3GPP TSG RAN WG1 Meeting#105-e			R1-2105843

e-Meeting, May 19th – May 27th, 2021

Agenda item:	8.12.2

Source:	Chengdu TD Tech, TD Tech

Title:	Discussion on reliability for RRC_CONNECTED UEs

Document for:	Discussion and Decision

- Introduction

The work item “NR Multicast and Broadcast Services” [1] was agreed in RAN#88-e to provide MBS in a cell. Based on the agreements on NR MBS so far, the discussion on the reliability for RRC_CONNECTED UEs is made with the focus on the HARQ-ACK feedback for SPS MRB of the PTM bearer and the related proposals are suggested in the contribution.

- HARQ-ACK feedback for SPS MRB of PTM bearer

In order to distinguish the RBs of an MBS session from the RBs of a unicast session, MRB is used to indicate the RB of an MBS session.

For an MBS session with delivery mode 1, the PTM bearer and the PTP bearer per UE can be configured to send the MBS session to a group of RRC_CONNECTED UEs. G-RNTI and G-CS-RNTI can be allocated to the MBS session for identifying each non-SPS MRB and each SPS MRB of the PTM bearer of the MBS session over Uu respectively. For the MBS session, the PTP bearer of a UE uses C-RNTI and CS-RNTI to indicate each non-SPS MRB and each SPS MRB of the MBS session over Uu respectively.

In the last RAN1 meeting, the HARQ-ACK feedback for each SPS MRB of the PTM bearer was agreed. The detailed HARQ-ACK feedback method is discussed as follows.

For each SPS MRB of an MBS session, the SPS group-common PDSCH scrambled with G-CS-RNTI is used by the PTM bearer to send the data of the SPS MRB periodically. When the initial transmission of a TB of the SPS MRB is NACKed by at least one UE, the retransmission of the TB should be provided.

If the ACK/NACK based HARQ-ACK feedback is supported, the different RRC_CONNECTED UEs use the different SPS PUCCH resources to feed back the ACK/NACK information of each TB. gNB can detect the ACK/NACK information from each UE. Therefore, gNB can make the retransmission of the NACKed TB with the PTM bearer and/or the PTP bearer.

If the NACK-only based HARQ-ACK feedback is supported, the different RRC_CONNECTED UEs share the same SPS PUCCH resources to feed back the NACK-only information. gNB can detect each PUCCH in the SPS PUCCH resources. But for each detected PUCCH, gNB can’t know which UE/UEs sent the PUCCH. Therefore, gNB can only use the PTM bearer to retransmit the NACKed TB.

Based on the above discussion, the following proposals are suggested.

Proposal 1: Support the following HARQ-ACK feedback methods for each SPS MRB of the PTM bearer of an MBS session:

- ACK/NACK based HARQ-ACK feedback with the different SPS PUCCH resources for the different RRC_CONNECTE UEs.

  - FFS: details for the ACK/NACK based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

- NACK-ONLY based HARQ-ACK feedback with the different RRC_CONNECTED UEs sharing the same SPS PUCCH resources

  - FFS: details for the NACK-ONLY based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

  - FFS: whether or not other information can be fed back with the NACK-ONLY information.

Proposal 2: The HARQ-ACK feedback method for each SPS MRB of the PTM bearer of an MBS session can be set independently. The HARQ-ACK feedback method for each SPS MRB can be signalled to UE by

- Option 1: Use RRC signalling to enable one of the HARQ-ACK feedback method

- Option 2:

  - Use RRC signalling to enable one of the HARQ-ACK feedback method

  - Use DCI format to enable/disable the HARQ-ACK feedback method dynamically

- FFS: Which option to use

Proposal 3: For the ACK/NACK based HARQ-ACK feedback for an SPS MRB of the PTM bearer of the MBS session, both the PTP bearer and the PTM bearer with PTM scheme 1 can be used for the retransmission of the NACKed TB.

Proposal 4: For the NACK-ONLY based HARQ-ACK feedback for an SPS MRB of the PTM bearer of the MBS session, the PTM bearer with PTM scheme 1 can be used for the retransmission of the NACKed TB, where PTM scheme 1 can use beam sweeping or partial beam sweeping.

- Conclusion

Based on the discussion in the above section, the following proposals are suggested.

Proposal 1: Support the following HARQ-ACK feedback methods for each SPS MRB of the PTM bearer of an MBS session:

- ACK/NACK based HARQ-ACK feedback with the different SPS PUCCH resources for the different RRC_CONNECTE UEs.

  - FFS: details for the ACK/NACK based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

- NACK-ONLY based HARQ-ACK feedback with the different RRC_CONNECTED UEs sharing the same SPS PUCCH resources

  - FFS: details for the NACK-ONLY based HARQ-ACK feedback: SPS PUCCH resource allocation, timing between SPS PDSCH and SPS PUCCH, other aspects.

  - FFS: whether or not other information can be fed back with the NACK-ONLY information.

Proposal 2: The HARQ-ACK feedback method for each SPS MRB of the PTM bearer of an MBS session can be set independently. The HARQ-ACK feedback method for each SPS MRB can be signalled to UE by

- Option 1: Use RRC signalling to enable one of the HARQ-ACK feedback method

- Option 2:

  - Use RRC signalling to enable one of the HARQ-ACK feedback method

  - Use DCI format to enable/disable the HARQ-ACK feedback method dynamically

- FFS: Which option to use

Proposal 3: For the ACK/NACK based HARQ-ACK feedback for an SPS MRB of the PTM bearer of the MBS session, both the PTP bearer and the PTM bearer with PTM scheme 1 can be used for the retransmission of the NACKed TB.

Proposal 4: For the NACK-ONLY based HARQ-ACK feedback for an SPS MRB of the PTM bearer of the MBS session, the PTM bearer with PTM scheme 1 can be used for the retransmission of the NACKed TB, where PTM scheme 1 can use beam sweeping or partial beam sweeping.

- Reference

- RP-201038, Revised Work Item on NR Multicast and Broadcast Services, RAN Meeting #88-e

- R1-2101637, Study on the reliability for RRC_CONNECTED UEs, Chengdu TD Tech, TD Tech, RAN1 Meeting #104-e