3GPP TSG RAN WG1 #105-e	R1-21xxxxx

e-Meeting, May 10th – 27th, 2021

Agenda item:		7.2.12

Source:	Moderator (China Telecom)

Title:	Summary of Rel-16 uplink Tx switching

Document for:		Discussion and Decision

# Introduction

In RAN #85 meeting, the following objective was added in the revised WID of “RF requirements for NR frequency range 1” [1].

- Specify UE requirements to allow switching between case 1 and case 2 as below for two uplink carriers case inter-band EN-DC without SUL, inter-band UL CA and standalone SUL for UE supporting maximum two concurrent transmission

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

    - UE RF requirements, e.g., time mask RF requirements and other necessary RF requirements if any

    - The options agreed at RAN4 #92 in R4-1910531 can be considered as starting point

    - Study if there are any impact to interruption and delay requirements, and specify the RRM requirements if needed

    - RAN1 will further study by Dec 2019 if there are any RAN1 potential impacts based on RAN4 LS if any

    - No new TDM pattern will be defined, i.e. scheduling-based switching is assumed.

    - Finalization of RAN4 requirements and approval of RAN4 CRs shall be based on RAN1 LS

    - Strive to minimize RAN1 impact.

    - Strive to achieve no impact to RAN1 E-UTRAN spec

    - Strive to avoid defining location of switching period impacting RAN1 spec

    - Define per band per band combination or per band combination UE capability signaling if needed

Note 1: Only addressing the case of co-located and synchronized network deployment for the two UL carriers

Note 2: Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA

Note 3: The above objectives will not relax the existing requirements specified in Rel-15 38.101-3 for band combinations allowing single uplink transmission

Note 4: The UE is configured with two different uplink carrier frequencies.

This contribution summarizes the maintenance issues for Rel-16 uplink Tx switching.

# Summary of maintenance issues

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: Correction on RRC parameter “tdm-PatternConfig-r15” and “tdm-PatternConfig-r16” in TS 38.214 | R1-2104731 | Suggest to be discussed in RAN1#105-e |
| Issue#2: Clarification on CA based SRS carrier switching | R1-2104325, R1-2104653, R1-2105925 | Suggest to be discussed in RAN1#105-e |
| Issue#3: Clarification on SRS antenna switching | R1-2104653 | This was discussed in RAN1#104b-e. It seems no consensus can be reached on whether it is a valid case. Suggest NOT to be discussed in RAN1#105-e |

| Company | Views |
| --- | --- |
| CATT | We are ok to discuss about Issue#1 and issue#2. Regarding issue#3, we agree with FL ‘s suggestion on “NOT to be discussed in RAN1#105-e” |
| OPPO | Support to discussion Issue#1. In MR-DC/CA session, the similar correction was agreed only for power control part. We also need correction for UL Tx switching |

# References

- RP-192282, WID revision: RF requirements for NR frequency range 1 (FR1), Huawei, HiSilicon, China Telecom, RAN #85, Sep. 2019.

- R1-2104325, Remaining Issues of Rel-16 UL Tx Switching, ZTE, RAN1#105-e, May 10th – 27th, 2021.

- R1-2104653, Remaining issues for 1Tx-2Tx switching, Qualcomm Incorporated, RAN1#105-e, May 10th – 27th, 2021.

- R1-2104731, Text Proposals for Tx Switching between Two Uplink Carriers, OPPO, RAN1#105-e, May 10th – 27th, 2021.

- R1-2105925, Discussion on the remaining problems of supporting Tx switching between two uplink carriers, Huawei, HiSilicon, RAN1#105-e, May 10th – 27th, 2021.