3GPP TSG RAN WG1 #106-e	R1-21xxxxx

e-Meeting, August 16th – 27th, 2021

Agenda item:		7.2.12

Source:	Moderator (China Telecom)

Title:	Summary of Rel-16 uplink Tx switching

Document for:		Discussion and Decision

# Introduction

In RAN #85 meeting, the following objective was added in the revised WID of “RF requirements for NR frequency range 1” [1].

- Specify UE requirements to allow switching between case 1 and case 2 as below for two uplink carriers case inter-band EN-DC without SUL, inter-band UL CA and standalone SUL for UE supporting maximum two concurrent transmission

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

    - UE RF requirements, e.g., time mask RF requirements and other necessary RF requirements if any

    - The options agreed at RAN4 #92 in R4-1910531 can be considered as starting point

    - Study if there are any impact to interruption and delay requirements, and specify the RRM requirements if needed

    - RAN1 will further study by Dec 2019 if there are any RAN1 potential impacts based on RAN4 LS if any

    - No new TDM pattern will be defined, i.e. scheduling-based switching is assumed.

    - Finalization of RAN4 requirements and approval of RAN4 CRs shall be based on RAN1 LS

    - Strive to minimize RAN1 impact.

    - Strive to achieve no impact to RAN1 E-UTRAN spec

    - Strive to avoid defining location of switching period impacting RAN1 spec

    - Define per band per band combination or per band combination UE capability signaling if needed

Note 1: Only addressing the case of co-located and synchronized network deployment for the two UL carriers

Note 2: Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA

Note 3: The above objectives will not relax the existing requirements specified in Rel-15 38.101-3 for band combinations allowing single uplink transmission

Note 4: The UE is configured with two different uplink carrier frequencies.

This contribution summarizes the maintenance issues for Rel-16 uplink Tx switching.

# Summary of maintenance issues

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: UL MIMO coherence for Tx switching | R1-2106501, R1-2106786, R1-2107307, R1-2107960, R1-2107961, R1-2107962 | Depend on the outcome of preparation phase for AI 5 |
| Issue#2: CA based SRS carrier switching | R1-2106501, R1-2107322 | Suggest to be discussed in AI 7.2.12 in RAN1#106-e |
| Issue#3: Back-to-back switching with SRS carrier switching | R1-2106501 | Suggest to be discussed in AI 5.1 in RAN1#106-e. |

| Company | Views |
| --- | --- |
| CATT | Issue#1 and Issue#2 can be handled under AI 7.2.12. Issue#3 can continue to be discussed under AI 5.1. |
| ZTE | Ok to discuss all these three issues. For Issue#3, if something is going to be introduced in Rel-16, something with less impact is preferred. |
| Huawei, HiSilicon | Agree with the arrangement. On issue#1, we prefer to discuss it in this session. On Issue#3, we feel it has impacted on Rel-16 as well, so we prefer to discuss it in this session. Speaking of less spec impact, please note that a very similar timeline for general SRS carrier switching was agreed in CR R1-2104043 in RAN1#104b-e, which can be good reference and reused in some way. |

# References

- RP-192282, WID revision: RF requirements for NR frequency range 1 (FR1), Huawei, HiSilicon, China Telecom, RAN #85, Sep. 2019.

- R4-2107765, LS on UL MIMO coherence for Tx switching between two carriers, RAN4, China Telecom, RAN4 #99-e, 19 - 27 May, 2021.

- R1-2106501, Discussion on the remaining problems of supporting Tx switching between two uplink, Huawei, HiSilicon, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2106786, Discussion on RAN4 LS on UL MIMO coherence for Tx switching between two carriers, ZTE, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107307, Discussion of RAN4 LS on UL MIMO coherence for Tx switching between two carriers, Qualcomm Incorporated, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107322, Remaining issues for 1Tx-2Tx switching, Qualcomm Incorporated, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107960, Discussion on LS on UL MIMO coherence for Tx switching between two carriers, vivo, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107961, [Draft] Reply LS on UL MIMO coherence for Tx switching between two carriers, vivo, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107962, [Draft] CR on UL MIMO coherence for Tx switching between two carriers, vivo, RAN1 #106-e, August 16th – 27th, 2021.