3GPP TSG RAN WG1 #106-e	R1-21xxxxx

e-Meeting, August 16th – 27th, 2021

Agenda item:		7.2.12

Source:	Moderator (China Telecom)

Title:	Summary of Rel-16 uplink Tx switching

Document for:		Discussion and Decision

# Introduction

In RAN #85 meeting, the following objective was added in the revised WID of “RF requirements for NR frequency range 1” [1].

- Specify UE requirements to allow switching between case 1 and case 2 as below for two uplink carriers case inter-band EN-DC without SUL, inter-band UL CA and standalone SUL for UE supporting maximum two concurrent transmission

| Case 1 | 1 Tx on carrier 1 and 1 Tx on carrier 2 |
| --- | --- |
| Case 2 | 0 Tx on carrier 1 and 2 Tx on carrier 2 |

    - UE RF requirements, e.g., time mask RF requirements and other necessary RF requirements if any

    - The options agreed at RAN4 #92 in R4-1910531 can be considered as starting point

    - Study if there are any impact to interruption and delay requirements, and specify the RRM requirements if needed

    - RAN1 will further study by Dec 2019 if there are any RAN1 potential impacts based on RAN4 LS if any

    - No new TDM pattern will be defined, i.e. scheduling-based switching is assumed.

    - Finalization of RAN4 requirements and approval of RAN4 CRs shall be based on RAN1 LS

    - Strive to minimize RAN1 impact.

    - Strive to achieve no impact to RAN1 E-UTRAN spec

    - Strive to avoid defining location of switching period impacting RAN1 spec

    - Define per band per band combination or per band combination UE capability signaling if needed

Note 1: Only addressing the case of co-located and synchronized network deployment for the two UL carriers

Note 2: Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA

Note 3: The above objectives will not relax the existing requirements specified in Rel-15 38.101-3 for band combinations allowing single uplink transmission

Note 4: The UE is configured with two different uplink carrier frequencies.

This contribution summarizes the maintenance issues for Rel-16 uplink Tx switching.

# Summary of maintenance issues

| Issues | Related contributions | Initial assessment |
| --- | --- | --- |
| Issue#1: UL MIMO coherence for Tx switching | R1-2106501, R1-2106786, R1-2107307, R1-2107960, R1-2107961, R1-2107962 | Depend on the outcome of preparation phase for AI 5 |
| Issue#2: CA based SRS carrier switching | R1-2106501, R1-2107322 | Suggest to be discussed in AI 7.2.12 in RAN1#106-e |
| Issue#3: Back-to-back switching with SRS carrier switching | R1-2106501 | Suggest to be discussed in AI 5.1 in RAN1#106-e. |

| Company | Views |
| --- | --- |
| CATT | Issue#1 and Issue#2 can be handled under AI 7.2.12. Issue#3 can continue to be discussed under AI 5.1. |
| ZTE | Ok to discuss all these three issues. For Issue#3, if something is going to be introduced in Rel-16, something with less impact is preferred. |
| Huawei, HiSilicon | Agree with the arrangement. On issue#1, we prefer to discuss it in this session. On Issue#3, we feel it has impacted on Rel-16 as well, so we prefer to discuss it in this session. Speaking of less spec impact, please note that a very similar timeline for general SRS carrier switching was agreed in CR R1-2104043 in RAN1#104b-e, which can be good reference and reused in some way. |
| Qualcomm | Issue #1 Propose to discuss this under AI 7.2.6 which follows the initial assessment from Mr. Chair. Issue #2 As Rel-16 UL Tx switching has been delayed for several meetings already and the ambiguity of SRS carrier switching might not be able to be solved in a short time, we propose to conclude that the combination of SRS carrier switching, and UL Tx switching is not supported in R16. Furthermore, we would suggest solving the issues in Rel-17 for this combined feature. Issue #3 We support FL’s proposal. |

FL comments:

For issue #1, according to Chair’s recommendation after 1st round of comments under AI 5, it is suggested to be handled under AI 7.2.12.

For issue #2, it seems the majority are fine to continue the discussion under AI 7.2.12 in RAN1#106-e.

For issue #3, it seems the majority are fine to continue the discussion in under AI 5.1 in RAN1#106-e. Regarding the relation with Rel-16, suggest to add one note that the agreements or conclusions may also be applicable to Rel-16.

Proposal:

- The following issues are discussed under AI 7.2.12 in RAN1#106-e

  - Issue#1: UL MIMO coherence for Tx switching

  - Issue#2: CA based SRS carrier switching

- The following issues is discussed under AI 5.1 in RAN1#106-e

  - Issue#3: Back-to-back switching with SRS carrier switching

  - Note: the agreements or conclusions may also be applicable to Rel-16.

| Company | Views |
| --- | --- |
| CATT | Basically we are fine with FL proposal. Regarding Note description of Issue#3, our modification is as follow: Note: if necessary, the agreements or conclusions may also be applicable to Rel-16. |
| Qualcomm | For Issue #2, we don’t think any discussion under this AI would provide any help as carrier prioritization during SRS carrier switching would be handled in another parallel email thread. Given most companies reached the consensus on the dependency with SRS carrier switching CR thread since RAN1 #104b-emeeting, we strongly suggest NOT handle this issue under AI 7.2.12. Moreover, given Rel-16 already was frozen more than one year, we would suggest concluding the combination of SRS carrier switching and UL Tx switching is not supported in R16. Furthermore, we would suggest solving the issues in Rel-17 for this combined feature. |

# References

- RP-192282, WID revision: RF requirements for NR frequency range 1 (FR1), Huawei, HiSilicon, China Telecom, RAN #85, Sep. 2019.

- R4-2107765, LS on UL MIMO coherence for Tx switching between two carriers, RAN4, China Telecom, RAN4 #99-e, 19 - 27 May, 2021.

- R1-2106501, Discussion on the remaining problems of supporting Tx switching between two uplink, Huawei, HiSilicon, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2106786, Discussion on RAN4 LS on UL MIMO coherence for Tx switching between two carriers, ZTE, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107307, Discussion of RAN4 LS on UL MIMO coherence for Tx switching between two carriers, Qualcomm Incorporated, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107322, Remaining issues for 1Tx-2Tx switching, Qualcomm Incorporated, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107960, Discussion on LS on UL MIMO coherence for Tx switching between two carriers, vivo, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107961, [Draft] Reply LS on UL MIMO coherence for Tx switching between two carriers, vivo, RAN1 #106-e, August 16th – 27th, 2021.

- R1-2107962, [Draft] CR on UL MIMO coherence for Tx switching between two carriers, vivo, RAN1 #106-e, August 16th – 27th, 2021.