3GPP TSG RAN WG1 #106-e	R1-2106424

e-Meeting, August 16th – 27th, 2021

3GPP TGS-RAN WG4 Meeting #99e	R4-2107950
Electronic Meeting, May 19 – 27, 2021

Title:	LS reply on maximum UE EIRP and conducted power

Response to:	R1-2104061

Release:	Rel-17

Work Item:	NR_ext_to_71GHz

Source:	TSG RAN WG4

To:	TSG RAN WG1

Contact person:	Aida Vera Lopez

aida.l.vera.lopez@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN4 would like to thank RAN1 for the LS. Before addressing the question, we provide a brief background on our discussions. Currently, power classes are a package of four parameters: minimum peak EIRP, EIRP spherical coverage, maximum TRP and maximum EIRP (regulatory defined, captured in specs for reference). Note that all of the parameters are radiated. RAN4 specification of the minimum peak EIRP and spherical coverage performance requirements is done per-band, with a reference UE type. The maximum TRP parameter considers both UL co-channel interference and regulatory limits. At this time, RAN4 is reviewing the regulatory requirements (maximum EIRP and maximum TRP) for this frequency range.

RAN1 description and question

Answer

RAN4 can confirm that the current regulatory limits, i.e. max EIRP and max TRP, are higher than the above values. We further note that for the 52.6 to 71 GHz frequency range, regulations in some regions also specify a maximum spectral power density (EIRP).

Regarding what minimum peak EIRP value RAN4 will specify for a power class in this frequency range, it is premature to answer at this stage. RAN4 will continue discussing the power classes, reference UE type, antenna array size, and design considerations to make this assessment. While power class performance is TBD, an FWA UE is expected to yield the highest minimum peak EIRP, and it may be specified around 25 dBm or higher. However, further study is needed to confirm this and provide an exact minimum peak EIRP value. A power ranging from minimum peak EIRP to below the regulatory maximum EIRP limit, is technically valid for the UE to transmit out.

For additional context, different radiated output powers in 38.101-2 are summarized in the table below. Please note that there are no conducted output power requirements defined for FR2 in RAN4 specifications.

Table 1. FR2 minimum peak EIRP requirements

| Power class | Max TRP [dBm] | FR2 band | Min peak EIRP [dBm] | Max EIRP [dBm] |
| --- | --- | --- | --- | --- |
| Power class 1 Fixed wireless access UE | 35 | n257/n258/n261 | 40.0 | 55 |
|  |  | n260 | 38.0 |  |
| Power class 2 Vehicular UE | 23 | n257/n258/n261 | 29.0 | 43 |
| Power class 3 Handheld UE | 23 | n257/n258/n261 | 22.4 | 43 |
|  |  | n260 | 20.6 |  |
|  |  | n259 | 18.7 |  |
|  |  | n262 | 16.0 |  |
| Power class 4 High-power non-handheld UE | 23 | n257/n258/n261 | 34.0 | 43 |
|  |  | n260 | 31.0 |  |
| Power class 5 Fixed wireless access UE | 23 | n257 | 30.0 | 43 |
|  |  | n258 | 30.4 |  |

TRP: Total Radiated Power

EIRP: Effective Isotropic Radiated Power

Considering RAN4 is in the early stages of our discussions, this is the information we can provide at this time. Further guidance will be provided as power class discussions progress in RAN4.

# 2	Actions

To 3GPP RAN1

ACTION: RAN4 respectfully asks RAN1 to take the information detailed above into account.

# 3	Dates of next TSG RAN WG4 meetings

TSG RAN WG4 Meeting #100-e	August 16 – 27, 2021	          E-meeting

TSG RAN WG4 Meeting #101-e	November 1 – 12, 2021	          E-meeting

# 4	References

- R1-2104061, “LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band,” RAN1, RAN1 #104Bis-e, April 2021

- 3GPP TS 38.101-2, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” version 17.1.0