3GPP TSG RAN WG1 #106bis-e		R1-2108705

e-Meeting, October 11th – 19th, 2021

3GPP TSG-RAN4 Meeting #100-e 		R4-2115347

E-meeting, 16th Aug – 27th Aug, 2021

Title:	Reply LS on NTN UL time and frequency synchronization requirements

Response to:	R1-2102263 LS on NTN UL time and frequency synchronization requirements

Release:	Rel-17

Work Item:	NR_NTN_solutions-Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

- Name:			Xuhua Tao

- E-mail Address:	taoxuhua@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator:  mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

RAN4 would like to thank RAN1 for the LS on NTN UL time and frequency synchronization requirement for initial access (i.e. PRACH transmission) and UL transmissions in RRC_CONNECTED state (R1-2102263). RAN4 is currently working on defining FR1 NTN UE timing requirement as per RAN plenary#92 decision that Ka band and FR2 aspects to be postponed until March 2022. Question 1 in R1-2102263 has been discussed and achieved the following conclusions:

Question 1: What are the NTN UL time synchronization requirements?

- For initial access (i.e. PRACH transmission)

- For UL transmissions in RRC Connected State

[RAN4]: The UL timing requirements for NTN UE will be specified in RAN4 are summarized as follows:

- For initial access (i.e. PRACH transmission)

- Initial transmit timing error requirement

The UE initial transmission timing error requirement for initial access is defined when the PRACH transmission or the msgA transmission. And the UE initial transmission timing error shall be less than or equal to Te_NTN where the timing error limit value Te_NTN is specified in Table 1. In order to derive Te_NTN, RAN4 assumed Te_NTN = Te + Te_GNSS + Te_SAT, where,

- Te is the legacy timing error

- Te_GNSS is the GNSS accuracy

- Te_SAT is the serving-satellite position estimation error

Table 1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| 1 | 15 | 15 | [X1]*64*Tc |
|  |  | 30 | [X2]*64*Tc |
|  |  | 60 | [X3]*64*Tc |
|  | 30 | 15 | [Y1]*64*Tc |
|  |  | 30 | [Y2]*64*Tc |
|  |  | 60 | [Y3]*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 |  |  |  |

- RAN4 will continue to discuss the value of (X1-X3) and (Y1-Y3) for Te_NTN.

- FFS on whether the requirement will be developed for all UL SCSs

- For UL transmissions in RRC_CONNECTED state

- Initial transmit timing error requirement

The UE initial transmission timing error requirement for RRC_CONNECTED is defined when it is the first transmission in a DRX cycle for PUCCH, PUSCH and SRS in RRC_CONNECTED state. The requirement is the same as Table 1.

- Gradual timing adjustment requirement

When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE is required to adjust its timing to within Te_NTN. The definition of reference timing is still under discussion. All adjustments made to the UE uplink timing shall follow these rules:

- The maximum amount of the magnitude of the timing change in one adjustment shall be Tq_NTN.

- The minimum aggregate adjustment rate shall be Tp_NTN per [X]ms.

  - FFS whether X can be different for different types of satellites, altitude, etc.

- The maximum aggregate adjustment rate shall be Tq_NTN per [Y]ms.

  - FFS whether Y can be different for different types of satellites, altitude, etc.

| Frequency Range | SCS of uplink signals (kHz) | Tq_NTN | Tp_NTN |
| --- | --- | --- | --- |
| 1 | 15 | [Z1]*64*Tc | [Z1]*64*Tc |
|  | 30 | [Z2]*64*Tc | [Z2]*64*Tc |
|  | 60 | [Z3]*64*Tc | [Z3]*64*Tc |
| NOTE: Tc is the basic timing unit defined in TS 38.211 |  |  |  |

  - RAN4 will continue to discuss the value of Z1-Z3, and FFS whether different values can be defined for different types of satellites, altitude, etc.

  - FFS on whether the requirement will be developed for all UL SCSs

  - FFS on whether the requirement will be applicable for LEO

- TA adjustment accuracy requirement

RAN4 is working on how to define TA adjustment accuracy in response to timing advance command.

# 2. Actions:

To TSG RAN WG1

ACTION: 	RAN4 respectfully ask RAN1 to take the above information into account.

# 3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG4 Meeting #101-e        01 – 12 Nov 2021	Electronic Meeting

TSG-RAN WG4 Meeting #102-e        21 – 25 Feb 2022	Athens