3GPP TSG RAN WG1 Meeting #107-e											      R1-2110767

e-Meeting, November 11th – 19th, 2021

Agenda Item:		8.1.4

Source:				InterDigital, Inc.

Title:						Remaining Details on CSI Enhancements for NCJT MTRP

Document for:		Discussion and Decision

# Background

The MIMO WID was approved in RAN#86 [1] and it was agreed to study enhancements on CSI measurement and reporting:

- Enhancement on CSI measurement and reporting: - Evaluate and, if needed, specify CSI reporting for DL multi-TRP and/or multi-panel transmission to enable more dynamic channel/interference hypotheses for NCJT, targeting both FR1 and FR2

In the last meeting [2], further progress was made on this topic, however some issues remained for FFS. In this contribution, we provide our views on the remaining details for the NCJT CSI reporting enhancement.

# CSI ENHANCEMENTS FOR NCJT

PMI sharing

For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT measurement hypothesis, study whether to support non-PMI CSI reporting with reportQuantity set to "CRI-RI-CQI" in Rel-17 - Related details, if needed, are to be discussed in RAN1#106bis. - Interested companies are encouraged to share details and related specification impact if support

It has been agreed to support a CSI report with multiple measurement hypothesis which includes sTRP and NCJT assumptions. With option 1, a CSI report is associated with X sTRP measurement hypothesis and one NCJT hypothesis, where X=0,1, or 2. The details related to the PMI reporting and sharing were discussed during the last meeting but no conclusion was made.

The CSI overhead grows large considering all the hypothesis. For example, the case with two single TRP assumptions and NCJT assumption (Option 1 with X+1, X=2) requires one PMI per TRP at least for the sTRP case. For NCJT, PMIs per TRP are also necessary and they are not necessarily the same as the ones chosen for the sTRP case. For example, in the NCJT case, the inter-layer interference needs to be considered when selecting the PMIs. An optimal precoder for sTRP may generate some interference towards the signal received from the other TRP that it may not still be optimal for the NCJT hypothesis.

However, it is also likely that the same PMI is optimal for either hypothesis. For example, due to a CBSR or UE location, only a few PMIs may have a signal quality greater than a threshold that satisfies the UE. Then, CSI reporting overhead can be reduced by sharing the PMI reported for sTRP hypothesis and also apply it for the NCJT hypothesis. Hence, the network should be able to reconfigure the CSI reporting configuration by enabling PMI sharing. The sharing can be turned on or off by RRC signalling. Similarly, this also applies to RI sharing between hypothesis.

Observation 1: The optimal RIs/PMIs for sTRP and NCJT measurement hypothesis are not necessarily the same.

Proposal 1: Support sharing of RI/PMI for sTRP and NCJT CSI. The sharing can be RRC configured on/off.

UCI payload construction priority

To confirm the order of UCI payload construction for reported CSIs, study following Alternatives and down-select one or more Alternative(s) for required specification changes in RAN1 106bis: - Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214. - Alt 2: modify the table of priority reporting levels for Part 2 CSI, i.e., Table 5.2.3-1 in 38.214. - Alt 4: modify mapping order of CSI fields of one CSI report, i.e., Table 6.3.2.1.2-3/4/5 in 38.212

A UE is expected to report CSI on PUCCH or PUSCH resources. If the CSI overhead is too large to fit in the available UL resources, then CSI omission, as defined in Rel-16, can be used. The omission rules are defined for the sTRP case because that was the only hypothesis supported in the single CSI reporting setting. For Rel-17, different alternatives have been proposed for handling the priority of different measurement hypothesis in the single CSI reporting setting. Similarly, this issue was discussed last meeting but not decided yet.

In Alt 2, the table of priority reporting levels is modified. The table defines the priority of wideband over subband reporting and prioritizing even over odd subbands. Introducing priority levels specific to different hypothesis in this context seems cumbersome to introduce in the specification. In Alt 4, the ordering of the CSI fields is modified which similarly would produce significant change to the specification.

Our preference is for Alt 1 that is based on modifying the priority equation. In the specifications 38.214, the priority equation is defined for a CSI report, and a first CSI report is given priority over another CSI report if the associated priority value is lower for the first report than the second report. If each hypothesis is treated as a separate report within the same CSI configuration, then each measurement hypothesis is assigned a priority level and the UE determines the payload to include based on the hypothesis with the lowest priority. The sTRP hypothesis should be prioritized over the NCJT hypothesis at least for up to X=1 because obtaining a reliable single-TRP channel quality is often more important than that of a single NCJT as it may better to ensure UE connectivity. With X=2, the NCJT hypothesis can be prioritized to ensure that at least one of the reported CSI corresponds to the NCJT hypothesis. We think it is more straightforward to accomplish this using a modified priority equation where a different priority number is assigned for the CSI corresponding to sTRP from first TRP, sTRP from second TRP, and NCJT.

Observation 2: Obtaining a reliable single-TRP channel quality is often more important than that of a single NCJT.

Proposal 2: Support Alt 1: modify priority equation, i.e., Section 5.2.5 in 38.214.

# Conclusions

This contribution discussed the remaining details on CSI enhancements for NCJT MTRP. Based on the presented discussion, we make the following observations and proposals:

Observation 1: The optimal RIs/PMIs for sTRP and NCJT measurement hypothesis are not necessarily the same.

Observation 2: Obtaining a reliable single-TRP channel quality is often more important than that of a single NCJT.

Proposal 1: Support sharing of RI/PMI for sTRP and NCJT CSI. The sharing can be RRC configured on/off.

Proposal 2: Support Alt 1; modify priority equation, i.e., Section 5.2.5 in 38.214.

# References

- RP-193133, “New WI: Further enhancements on MIMO for NR”, Samsung, 3GPP RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #106b-e, October 11th – 19th, 2021