3GPP TSG RAN WG1 Meeting #107-e	R1-2110794

e-Meeting, November 11th –19th, 2021

Agenda Item:     8.16.8

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NR coverage enhancement

Document for:	Discussion and Decision

Introduction

In this contribution, our views on UE capabilities of NR coverage enhancement are provided.

Discussion on NR coverage enhancement UE capabilities

Whether the features are Mandatory or Optional

As enhanced features in Rel-17, considering UE capabilities and flexibility, all the UE features for coverage enhancement listed in [1] should be optional with capability signaling.

Specifically, for FG 30-6 related to Msg3 repetition, the feature can realize optional based on whether to access via separate RACH resources without capability signaling. However, capability signaling after beyond the RACH procedure can help the gNB properly configure RACH resources for different purposes. Thus, FG 30-6 should be optional with capability signaling.

Proposal 1: For FG 30-1 to FG 30-6, all the UE features should be optional with capability signaling.

The need of FDD/TDD differentiation

For all the features, there seems no justification or evidence to support the need for FDD/TDD differentiation. The FDD/TDD differentiation is unnecessary.

Proposal 2: For FG 30-1 to FG 30-6, the FDD/TDD differentiation is unnecessary.

The structure of UE features about DMRS bundling

On the basis of the agreements reached in AI 8.8.1.3, whether DMRS bundling for repetition type B and TBoMS and so on are supported, should depend on the respective UE capabilities. The structure of FG 30-4a to FG 30-4c should be kept.

The capabilities of DMRS bundling for back-to-back case and non-back-to-back case may be different, because the gap between transmissions of non-back-to-back case brings in other uncertainties that may affect power consistency and phase continuity. Actually, according to the RAN4’s input, compared to the  non-back-to-back transmissions with the gap consist of unscheduled symbols, it is more difficult for the UE to maintain power consistency and phase continuity when there are other UL signals/channels inserted in the gap between transmissions, even with same setting. Thus, it is preferable that different use cases have separate FGs. At least the feature for non-back-to-back with other UL signals/channels inserted in the gap between repetitions with same setting should be separate.

Besides, in generally, it is reasonable to set the features for PUCCH and PUSCH separately.

Proposal 3: The structure of UE features about DMRS bundling should be as follows:

- DMRS bundling for each type of transmission, e.g. repetition type A, repetition type B, TBoMS, should have separate FG.

- Introduce additional FGs for non-back-to-back with the gap consists of unscheduled symbols and non-back-to-back with other UL signals/channels inserted in the gap between repetitions with same setting, separately.

- DMRS bundling for PUSCH and PUCCH should have separate FG.

The value of maximum duration

The values of the maximum duration are related to the phase continuity and power consistency tolerance, which is under discussion in RAN4 now. The tolerance may be related to modulation order. Thus, whether and how to report different value of the maximum duration for DMRS bundling for different modulation orders is up to RAN4.

For the back-to-back transmissions and non-back-to-back transmissions, although a single value of maximum duration may be sufficient, it is safer to wait for RAN4’s input.

Proposal 4: For FG 30-4, whether and how to report different value of the maximum duration for DMRS bundling for (a) different modulation orders (b) back-to-back and non-back-to-back, is up to RAN4.

The need of FR1/FR2 differentiation for features related to DMRS bundling

For a given tolerance, the CFOs of different operating frequency are different, which may cause the “compensation leftover for frequency error” be different. And UE’s implementation on different FR range to maintain the phase continuity could be different. Thus, the differentiation of FR1/FR2 for the feature of maximum duration is necessary.

Proposal 5: For FG 30-4, FR1/FR2 differentiation is necessary.

For FG 30-4x, as previously described, the RF characteristics on different FR range are different, so the capabilities to support DMRS bundling on different FRs may be different for the same UE. The differentiation of FR1/FR2 is necessary for the features related to supporting DMRS bundling.

Proposal 6: For FG 30-4x, FR1/FR2 differentiation is necessary.

The type of granularity

First of all, it should be noted that, there are certain reasons to define that the FG 30-4 is set per band. As previously described, the compensation leftover for frequency error and RF characteristics may be different for different bands. However, analyzing RF and determining the values of the maximum duration for all bands will result in very heavy workload and is unnecessary. Considering above, it is a good choice to define the UE feature of maximum duration, i.e. FG 30-4, based on the granularity of Per UE and with FR1/FR2 differentiation.

For other features except for FG 30-4, there is no particular reason to apply other granularities. Per UE is sufficient.

Proposal 7: All the UE feature for NR coverage enhancement should be based on the granularity of Per UE.

The structure of the features related to enhanced PUSCH repetition type A and TBoMS

In our view, only one FG for both CG and DG is sufficient. But we are also open to splitting that into CG and DG, or into type 1 CG, type 2 CG and DG, which enables more flexibility in supporting and reporting UE capabilities. The details of components and prerequisite feature groups are up to the structure.

Conclusions

In this contribution, we discussed the various aspects for the NR coverage enhancement UE capabilities for Rel-17. The proposals are listed in the following:

Proposal 1: For FG 30-1 to FG 30-6, all the UE features should be optional with capability signaling.

Proposal 2:  For FG 30-1 to FG 30-6, the FDD/TDD differentiation is unnecessary.

Proposal 3: The structure of UE features about DMRS bundling should be as follows:

- DMRS bundling for each type of transmission, e.g. repetition type A, repetition type B, TBoMS, should have separate FG.

- Introduce additional FGs for non-back-to-back with the gap consists of unscheduled symbols and non-back-to-back with other UL signals/channels inserted in the gap between repetitions with same setting, separately.

- DMRS bundling for PUSCH and PUCCH should have separate FG.

Proposal 4: For FG 30-4, whether and how to report different value of the maximum duration for DMRS bundling for (a) different modulation orders (b) back-to-back and non-back-to-back, is up to RAN4.

Proposal 5: For FG 30-4, FR1/FR2 differentiation is necessary.

Proposal 6: For FG 30-4x, FR1/FR2 differentiation is necessary.

Proposal 7: All the UE feature for NR coverage enhancement should be based on the granularity of Per UE.

- References

- R1-2108679, “Preliminary RAN1 UE features list for Rel-17 NR”, AT&T, NTT DOCOMO, INC., Aug. 2021.

- 3GPP TR 38.830, “Study on NR coverage enhancements,” Dec, 2020.

- R1-2109714, “Summary on UE features for NR coverage enhancement”, NTT DOCOMO, INC. Oct. 2021.

- Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is jointly coded with SLIV in TDRA list. A row index of the TDRA list is indicated by a Type 1 configured grant configuration. | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-17] | Yes | N/A | UE does not support PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. RV is cycled across transmission occasions. | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS No | No | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS whether to report different values of maximum duration for (a) different modulation orders (b) back-to-back and non-back-to-back |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS No | No Yes | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A. This is applicable for back-to-back transmissions. | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for back-to-back PUSCH repetition type A | [Per UE] | FFS No | No Yes | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B. This is applicable for back-to-back transmissions. | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for back-to-back PUSCH repetition type B | [Per UE] | FFS No | No Yes | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH. This is applicable for back-to-back transmissions. | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for back-to-back TB processing over multi-slot PUSCH | [Per UE] | FFS No | No Yes | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions. This is applicable for back-to-back transmissions. | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for back-to-back PUCCH repetitions | [Per UE] | FFS No | No Yes | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS No | No Yes | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced Support inter-slot frequency hopping with inter-slot bundling for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping with inter-slot bundling for PUCCH repetitions with DMRS bundling | [Per UE] | FFS No | No Yes | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the dynamic events that violate power consistency and phase continuity] | Support restarting DM-RS bundling after the dynamic events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the dynamic events that violate power consistency and phase continuity | [Per UE] | FFS No | No Yes | N/A |  | [Optional with capability signalling] |
| 30. NR_cov_enh | 30-4h | DMRS bundling for non-back-to-back transmissions with the gap consists of unscheduled symbols | Support for DMRS bundling for non-back-to-back transmissions across consecutive slots with unscheduled symbols in the gap between transmissions | 30-4a or 30-4b or 30-4c or 30-4d | Yes | N/A | UE does not support for DMRS bundling for non-back-to-back transmissions across consecutive slots with unscheduled symbols in the gap between transmissions | Per UE | No | Yes | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4i | DMRS bundling for non-back-to-back transmissions with other UL signals/channels in the gap between repetitions with same setting | Support for DMRS bundling for non-back-to-back transmissions across consecutive slots with other UL signals/channels in the gap between transmissions with same setting | 30-4a or 30-4b or 30-4c or 30-4d | Yes | N/A | UE does not support for DMRS bundling for non-back-to-back transmissions across consecutive slots with other UL signals/channels in the gap between transmissions with same setting | Per UE | No | Yes | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support dynamic PUCCH repetition indication | [4-23] | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Msg3 repetition | Support of Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. |  | Yes | N/A | UE does not support Msg3 repetition for Msg3 initial transmission and re-transmission in RRC connected mode. | [Per UE] | No FFS | No | N/A |  | [Optional with capability signalling] |