3GPP TSG RAN WG1 Meeting #107-e	R1-2110808

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.15.1

Source:	Huawei, HiSilicon

Title:	Discussion on time and frequency synchronization enhancement for IoT in NTN

Document for:	Discussion and Decision

Introduction

In RAN1#106bis-e, the following were agreed for time and frequency synchronization enhancement for IoT in NTN [1].

Agreement: The validity timer for UL synchronization is started/restarted with configured timer validity duration at the epoch time of the assistance information (i.e. serving satellite ephemeris data). - FFS: Precise definition of epoch time taking into account SIB repetitions Agreement: A single validity duration for both serving satellite ephemeris and common TA related parameters is defined at least if serving satellite ephemeris and common TA parameters are signalled in the same SIB message. Agreement: Configuration of UL transmission segment is indicated on SIB at least for initial access - FFS via UE-specific RRC signalling in RRC_CONNECTED. Agreement: For NB-IOT, the same value is used for segment durations for all NPRACH preambles for a particular NPRACH format Agreement: In eMTC/NB-IoT, NTA update based on TA Command field in msg2 and MAC CE TA command is used for UL timing alignment correction as follows: - No extension on TAC 11-bit field in Random Access Response - When TAC (TA) in Msg2 is received, UE first adjustment and NTA is adjusted as follows: NTA,new = TA 16, where TA is the timing advance command in msg2. - When TACs (TA) provided within the MAC CE is received, NTA is updated as follows: - NTA_new=NTA_old+TA-31.16 , - Where TA is the TAC field received in MAC CE command. Agreement: RAN1 has discussed the following aspects and leaves it up to RAN2 to specify UE behaviour related to expiry of UL synchronization validity timer and determine which of the following aspects are to be specified: - Mechanisms for UE to declare loss of UL synchronization including mechanisms for UL synchronization recovery procedure when UL synchronization is lost if UL synchronization validity timer expires in RRC_CONNECTED - It is up to RAN2 to specify this new behaviour for connected UE within RLF set of procedures or a new procedure for re-acquiring satellite ephemeris - Mechanism for UL synchronization includes re-acquiring the satellite ephemeris and common TA parameters if indicated on SIB - A new clause of RLF for loss of UL synchronization if validity timer for UL synchronization expires assuming a new re-interpretation of RLF set of procedures is specified for recovery of UL synchronization with re-acquisition of satellite ephemeris and common TA parameters if indicated - Potential additional RACH after re-acquisition of satellite ephemeris and common TA parameters if indicated for the UL synchronization recovery procedure in case of potential residual TA error. - If validity timer for UL synchronization expires and no UL synchronization recovery mechanisms specified as above, UE behaviour shall declare RLF and go into idle mode autonomously to re-acquire ephemeris SIB. UE will then need to re-access the cell via Random Access procedure. - UE signalling to indicate the validity timer for UL synchronization is about to expire

In this contribution, we discuss the remaining issues of time and frequency enhancements for IoT in NTN.

Discussion

UL time and frequency synchronization enhancement

GNSS Measurements

GNSS capability in the UE is taken as the working assumption of WI on NB-IoT/eMTC support for NTN [2]. In RAN1#106-e, it was agreed that for sporadic short transmission, UE in RRC_CONNECTED should go back to idle mode and re-acquire a GNSS position fix if GNSS becomes outdated. In RAN1#106bis-e, the UE behaviour before GNSS position fix becomes outdated for IoT-NTN UE in RRC_CONNECTED was discussed. In order to avoid the potential interference from the UE without valid GNSS position fix, UE in RRC_CONNECTED can report its remained validity duration of GNSS to the network. Based on this assistant information, the network can decide whether to move the UE to RRC_IDLE. As a side effect, the eNB can perform a better scheduling and potentially reduce the resource waste by ensuring that GNSS position fix is valid for a given transmission. In addition, based on the assistant information, the eNB would also be able to potentially configure a measurement gap for a new GNSS position fix if the UE does not support simultaneous GNSS and NTN NB-IoT/eMTC operation.

Proposal 1: UE in RRC_CONNECTED can report its GNSS position fix validity duration to the network.

Proposal 2: Based on the UE reported GNSS position fix validity duration, the network can configure a measurement gap for a new GNSS position fix if the UE does not support simultaneous GNSS and NTN NB-IoT/eMTC operation.

NPUSCH transmission segmentation duration

The following agreements were made on segment duration for long NPUSCH transmissions in 
RAN1#106-e.

Agreement: Duration of UL transmission segment for UE pre-compensation for PUSCH transmission is a number of PUSCH repetition units configured by the network - For NB-IoT, repetition unit is MidenticalNPUSCH×NslotUL×Tslot … Agreement: - For NB-IoT/eMTC NTN, the network configures one of K candidate values for the UL transmission segment duration of NPUSCH/PUSCH in a k-bit field. - For NB-IoT, maximum 3-bit field with a maximum number of K=8 candidate values 2 ms, 4 ms, 8 ms, 16 ms, 32 ms, 64 ms, 128 ms, 256 ms - FFS: Down scoping of K candidate values, size of k-bit field

For long NPUSCH transmission, the eNB shall indicate one value from K segment durations to an NB-IoT UE. The UL segment duration for UE pre-compensation for NPUSCH should not exceed the timing error requirements and should also be compatible with the NPUSCH repetition unit defined in TR 36.211. In other words, the UL segment duration X should be a number of N NPUSCH repetition units which does not exceed the maximum allowed continuous transmission time Tseg_max based on timing drift rate and the maximum timing error tolerance Te, as in:

X ms=N×Trep_unit ,

N=Tseg_maxTrep_unit.

Note that the NPUSCH repetition unit relates to the numerology defined in Table 10.1.2.3 in TR 36.211. For FDD, NPUSCH transmission in NB-IoT NTN can be summarized in the following scenarios:

- Case 1 - NPUSCH format 1, SCS 3.75kHz, 1 Tone, NslotsUL = 16 and NsymbUL = 7;

- Case 2 - NPUSCH format 1, SCS 15kHz, 1 Tone, NslotsUL = 16 and NsymbUL = 7;

- Case 3 - NPUSCH format 1, SCS 15kHz, 3 Tone, NslotsUL = 8 and NsymbUL = 7;

- Case 4 - NPUSCH format 1, SCS 15kHz, 6 Tone, NslotsUL = 4 and NsymbUL = 7;

- Case 5 - NPUSCH format 1, SCS 15kHz, 12 Tone, NslotsUL = 2 and NsymbUL = 7;

- Case 6 - NPUSCH format 2, SCS 3.75kHz, 1 Tone, NslotsUL = 4 and NsymbUL = 7;

- Case 7 - NPUSCH format 2, SCS 15kHz, 1 Tone, NslotsUL = 4 and NsymbUL = 7;

For the above 7 cases, the maximum number of NPUSCH repetition units in one segmentation is bounded by Te for different elevation angles as shown in Figure 2.

Figure 2.  Number of NPUSCH repetition units in a UL segment for LEO-600.

For each numerology, the eNB can determine the UL segment duration X for UL pre-compensation during long UL transmissions. Besides, considering that the time-duration of NPUSCH repetition unit, i.e. MidenticalNPUSCH×NslotUL×Tslot, only consists of a few values {2ms, 4ms, 8ms, 16ms, 32ms} for frame structure type 1, the values of X for all numerology cases can be determined for different elevation angles. In the following, we provide the values of UL segment duration X for the LEO-600km in Table 2.

- Table 2. UL segment duration for long UL transmission for NPUSCH for NB-IoT NTN

| - Elevation angle |  |  | - 10o | - 20 o | - 30 o | - 40 o | - 50 o | - 60 o | - 70 o | - 80 o | - 83 o~90o |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Max-continuous transmission time based on Te (ms) |  |  | - 28.7 | - 30.08 | - 32.63 | - 36.89 | - 43.97 | - 56.52 | - 82.63 | - 162.8 | - 256 |
| - Case 1 | - X (ms) | - Mrep≥8 | - - | - - | - 32 | - 32 | - 32 | - 32 | - 64 | - 160 | - 256 |
|  |  | - Mrep=4 | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ |
|  |  | - Mrep=1,2 | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ |
| - Case 2 | - X (ms) | - Mrep≥8 | - 24 | - 24 | - 32 | - 32 | - 40 | - 56 | - 80 | - 160 | - 256 |
|  |  | - Mrep=4 | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ |
|  |  | - Mrep=1,2 | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ |
| - Case 3 | - X (ms) | - Mrep≥8 | - 16 | - 16 | - 32 | - 32 | - 32 | - 48 | - 80 | - 160 | - 256 |
|  |  | - Mrep=4 | - 24 | - 24 | - 32 | - 32 | - 40 | - 56 | - 80 | - 160 | - 256 |
|  |  | - Mrep=1,2 | - 28 | - 28 | - 32 | - 36 | - 40 | - 56 | - 80 | - 160 | - 256 |
| - Case 4 | - X (ms) | - Mrep≥8 | - 24 | - 24 | - 32 | - 32 | - 40 | - 56 | - 80 | - 160 | - 256 |
|  |  | - Mrep=4 | - 28 | - 28 | - 32 | - 36 | - 40 | - 56 | - 80 | - 160 | - 256 |
|  |  | - Mrep=1,2 | - 28 | - 30 | - 32 | - 36 | - 42 | - 56 | - 82 | - 162 | - 256 |
| - Case 5 | - X (ms) | - Mrep≥8 | - 28 | - 28 | - 32 | - 36 | - 40 | - 56 | - 80 | - 160 | - 256 |
|  |  | - Mrep=4 | - 28 | - 30 | - 32 | - 36 | - 42 | - 56 | - 82 | - 162 | - 256 |
|  |  | - Mrep=1,2 | - 28 | - 30 | - 32 | - 36 | - 43 | - 56 | - 82 | - 162 | - 256 |
| - Case 6 | - X (ms) | - Mrep≥8 | - 24 | - 24 | - 32 | - 32 | - 40 | - 56 | - 80 | - 160 | - 256 |
|  |  | - Mrep=4 | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ |
|  |  | - Mrep=1,2 | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ |
| - Case 7 | - X (ms) | - Mrep≥8 | - 28 | - 30 | - 32 | - 36 | - 42 | - 56 | - 82 | - 162 | - 256 |
|  |  | - Mrep=4 | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ |
|  |  | - Mrep=1,2 | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ | - ~ |

In Table 2, the UL segment durations X for NPUSCH during long transmissions are evaluated for different elevation angles covering all numerology cases, where the elevation angle are chosen equally from 10o to 90o. Specifically, the TA rate at elevation angle θ≥ 83.67o results in a maximum allowed time-continuous transmission of 256ms. Therefore the last column in Table 2 has an applicable elevation range of 83o~90o.  Besides, for each case, Table 2 provides X for different repetition times, i.e., MrepNPUSCH. For case 1, 2, 6 and 7, the segmentation duration are identical for all possible MrepNPUSCH.

Since the NPUSCH segmentation duration X is dependent on the delay drift rate which is essentially coupled with the elevation angle. One possible way is to pre-define a table and the eNB can indicate one set of NPUSCH segmentation durations for each numerology and number of NPUSCH repetitions. As an example, one can indicate the column index in Table 2 based on the elevation angle. Another solution is to pre-define as set of K candidate values and the eNB indicates one of the K candidate values via k bit field. This alternative is simpler than the first scheme since it assumes that the segmentation duration is same for all combinations of numerology and number of NPUSCH repetitions for a give elevation angle. On the number of K it can be observed that the values of 2ms, 4ms and 8ms are not applicable according to the analysis in Table 2. Therefore only segment duration between 16 ms and 256 ms should be specified.

In addition, for different orbit heights, the max-allowed segment lengths could be different at same elevation angles. According to the calculation, for higher orbit case such as LEO-1200, the segmentation under 10o~90o elevation ranges from 32ms to 256ms, which has a longer allowed segment length compared to LEO-600 at the worst elevation angle of 10o. On the other hand, the lower orbits have larger TA rate. Thus the allowed segment duration for lower orbit could be set shorter than in LEO-600 case. While, based on our analysis, the segmentation for LEO-300 has the same value range compared to LEO-600, which is also from 16ms to 256ms. To our understanding, the NPUSCH segment length should be configured by NW to UEs in a unified manner with respect to different orbit types. Therefore only the segment length of the worst case, i.e., the LEO-300, can be specified for the sake of simplicity. And no need to down scope the candidate values per satellite orbit.

Proposal 3: For all satellite orbits, network configures one of 5 candidate values for the UL transmission segmentation duration of NPUSCH in a 3-bit field, where the 5 candidate values are

- {16 ms, 32 ms, 64 ms, 128 ms, 256 ms}

According to the satellite parameter sets specified in [7], for LEO-600 scenario, parameters in set-4 specified the largest footprint of satellite beams which have a beam diameter of 1700km and central beam edge elevation of 30o at the nadir beam. For satellite beam with such a large footprint, UEs in RRC_CONNECTED in the same cell could experience quite TA drift rate. For example, connected UEs at nadir point of a set-4-beam have nearly 90o elevation angle, thus the NPUSCH segment duration for these UEs could be signaled with the largest allowed segment length of 256ms according to Table 2. On the other hand, connected UEs in the cell edge of a set-4-beam would experience much worse TA rate due to a much larger timing draft rate. Therefore, a cell-edge UE should be signaled with a much smaller segment duration (i.e., 32ms according to Table 2) compared to the cell-center UEs. Therefore, in addition to cell-specific configuration, the NPUSCH segment duration can also be configured via UE-specific signaling, e.g. segment duration is determined by UE’s location report.

Proposal 4: The NPUSCH segmentation duration can be configured via UE-specific signaling.

NPRACH transmission segmentation duration

RAN1 has made the agreement on the NPRACH repetition units as:

Agreement: Duration of UL transmission segment for UE pre-compensation for PRACH transmission is a number of RACH repetition units configured by the network - For NB-IoT, repetition unit is P symbol groups. - For eMTC, repetition unit is one preamble including guard period. - FFS: Configuration details

The value of P and the PRACH repetition unit time-duration for preamble format 0, 1 and 2 is given in Table 1, respectively. For the details on the maximum allowed number of NPRACH repetition units for time-continuous transmission, we propose the similar idea as in deriving the segmentation for NPUSCH for NB-IoT, which is based on the timer error Te and the TA drift rate at different elevation angles.  Based on the discussion,  an example for LEO-600 scenario of the allowed NPRACH repetition units N, i.e. in P symbol groups, at different elevation angles are illustrated in Figure 3, where it is observed that the N values are quite similar among lower elevation angles, e.g. <50 degrees. Besides, Table 3 summarized the different values of N corresponding to each elevation angle bounded by the Te requirements.

Figure 3.  Maximum allowed time-continuous transmission and TA drift rate for LEO-600km

- Table 3. Number of NPRACH repetition units at different elevation angles.

| Elevation | 10o | 20o | 30o | 40o | 50o | 60o | 70o | 80o | 90o |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Format 0 | 5 | 5 | 5 | 6 | 7 | 10 | 14 | 29 | 64 |
| Format 1 | 4 | 4 | 5 | 5 | 6 | 8 | 12 | 25 | 64 |
| Format 2 | 1 | 1 | 1 | 1 | 2 | 2 | 4 | 8 | 16 |

Proposal 5: Network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field in the system information where the size of k and K values are:

- Format 0 and format 1: 3-bit field, K=6 candidate values 2*4*(TCP+TSEQ), 4*4*(TCP+TSEQ), 8*4*(TCP+TSEQ), 16*4*(TCP+TSEQ), 32*4*(TCP+TSEQ), 64*4*(TCP+TSEQ)

- Format 2: 3-bit field, K=5 candidate values 1*6*(TCP+TSEQ), 2*6*(TCP+TSEQ), 4*6*(TCP+TSEQ), 8*6*(TCP+TSEQ), 16*6*(TCP+TSEQ)

Validity timer for UL synchronization

In RAN1#106bis-e, it was agreed that a single validity duration for both serving satellite ephemeris and common TA parameters is defined at least if serving satellite ephemeris and common TA parameters are signaled in the same SIB message. The validity timer for UL synchronization is started/restarted with configured timer validity duration at the epoch time of the assistance information (i.e. serving satellite ephemeris data). Upon reads the SIB, the UE should immediately (re)start the indicated validity timer(s) for UL synchronization. And the UE in RRC_CONNECTED may triggers RLF if the validity timer(s) for UL synchronization expires. The epoch time for serving satellite ephemeris and common TA should also be the same when they are signaled in the same SIB message. Therefore, to simplify the implementation of UE, it is better to signal ephemeris and common TA in the same SIB message as UE have only one validity timer to maintain.

Similar to NR NTN, the reference point for common TA and satellite ephemeris epoch time can be set at the satellite as it has longer validity duration compared to set at NTN GW. At the eNB, it should take the feederlink delay into consideration for the indicated common TA parameters and ephemeris. At the UE side, as the delay of service link is different among UE, UE can propagate the exact ephemeris and Common TA by its service link delay.

For the validity timer for UL synchronization, it is started/restarted with configured timer validity duration at the epoch time of the common TA and ephemeris. As there may be multiple repetitions for each SI, the epoch time for common TA and satellite ephemeris can be defined as the ending time of the SI window carrying the common TA and satellite ephemeris.

Proposal 6: The reference point for epoch time is set at the serving satellite transmitter.

Propose 7: The epoch time for common TA and satellite ephemeris is defined as the ending time of the SI window carrying the common TA and satellite ephemeris.

New UL gaps for long UL transmission

In the section above, we have discussed and proposed that extra UL gaps should be introduced during long UL transmissions for TA adjustment. According to FL summary of RAN1#106bis-e, there are two options concerning the TA adjustment between UL segments, as follows:

Initial Proposal – Section 4.2.4: Companies are encouraged to comment on options for new gaps between UL transmission segments: - Option 1: New gaps for NPUSCH/PUSCH: - 1 ms: - 40 ms (legacy UL gap) - Smaller than 1 ms - one punctured SC-FDMA symbol in NPUSCH/PUSCH - one dropped/punctured time duration in NPUSCH/PUSCH - Indicated by system Information - Option 2: No new gaps between UL transmission segments: (Please, say if this has impact on the specifications or can be up to UE implementation based on RAN4 timing requirements). - Small TA adjustment is implemented by dropping tail samples of a segment for NPUSCH/PUSCH - Small TA adjustment is implemented by delaying a few samples for UL transmission for NPUSCH/PUSCH - Large TA adjustment is done within the NPRACH/RACH preamble’s cyclic prefix or the guard band - Other ways up to UE implementation

Basically, option 1 is to introduce extra gaps between segment transmissions, so as to address issue of TA adjustment and the overlapping between adjacent segments as TA rate increases. It was argued that option 1 will cause more waste on the time resource, especially when the gap length is large, such as 40ms. Then option 2 is one way to solve the issue within the tail time of a previous segment for NPUSCH or the initial time of a subsequent segment for NPRACH. For NPRACH repetition, applying TA adjustment within the preamble’s CP is equivalent to shorten the CP length. For connected UEs at cell/beam edge who experience small elevation angles such as 10~30 degrees, the multipath effect during transmissions shall not be neglected, especially at down-town or mountainous area where massive reflectors exists. In such cases the shortened CP may induce performance loss. While for NPUSCH repetition transmission, directly dropping the tail samples of a segment may cause detection issue for the related symbols.

Based on the above analysis, option 1 with appropriate UL gap (can be much less 40ms) for TA adjustment is preferable. In fact, the gaps should be able to cover the issue of overlapping in segments as TA drift rate changes from small values to large values, i.e., as satellite moves away from connected UEs. Based on this, the metric for gap length determination could be the maximum TA variation during one segment transmission. The segment duration during the UL transmission is determined by the TA drift rate, given the timing error requirements (80*Ts for NB-IoT). Therefore theoretically, a gap length of 80Ts should be enough to cover the issue of overlapping and satisfy the TA adjustment. However, in order to avoid slot misalignment at eNB, a gap of 1ms can be adopted for UL gap and will not cause much waste of resources.  At the eNB side, the timing for scheduling/detection at eNB should take the inserted gaps into consideration at the same time.

Proposal 8: Support 1ms of UL gap for NB-IoT over NTN.

DL synchronization

The DL synchronization error during the initial access of IoT NTN caused by large differential Doppler shift and UE oscillator error was discussed in the last two meetings. And two candidate solutions are to be determined based on FL summary, namely the new channel raster with step size greater than 100 kHz, and including (part of) ARFCN-indication-in-MIB. From Table 2 in [5], the differential Doppler frequency can be up to +/-39.9 kHz with set-4 LEO-600. Besides, with 20 ppm oscillator error at UE, there could be extra frequency offsets as +/-40 KHz. The total uncertainty on DL raster exceeds half of 100 kHz channel raster of terrestrial NB-IoT/eMTC, which would cause error in (N)Cell frequency selection. Introducing the new channel raster with larger step size (100 kHz) is a straightforward solution without extra signalling overhead compared to ARFCN indication.

Proposal 9: Support introducing the new channel raster with step size greater than 100 kHz for DL synchronization in IoT NTN.

The issue of DL frequency compensation was discussed in NR NTN. Compare to commercial smart phones, the IoT devices are more sensitive to complexity and power consumption, introducing DL frequency compensation is beneficial for IoT NTN to reduce the CFO estimation complexity during initial access. Otherwise, the UE cell search complexity are increased with additional power consumption. The indication of frequency pre-compensation as discussed in our companion paper in [3] can reused for IoT-NTN, i.e. the value range can be 0~25.5ppm with granularity of 0.01ppm.

Observation: Without DL frequency pre-compensation, UE initial cell search complexity and latency will introduce large power consumption.

Proposal 10: Support DL frequency pre-compensation in IoT NTN and use 12-bit to indicate the value of DL frequency pre-compensation with range [0,…, 4095] and granularity of 0.01ppm.

RRC parameters

According to the discussion above, the value range for TransmissionDurationNPRACH-NB-r17 and TransmissionDurationNPUSCH-NB-r17 in [6] needs to be updated. Firstly, the segment duration for NPRACH transmission with preamble format 2 should add a minimum segment length of 6*(TCP+TSEQ). Secondly, the {2,4,8} ms can be removed for TransmissionDurationNPUSCH-NB-r17. Additionally, the segment duration for the UL transmission can be configured by NW in a cell-specific or UE specific way. The RRC should also include the UL gap with duration of 1ms and DL pre-compensation of serving and neighbor cell with range of 0~25.5ppm.

The RRC parameter list can be referred in the appendix with the proposed revisions highlighted in yellow.

Proposal 11: Update the RRC parameters according to the Table provided in the Appendix.

Conclusion

In this contribution, we discuss the time/frequency adjustment, RACH enhancement and power consumption introduced by GNSS and NTN related SIB reading in IoT NTN. The following observations and proposals are presented.

Observation: Without DL frequency pre-compensation, UE initial cell search complexity and latency will introduce large power consumption.

Proposal 1: UE in RRC_CONNECTED can report its GNSS position fix validity duration to the network.

Proposal 2: Based on the UE reported GNSS position fix validity duration, the network can configure a measurement gap for a new GNSS position fix if the UE does not support simultaneous GNSS and NTN NB-IoT/eMTC operation.

Proposal 3: For all satellite orbits, network configures one of 5 candidate values for the UL transmission segmentation duration of NPUSCH in a 3-bit field, where the 5 candidate values are

- {16 ms, 32 ms, 64 ms, 128 ms, 256 ms}

Proposal 4: The NPUSCH segmentation duration can be configured via UE-specific signaling.

Proposal 5: Network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field in the system information where the size of k and K values are:

- Format 0 and format 1: 3-bit field, K=6 candidate values 2*4*(TCP+TSEQ), 4*4*(TCP+TSEQ), 8*4*(TCP+TSEQ), 16*4*(TCP+TSEQ), 32*4*(TCP+TSEQ), 64*4*(TCP+TSEQ)

- Format 2: 3-bit field, K=5 candidate values 1*6*(TCP+TSEQ), 2*6*(TCP+TSEQ), 4*6*(TCP+TSEQ), 8*6*(TCP+TSEQ), 16*6*(TCP+TSEQ)

Proposal 6: The reference point for epoch time is set at the serving satellite transmitter.

Propose 7: The epoch time for common TA and satellite ephemeris is defined as the ending time of the SI window carrying the common TA and satellite ephemeris.

Proposal 8: Support 1ms of UL gap for NB-IoT over NTN.

Proposal 9: Support introducing the new channel raster with step size greater than 100 kHz for DL synchronization in IoT NTN.

Proposal 10: Support DL frequency pre-compensation in IoT NTN and use 12-bit to indicate the value of DL frequency pre-compensation with range [0,…, 4095] and granularity of 0.01ppm.

Proposal 11: Update the RRC parameters according to the Table provided in the Appendix.

- References

RAN1#106bis-e Chairman’s Notes

RP-211601, NB-IoT/eMTC support for Non-Terrestrial Networks

R1-2110805, “Discussion on UL time and frequency synchronization enhancement for NTN”, Huawei, HiSilicon

TR 36.133, Requirements for support of radio resource management

R1-2102344, “Discussion on time and frequency synchronization enhancement for IoT in NTN”, Huawei, HiSilicon.

R1-2110572, “Consolidated higher layers parameter list for Rel-17 LTE”.

3GPP TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)”.

- Appendix A: Revised (highlighted in yellow) higher layers parameter list for Rel-17 LTE.

| WI code | Sub-feature group | RAN1 specific-ation | Sect-ion | RAN2 Parant IE | RAN2 ASN.1 name | Parameter name in the spec | New or existing? | Parameter name in the text | Description | Value range | Default value aspect | Per (UE, cell, TRP, …) | UE-specific or Cell-specific | Specific-ation | Comment | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LTE_NBIOT_eMTC_NTN | Basic IoT over NTN support / UL synchronization-Transmission-IoT NTN | 36.213 |  |  |  | TransmissionDurationNPRACH-NB-r17 | new | TransmissionDurationNPRACH-NB-r17 | The UL transmission segment length for NPRACH refers to the duration of time during which the applied pre-compensation shall not be changed by the UE. Configuration of UL transmission segment is indicated on SIB at least for initial access • FFS via UE-specific RRC signalling in RRC_CONNECTED. Duration of UL transmission segment for UE pre-compensation for NPRACH transmission is a number of NRACH repetition units configured by the network for NB-IoT NTN | Format 0 and 1: [2.4.(TCP+TSEQ), 4.4.(TCP+TSEQ), 8.4.(TCP+TSEQ), 16.4.(TCP+TSEQ), 32.4.(TCP+TSEQ), 64.4.(TCP+TSEQ)] -Format 2: [1. 6.(TCP+TSEQ), 2.6.(TCP+TSEQ), 4.6.(TCP+TSEQ), 8.6.(TCP+TSEQ), 16.6.(TCP+TSEQ) ] | 0 | Per cell | Cell-specific | 36.331 | For NB-IoT NTN, the network configures one of K values for the UL transmission segment duration of each PRACH preamble format in a k-bit field, where the size of the k-bit field and the number of K candidate values depend on the preamble format. Format 0 and format 1: 3-bit field, K=6 candidate values Format 2: 2-bit field, K=4 candidate values For NB-IOT, the same value is used for segment durations for all NPRACH preambles for a particular NPRACH format FFS: Down scoping of K candidate values, size of k-bit field | Stable |
| LTE_NBIOT_eMTC_NTN | Basic IoT over NTN support / UL synchronization-Transmission-IoT NTN | 36.213 |  |  |  | TransmissionDurationNPUSCH-NB-r17 | new | TransmissionDurationNPUSCH-NB-r17 | The UL transmission segment length for NPUSCH refers to the duration of time during which the applied pre-compensation shall not be changed by the UE. For NB-IoT, maximum 3-bit field with a maximum number of K=8 candidate values Configuration of UL transmission segment is indicated on SIB at least for initial access • FFS via UE-specific RRC signalling in RRC_CONNECTED. Duration of UL transmission segment for UE pre-compensation for NPUSCH transmission is a number of NPUSCH repetition units configured by the network - For NB-IoT, repetition unit is M_identical^NPUSCH×N_slot^UL×T_slot | [2 ms, 4 ms, 8 ms, 16 ms, 32 ms, 64 ms, 128 ms, 256 ms ] | 0 | Per cell or pre UE | Cell-specific or UE-specific | 36.331 | For NB-IoT NTN, the network configures one of K candidate values for the UL transmission segment duration of NPUSCH in a k-bit field. FFS: Down scoping of K candidate values, size of k-bit field | Stable |
| LTE_NBIOT_eMTC_NTN | Basic IoT over NTN support / UL synchronization-Transmission-IoT NTN | 36.213 |  |  |  | ULTransmissionGapNPUSCH-NB-r17 | new | ULTransmissionGapNPUSCH-NB-r17 | The UL gap for NPUSCH refers to the duration of UL transmission gap which is applied between two segmentations. | 1ms | 1ms | Per cell | Cell specific |  | For NB-IoT NTN, 1ms of UL transmission gap is applied between two segmentations. | Stable |
| LTE_NBIOT_eMTC_NTN | Basic IoT over NTN support / UL frequency synchronization-Transmission-IoT NTN | 36.213 |  |  |  | DLfrequencyPrecompensation-NB-r17 | new | DLfrequencyPrecompensation-NB-r17 | The DL frequency pre-compensation refers to the amount of DL frequency pre-compensation at the eNB | (0~25.5ppm) | 0 | Per cell | Cell specific |  | For NB-IoT NTN, DL frequency pre-compensation is applied according the minimum Doppler shift of the service link of the cell | Stable |