3GPP TSG RAN WG1 Meeting #107-e	R1-2110827

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.2.1

Source:	Huawei, HiSilicon

Title:	Initial access signals and channels for 52-71GHz spectrum

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss our views on initial access including SS/PBCH blocks, discovery burst transmission window (DBTW), CORESET#0 multiplexing patterns and PRACH configuration.

SS/PBCH blocks transmission

SSB pattern

Agreement (RAN1 #106-e)[1]:

For 480kHz and 960kHz sub-carrier spacing, first symbols of the candidate SSB have index {2, 9} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

In RAN1#106-e, the following agreement on SSB pattern have been reached.

Number of candidate SSB position within half frame: For operation without shared spectrum, Lmax=64 is sufficient for both 480 kHz and 960 kHz SCS to support at most 64 beams according to the WID. For operation with shared spectrum, additional candidate SSB positions is beneficial to deal with a LBT failure prior to SSB transmission in 480 kHz or 960 kHz SCS when Contention Exempt Short Control Signaling is not applicable.  Lmax=128 is a reasonable choice considering the tradeoff between increased channel access opportunities and increased candidate SSB index to be indicated in PBCH. On the other hand, the working assumption on support of at most 64 candidate SSB position for 120 kHz SCS in RAN1#106 does not imply there is less chance to have LBT failure in 60GHz unlicensed band. It is mostly due to large standard impact to introduce additional SSB candidates for 120 kHz SCS. We think the working assumption should be confirmed.

Proposal 1: For SSB with 120 kHz SCS, confirm the working assumption on 64 candidate SSBs within a half frame. For SSB with 480 kHz and 960 kHz SCS, 64 candidate SSBs is sufficient for operation without shared spectrum while 128 candidate SSBs should be supported for operation with shared spectrum.

Agreement:

Supported value of n for 480/960kHz SSB slot pattern:

- ALT A) non-contiguous, N slot gap (slots that do not contain SSB) every M slots that contain SSB

  - same pattern will apply to 480kHz and 960kHz (i.e same N and M for 480 and 960 kHz)

  - N = 2, M = 8

  - FFS: starting position of n

- ALT B) non-contiguous, N slot gap (slots that do not contain SSB) every M slots that contain SSB

  - scaled version pattern will apply between 480 and 960 kHz (i.e. N and M for 480kHz, 2N and 2M for 960 kHz)

  - N = 2, M = 8

  - FFS: starting position of n

- ALT C) slots that do not contain SSB correspond to the slots that do not contain SSB in 120 kHz Case D.

  - Note: ALT 4 means that only slots 32-39 for 480 kHz SSB pattern are reserved for UL and 960 kHz SSB pattern is contiguous.

UL slots within SSB burst: In RAN1#106b-e, following three alternatives have been identified for down-selection on the SSB contained slot indexes for 480kHz and 960kHz SCS.

All the 3 alternatives are expected to reserve UL slot within SSB burst to help UL transmission with short latency requirement. ALT A) and ALT B) allows shorter round trip time than ALT C) because the number of UL gaps per time unit is increased with the numerology. However, due to the almost same absolute timeline of beam switching and PDSCH/PUSCH processing as 120 kHz SCS, the increased opportunities for UL transmission do not provide additional reduction on the latency. On the contrary, the overhead of DL/UL switching caused by UL slots insertion is increased. Meanwhile, there is no additional requirement to support latency shorter than already supported values in Rel-15/16 in FR2-2.  Hence, we propose to adopt ALT C) slots that do not contain SSB correspond to the slots that do not contain SSB in 120 kHz Case D. In Case D for 120 kHz, 8 slot indexes Aμ=A3={8-9, 18-19, 28-29, 38-39} within a half frame are reserved for UL transmission. These slots correspond to slots A5={32-39, 72-79, 112-119, 152-159} within a half frame in 480 kHz and slots A6={64-79, 144-159, 224-239,304-319} within a half frame in 960 kHz. When designing SSB pattern for 480 kHz and 960 kHz SSBs, slots in A5 and A6 may be reserved for UL transmission, respectively. However, for operations without shared spectrum whereLmax=Lmax=64, 64 SSB candidates can be mapped to the first 32 slots 0-31 in 480 kHz or 960 kHz without an overlap with any slot in A5 or A6, respectively. As such, there is no need to reserve UL slots within the SSB burst. For operations with shared spectrum where Lmax=128, 128 SSB candidates can be mapped to slots 0-63 in 960 kHz without any overlap with A6. Therefore, there is no need to reserve UL slots within the SSB bust in this case either. In turn, 128 SSB candidates would occupy 64 slots in 480 kHz which would have overlap with slots 32-39 in A5. As such, we propose to map SSB candidates in 480kHz for operations with shared spectrum on the first 64 available slots 0,1,…,31, 40, 41,…71.

Figure 1  SSB pattern for different SCSs

Figure 1 shows our proposed SSB patterns for 120 kHz, 480 kHz and 960 kHz in operations with shared and without shared spectrum.

Proposal 2: Support following patterns (ALT C) for SSB with 480 kHz and 960 kHz SCS:

- For operations without shared spectrum:

  - {2,9}+14n, (n=0,1,2,…,31) for both 480 kHz and 960 kHz SCS.

- For operations with shared spectrum:

  - {2,9}+14n, (n=0,1,2,…,31,40,…,71) for 480 kHz SCS;

  - {2,9}+14n, (n=0,1,2,…,63) for 960 kHz SCS.

Since we propose the number of candidate SSBs in 480 kHz and 960 kHz SCS to be Lmax=128 for operation with shared spectrum, one more bit is required to indicate the candidate SSB index compared to Rel-15 FR2. We propose to borrow the 4th LSB of SFN in the PBCH payload to indicate the 7th bit of the candidate SSB index in operation with shared spectrum. In turn, we can transfer the 4th LSB of SFN to MIB payload. Note that if the 4th LSB of SFN is indicated in the MIB payload, the periodicity of update in MIB payload is still kept to 80 ms as in Rel-15.

Proposal 3: For operation with shared spectrum and for 480 kHz and 960 kHz SSBs, indicate the 7th bit of the candidate SSB index by borrowing the 4th LSB of SFN in the PBCH payload. Indicate the 4th LSB of SFN with spare bit in MIB payload.

Discovery burst transmission window

In RAN1#106-e, RAN1 conclude to continue discussions to develop solutions for supporting DBTW. Moreover, if DBTW is supported, similar DBTW lengths of {0.5, 1, 2, 3, 4, 5} ms as Rel-16 NR-U for 120 kHz SCS was agreed. In RAN1#106b-e, the following agreements and working assumption on DBTW

Working assumption:

Support DBTW for 120 kHz.

- FFS: Support for 480 kHz and 960 kHz

Agreement:

No other values of n other than agreed previously is supported for 120kHz SCS, where parameter ‘n’ is the set of values to determine the first symbols of the candidate SSB blocks for 120kHz SCS in agreement from RAN1 #104-bis-e.

are achieved.

In operation with shared spectrum, the transmission of SSBs in some candidate locations may be dropped due to a LBT failure. This increases the UE initial access latency if there is no more opportunity to transmit the dropped SSB within a DBTW. Note that, to our understanding, contention exempt short control signaling for operation with shared spectrum is not supported in all regions (e.g., Japan). Therefore, even if the duty cycle of the 480/960 kHz SSB burst is less than 10% within every 100 ms, LBT is mandated in these regions. Therefore, to facilitate transmitting the dropped SSBs due to LBT failure, we propose to support DBTW for all numerologies in Rel-17.

Observation 1: Contention exempt short control signaling for operation with shared spectrum is not supported in all regions. Therefore, even if the duty cycle of the 480/960 kHz SSB burst is less than 10% within every 100 ms, LBT must be used prior to 480/960 kHz SSB burst transmission in these regions.

Proposal 4: Confirm the working assumption to support DBTW for 120 kHz and further support DBTW for 480 kHz and 960 kHz SSB.

Agreement:

For 120kHz SCS, for NSSBQCL values:

- If 2 bits are available in MIB for NSSBQCL, at least support {16, 32, 64}

- If 1 bit is available in MIB for NSSBQCL, support {32, 64}

  - FFS: methods to indicate more NSSBQCL values without increasing used number of bits, e.g., {16, 32, 64}

- Note: value NSSBQCL < 64 indicates DBTW enabled/supported and operation with shared spectrum.

- Note: For operation without shared spectrum channel access, a UE expects to be configured with NSSBQCL = 64. Use of NSSBQCL=64 in shared spectrum is not precluded.

- FFS: 1 bit or 2 bits used for NSSBQCL

In NR-U Rel-16, the parameter NSSBQCL=1,2,4,8 is used for UE to determine the candidate SSB positions with same QCL assumption in the DBTW. Value of NSSBQCL  is indicated in MIB which allows the UE during initial access to only monitor the RMSI PDCCHs and RMSI PDSCHs associated with the candidate SSB indexes with the same QCL assumption as the detected SSB. Value of NSSBQCL is also indicated through cell specific or dedicated RRC signaling to facilitate intra and inter cell RRM measurement. In RAN1#106b-e, 2 alternative sets of NSSBQCL are identified FFS for 120 kHz SCS.

We think 2 bits of NSSBQCL indicating at least of {16, 32, 64} provide more flexibility to cater to gNB with different number of beams for all numerologies. The 2 bits can be from subCarrierSpacingCommon and one bit in pdcch-ConfigSIB1 in MIB to be discussed in the following sections.

Proposal 5: At least  NSSBQCL=16,32,64 should be indicated in MIB for all three numerologies.

In NR-U Rel-16, UE is always assuming LBT is ON and DBTW is ON by default. However, in FR2-2, it is possible that LBT is not mandatory for unlicensed band operation in certain regions. Letting UE know DBTW is OFF before UE acquires SIB1 could be beneficial to reduce blind detection of Type0 PDCCH CSS associated with candidate SSB position not transmitted at all. For example, UE may not need to detect monitoring occasions of Type0 PDCCH CSS associated with candidate SSB indexes in the range of 64 – 127 if DBTW is OFF for 480/960 kHz SCS. As stated in the above agreement, NSSBQCL<64 indicates DBTW enabled/supported and operation with shared spectrum for 120 kHz. Moreover, NSSBQCL=64 effectively means that DBTW is disabled for 120 kHz. However, if the number of candidate SSB positions is larger than 64 for operation with shared spectrum for 480/960 kHz, NSSBQCL=64 does not indicate whether DBTW is disabled or enabled as, at the time of initial SSB detection, UE may not know if it is operating with shared or without shared spectrum. In such a case, a separate entry of  NSSBQCL should be reserved for disabling DBTW. Since 120 kHz does not require such an indication using a separate entry of NSSBQCL, additional value of NSSBQCL=8 can be considered for 120 kHz.

Proposal 6: UE should be able to identify DBTW enable/disable before acquiring SIB1. A specific entry of NSSBQCL in MIB should be reserved to disable DBTW if the number of candidate SSB positions is larger than 64 for 480 kHz and 960 kHz SCS.

In NRU Rel-16, the DBTW length is indicated in SIB1 to facilitate UE to reduce the number of detection of candidate SSB position and the associated Type0 PDCCH monitoring occasion if co-channel interference is not severe. Although, as discussed above, it is possible to disable DBTW through MIB implicitly or explicitly for the scenario where LBT is not required, finer adjustment of DBTW length in SIB1 as in NRU Rel-16 is still beneficial when DBTW is enabled and NSSBQCL is small.  For example, if  NSSBQCL=16, half of the SSB measurement can be saved if DBTW length of 2.5ms is indicated compared to the case that DBTW length is 5ms for 120 kHz. Note that, when we increase the maximum number of candidate SSB indices to 128 for 480kHz (960kHz) SCS in operation with shared spectrum, the maximum number of slots covering all candidate SSB positions and reserved UL slots in-between is 72 (64)  which equals to 2.25ms (1ms). The configurable DBTW lengths smaller than the above maximum lengths may be obtained by scaling down the supported DBTW lengths of {4, 3, 1, 1, 0.5} for 120 kHz SCS by a factor of 4 (8) for 480 (960) kHz.

Proposal 7: Configure DBTW length in SIB1 for operation with shared spectrum in 52.6GHz to 71GHz with the following values:

- 480 kHz SCS: {72, 32, 24, 16, 8, 4} slots = {2.25, 1, 0.75, 0.5, 0.25, 0.125} ms

- 960 kHz SCS: {64, 32, 24, 16, 8, 4} slots = {1, 0.5, 0.375, 0.25, 0.125, 0.0625} ms

Agreement:

Same DCI size for DCI 1_0 in CSS regardless of channel access mode (i.e., LBT on/off).

- Existing DCI size alignment in TS38.212 applies to DCI 1_0 and 0_0 in CSS.

In RAN1#106b-e, the DCI size ambiguity of DCI format 1-0/0-0 in the initial access procedure is resolved according to the following agreement and, therefore, there is no need to indicate LBT on/off in MIB.

After UE acquires SIB1 and before RRC connection is set up, the “ChannelAccess-CPext” field in DCI format 0-0/1-0 scrambled with TC_RNTI for msg3 retransmission/msg4 and in RAR UL grant for msg3 transmission should be added for UE operating in LBT ON mode. Otherwise, the bits for the “ChannelAccess-CPext” field are reserved. Hence, an indication of LBT on/off in SIB1 is necessary to help UE determine the existence of “ChannelAccess-CPext” field in DCI format 1-0/0-0.

Proposal 8: Use of LBT should be indicated in SIB1 to help UE determine the existence of “ChannelAccess-CPext” field in DCI format 1-0/0-0 scrambled with TC-RNTI and in RAR UL grant.

UE assumption regarding ssb-PositionsInBurst in SIB1

ssb-PositionsInBurst is provided in SIB1 to indicate time domain positions of the transmitted SSBs in a SSB burst. In Rel-16 NR-U, ssb-PositionsInBurst is comprised of a single 8-bit bitmap inOneGroup. If MSB k (k>=1) of inOneGroup is set to 1, UE assumes that one or more SSBs within DBTW and with the candidate SSB index corresponding to the SSB index k-1 may be transmitted. In turn, If MSB k of ssb- inOneGroup is set to 0, UE assumes that the SSBs with the candidate SSB indexes corresponding to the SSB index k-1 are not transmitted.  Further, “The UE assumes that a bit at position k > NSSBQCL is 0” [4]. Above interpretations of the bits of ssb-PositionsInBurst cannot be directly carried over in 60 GHz shared spectrum:  In Rel-16 NR-U, maximum number of transmitted SSB indexes is 8 which is equal to the size of the single bitmap inOneGroup in ssb-PositionsInBurst. In turn, in 60 GHz shared spectrum, following the FR2 convention, ssb-PositionsInBurst is comprised of two 8-bit bitmaps of groupPresense and inOneGroup while the maximum number of the transmitted SSB indexes is 64. To clarify UE interpretation of ssb-PositionsInBurst bits for operation with shared spectrum in 60 GHz where ssb-PositionsInBurst is configured with both inOneGroup and groupPresense bitmaps, if MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense are set to 1, UE should assume that SSBs within DBTW with candidate SS/PBCH block index(es) corresponding to SSB index equal to k-1+m-1×8 may be transmitted; otherwise, the UE should assume that the SSB(s) are not transmitted.

Proposal 9: In operation with shared spectrum in 60 GHz, for MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense of ssb-PositionsInBurst:

- if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with candidate SSB index(es) corresponding to SSB index equal to k-1+(m-1)×8 may be transmitted;

- if MSB k of inOneGroup or MSB m of groupPresense are set to 0, the UE assumes that the SSB(s) are not transmitted.

Further, it should be understood that, regardless of the value of the MSB k of inOneGroup and MSB m of groupPresense, if k-1+m-1×8> NSSBQCL, UE assumes that candidate SSB index(es) corresponding to SSB index equal to k-1+m-1×8 are not transmitted.

Proposal 10: Regardless of the value of the MSB k of inOneGroup and MSB m of groupPresense in ssb-PositionsInBurst configured in SIB1, if k-1+m-1×8> NSSBQCL, UE assumes that candidate SSB index(es) corresponding to SSB index equal to k-1+m-1×8 are not transmitted.

CORESET#0/Type0-PDCCH common search space design

It was agreed in RAN 92-e that, for all three supported SCSs of 120, 480, and 960 kHz, Type0-PDCCH and the corresponding SSB have the same SCS and, further, SSB-CORESET#0 multiplexing pattern 1 is prioritized [3]. In addition, the following agreements regarding CORESET#0/Type0-PDCCH for 120 kHz SCS has been reached:

Agreement (RAN1 #104-e): For CORESET#0 and Type0-PDCCH search space configured in MIB: - Support {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS equal to {120, 120} kHz - Support at least SSB and CORESET#0 multiplexing patterns, number of RBs for CORESET#0, number of symbols (duration of CORESET#0) that are supported in Rel-15/16 for {SS/PBCH Block, CORESET#0 for Type0-PDCCH} SCS = {120, 120} kHz. - FFS: Supporting additional values - FFS: Supported values for SSB to CORESET#0 offset RBs Agreement (RAN1 #106-e): For ‘controlResourceSetZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, - Support the following set of parameters. SS/PBCH block and CORESET multiplexing pattern; Number of RBs; Number of Symbols. 1; 24; 2. 1; 48; 1. 1; 48; 2. - Note: the number of entries corresponding the same {mux pattern, number of RB, number of symbol} tuple (listed above) will depend on required RB offsets that needs to be supported based on channel and sync raster design. FFS: addition other set of parameters Working assumption: (RAN1#106b-e): - For {SSB, CORESET#0/Type0-PDCCH} = {120, 120} kHz, support multiplexing pattern 1 with 96 PRB CORESET#0, and {1, 2} symbol durations - Note: the working assumption can be confirmed once RAN1 agrees on the number of needed SSB-CORESET0 offsets for 24 and 48 RB CORESET0 based on RAN4 channelization design

In particular, above agreement states that for {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, both multiplexing Pattern 1 and multiplexing Pattern 3 are supported. For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {480, 480} kHz and {960, 960} kHz, we believe that supporting multiplexing Pattern 1 is sufficient. As such, we propose the following.

Proposal 11:  For CORESET for Type0-PDCCH in 52.6GHz to 71GHz spectrum, support the following:

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, support multiplexing pattern 1 and multiplexing pattern 3 as per Agreement in RAN1 104-e.

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {480, 480} kHz, support multiplexing pattern 1 only.

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {960, 960} kHz, support multiplexing pattern 1 only.

CORESET#0 configuration for 120 kHz SCS

In Rel-15/Rel-16, each CORESET#0 configuration index is associated with a RB offset from the smallest RB index of the CORESET#0 to the smallest RB index of the common RB overlapping with the first RB of the corresponding SSB. Although the exact values of supported RB offsets from CORESET#0 to SSB in 52.6GHz to 71GHz spectrum maybe restricted by the synch raster and channel raster design in FR2-2, RAN1 can still discuss and agree on the design principles and nominal values of RB offsets from CORESET#0 to SSB. Assuming that there will not be any restriction imposed by the synch raster and channel raster design in FR2-2 on RB offsets from CORESET#0 to SSB, the RB offsets defined in Rel-15/Rel-16 FR2 for the supported (Mux pattern, NRBCORESET, NsymbCORESET ) tuples can be inherited without any modification. In addition, for CORESET0 with 48 and 96 PRB with multiplexing pattern 1, it would be beneficial for gNB to transmit portion of discovery burst (e.g. SSB, Type0-PDCCH, RMSI PDSCH) with the same beam and as compact as possible in time and frequency domain to save LBT and beam switching overheads.  Considering only frequency domain resource allocation type 1 is allowed for RMSI PDSCH, aligning SSB with the two extremities of CORESET#0 in the frequency domain frees up more contiguous RBs and allows a more flexible and potentially a larger allocation for PDSCH carrying SIB1. Similar concept has been adopted in NR-U Rel-16 due to the same reason.

Proposal 12: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={120, 120} kHz:

- For CORESET#0 with 24 RBs and 48 RBs: the same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: additional RB offsets values of 0 and 28 RBs can be considered for multiplexing pattern 1.

- For CORESET#0 with 96 RBs: RB offsets values of 0 and 76 RBs can be considered for multiplexing pattern 1.

- Note: All above RB offsets are nominal and may need to be modified after finalizing synch raster and channel raster design in FR2-2.

Figure 2 Example of RB-level offset configuration for {SS/PBCH block, PDCCH}={120, 120} kHz when 96 RB CORESET0 is configured.

The new entries (red rows) for CORESET0 configuration discussed above can be added in table 13-8 of TS38.213 as follows:

Table 1 Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 3 | 24 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 5 | 3 | 24 | 2 | 24 |
| 6 | 3 | 48 | 2 | -20 if kSSB=0 -21 if kSSB>0 |
| 7 | 3 | 48 | 2 | 48 |
| 8 | 1 | 48 | 1 | 0 |
| 9 | 1 | 48 | 1 | 28 |
| 10 | 1 | 48 | 2 | 0 |
| 11 | 1 | 48 | 2 | 28 |
| 12 | 1 | 96 | 1 | 0 |
| 13 | 1 | 96 | 1 | 76 |
| 14 | 1 | 96 | 2 | 0 |
| 15 | 1 | 96 | 2 | 76 |

CORESET#0 configuration for 480 kHz and 960 kHz SCS

As the beam switching delay may be too large to be absorbed in CP in 480/960 kHz SCS, it is critical to have the possibility of transmitting SSB and its associated RMSI PDCCH/PDSCH without a need for a switching gap. Aligning SSB with the two extremities of CORESET#0 in the frequency domain frees up more contiguous RBs and allows a more flexible and potentially a larger allocation for PDSCH carrying SIB1. Aligning the lowest (highest) RBs of SSB and CORESET#0 in the frequency domain is already supported in Rel-15/16 for 24 RB CORESET#0 and multiplexing pattern 1 in {SSB, CORESET#0} ={120, 120} kHz, which can be directly reused for 480 kHz and 960 kHz SCS. For 48 RB CORESET#0, at least the RB offset of 0 and 28 RBs should be supported. An example is shown in the following figure.

Figure 3 Example of RB-level offset configuration for {SS/PBCH block, PDCCH}={480, 480} kHz and {960, 960} kHz when 48 RB CORESET0 is configured.

Proposal 13: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={480, 480} kHz and {960, 960} kHz:

- For CORESET#0 with 24 RBs: the same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: In addition to the offset of 14 RBs already supported in Rel-16, additional values of 0 and 28 RBs can be considered for multiplexing pattern 1.

The set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz and {960, 960} kHz SCS is provided in the following table.

Table 2   Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz and {960, 960} kHz for operation with share spectrum

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs | Number of Symbols | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 14 |
| 3 | 1 | 48 | 2 | 14 |
| 4 | 1 | 48 | 1 | 0 |
| 5 | 1 | 48 | 1 | 28 |
| 6 | 1 | 48 | 2 | 0 |
| 7 | 1 | 48 | 2 | 28 |

Type0-PDCCH common search space configuration

According to TS38.213 section 13, the associated Type0-PDCCH monitoring occasions corresponding to a SS/PBCH block index i are included in two consecutive slots starting from slot n0. The UE determines an index of slot n0 as n0=O⋅2μ+i⋅M mod Nslotframe, μ that is in a frame with system frame number (SFN) SFNC satisfying SFNC mod 2=0 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=0, or in a frame with SFN satisfying SFNC mod 2=1 if O⋅2μ+i⋅M/Nslotframe, μ mod 2=1. Parameters O and M are provided in Table 13-12 of 38.213 (copied in below Table 3) for all combinations of {SSB, CORESET#0/Type0-PDCCH} numerologies supported in FR2-1, including {240, 120}, {120, 120}, {240, 60}, and {120, 60} kHz.

- 120kHz SSB/CORESET0

The parameter O is used to determine the offset between Type0-PDCCH CSS MO associated with the candidate SSB index 0 in the unit of msec from the beginning of the frame.  In FR2-1, the subset of O=0, 2.5, 5 is used when SSB burst set is located in the 1st half frame. gNB uses O=2.5(5) to configure the  Type0-PDCCH CSS monitoring window after the whole associated 240kHz (120kHz) SSB burst set is transmitted.  Similarly, when SSB burst set is in the 2nd half frame, the subset of  O=0, 5,7.5 is used. The value of  O= 7.5 (0) is configured to shift the Type0-PDCCH CSS monitoring window after the whole associated 240kHz (120kHz) SSB burst set  is transmitted. An example is illustrated in Figure 4. For 120 kHz SCS in FR2-2, the O values of {2.5, 7.5} are not necessary because they are used for mix numerology configuration for SSBs with 240 kHz SCS in FR2-1. Thus, we propose to adopt the parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR2-2 when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz in Table 3.

Proposal 14: The parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 with {120, 120} kHz in Table 3 should be supported.

Figure 4	Examples of SSB/COREST0 multiplexing pattern 1

Table 3	  Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR2-2 when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 5 | 1 | 1 | 0 |
| 2 | 0 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 3 | 5 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 4 | 0 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 5 | 5 | 2 | 1/2 | {0, if i is even}, {NsymbCORESET, if i is odd} |
| 6 | 0 | 1 | 2 | 0 |
| 7 | 5 | 1 | 2 | 0 |

- 480/960kHz SSB/CORESET0

Agreement:

For ‘searchSpaceZero’ configuration for {SSB, CORESET#0/Type0-PDCCH} = {480, 480} kHz and {960, 960} kHz, use the following table for multiplexing pattern 1:

- FFS: The value of X (> 0)

- FFS: whether or not to use different X value depending on whether DBTW is ON/OFF

- FFS: whether or not to use same or different X value for 480 and 960 kHz

- FFS: whether Y =NsymbCORESET, or Y=NsymbCORESET+1, or whether to remove entries with Y

| Index | O | Number of search space sets per slot | M | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 2 | X | 1 | 1 | 0 |
| 3 | X | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 6 | 0 | 2 | 1/2 | {0, if i is even}, {Y, if i is odd} |
| 7 | X | 2 | 1/2 | {0, if i is even}, {Y, if i is odd} |
| 8 | 5 | 2 | 1/2 | {0, if i is even}, {Y, if i is odd} |
| 9 | 5 + X | 1 | 1 | 0 |
| 10 | 5 + X | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 11 | 5 + X | 2 | 1/2 | {0, if i is even}, {Y, if i is odd} |
| 12 | 0 | 1 | 2 | 0 |
| 13 | 5 | 1 | 2 | 0 |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

In RAN1#106b-e, the following agreement was achieved on the type 0 PDCCH CSS configuration for 480/960 kHz SCS.

In addition to the existing entries for O={0, 5}, the entries with O={X, 5+X} have been discussed which allow gNB to transmit RMSI PDCCH/PDSCH immediately after the transmission of SS burst set in the same half frame. In Rel-15, X=2.5 is supported to facilitate transmitting RMSI PDCCH/PDSCH immediately after 240kHz SSB burst set as the SS burst set with 240kHz is only around half of the 5ms SSB burst length of 120kHz SCS. For the 480 kHz and 960 kHz SSB burst set, the X values can be further scaled down  for μ=5 and μ=6 i.e. X=52μ-3 when DBTW is OFF. If DBTW is ON and assuming at most 128 candidate SSB position for 480 kHz and 960 kHz SSB burst set, X=52μ-4 should be used. Note that, for  μ=5 and μ=6, DBTW OFF should be indicated in MIB using a value of NSSBQCL. This enables UE to choose the correct value of X from 2μ-3, 2μ-4

As for the first symbol index of {0, if i is even}and {Y, if i is odd}, there are three alternatives of Y, i.e. Y =NsymbCORESET, or Y=NsymbCORESET+1, or whether to remove entries with Y.  Y =NsymbCORESET does not work because CP is too short to accommodate potential beam switching gap between two monitoring occasions transmitted with different beams. Y =NsymbCORESET is even more problematic if  Type0-PDCCH CSS monitoring window and SSB burst set start in the same half-frame. In such a case, since we agreed that “first symbols of the candidate SSB have index {2, 9} + 14*n”, to accommodate both Type0-PDCCHs and the first candidate SSB in the slot,  NsymbCORESET can only be equal to one. Moreover, gNB should perform two beam switching between the Type0-PDCCHs in OS#0 and OS#1 and the Type0-PDCCH in OS#1 and the SSB candidate in OS#2. For Y=NsymbCORESET+1, the beam switching gap between two consecutive monitoring occasions can be inserted. However, it is problematic if Type0-PDCCH CSS monitoring window and SSB burst set start in the same half-frame. In such a case, since we agreed that “first symbols of the candidate SSB have index {2, 9} + 14*n”, the Type0-PDCCHs monitoring occasion associated with the second candidate SSB in the slot collides with the first SSB even if  NsymbCORESET is equal to one. On the other hand, since the first symbols of the candidate SSB in a slot have index {2, 9}, the configuration with monitoring occasions starting symbols of {0, if i is even} and {7 if i is odd} to support COREST0 with 2 OS is very well justified. Based on the above discussion, we propose to update the parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 for FR2-2 in the following table.

Table 4 Parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 and FR2-2 when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz or {960, 960} kHz

| Index |  | Number of search space sets per slot |  | First symbol index |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 2 | X | 1 | 1 | 0 |
| 3 | X | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 4 | 5 | 1 | 1 | 0 |
| 5 | 5 | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 6 | 5+X | 1 | 1 | 0 |
| 7 | 5+X | 2 | 1/2 | {0, if i is even}, {7, if i is odd} |
| 8 | 0 | 1 | 2 | 0 |
| 9 | 5 | 1 | 2 | 0 |
| 10-15 | Reserved |  |  |  |

Proposal 15: Support the parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 with {480, 480} kHz and {960, 960} kHz in Table 4 that is obtained from the agreed Table in RAN1 106b-e by selecting  X=52μ-3 when DBTW is OFF and X=52μ-4 when DBTW is ON and removing entries with Y.

- Note: DBTW OFF is indicated in MIB using a value of NSSBQCL

Offset between off-sync raster SSB and CORESET#0

In Rel-16 NRU, off-synch raster SSBs may configure CORESET#0 to support ANR on SCells. When a UE is configured to report the CGI associated with an off-synch raster SSB, the UE finds the frequency offset from CORESET#0 to the off-synch raster SSB according to a sum of the following first offset and the second offset:

- First offset: Provided in Table 13-1A and Table 13-4A defined in [4][2], and

- Second offset: Determined as the offset from a smallest RB index of the common RB overlapping with the first RB of the off-synch raster SSB indicated in the measurement configuration to a smallest RB index of the common RB overlapping with the first RB of a SSB hypothetically located at the GSCN of a synch raster entry where the single synch raster entry is located in the same channel as the SSB used for the shared spectrum channel access procedure [4].

Above offsets are defined with respect to the SCS of the CORESET#0, which for operation with shared spectrum, is the same as the SCS of the corresponding SSB. The frequency location of the CORESET#0 associated with the off-synch raster SSB can be unambiguously determined from the above sum since only a single synch raster entry is located in the same channel as the configured off-synch raster SSB. The reason for the one to one correspondence between synch raster entry and the channel in Rel-16 NRU is that, in Rel-16 NRU:

1) Each channel has a 20 MHz nominal channel bandwidth as defined in [5] and;

2) Synch raster entries in unlicensed bands are allocated roughly 20 MHz apart such that one and only one on-synch raster SSB can be fully contained in each channel.

However, in 60GHz unlicensed spectrum, neither of the above two conditions are met: First, regulations do not define a fixed nominal channel bandwidth [6]. Second, RAN4 has not determined the synch raster locations yet. Further, given the flexible nominal channel bandwidth in 60GHz unlicensed spectrum, it seems impossible to guarantee only one on-synch raster SSB per channel. Above discussion hints to the conclusion that the same method as in Rel-16 NRU cannot be used to find the offset between an off-synch raster SSB and the corresponding CORESET#0 in 60GHz unlicensed spectrum. However, without having to wait for RAN4 to allocate the synch raster, above issue may be independently resolved in RAN1 by uniquely determining the hypothetical on-synch raster SSB that serves as the reference for the above second offset in case multiple synch raster (and hence, multiple hypothetical on-synch raster SSBs) are included in a channel bandwidth.

Proposal 16: To find the offset between an off-synch raster SSB and the corresponding CORESET#0 in 60GHz unlicensed spectrum, RAN1 should uniquely determine the hypothetical on-synch raster SSB that serves as the reference for the offset to the off-synch raster SSB in case more than one synch rasters are included in a channel bandwidth.

Fields in PBCH payload and MIB

Agreement:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

In RAN1#104bis-e, the following agreement was reached in which some restrictions are set on the bit size of PBCH and MIB.

According to the discussion in previous sections, several features might have impact on the PBCH payload and MIB content. For example, the bit#6 (subCarrierSpacingCommon) and bit#19 (pdcch-configSIB1) can be re-interpreted to indicate NSSBQCL. For 480kHz and 960kHz SCS SSB with DBTW is ON, the bit#23 (4th LSB of SFN) could be used to indicate the 7th bit of candidate SSB index if 128 candidate SSB positions are supported. The 4th LSB of SFN can take the spare bit in bit#22.

Proposal 17: the MIB content and PBCH payload in Table 5 and Table 6 should be supported for 120 kHz, 480 kHz and 960 kHz SSB.

Table 5 MIB and PBCH payload bit allocation for 120kHz SCS SSB

| bit |  | FR2-1 | FR2-2 |
| --- | --- | --- | --- |
|  |  | 120kHz | 120kHz |
| MIB | 0 | 10 - 5 MSB of SFN | 10 - 5 MSB of SFN |
|  | 1 |  |  |
|  | 2 |  |  |
|  | 3 |  |  |
|  | 4 |  |  |
|  | 5 |  |  |
|  | 6 | subCarrierSpacingCommon | 1 bit for NSSBQCL (sec 2.2) |
|  | 7 | ssb-SubcarrierOffset | ssb-SubcarrierOffset |
|  | 8 |  |  |
|  | 9 |  |  |
|  | 10 |  |  |
|  | 11 | dmrs-TypeA-Position | dmrs-TypeA-Position |
|  | 12 | pdcch-ConfigSIB1 /controlResourceSetZero | controlResourceSetZero (Sec 3.1 Table 1) |
|  | 13 |  |  |
|  | 14 |  |  |
|  | 15 |  |  |
|  | 16 | pdcch-ConfigSIB1 /searchSpaceZero | searchSpaceZero (Sec 3.3 Table 3) |
|  | 17 |  |  |
|  | 18 |  |  |
|  | 19 |  | 1 bit for NSSBQCL (sec 2.2) |
|  | 20 | cellBarred | cellBarred |
|  | 21 | intraFreqReselection | intraFreqReselection |
|  | 22 | spare | Spare bit |
| PBCH payload | 23 | 4th LSB of SFN | 4th LSB of SFN |
|  | 24 | 3th LSB of SFN | 3th LSB of SFN |
|  | 25 | 2th LSB of SFN | 3th LSB of SFN |
|  | 26 | 1th LSB of SFN | 3th LSB of SFN |
|  | 27 | half frame indication | half frame indication |
|  | 28 | 6th bit of candi. SSB index | 6th bit of candi. SSB index |
|  | 29 | 5th bit of candi. SSB index | 5th bit of candi. SSB index |
|  | 30 | 4th bit of candi. SSB index | 4th bit of candi. SSB index |

Table 6 MIB and PBCH payload bit allocation 480 kHz and 960 kHz SCS SSB

| bit |  | FR2-1 | FR2-2 |  |
| --- | --- | --- | --- | --- |
|  |  | 120kHz | 480kHz and 960kHz |  |
|  |  |  | DBTW OFF | DBTW ON |
| MIB | 0 | 10 - 5 MSB of SFN | 10 - 5 MSB of SFN |  |
|  | 1 |  |  |  |
|  | 2 |  |  |  |
|  | 3 |  |  |  |
|  | 4 |  |  |  |
|  | 5 |  |  |  |
|  | 6 | subCarrierSpacingCommon | 1 bit for NSSBQCL (sec 2.2) |  |
|  | 7 | ssb-SubcarrierOffset | ssb-SubcarrierOffset |  |
|  | 8 |  |  |  |
|  | 9 |  |  |  |
|  | 10 |  |  |  |
|  | 11 | dmrs-TypeA-Position | dmrs-TypeA-Position |  |
|  | 12 | pdcch-ConfigSIB1 /controlResourceSetZero | controlResourceSetZero (sec 3.2 Table 2) |  |
|  | 13 |  |  |  |
|  | 14 |  |  |  |
|  | 15 |  | searchSpaceZero (Sec 3.3 Table 4) |  |
|  | 16 | pdcch-ConfigSIB1 /searchSpaceZero |  |  |
|  | 17 |  |  |  |
|  | 18 |  |  |  |
|  | 19 |  | 1 bit for NSSBQCL (sec 2.2) |  |
|  | 20 | cellBarred | cellBarred |  |
|  | 21 | intraFreqReselection | intraFreqReselection |  |
|  | 22 | spare | Spare bit | 4th LSB of SFN |
| PBCH payload | 23 | 4th LSB of SFN | 4th LSB of SFN | 7th bit of candi. SSB index (sec 2.1) |
|  | 24 | 3th LSB of SFN | 3th LSB of SFN |  |
|  | 25 | 2th LSB of SFN | 3th LSB of SFN |  |
|  | 26 | 1th LSB of SFN | 3th LSB of SFN |  |
|  | 27 | half frame indication | half frame indication |  |
|  | 28 | 6th bit of candi. SSB index | 6th bit of candi. SSB index |  |
|  | 29 | 5th bit of candi. SSB index | 5th bit of candi. SSB index |  |
|  | 30 | 4th bit of candi. SSB index | 4th bit of candi. SSB index |  |

Random access design

Resolving RA-RNTI/ MsgB-RNTI overflow issue

When a random access preamble is transmitted, UE shall monitor RAR with RA-RNTI provided by

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id	(1)

where s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14), t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80), f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8), and ul_carrier_id is the UL carrier used for preamble transmission (0 for NUL carrier, and 1 for SUL carrier). The maximum value of t_id is 79 as there are 80 slots per system frame in 120 kHz SCS. With the support of 480kHz and 960kHz PRACH in FR2-2, the maximum number of slots per system frame increases to 640 slots. A straightforward modification of above RA-RNTI formula would then be

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 640 × f_id + 14 × 640 × 8 × ul_carrier_id. 	(2)

However, such modification would result in the maximum value of RA-RNTI exceeding the highest possible value of RNTI, i.e. 2^16-1.

Recall that we have agreed in RAN1 106-e that when the number of PRACH slots in a reference slot is 1, nslotRA=[7] for 480kHz and nslotRA=[15] for 960kHz PRACH and when the number of PRACH slots in a reference slot is 2, nslotRA=[3,7] for 480kHz and nslotRA=[7,15] for 960kHz PRACH. Assuming that the values in brackets are confirmed, the index of any two PRACH slots within a system frame differs at least by the value of 4 (8) in 480 kHz (960 kHz) or, in a more compact form, by the value of 2μ-3for μ=5, 6.

As all ROs are still maintained in the original PRACH slot nslotRA, the index of any two PRACH slots within a system frame differs at least by the value of 2μ-3, gNB can compress the index of the slot of the received RO in a system frame (tid) in the RA-RNTI formula to floor(tid2μ-3) without any loss of information and UE can recover the index of the PRACH slot from

RA-RNTI = 1+s_id+14×floor(tid2μ-3)+14×80×f_id+14×80×8×ul_carrier_id	(3)

Proposal 18: The RA-RNTI corresponding to 480 kHz and 960 kHz ROs can be generated according to equation (3) by compressing the t_id to tid2μ-3.

RAR window

In Rel-16 NR-U, RAR window was enlarged to the maximum of 40 ms to cope with the possible latency due to the LBT failure prior to msg2 transmission at gNB. In 2-step RACH, the same maximum length of 40 ms was also used to take into account the additional PUSCH processing delay at gNB as gNB needs to decode UE’s PUSCH appended to MsgA prior to sending MsgB. Issues of LBT failure prior to msg2 transmission at gNB for operation with shared spectrum and PUSCH processing delay at gNB prior to sending MsgB for both operations with and without shared spectrum should also be taken into account in 52.6GHz to 71GHz spectrum. Therefore, we propose to also enlarge ra-ResponseWindow for operation with shared spectrum and msgB-ResponseWindow for both operations with and without shared spectrum to the maximum of 40 ms. In this case, similar to Rel-16, the two bits in DCI format 1_0 with CRC scrambled by RA-RNTI (MsgB-RNTI) should also be used to indicate the two LSBs of SFN at which the gNB has received msg1 (MsgA).

Proposal 19: Support maximum of 40 ms for ra-ResponseWindow for operation with shared spectrum and msgB-ResponseWindow for both operations with and without shared spectrum. Support indicating two LSBs of SFN at which gNB has received msg1 (MsgA) in DCI format 1_0 with CRC scrambled by RA-RNTI (MsgB-RNTI).

Conclusions

In this paper, we discuss the issues related to initial access. The following observations and proposals are made.

Proposal 1: For SSB with 120 kHz SCS, confirm the working assumption on 64 candidate SSBs within a half frame. For SSB with 480 kHz and 960 kHz SCS, 64 candidate SSBs is sufficient for operation without shared spectrum while 128 candidate SSBs should be supported for operation with shared spectrum.

Proposal 2: Support following patterns (ALT C) for SSB with 480 kHz and 960 kHz SCS:

- For operations without shared spectrum:

  - {2,9}+14n, (n=0,1,2,…,31) for both 480 kHz and 960 kHz SCS.

- For operations with shared spectrum:

  - {2,9}+14n, (n=0,1,2,…,31,40,…,71) for 480 kHz SCS;

  - {2,9}+14n, (n=0,1,2,…,63) for 960 kHz SCS.

Proposal 3: For operation with shared spectrum and for 480 kHz and 960 kHz SSBs, indicate the 7th bit of the candidate SSB index by borrowing the 4th LSB of SFN in the PBCH payload. Indicate the 4th LSB of SFN with spare bit in MIB payload.

Observation 1: Contention exempt short control signaling for operation with shared spectrum is not supported in all regions. Therefore, even if the duty cycle of the 480/960 kHz SSB burst is less than 10% within every 100 ms, LBT must be used prior to 480/960 kHz SSB burst transmission in these regions.

Proposal 4: Confirm the working assumption to support DBTW for 120 kHz and further support DBTW for 480 kHz and 960 kHz SSB.

Proposal 5: At least  NSSBQCL=16,32,64 should be indicated in MIB for all three numerologies.

Proposal 6: UE should be able to identify DBTW enable/disable before acquiring SIB1. A specific entry of NSSBQCL in MIB should be reserved to disable DBTW if the number of candidate SSB positions is larger than 64 for 480 kHz and 960 kHz SCS.

Proposal 7: Configure DBTW length in SIB1 for operation with shared spectrum in 52.6GHz to 71GHz with the following values:

- 480 kHz SCS: {72, 32, 24, 16, 8, 4} slots = {2.25, 1, 0.75, 0.5, 0.25, 0.125} ms

- 960 kHz SCS: {64, 32, 24, 16, 8, 4} slots = {1, 0.5, 0.375, 0.25, 0.125, 0.0625} ms

Proposal 8: Use of LBT should be indicated in SIB1 to help UE determine the existence of “ChannelAccess-CPext” field in DCI format 1-0/0-0 scrambled with TC-RNTI and in RAR UL grant.

Proposal 9: In operation with shared spectrum in 60 GHz, for MSB k, k≥1, of inOneGroup and MSB m, m≥1, of groupPresense of ssb-PositionsInBurst:

- if MSB k of inOneGroup and MSB m of groupPresense are set to 1, the UE assumes that SSB(s) within DBTW with candidate SSB index(es) corresponding to SSB index equal to k-1+(m-1)×8 may be transmitted;

- if MSB k of inOneGroup or MSB m of groupPresense are set to 0, the UE assumes that the SSB(s) are not transmitted.

Proposal 10: Regardless of the value of the MSB k of inOneGroup and MSB m of groupPresense in ssb-PositionsInBurst configured in SIB1, if k-1+m-1×8> NSSBQCL, UE assumes that candidate SSB index(es) corresponding to SSB index equal to k-1+m-1×8 are not transmitted.

Proposal 11:  For CORESET for Type0-PDCCH in 52.6GHz to 71GHz spectrum, support the following:

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {120, 120} kHz, support multiplexing pattern 1 and multiplexing pattern 3 as per Agreement in RAN1 104-e.

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {480, 480} kHz, support multiplexing pattern 1 only.

- For {SS/PBCH Block, CORESET for Type0-PDCCH} SCS equal to {960, 960} kHz, support multiplexing pattern 1 only.

Proposal 12: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={120, 120} kHz:

- For CORESET#0 with 24 RBs and 48 RBs: the same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: additional RB offsets values of 0 and 28 RBs can be considered for multiplexing pattern 1.

- For CORESET#0 with 96 RBs: RB offsets values of 0 and 76 RBs can be considered for multiplexing pattern 1.

- Note: All above RB offsets are nominal and may need to be modified after finalizing synch raster and channel raster design in FR2-2.

Proposal 13: Support the following CORESET#0 RB offsets values for {SSB, CORESET#0} SCS={480, 480} kHz and {960, 960} kHz:

- For CORESET#0 with 24 RBs: the same as supported values in Table 13-8 of 38.213.

- For CORESET#0 with 48 RBs: In addition to the offset of 14 RBs already supported in Rel-16, additional values of 0 and 28 RBs can be considered for multiplexing pattern 1.

Proposal 14: The parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 with {120, 120} kHz in Table 3 should be supported.

Proposal 15: Support the parameters for PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 1 with {480, 480} kHz and {960, 960} kHz in Table 4 that is obtained from the agreed Table in RAN1 106b-e by selecting  X=52μ-3 when DBTW is OFF and X=52μ-4 when DBTW is ON and removing entries with Y.

- Note: DBTW OFF is indicated in MIB using a value of NSSBQCL

Proposal 16: To find the offset between an off-synch raster SSB and the corresponding CORESET#0 in 60GHz unlicensed spectrum, RAN1 should uniquely determine the hypothetical on-synch raster SSB that serves as the reference for the offset to the off-synch raster SSB in case more than one synch rasters are included in a channel bandwidth.

Proposal 17: the MIB content and PBCH payload in Table 5 and Table 6 should be supported for 120 kHz, 480 kHz and 960 kHz SSB.

Proposal 18: The RA-RNTI corresponding to 480 kHz and 960 kHz ROs can be generated according to equation (3) by compressing the t_id to tid2μ-3.

Proposal 19: Support maximum of 40 ms for ra-ResponseWindow for operation with shared spectrum and msgB-ResponseWindow for both operations with and without shared spectrum. Support indicating two LSBs of SFN at which gNB has received msg1 (MsgA) in DCI format 1_0 with CRC scrambled by RA-RNTI (MsgB-RNTI).

- References

- Chair’s Notes RAN1#106-e

- R1-2108589, “Summary #6 of email discussion on initial access aspect of NR extension up to 71 GHz”, Moderator (Intel Corporation)

- RP-211584, “Revised WID:  Extending current NR operation to 71 GHz” RAN #92-e, Qualcomm, Intel.

- TS 38.213.

- ETSI EN 301 893.

- ETSI EN 302 567.

- TS 38.331.