3GPP TSG RAN WG1 Meeting #107-e	R1-2110859

e-Meeting, November 11th – November 19th, 2021

Agenda Item:	8.16.9

Source:	Huawei, HiSilicon

Title:	Rel-17 UE features for NB-IoT and LTE-MTC enhancements

Document for:	Discussion and Decision

- Introduction

This contribution discusses the UE features for NB-IoT and LTE-MTC enhancements.

- Rel-17 UE features

In last meeting, the discussion for Rel-17 UE features for NB-IoT and LTE-MTC enhancements is summarized in [1].

For Rel-17 NB-IoT UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH, the need of FDD/TDD differentiation is [Yes] in the preliminary version. Regarding support of 16-QAM, we failed to see the difference between TDD and FDD. Furthermore, for the legacy NB-IoT, the MCS table for unicast NPDSCH and unicast NPUSCH is the same for TDD and FDD. Therefore, the need of FDD/TDD differentiation should be no for UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH.

Proposal 1: The need of FDD/TDD differentiation should be no for UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH.

For the UE capability of supporting Alt-1 and Alt-2e HARQ-ACK delay, Alt-1 HARQ-ACK delay can be used for all the bitmap configured and Alt-2e HARQ-ACK delay can be used for configurations with low percentage of non-BL/CE subframes to save the DCI signaling. Therefore, the UE should support both Alt-1 and Alt-2e.

Proposal 2: For FG 1-3, UE should support both Alt-1 and Alt-2e HARQ-ACK delays.

- Conclusion

In this contribution, our views on Rel-17 UE features for NB-IoT and LTE-MTC enhancements are provided. The following proposals are made.

Proposal 1: The need of FDD/TDD differentiation should be no for UE features 16-QAM for unicast NPDSCH and 16-QAM for unicast NPUSCH.

Proposal 2: For FG 1-3, UE should support both Alt-1 and Alt-2e HARQ-ACK delays.

- References

- R1-2109715	Summary on UE features for NB-IoT and LTE-MTC enhancements	Moderator (NTT DOCOMO, INC.)