3GPP TSG RAN WG1 #107-e	R1-2110866

e-Meeting, November 11 – November 19, 2021

Agenda item:		8.8.2

Source:	Nokia, Nokia Shanghai Bell

Title:	PUCCH coverage enhancements

Document for:		Discussion and Decision

# Introduction

The following can be noted from the work item description (WID) for Rel-17 coverage enhancement [1]:

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DM-RS bundling across PUCCH repetitions [RAN1, RAN4]

As described in the WID, two main features are introduced for PUCCH coverage enhancements in Rel-17: the support of dynamic PUCCH repetition factor indication and the support of DM-RS bundling (or joint channel estimation) across PUCCH repetitions. This document expresses our views on these two features, stemming from agreement in RAN1 #106-bis-e.

# Discussion

## Dynamic PUCCH repetition factor indication

In RAN1 #106-e, it was agreed to confirm the working assumption agreed in RAN1 #105-e regarding the dynamic indication of the PUCCH repetition factor:

Confirm the following working assumption Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication. - Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor. - FFS: RRC signaling enhancement details

The confirmed working assumption has however an FFS point on RRC signaling enhancement details, which is left open for further discussion. Such details on integration of dynamic repetition factor per PUCCH resource should be left to RAN2, even though a technical implication of such integration has relevant RAN1 impact. Consider, for instance, Figure 1 and Figure 2, where an example implementation for configuring PUCCH repetition factor per PUCCH resource and the current per-format PUCCH repetition factor configuration via the parameter nrofSlots in PUCCH-FormatConfig IE, respectively, are provided. The latter configuration, in particular, applies to PUCCH formats 1,3 and 4.

Figure 1. RRC configuration details for PUCCH dynamic repetition factor

Figure 2. Configuration of per-format PUCCH repetition factor as per TS 38.331

In this context, in case the Rel-17 parameter repetitionFactor is configured for a specific PUCCH resource, ambiguity may exist at the UE on which repetition factor to apply for the PUCCH, since the Rel-15/16 parameter nrofSlots (in PUCCH-FormatConfig) is also configured and applies per PUCCH format. Conversely, in case the parameter repetitionFactor is not configured per PUCCH resource, no ambiguity exists, and UE uses the repetition factor indicated by nrofSlots, if applicable.

This interaction between the two RRC configured repetition factors was further discussed in RAN1 #106-e, and the following agreement was reached:

Agreement: for a PUCCH resource, if both a new repetition parameter corresponding to Rel-17 dynamic PUCCH repetition factor indication and the Rel-15/16 nrofSlots are configured, the new repetition parameter overrides nrofSlots.

In our view, a clarification for this agreement is necessary when considering that it may not apply exclusively to the case of a PUCCH with associated scheduling DCI. Indeed, when configuring PUCCH resources for semi-static CSI reporting, the network would refer to the resource index of a PUCCH resource configured in PUCCH-Config. So, if the selected PUCCH resource is characterized by a dynamic repetition factor, the periodic CSI report would be repeated as many times as indicated in the dynamic repetition factor field.

We believe this creates a constraint in terms of configuration of the PUCCH resource for a specific CSI report. Indeed, with the legacy configuration of number of repetitions through the parameter nrofSlots, it is always possible to configure the repetition number independently from the specific PUCCH resource and only based on SNR conditions of the specific UE. Based on the number of bits to transmit and the network load, the network can then select an appropriate PUCCH resource (in the time-frequency grid) for a particular reportQuantity of the CSI report. Conversely, this may not be possible if the agreement above would apply also to PUCCH resources for CSI reporting. The network would indeed be constrained in the allocation of the PUCCH resources for CSI reporting to only the ones having a repetition factor that matches the SNR conditions of the specific UE. This is not desirable from the point of view of configuration of a periodic or semi-persistent CSI report, since correspondent PUCCH resources of several UEs need to be properly fit in the time-frequency grid of the uplink or flexible slots.

Observation 1: overriding nrofSlots with the new repetition parameter in the case the PUCCH resource is associated with a CSI report is not desirable from the point of view of configuration of a periodic or semi-persistent CSI report

This issue was further discussed during RAN1 #106-bis-e, with the resulting FL’s proposal:

FL proposal 1-1: For a PUCCH resource to transmit a PUCCH without an associated scheduling DCI (e.g. P/SP-CSI or SR), if the PUCCH resource is configured with RRC parameter “nrofSlots-r17”, “nrofSlots-r17” is ignored and the RRC parameter “nrofSlots” is used for determining the repetition factor of the specific PUCCH resource.

In view of these considerations, and also considering the most recent discussion in RAN1 #106-bis-e, we propose to extend the above agreement (in RAN1 #106-e) to clarify that in the case the PUCCH resource is associated with a CSI report (or, more generically, the PUCCH resource is not associated with a scheduling DCI) and the PUCCH resource is characterized by a dynamic repetition factor, the new repetition parameter does not override the parameter nrofSlots but rather nrofSlots is used for determining the repetition factor of the specific PUCCH resource.

Proposal 1: in the case a PUCCH resource is not associated with a scheduling DCI (e.g. PUCCH resource associated with CSI report) and the PUCCH resource is characterized by a dynamic repetition factor, the parameter nrofSlots is used for determining the repetition factor of the specific PUCCH resource.

In RAN1 #106-e, it was also agreed the following on the number of PUCCH repetitions:

Agreement​ - In Rel-17, reuse the Rel-16 PUCCH repetition factors 2, 4, 8. ​ - Do not support PUCCH repetition factor larger than 8 In Rel-17.

A behavior for the case when the network wants to configure and dynamically indicate a PUCCH repetition factor equal to 1 (no repetition) has not been defined. Indeed, at this point, the only way for having a PUCCH resource with repetition factor equal to 1 is to define the PUCCH resource without the new repetition factor parameter and set the field nrofSlots as empty. In our view, this creates strong limitations for example in the cases the UE experiences a successful RRC connection at low SNR, at which point nrofSlots (i.e. 2, 4 or 8 repetitions) was configured and tailored to the specific channel condition, but then starts observing good SNR conditions. In line with the motivations of introducing the dynamic indication of repetition factor for PUCCH, RAN1 should not only consider the cases where the UE worsens its channel conditions but also those cases in which the UE improves its channel conditions, compared to the state during RRC connection. For this reason, we believe that a PUCCH repetition factor equal to 1 should be additionally supported in Rel-17.

Proposal 2: support PUCCH repetition factor equal to 1 in Rel-17

## DM-RS bundling across PUCCH repetitions

In RAN1 #104-e, the following agreements on DM-RS bundling across PUCCH repetitions were made:

Agreements: Subject to the prerequisites of DM-RS bundling for PUCCH repetitions, support enabling PUCCH repetitions with DM-RS bundling via RRC configuration. ​ ​ - FFS: the configuration is per UE or per PUCCH resource. ​ - FFS: whether additional dynamic signaling is needed to enable/disable PUCCH repetitions with DM-RS bundling​ - FFS: necessity of additional signaling/configuration of DM-RS bundling duration/window and associated size

In RAN1 #106-e some progress was made on the first two FFS bullets of the agreement in RAN1 #104-e, but no actual agreement was made as can be noticed from FL summary [4].

A relevant excerpt of [4] is reported below:

FL proposal 6: The granularity of RRC configuration for DM-RS bundling across PUCCH repetitions is down selected from the following two options: - Option 2: per UL BWP - Option 3: per PUCCH resource format Proposed conclusion 2: Dynamic signaling to enable/disable DM-RS bundling for PUCCH or PUSCH repetitions is not supported in Rel-17. - Support by: Sharp, CATT, LG, OPPO, Panasonic, Intel, Lenovo/Motorola, Ericsson, Apple, Docomo, Nokia/NSB, Intel, CT, ZTE. - Object by:

Further discussion on the second FFS bullet occurred in RAN1 #106-bis-e, and the following agreement was reached:

Agreement: Dynamic signaling to enable/disable DM-RS bundling for PUCCH or PUSCH repetitions is not supported in Rel-17.

RRC configuration for DM-RS bundling across PUCCH repetitions

Based on the discussion in RAN1 #106-e, it emerges that many companies support a down-selection of the RRC configuration for DM-RS bundling across PUCCH repetitions to either configuration per UL BWP or per PUCCH resource format.

In our view, configuring DM-RS bundling per UL BWP (i.e., in the IE PUCCH-Config) is enough and rather straightforward, since the UE specific PUCCH parameters are RRC configured per BWP. This way, the feature will directly apply to all PUCCH resources with repetitions and DM-RS for channel estimation. Such configuration would finally be based on UE capability (Rel-17 UE).

Proposal 3. The configuration of DM-RS bundling across PUCCH repetitions is per UL BWP and subject to UE capability.

### Inter-slot FH with DM-RS bundling

In RAN1 #106-bis-e, the following was agreed for the interaction of inter-slot FH and DM-RS bundling:

Agreement: For the interaction between inter-slot frequency hopping and DM-RS bundling for PUCCH/PUSCH repetitions, a UE perform the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering. One option of the following options is to be selected. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - Option 2: “configured TDW determination” -> “hopping intervals determination” -> “actual TDW determination” - Option 4: “configured TDW determination” -> “actual TDW determination” and “hopping intervals determination” Note: option 1 and 2 assume a hopping interval can be different than an actual TDW. Option 4 assumes a hopping interval is the same as an actual TDW.

Before discussing the agreed options in greater detail, we observe the following:

- TDWs are configured and determined based on the counting method for the slots (based on consecutive physical or available slots), and DM-RS belonging to two different configured TDWs cannot be bundled together. Therefore, determining hopping intervals and configured TDW are two operations whose order can be inverted, without altering the result, i.e., Option 1 and Option 2 would lead to the same outcome.

- The only difference between the three considered options is the role of FH in the context of the actual TDWs determination. For Option 1 and Option 2, FH is a semi-static event that determines the end of an actual TDW, and the start of a new one within the same configured TDW, if applicable. Conversely, FH is not considered a semi-static event which determines the end of an actual TDW, and the start of a new one within the same configured TDW, if applicable. In this case, FH is not a semi-static event which impacts actual TDW determination.

- Down-selection among Option 1, Option 2 and Option 4 is equivalent to agree on an answer to the question “is FH a semi-static event which impacts actual TDW determination?”

Consider now the example scenario of Figure 3, where a UE is expected to transmit 8 repetitions in 8 contiguous slots, with two events violating power consistency and phase continuity between repetition 2 and 3 (rep2 and rep3) and repetitions 6 and 7 (rep6 and rep7). Here we assume that the frequency hopping interval is configured as half the number of repetitions (4 slots) and the configured window has a length L=8 slots. In addition, in the remainder of this Section, we will analyse Option 1 and Option 2 together. This is because:

If the UE were to comply with Option 1 and/or Option 2, the UE would transmit the PUCCH repetitions as shown in Figure 4. At most two PUCCH repetitions could be bundled together for joint channel estimation and UE transmits in one frequency hop for half of the repetitions (4 out of 8 slots), as determined in the first or second step.

Differently, if the UE were to comply with Option 4, the UE would transmit the PUCCH repetitions as shown in Figure 5. Although, as in Option 1 and 2, the UE still transmits in one frequency hop for half of the repetitions (4 out of 8 slots), yielding a same frequency diversity gain, in this case the joint channel estimation is not anymore limited to two PUCCH repetitions but can be extended to 4 PUCCH repetitions for the 2nd hop, offering a better reliability of the channel estimate and hence coverage enhancements.

Based on the above analysis, it is evident that Option 4 offers the best performance and interaction between frequency hopping and joint channel estimation, enabling improved performance of the channel estimation but still allowing a same degree of frequency diversity.

Observation 2. Option 4 offers the best performance and interaction between frequency hopping and joint channel estimation, enabling improved performance of the channel estimation but still allowing a same degree of frequency diversity.

Proposal 4. Select Option 4: “configured TDW determination” -> “actual TDW determination” and “hopping intervals determination”, for the interaction between inter-slot frequency hopping and DM-RS bundling for PUCCH/PUSCH repetitions

Figure 3. PUCCH repetitions across 8 slots with two events violating the conditions of power consistency and phase continuity

Figure 4. Determination of frequency hopping interval, configured TDW and actual TDW in case of Option 1 and/or Option 2

Figure 5. Determination of frequency hopping interval, configured TDW and actual TDW in case of Option 4

In RAN1 #106bis-e, companies raised concerns regarding the poorer spectral efficiency that Option 4 would lead to, as compared to Option 1 and Option 2. More precisely, it was argued that Option 1 would allow a gNB to allocate resources in an efficient manner and would enable interlaced hopping of UEs sharing the same RBs (startingPRB and secondHopPRB), even if only a part of the UEs is using the DM-RS bundling feature. In our view, this would be correct and possible only if the UEs using the DM-RS bundling feature still comply with the hopping pattern defined in Rel-16 (i.e. slot-number-based hop), which would however not allow DM-RS bundling since phase continuity cannot be kept with frequency hopping and for a non-back-to-back PUCCH/PUSCH repetitions with a gap larger than 13 symbols. Conversely, if the frequency hopping pattern is modified for the UEs using DM-RS bundling, we do not see how a gNB could be able to optimize the spectral utilization between such UEs and legacy UEs. Indeed, spectral optimization would now become a more complex problem, wherein a knowledge of whether the slot is even or odd is not enough anymore, but other factors such as the UE specific frequency hopping interval size and number of repetitions should be considered in the optimization. In other words, the scheduling optimization problem to minimize spectral inefficiencies would be the same for all 3 considered options.

As an example, we could think of a scenario in which a gNB needs to optimize spectral allocation for a first Rel-16 UE transmitting PUCCH repetitions without DM-RS bundling and a second Rel-17 UE transmitting PUCCH repetitions using DM-RS bundling, in an FDD band, for simplicity, as shown in Figure 6. No discussion has occurred so far in RAN1 regarding the definition of a new frequency hopping pattern for UEs supporting DMRS bundling. Indeed, re-using Rel-16 frequency hopping pattern does not seem to be an option since, as clarified also by RAN4, phase continuity cannot be maintained when frequency hopping is enabled. Hopping intervals longer than 1 slot need to be discussed and defined in RAN1, for enabling DMRS bundling even when frequency hopping is active.

Figure 6. Frequency multiplexing of frequency hopping UEs with different hopping patterns

It is readily visible that if the Rel-16 UE and Rel-17 UE use the same (but reversed) PRBs for PUCCH transmission and inter-slot frequency hopping, half of the slots will be subject to collision. So, even in this case aligned with Option 1 above, gNB will likely not be able to use the same PRBs for two different UEs and hence optimize spectral allocation. To avoid collisions, gNB would need to consider both startingPRB and secondHopPRB of one of the two UEs as occupied when scheduling PUCCH transmission of the other UE. This operation mechanism will finally be applicable to all the options listed above (Option 1, Option 2 and Option 4) without any complication.

Observation 3. To avoid collisions between UEs with different frequency hopping patterns, gNB needs to consider both startingPRB and secondHopPRB of one UE as occupied when scheduling PUCCH transmission of one other UE. This operation mechanism is applicable to all the options listed above (Option 1, Option 2 and Option 4) without any complication.

Switching the focus to the frequency hopping patterns, we observe that no discussion has occurred so far in RAN1 regarding the definition of a new frequency hopping pattern for UEs supporting DM-RS bundling. However, and as discussed above, such definition is paramount to ensure the good success of the feature. Indeed, re-using Rel-16 frequency hopping pattern does not seem to be an option for DM-RS bundling since, as clarified also by RAN4, phase continuity cannot be maintained when frequency hopping is enabled. Hopping intervals longer than 1 slot need to be discussed and defined in RAN1, for enabling DM-RS bundling even when frequency hopping is active.

Proposal 5. RAN1 to discuss and define hopping intervals longer than 1 slot

FFS: details for definition of hopping intervals

### PUCCH TDW restart

In RAN1 #106-e, the following was agreed in AI 8.8.1.3 regarding the restart of the DM-RS bundling TDW after an event:

Agreement Down-sselect one of the following options: - Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DM-RS bundling is applied only to dynamic events. - Option 2: UE capability of restarting DM-RS bundling is applied to both semi-static events and dynamic events.

In our opinion, such capability should apply also to DM-RS bundling across PUCCH repetitions, without having to define a completely new capability. In case this is deemed not possible for practical reasons, we believe that at least the two capabilities operate the same manner. The way the capability applies to dynamic and semi-static events should be the same between PUSCH and PUCCH.

Proposal 6. UE capability of restarting DM-RS bundling after a semi-static or dynamic event is the same for PUCCH and PUSCH.

This agreement is directly related to the discussion in Section 2.2.1, when we consider frequency hopping as an event violating phase continuity. Indeed, if UE capability of restarting DM-RS bundling is applied to both semi-static events and dynamic events, it would mean:

- For Option 1 and 2 in Section 2.2.1, as soon as UE changes frequency hop, DM-RS bundling cannot be applied to the other hop, if within a same configured window. This would de facto prohibit the application of DM-RS bundling until the end of the configured TDW, regardless of how many actual TDWs are created inside the configured TDW itself. This would negatively impact the channel estimation performance of at the least the second hop and would implicitly exclude the possibility of enabling frequency hopping together with DM-RS bundling, de facto making the whole feature meaningless in practice.

- For Option 4, this would have zero impact on the performance of the DM-RS bundling, given that the frequency hooping interval determination would occur after the actual TDW determination.

Conversely, if UE is mandated to support restarting DM-RS bundling due to semi-static events and UE capability of restarting DM-RS bundling is applied only to dynamic events, all the 3 Options for the interaction of inter-slot FH and DM-RS bundling in Section 2.2.1 could be supported, and the DM-RS bundling feature would be meaningful in practice. For this reason, we think that the support of restarting DM-RS bundling due to semi-static events is mandatory for the UE.

Observation 4. Option 2 “UE capability of restarting DM-RS bundling is applied to both semi-static events and dynamic events.” creates complications and overhead in the case frequency hopping and DM-RS bundling are jointly enabled.

Proposal 7. For the restart of the DM-RS bundling TDW for PUCCH, down-select Option 1: If DM-RS bundling is supported, the support of restarting DM-RS bundling due to semi-static events is mandatory for the UE. UE capability of restarting DM-RS bundling is applied only to dynamic events.

### Discussion on use cases for DM-RS bundling

The following agreement was made in RAN1 #106-e regarding the use cases for DM-RS bundling:

Agreement​: - For DM-RS bundling for PUCCH repetitions, RAN1 at least prioritize use cases 3 and 4a in R1-2104119.​

The use cases 3 and 4(a) are defined as follows:

- Use case 3: back-to-back PUCCH repetitions across consecutive slots.

- Use case 4: non-back-to-back PUCCH repetitions across consecutive slots.

  - Use 4a: no uplink transmission in the middle of two PUCCH repetitions

In our view, differently from use case 3, which seems to be targeting repetitions of long PUCCH format spanning the whole slot, further discussion is needed in RAN1 for clear definition of use case 4a. In particular, it is important to discuss and understand the UE behaviour in the case an uplink transmission occurs in between two non-back-to-back PUCCH repetitions.

Let us take as an example the scenario of Figure 3, where the UE is expected to transmit an UL channel (e.g., periodic CSI report) in between two PUCCH repetitions. This scenario does not comply with the conditions of use case 4a and because of this, the UE would either break phase continuity between rep2 and rep3 for transmission of the UL channel or ignore the UL occasion and keep phase continuity across all 4 repetitions (assuming max duration constraint is fulfilled).

In this case, to avoid ambiguity between gNB and UE operations, gNB needs to indicate to UE which behaviour to follow across the PUCCH repetitions (i.e., within the TDW) in either a dynamic or semi-static way, before the start of the PUCCH repetitions. For example, and taking the WA on TDW made in RAN1 #106b-e as a reference, if the UE supports a max duration of the TDW equal to 4 consecutive slots but gNB indicates to UE to transmit other UL channels during the TDW, the UE will break phase continuity after the second repetition and automatically split the TDW is two actual time domain windows of length 2 slots. Conversely, gNB could indicate to ignore any other UL transmission occurring in between the PUCCH repetitions, implicitly instructing the UE to keep phase and power continuity across the whole TDW.

Observation 5. To avoid ambiguity between gNB and UE operations, gNB needs to indicate to UE which behaviour to follow across the PUCCH repetitions (i.e., within the time domain window) for transmission of other UL channels

However, such behaviours (dropping or transmission of the other UL channel) can be regarded as sub-optimal since they would either impact the performance of joint channel estimation or the information available at gNB for the specific UE (e.g., in case the UL transmission carries a periodic CSI report). To ensure system performance does not incur unnecessary loss, one option would be for the UE to dynamically adapt the settings of the other UL transmission to avoid breaking phase continuity and still transmit the UL channel. In our understanding, this would still comply with the conditions of use case 4a where the uplink transmission between two PUCCH repetitions is intended as having different settings than the PUCCH repetitions. Indeed, according to the feedback from RAN4, phase continuity cannot be guaranteed whenever the other UL transmission has at least one different setting than the PUCCH repetitions in terms of antenna port, occupied PRBs and UL power.

Figure 7. UL transmission in between two PUCCH repetitions

Adaptation of UL transmission to avoid breaking phase continuity and still transmit the UL channel can be realized by adapting the settings of the other UL transmission to make it the same as the one used for transmitting PUCCH repetitions. This way, this scenario is turned into the case of “other UL transmission” with same settings as PUCCH repetitions, as shown in Figure 4 where a PUCCH with different settings (e.g. semi-statically configured PUCCH) occurs between rep2 and rep3.

Figure 8. An example of adapting the settings of the PUCCH to be the same as PUCCH when the PUCCH is configured with different settings than the PUCCH repetitions.

For the sake of discussion, if we assume that the other UL transmission is a configured PUCCH (as in Figure 3), the most challenging setting is to have the same occupied PRBs between the configured PUCCH and PUCCH repetitions. Indeed, antenna port and UL power can be dynamically controlled by the gNB but the occupied PRBs for the configured PUCCH is semi-statically configured.

Therefore, a rule needs to be specified to adapt the occupied PRBs of the configured PUCCH to be the same as the ones for PUCCH repetitions. Some specification impacts may be envisioned to apply this option to PUCCH formats 0/1/4, which support only 1 PRB. Conversely, no issue is envisioned for supporting this option for PUCCH formats 2/3 (which support up to 16 PRBs). This does not seem to cause any practically relevant issues for real deployments given that:

- PF2 and PF3 are arguably the formats for which coverage shortage over the PUCCH of a single UE is expected to occur (for both FR1 and FR2). Indeed, their payload size is larger than what can be transmitted over PF0/PF1. Their relevance is further confirmed by the fact that they are the typical solution to realize a P-CSI report. Relevance of PF4 in this context may be application dependent.

- A small number of PRBs is expected to be scheduled in coverage shortage situations for PUCCH repetitions.

- Depending on the availability of the REs within the occupied PRBs of PUCCH repetitions, the gNB should be able to indicate to the UE whether this option can be adopted or not.

From the above analysis, we propose the following:

Proposal 8. For non-back-to-back PUCCH transmissions, in case the other UL transmission in between two successive PUCCH repetitions has different settings than PUCCH, the gNB indicates one of the following options to the UE:

- Option 1: Drop the other UL transmission with different settings.

- Option 2: Transmit the other UL transmission with different settings and break the phase continuity.

- Option 3: Adapt the settings of the other UL transmission to make it be the same as the PUCCH repetitions.

# Conclusion

In this contribution, we expressed our views on dynamic PUCCH repetition factor indication and DM-RS bundling across PUCCH repetitions.

We made the following proposals and observations:

Observation 1. Overriding nrofSlots with the new repetition parameter in the case the PUCCH resource is associated with a CSI report is not desirable from the point of view of configuration of a periodic or semi-persistent CSI report

Observation 2. Option 4 offers the best performance and interaction between frequency hopping and joint channel estimation, enabling improved performance of the channel estimation but still allowing a same degree of frequency diversity.

Observation 3. To avoid collisions between UEs with different frequency hopping patterns, gNB needs to consider both startingPRB and secondHopPRB of one UE as occupied when scheduling PUCCH transmission of one other UE. This operation mechanism is applicable to all the options listed above (Option 1, Option 2 and Option 4) without any complication.

Observation 4. Option 2 “UE capability of restarting DM-RS bundling is applied to both semi-static events and dynamic events.” creates complications and overhead in the case frequency hopping and DM-RS bundling are jointly enabled.

Observation 5. To avoid ambiguity between gNB and UE operations, gNB needs to indicate to UE which behaviour to follow across the PUCCH repetitions (i.e., within the time domain window)

Proposal 1. In the case a PUCCH resource is not associated with a scheduling DCI and the PUCCH resource is characterized by a dynamic repetition factor, the parameter nrofSlots is used for determining the repetition factor of the specific PUCCH resource.

Proposal 2. Support PUCCH repetition factor equal to 1 in Rel-17

Proposal 3. The configuration of DM-RS bundling across PUCCH repetitions is per UL BWP and subject to UE capability.

Proposal 4. Select Option 4: “configured TDW determination” -> “actual TDW determination” and “hopping intervals determination”, for the interaction between inter-slot frequency hopping and DM-RS bundling for PUCCH/PUSCH repetitions

Proposal 5. RAN1 to discuss and define hopping intervals longer than 1 slot

FFS: details for definition of hopping intervals

Proposal 6. UE capability of restarting DM-RS bundling after a semi-static or dynamic event is the same for PUCCH and PUSCH.

Proposal 7. For the restart of the DM-RS bundling TDW for PUCCH, down-select Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DM-RS bundling is applied only to dynamic events.

Proposal 8. For non-back-to-back PUCCH transmissions, in case the other UL transmission in between two successive PUCCH repetitions has different settings than PUCCH, the gNB indicates one of the following options to the UE:

- Option 1: Drop the other UL transmission with different settings.

- Option 2: Transmit the other UL transmission with different settings and break the phase continuity.

- Option 3: Adapt the settings of the other UL transmission to make it be the same as the PUCCH repetitions.

# References

- RP-202928	New WID on NR coverage enhancements, China Telecom (moderator), December 2020.

- R4-2103393 Reply LS on PUCCH and PUSCH repetition, Qualcomm (moderator), February 2021.

- R1-2106657	Joint channel estimation for PUSCH coverage enhancements, Nokia/NSB, August 2021.

- R1-2108619, FL summary of PUCCH coverage enhancement, August 2021.