3GPP TSG-RAN WG1 Meeting #107-e 	R1-2110869

e-Meeting, November 11th – 19th, 2021

| oCR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [Draft] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | - | rev | - | Current version: | 15.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the description of SSBRI report in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2021-11-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the description in section 5.2.1.4.2 of TS 38.214 as follows, the SSBRI should always be reported. For example, the number of resources in csi-SSB-ResourceList is 1, the SSBRI should also be reported with value 0. TS 38.214 section 5.2.1.4.2 “If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-RSRP', the UE shall report SSBRI, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet. ” However, the bitwidth of SSBRI field is 0 when the number of resources in csi-SSB-ResourceList is 1 according to section 6.3.1.1.3 of TS 38.212 as below. TS 38.212 section 6.3.1.1.2 “The bitwidth for CRI, SSBRI, RSRP, and differential RSRP are provided in Table 6.3.1.1.2-6. Table 6.3.1.1.2-6: CRI, SSBRI, and RSRP Field; Bitwidth. CRI. SSBRI. RSRP; 7. Differential RSRP; 4. where is the number of CSI-RS resources in the corresponding resource set, and is the configured number of SS/PBCH blocks in the corresponding resource set for reporting 'ssb-Index-RSRP'.” Therefore, there is a mismatch in SSBRI reporting when the number of SSB in the corresponding resource list is 1. |
| T |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | To exclude the case that the number of SSB in the corresponding resource list is 1 when reporting SSBRI in section 5.2.1.4.2 of 38.214; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behavior is unclear in terms of SSBRI reporting when the number of SSB in the corresponding resource list is 1. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis: - No impact to existing gNB and UE implementation. . |  |  |  |  |  |  |  |  |

##### 5.2.1.4.2	Report Quantity Configurations

< Unchanged parts are omitted >

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'ssb-Index-RSRP', the UE shall report SSBRI except the case where the size of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet is 1, where SSBRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated csi-SSB-ResourceList in the corresponding CSI-SSB-ResourceSet.

< Unchanged parts are omitted >