3GPP TSG RAN WG1 #107-e	R1-2110870

# e-Meeting, November 11th – 19th, 2021‎

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR |  | rev | - | Current version: | 15.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on data and control multiplexing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the section of Data and control multiplexing, if the number of HARQ-ACK bits is less than or equal to 2, and only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, the reserved resource for potential HARQ-ACK transmission is not subtracted when calculating the number of coded bit for CSI part 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Under the condition of “if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH”, add a sub-condition of “if the number of HARQ-ACK information bits is more than 2,” before the exiting formula to calculate the number of coded bit of CSI part 1 in the first hop, and add the formula for the other cases, |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE may derive wrong number of coded bit for CSI part 1 if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH and the HARQ-ACK bit is no more than 2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.7	Data and control multiplexing

< Unchanged part is omitted >

If frequency hopping is configured for the PUSCH,

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol after the first set of consecutive OFDM symbol(s) carrying DMRS in the second hop.

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the first hop;

-	denote  as the OFDM symbol index of the first OFDM symbol that does not carry DMRS in the second hop;

-	if HARQ-ACK is present for transmission on the PUSCH with UL-SCH, let

-	 and ;

-	if CSI is present for transmission on the PUSCH with UL-SCH, let

-	;

-	;

-	; and

-	;

-	if only HARQ-ACK and CSI part 1 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2,  GCSI-part11=M1∙NL∙Qm-GACK1; otherwise, GCSI-part11=M1∙NL∙Qm-GrvdACK1

-	;

-	if HARQ-ACK, CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	if the number of HARQ-ACK information bits is more than 2,	; otherwise,

-	;

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise; and

-	 if the number of HARQ-ACK information bits is no more than 2, and  otherwise;

-	if only CSI part 1 and CSI part 2 are present for transmission on the PUSCH without UL-SCH, let

-	;

-	;

-	; and

-	;

-	let , and denote ,  as the number of OFDM symbols of the PUSCH in the first and second hop, respectively;

-	 is the number of transmission layers of the PUSCH;

-	 is the modulation order of the PUSCH;

-	;

-

-	.

< Unchanged part is omitted >