3GPP TSG RAN WG1 Meeting #107-e	R1-2110885

e-Meeting, November 11 – 19, 2021

Agenda Item:	8.14.1

Source:	Futurewei

Title:	XR evaluation results

Document for:	Discussion and decision

# Introduction

In RAN1#104-bis-e meeting, the Rel. 17 FS_NR_XR_eval SID [1] was discussed and many agreements were made in the context of evaluation methodology and traffic model characterization [2].

In [3][4][5][6], we provided some initial XR evaluation results with focus on capacity analysis. The network deployment assumed were the Urban Macro FR1 and Dense Urban FR1 scenarios. Two transmission schemes were evaluated. A baseline zero-forcing transmission scheme and a transmission scheme based on cooperative MIMO. It is shown that the XR system capacity is limited with few satisfactory UEs that satisfy the reliability and latency requirements with the assumption of the baseline zero-forcing scheme. A precoding technique based on DL interference probing based on flexible A-SRS triggering was then evaluated and compared to baseline zero-forcing precoding.

In this paper, on top of the more downlink simulation configurations that we presented in [6], such as Dense Urban FR1 vs. Urban Macro UMa, SU-MIMO vs. MU-MIMO performances, downlink slot configuration of [DDDSU] vs. [DDDUU], and data rates of 45Mbps vs. 30Mbps, we will present more simulations: We split the XR applications to AR/XR and CG, and consider PDB of 7ms, 13ms, 15ms.

Like the analysis presented in [3][4][5][6], the performance assuming the two different precoding schemes are compared. It is shown that the gain in terms of XR system capacity with enhanced cooperative MIMO scheme with MU-MIMO is substantial in the range of 50~90% for various configurations.

In addition to the initial uplink results for XR “Pose and Control” data for both Urban Macro and Dense Urban scenarios, in this paper, we will present uplink data “stream” performances.

# FR1 Simulation Assumptions

## Simulation Assumptions

This section includes the simulation assumptions used for Dense Urban FR1 and Urban Macro FR1 scenarios which follow the agreements made in meeting RAN1 #104-e and RAN1 #104-bis-e. The corresponding results are presented in Section 3 and Section 4. Table 1 summarizes the System Level Simulation SLS assumptions while Table 2 summarizes the traffic model parameters assumptions that were made to obtain the capacity results in Section 3 and Section 4. We further discuss the precoding, scheduling, traffic arrival offset assumptions made to compute the capacity of the system.

In RAN1 #104-bis-e meeting, RAN1 agreed on the exact parameters of Truncated Gaussian for the packet size distribution as well as the parameters of the jitter distribution which are presented in Table 2. The air interface delay budget used for the evaluations is 10 msec and the delay is measured from the point when a packet arrives at gNB to the point when it is successfully delivered to UE.

Table 1: Simulation Assumptions

| Scenario | Dense Urban FR1 | Urban Macro FR1 |
| --- | --- | --- |
| Channel Model | UMa TR 38.901 | UMa TR 38.901 |
| Layout | 21 cells with wraparound | 21 cells with wraparound |
| ISD | 200 m | 500 m |
| Carrier | 4 GHz | 4 GHz |
| SCS | 30 kHz | 30 kHz |
| BS height | 25m | 25m |
| BS noise figure | 5 dB | 5 dB |
| UE noise figure | 9 dB | 9 dB |
| UE receiver | MMSE-IRC | MMSE-IRC |
| Channel estimation DMRS | Realistic | Realistic |
| UE speed | 3km/hr | 3km/hr |
| MCS | Up to 256 QAM | Up to 256 QAM |
| BS antenna pattern | 3 sector radiation pattern, 8 dBi | 3 sector radiation pattern, 8 dBi |
| UE antenna pattern | Omni-directional, 0 dBi, | Omni-directional, 0 dBi, |
| UE distribution | 80% indoor and 20% outdoor | 80% indoor and 20% outdoor |
| BS antenna | Option 1: 64 TxRUs (8,8,2,1,1;4,8) (0.5 lambda, 0.5 lambda) | Option 1: 64 TxRUs (8,8,2,1,1;4,8) (0.5 lambda, 0.5 lambda) |
| Downtilt | 12 degree | 6 degree |
| BW | 100 MHz | 100 MHz |
| UE antenna | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |
| BS Tx power | 44 dBm per 20 MHz | 49 dBm per 20 MHz |
| UE Tx power | 23 dBm | 23 dBm |
| HARQ | Number of retransmission: 4 | Number of retransmission: 4 |
| TDD configuration | Options: DDDUU, DDDSU | Options: DDDUU, DDDSU |
| UE height in meters | hUT=3(nfl – 1) + 1.5 nfl=1 for outdoor UEs nfl ~ uniform (1,Nfl) where Nfl ~uniform (4,8) | hUT=3(nfl – 1) + 1.5 nfl=1 for outdoor UEs nfl ~ uniform (1,Nfl) where Nfl ~uniform (4,8) |

Table 2: Downlink Traffic Model Simulation Assumptions for Dense Urban FR1 and Urban Macro FR1 Scenarios

| PDB for DL video stream | 10 msec |
| --- | --- |
| Average Data Rates | 30 Mbps and 45 Mbps |
| Frame rate | 60 fps |
| Jitter | - Truncated Gaussian - Mean 0 msec - STD 2 msec - Range [-4, 4] ms |
| Packet Size Distribution | - Truncated Gaussian - Mean 30 Mbps x 16.667 ms/frame = 500 kbits/frame - STD 10.5% x mean - Max: 1.5 x mean - Min: 0.5 x mean |

Further assumption on DL traffic model

In RAN1 #104-bis-e the following agreement was made for the case of single stream per UE in DL.

Agreement:

In case of single stream per UE in DL, a UE is declared a satisfied UE if more than X (%) of packets are successfully delivered within a given air interface PDB.

- The baseline X value is 99.

- Other values of X can be optionally evaluated, e.g., X < = 95, X=99.9.

- Additional combinations of (X, PDB) values can be optionally evaluated, e.g.,

  - (99, 7), (95, 13) for VR/AR

  - (99, 12), (95, 18) for CG

- FFS: Different values for I-frame and P-frame if evaluation of them is agreed.

In this contribution, the single stream per UE in DL is assumed and a UE is declared as a satisfied UE if more than 99% of packets are successfully delivered within 10 msec PDB.

In RAN1#103-e meeting the following agreement was made regarding the system capacity [3]

Agreement:

System capacity is defined as the maximum number of users per cell with at least X % of UEs being satisfied.

- X=90 (baseline) or 95 (optional)

- Other values of X can also be evaluated optionally

Note: The exact ‘satisfied’ requirements will be discussed separately

FFS: how to calculate the percentage of satisfied users across multiple drops of simulations

We focus on the exact value of X being 90%, that is the system capacity is defined as the maximum number of users per cell such that at least 90% of UEs being satisfied.

Assumption on the traffic arrival offset

An important consideration that may affect the XR system capacity is the consideration of whether the periodic traffic pattern of XR users is aligned or have an offset relative to each other. In this paper, to reduce the number of evaluation results, we only consider the latter case, which represents the best case with the anticipated better performance where the UEs have arrival offsets uniformly spread out within the packet arrival period.

Assumption on the gNB precoding

The baseline performance assumes the gNB performs zero forcing precoding to null the MU-MIMO interference. The corresponding results are presented in Section 3. A cooperative MIMO scheme based on DL interference probing (by uplink SRS utilizing TDD channel reciprocity) is then evaluated with the corresponding results in section 4. The description of the enhanced precoding is described in detail therein.

Assumption on the uneven/even load

For “uneven load”, it means the average number of UE’s across all cells. In this case, the actual number of UE’s from cell to cell can be different, depending on the cell association. For “even load”, every cell has exactly same number of UE’s associated with it. In this contribution, we focus on the “uneven load” only, to reduce the number of simulations.

# Capacity Results with Zero forcing Precoding

## Capacity Results with Zero forcing Precoding

Section 3.2 will present all different downlink simulation results based on various configurations. In this section, we present the downlink MU-MIMO simulation of data packet rate 45Mbps, with slot configuration of [DDDSU].  Please note in last contribution [5], we presented simulation of data packet rate 30Mbps, with slot configuration of [DDDUU].

For the special slot “S” in the slot configuration, the throughput ratio is 10/14 for downlink data and 2/14 for uplink data. See following figure.

Figure 1: S slot in [DDDSU]

With the simulation assumptions made in Section 2.1 and while assuming zero-forcing precoding, the percentages of satisfied UEs with satisfaction rate of 99% for Dense Urban and Urban Macro results are shown in figure below. For each scenario, uneven is investigated. With the threshold of 90%, the capacity of XR system is limited to ~7.6 users/cell for Dense Urban scenario, and ~4.9 users/cell for Urban Macro scenario.

Figure 2: Zero forcing Precoding Capacity Results

Table 3: Capacity of the XR system assuming ZF precoding (with offset), data packet rate of 45Mbps, with slot configuration of [DDDSU]

| Scheme | Capacity (users/cell) (Uneven UE Load) |
| --- | --- |
| Dense Urban FR1 | ~7.6 |
| Urban Macro FR1 | ~4.9 |

Observation 1: For the FR1 Dense Urban Scenario and with the assumption of zero forcing precoding, the capacity of the XR system is ~7.6 UEs/cell  for uneven UE load, with data packet rate of 45Mbps, with slot configuration of [DDDSU].

Observation 2: For the FR1 UMa Scenario and with the assumption of zero forcing precoding, the capacity of the XR system is ~4.9 UEs/cell  for uneven UE load, with data packet rate of 45Mbps, with slot configuration of [DDDSU].

Proposal 1:  Study enhancements to MU-MIMO with a large number of antennas in order to increase the system capacity of XR

## Capacity Results with Zero forcing Precoding for More Configurations

The following table shows the performance of various configurations according to the table format of last standards meeting 106bis-e, all with Zero forcing Precoding.

Table 4: Capacity of the XR system assuming ZF precoding (with offset)

| Scenario | App | PDB | Bit rate | Fps | MIMO | Capacity |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DDDUU | DDDSU |
|  |  |  |  |  |  | ZF | ZF |
| DU | AR/VR | 10ms | 45Mbps | 60 | SU | 4.0 | 6 |
|  |  |  |  |  | MU | 5.2 | 7.3 |
|  |  |  | 30 Mbps | 60 | SU | 7.6 | 9.7 |
|  |  |  |  |  | MU | 8.9 | 12.3 |
|  |  | 7ms | 30 Mbps | 60 | MU | 6.4 | 8.4 |
|  |  | 13ms | 30 Mbps | 60 | MU | 11.4 | 14.7 |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 10.3 | 12.4 |
|  |  |  |  |  | MU | 12.3 | 17.1 |

| Scenario | App | PDB | Bit rate | Fps | MIMO | Capacity |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DDDUU | DDDSU |
|  |  |  |  |  |  | ZF | ZF |
| UMa | AR/VR | 10ms | 45Mbps | 60 | SU | 3.3 | 4.4 |
|  |  |  |  |  | MU | 3.6 | 4.9 |
|  |  |  | 30 Mbps | 60 | SU | 5.4 | 7 |
|  |  |  |  |  | MU | 6.3 | 7.7 |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 7.2 | 9.7 |
|  |  |  |  |  | MU | 8.4 | 11.1 |

# Capacity Results with Cooperative MIMO

## Cooperative MIMO for TDD with Downlink Interference Probing via SRS

Massive MIMO is considered as the major technique to achieve very high SE performance. One of the main issues in Massive MIMO is how to obtain accurate knowledge of the channels to enable spatial multiplexing of transmissions to multiple users without strong inter-user interference. For FDD systems, a relatively smaller number of antennas in the system limits the potential of performance, and in addition, without channel reciprocity, CSI acquisition needs to rely on CSI feedback which can lead to higher overhead, longer latency, lower accuracy, and hence limited SE performance. For TDD systems, Massive MIMO works relatively well thanks to accurate CSI acquisition through channel UL-DL reciprocity and generally provides higher SE than FDD.

However, channel UL-DL reciprocity is so far only utilized for single-cell (non-cooperative) MU-MIMO for TDD system and the gain is still far from what is promised with the massive number of antennas.

On the other hand, cooperative MIMO (e.g., CoMP in LTE) has not been found successful yet in practice. Here are some insights on the causes:

- Reciprocity in TDD is utilized (only) for channel information estimation for desired signals.

- Intra-cell interference (from MU-pairing) is handled relatively well, but for most scenarios with Massive MIMO inter-cell interference becomes the dominant limiting factor.

- Handling inter-cell interference using channel information of neighbouring cells is not successful even for TDD system:

  - Each individual interfering link may not be strong enough to obtain reliable channel information, by CSI feedback or by SRS.

  - The number of interfering links to consider is generally large, resulting in overhead, complexity, and robustness issues.

  - Each cell (or cell group) uses a centralized approach, which leads to significant complexity, robustness, backhaul, and edge-effect issues.

To resolve these issues and obtain the promised gain from the massive number of antennas, a new approach is proposed here. Instead of trying to obtain channel state information of interfering links and then determine precodings jointly at a centralized entity, SRS is enhanced to directly reflect DL interference spatial information (utilizing UL-DL reciprocity). Each gNB measures the corresponding SRS resources to obtain such information and adjusts its precoding to achieve interference coordination/avoidance.

To utilize TDD UL-DL reciprocity for interference measurement, the key is to tie some sounding activities to scheduled DL transmissions, which may be referred to as DL Interference Probing or SRS Probing. Then by TDD reciprocity, a gNB seeing strong UL interference from a certain spatial direction on the SRS resources (for example, via estimating the UL spatial covariance of interference signal) implies that in DL transmission the gNB will cause strong interference to UE(s) in that direction. The gNB can then adjust the precoding for DL interference avoidance. The gNBs in the network can coordinate the probing SRS resources in a semi-static fashion a priori, but essentially no inter-gNB information exchange is required on the fly. To summarize, DL Interference Probing from UL is possible if the network controls the UEs to transmit SRS in a way that best reflects prospective DL interference.

The above approach is illustrated in figure below, in which the sounding from UEs is based on prescheduling and reflects UE receive beamforming capabilities. As a result, the gNB can acquire DL interference spatial information. The gNB adjusts the precoders and thus DL SINR and SE can be improved. Furthermore, depending on the network implementation, more users/layers may be paired for MU-MIMO (such as via less conservative scheduling), yielding even higher SE gains. This approach of interference probing and mitigation, namely SRS probing, can be used to enable a variety of communication schemes to be implemented, generally in the category of Cooperative Massive MIMO.

Figure 3: TDD DL SE performance enhancement via interference probing and mitigation. To convey spatial information about the interference (both intra-cell and inter-cell) to the network, flexible A-SRS triggering based on prescheduling may be used.

The above approach is effective to suppress both inter-cell and intra-cell interference. This approach also has the following advantages:

- Distributed across gNBs (or TRPs) in the network, with low computational complexity for each gNB

- No or little channel information exchange among gNBs

- Can adapt to inter-cell interference, including unknown interference from non-cooperative gNBs or outside of the network (e.g., from other service providers, small cells, etc.)

- No need to estimate element-wise channel, reducing the complexity and overhead.

One specific way to implement is called bi-directional training (BiT); see, e.g., [7][8][9] (and references therein) for details for algorithms for narrowband systems, wideband systems, and dynamic TDD systems. Alternatively, BiT without multiple iterations (i.e., only one shot of interference probing before the PDSCH transmission, as shown in figure above can be performed, in which beamformed sounding based on conventional DL channel measurement and interference measurement is used. The latter is more practical for typical wireless communications.

## Capacity Results with BiT precoding

In this section, the results herein are divided into Dense Urban and UMa while assuming the cooperative MIMO transmission scheme.

Same as Section 3.1, we present sample results of the downlink MU-MIMO simulation of data packet rate 45Mbps, with slot configuration of [DDDSU]. Capacity Results with BiT Precoding for various configurations will be presented in Section 4.4 and compare with results of Zero-Forcing precoding baseline.

Dense Urban Results

With the assumption of BiT precoding described in Section 4.1, the capacity of the XR system has increased to ~14.3 UEs with the assumption of having uneven users traffic arrive with random offset. The advantage of BiT comes from the fact that inter and intra cell interference is nulled which may be achieved as explained in Section 4.1.

Figure 4: Capacity Results with BiT precoding: Dense Urban FR1, Uneven loads

Urban Macro Results

The capacity of the UMa scenario assuming the BiT precoding increases to ~7.6 UEs with the assumption of having BiT precoding.

Figure 5: Capacity Results with BiT precoding: Urban Macro FR1, Uneven UE loads

Table 5: Capacity of the XR system assuming BiT precoding (with offset) ), data packet rate of 45Mbps, with slot configuration of [DDDSU]

| Scheme | Capacity (users/cell) (Uneven UE Load) |
| --- | --- |
| Dense Urban FR1 | ~14.3 |
| Urban Macro FR1 | ~7.6 |

Observation 3: For the Dense Urban Scenario with the BiT precoding assumption, the gain is 88% for the XR system capacity with the assumption of having uneven, with data packet rate of 45Mbps, with slot configuration of [DDDSU].

Observation 4: For the UMa Scenario with the BiT precoding assumption, the gain is 60% for the XR system capacity with the assumption of having uneven load, with data packet rate of 45Mbps, with slot configuration of [DDDSU].

Observation 5: TDD ZF performance can be significantly improved by flexible A-SRS triggering with dynamically indicated partial frequency sounding.

Proposal 2: Capture in the TR results for TDD with and without Cooperative MIMO via DL interference probing based on SRS enhancements to improve XR system capacity.

Proposal 3: Capture in the TR the following description for TDD Cooperative MIMO via DL interference probing based on SRS enhancements:

- Bi-directional Training (BiT) is a spatial-domain interference avoidance scheme in a TDD Cooperative MIMO system. In BiT, DL interference probing is performed on uplink sounding resources semi-statically coordinated among gNBs. On the sounding resources, each gNB triggers SRS transmissions with parameters associated with corresponding DL transmissions. Then DL interference mitigation in spatial domain is performed by each gNB for its DL transmissions based on the interference probing outcome.

## Further Performance Analysis Dense Urban FR1 scenario

The performance analysis in this section include: Resource Utilization (RU), 5%ile UPT and SINR CDF. It is assumed that there are 12 UEs/cell. The assumptions include a traffic arrival offset for UEs. The simulations have demonstrated generally similarly high gains of BiT over baseline ZF.

Note that this section uses the same configuration as in [5], i.e. data packet rate of 30Mbps and downlink slot configuration of [DDDSU], as the performances of RU, UPT and SINR is similar regardless of the configurations.

### Resource Utilization

In figure below, we also see that BiT can substantially reduce the RU with the same assumption of traffic load as that of ZF. The gain of (15%) results from the increased spectrum efficiency due to much higher SINR (as shown in Figure 8) resulting from better interference suppression with BiT. With the reduction of RU the interference in the network is reduced, further improving the SINR and UPT in the system as shown in the section 4.3.2 and 4.3.3.

Figure 6: RU comparison of BiT (with one-shot interference probing) and ZF-based Massive MIMO

### 5%UPT

With one-shot BiT, significant UPT gains (52% for 5%ile UPT) are achieved over ZF as shown in figure below. The gains primarily come from increased DL SINR and the increased total number of paired layers due to effective inter-cell interference avoidance.

Figure 7: Performance comparison of BiT (with one-shot interference probing) and ZF-based Massive MIMO for 5%ile UPT

### SINR

In figure below, a comparison of BIT and ZF show an SINR improvement of about 6 dB at 50%ile due to the superb capability of interference mitigation by BiT. The gains on DL SINR is due to effective inter-cell interference avoidance which results in increase in UPT, decrease in RU and decrease in PER.

Figure 8: Performance comparison of BiT (with one-shot interference probing) and ZF-based Massive MIMO for DL SINR

## Capacity Results with BiT Precoding for More Configurations

The following table shows the performance of various configurations according to the table format of last standards meeting 106bis-e, for both Zero-Forcing and BiT precodings.

The performances of various configurations are as shown in table below. Please note that in the table:

- The first row is for 30Mbps, MU, [DDDUU]. This is same results that we presented in last contribution [5].

- All other rows are the new simulations results in this contribution.

- The sample performance presented in Section 3.1 and Section 4.2 is summarized in row 5 of the table.

From the performances of various configurations, we can see two phenomena regarding to the gains of BiT vs. ZF:

- MU experiences higher gains than SU for BiT vs. ZF precoding: Through the analysis, we see a lot more layers are scheduled in MU. The MCS assignment generally is saturated in SU simulations. Therefore, BiT can’t show greater improvements on ZF.

- DU scenario has higher gains than Uma scenario:  This is due to the shorter inter-cell distance in DU scenario. In this case, interference is more dominating than noise. BiT can do good job to supress interference, but not meant to suppress noise level.

Table 6: Capacity of the XR system assuming ZF and BiT precoding (with offset)

| Scenario | App | PDB | Bit rate | Fps | MIMO | Capacity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | [DDDUU] |  |  | [DDDSU] |  |  |
|  |  |  |  |  |  | ZF | BiT | BiT Gain | ZF | BiT | BiT Gain |
| DU | AR/VR | 10ms | 45Mbps | 60 | SU | 4.0 | 4.7 | 18% | 6 | 7 | 17% |
|  |  |  |  |  | MU | 5.2 | 10.6 | 103% | 7.3 | 14.3 | 88% |
|  |  |  | 30 Mbps | 60 | SU | 7.6 | 9.4 | 24% | 9.7 | 11.7 | 21% |
|  |  |  |  |  | MU | 8.9 | 16.4 | 89% | 12.3 | 20.3 | 65% |
|  |  | 7ms | 30 Mbps | 60 | MU | 6.4 | 12.7 | 98% | 8.4 | 16.9 | 101% |
|  |  | 13ms | 30 Mbps | 60 | MU | 11.4 | 18.6 | 63% | 14.7 | 22.1 | 50% |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 10.3 | 11.4 | 11% | 12.4 | 14.9 | 20% |
|  |  |  |  |  | MU | 12.3 | 19.7 | 60% | 17.1 | 22.9 | 34% |

| Scenario | App | PDB | Bit rate | Fps | MIMO | Capacity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | [DDDUU] |  |  | [DDDSU] |  |  |
|  |  |  |  |  |  | ZF | BiT | BiT Gain | ZF | BiT | BiT Gain |
| UMa | AR/VR | 10ms | 45Mbps | 60 | SU | 3.3 | 3.7 | 12% | 4.4 | 5.4 | 23% |
|  |  |  |  |  | MU | 3.6 | 5.5 | 53% | 4.9 | 7.7 | 60% |
|  |  |  | 30 Mbps | 60 | SU | 5.4 | 6.5 | 20% | 7 | 8.9 | 27% |
|  |  |  |  |  | MU | 6.3 | 9.5 | 56% | 7.7 | 11.6 | 51% |
|  | CG | 15 ms | 30 Mbps | 60 | SU | 7.2 | 8.7 | 38% | 9.7 | 11.4 | 18% |
|  |  |  |  |  | MU | 8.4 | 12.4 | 48% | 11.1 | 14.2 | 28% |

Observation 6: With the BiT precoding assumption, MU-MIMO experiences higher gains than SU-MIMO relative to Zero-Forcing precoding. MU-MIMO experiences gains in the range of 50~90%. SU-MIMO experiences gains around 20%.

Observation 7: DU scenario experiences higher gains than Uma scenario with the BiT precoding relative to Zero-Forcing precoding, due to the shorter inter-cell distance, in which interference is more dominating than noise.

# Uplink Pose and Control Simulation Assumptions and Results

## Simulation Assumptions

The following table shows the CG/VR single stream (pose/control) traffic model. The Period is 4ms (i.e. 250 fps) with no jitter, with fixed packet size of 100 bytes. PDB is still 10ms.

Table 7: Uplink Traffic Model Simulation Assumptions for Dense Urban FR1 and Urban Macro FR1 Scenarios

| PDB for UL Pose/Control | 10 msec |
| --- | --- |
| Average Data Rates | 0.2 Mbps |
| Frame rate | 250 fps |
| Jitter | - None |
| Packet Size Distribution | - Fixed length 800 bits/frame |

## Simulation Results

To reduce the simulation load of big number of UEs (usually hundreds) in the cell, we use FDM method: we only simulate fractional of the whole bandwidth of 100MHz. Approximately, the number of UE satisfying the PDB requirement can be multiplied by the fraction of the full bandwidth we are simulating.

Specifically, for 100MHz full bandwidth, we only simulate 12.5MHz, i.e. 1/8 of the whole bandwidth, to find the number of UEs that can met the PDB requirements. Then finally, the number of UE is multiplied by 8 to approximate the full bandwidth performance.

The simulation performance is shown below. We can see that capacity for UMa and DU are about 142.4 (=17.8*8) UEs, and 178.8 (=22.1*8) UEs, respectively.

Figure 9: Capacity Results of Uplink Pose/Control Simulation with FDM of bandwidth of 12.5MHz

In these simulations, we are limiting uplink schedule to SU and UE rank to be 1. Better performances can be achieved in future with more sophisticated uplink scheduler.

Observation 8: The capacity of uplink control and pose packet for DU scenario is about 178 UEs and for UMa scenario is about 142 UEs in a simple uplink scheduling simulation.

# Uplink Scene Stream Simulation Assumptions and Results

## Simulation Assumptions

The following table shows the CG/VR single scene stream traffic model. The Period is 60 fps with jitter, with mean packet size of 10Mbps bytes. PDB is still 30ms.

Table 8: Uplink Traffic Model Simulation Assumptions for Dense Urban FR1 and Urban Macro FR1 Scenarios

| PDB for UL stream | 30 msec |
| --- | --- |
| Average Data Rates | 10 Mbps |
| Frame rate | 60 fps |
| Jitter | - Truncated Gaussian - Mean 0 msec - STD 2 msec - Range [-4, 4] ms |
| Packet Size Distribution | - Truncated Gaussian - Mean 10 Mbps x 16.667 ms/frame = 166.666 kbits/frame - STD 10.5% x mean - Max: 1.5 x mean - Min: 0.5 x mean |

## Simulation Results

We use FDM method to only simulate UL data stream performance for 50MHz bandwidth. This is due to the computation power limitation based on our system simulator, which simulates number of subbands instead per PRB. Too many subbands would cause the simulation load too big. With the number of UEs that can met the PDB requirements of 50MHz, multiplying by 2 would approximate the performance full bandwidth of 100MHz.

The simulation performance is shown below for DU. We can see that capacity for DU are about 7.4 UE. For Urban Macro, it is less than 1 UE.

Figure 10: Capacity Results of Uplink Scene Simulation with FDM of bandwidth of 50MHz for Dense Urban

The performances of UL pose and control stream and scene stream for DY and UMa are summarized in the table beow.

Table 9: Uplink Simulation Pose and Control and Scene Stream Dense Urban FR1 and Urban Macro FR1 Scenarios

| Scenario | App | PDB (ms) | Bit rate (Mbps) | FPS | MIMO | Capacity result |
| --- | --- | --- | --- | --- | --- | --- |
| DU | VR/CG (1 stream: Pose) | 10 | 0.2 | 250 | SU | 178.4 |
|  | AR (1 stream: Scene) | 30 | 10 | 60 | SU | 7.4 |
| UMa | VR/CG (1 stream: Pose) | 10 | 0.2 | 250 | SU | 142.4 |
|  | AR (1 stream: Scene) | 30 | 10 | 60 | SU | <1 |

Observation 9: The capacity of uplink scene stream for DU scenario is about 7.4 UEs and for UMa is less than 1 UE in a simple uplink scheduling simulation.

# Conclusions

In this contribution, we present our views on evaluation methodology. Cooperative MIMO scheme via precoding technique based on DL interference probing is evaluated and compared to baseline MU-MIMO using zero forcing precoding. It is shown that the former increases the XR system capacity by 50~90% for the DU and UMa scenarios for various configurations, when UEs have traffic arrival offset and uneven UE loads. Based on the discussions in the previous sections we propose the following:

Proposal 1:  Study enhancements to MU-MIMO with a large number of antennas in order to increase the system capacity of XR

Proposal 2: Capture in the TR results for TDD with and without Cooperative MIMO via DL interference probing based on SRS enhancements to improve XR system capacity.

Proposal 3: Capture in the TR the following description for TDD Cooperative MIMO via DL interference probing based on SRS enhancements:

- Bi-directional Training (BiT) is a spatial-domain interference avoidance scheme in a TDD Cooperative MIMO system. In BiT, DL interference probing is performed on uplink sounding resources semi-statically coordinated among gNBs. On the sounding resources, each gNB triggers SRS transmissions with parameters associated with corresponding DL transmissions. Then DL interference mitigation in spatial domain is performed by each gNB for its DL transmissions based on the interference probing outcome.

Observation 1: For the FR1 Dense Urban Scenario and with the assumption of zero forcing precoding, the capacity of the XR system is ~7.6 UEs/cell  for uneven UE load, with data packet rate of 45Mbps, with slot configuration of [DDDSU].

Observation 2: For the FR1 UMa Scenario and with the assumption of zero forcing precoding, the capacity of the XR system is ~4.9 UEs/cell  for uneven UE load, with data packet rate of 45Mbps, with slot configuration of [DDDSU].

Observation 3: For the Dense Urban Scenario with the BiT precoding assumption, the gain is 88% for the XR system capacity with the assumption of having uneven, with data packet rate of 45Mbps, with slot configuration of [DDDSU].

Observation 4: For the UMa Scenario with the BiT precoding assumption, the gain is 60% for the XR system capacity with the assumption of having uneven load, with data packet rate of 45Mbps, with slot configuration of [DDDSU].

Observation 5: TDD ZF performance can be significantly improved by flexible A-SRS triggering with dynamically indicated partial frequency sounding.

Observation 6: With the BiT precoding assumption, MU-MIMO experiences higher gains than SU-MIMO relative to Zero-Forcing precoding. MU-MIMO experiences gains in the range of 50~90%. SU-MIMO experiences gains around 20%.

Observation 7: DU scenario experiences higher gains than Uma scenario with the BiT precoding relative to Zero-Forcing precoding, due to the shorter inter-cell distance, in which interference is more dominating than noise.

Observation 8: The capacity of uplink control and pose packet for DU scenario is about 178 UEs and for UMa scenario is about 142 UEs in a simple uplink scheduling simulation.

Observation 9: The capacity of uplink scene stream for DU scenario is about 7.4 UEs and for UMa is less than 1 UE in a simple uplink scheduling simulation.

# References

RP-201145, “Revised SID on XR Evaluations for NR”, Qualcomm, RAN#88e.

3GPP RAN1, RAN1#104-e, Chairman Notes.

R1-2102771, “XR initial evaluations”, Futurewei, RAN1#104-bis-e

R1-2104209, “XR initial evaluations”, Futurewei, RAN1#105-e

R1-2107088, “XR Further Evaluations”, Futurewei, RAN1#106-e

R1-2108799, “XR Further Evaluations”, Futurewei, RAN1#106bis-e

H. Zhou, J. Liu, Q. Cheng, D. Maamari, W. Xiao, and A. C. K. Soong, "Bi-Directional Training with Rank Optimization and Fairness Control," in 2018 IEEE 88th Vehicular Technology Conference (VTC-Fall), 2018.

J. Liu, Q. Cheng, W. Xiao, D. Maamari, and A. C. K. Soong, "Bi-directional Training for Wideband Systems," in 2019 IEEE 89th Vehicular Technology Conference (VTC-Fall), 2019.

R1-2102765, “Enhancements on SRS flexibility, coverage and capacity”, FUTUREWEI, RAN1#104-bis-e.