GPP TSG RAN WG1 Meeting #107-e	R1-2110894

E-meeting, November 11 – November 20,2021

Agenda Item:	8.2.6

Source:	Guangdong Communications and Networks Institute

Title:	Discussion on relation between sensing beam and transmission beam

Document for:	Discussion and Decision

Introduction

In the meeting RAN1#106-e, it was agreed that 3GPP specifications considers defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define the sensing beam for LBT, where at least the sensing beam(s) “covers” the transmission beam(s). Two main alternatives were formulated for potential down selection considering two approaches to define “cover”; the first uses the physical properties of the beams involved and the second extends the beam correspondence framework and/or QCL/TCI framework as follows:

Agreement: 3GPP specification consider defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s), considering following alternatives. Target down-selection by RAN1 #106bis-e - Alt 1: Specify necessary requirement/test procedure to guarantee sensing beam “covers” the transmission beam - Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is ncluding in the [X, FFS] dB beamwidth of the sensing beam. - Alt-1B: the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain - Alt-1C: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP. The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions. - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams. - Sending LS to RAN4 and inform them the above and request them to make the final choice - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well - Alt 2. Extending the beam correspondence framework and QCL/TCI/SpatialRelationInfo framework to define “cover” and to indicate sensing beam(s) associated with a transmission beam(s) - On gNB side sensing beam selection for a DL transmission beam, - Option 1: The selection of eligible sensing beam for a transmission beam is left for gNB implementation - No testing or enforcement introduced in 3GPP spec for this option - Option 2: Beam correspondence at gNB side is assumed. Supporting one or more of the following behaviors - A1. For a gNB transmission beam corresponding to TCI state A for a certain UE, the gNB can use the same beam for sensing - A2. If TCI B is used as QCL source (Type D) for TCI A for a certain UE, then gNB transmission beam corresponding to TCI B can be used as the sensing beam for transmission with TCI A. - A3. If TCI C is NOT used as QCL source (Type D) for TCI A for any UE, then gNB cannot use the transmission beam corresponds to TCI C as the sensing beam for transmission with TCI A. - FFS: How and if to support sensing with a beam without corresponding RS sent? For example, how to use quasi-Omni beam for sensing if there is no SSB transmitted with quasi-omni beam - On UE side sensing beam selection for a UL transmission beam - Beam correspondence is assumed at UE - FFS: What if beam correspondence is not supported at UE. - Supporting one or more of the following behaviors - If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing - Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing - FFS: How and if to support a wider sensing beam (such as pseudo-omni beam, which is supported in WiFi) to be used for a narrower transmission beam under QCL/TCI framework - Option 0: Not supported - Option 1: UE implementation. - No testing or enforcement introduced in 3GPP spec for this option - Option 2: gNB indication. - FFS details. - FFS: How and if to support multiple sensing beams to be used for a transmission beam under QCL/TCI framework - Note: Supporting both alternatives or a combination of the two alternatives is not precluded

However, there is no consensus on this topic in the meeting RAN1#106b-e. In our view, if Alt 1 is going to be agreed in the future to define directional sensing beam covering directional transmission beam, more discussion is needed since the listed alternatives above seems have risk of producing excessive interference. Alt-1A only considers the spatial relation between two beams and does not take their respective gains into account. Alt-1B and Alt-1E only consider the direction of peak transmission direction and does not take their shapes into account. Alt-1C and Alt-1D consider both shape and gain but does not guarantee avoiding regulation violation. In the following discussion, we would like to complement discussion on the relation of two beams by involving shape and gain. We would like to emphasis the importance to guarantee that a directional transmission allowed by a directional sensing will not produce excessive interference in terms of regulation.

Discussion on relation between sensing beam and transmission beam

If there are different beams for transmission and sensing, key issue is the validity of sensing result for transmission. In principle, correlation between sensing beam and transmission beam is the fundamental criteria to describe relation in quantity between these two beams. In this contribution, we study the impacts of relation between sensing beam and transmission beam with respect to beam shape and gain distribution in a straightforward way. The ideal configuration of identical sensing beam and transmission beam is used as baseline when consider performance loss.

We firstly assume same/similar peak gain for both sensing beam and transmission beam, and classify the relation by means of shape and direction. In this way, we focus on the impacts of beam shape and direction by normalizing beam gain. Generally, there are following configurations:

- configuration 1: similar shape and aligned direction

- configuration 2: similar shape and unaligned direction

- configuration 3: different shape and aligned direction

- configuration 4: different shape and unaligned direction

For configuration 1, it is strongly confident that sensing result effectively reflects the channel status for transmission. For configuration 4, it is obvious that sensing result is irrelevant to the channel status of interest. Configuration 2 and 3 are studied as below.

- Similar shape and unaligned direction (configuration 2)

A typical scenario with configuration 2 is depicted in figure 1. It is the most sensitive for sensing at the direction of sensing beam maxima, and is the most powerful for radiating energy at the direction of transmission beam. Consider two cases in which a neighboring user is at position 1 and position 2 respectively, corresponding to the directions of two beams. As shown in figure 1, gain of sensing beam at the direction of position 1 is XdB less than gain of transmission beam. Similarly, transmission beam gain at the direction of position 2 is YdB less than gain of sensing beam. Under the condition of similar shape, X and Y are positively correlated, and a smaller X and Y indicates more overlaps between two beams. Assume in both cases the same energy is detected by directional sensing, for example, due to a longer distance of position 2 than position 1 with the same neighboring transmission power，and is lower than predetermined threshold. Then the transmitter will transmit with directional transmission beam without discriminating any of these two cases. However, the interference level at position 2 is (X+Y)dB lower than position 1, with the same transmit power and transmission beam. In other words, there is a power boost room of (X+Y)dB for directional transmission when neighboring user is at position 2, in order to product the same interference level to the case of neighboring user at position 1. However, transmitter is unaware of this power boost room since detected energies for these two cases are the same.

X dB Y dB Position 1 Position 2 Sensing Transmission

Figure 1. configuration 2- similar shape and unaligned direction

With conventional LBT process, it is safe with respect to interference in the direction range starting from the cross-point of two beams towards sensing beam maxima, and is dangerous in the direction range starting from the cross-point towards transmission beam maxima. In order to avoid violating regulations, it is necessary to identify the maximal gap in gain of the same direction between transmission beam and sensing beam within dangerous direction range (denoted as transmission-over-sensing gain X), and a power fallback with this maximal gap is applied to all directions since there is no direction information of neighbors. By identifying the maximal gap in gain of the same direction between transmission beam and sensing beam within safe direction range (denoted as sensing-over-transmission gain Y), and the maximum performance loss is the sum of maximal transmission-over-sensing gain(Xmax) and maximal sensing-over-transmission gain(Ymax). It also can be seen that the average performance loss is determined by the correlation of these two beams.

Observation 1: By identifying maximal transmission-over-sensing gain(Xmax) and maximal sensing-over-transmission gain(Ymax) as mentioned above for similar shape and unaligned direction configuration of sensing beam and transmission beam, there could be a performance loss of (Xmax + Ymax)dB at the most in terms of transmit power in order to avoid violating regulations when there is no direction information about the neighboring user.

- Different shape and aligned direction (configuration 3)

For configuration 3, there are two sub-configurations 3-1 and 3-2 as depicted in figure 2. In figure 2(a), sensing beam is completely covered by transmission beam, then sensing gain is less than transmission gain in almost all directions. The consequence is that sensed interfering zone shrinks inconsistently with wider transmission beam. So actual transmit power has to be reduced to be lower than the value when sensing gain is identical to transmission gain, otherwise regulations will be not fulfilled. To do this, transmitter should be aware of the gap of gains between two beams, and either reduce power if transmission is allowed, or modify the threshold accordingly. However, there is no way to know about this information due to not knowing about direction of interfered neighbor. A practical way for transmitter is to always reduce power with the maximal gap of gains between two beams in all directions. Thus, the maximum performance loss is the maximal transmission-over-sensing gain which occurs at the direction of their aligned pointing direction.

| Position 1 Position 2 Sensing Transmission Xmax | Position 1 Position 2 Sensing Transmission Ymax |
| --- | --- |
| - sub-configuration 3-1 sensing beam is within transmission beam | (b) sub-configuration 3-2 transmission beam is within sensing beam |
| Figure 2. configuration 3- different shape and aligned direction |  |

In figure 2(b), transmission beam is completely covered by sensing beam. Then sensed interfering zone is enlarged, and the transmit power determined by conventional LBT process is conservative due to the transmission gain is less than sensing gain in almost all directions. It is possible to allow transmitter to boost its power to compensate the gap of gain of sensing beam over transmission beam without violating regulations. However, there is no way to know about this gap information due to not knowing about direction of interfered neighbor. It can be seen that performance loss is maximized at the direction where the gap of gain between two beams is maximal.

From the above analysis, there is simple way for configuration 2 to indicate the difference between sensing beam and transmission beam in order to predict the performance. In comparison, it is difficult for configuration 3 to indicate the difference between sensing beam and transmission beam. However, sub-configuration 3-2 might facilitate implementation in some cases, for example, covering multiple transmission beams with single sensing beam in order to reduce complexity and cost.

A more general configuration is depicted in figure 3, without limiting the shapes and directions of two beams. By identifying the maximal transmission-over-sensing gain(Xmax dB) and maximal sensing-over-transmission gain(Ymax dB), the maximum performance loss is (Xmax + Ymax)dB, and the average performance loss is determined by the correlation of these two beams. It can be seen that for sub-configuration 3-1 Xmax is equal to 0, and for sub-configuration 3-2 Ymax is equal to 0.

Sensing Transmission Ymax Xmax

Figure 3. general configuration for directional sensing and transmission

Observation 2: By identifying maximal transmission-over-sensing gain(Xmax) or maximal sensing-over-transmission gain(Ymax) as mentioned above for different shape and aligned direction configuration of sensing beam and transmission beam, there could be a performance loss of Xmax or Ymax dB at the most in terms of transmit power in order to avoid violating regulations when there is no direction information about the neighboring user.

Observation 3: For general case, by identifying maximal transmission-over-sensing gain(Xmax)dB and maximal sensing-over-transmission gain(Ymax)dB, power control with Xmax can be used to avoid violating regulations, and then a maximum performance loss is expected to be (Xmax + Ymax)dB.

Proposal: for directional sensing, maximal transmission-over-sensing gain and maximal sensing-over-transmission gain can defined as parameters for power control and performance evaluation.