3GPP TSG RAN WG1 #106bis-e                                        R1-2110916

e-Meeting, November 11th – 19th, 2021

Source:	ZTE

Title:          Discussion on enhanced intra-UE multiplexing

Agenda item:	8.3.3

Document for: Discussion and Decision

- Introduction

In the RAN1#106bis-e meeting, the potential enhancements for intra-UE multiplexing between different priorities and simultaneously PUCCH/PUSCH transmission were discussed, and the following agreements were reached [1].

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, in case the total number of LP and HP HARQ-ACK bits is 2: - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). Agreement For determining the PUCCH resource to carry the multiplexed high-priority and low-priority HARQ-ACKs, - The number of RBs for multiplexing HP HARQ-ACK and LP HARQ-ACK on a PUCCH format 3 is determined as following: - If OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm ≤MRBPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH, the minimum number of RBs is determined as the number of MRB, minPUCCH, satisfying OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm≤MRB,minPUCCH∙Nsc, ctrlRB∙Nsymb-UCIPUCCH and OHP_UCI+OCRC, HP_UCIrHP_UCI∙Qm+OLP_UCI+OCRC, LP_UCIrLP_UCI∙Qm>MRB,minPUCCH-1∙Nsc, ctrlRB∙Nsymb-UCIPUCCH. - Note: rHP_UCI∙rLP_UCI∙Qm is multiplied at both sides to avoid mismatch between gNB and UE due to floating point operation. Editor to capture as suggested. - Otherwise, - Alt1: the number of RBs is MRBPUCCH. FFS: Whether/How LP HARQ-ACK is dropped. - Alt2: the number of RBs is determined by HP ACK payload size. LP HARQ-ACK is fully dropped. - Other alternatives are not precluded. - r_HP_UCI is maxCodeRate configured for HP bits and r_LP_UCI is maxCodeRate configured for LP bits in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK). - FFS whether more than one maxCodeRate can be configured for one priority. - If MRB, minPUCCH is not equal to according to [4, TS 38.211], MRB, minPUCCH is increased to the nearest allowed value of nrofPRBs for PUCCH-format3 provided by the second PUCCH-Config [12, TS 38.331]. - HP coded bits and LP coded bits are not transmitted using the same RE(s) - FFS for PUCCH format 2. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, - HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.1 and Clause 5.3.3. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK in principle. FFS details. - For LP HARQ-ACK, reuse R15 Part 1 CSI rate matching and RE mapping. Agreement The following working assumption is confirmed. For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable Agreement For both the subslot-based PUCCH and slot-based PUCCH, if simultaneous PUCCH/PUSCH transmission is not enabled, reuse Rel-16 procedure for Step 1 Agreement For collision between HP CG PUSCH and LP DG PUSCH, if MAC delivers two MAC PDUs to PHY, PHY layer can make the prioritization so that the UE is expected to transmit the CG PUSCH and cancel the DG PUSCH at latest from the first symbol that is overlapping with the CG PUSCH. - Note: For the DG PUSCH, it is up to UE implementation to handle OFDM symbols of the DG PUSCH before the start of HP CG PUSCH which are nonoverlapping with the HP CG PUSCH. - FFS: How to handle the collision when there is repetition for CG and/or DG PUSCH

In this contribution, we further discuss the potential remaining issues and provide our views.

- Multiplexing UCIs of different priorities in a PUCCH

  - Coding, rate matching, RE mapping and power control

In RAN1#106-e meeting, for the case that the total number of LP and HP HARQ-ACK bits >2 when the two UCIs with different priorities are transmitted in a same PUCCH resource, the coding schemes for 1 bit and 2 bits are discussed and two options are compared to be down selected:

- Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

  - Reuse R15 scrambling for PUSCH as baseline.

- Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding.

For 2 bits of LP or HP HARQ-ACK case, we have provided the simulation results in our previous contribution [2] to prove that padding method, i.e., option 2, has worse performance than simplex coding, i.e., option 1. Figure 1 is the update of simulation results of 2 bits HARQ-ACK, we decode the RM(32,3) as RM(32,2), 0.5dB performance gap is still observed between option 1(yellow) and option 2(green). Moreover, un-equal protection for the 2bits of option 2 is also observed in previous contribution. It is harmful for the HARQ information bits.

Figure 1 Coding schemes comparison for 2 bit HARQ-ACK

Furthermore, the coding scheme in Clause 5.3.3.2 doesn’t have the issue of scrambling. The placeholder doesn’t take effect if the Qm = 1 or 2 when number of information bits = 2, and it is certainly that only BPSK or QPSK modulation is for PUCCH.

For 1 bits of LP or HP HARQ-ACK case, we newly provide the simulation results below.

Figure 2 Coding schemes comparison for 1 bit HARQ-ACK

From Figure 2, the yellow curve for option 1, i.e.,1 bit based on Clause 5.3.3.1 almost coincide with the pink curve for option 2, i.e., padding to 3bits but with 1bit decoding. If some kind of optimization for option 1, e.g. constellation restriction, is adopted, the purple curve may provide some gain but the gain is tiny. At least we can conclude the decoding performance provided by option 1 is not worse than that of option 2.

Proposal 1: For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding and reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit, reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

For other payload size, i.e., larger than 2, the existing coding schemes defined in current specification should be reused to minimize the standard efforts. That is, if the payload size of a LP or HP HARQ-ACK is more than 2 but less than 12, RM code is performed, otherwise, Polar code is applied.

Proposal 2: When the two UCIs with different priorities will be multiplexed on a PUCCH format 2/3/4 by separate coding, for a certain priority UCI,

- If the payload size is more than 2 but less than 12, RM code is performed.

- If the payload is more than 11 bits, Polar coding is performed.

In RAN1#106-e meeting, an agreement is achieved as:

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - HP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - LP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for CSI-2. Above applies at least for PUCCH format 3 and 4.

To be exactly, this agreement should only be applied to the case of total number of LP and HP HARQ-ACK bits >2. If total number of LP and HP HARQ-ACK bits = 2, the code domain multiplexing is used and it has no issue about rate matching and RE mapping.

Proposal 3: Modify the agreement in RAN1#106-e to:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17 in case of the total number of LP and HP HARQ-ACK bits >2,

- HP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

- LP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for CSI-2.

Above applies at least for PUCCH format 3 and 4.

The remaining issue about coding, rate matching equation and RE mapping is related to PUCCH format 2. PUCCH Format 2 doesn’t support CSI part 2 and only one coding chain is provided in Rel-15/16, then anyway, if separate coding for HP HARQ-ACK and LP HARQ-ACK is supported, two coding chains are needed. Also there is no legacy rate matching and RE mapping rules to be reused for this scenario, then the scheme of rate matching and RE mapping for PF2 should be further investigated.

As the two coding chains are defined for HP HARQ-ACK and LP HARQ-ACK multiplexing in PUCCH in Rel-17, an easy way to extend the scheme of rate matching and RE mapping rules for PUCCH format 2 is mapping the coded bits of HP HARQ-ACK and LP HARQ-ACK continuously in the time-frequency resources, for example, mapping the coded bits of HP HARQ-ACK firstly and mapping the coded bits of LP HARQ-ACK secondly. The advantage of this localized RE mapping for each kind of HARQ-ACK bits and cascaded HP HARQ-ACK and LP HARQ-ACK is the lower complexity for implementation and specification effort. For the distributed RE mapping, there are much specification effort to be considered for example, the distance between REs if the distributed mapping is applied. For the narrow band PUCCH, the benefit from distribution RE mapping is marginal.

Figure 3 RE mapping of multiplexed HP HARQ-ACK and LP HARQ-ACK in PF2

Proposal 4: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2,

- Coded bits of HP HARQ-ACK and LP HARQ-ACK are continuously mapped in the time-frequency resources for PF2.

  - Multiplexing enable/disable mechanism

The remaining issue is whether to adopt the scheme on RRC+DCI-based enabling/disabling.

An important principle for intra-UE multiplexing UCI with different priorities is that the latency and reliability of high priority transmission should not be affected. Even if the multiplexing timelines are met, whether the UE actually multiplexes or prioritizes the transmission of PUCCH/PUSCH with high priority should be determined and configured by the network. The principle of determination should satisfy the requirement of latency and reliability and avoid the bad multiplexing scenarios, e.g., insufficient PUCCH resources. Therefore, it is necessary to indicate whether a high priority channel can be multiplexed with a low priority channel by dynamic scheduling DCI. If the multiplexing indicator is disabled, the UE will prioritize the transmission(s) of PUCCH/PUSCH with high priority as Rel-16. Otherwise, the UE will use the enhanced Rel-17 methods for multiplexing. In the discussion of framework of intra-UE multiplexing, for step 2 to handle multiplexing between the different priorities, the multiplexing timeline may not be satisfied, then it is reasonable to indicate dynamically to disable the multiplexing.

Proposal 5: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support DCI+RRC configuration for gNB to enable/disable the multiplexing when DCI is applied.

- For SPS HARQ-ACK, the enable/disable scheme falls back to RRC configuration.

Another issue is where the multiplexing indicator should be carried on. It is unnecessary to configure the multiplexing indicator in DCI field or RRC parameter for both high priority transmission and low priority transmission. Moreover, in order to decrease the possibility of multiplexing indicator missing, the multiplexing indicator should be carried on the DCI and RRC parameter for the high priority transmission, which is more reliable than the low priority one.

Proposal 6: The indicator of intra-UE multiplexing UCI with different priorities should be carried on the scheduling DCI or RRC parameter for the high priority transmission.

  - Ambiguity on LP HARQ-ACK existence or LP HARQ-ACK type-2 codebook size due to DCI miss-detection

In RAN1#106-e meeting, progress about PUCCH resource determination and mapping for multiplexing has been achieved. Some remaining issues about number of PRB determination and impact due to missing DCI etc., still need to be solved.

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - PUCCH resource set determination is based on: UCI payload size = the number of HP UCI bits + the number of LP UCI bits. - FFS PRB number determination for HP A/N and LP A/N, e.g. based on their coding rates. - FFS the impact to the number of LP UCI bits due to missed DCI and potential solutions - Note: the number of LP UCI bits in the above agreement does may not necessarily mean the actual number of LP UCI bits until the second FFS is resolved

    - Ambiguity on LP HARQ-ACK existence

For the problem of ambiguity on LP HARQ-ACK existence or LP HARQ-ACK type-2 codebook size due to DCI miss-detection, how the ambiguity would happen should be identified first.

For the ambiguity due to LP HARQ-ACK non-existence, in case the total number of LP and HP HARQ-ACK bits is more than 2, PUCCH format 2, 3, or 4 is used. If the DCI corresponding to LP HARQ-ACK is missing, the gNB can blindly decode the PUCCH based on the hypothesis of different payload size under the condition that whether the LP HARQ-ACK is multiplexed with HP HARQ-ACK or not. gNB will correctly obtain the HP UCI without ambiguity and retransmit the PDSCH with low priority as the corresponding DCI is missing. The only cost is the gNB complexity increase due to blind detection.

But in the case that the total number of UCI bits is no more than 2 bits and for LP HARQ-ACK and HP HARQ-ACK are multiplexing on PUCCH format 0, there is ambiguity for gNB on determining whether the LP HARQ-ACK is multiplexed with HP HARQ-ACK or not. If the DCI corresponding to LP HARQ-ACK is missing detection, the single HP HARQ-ACK and the multiplexed HARQ-ACKs would map to the same mcs, which will cause misunderstanding about the detection of LP transmission at gNB side. For example, in Figure 4, according to the current specification in section 9.2.3 in TS38.213, for PUCCH format 0, if the DCI corresponding to LP PUCCH is missing detection, UE will transmit only 1 bit HP UCI with mcs =0 for NACK or 6 for ACK. While at gNB side, the gNB will assume 2 bits UCI reception and consider the result as {HP=NACK, LP=NACK} or {HP=ACK, LP=ACK}. Obviously when mcs =6 (in case the HP transmission is correctly decoded), the gNB will assume the LP transmission is also correctly detected by UE but it fails at UE side in fact. Hypothesis detection based on payload size doesn’t work well for this case.

Figure 4 ambiguity for gNB on determining whether the LP HARQ-ACK is multiplexed with HP HARQ-ACK or not in the case that the total number of UCI bits is no more than 2 bits

Observation 1: The ambiguity problem due to LP HARQ-ACK non-existence, in case the total number of LP and HP HARQ-ACK bits is no more than 2 bits, is very severe and need to be solved.

To solve this severe ambiguity problem, two possible solutions are studied. 1) Different initial sequences can be configured for multiplexed UCIs and single HP UCI respectively, or 2) The extended PUCCH resource can be configured for the multiplexed UCIs. Different initial sequences design will need more specification effort and more sequences resources. Simply extending the PUCCH time-frequency resource for multiplexed HP/LP HARQ-ACK will cost much resource overhead. A better approach should be strive to reuse the current PUCCH resources and less specification effort.

The extended PUCCH resource for multiplexed UCIs can be implicitly indicated to UE. For example, in Figure 5, the PRI in the HP DCI indicates the PUCCH resource from the PUCCH set in the PUCCH-config with high priority for HP UCI only if no LP UCI needs to be multiplexed. And the PUCCH resource corresponding to PRI+x is defined as the extended PUCCH resource from the same PUCCH set in the PUCCH-config with high priority for multiplexed UCIs, the x can be predefined, e.g., x=1. Obviously, the extended PUCCH resource is still the PUCCH resource in the same PUCCH set in the PUCCH-Config with high priority and be well consistent with the previous agreement. The number of bits for PRI field doesn’t need to increase.

The example solution of PRI+x doesn’t need new defined dedicated PUCCH resource. That is the reason why x cannot be absorbed in PRI. On the other hand, this scheme avoids the additional overhead of PUCCH resource.

We prefer the extended PUCCH resource for multiplexed UCIs can be implicitly indicated to UE in second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) as the minor specification impact.

Figure 5 Example of PRI+x solution for the extended PUCCH resource for multiplexed UCIs

Proposal 7: For the case that the total number of bits is no more than 2 bits, the PRI+x in the HP DCI is used to implicitly determine an extended PUCCH resource from the same PUCCH set in the PUCCH-config with high priority for the multiplexed UCI.

- x is predefined, e.g., x=1.

    - Ambiguity on LP HARQ-ACK type-2 codebook size due to DCI miss-detection

The other option 3/4/5 in the FL’s proposals aim to solve the ambiguity on LP HARQ-ACK type-2 codebook size due to DCI miss-detection. But actually the issue of misalignment size of type 2 codebook is not so severe. The worst case is the missing of last LP DCI, due to the HP UCI and LP UCI are separately coded and separately mapping in PUCCH, in case that the total number of bits is more than 2 bits, gNB could blind decode the PUCCH based on the hypothesis of different payload size under the condition that whether the LP HARQ-ACK is multiplexed with HP HARQ-ACK or not. HP HAQR-ACK is always guaranteed to be received, and LP HARQ-ACK could be dropped as usual.

Observation 2: The ambiguity problem on LP HARQ-ACK type-2 codebook size due to DCI miss-detection, is not severe and could be solved by gNB implementation.

Even if the size ambiguity in this case should be solved, the option 3/4/5 sound not so good.

Option 3a/3b fix the LP HARQ-ACK payload size to a reference size or a reserved size. The problem is if the reference or reserved size is less than the actual size, anyhow some PDSCHs need retransmissions.

Option 4 increases the overhead of DCI, it seems like to indicate the T_DAI twice. Except the original T_DAI for LP transmission is carried in LP DCI, the similar indication for LP transmission is also carried in HP DCI.

Same as option4, option 5 also increases the overhead of DCI and is less efficient to over-optimize a low probability of DCI missing event.

Observation 3: The option 3/4/5 aiming to solve issue of ambiguity on LP HARQ-ACK type-2 codebook size due to DCI miss-detection, have kinds of shortcomings, such as DCI overhead increase and less efficient to over-optimize a low probability event.

  - Multiplexing HARQ-ACK and SR with different priorities

    - PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0

Some candidate options are:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, down-select the following options: - Opt.1b: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.3: No enhancement over Rel-16. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Further discussion could be based on option 1b, option 2c and option 3.

To satisfy the latency and reliability of HP SR, when a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, the multiplexed UCI should consider to transmit on the PUCCH resource with higher priority, especially for the positive SR. The option 1b is preferred.

If a UE would transmit positive SR in a resource using PUCCH format 0 and at most two HARQ-ACK information bits in a resource using PUCCH format 0 in a slot, the UE transmits the PUCCH in the resource using PUCCH format 0 in PRB(s) for SR. The same way as Table 9.2.3-3 (i.e.=0, 6) and Table 9.2.3-4 (i.e.=0, 3, 6, 9) in TS 38.213[3] can be reused for the UE to determine a value of  and (based on HARQ-ACK) for computing a value of cyclic shift.

Otherwise, the UE transmits only the PUCCH with HARQ-ACK information bits in a resource using PUCCH format 0 when negative SR.

If option 1b is adopted, the HP SR and LP SR are multiplexed into different PUCCH resources, and it is easy to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK.

If option 2c is adopted, it is hard to differentiate HP SR and LP SR if the multiplexing way as Rel-15 is reused, and more specification effort is needed.

There are more issues about option 2c. According to option 2c, whatever SR is, the resources for HARQ will be used to multiplex SR. When SR is positive, the cyclic shift set is {1, 4, 7, 10}, when SR is negative, the cyclic shift set is {0, 3, 6, 9}. The distance between the cyclic shifts is only 1, it is too small, and gNB is difficult to distinguish the SR positive or negative. In addition, if the DCI for HARQ is missing, UE doesn’t get the resources for HARQ, and SR will not be transmitted as demand on resources for HARQ.

For these reasons, Option 1b is more feasible than option 2c.

    - PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1

Some candidate options are:

When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, down-select the following options: - Opt.1b: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource (i.e. No enhancement over Rel-16). - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

The basic principle is the same as above section. Option 1b is our preference. To be more specific, the multiplexing way reuses Rel-15. If a UE would transmit positive SR in a resource using PUCCH format 0 and at most two HARQ-ACK information bits are multiplexed, the UE transmits the PUCCH in the resource using PUCCH format 0 in PRB(s) for SR. The same way as Table 9.2.3-3 (i.e.=0, 6) and Table 9.2.3-4 (i.e.=0, 3, 6, 9) in TS 38.213[3] can be reused for the UE to determine a value of  and (based on HARQ-ACK) for computing a value of cyclic shift.

Option 4 is not a good alternative as no enhancement over Rel-16.

    - PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0

Some candidate options are:

When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, down-select the following options: - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: No enhancement over Rel-16. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Option 3 is slightly preferred than option 2c. Option 4 is not acceptable as no enhancement.

If a UE would transmit positive SR in a resource using PUCCH format 1 and HARQ-ACK information bits in a resource using PUCCH format 0 in a slot, the UE transmits the PUCCH in the resource using PUCCH format 1 in PRB(s) for SR. The value of cyclic shift of sequence, i.e.,, of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For example, if HARQ-ACK value is 0, the UE transmits the PUCCH by setting b(0) = 0 and = 0. If HARQ-ACK value is 1, the UE transmits the PUCCH by setting b(0) = 0 and = 6.

Else, the UE transmits only the PUCCH with HARQ-ACK information bits in a resource using PUCCH format 0 and drops the PUCCH with negative SR.

For the issue of how to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK, if option 3 is adopted, the HP SR and LP SR are multiplexed into different PUCCH resources, and it is easy to differentiate HP SR and LP SR.

    - PUCCH carrying HP SR with PF0/1 overlaps with a PUCCH carrying LP HARQ-ACK with PF2/3/4

If a UE would transmit positive SR in a resource using PUCCH format 0/1 and HARQ-ACK information bits in a resource using PUCCH format 2/3/4, the UE reuses Rel-15 rules for multiplexing.

Else, the UE transmits only the PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.

The only difference with the principle of Rel-15 multiplexing is the dropping of SR if SR is negative.

    - PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF1

If a UE would transmit SR in a resource using PUCCH format 1 and HARQ-ACK information bits in a resource using PUCCH format 1 in a slot, the UE reuses Rel-15 rules for multiplexing.

Proposal 8: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. The principle is applied at least for three cases:

- PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0

- PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1

- PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0

Proposal 9: Adopt the following rules to multiplex high priority SR and low priority HARQ-ACK.

| HARQ-ACK SR | PUCCH format 0 | PUCCH format 1 | PUCCH format 2/3/4 |
| --- | --- | --- | --- |
| PUCCH format 0 | For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 0 in PRB(s) for SR. The same way in Rel-15 can be reused for the UE to determine the value of and for computing the value of cyclic shift . For negative SR, the UE transmits only a PUCCH with HARQ-ACK information. |  | For positive SR, the UE Reuse Rel-15 rules. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. |
| PUCCH format 1 | For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 in PRB(s) for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. | Reuse Rel-15 rules. |  |

  - Multiplexing in case a PUCCH overlaps with more than one PUCCH

In RAN1 #104-e meeting [4], the case that a PUCCH overlaps with more than one PUCCHs containing slot based resources or sub-slot based resources is discussed, there are some problems mentioned to be further studied, for example:

- Determination of the PUCCH time unit for handling the PUCCH collision case when different PUCCH time units of different priorities are configured, when a LP HARQ-ACK PUCCH overlaps with more than one HP sub-slot

- How to handle the cases when a LP HARQ-ACK PUCCH overlaps with multiple HP SR PUCCHs.

- When a LP HARQ-ACK overlaps with HP HARQ-ACK PUCCH and HP SR PUCCH simultaneously, how to handle the case if the LP HARQ-ACK can be multiplexed with HP HARQ-ACK but the LP HARQ-ACK cannot be multiplexed with HP SR.

In RAN1 #105-e meeting [5], the issue of multiplexing in case a PUCCH overlaps with more than on PUCCH is discussed under general framework for intra-UE multiplexing topic, and the last round proposals from feature leader are:

Proposal for 3rd round discussion: For handling the scenarios where a PUCCH of a given priority crosses the sub-slot boundary of the PUCCH config of another priority and overlaps with a PUCCH of another priority, adopt the following procedure: - The time unit of high priority PUCCH is used as the time unit for multiplexing of low-priority PUCCH and high-priority PUCCH. (assuming the time unit of HP PUCCH is equal to or smaller than that of LP PUCCH.) - FFS: How to determine an associated HP PUCCH time unit for the LP HARQ-ACK PUCCH if the LP HARQ-ACK PUCCH overlaps with multiple HP PUCCH time units. Options under consideration include: - Option 1: The low priority PUCCH performs multiplexing or dropping procedure in the first overlapping time unit that contains high priority PUCCH.

For the issue of the PUCCH time unit for handling the PUCCH collision case when different PUCCH time units of different priorities are configured, when a LP HARQ-ACK PUCCH overlaps with more than one HP sub-slot, the main bullet of proposal is fine and can be the starting point for further study, but the option 1 for the FFS bullet should have an additional restriction on timeline requirement. The additional restriction could be added as below with RED:

For handling the scenarios where a PUCCH of a given priority crosses the sub-slot boundary of the PUCCH config of another priority and overlaps with a PUCCH of another priority, adopt the following procedure: - The time unit of high priority PUCCH is used as the time unit for multiplexing of low-priority PUCCH and high-priority PUCCH. (assuming the time unit of HP PUCCH is equal to or smaller than that of LP PUCCH.) - FFS: How to determine an associated HP PUCCH time unit for the LP HARQ-ACK PUCCH if the LP HARQ-ACK PUCCH overlaps with multiple HP PUCCH time units. Options under consideration include: - Option 1: The low priority PUCCH performs multiplexing or dropping procedure in the first overlapping time unit that contains high priority PUCCH if overlapped low priority PUCCH and high priority PUCCH meet the multiplexing timeline.

The reason for adding this restriction is, the first overlapping time unit may not satisfy the multiplexing timeline requirement. For example, in Figure 6, if the UE capabilities are not same for high and low priority transmissions, and N2 for high and low priority transmission are different, it is possible the N2 of low priority transmission is larger than the N2 of high priority transmission. Although LP transmission overlaps with the first time unit containing the HP transmission, but the multiplexing timeline of LP transmission is later than the first HP time unit and the timeline is only satisfied in the second HP time unit.

Figure 6 illusion of timeline for handling the PUCCH collision case when different PUCCH time units of different priorities are configured

Proposal 10: To determine an associated HP PUCCH time unit for the LP HARQ-ACK PUCCH if the LP HARQ-ACK PUCCH overlaps with multiple HP PUCCH time units, the low priority PUCCH performs multiplexing or dropping procedure in the first overlapping time unit that contains high priority PUCCH if overlapped low priority PUCCH and high priority PUCCH meet the multiplexing timeline.

- Multiplexing UCIs of different priorities in a PUSCH

  - Coding, rate matching and RE mapping

Some general common senses about coding, rate matching and RE mapping are achieved in previous meetings:

- Supporting separate coding;

- Reusing the legacy coding scheme, rate matching and RE mapping as much as possible;

- The numbers of coding chain should not exceed the legacy UE capability.

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, the main issues to be solved are:

- The coding scheme, rate matching and RE mapping of HP HARQ-ACK, LP HARQ-ACK and HP/LP CSI part 1/2;

- Which one among HP HARQ-ACK, LP HARQ-ACK and HP/LP CSI part 1/2 should be dropped if the maximum coding chain including for UL-SCH is four?

- What is the difference of solutions for above questions if PUSCH is not conveying UL-SCH?

The HP HARQ-ACK should be prioritized, and in any case, HP HARQ-ACK will be multiplexed into PUSCH. The coding scheme, rate matching and RE mapping of HP HARQ-ACK can reuse the mechanism of HARQ-ACK multiplexed in PUSCH in Rel-15.

Proposal 11: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, the coding scheme, rate matching and RE mapping of HP HARQ-ACK reuse the mechanism of HARQ-ACK multiplexed in PUSCH in Rel-15.

An agreement in RAN1#106b-e meeting is:

Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH without CSI, - HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.1 and Clause 5.3.3. - Reuse R15 HARQ-ACK rate matching/puncturing and RE mapping for HP HARQ-ACK in principle. FFS details. For LP HARQ-ACK, reuse R15 Part 1 CSI rate matching and RE mapping.

One possible remaining issue for this agreement is the ambiguity on the size of LP HARQ-ACK due to LP DCI missing. If the LP DCI is missing, especially the last LP DCI, the number of bits of LP HARQ-ACK doesn’t align with the expectation of gNB, and the ambiguity will negatively affect the decoding of LP HARQ-ACK and subsequent PUSCH or CSI part. The protection on the codebook size determination of LP HARQ-ACK should be considered and specified. One simple way is reusing the mechanism of legacy T-DAI, i.e., adding the new T-DAI field for LP HARQ-ACK indicated in HP DCI.

Proposal 12: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH, a new T-DAI field for LP HARQ-ACK is added in HP DCI.

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and two-part LP CSI would be transmitted on LP PUSCH conveying UL-SCH, The CSI part 2 should be dropped firstly. LP HARQ-ACK is coded separately from HP HARQ-ACK and CSI part 1. Mechanism of coding, rate matching and RE mapping of LP HARQ-ACK and LP CSI part 1 will respectively follow the rules of Rel-15 CSI-part 1 and Rel-15 CSI-part 2. If the leftover resources for LP HARQ-ACK and LP CSI part 1 is not sufficient, LP HARQ-ACK has higher priority than LP CSI part 1, and LP CSI part 1 may be partially dropped or compressed, the schemes of UCI partially dropping and compressing are discussed in section 3.3.

Proposal 13: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH,

- The LP CSI part 2 is dropped firstly.

- LP HARQ-ACK is coded separately from HP HARQ-ACK and CSI part 1.

- The coding scheme, rate matching and RE mapping of LP HARQ-ACK and LP CSI part 1 will respectively follow the rules of Rel-15 CSI-part 1 and Rel-15 CSI-part 2.

- If the leftover resources for LP HARQ-ACK and LP CSI part 1 is not sufficient, LP HARQ-ACK has higher priority than LP CSI part 1, and LP CSI part 1 may be partially dropped or compressed.

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK and two-part HP A-CSI would be transmitted on HP PUSCH conveying UL-SCH, there are two alternatives for the first dropping:

- Alt1: Dropping LP HARQ-ACK, it is natural as the lower priority of LP HARQ-ACK;

- Alt2: Dropping HP A-CSI part 2, as A-CSI part 2 is a supplementary channel state report, the cancellation of A-CSI part 2 would not severely affect the performance.

Alt2 (i.e., dropping HP A-CSI part 2) is slightly preferred.

Proposal 14: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and HP CSI consisting of two parts would be transmitted on HP PUSCH conveying UL-SCH,

- Dropping HP A-CSI part 2.

- The coding scheme, rate matching and RE mapping of LP HARQ-ACK and HP CSI part 1 will respectively follow the rules of Rel-15 CSI-part 1 and Rel-15 CSI-part 2.

- If the leftover resources for LP HARQ-ACK and HP CSI part 1 is not sufficient, LP HARQ-ACK has lower priority than HP CSI part 1, and LP HARQ-ACK may be partially dropped or compressed.

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK and LP CSI would be transmitted on HP PUSCH conveying UL-SCH, LP CSI could be dropped and multiplexing with HP PUSCH is not allowed.

Proposal 15: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on HP PUSCH conveying UL-SCH, LP CSI is dropped and multiplexing with HP PUSCH is not allowed.

If HP HARQ-ACK, LP HARQ-ACK and HP CSI would be transmitted on LP PUSCH conveying UL-SCH, the HP CSI could be multiplexed with LP PUSCH. And the multiplexing principle follows the way which HP HARQ-ACK, LP HARQ-ACK, and HP CSI consisting of two parts are transmitted on HP PUSCH conveying UL-SCH.

Proposal 16: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and HP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH, HP CSI is allowed to multiplex with LP PUSCH. The multiplexing principle follows the way which HP HARQ-ACK, LP HARQ-ACK, and HP CSI consisting of two parts are transmitted on HP PUSCH conveying UL-SCH.

As the coding chain for PUSCH is LDPC coding, the coding chain for UL-SCH could not be used for UCI, it means the UE behaviour follows the above proposals just like the UL-SCH exists.

Proposal 17: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and HP/LP CSI consisting of two parts would be transmitted on HP/LP PUSCH not conveying UL-SCH, UE follows the same behaviour as that in case of PUSCH conveying UL-SCH.

  - Enhancements for multiplexing parameters

    - Multiplexing enable/disable mechanism

According to the agreement of RAN1#104-e meeting, for multiplexing LP HARQ-ACK in a HP PUSCH, 0< beta_offset <1 should be supported, and beta_offset =0 or a value disabling the multiplexing is FFS.

On FFS issue remaining in RAN1#104b-e meeting, as some companies said, the enable/disable mechanism by setting beta_offset = 0 is not feasible.

Dynamic explicit indication on enabling and disabling is helpful for UE to identify the premise of multiplexing, and UE is easy to distinguish the new specific indication field in DCI, for example, the new fields of beta_offset value, T-DAI for the size determination for LP HARQ-ACK, etc. In the discussion of framework of intra-UE multiplexing, for step 2 to handle multiplexing between the different priorities, the multiplexing timeline may not be satisfied, then it is reasonable to indicate dynamically to disable the multiplexing.

From the aspect of signaling to enable or disable multiplexing, the same mechanism of intra-UE multiplexing UCI on PUCCH should be applied to intra-UE multiplexing UCI with data. Moreover, the indication on multiplexing enable/disable should exist in the scheduling DCI or RRC parameter for the high priority transmission, but not implicitly be set by beta_offset=0 or other values.

Proposal 18: The beta_offset should not be used to disable the intra-UE multiplexing UCI with data.

Proposal 19: The indicator of intra-UE multiplexing UCI with data exists in the scheduling DCI or RRC parameter for the high priority transmission.

    - Remaining issues of Beta-offset

In RAN1#106-e meeting, agreement of set of beta offset values is:

Agreement In NR Rel-17, [at least] 2 new set of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases: - Multiplexing LP HARQ-ACK on HP PUSCH - Multiplexing HP HARQ-ACK on LP PUSCH

The agreement implies that two options for the whole sets are:

- Option 1: Up to 3 sets of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases:

  - Multiplexing HARQ-ACK on the PUSCH with same priority

  - Multiplexing LP HARQ-ACK on HP PUSCH

  - Multiplexing HP HARQ-ACK on LP PUSCH

- Option 2: Up to 4 sets of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases:

  - Multiplexing LP HARQ-ACK/UCI on LP PUSCH

  - Multiplexing HP HARQ-ACK/UCI on HP PUSCH

  - Multiplexing LP HARQ-ACK on HP PUSCH

  - Multiplexing HP HARQ-ACK on LP PUSCH

If no further agreement is achieved on the number of sets of beta offset values, option 1 will be the default configuration for sets of beta offset values.

Proposal 20: Up to 3 sets of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases:

- Multiplexing HARQ-ACK on the PUSCH with same priority

- Multiplexing LP HARQ-ACK on HP PUSCH

- Multiplexing HP HARQ-ACK on LP PUSCH

  - Resource mapping

When the LP UCI is multiplexed in HP PUCCH or HP PUSCH, the LP UCI may not be accommodated totally since the HP UCI or HP PUSCH should not be affected as possible and the LP UCI can only be mapped to the leftover PUCCH or PUSCH resource. In this case, the payload size for the LP UCI should be reduced until it can be carried by the remaining resource. There are two options for the payload size reduction, i.e., LP UCI is dropped partially and LP UCI compression

- Partially LP UCI dropping

This is the most straightforward way. The UE just drop some or all UCI depending on the available resource. For the dropped ACK/NACK, the network has to assume NACK and retransmit the corresponding PDSCH. This may reduce the resource efficiency since the UE may have already decode the PDSCH correctly. Anyway, it is better than the operation in Rel-16, where the LP UCI is totally dropped.

- LP UCI compression

Another solution is LP UCI compression. For example, HARQ-ACK bundling is widely used to reduce the HARQ-ACK codebook overhead in LTE and NR. After bundling, if the NACK is transmitted, the network still does not know the exact HARQ-ACK for each PDSCH. This situation is the same as the partial LP UCI dropping. However, if ACK is obtained by bundling, it means all the corresponding PDSCH is ACK and the network can get the exact feedback for each PDSCH. Since the possibility of ACK is much higher than NACK in practice, it can further avoid the unnecessary retransmission, which can further improve the resource efficiency compared to partial LP UCI dropping. Therefore, it is slightly preferred

Proposal 21: LP UCI compression is slightly preferred in case there is no enough resource left for LP UCI.

  - Timeline and latency requirements

For the latency requirement, the HP HARQ-ACK information should be mapped to the LP PUSCH resources no later than the last symbol of PUCCH resource for HARQ-ACK which is shown in Figure 7.

Figure 7. High priority HARQ-ACK multiplex with low priority PUSCH

Proposal 22: For the overlapping between high priority HARQ-ACK and low priority PUSCH, if the gNB allows a UE to multiplex the HARQ-ACK on PUSCH, the UE maps this HARQ-ACK to PUSCH resource elements no later than the last symbol of PUCCH resource for HARQ-ACK.

- Simultaneous x-CC PUCCH/PUSCH transmissions for inter-band CA

The agreements and conclusion about simultaneous x-CC PUCCH/PUSCH transmissions in previous meetings of RAN1#102[6], #104 and #106 are:

Agreements: Support simultaneous PUCCH/PUSCH transmissions on different cells at least for inter-band CA. - FFS how to trigger this function. - FFS for intra-band CA. Agreements: Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication Conclusion Simultaneous PUCCH/PUSCH transmission on the same cell is not supported in Rel-17.

It is clear that the process of simultaneous transmissions conflict with the multiplexing/prioritizing among the channels. The applied scope of multiplexing/prioritizing and simultaneous-transmissions should be clearly defined to prevent the process collisions. And the processing order between multiplexing/prioritizing and simultaneous-transmissions should also be specified to avoid ambiguity between gNB and UE.

Based on the previous agreements and conclusion, we assume the scope of simultaneous transmissions only applies to PUCCH/PUSCH transmission of different PHY priorities over different cells under inter-band CA case, if simultaneous transmissions are enabled. We assume the simultaneous PUCCH/PUSCH transmissions under intra-band CA is not allowed as no agreements on this are achieved.

It is natural to multiplex or prioritize firstly and handle the simultaneous PUCCH/PUSCH transmissions secondly as the simultaneous PUCCH/PUSCH transmissions are only supported in different cells under inter-band CA. So the proposed processing order is:

- The PUCCH/PUSCH on a same cell or on different cells under intra-band CA are multiplexed or prioritized, following the same procedure with simultaneous PUCCH/PUSCH disabled.

- The PUCCH/PUSCH on different cells within the same PUCCH group under inter-band CA are transmitted simultaneously at least when PUCCH/PUSCH have different priorities

  - FFS: whether to apply simultaneous PUCCH/PUSCH transmissions when PUCCH/PUSCH have the same priority.

Proposal 23: If simultaneous PUCCH/PUSCH over different cells is configured, the processing order between multiplexing/prioritizing and simultaneous-transmissions is proposed to be:

- The PUCCH/PUSCH on a same cell or on different cells under intra-band CA are multiplexed or prioritized, following the same procedure with simultaneous PUCCH/PUSCH disabled.

- The PUCCH/PUSCH on different cells within the same PUCCH group under inter-band CA are transmitted simultaneously at least when PUCCH/PUSCH have different priorities

  - FFS: whether to apply simultaneous PUCCH/PUSCH transmissions when PUCCH/PUSCH have the same priority.

- Discussion on the overlapping between dynamic grant PUSCH and configured grant PUSCH

In the RAN1#106bis meeting, the collision between HP CG and LP DG has been solved.

- LP CG vs HP DG

Similarly, if the UE processes the CG PUSCH first, more time is needed for the UE to cancel the CG PUSCH and prepare the DG PUSCH. It means the time interval between the DG PUSCH and the corresponding scheduling PDCCH should be large enough. The timeline defined in Rel-16 can be reused. Regarding the UE capability, it can be discussed after the solution is stable.

Proposal 24: For the overlapping between LP CG and HP DG, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. Further, a UE expects that the first [overlapping] symbol of the high priority DG PUSCH is not earlier than Tproc,2+d1 after the last symbol of the PDCCH with the DCI format scheduling the high priority channel.

- Conclusion

According to the analysis given above, we have the following observations and proposals:

Observation 1: The ambiguity problem due to LP HARQ-ACK non-existence, in case the total number of LP and HP HARQ-ACK bits is no more than 2 bits, is very severe and need to be solved.

Observation 2: The ambiguity problem on LP HARQ-ACK type-2 codebook size due to DCI miss-detection, is not severe and could be solved by gNB implementation.

Observation 3: The option 3/4/5 aiming to solve issue of ambiguity on LP HARQ-ACK type-2 codebook size due to DCI miss-detection, have kinds of shortcomings, such as DCI overhead increase and less efficient to over-optimize a low probability event.

Proposal 1: For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding and reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit, reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

Proposal 2: When the two UCIs with different priorities will be multiplexed on a PUCCH format 2/3/4 by separate coding, for a certain priority UCI,

- If the payload size is more than 2 but less than 12, RM code is performed.

- If the payload is more than 11 bits, Polar coding is performed.

Proposal 3: Modify the agreement in RAN1#106-e to:

For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17 in case of the total number of LP and HP HARQ-ACK bits >2,

- HP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1.

- LP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for CSI-2.

Above applies at least for PUCCH format 3 and 4.

Proposal 4: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2,

- Coded bits of HP HARQ-ACK and LP HARQ-ACK are continuously mapped in the time-frequency resources for PF2.

Proposal 5: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support DCI+RRC configuration for gNB to enable/disable the multiplexing when DCI is applied.

- For SPS HARQ-ACK, the enable/disable scheme falls back to RRC configuration.

Proposal 6: The indicator of intra-UE multiplexing UCI with different priorities should be carried on the scheduling DCI or RRC parameter for the high priority transmission.

Proposal 7: For the case that the total number of bits is no more than 2 bits, the PRI+x in the HP DCI is used to implicitly determine an extended PUCCH resource from the same PUCCH set in the PUCCH-config with high priority for the multiplexed UCI.

- x is predefined, e.g., x=1.

Proposal 8: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. The principle is applied at least for three cases:

- PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0

- PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1

- PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0

Proposal 9: Adopt the following rules to multiplex high priority SR and low priority HARQ-ACK.

| HARQ-ACK SR | PUCCH format 0 | PUCCH format 1 | PUCCH format 2/3/4 |
| --- | --- | --- | --- |
| PUCCH format 0 | For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 0 in PRB(s) for SR. The same way in Rel-15 can be reused for the UE to determine the value of and for computing the value of cyclic shift . For negative SR, the UE transmits only a PUCCH with HARQ-ACK information. |  | For positive SR, the UE Reuse Rel-15 rules. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. |
| PUCCH format 1 | For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 in PRB(s) for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. | Reuse Rel-15 rules. |  |

Proposal 10: To determine an associated HP PUCCH time unit for the LP HARQ-ACK PUCCH if the LP HARQ-ACK PUCCH overlaps with multiple HP PUCCH time units, the low priority PUCCH performs multiplexing or dropping procedure in the first overlapping time unit that contains high priority PUCCH if overlapped low priority PUCCH and high priority PUCCH meet the multiplexing timeline.

Proposal 11: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, the coding scheme, rate matching and RE mapping of HP HARQ-ACK reuse the mechanism of HARQ-ACK multiplexed in PUSCH in Rel-15.

Proposal 12: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK and LP HARQ-ACK would be transmitted on HP/LP PUSCH, a new T-DAI field for LP HARQ-ACK is added in HP DCI.

Proposal 13: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH,

- The LP CSI part 2 is dropped firstly.

- LP HARQ-ACK is coded separately from HP HARQ-ACK and CSI part 1.

- The coding scheme, rate matching and RE mapping of LP HARQ-ACK and LP CSI part 1 will respectively follow the rules of Rel-15 CSI-part 1 and Rel-15 CSI-part 2.

- If the leftover resources for LP HARQ-ACK and LP CSI part 1 is not sufficient, LP HARQ-ACK has higher priority than LP CSI part 1, and LP CSI part 1 may be partially dropped or compressed.

Proposal 14: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and HP CSI consisting of two parts would be transmitted on HP PUSCH conveying UL-SCH,

- Dropping HP A-CSI part 2.

- The coding scheme, rate matching and RE mapping of LP HARQ-ACK and HP CSI part 1 will respectively follow the rules of Rel-15 CSI-part 1 and Rel-15 CSI-part 2.

- If the leftover resources for LP HARQ-ACK and HP CSI part 1 is not sufficient, LP HARQ-ACK has lower priority than HP CSI part 1, and LP HARQ-ACK may be partially dropped or compressed.

Proposal 15: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and LP CSI consisting of two parts would be transmitted on HP PUSCH conveying UL-SCH, LP CSI is dropped and multiplexing with HP PUSCH is not allowed.

Proposal 16: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and HP CSI consisting of two parts would be transmitted on LP PUSCH conveying UL-SCH, HP CSI is allowed to multiplex with LP PUSCH. The multiplexing principle follows the way which HP HARQ-ACK, LP HARQ-ACK, and HP CSI consisting of two parts are transmitted on HP PUSCH conveying UL-SCH.

Proposal 17: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, if HP HARQ-ACK, LP HARQ-ACK, and HP/LP CSI consisting of two parts would be transmitted on HP/LP PUSCH not conveying UL-SCH, UE follows the same behaviour as that in case of PUSCH conveying UL-SCH.

Proposal 18: The beta_offset should not be used to disable the intra-UE multiplexing UCI with data.

Proposal 19: The indicator of intra-UE multiplexing UCI with data exists in the scheduling DCI or RRC parameter for the high priority transmission.

Proposal 20: Up to 3 sets of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases:

- Multiplexing HARQ-ACK on the PUSCH with same priority

- Multiplexing LP HARQ-ACK on HP PUSCH

- Multiplexing HP HARQ-ACK on LP PUSCH

Proposal 21: LP UCI compression is slightly preferred in case there is no enough resource left for LP UCI.

Proposal 22: For the overlapping between high priority HARQ-ACK and low priority PUSCH, if the gNB allows a UE to multiplex the HARQ-ACK on PUSCH, the UE maps this HARQ-ACK to PUSCH resource elements no later than the last symbol of PUCCH resource for HARQ-ACK.

Proposal 23: If simultaneous PUCCH/PUSCH over different cells is configured, the processing order between multiplexing/prioritizing and simultaneous-transmissions is proposed to be:

- The PUCCH/PUSCH on a same cell or on different cells under intra-band CA are multiplexed or prioritized, following the same procedure with simultaneous PUCCH/PUSCH disabled.

- The PUCCH/PUSCH on different cells within the same PUCCH group under inter-band CA are transmitted simultaneously at least when PUCCH/PUSCH have different priorities

  - FFS: whether to apply simultaneous PUCCH/PUSCH transmissions when PUCCH/PUSCH have the same priority.

Proposal 24: For the overlapping between LP CG and HP DG, PHY layer can make the prioritization so that the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. Further, a UE expects that the first [overlapping] symbol of the high priority DG PUSCH is not earlier than Tproc,2+d1 after the last symbol of the PDCCH with the DCI format scheduling the high priority channel.

- Reference

- 3GPP, Chairman's Notes RAN1#106bis-e final.

- R1-2104329, Discussion on enhanced intra-UE multiplexing, ZTE

- 3GPP TS 38.213-g60, Physical layer procedures for control, Release 16.

[4]  3GPP, Chairman's Notes RAN1#104-e final.

[5]  3GPP, Chairman's Notes RAN1#105-e final.

[6]  3GPP, Chairman's Notes RAN1#102-e final.