3GPP TSG RAN WG1 Meeting #107-e                                      R1-2110962

e-Meeting, November 11th – 19th, 2021

Source:         ZTE

Title:            UE features for feMIMO

Agenda Item:    8.16.1

Document for:   Discussion and Decision

Introduction

The RAN1 features developed in NR Rel-17 has become more and more stable. A preliminary RAN1 UE features list has been compiled in [1]. Nine feature groups are given in [1] to categorize all the feMIMO UE features, including multi-beam, multi-TRP PDCCH, multi-TRP PUSCH/PUCCH, inter-cell multi-TRP, multi-TRP beam management, SFN based HST, multi-TRP CSI, SRS and FeType II CSI. Further, after RAN1#106bis-e, an updated list of UE features is given in [2]. We discuss the remaining open issues in this updated list for feMIMO.

Multi-beam enhancements (23-1)

Four features (23-1-1~4) are developed in [1] for Rel-17 beam management, and then we discuss some further details for 23-1 series in this section, especially for the missing new sub-features.

23-1-1	Unified TCI for intra- and inter-cell beam management

Firstly, in the main feature, we need to explicit that the following functionalities are supported as part of the basic feature in 23-1-1:

- The common TCI state ID update and activation for CA

- Beam alignment for PL-RS

- MAC-CE-only based TCI state indication

Then, a new UE feature of beam misalignment has been agreed to be introduced as a sub-feature for 23-1-1, and by default the UE should mandatorily support the case of beam alignment. Also, TCI application timing for DCI based TCI state indication should be introduced as well.

After that, we have the following comments for other potential UE features captured by the moderator.

- Considering that only up to 128 TCI state is supported for unified TCI framework, we think that the further UE feature about maximum number of configured TCI state and TCI state pool cross CC is unnecessary at all (for component-4/5/6).

- Regarding activated TCI state (component-7/8), we can support to have UE capability signaling of maximum number of joint, separate DL and separate UL TCI states, respectively.

- Regarding QCL rules (component-10), we think that it should be supported by default considering that Rel-15 QCL rule is mandatory feature without capability signaling for NR UE.

- Regarding maximum number of PCI (component-12), we think that we need to discuss this issue in the agenda of inter-cell mTRP firstly. We may just reuse the corresponding conclusion/UE feature.

Proposal 1: For unified TCI for intra- and inter-cell beam management, the following modification in red is proposed.

| 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [ (PLRS and other, including handling of beam [alignment/misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states in an active BWP in a CC across all BWPs and all CCs in a band] [8a. The maximum number of MAC-CE activated separate DL and UL TCI states in an active BWP in a CC 8b. The maximum number of MAC-CE activated UL TCI states in an active BWP in a CC across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states] |
| --- | --- | --- |
| 23-1-1a | Support of beam misalignment | Support of beam misalignment for PLRS |
| 23-1-1b | TCI application time | X symbols after transmitting HARQ-ACK, the indicated TCI state is applied to a downlink or uplink signal, in the case of DCI based TCI state indication |

23-1-2	Inter-cell measurement and reporting (for inter-cell BM and mTRP)

Firstly, regarding PCI(s) different from serving cell PCI, we prefer to reuse the legacy UE feature of [2-24] SSB/CSI-RS for beam measurement as a starting point.

Then, based on already agreement, the maximum value of supported K is a UE capability, and then the corresponding candidate value should be provided, e.g., {1, 2, 4} as legacy beam reporting.

After that, we have the following comments for other potential UE features captured by the moderator.

- Regarding #. of PCI in FR1/FR2 (component-3/4),  as we mentioned before, we suggest that this feature should be discussed in inter-cell mTRP firstly, and then we can further review whether we need to introduce the same/similar UE capability signaling accordingly.

- Regarding #. of SSB associated with different PCI from serving cell (component-5/6), we do not identify the necessary of introducing the corresponding UE feature, considering that it is just the normal L1-measurement and there is no additional complexity for UE implementation.

- Regarding the indication whether SSB for L1 measurement can be different from L3 measurement (component-7), we think that it should be supported by default. No UE feature is needed.

Proposal 2: For inter-cell measurement and reporting (for inter-cell BM and mTRP), the following modification is proposed in red

| 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM [and mTRP]) | 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of Kmax SSBRI-RSRP[,SS-RSRP] [pairs/beams], with where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] |
| --- | --- | --- |

23-1-3	MPE mitigation

Based on the following agreement, the N P-MPR values and corresponding SSBRI(s)/CRI(s) can be reported. In our views, M should be fixed to 1, and if sharing the same MPE impacts, the UE can provide same P-MPR values for different SSBRI(s)/CRI(s), respectively, in a report instance. Therefore, a new sub-feature of maximum number of P-MPR and SSBRI/CRI pairs can be introduced, and reusing legacy candidate values for normal beam report can be considered as a starting point, i.e., {1, 2, 4}.

Then, based on the following highlighted part, we need to further consider another new UE feature of maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation. As a candidate value, we think that at least the value of ‘64’ should be supported.

Agreement

On Rel.17 enhancements to facilitate MPE mitigation, confirm the following working assumption (in the midst of the previous agreement) as an agreement with the following refinement (highlighted in red):

On Rel.17 enhancements to facilitate MPE mitigation, support the following enhancement on the Rel-16 event-triggered P-MPR-based reporting (included in the PHR report when a threshold is reached, reported via MAC-CE): - In addition to the existing field in the PHR MAC-CE, N≥1 P-MPR values can be reported - The N P-MPR values are reported together with the following: - (Working Assumption) For each P-MPR value, up to M SSBRI(s)/CRI(s), where the SSBRI(s)/CRI(s) is selected by the UE from a candidate SSB/CSI-RS resource pool (FFS: how to perform the selection) - Support M=1 - FFS: The supported value(s) of M - FFS: Additional reporting quantities, e.g. SSBRI/CRI, MPR+DL RSRP, or modified virtual PHR - FFS: additional signaling (e.g. CSI triggering) from the NW

Proposal 3: For MPE mitigation, the following modification is proposed in red

| 23-1-3 | MPE mitigation | 1. Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and [associated] P-MPR in PHR reporting) [2. Support of reporting up to Nmax P-MPR and SSBRI/CRI pairs] [3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation] |
| --- | --- | --- |

Multi-TRP PDCCH enhancements (23-2)

Three UE features (23-2-1, 23-2-1, and 23-2-3) are listed for MTRP PDCCH enhancements in [2]. We have some comments on these FGs as below.

For the third bullet in FG 23-2-1, based on previous RAN1 discussion, when 2 BDs are counted for the two linked PDCCH candidates, UE can do ether individual detection or soft combining. And when 3 BDs are counted for the linked candidates, it is assumed that UE has to do both individual detection and soft combining. That’s why the total number of BDs is 3 which is larger than 2. Therefore, it is clear that the component 3 is only needed when 2 BDs are counted.

In addition, there is a conclusion made in RAN1#106bis-e meeting, i.e. there is no consensus to introduce RRC configuration for the number of BDs. Hence, reporting more than one BD numbers is precluded for component 2. Otherwise, gNB cannot configure which one is used if UE reports two BD numbers.

For component 1, we think it should be used for both USS and CSS type 3 as there is no much difference for PDCCH decoding with repetition for USS and CSS from UE complexity perspective.

Proposal 4: For FG 23-2-1,

- Component 1 is for both USS and CSS type 3.

- For component 2, support of reporting one number as required number of BDs for the two PDCCH candidates

- Component 3 is only for the case of BD = 2.

For 23-2-3 on support of PDCCH repetition for Type 3 CSS, as aforementioned, we think this extra FG is unnecessary. The main FG 23-2-1 is sufficient from our view as there is no much difference between USS and CSS to process PDCCH repetition operation from UE side. No matter two PDCCH is from USS or CSS, the decoding of two PDCCH repetition is the same, i.e. separate decoding or soft combining.

Proposal 5: Remove FG 23-2-3 on support of PDCCH repetition for Type 3 CSS

In addition, inter-span PDCCH repetition was agreed as a UE optional feature in RAN1#106bis-e meeting. Hence, one new FG should be introduced.

Proposal 6: Introduce a new FG on support of inter-span PDCCH repetition.

Here is our suggested text change for Multi-PDCCH enhancement

| 23-2-1 | Multi-TRP PDCCH repetition | 1. Support of PDCCH repetition based on two linked SS sets associated with corresponding CORESETs 2. Support of reporting one number as required number of BDs for the two PDCCH candidates 3. If 2 is reported in component 2, report whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling 4. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |
| --- | --- | --- |
| 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition |
| 23-2-4 | Inter-span PDCCH repetition | Support of inter-span PDCCH repetition |

Multi-TRP PUSCH/PUCCH enhancements (23-3)

Three UE features (23-3-1, 23-3-1-1 and 23-3-2) have been respectively listed for MTRP PUSCH and PUCCH enhancements in [2]. We have some further comments on the details as below.

FG 23-3-1: Multi-TRP PUSCH repetition (type A)

On component 6, it means sequential mapping should be mandatory to MTRP PUSCH scheme. However, both sequential mapping and cyclic mapping for MTRP PDSCH scheme in Rel-16 are NOT explicitly specified as the UE basic feature, because both of them are supported by the UE in default according to RAN1 specifications. Likewise, it is unnecessary to specify sequential mapping as one UE feature, hence we propose to remove component 6.

On component 7, it is NOT in line with any agreement in the previous RAN1 meetings so far, hence we propose to remove component 7.

On component 10, it means STRP/MTRP dynamic switching can be support or not supported to the UE. If so, when the UE cannot support this component, it means the UE can only be scheduled to always transmit PUSCH to multiple TRP, which is an error case of UE design in reality. Hence we propose to remove component 10.

On component 11, we fail to see the necessity for the number of SRS resources when CB and NCB schemes, which is the same as Rel-15/16 and can be captured by RAN1 specifications. Besides, RAN1 has agreed that the number of SRS resources in two SRS resource sets should be the same, so it is unclear to the intention of this component. Hence we propose to remove component 11.

Proposal 7: For FG 23-3-1, remove components 6, 7 10 and 11.

FG 23-3-2: Multi-TRP PUCCH repetition

On component 3, we agree to add the condition “when the number of repetitions is larger than 2” for cyclic mapping, and we are general fine with either way to add the text in component 3, e.g. removing the text in the brackets.

Proposal 8: For FG 23-3-2, support to add the text “when the number of repetitions is larger than 2” in component 3.

Based on the above proposals, we suggest the following change on 23-3-2.

| 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] |
| --- | --- | --- |
| 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable |
| 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0 /] 1_1 / 1_2 |

Inter-cell Multi-TRP (23-4)

One UE feature (23-4) has been listed for inter-cell MTRP enhancements in [2]. We have some further comments on the details as below.

FG 23-4: IntCell-mTRP

On component 3 and component 4, the statements of both X1 (case 1) and X2 (case 2) are opposite with the existing agreement in RAN1#106b-e, where

- X1 (case 1): SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI

- X2 (Case 2): SSB time domain positions and periodicity of the additional PCIs is not according to case 1

Hence the statements of both X1 (case 1) and X2 (case 2) should be revised to be in line with the existing agreement.

Besides, we can be general fine with the differentiation of X in FR1 and FR2, but we think it can be distinguished by the column “FR1-FR2 DIFF”, if necessary. Hence we propose to remove the texts “FR1” and “FR2” in component 3 and component 4, respectively.

On component 5, it is NOT in line with any agreement in the previous RAN1 meetings so far, hence we propose to remove component 5.

Proposal 9: For FG 23-4,

- revise the statement of X1 as “SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI” in component 3.

- revise the statement of X2 as “SSB time domain positions and periodicity of the additional PCIs is not according to case 1” in component 4.

- remove the text “FR1” and “FR2” in component 3 and component 4, respectively.

- remove component 5.

Based on the above proposals, we suggest the following change on 23-4.

| 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [case1] (X1)] [SSB time domain positions and periodicity of the additional PCIs is the same as SSB time domain positions and periodicity of the serving cell PCI] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [case2] (X2)] [SSB time domain positions and periodicity of the additional PCIs is not according to case 1] [5 |
| --- | --- | --- |

Multi-TRP beam management (23-5)

In our views, the explicit sub-features of supporting Rel-17 group-based reporting and supporting Rel-17 mTRP BFR should be introduced for [23-5-1] and [23-5-2], respectively.

- Regarding group based report [23-5-1]

  - Regarding #. RS for both CMR sets (component-3/4), we are open to have some further discussion.

  - Regarding scope of group based reporting (component-2/5), we think that it should be supported for mTRP and should have 2 resource sets that can be merged with first component.

  - Then, for inter-cell mTRP, we are open to have some further discussion.

- Regarding MTRP BFR enhancement [23-5-2]

  - For mTRP-BFR, 2 BFD-RS sets should be supported, and so we do not think that we need the component-2, and for clarification, we can refine the component-1 accordingly.

  - Regarding Component 3/4/5, we are open to have some further discussion, but we do not think that Component-7 is needed under Component-1.

  - Regarding Component 6, it is needed based on the already RAN1 agreement.

  - Regarding Component 8, it seems that only m-DCI mTRP BFR is complete, and we are afraid that s-DCI mTRP BFR may be precluded in Rel-17 mTRP BFR.

Hence, we have the following proposal.

Proposal 10: For MPE mitigation, the following modification in red is proposed

| 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Support a single L1-RSRP reporting instance consisting of N beam groups and M=2 beams per [pair/group] [in one report instance], based on measurement on two CMR resource sets. 2. [Support of enhanced group-based reporting for Rel-17 M-TRP] 3. [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] 4. [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] 5. [Support beam measurement on two CMR resource sets] 6. [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP] |
| --- | --- | --- |
| 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set for two BFD-RS sets 2. [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] 3. [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] 4. [Maximal number of different SSBs of both BFD-RS sets across all CCs] 5. [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] 6. [Support up to two PUCCH-SR resources for MTRP BFRQ] 7. [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] 8. [Support of [single DCI/multi-DCI] based BFR enhancement] |

SFN based HST enhancements (23-6)

For SFN based HST enhancements, several FGs (23-6-1 ~ 23-6-3) are listed in [2]. We generally agree with the first two. However, the third one, i.e., 23-6-3 on support of simultaneous activation of two TCI states for PDCCH across multiple CCs, is not needed from our view. That’s because whether UE supports such feature can completely be inferred by the existing Rel-16 UE FG 16-1b-1 combined with the Rel-17 FG 23-6-1 or 23-6-2.

| 16-1b-1 | TCI state activation across multiple CCs | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH |
| --- | --- | --- |

As we know, for a UE support FG 16-1b-1, some configured CCs in which TCI states can be simultaneously updated for CORESETs with the same ID should share the same RF chain or similar channel conditions for analog beamforming. This is regardless of the number of activated TCI states for each CORESET. So, as long as UE supports FG 16-1b-1, and also supports FG 23-6-1 or 23-6-2, it is natural for this UE to support simultaneous activation of two TCI states across set of configured component carriers.

Proposal 11: Remove FG 23-6-3 on support of simultaneous activation of two TCI states for PDCCH across multiple CCs for HST.

Multi-TRP CSI (23-7)

Several UE features (23-7-1 ~ 23-7-5) are listed for MTRP CSI enhancements in [2]. We generally agree with these UE features, as they are aligned with previous agreements. Some suggestions are given as follows

Proposal 12: For MTRP CSI enhancements

- In FG 23-7-1,

  - a default value of Ks,max should be added as the basic component of this feature. We suggest Ks,max =4 as a compromised value between 2 and 8.

  - 24 ports of max # of Tx ports per source in a resource set in FG 23-7-2 should be supported by default

- FG 23-7-2 is used to support 32 ports per source in a resource set

- FG 23-7-4 is used to support of Nmax=2.

- FG 23-7-3 should be removed as it is similar as component 2 of FG 23-7-1

SRS enhancements (23-8)

Eight UE features (23-8-1 ~ 23-8-8) are listed for SRS enhancements in [2]. We generally agree with having these UE features, as they are aligned with previous agreements. The major remaining issue is the types of these UE features. In general, from NW perspective, it is desirable to avoid too small granularity (e.g., per BC) for the SRS features. For instance, 23-8-2 (Triggering SRS only in DCI 0_1/0_2) is not related with any band-specific or BC-specific characteristics. Hence it should be per UE.

Proposal 13: Strive to avoid per BC features for SRS, e.g., the type of 23-8-2 (Triggering SRS only in DCI 0_1/0_2) should be ‘per UE’.

In RAN1#106bis-e, a new UE optional feature is supported based on the following agreement.

Agreement

For extension of aperiodic antenna switching SRS configurations for <=4Rx, support N=4 for 1T4R and N=2 for 1T2R/2T4R.

- The above extension is UE optional

Hence a new FG is needed for such extension of aperiodic antenna switching SRS of 1T4R and 1T2R/2T4R. We have the following proposal.

Proposal 14: Add the following new FG

| 23-8-9 | Extension of aperiodic SRS configuration for 1T4R, 1T2R and 2T4R | Support of 4 aperiodic SRS resource sets for 1T4R and 2 aperiodic resource sets for 1T2R/2T4R. |
| --- | --- | --- |

For other yellow parts in 23-8-1 ~ 23-8-8, we have the following proposal.

Proposal 15: Update the yellow parts in 23-8-1 ~ 23-8-8 as following.

| 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] |
| --- | --- | --- |
| 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI |
| 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 [2. Report whether the uplink TX switching impact to downlink receiving in a band] [3. Report whether the UL Tx is switched together with UL Tx in another band] |
| 23-8-4 | Maximum 2 SP [and 1 periodic] SRS sets for antenna switching | Support of maximum [2] [SP] SRS resource sets [and maximum 1 periodic SRS resource set] for antenna switching |
| 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource |
| 23-8-6 | Partial frequency sounding of SRS | Support of partial [(RB level)] frequency sounding for SRS |
| 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial [(RB level)] frequency SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS] |
| 23-8-8 | Comb-8 SRS | Support of comb-8 for SRS other than for positioning |

FeType II port selection CSI (23-9)

Four features (23-9-1, 23-9-1a ~ 23-9-3) are developed in [2] for FeType II port selection CSI, which is analogous to eType II CSI. We discuss some further details for 23-9 series in this section.

Optional support of Mv = 2

The complexity and UE hardware cost to support Mv=2 can be different for different triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}. In the basic feature 23-9-1, a list of such triplets are reported for Mv=1. Hence it is needed to report another list of triplets for Mv=2. Otherwise, UE has to consider the worst case when determining whether it can supports Mv=2. For example, if UE can support Mv=2 for small number of Tx ports, but it cannot support Mv=2 for large number of Tx ports, it has to report no support of Mv=2 if the report of Mv=2 does not include the list of triplets. This limits NW to realize the better performance from Mv=2 for small number of Tx ports. Hence we have the following proposal.

Proposal 16: Support to report a list of triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for Mv=2 as follows.

| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=2 |
| --- | --- | --- | --- |

Optional support of R=2

RAN1 has agreed to support R=2 for FeType II PS codebook based on the following agreement.

Agreement

For Rel-17 PS codebook, support R=2 when M=2

- Note that this R is optional, whereas how to support R=2 in Rel-17 UE capability signalling is FFS, e.g. similar with Rel-16 eType II codebook.

A new UE feature about the supported triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for R=2 needs to be added, which is same as eType II codebooks.

Proposal 17: Support to report a list of triplets {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for R=1 as following.

| 23-9-4 | Support of R=2 for FeType-II | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with R=2 |
| --- | --- | --- |

Conclusion

We discuss detailed issues on the NR feMIMO UE feature list in this contribution. Seventeen proposals are given together with detailed update of the FG list are given in Sections 1 - 9.

References

[1] R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.)

[2] R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.)