3GPP TSG RAN WG1 #107-e		R1-2110992

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Remaining issues on inter-cell MTRP operation

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

Introduction

Most critical issues have been resolved in previous RAN1 meetings. There are few issues could be further discussed, in this contribution we discuss few issues which deserve concluding.  In [2], many proposals from individual companies are listed in others sections, according to the comments received during email discussion during RAN1#106b-e, there seem to be no consensus on any of them. Those can be handled in this meeting if there is consensus.

Agreements made in previous meeting can be found in Appendix.

QCL indication for CORESETs with common search space

It should be noted that it is also possible for the network to indicate the QCL source for the CORESETs with common search space, which is typically configured through MIB or SIBs. UE behaviour for these CORESETs should be clarified.

For example, if CORESET #0 TCI state is updated (through dedicated MAC CE signaling) to be associated with QCL source from SSB of another PCI, UE may begin to decode the SIB information of the target cell. It should be clarified whether such behaviour is allowed or not in Rel-17 FeMIMO inter-cell multi-TRP operation.

Above clarification may not be limited for CORESET #0, but also needed for CORESETs associated with type 0/1/2 search spaces.

Proposal 1: Clarify that it is not expected for CORESETs associated with type 0/1/2 SS to be configured/activated with TCI states associated with SSB of PCI different from the serving cell PCI.

On rate matching of non-serving cell SSB

In RAN1#104-e, it was agreed that PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI. However, following two cases are left for further discussion:

- whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

gNB may configure rate matching pattern including SSB pattern(s) by implementation, hence for flexibility of implementation further restriction on rate matching behavior seems not necessary.

Proposal 2: PDSCH in non-serving cell is not rate matched around SSB from serving cell and PDSCH in serving cell is not rate matched around SSB from non-serving cell.

Another issue that needs discussion is on how to interpret “PDSCH/PDCCH from non-serving cell (PCI)” in the following agreement:

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

The straightforward understanding on “PDSCH/PDCCH from non-serving cell (PCI)” is the PDSCH/PDCCH QCL’ed indirectly with an SSB with different PCI from serving cell. Note that it has already been clarified in 8.1.1 that when RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel.

Proposal 3: Clarify that “PDSCH from non-serving cell (PCI)” are those PDCH/PDCCH that use SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference.

- Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

Another aspect to clarify is that it might only be applicable for PDSCH.

Proposal 4: Update previous agreement on rate matching as following:

- PDSCH that uses SSB associated with a physical cell ID as an indirect QCL reference is rate matched around SSB with the same PCI as the indirect QCL reference of the PDSCH.

  - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

Conclusions

In this paper we discussed remaining issues on inter-cell multi-TRP operations, we have following proposals:

Proposal 1: Clarify that it is not expected for CORESETs associated with type 0/1/2 SS to be configured/activated with TCI states associated with SSB of PCI different from the serving cell PCI.

Proposal 2: PDSCH in non-serving cell is not rate matched around SSB from serving cell and PDSCH in serving cell is not rate matched around SSB from non-serving cell.

Proposal 3: Clarify that “PDSCH from non-serving cell (PCI)” are those PDCH/PDCCH that use SSB associated with a physical cell ID different from that of the serving cell as an indirect QCL reference.

- Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

Proposal 4: Update previous agreement on rate matching as following:

- PDSCH that uses SSB associated with a physical cell ID as an indirect QCL reference is rate matched around SSB with the same PCI as the indirect QCL reference of the PDSCH.

  - Note: When RS X is an indirect QCL reference of a target channel, there exists at least one other source signal on the QCL chain between RS X and the target channel

References

[1] RAN1#106b-e, Chairman’s notes

[2] R1-2110482, “Feature lead summary#1 on Enhancements on Multi-TRP inter-cell operation”, RAN1#106b-e

Appendix

In RAN1 #102-e meeting, the following agreements were made:

Agreement Study the following aspects of QCL /TCI-related enhancement to enable inter-cell multi-DCI based multi-TRP operation. - Details on configuration of non-serving cell RS; - Allowed source and target RS types for RS transmitted from the non-serving cell TRP ; - Allowed QCL types for RS transmitted from the non-serving cell TRP ; - Measurement and reporting related to QCL /TCI enhancement except for that in 8.1.1, if any; - Clarification on potential UE behavior for associating/multiplexing non-serving cell RS with other RS/channels; - Other details not precluded.

Agreement

For QCL /TCI related enhancement for enhanced inter-cell multi-TRP operations, support RRC configuration of non-serving cell information

- Non-serving cell information can be associated with the TCI state and/or QCL -info at least when “neighbor cell SSB” is used as “QCL referenceSignal ”

  - FFS : Whether beam indication enhancement is needed in addition to QCL -info enhancement

  - FFS : Whether the association is explicit or implicit

Agreement

The information provided by SSB-Configuration-r16/ssb-InfoNcell-r16 and/or MeasObject can be starting point for providing non-serving cell information

For future meetings

Consider rate matching behavior related to non-serving cell SSB.

In RAN1#103-e meeting, further agreements were made as below:

In RAN1#104-e meeting, further agreements were made as below:

Agreement

Non-serving cell information at least includes non-serving cell PCI to support inter-cell multi-DCI multi-TRP operation

- FFS: Whether the indication of PCI is implicit or explicit

Conclusion

Reuse Rel-15/16 QCL rule between the source and target RS/channel for non-serving cell RS/channel.

Agreement

At least following non-serving cell SSB information are needed in inter-cell MTRP operation

- SSB time domain position

- SSB transmission periodicity

- SSB transmission power

FFS: Other non-serving cell information

FFS: Whether indication of these information is implicit or explicit

Agreement

For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e

- Option1: Indicate/associate non-serving cell PCI in the TCI state

  - FFS other non-serving cell information

- Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell

  - FFS: how the flag is linked to non-serving cell

- Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively.

  - FFS: Each group is associated with a CORESETPoolIndex value.

  - FFS: how to link the group of TCI states to non-serving cell.

- Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS

  - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, …

  - FFS: detailed re-indexing rule(s) of non-serving cell RSs

- Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with

  - FFS: how the indicator is linked to non-serving cell

  - Note: when there is only one non-serving cell, it means the same as Option2.

Agreement

Agree on scheme1

- Scheme1: PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI

- FFS: whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- FFS: whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Conclusion

The UE may assume received DL transmission from multiple TRP within a CP in FR1 and FR2.

- Note: This does not imply that RAN1 intends to ask RAN4 to tighten network synchronization requirements.

In RAN1#104b-e meeting, further agreements were made as below:

Agreement

- For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC

  - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC.

  - Applicable at least for non-cross carrier QCL indication

    - FFS: Cross carrier scheduling QCL indication

- RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP

- Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

Conclusion

Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specifcation point of view

Agreement

For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e

- Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex

- Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex

- Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex

Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e

Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e

In RAN1#106-e meeting, further agreements were made as below:

Agreement

Introduce a new RRC indicator/signalling (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with, where the new indicator/signaling is not the exact PCI value

- Detailed signalling design is up to RAN2

Agreement

Rel. 17 inter-cell MTRP, the maximum number of additional RRC -configured PCIs per CC is denoted X and can be reported as a UE capability

- For the report value of X, multiple candidate values including 1 is supported.

  - FFS : Which values to support other than 1.

  - Values larger than 7 are precluded

  - RAN1 needs to agree on value(s) of X other than 1

- Down-select one of the following alternatives:

  - Alt 1: A single value of X is reported as UE capability for any possible SSB time domain position and periodicity

  - Alt 3: At least Two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on SSB time domain position and periodicity with respect to serving cell SSB

- The serving cell PCI is always associated with active TCI states, only 1 additional PCI can be associated with the active TCI States

Agreement

- For inter-cell mTRP , one PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with one CORESETPoolIndex , another PCI associated with one or more of activated TCI states for PDSCH/PDCCH is associated with another CORESETPoolIndex

- FFS : The association between PCI and CORESETPoolIndex when switching between intra-cell mTRP and inter-cell mTRP

Agreement

For a CSI-RS QCLed with a neighboring cell SSB, the CSI-RS EPRE is calculated based on powerControlOffsetSS and the SSB transmission power in the neighboring cell information.

Agreement

LS to RAN2 on multi-TRP inter-cell is endorsed in R1-2108633.

In RAN1#106b-e meeting, further agreements were made as below:

Agreement

Support two independent X values (X1, X2) are reported as a UE capability for at least two different assumptions on additional SSB time domain position and periodicity with respect to serving cell SSB.

- X1 (Case 1)= The maximum number of configured additional PCIs when the each configurations of SSB time domain positions and periodicity of the additional PCIs are is the same as SSB time domain positions and periodicity of the serving cell PCI

- X2 (Case 2)= The maximum number of configured additional PCIs when at least one of the configurations of SSB time domain positions and periodicity of the additional PCIs is not according to Case 1

- Note: By definition, Case 1 and Case 2 cannot be enabled simultaneously

- Supported values for X1 and X2 includes at least 0,1,2,3 and 7. FFS on other values

- This UE capability has FR1 and FR2 differentiation (FFS : Whether this UE capability is per UE or per

Updated Proposal 6:

- Center frequency, SCS, SFN offset are assumed to be the same for SSBs from the serving cell and the configured cells having TRPsSSBs with different PCI different from the serving cell for inter-cell multi TRP operation.

- For non-serving cell SSB information, the information related to “SSB time domain position” for non-serving cell SSB with PCI different from the serving cell consists of [halfFrameIndex and] ssb-PositionsInBurst