3GPP TSG RAN WG1 #107-e		R1-2110993

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Remaining issues on MTRP multibeam enhancement

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

Beam reporting for MTRP

In Rel-16, both ideal backhaul and non-ideal backhaul are considered for MTRP with the same beam reporting framework as Rel-15. The MTRP transmission defined in Rel-16 such as NCJT, URLLC SDM, FDM schemes requires simultaneous reception by the UE. And in these use cases above, MTRP based beam management is beneficial for traffic offload between TRPs, where the network can configure UE to report the beams from different TRPs which can be transmitted simultaneously.  Some commercial UEs have the capability to activate multiple Rx panels indicating that the UE can connect to and receive from different TRPs with multiple Rx panels simultaneously. Beam reporting for MTRP should take all the above into account.

Rel-17 MTRP beam reporting enhancement should take into account both ideal backhaul and non-ideal backhaul scenarios to support simultaneous transmission at the network side and simultaneous reception at the UE side.

Beam reporting scheme in ideal backhaul MTRPs

In RAN1#104-e meeting [1], Option 2, that is a single CSI-report consisting of N beams pairs/groups and M (M=2) beams per pair/group, and different beams within a pair/group can be received simultaneously was agreed. In ideal backhaul MTRPs, the backhaul delay between TRPs can be negligible so that the TRP receiving the reported beam information can forward it to the other TRP. In that case, Option 2 can effectively achieve simultaneous transmission on the TRP side and simultaneous reception on the user side. After discussion in previous meetings, the procedure of Option 2 is almost complete: two CSI-RS/SSB resource sets are configured to measure, and after measurement, X beam pairs that can be received simultaneously could be reported, where X is no more than 4 and is configured by the network based on UE capability, and UCI design for the reported beam information also has been determined. However, two remaining issues need to be further determined and resolved for Option2 to work well. Detailed discussion on the issues are as follows:

Activation of two semi-persist CSI-RS resource sets

SP CSI-RS/CSI-IM Resource Set Activation MAC CE

- In Rel-15, only one semi-persist CSI-RS resource set can be activated by a MAC CE activation command as shown in Figure 1. Therefore, how to activate two semi-persist CSI-RS resource sets is a potential issue to be considered.  In our view, two CSI-RS resource sets can be activated by one activation command together or activated by two activation commands separately. Compared with the former, the latter can reuse the format of the MAC CE activation command in Rel-15. But for some specific cases, such as one of the activation commands can not be received by UE or both activation commands can not be received simultaneously, UE behavior needs to be clarified.  As for the former, two CSI-RS resource sets can be activated simultaneously, but the shortcoming is the format of the MAC CE activation command need to be redesigned.

Support enhancement on activation of two semi-persist CSI-RS resource sets simultaneously

Default beam of aperiodic CSI-RS

- In Rel-16, the default beam of aperiodic CSI-RS in single-DCI-based MTRP and multi-DCI-based MTRP scenarios had been specified. For multi-DCI-based MTRP, if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet is smaller than the UE reported threshold beamSwitchTiming/beamSwitchTiming-r16, the UE applies the QCL assumption of the other DL signals satisfying following conditions to receive the aperiodic CSI-RS:

- overlap fully in symbols with the aperiodic CSI-RS

- associate with the same CORESETPoolindex as the aperiodic CSI-RS

- If there is no DL signal meeting the requirements, UE will apply the QCL parameter(s) of the CORSET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH triggering that aperiodic CSI-RS. However, due to the agreed two CSI-RS resource sets for beam measurement and report in MTRP scenarios, it means for aperiodic CSI-RS resource, a DCI would trigger two resource sets. In this case, directly reusing the default beam mechanism defined in Rel-16 may be ambiguous, as a result of both sets may associate the same CORESETPoolindex. Therefore, the default beam of aperiodic CSI-RS for beam management in multi-DCI-based MTRP scenarios needs to be clarified. In our view, defining the default beam per TRP is the most straightforward way.

Default beam of aperiodic CSI-RS resource should be clarified when a DCI trigger two CSI-RS resource sets

Interference measurement and report metric

In previous meetings, solutions for this issue were discussed but no agreement was reached. In our opinion, when the UE measures the CSI-RS/SSB resources, the result of L1-SINR may reflect the inter-beam interference, but it can’t reflect the quality of the simultaneous reception. For example, in the NCJT scenario, the interference between different beams does not necessarily correspond to the effective post-SINR if simultaneously received by two Rx panels. L1-SINR value measured as strong interference between two beams may also imply that the two beams contribute higher spatial diversity to the MIMO capacity. Generally, the factors such as received signal power, spatial diversity, and spatial correlation determine the MIMO performance of NCJT, which cannot be reflected by the L1-SINR computation based on current specification.

We analyze the throughput performance of reported beams based on L1-SINR criterion and L1-RSRP criterion respectively. In the simulation, we assume one panel on the TRP side and two panels (back to back) on the UE side.  Each panel contains one TXRUs with dual-polarization. While sweeping beams, UE only selects and reports one optimal beam pair based on L1-RSRP or L1-SINR. More simulation parameters can be found in Appendix A.

The methods of selecting beam pair for an MTRP UE are given as below:

L1-RSRP criterion: a UE selects one beam with the highest L1-RSRP per Rx panel, each corresponding to a different TRP. Independent spatial domain filters per Rx panel are assumed, implying any beam pairs can be received simultaneously by different spatial domain filters.

L1-SINR criterion: a UE selects one beam pair with the highest capacity based on L1-SINR regarding the CMR resources from another TRP as interference. Assuming beam 1 is for by Rx panel 1 and beam 2 is for Rx panel 2, then the leakage of beam 1 received by Rx panel 2 is deemed as the interference, and so is beam 2. The capacity of a beam pair is calculated as follows:

capacity=log2(1+SINR1)+log2(1+SINR2)

where SINR1 and SINR2 are L1-SINR of the beam pairs for two Rx panels respectively, given by

SINR1=P1P2'+I1

SINR2=P2P1'+I2

where P1, P2 are signal power of beam 1, beam 2 received by Rx panel 1 and Rx panel 2 respectively, P2', P1' are interference power of beam 2, beam 1 measured by Rx panel 1 and Rx panel 2 respectively. I1 and I2 are interference outside the two TRPs.

We provide UPT performance and comparison for FTP model 1 with RU for baseline L1-RSRP setting to 4%, 16% and 61% for FR2 Indoor Hotspot, where UE is not allowed to rotate and backhaul delay is negligible between TRPs. We set the same packet arrival rate (λ) for the cases with the same value of RU. Considerable UPT loss can be observed at 5% and 50% UPT, and mean UPT as well.

UPT with different measurement and report metrics in MTRP, FR2 InH

| RU | Report metric | Mean | 5% | 50% |
| --- | --- | --- | --- | --- |
| 4% | L1-RSRP | 519 MHz | 291 MHz | 444 MHz |
|  | L1-SINR | 473 MHz | 263 MHz | 444 MHz |
|  | Loss | -8.86% | -9.62% | 0.00% |
| 16% | L1-RSRP | 434 MHz | 147 MHz | 434 MHz |
|  | L1-SINR | 363 MHz | 117 MHz | 405 MHz |
|  | Loss | -16.36% | -20.41% | -6.68% |
| 61% | L1-RSRP | 334 MHz | 75.1 MHz | 337 MHz |
|  | L1-SINR | 217 MHz | 33.3 MHz | 188 MHz |
|  | Loss | -35.03% | -55.66% | -44.21% |

From the above simulation results, we observe that beam pair selected on L1-SINR criterion have obvious performance loss compared to that of L1-RSRP criterion, especially for the user at the cell boundary. The reason for the performance degradation of L1-SINR criterion is that regarding the CMR resources from another TRP as interference directly may ignore some signal information, especially for NCJT.

For MTRP, performance loss is observed for L1-SINR-based beam pair report with interference calculated between the reported beam pair.

Compared with L1-SINR, we think the CQI for MTRP CSI can precisely reflect inter-beam interference and determine whether two beams are suitable for grouping. However, feeding back the CQI per beam pair could increase too much calculation complexity at UE to implement. Thus, L1-RSRP discussion should be prioritized in Rel-17.

Do not support L1-SINR report with interference calculated between the reported beam pair.

Beam failure recovery for MTRP

In RAN1#106bis-e meeting [2], following agreement on BFR for MTRP was achieved.

Agreement To associate BFD-RS set k and NBI-RS set j - Alt-1: 1-to-1, fixed in spec - Whether NBI-RS configuration is mandatory is separate discussion Agreement Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. - FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case. Agreement For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with CORESETPoolIndex k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD -RS set k (k=0,1) in the MAC-CE for TRP -specific BFR - The above applies to Scell and SpCell - The above applies for the multi-DCI case Agreement SCS of the 28 symbols is the smallest SCS of the active DL BWP for the response reception CC and of the active DL BWP (s) of the CC(s) with the failed TRP link(s) reported in BFR MAC CE. Agreement FL proposal 2.12-1: RACH-based transmission can be triggered on a SpCell at least in the following scenarios - Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell - FFS: other scenarios - Scenario 2: at least one TRP fails on SpCell - Scenario 3: at least one pre-defined TRP fails on SpCell - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available - Scenario 5: If MAC-CE based reporting does not work (details FFS) - Scenario 6: When no PUCCH-SR is configured Agreement For RACH-based transmission, at least when all BFD-RS sets fail in SPCell, CBRA is supported Conclusion Design of MAC-CE related to SpCell when transmitted on msg3, msgA is up to RAN2.

TRP specific BFR

For TRP-specific BFR procedure, TRP-specific BFD, TRP-specific new candidate beam identification, TRP-specific BFRQ, and TRP-specific BFRR have been agreed in previous meetings and some details, such as the number of BFD-RS set, and the contents of BFR MAC CE also have been determined. We think both single-DCI-based MTRP and multi-DCI-based MTRP should be considered for TRP-specific BFR enhancement. Even though single-DCI-based MTRP is designed to conquer the possible blockage happened in FR2 by transmitting different layers or frequency resources of a PDSCH or PDSCH transmission occasions from two TRPs, the network would achieve more reliable PDCCH transmission or better PDSCH/PUSCH transmission scheme if the network was able to know beam failure of the radio link between either TRP and the UE in time.

In multi-DCI-based MTRP operation, TRP can be identified by the configured CORESETPoolIndex. However, there is no explicit TRP index in single-DCI-based MTRP scenario. With the agreement that only explicit configuration of BFD-RS is allowed for s-DCI case in the RAN#1 106-e meeting, BFD-RS set ID can be used to achieve subsequent TRP-specific BFR procedure,  but the signaling structure may be more complicated due to introducing an extra association at least between CORESETs and BFD-RS set ID. Additionally, considering multi-DCI has higher priority than single-DCI agreed in RAN#1 104-e meeting, we think the procedure of TRP-specific BFR in single-DCI-based MTRP scenarios can be simplified as that UE only indicate the failure event in BFR MAC CE based on the explicitly configured BFD-RS, and no TRP-specific NBI-RS configuration and beam resetting.  In this case, UE only reports the beam failure event and does not need to reset beam for the failed TRP and thus reuses the procedure defined for multi-DCI-based MTRP BFR procedure as much as possible.

Simplify the procedure of TRP-specific BFR in Single-DCI-based MTRP scenarios due to low priority

Support indication of failure event in BFR MAC CE based on the explicitly configured BFD-RS

No further enhancement on the configuration of TRP-specific NBI-RS and beam resetting

TRP-specific Beam failure detection

Independent BFD-RS configuration per-TRP has been agreed, where each TRP is associated with a BFD-RS set and the total number of BFD-RS set per BWP is 2, and the maximum number of BFD-RSs in each set is up to UE capability. No matter it is single-DCI-based MTRP or multi-DCI-based MTRP, two sets of BFD-RS, e.g., q0,0, q0,1, can be explicitly configured was agreed, each set consisting of the BFD-RS(s) associating with one TRP.  As for the implicit configuration method, it is only allowed to apply for multi-DCI-based MTRP scenarios to determine two sets of BFD-RSs.

After the discussion in the last meeting, two issues about the configuration of BFD-RS were left and need to be resolved. One of them is how to explicitly configure two sets of BFD-RSs, and some candidate schemes were proposed in the last meeting as follows:

- Alt 1: BFD RS is configured and updated by RRC

- Alt 2: BFD RS is configured by RRC

  - Alt 2-1: Support MAC CE to update the TCI state for BFD-RS resource

  - Alt 2-2: support MAC CE to update the BFD-RS resource

- Alt 3: BFD RS is configured and updated by MAC CE

From the perspective of specification impact, Alt 1 follows the legacy configuration method in Rel-15 so that it has very small specification impact. From the perspective of maintaining beam alignment between PDCCH and BFD-RS when there is a beam switch for PDCCH, we are also fine with Alt 2-1 as it is the simplest one between Alt 2-1, Alt 2-2, and Alt 3.

For TRP-specific BFD-RS explicit configuration and update,

Support  RRC configuration ;

Support update by RRC reconfiguration or MAC CE to update the QCL assumption of BFD-RS.

Another remaining issue is the value of X, e.g., the number of TCI states of CORESETs with CORESETPoolindex=k to determine the BFD-RS set k implicitly, and how to select TCI state(s) when the value of X exceeds UE capability. In our view, this issue can be resolved by network implementation or UE implementation. From the perspective of BFD capability definition, it should be assumed that the value of X is configured no more than the UE capability. While from the perspective of scheduling flexibility, e.g., each CORESET QCLed with different periodic CSI-RS resources, thus the value of X may exceed the UE capability. If exceeds, TCI state selection based on UE implementation is most straightforward, rather than designing a certain selection rule to complicate the specification.

For TRP-specific BFD-RS implicit configuration,

the value of X should be configured no more than UE capability.

If the value of X exceeds UE capability, TCI state selection is up to UE implementation.

Except for the above issues about the configuration of BFD-RS, some details on the procedure of TRP-specific beam failure detection also should be discussed and determined. TRP-specific beam failure detection is carried out for each BFD-RS set respectively and the periodicity of indicating the failed BFD-RS set index to high layer had been agreed to be determined by the maximum value of {minimal periodicity of BFD RS in the set, 2ms} in 104bis-e meeting. Therefore, considering that different TRP may have different sensitivity for beam failure detection, we think some RRC parameters about beam failure detection should be configured per BFD-RS set, such as beamFailureInstanceMaxCount and beamFailureDetectionTimer.

For TRP-specific beam failure detection, support configure TRP-specific RRC parameters, e.g., beamFailureInstanceMaxCount-1, beamFailureInstanceMaxCount-2,  and beamFailureDetectionTimer-1, beamFailureDetectionTimer-2.

Configuration of TRP-specific NBI-RS sets

As the independent configuration of the NBI-RS set per TRP and the 1-to-1 association between each BFD-RS set and an NBI-RS set has been supported, new candidate beam selection based on the TRP in beam failure is enabled if TRP-specific NBI-RS set is configured. However, for multi-DCI-based MTRP, we don’t think the configuration of TRP-specific NBI-RS set(s) for TRP-specific BFR is a necessity. For example, if one TRP fails and its corresponding TRP-specific NBI-RS set is not configured, an aperiodic CSI report can be triggered by the TRP in a good radio link to obtain one or more new beams to recover the failed TRP. Compared with periodic measurement of NBI-RS resources, aperiodic beam measurement consumes less resource, which is beneficial for the network to schedule various services of users within the limited UE capability flexibly.

If NBI-RS resources are configured, to fully leverage the benefit of MTRP operation, UE should be able to freely select new candidate beams of other TRPs but not limit to the TRP in beam failure, so that any TRP providing good radio links can be selected to serve the UE. Therefore, new beams of more other TRPs should be configured in the candidate beam set. For example, as illustrated in Figure 2, two NBI-RS sets can be configured, each associating with a BFD-RS set and including the new beams of the associating TRP and other TRPs that the UE can select. In this way, the UE can select one new beam out of the new beam set corresponding to the beam link pair of the TRP in failure and of other TRPs except the one in good radio link, rather than limiting to the beam of the TRP in beam failure. Otherwise, restricting the new candidate beam of the TRP in beam failure will achieve inferior performance to the free selection case.

Example of NBI-RS set configuration

However, whether the new candidate beam can be the beam with the TRP still in good radio link other than the one still in use needs further study. If it is allowed, the network would fall back to single TRP transmission once the new candidate beam is selected as the beam of the TRP in a good radio link.

Support to optionally configure TRP-specific NBI-RS for multi-DCI-based MTRP.

TRP-specific BFRQ

One of the benefits of TRP-specific BFR is that a UE connecting with multiple TRPs can help itself avoid the break of all radio links. This can also be reflected by the beam failure recovery request reliability. The BFRQ can be transmitted through the good radio link to the TRP not in beam failure. One or two dedicated PUCCH-SR resources in a cell group had been supported in previous meetings.

For the case of two dedicated PUCCH-SR resources, the configuration of two PUCCH-SR resources was discussed but no consensus was reached in the previous meetings. In Rel-16, high-layer parameters including sr-ProhibitTimer, sr-TransMax and SR_COUNTER are configured per SR configuration. If two dedicated PUCCH-SR resources are under the same SR configuration in Rel-17, the set of higher layer configured parameters would be shared between two TRPs. This would be less flexible compared to TRPs each having their own set of parameters.

As for the spatial relation per PUCCH-SR resource, when a UE is configured with two PUCCH-SR resources in a cell group, each associated with a TRP/BFD-RS set and a spatial filter for its corresponding TRP or the other TRP, the UE can select the appropriate PUCCH-SR resource to request a UL grant for MAC CE with BFRQ. With TRP-specific PUCCH-SR resources, the network is able to know the TRP in beam failure and may not schedule the failed TRP to transmit. However, when a UE is configured with only one PUCCH-SR resource in a cell group, the PUCCH-SR resource may be configured with one or two spatial relations. And for the case of two spatial relations, we think it is used as agreed for PUCCH repetition to facilitate reliability, not for selecting one spatial relation towards a certain TRP to transmit PUCCH.

For the dedicated PUCCH-SR resources for TRP-specific BFRQ,

For the case when two PUCCH-SR resources are configured for TRP-specific BFRQ in a cell group,

Clarify whether the two PUCCH-SR share the same set of higher layer parameters (sr-ProhibitTimer, sr-TransMax and SR_COUNTER) if two PUCCH-SR resources are under the same SR configurations.

Each PUCCH-SR resource associated with a BFD-RS set of one TRP can be configured with the spatial filter towards its corresponding TRP or the other TRP in FR2.

For the case when one PUCCH-SR resource is configured for TRP-specific BFRQ in a cell group,

The PUCCH-SR resource can be associated with one or two spatial relations.

The case of two spatial relations is used as agreed for PUCCH repetition.

For the case of BFR of one TRP, when two dedicated PUCCH-SR resources are configured in a cell group, it was agreed that each PUCCH-SR resource should be associated with one BFD-RS set on SpCell in the last meeting. Besides SpCell, we think the association between a BFD-RS set on SCell and a PUCCH-SR resource is also necessary to achieve TRP-specific BFRQ.

In the previous meetings, three alternatives for PUCCH-SR resource selection were discussed and they are listed as follows:

Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set

Alt-2: PUCCH-SR resource associated with failed BFD-RS set

Alt-3: Leave it up to UE implementation

According to different use cases, we think three alternatives can be divided into two categories. For the case that at most one TRP fails in each CC and all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, Alt1 and Alt2 are appropriate to be used. Compared Alt1 and Alt2, we think the only difference between them is the spatial relation, where the spatial relation of PUCCH-SR resource in Alt1 towards the TRP which the PUCCH-SR resource associates with, while the spatial relation of PUCCH-SR resource in Alt2 towards the TRP which the other PUCCH-SR resource associates with. Therefore, we are fine with Alt1 and Alt2. For the other cases, Alt-3 is applied. In addition to the PUCCH-SR resource, the selection rule for UL grant should also be considered when more than one UL grants are available to transmit BFR MAC CE.

For TRP-specific BFRQ,

when SR is triggered and 2 PUCCH-SR resources are configured, support to configure an association between PUCCH-SR resource and BFD-RS set on each CC.

If at most one TRP fails in each CC and all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, either Alt1 or Alt2 can be used to select one PUCCH-SR resource to be transmitted.

Otherwise, Alt-3 should be applied.

When more than one UL grants are available to transmit BFR MAC CE, the selection rule should also be considered.

gNB response enhancement

For the case of BFR of one TRP, after the UE transmits the BFRQ carrying the new beam with a first PUSCH MAC CE, it can monitor PDCCH for scheduling a PUSCH transmission with the same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value like legacy procedure designed for SCell beam failure. When one TRP is in failure and no new beam has been found and reported based on the corresponding NBI-RS resource(s), the network can deactivate the failed TRP through MAC CE deactivation command to achieve the switch of transmission hypothesis from MTRP to STRP.

Support to deactivate TRP through MAC CE if there is no new beam found.

UE behavior upon receiving BFRR and not receiving BFRR

UE behavior upon receiving BFRR should consider the MTRP mode, the number of beam links in failure, and the number of new beams. As for UE behavior, beam reset, TRP deactivation, and beam training may be conducted optionally in different cases, detailed analyses are as follows.

For a UE operating with multi-DCI-based MTRP, and in the case of BFR for one TRP, the behavior of beam reset that after 28 symbols from receiving the legacy BFR response(PDCCH), the QCL assumption of all CORESETs associated with the failed BFD-RS set reported in the MAC-CE for TRP-specific BFR is updated by the RS resource associated with the latest reported new candidate beam associated with the failed BFD-RS set was agreed in the last meeting. As for updating the QCL assumption for other DL channels/RSs, UL spatial filter/power control assumption for PUCCH, and other UL channels/RSs, we think it may introduce extra signaling overhead but no obvious benefits. Therefore, we don’t support discussing other channels in the subsequent limited time.

For a UE operating with multi-DCI-based MTRP,

For the case of BFR of one TRP, if it has reported the new beam,

after 28 symbols from receiving the legacy BFR response (PDCCH), do not support update QCL assumption for other DL channels/RSs except for PDCCH, UL spatial filter/power control assumption for PUCCH, and other UL channels/RS

For the case of BFR of one TRP but no new beam has been found and reported, UE does not reset the beam.

UE may fall back to single TRP reception when BFRR is a MAC CE deactivation command for the failed TRP.

It is not necessary to define beam resetting behavior for UE operating with single-DCI-based MTRP operation.

BFR for both failed TRPs

In section 2.1, the procedure of TRP-specific BFR for one TRP link failure has been discussed. And in this section, the topic to be discussed is the procedure of BFR for two failed TRPs, which also includes BFD, NBI, BFRQ, BFRR and update of QCL assumption.

For beam failure detection for both TRPs, according to the previous agreement achieved in the last meeting, that RACH-based transmission will be triggered when beam failure is detected on all BFD-RS sets on the SpCell,  it has been supported at least for SpCell that beam failure detection for both TRPs is based on the two TRP-specific BFD-RS set, rather than configure one extra cell-specific BFD-RS set. For SCell, we think it can reuse the same mechanism. However, there is ambiguity regarding how to determine both BFD-RS sets are in beam failure. Rarely in practical scenarios would the two TRPs fail exactly at the same moment. It is highly possible that even when the radio link condition of two TRPs is not suitable for transmission anymore, the failure event detection would happen sequentially within a short period. Thus it is more reasonable to define a time window to determine whether two TRPs fail simultaneously.

For beam failure detection when two TRP-specific BFD-RS sets are configured, if the failure events of different BFD-RS sets are declared within a predefined time window, the failure event of two TRPs failing simultaneously would be declared.

For new beam identification when both TRPs fail, TRP-specific new beam identification for each failed TRP would serve the purpose and thus there is no need to configure a cell-specific NBI-RS set. In this case, each failed TRP may obtain its new beam.

It is not necessary to configure cell-specific NBI and TRP-specific NBI simultaneously.

For BFRQ when both TRPs fail, it should be discussed separately for SpCell and SCell. If two TRP fails simultaneously in SpCell, it has been agreed that CBRA-based BFRQ would be triggered in the last meeting. If two TRP fails simultaneously in SCell, PUCCH-based BFRQ will be triggered, like BFR for SCell in Rel-16. However, if two TRP fails at different time but within the predefined window, one TRP-specific BFRQ may be triggered after the first BFD-RS set is detected with beam failure, in this case, we need to consider the timeline of the TRP-specific BFRQ when the second BFD-RS set is declared with beam failure, such as whether to cancel the TRP-specific BFRQ before the start of the beam failure recovery request reporting two TRP link failure. For BFRQ when both TRPs fail, the contents included in BFR MAC CE should also be considered. In RAN#1 106-e meeting, the contents of BFR MAC CE for the case of BFR of one TRP had been determined, which includes indices of failed BFD-RS set, indices of CC containing the failed TRP link, an indicator of whether a new candidate beam is identified in the NBI-RS set associated with the failed BFD-RS set, and a resource indicator representing the new candidate beam (if identified) based on the number of NBI-RS resources in the corresponding NBI-RS set. In our view, to recover both failed TRP, beam failure events and new beam information of two failed TRPs should be included in BFR MAC CE.

For BFRQ when both TRPs are in failure,

- Reuse PUCCH-based BFRQ for SCell when two TRP are declared with beam failure simultaneously;

- The timeline of TRP-specific BFRQ should be considered when two TRP are declared with beam failure at different time but within the predefined window;

- Beam failure events and new beam information of two failed TRPs should be included in BFR MAC CE.

For BFRR when both TRPs fail, it can reuse the legacy BFRR mechanism defined in Rel-15 if both TRPs in SpCell fail, or reuse the legacy BFRR mechanism defined in Rel-16 if both TRPs in SCell fail.

For the update of QCL assumption, after 28 symbols from receiving the BFR response, for each failed BFD-RS set, the QCL assumption of all CORESETs associated with the failed BFD-RS set reported in the MAC-CE for TRP-specific BFR is updated by the RS resource associated with the latest reported new candidate beam associated with the failed BFD-RS set.

After receiving the BFR response, the QCL assumption of all CORESETs associated with the failed BFD-RS sets should be updated by their corresponding new candidate beams reported in BFR MAC CE simultaneously.

Conclusion

In this contribution, we analyze enhancements of beam management for MTRP in Rel-17, including beam reporting and TRP-specific BFR. To summarize, we have the following observations and proposals.

- Option 3 provides the highest Scheduling flexibility with the same UCI payload size per PUCCH/PUSCH resource for non-ideal backhaul scenarios.

For non-ideal backhaul cases, some throughput gain can be achieved by Option 3, while outdated beam pair applied due to backhaul delay for Option 1 and Option 2 when forwarding the beam report from one TRP to the other TRP.

For MTRP, performance loss is observed for L1-SINR-based beam pair report with interference calculated between the reported beam pair.

- Rel-17 MTRP beam reporting enhancement should take into account both ideal backhaul and non-ideal backhaul scenarios to support simultaneous transmission at the network side and simultaneous reception at the UE side.

Support enhancement on activation of two semi-persist or aperiodic CSI-RS resource sets simultaneously

Default beam of aperiodic CSI-RS resource should be clarified when a DCI trigger two CSI-RS resource sets

Do not support L1-SINR report with interference calculated between the reported beam pair.

Simplify the procedure of TRP-specific BFR in Single-DCI-based MTRP scenarios due to low priority

Support indication of failure event in BFR MAC CE based on the explicitly configured BFD-RS

No further enhancement on the configuration of TRP-specific NBI-RS and beam resetting

For TRP-specific BFD-RS explicit configuration and update,

Support  RRC configuration ;

Support update by RRC reconfiguration or MAC CE to update the QCL assumption of BFD-RS.

For TRP-specific BFD-RS implicit configuration,

the value of X should be configured no more than UE capability.

If the value of X exceeds UE capability, TCI state selection is up to UE implementation.

For TRP-specific beam failure detection, support configure TRP-specific RRC parameters, e.g., beamFailureInstanceMaxCount-1, beamFailureInstanceMaxCount-2,  and beamFailureDetectionTimer-1, beamFailureDetectionTimer-2.

Support to optionally configure TRP-specific NBI-RS for multi-DCI-based MTRP.

For the dedicated PUCCH-SR resources for TRP-specific BFRQ,

For the case when two PUCCH-SR resources are configured for TRP-specific BFRQ in a cell group,

Clarify whether the two PUCCH-SR share the same set of higher layer parameters (sr-ProhibitTimer, sr-TransMax and SR_COUNTER) if two PUCCH-SR resources are under the same SR configurations.

Each PUCCH-SR resource associated with a BFD-RS set of one TRP can be configured with the spatial filter towards its corresponding TRP or the other TRP in FR2.

For the case when one PUCCH-SR resource is configured for TRP-specific BFRQ in a cell group,

The PUCCH-SR resource can be associated with one or two spatial relations.

The case of two spatial relations is used as agreed for PUCCH repetition.

For TRP-specific BFRQ,

when SR is triggered and 2 PUCCH-SR resources are configured, support to configure an association between PUCCH-SR resource and BFD-RS set on each CC.

If at most one TRP fails in each CC and all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, either Alt1 or Alt2 can be used to select one PUCCH-SR resource to be transmitted.

Otherwise, Alt-3 should be applied.

When more than one UL grants are available to transmit BFR MAC CE, the selection rule should also be considered.

Support to deactivate TRP through MAC CE if there is no new beam found.

For a UE operating with multi-DCI-based MTRP,

For the case of BFR of one TRP, if it has reported the new beam,

after 28 symbols from receiving the legacy BFR response (PDCCH), do not support update QCL assumption for other DL channels/RSs except for PDCCH, UL spatial filter/power control assumption for PUCCH, and other UL channels/RS

For the case of BFR of one TRP but no new beam has been found and reported, UE does not reset the beam.

UE may fall back to single TRP reception when BFRR is a MAC CE deactivation command for the failed TRP.

It is not necessary to define beam resetting behavior for UE operating with single-DCI-based MTRP operation.

For beam failure detection when two TRP-specific BFD-RS sets are configured, if the failure events of different BFD-RS sets are declared within a predefined time window, the failure event of two TRPs failing simultaneously would be declared.

It is not necessary to configure cell-specific NBI and TRP-specific NBI simultaneously.

For BFRQ when both TRPs are in failure,

- Reuse PUCCH-based BFRQ for SCell when two TRP are declared with beam failure simultaneously;

- The timeline of TRP-specific BFRQ should be considered when two TRP are declared with beam failure at different time but within the predefined window;

- Beam failure events and new beam information of two failed TRPs should be included in BFR MAC CE.

After receiving the BFR response, the QCL assumption of all CORESETs associated with the failed BFD-RS sets should be updated by their corresponding new candidate beams reported in BFR MAC CE simultaneously.

References

Chairman’s notes, RAN1#104-e.

Chairman’s notes, RAN1#106bis-e.

Appendix A: Simulation parameters

SLS assumption for MTRP beam report enhancement

| Parameters |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Indoor hotspot (InH) |
| Frequency Range |  | FR2, 30GHz |
| Inter-BS distance |  | 20m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at TRP |  | 2 Tx ports: (M, N, P, Mg, Ng) = (4,4,2,1,1) |
| Antenna setup and port layouts at UE |  | 4Rx Port: (M, N, P, Mg, Ng, Mp,Np) = (2,4,2,1,2,1,2) (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 23dBm |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 120kHz |
| Number of RBs |  | 52 |
| Simulation bandwidth |  | 80 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| Configuration for MTRP | Cluster | 4 neighboring TRPs |
|  | Maximal number of coordinating TRPs | 2 |
|  | Backhaul assumption | non-ideal |
| Feedback assumption |  | Beam and CSI feedback periodicity: 5 ms Scheduling delay (from CSI/beam feedback to time to apply in scheduling): 4 ms Subband PMI, subband CQI Rank 1 or rank 2 per TRP |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Ideal |