3GPP TSG RAN WG1 #107		R1-2111003

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Remaining issues on channel access mechanism for NR operation from 52.6GHz to 71 GHz

Agenda Item:	8.2.6

Document for:	Discussion and Decision

- Introduction

The revised WID for operations extended to 71GHz was approved in RAN#91e [1]. The physical layer procedures such as channel access were agreed for further investigation:

- Physical layer procedure(s) including [RAN1]:

  - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

    - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

    - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

    - Study, and if needed specify, energy detection threshold enhancement

In this contribution, we will discuss the details of all these channel access mechanisms, as well as the channel access mechanism selection in this frequency range.

- Discussion

LBT procedure related issues

In the regulation defined by ETSI for 60GHz band [2], there are no definitions for LBT bandwidth. In NR, UE may support different bandwidth, which will impact on the LBT bandwidth when accessing the channel in 60GHz band. Meanwhile, the ED threshold in the regulation is not specified for the LBT procedure when applying directional LBT beams. For some mechanisms used in NR-U such as Cat 2 LBT multi-channel LBT, etc., they are not clearly specified in the regulation. In the following sections, we will discuss the details of the mentioned issues.

Cat.2 LBT

Cat.2 LBT has been discussed and the following agreements were achieved in RAN1#104-e [3]:

Agreement:

If Cat 2 LBT is introduced, the following use cases can be further studied:

- Resume transmission after a gap Y:  Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y)

- COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT

- Multi-Beam LBT:  Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z)

- Rx-Assistance:  Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling

Other use cases not precluded.

FFS if Cat 2 LBT is mandated for each use case or not.

The introduction of the Cat. 2 LBT should be limited to some scenarios in addition to COT sharing in certain region. For example, for multi-beam LBT, Cat 2 LBT may be used before switching to a new transmission beam (not used in the earlier part of the COT) in a COT with TDM beams. This is due to the fact that the channel for the newly switched beam has not been occupied during the past transmission. Therefore, it is better to perform a CCA check before the transmission to make sure that the channel is still available in this direction. For receiver-assisted LBT, if LBT is mandatary at the receiver side, then Cat 2 LBT is the best choice for the sake of transmission efficiency. Besides, if Type-B multi-channel channel access scheme is introduced, Cat 2 LBT should also be applied to the channels other than the primary channel.

However, for the resumed transmissions, if the resumed transmission is after multiple switching points, according to the regulation, an equipment (initiating or not initiating transmission) can skip the CCA upon correct reception of a packet intended for this equipment, and immediately proceed with transmission in response to received frames. Therefore, no additional LBT is required. If there is no transmission from the responding device, the resume transmission can also follow the case with multiple switching points, no additional LBT is required.

Proposal 1: The Cat 2 LBT can be used before switching to a new beam in a COT with TDM beams, before response with assistant information at the receiver, and in the Type B multi-channel access scheme.

Multi-channel channel access

The multi-channel channel access has been discussed in RAN1#104 [3]

Agreement:

Define Type A and Type B multi-channel channel access as:

- Type A: Perform independent eCCA for each channel

- Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot

Down-selection between

- Alt1: Support Type A multi-channel channel access only

- Alt2: Support both Type A and Type B multi-channel channel access.

Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

For multi-channel channel access scheme, Type B multi-channel channel access requires to perform Cat 2 LBT for the non-primary channels. Therefore, if Cat 2 LBT is finally agreed to be introduced in B52.6GHz frequency range for other use cases as discussed in section 2.1.1, it can also be applied to Type B multi-channel channel access. In this case, both Type A and Type B multi-channel channel access can be supported.

Proposal 2: Both Type A and Type B multi-channel channel access can be supported.

ED threshold

The ED threshold has been discussed during RAN1#104 and RAN1#104b meeting. The interpretation of the parameters is still not clear. The following agreement and working assumption are made during the discussion.

Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

Working assumption:

For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

According to the regulation, the RF output power is the mean EIRP for the equipment during a transmission burst [2]. No RF output power for the responding device is considered. Therefore, the working assumption should be modified as below:

Revised working assumption:

For Pout in EDT determination, define Pout as the maximum of mean EIRP of transmission burst for the node determining EDT during a COT.

To support the revised working assumption, the transmission burst should be clarified. The definition of transmission burst in NR-U can be considered as a baseline: the transmission burst is a set of transmissions from gNB/UE without any gaps greater than [16us]. However, consider the directional transmission and LBT procedure in high frequency band, the transmission burst should be defined further considering the sensing beam and the transmission beam. The transmission burst is a set of transmissions from gNB/UE from one or more transmission beams within a sensing beam without any gaps greater than [16us]. That is to say, if the sensing beam only covers one transmission beam, i.e., per-beam LBT, then the transmission burst is a set of transmissions from gNB/UE from that transmission beam without any gaps greater than [16us]. Transmissions in different beams are considered as different transmission bursts. Therefore, different EDTs can be applied to the different transmission beams. If the sensing beam covers more than one transmission beam, i.e., wide beam LBT, then the transmission burst is a set of transmissions from gNB/UE from all transmission beams within a sensing beam without any gaps greater than [16us]. The duration of the gap in B52.6GHz can be smaller than 16us.

Proposal 3: The transmission burst is a set of transmissions from gNB/UE from one or more transmission beams which are “covered” by a sensing beam without any gaps greater than [16us].

Proposal 4: For Pout in EDT determination, define Pout as the maximum of mean EIRP of transmission burst for the node determining EDT during a COT.

In addition, when calculating the ED threshold, the operating channel bandwidth is not clearly defined yet for 3GPP system. Since the ED threshold is used to evaluate the energy level in a certain frequency range, the most natural way is to refer the operating channel bandwidth to the LBT bandwidth, i.e., channel bandwidth or BWP bandwidth.

Proposal 5: The LBT bandwidth should be used as the operating channel bandwidth for EDT evaluation.

In RAN1#106b meeting [7], the adjustment of the EDT was further classified as below:

- Scenario 1: For UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1} and when the same TX beam is used for sensing, no additional EDT adjustment is introduced

- Scenario 2: For other cases (other than scenario 1) where sensing beam “covers” the transmission beam and has lower beamforming gain, the EDT is adjusted higher lower/tighter by the difference between the antenna gains of the sensing beam and transmission beam

  - Note: This is to make sure the same jammer at the transmission beam direction can be detected with the lower gain sensing beam

- Scenario 3: If UE uses omni beam for sensing, no additional EDT adjustment is introduced. If UE is using a directional beam for sensing (with positive antenna gain, so the UE will see higher energy level compared with omni sensing beam), either EDT is adjusted higher/looser by the antenna gain or the measurement energy is adjusted lower by the antenna gain before measured energy is compared with EDT

During the discussion of last meeting, it looks like that people have different views on the already agreed EDT calculation. In our opinion, the previous agreed EDT equation is feasible for sensing with omni-directional antenna. The ED threshold is defined assuming a 0 dBi antenna gain, and the transmission antenna gain is already included in the Pout. When directional LBT is applied, the beamforming gain for the receiving antenna is no longer 0 dBi, and the impact of the beamforming gain should be counted in when calculating the ED threshold. Otherwise, the affected surrounding nodes in the interference zone will be different. The antenna gain of the transmission antenna should not be additionally considered since it is already included in the Pout. A straightforward way to consider the beamforming gain impact can be realized by adding the antenna gain to the ED threshold formula. Different LBT beams will lead to different ED thresholds depending on their beamforming gain. However, we are also flexible to adjust the measurement energy by deducting the antenna gain. That is to say, we support scenario 3.

Proposal 6: The previously agreed EDT calculation equation is applicable for omni-directional sensing beam.

Proposal 7: If UE is using a directional beam for sensing, either EDT is adjusted higher/looser by the antenna gain or the measurement energy is adjusted lower by the antenna gain before measured energy is compared with EDT.

For MU-MIMO, it is possible that at the beginning of a COT, gNB performs LBT for beam 1 and beam 2, and transmits to UEs with beam1 and beam 2, then gNB receives UL transmissions with beam 1 and/or beam 2. At the meantime, gNB can perform LBT for beam3, in the next transmission burst, gNB transmits to UEs with beam1 and beam 3. After that, gNB transmits to UE with beam 3 only. In this case, different beams may maintain different COTs as shown in Figure 1. Beam 1 and beam 2 share the same COT since the LBTs for them are finished right before the COT. The LBT for beam 3 is performed within the COT for beam 1 and beam 2, and the COT duration of beam 3 is independently maintained. Furthermore, the COT duration can be indicated per-sensing beam, UE then determines whether to share the gNB-initiated COT based on both the COT duration and the sensing beam information.

Figure 1 Overlapped COTs

Proposal 8: A node can initiate two (or more) (partially) overlapping COTs in two (or more) different beams.

Directional LBT

LBT for SDM and TDM transmission

The alternatives for directional LBT were further clarified as following in RAN1#104-e [3].

Agreement:

For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both)

- Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

Agreement:

Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations

- Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

  - FFS: Details on the definition of "cover"

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

- Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

For spatial multiplexing, Alt 1 with one LBT beam covers all transmission beams provides a simpler LBT procedure. However, it may overprotect the transmitting node since any interference within the wide beam coverage will block all transmissions. The merit of the spatial multiplexing is degraded by the overprotection of the wide beam LBT. Multiple LBT beams covering multiple transmission beams lead to multiple LBT operations, but provide more precise LBT results. The transmitting node can still transmit to the receivers in other directions even if the channel of one direction is sensed as busy. Therefore, considering the pros and cons of the two alternatives, both of them should be supported for MU-MIMO (SDM) transmission.

Proposal 9: For a COT with MU-MIMO (SDM) transmission, both Alt 1 and Alt 2 are supported.

Similar as SDM transmission, both wide beam and per-beam LBT should be supported for the COT with TDM transmission. However, for per-beam LBT, there are two options. The only difference between Alt 2 and Alt 3 is the channel access scheme for beam switching in TDM transmission. Since before transmitting with the newly switched beam, the channel in that direction has never been occupied. It is different from COT sharing, the channel has been occupied and then shared with the responder. In beam switching case, the transmission is performed one by one, the channels for later transmissions are not occupied after LBT until the start of the transmission. Therefore, in order to reduce potential interference to surrounding nodes, the transmitter should perform additional directional LBT for the transmissions when switching to a different beam in the middle of COT as shown in Figure 2. Independent per-beam LBT is performed at the beginning of the COT for all the beams used in the COT. Before each beam switching, an additional LBT is performed to make sure that the channel for the newly switched beam is still idle.

Figure 2 Directional LBT for multiple beams

Proposal 10: For a COT with TDM transmission, both Alt 1 and Alt 3 are supported.

Per-beam LBT

In RAN1#104b-e [4], the per-beam LBT was discussed. A few alternatives have been identified for the per-beam LBT for MU-MIMO transmission and the transmission with TDM beams.

Agreement:

For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed in TDM fashion

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Agreement:

Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed one after another in time domain

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

For MU-MIMO (SDM) transmission, when the equipment has the capability to transmit and receive with different beams simultaneously, the most efficient way to perform per-beam LBT is to perform LBT over different beams simultaneously in parallel. When the equipment does not support simultaneous LBT with different beams, the TDM LBT in Alt A-1 can be applied. With Alt A-2, we are not sure how to perform SDM transmission if the eCCA is performed on one beam and start transmission with the beam to occupy the COT and then move on to the eCCA on the other beam. With Alt A-3, the exact LBT procedure and benefit are not clear, different companies seem to have different proposals.

Proposal 11: For a COT with MU-MIMO (SDM) transmission, both Alt A-1 and Alt B are supported.

For transmitting with different beams within a COT in a TDM way, the per-beam LBT is performed based on the capability. If the equipment has the capability to simultaneously sense in different beams, e.g., with digital beams or with different TRPs, the per-beam LBT can be performed simultaneously in parallel. If the equipment does not have the capability to simultaneously sense with different beams, it should perform the LBT for different beams one after another in time domain. More specifically, Alt A-1 is supported, the node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle. Alt A-2 is not supported since gNB should always leave gap for LBT between transmissions. Due to the uncertainty of the channel availability, the time duration for eCCA is hard to estimate. Therefore, some of the transmissions with pre-defined pattern may not be transmitted due to the eCCA between transmissions. With Alt A-3, as we mentioned above, the procedure and the benefit are not clear.

Proposal 12: Alt A-1 and Alt-B are supported for the transmission within a COT with TDM of beams with beam switching.

Spatial relationship between sensing beam and transmission beam

The spatial relationship between all possible sensing beams and the transmission beams has been extensively discussed. It is agreed that the sensing beam should “cover” the transmission beam(s). However, the definition of “cover” is not clear yet. The potential options are discussed in RAN1#106b-e as below [7]:

Agreement:

- When UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, support the following behaviors

- If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing

- Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing

- FFS: The case when UE does not indicate a capability for beam correspondence

- Note: The UE should meet local regulatory requirements

Proposal:

For the following

- Sensing at gNB

- UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}

- UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={0} before UL beam management procedure

- UE chooses to uses a different beam for sensing than the beam used for transmission,

Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s)

- Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective

  - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam.

  - Alt-1B:  the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain

  - Alt-1C:  The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP.  The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

  - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain

  - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams.

- Sending LS to RAN4 and inform them the above and request them to make the final choice

  - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well

  - It is up to RAN4 to further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it for UE implementation

The intention of the proposal is to illustrate the cases when the sensing beam and the transmission beam are not perfectly matched, i.e., when using a wide sensing beam covers the transmission beam(s), or when the device does not support beam correspondence, as shown in Figure 3. In these cases, UE cannot perform directional LBT using the beams referring to the SRI or TCI as in the agreement. Besides, there is no beam correspondence requirement defined for gNB. Therefore, it is better to define “cover” for these cases. Since X dB beam width is a widely used concept in RAN4. One simple alternative is to define “cover” as the angle included in the [3] dB beam width of the transmission beam is included in the [X] dB beam width of the sensing beam.

Figure 3 Relationship between sensing beam and transmission beam(s)

Proposal 13: The “cover” for sensing beam is defined as: the angle included in the [3] dB beam width of the transmission beam(s) is included in the [X] dB beam width of the sensing beam.

Receiver assisted LBT

Receiver assisted LBT is observed to be beneficial for the receivers suffered from hidden node problem. However, the detailed design of the signaling is not discussed yet. In this section, we discuss the possible channels and contents for the related signal such as transmitter request, receiver feedback in NR. Some preliminary agreement has been achieved in RAN1#106 [6]:

Agreement:

For receiver to provide assistance in channel access, channel sensing and reporting need to be performed. The following schemes can be further considered. Target down-selection by RAN1 #106bis-e

- Scheme 1: L1-RSSI based receiver assistance

  - Resource used for RSSI measurement

    - Alt 1: RSSI measurement is based on the time/frequency resources configured for ZP-CSI-RS

      - FFS: any enhancement needed for ZP-CSI-RS for this purpose (eg., ZP-CSI-RS over all REs in BWP over one or more symbols).

    - Alt 2: Energy measurement on operating BW over indicated or specified number of symbols or time interval

  - L1-RSSI is reported in an AP-CSI report

  - L1-RSSI trigger in UL grant

    - FFS if L1-RSSI trigger can also be carried in DL grant

  - Timeline for L1-RSSI reporting is at least equal to AP-CSI reporting and RAN1 strives to tighten the timeline

    - Note: If L1-RSSI reporting timeline cannot be tighter than AP-CSI reporting timeline, this scheme is not needed

  - FFS: How to indicate the measurement beam for L1-RSSI

  - FFS: What is included in the L1-RSSI report, such as the value of RSSI measurement, comparison outcome with Energy Detection threshold, etc

- Scheme 2: CCA or eCCA based receiver assistance with existing phy channel/signals

  - Scheme 2-1: gNB schedules/triggers UL PUCCH/SRS transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUCCH (or SRS in the case of 1-bit Rx-assistance) to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens.

    - FFS if the downlink data transmission can be granted with the same DL DCI that schedules/triggers the first UL PUCCH/SRS transmission, in which case, the CCA or eCCA is performed for at least the first UL PUCCH/SRS transmission

  - Scheme 2-2: gNB schedules/triggers UL transmission PUSCH with the UL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUSCH to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, the downlink data transmission happens.

- Scheme 3: CCA or eCCA based receiver assistance with new RTS/CTS type transmission

  - New RTS/CTS-like signaling introduced.

  - gNB sends RTS-like signaling to UE. UE performs CCA or eCCA and if LBT passes, transmits CTS-like signaling to explicitly indicate the LBT outcome. gNB detects the CTS-like signaling to identify if the UE passed CCA or eCCA. After detecting the CTS-like signal, the data transmission happens

- Scheme 4: Legacy L3-RSSI with potential enhancements

  - FFS potential enhancements, e.g., supporting gNB indicating the beam used for UE RSSI measurement, supporting gNB indicating new reference SCS and measurement bandwidths

- Note: The schemes listed above are not mutually exclusive and should be discussed separately.

Scheme 2 uses the existing mechanism and can provide prompt assistant information. However, we think both Scheme 2-1 and Scheme 2-2 should be changed by deleting “data” in the last sentence if the original intention is to transmit PDSCH without PDCCH after receiving the assistant information. In current spec, at least PUCCH cannot be transmitted before the PDSCH, therefore, Scheme 2-1 by scheduling PUCCH and PDSCH with same DCI is not feasible. For Scheme 2-2, it does not make sense to schedule PDSCH in advance, and after receiving the PUSCH as the receiver assistant information, gNB transmits PDSCH immediately. gNB can simple schedule PDSCH after receiving the PUSCH, i.e., gNB can transmit PDCCH after receiving the assistant information. In our opinion, the assistant information should be trigger separately with PDSCH. gNB schedules/triggers UL PUCCH/SRS only transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. The triggering of PUCCH/SRS only transmission can be realized by setting the related fields to special values, which has very small spec impact. For the sake of progress, we are open to further decouple the downlink transmission and the assistant information, it can be up to gNB implementation. However, there must be an assistant information in case gNB want to refer to for DL transmission. Therefore, Scheme 2 should be modified as below:

- Scheme 2: CCA or eCCA based receiver assistance with existing phy channel/signals

  - Scheme 2-1: gNB schedules/triggers UL PUCCH/SRS only transmission with the DL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUCCH (or SRS in the case of 1-bit Rx-assistance) to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, gNB determines if the downlink data transmission happens depending on the detection of the Receiver-assistance information.

  - Scheme 2-2: gNB schedules/triggers UL transmission PUSCH with the UL assignment DCI and indicates CCA or eCCA in the DCI. UE performs CCA or eCCA for the scheduled/triggered UL transmission and if LBT passes, transmits the Receiver-assistance information (implicitly or explicitly) in the PUSCH to indicate the LBT outcome. gNB detects the scheduled UL transmission to tell if UE passes the CCA or eCCA. After detecting the Receiver-assistance information, gNB determines if the downlink data transmission happens depending on the detection of the Receiver-assistance information.

Proposal 14: Adopt the modified scheme 2.

Short control signaling

In RAN1#104b-e, it has been agreed that the contention exempt short control signalling can be applied to the transmission of SS/PBCH and the potential DL transmissions that can be multiplexed with SS/PBCH are investigated as below:

Agreement:

- Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH.

  - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule

  - FFS: Whether this can be applied to all supported SCS or specific SCS.

  - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH

  - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

- FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc

The discovery burst defined as in Rel-16 NR-U is agreed to be introduced in NR 52.6-71Ghz. In NR-U, the discovery burst is considered to be important and can be transmitted with Type 2A channel access mechanism if the duration is at most 1 ms. Therefore, the contention exempt short control signaling can be at least extended to the discovery burst defined in NR-U if the duration of the discovery burst is less than 1ms. That is to say, when it comes to contention exempt short control signaling, the SS/PBCH can be multiplexed with RMSI PDCCH, RMSI PDSCH, non-zero power CSI-RS. SS/PBCH multiplexing with other signals/channels can be further studied.

Proposal 15: The contention exempt short control signaling can be extended to discovery burst with duration at most 1ms.

Proposal 16: The contention exempt short control signaling based SS/PBCH can be multiplexed with RMSI PDCCH, RMSI PDSCH and CSI-RS.

In RAN1#105-e [5], the short control signalling was further discussed and the UL signals that can be considered as short control signalling are as below:

Agreement:

- Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS.

  - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals)

  - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell

  - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective

- FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

From our point of view, the regulation regarding short control signalling is for the actually transmitted signals, not the configured resources. Only when the signal is transmitted, it is considered as a short control signalling and the signal duration is counted in. if the signal is not transmitted, no transmission duration should be counted in the total short control signalling transmission time. Therefore, Alt 2 is supported for msg 1 and Msg A transmission when they are considered as short control signalling.

Proposal 17: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective.

- Conclusion

In this contribution, we focus on the channel access mechanisms for NR operation from 52.6GHz to 71GHz, and have the following proposals:

Proposal 1: The Cat 2 LBT can be used before switching to a new beam in a COT with TDM beams, before response with assistant information at the receiver, and in the Type B multi-channel access scheme.

Proposal 2: Both Type A and Type B multi-channel channel access can be supported.

Proposal 3: The transmission burst is a set of transmissions from gNB/UE from one or more transmission beams which are “covered” by a sensing beam without any gaps greater than [16us].

Proposal 4: For Pout in EDT determination, define Pout as the maximum of mean EIRP of transmission burst for the node determining EDT during a COT.

Proposal 5: The LBT bandwidth should be used as the operating channel bandwidth for EDT evaluation.

Proposal 6: The EDT calculation equation is applicable for omni-directional sensing beam.

Proposal 7: If UE is using a directional beam for sensing, either EDT is adjusted higher/looser by the antenna gain or the measurement energy is adjusted lower by the antenna gain before measured energy is compared with EDT.

Proposal 8: A node can initiate two (or more) (partially) overlapping COTs in two (or more) different beams.

Proposal 9: For a COT with MU-MIMO (SDM) transmission, both Alt 1 and Alt 2 are supported.

Proposal 10: For a COT with TDM transmission, both Alt 1 and Alt 3 are supported.

Proposal 11: For a COT with MU-MIMO (SDM) transmission, both Alt A-1 and Alt B are supported.

Proposal 12: Alt A-1 and Alt-B are supported for the transmission within a COT with TDM of beams with beam switching.

Proposal 13: The “cover” for sensing beam is defined as: the angle included in the [3] dB beam width of the transmission beam(s) is included in the [X] dB beam width of the sensing beam.

Proposal 14: Adopt the modified scheme 2.

Proposal 15: The contention exempt short control signaling can be extended to discovery burst with duration at most 1ms.

Proposal 16: The contention exempt short control signaling based SS/PBCH can be multiplexed with RMSI PDCCH, RMSI PDSCH and CSI-RS.

Proposal 17: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective.

- References

- RP-210862, Revised WID:  Extending current NR operation to 71 GHz, RAN #91e, Electronic Meeting, March 16-26, 2021.

- ETSI EN 302 567, v2.1.20, 2020-06.

- 3GPP RAN1#104-e, Chairman’s notes.

- 3GPP RAN1#104b-e, Chairman’s notes.

- 3GPP RAN1#105-e, Chairman’s notes.

- 3GPP RAN1#106-e, Chairman’s notes.

- 3GPP RAN1#106b-e, Chairman’s notes.