3GPP TSG RAN WG1 #107-e		R1-2111056

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Discussion on UE features for NR coverage enhancement

Agenda Item:	8.16.8

Document for:	Discussion and Decision

- Introduction

This contribution includes our proposals for Rel-17 coverage enhancement UE features. The discussion is based on the latest version of UE features summarized in R1-2110587 [1].

- Discussion

- TBoMS related features

In RAN1#106e, it has been agreed that the number of slots for TBoMS is counted based on available slots for UL transmission, and the available slots are determined based on the mechanisms determined for type-A PUSCH repetition counted on available slots. Hence, feature 30-2, i.e., type-A PUSCH repetitions counted on available slots, should be considered as prerequisite feature for feature 30-3.

On top of that, the candidate values of number of slots for a single TBoMS are being discussed in AI 8.8.1.2, and this should be captured in the component for feature 30-3.

Proposal 1: Feature 30-2 should be considered as prerequisite feature for feature 30-3. And value range of number of slots for a single TBoMS should be captured in the component for Feature 30-3.

The revised UE Feature 30-3 is provided in following table.

| Features | Index | Feature group | Components | Prerequisite feature groups | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH in RRC connected mode. Supported values for a single TBoMS include {2,4,8} | [11-6] 30-2 | [Optional with capability signalling] |

- DMRS bundling related features

For DMRS bundling related features, it depends on detailed RAN4 FR requirements on power consistency, phase continuity, and it may be FR specific or band specific, may be not per UE feature. In [2], RAN4 replied that the maximum duration (feature 30-4) maybe per band or per FR dependent, and Feature 30-4 is considered as prerequisite for other DMRS bundling features. Hence, whether features 30-4 and 30-4(a)-(g) are per band features, or need to be different for FR1/FR2, needs to be confirmed by RAN4.

Proposal 2: Whether feature 30-4 and 30-4(a-g) is per ‘band/UE’, or ‘need of FR1/FR2 differentiation’ should be confirmed by RAN4.

The suggested revision DMRS bundling related UE features are provided in following table.

| Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30-4a | [DM-RS bundling for PUSCH repetition type A] | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] [Per band] | FFS | No [YES] |
| 30-4b | [DM-RS bundling for PUSCH repetition type B] | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] [Per band] | FFS | No [YES] |
| 30-4c | [DM-RS bundling for TB processing over multi-slot PUSCH] | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] [Per band] | FFS | No [YES] |
| 30-4d | [DMRS bunding for PUCCH repetitions] | Support DM-RS bundling for PUCCH repetitions | [30-4], [4-23] | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] [Per band] | FFS | No [YES] |
| 30-4e | [Inter-slot frequency hopping with inter-slot bundling for PUSCH] | Support inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | UE does not support inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] [Per band] | FFS | No [YES] |
| 30-4f | [Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling] | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] [Per band] | FFS | No [YES] |
| 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] [Per band] | FFS | No [YES] |

- Msg3 repetition related features

For msg3 repetition related features 30-6, whether the capability reporting is needed depends on RAN2 further discussions, according to RAN2 agreements as follows.

Agreements - Msg3 repetition is applicable to all cases that trigger 4-step CBRA procedure (can come back if we identify that some specific case should not be covered) - A separate RSRP threshold is introduced for requesting Msg3 repetition - Extension of ra-ResponseWindow and ra-ContentionResolutionTimer are not needed for Msg3 repetition. - RAN2 confirms enhancing MAC RAR for indicating MSG3 repetition is not supported. - Postpone the discussion on UE capability (i.e. whether explicit UE capability is needed for indicating the support of Msg3 repetition).

Hence, whether Features 30-6 is ‘Optional with capability signaling’ or ‘Optional without capability signaling’ can be up to RAN2 discussion.

Proposal 3: Whether Features 30-6 is ‘Optional with capability signaling’ or ‘Optional without capability signaling’ can be up to RAN2 discussion.

- Conclusion

In this contribution, we discussed the coverage enhancement related UE features, and we have the following proposals.

Proposal 1: Feature 30-2 should be considered as prerequisite feature for feature 30-3. And value range of number of slots for a single TBoMS should be captured in the component for Feature 30-3.

Proposal 2: Whether feature 30-4 and 30-4(a-g) is per ‘band/UE’, or ‘need of FR1/FR2 differentiation’ should be confirmed by RAN4.

Proposal 3: Whether Features 30-6 is ‘Optional with capability signaling’ or ‘Optional without capability signaling’ can be up to RAN2 discussion.

- References

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e.

- R4-2114991, Reply LS on PUCCH and PUSCH transmissions, RAN4 #100-e.