3GPP TSG RAN WG1 #107-e		R1-2111057

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Discussion on UE feature for IAB enhancements

Agenda Item:	8.16.10

Document for:	Discussion and Decision

- Introduction

This contribution presents our view on Rel-17 IAB UE features. The discussion is based on the latest version of IAB UE feature list in [1] and appendix.

- Discussion

  - Timing case indication

Regarding the timing mode indication, case 6 timing mode indication and case 7 timing mode indication should be defined separately. Especially considering that, Case 6 timing mode indication is indicated from parent node to IAB node to enable the Case 6 timing mode, but Case 7 timing mode indication is indicated from IAB node which to enable the Case 7 timing mode for its child node. Since the transmission and reception nodes of Case 6 and Case 7 timing indication are different nodes, it is reasonable to split the timing mode indication into two sub-features.

Based on the updated features, Case 6 timing alignment and Case 7 timing alignment are described by FG 31-4 and FG 31-5, respectively. It is believed that Case 6 timing mode indication should be captured in this FG 31-4, and Case 7 timing mode indication should be captured in this FG 31-5.

Moreover, in RAN1#106e e-meeting, it was concluded that “Details on the design of the indication of when Case 6 timing and Case 7 timing is performed at the IAB-node are to be discussed under 8.10.1.”. That means, whether to use timing case indication or multiplexing case indication is still not decided.

Proposal 1: The FG for timing case indication is respectively defined for case 6 timing indication and case 7 timing indication, e.g., defined in FG 31-4 and FG 31-5, respectively.

Proposal 2: RAN1 further discuss whether to term the FG 31-4/FG 31-5 as ‘timing case alignment’ or ‘multiplexing case indication’.

Regarding the feature that child MT to receive a timing offset for Case 7 timing alignment, a separated FG can be defined, if timing case indication is finally replaced by multiplexing case indication.

Proposal 3: If FG 31-4/FG 31-5 as ‘multiplexing case indication’, additional FG is defined for child MT to receive a timing offset for Case 7 timing alignment.

- Conclusion

This contribution focus on UE features for eIAB, and the following are proposed,

Proposal 1: The FG for timing case indication is respectively defined for case 6 timing indication and case 7 timing indication, e.g., defined in FG 31-4 and FG 31-5, respectively.

Proposal 2: RAN1 further discuss whether to term the FG 31-4/FG 31-5 as ‘timing case alignment’ or ‘multiplexing case indication’.

Proposal 3: If FG 31-4/FG 31-5 as ‘multiplexing case indication’, additional FG is defined for child MT to receive a timing offset for Case 7 timing alignment.

- Reference

- R1-2110587,Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e (AT&T, NTT DOCOMO, INC.)

- Appendix

| - Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 31. NR_IAB_enh | 31-1 | Guard symbols | 1) Support [Rel-17 DesiredGuardSymbols] reporting 2) Support [Rel-17 ProvidedGuardSymbols] reception |  | Yes | N/A | Guard symbols reporting and reception associated with Case 6 and 7 timings are not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact Note: If an IAB node does not support a certain timing mode, the reported/provided values shall be ignored | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-2 | [Child IAB-DU beam restriction indication] | Support [DU Beam Indication] reception |  | Yes | N/A | Parent-node cannot indicate restricted beams at the IAB-DU. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-3 | [Child IAB-MT beam recommendation indication] | Support [MT Beam Indication] transmission 1) IAB-MT DL beam 2) IAB-MT UL beam |  | yes | N/A | IAB-node cannot indicate recommended/[non-preferred] MT DL/UL beam to parent node | Per IAB-node | no | no | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-4 | [Case 6 timing alignment] | Support [Case 6 transmission] |  | Yes | N/A | Case 6 timing at the IAB-node is not supported. Switching across different timing cases (i.e., Case 1 at IAB-node, Case 6 at IAB-node, and/or Case 7 at the Parent) is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-5 | Case 7 timing [alignment] | Support [Case7 transmission] |  | Yes | N/A | Parent-node cannot adopt both (and switch between) Case 1 and Case 7 timing. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-6 | DL TX power adjustment | 1.) Support [Desired DL TX Power Adjustment] reporting 2.) Support [DL TX Power Adjustment] reception |  | Yes | N/A | Parent-node’s DL TX power adjustment reporting and reception is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | 31-7 | [Desired] UL TX power adjustment | Support [Desired IAB-MT PSD range] reporting |  | Yes | N/A | Desired MT’s UL PSD range reporting is not supported. | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
| 31. NR_IAB_enh | [31-8] | [Dynamic indication of Rel-17 or FDM soft resource availability] | Support monitoring DCI Format 2_5 scrambled by AI-RNTI for indication of FDM soft resource availability to an IAB node |  | Yes | N/A | The IAB-node is unable to receive explicit availability indication for Soft resources | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| 31. NR_IAB_enh | 31-9 | Simultaneous transmission and reception from multiple parent nodes | Support simultaneous transmission and reception from multiple parent nodes |  | Yes | N/A | Simultaneous transmission and reception is not supported in DC scenario | per BC | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |