3GPP TSG RAN WG1 #107-e		R1-2111059

e-Meeting, November 11th – 19th, 2021

Source:	vivo

Title:	Discussion on UE features for MBS

Agenda Item:	8.16.12

Document for:	Discussion and Decision

- Introduction

In last meeting, Rel-17 NR UE features list has been preliminarily discussed and updated in [1]. In this contribution, our views on MBS part are further provided based on [1].

- Discussion

For broadcast, multiple G-RNTIs facilitate different services, and thus, support of  G-RNTIs should be included in broadcast feature 33-1.

For multicast, the capability for group-common PDSCH reception doesn’t have to be coupled with HARQ-ACK feedback. In general, there exist three reactions when UE receives group-common PDSCH: does not send any HARQ-ACK feedback; sends NACK-only based HARQ-ACK feedback; sends ACK/NACK based HARQ-ACK feedback. Thus, support of ACK/NACK based HARQ-ACK feedback should be separated from 33-2. Support of FDM-ed HARQ-ACK codebook should be separated from 33-3-2, and support of TDM-ed HARQ-ACK codebook should be separated from 33-3-3.  Furthermore, similar as that in NR Rel-15, support of type 1 and type 2 codebook should be separated as different UE capabilities. Regarding the retransmission for multicast, whether to differentiate PTP (re)transmissions for unicast and PTP (re)transmissions multicast is under discussion and thus, it’s better to wait for further progress to determine whether support of PTP retransmission for multicast is a separate UE capability.

According the achieved agreements in appendix, support of intra-slot TDM between unicast PDSCH(s) and group-common PDSCH(s), or among multiple group-common PDSCHs in a slot, and support of FDM between unicast PDSCH and group-common PDSCH should also apply to broadcast. Thus, 33-3-2 and 33-3-3 should take 33-1 or 33-2 as prerequisite feature groups.

Similar as dynamic scheduled group-common PDSCH reception, capability of HARQ-ACK feedback should be separated from data reception in 33-5-1.

For NACK-only based HARQ-ACK feedback, there is no consensus to support enabling/disabling HARQ-ACK feedback based on DCI and the motivation to support it is not clear. Thus, ‘ACK/NACK based’ should be kept in 33-7.

The corresponding track changes with respect to R1- 2110587 is shown below:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. - Support slot-level repetition for MTCH - Support of G-RNTIs for broadcast. FFS details of N |  | Yes | Optional without capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support PTM retransmission for multicast. - FFS: Support PTP retransmission for multicast. FFS whether/how to separate the above capabilities from FG 33-2 |  | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. | 33-1, 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). | 33-1 or 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-1 | ACK/NACK based HARQ-ACK feedback for multicast | - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured per G-RNTI by RRC signaling. | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-2 | TDM-ed Type-1 HARQ-ACK codebook for multicast | - Support TDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-2, 33-3-3, 33-4-1 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-3 | FDM-ed Type-1 HARQ-ACK codebook for multicast | - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. | 33-2, 33-3-2, 33-4-1 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-4 | Type 2 HARQ-ACK codebook for multicast | - Support Type-2 HARQ-ACK codebook for multicast. | 33-2, 33-4-1 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-4-5 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support [N>1] SPS group-common PDSCH configuration for multicast - Support [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling ACK/NACK basedHARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling ACK/NACK based HARQ-ACK feedback | 33-2 | Yes | Optional with capability signalling |

- Appendix

Agreements: (RAN1#102) For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. - FFS: TDM or SDM in a slot. Agreements: (RAN1#103) Support TDM between one unicast PDSCH and one group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs. Agreements: (RAN1#103) Further study the following cases for simultaneous reception of unicast PDSCH and group-common PDSCH in a slot based on UE capability for RRC_CONNECTED UEs. - Case 1: support TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot - Case 2: support TDM among multiple group-common PDSCHs in a slot - Case 3: support TDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot - Case 4: support FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot - Case 5: support FDM among multiple group-common PDSCHs in a slot - FFS: maximum number of PDSCHs in a slot simultaneous received per UE Agreement: (RAN1#104bis) At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs - Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - FFS: the value(s) of M - Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC - FFS: the value(s) of N - Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - FFS: the value(s) of K and L

- References

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.)