3GPP TSG RAN WG1 #107-e		R1-2111086

e-Meeting, November 11th – 19th, 2021

Agenda Item:     8.1.2.2

Source:	Spreadtrum Communications

Title:	Discussion on enhancement on multi-TRP inter-cell operation

Document for:	Discussion and decision

Introduction

According to R17 FeMIMO WID [1], the following objective is included in the scope:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception

In this paper, we will present our views on this issues.

Discussion

In Rel-15/Rel-16, for serving cell, all other signals and channels except for SSB could not map into the PRBs occupied by SSB, and only PDSCH or CSI-RS for beam management which is QCLed with a SSB can be transmitted in the same symbol as the SSB.  The motivation is straightforward and also important to ensure access performance for the cell. However, there is no any resource mapping restriction when referring to non-serving cell SSB. Thus, in our opinion, it seems there is no need to be rate-matched around non-serving cell SSB for all the signals/channels in the serving cell. Likewise, PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info also should not be rate matched around serving cell SSB.

Proposal 1:  For inter-cell multi-TRP operation, PDSCH/PDCCH from the serving cell should not be rate-matched around non-serving cell SSB.

Proposal 2: For inter-cell multi-TRP operation, PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is not rate matched around serving cell SSB.

Conclusion

In this paper, we provide our views on Multi-TRP inter-cell operation, and have the following proposals and observations.

Proposal 1:  For inter-cell multi-TRP operation, PDSCH/PDCCH from the serving cell should not be rate-matched around non-serving cell SSB.

Proposal 2: For inter-cell multi-TRP operation, PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is not rate matched around serving cell SSB.

Reference

- RP-202024, Further enhancements on MIMO for NR