3GPP TSG RAN WG1 #107-e		R1-2111120

e-Meeting, November 11th – 19th, 2021

Agenda Item:     8.16.12

Source:	Spreadtrum Communications

Title:	UE features for R17 NR MBS

Document for:	Discussion and decision

Introduction

In this paper, we provide our views on UE features for R17 NR MBS, based on R1-2110587 [1], and a full UE feature list with tracked changes is provided in section 5.

Discussion

FG33-2:

In Rel-17 MBS, HARQ-ACK feedback is introduced for multicast, and ACK/NACK based HARQ feedback and NACK-only are included. The PUCCH resource for each UE in the group is orthogonal by gNB’s implementation, to improve the reliability. But in our understanding, HARQ-ACK feedback is not the basic operation of multicast, as we have done in Rel-16 sidelink PC5, and also like FG33-4. In general, multicast transmission is more reliability than unicast. In addition, slot-level repetition is also can be utilized for the reliability. From the perspective of UE, power cost can be saved if it doesn’t support to feedback. Thus, we think whether to support HARQ-ACK feedback respectively of feedback option is up to UE capability.

Proposal 1: Component 6 in FG33-2 can be split as one separated FG, and as one optional UE capability.

Proposal 2: Component 7 in FG33-2 can be split as one separated FG, and as one optional UE capability.

Proposal 3: Component 8 in FG33-2 can be split as one separated FG, and as one optional UE capability.

FG33-3-2:

Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 2 and 3 in FG33-3-2 as separated FG.

Proposal 4: Component 2 and 3 in FG33-3-2 can be split as separated FG, and as optional UE capability.

FG33-3-3:

Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 6 and 7in FG33-3-3as separated FG.

Proposal 5: Component 6 and 7 in FG33-3-3 can be split as separated FG, and as optional UE capability.

FG33-5-1:

Given the above, HARQ-ACK feedback is not the basic operation for multicast. Thus, we prefer to split component 2, 3 in FG33-5-1 as separated FG.

Proposal 6: Component 2, 3 in FG33-5-1 can be split as separated FG, and as optional UE capability.

Regarding repetition for SPS, we have the following agreement made in RAN1#106b-e:

Agreement:

For slot-level repetition for SPS GC-PDSCH for multicast RRC_CONNECTED UEs.

- Config A or Config B can be configured to UE:

  - (Config A) UE can be optionally configured with pdsch-AggregationFactor per SPS-Config-Multicast.

  - (Config B) UE can be optionally configured with TDRA table with repetitionNumber as part of the TDRA table in PDSCH-Config-Multicast. If UE is configured with Config B, UE does not expect to be configured with Config A for the same SPS group-common PDSCH.

- For Config A, if pdsch-AggregationFactor in SPS-Config-Multicast is not configured, default value is

  - Alt1: equal to 1.

Dynamic repetition number indication and semi-static repetition number indication for SPS group-common PDSCH are considered. It is similar to repetition indication for dynamic group-common PDSCH. In our mind, there is no necessary to have additional FG for SPS. We are fine that FG 33-3-1 is also applied for SPS group-common PDSCH.

FG33-5-2:

For the component 1 of FG33-5-2, if taking FG12-2 as reference, it shall be per CFR configuration, and further constraint on per cell or cell group can also be considered.

Proposal 7: Component 1 in FG33-5-2 can be revised as ‘Support up to [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group’.

New FGs:

Last meeting, we have the following agreement:

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions can be configured between the following alternatives from the previous agreement:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Support of Alt. 1 is a UE capability

Thus, the following FG is preferred to be added to reflect the agreement.

Proposal 8: Suggest to add one new FG to reflect the following.

- For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions as follows

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast

Conclusion

In this paper, we have shown our views on MBS UE feature. A full UE feature list in tracked changes is provided in section 5.

- 4  References

- R1-2110587, ”Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), Oct. 2021

- 5  MBS UE features

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-1 | Broadcast | - Support of group-common PDCCH/PDSCH with CRC scrambled by MCCH-RNTI. - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for broadcast. - Support of CORESET and common search space for broadcast. - Support of DCI format 1_0 with CRC scrambled with G-RNTI/MCCH-RNTI for broadcast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. - Support MCCH change notification indication via DCI. |  | Yes |  |  | Per Band | No | No |  |  | Optional without capability signalling |
| 33. NR_MBS | 33-2 | Dynamic scheduling for multicast | - Support of group-common PDCCH/PDSCH with CRC scrambled by G-RNTI. - Support of CFR configuration for multicast. - Support of CORESET and common search space configuration for multicast. - Support of DCI format 1_0 / 1_1 with CRC scrambled with G-RNTI for multicast. - Support of inter-slot TDM between unicast PDSCH and group-common PDSCH in different slots. FFS whether/how to separate the above capabilities from FG 33-2 |  | Yes |  |  | Per Band | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-1 | Support ACK/NACK based HARQ-ACK feedback | - Support ACK/NACK based HARQ-ACK feedback, and support enabling/disabling ACK/NACK based HARQ-ACK feedback configured by RRC signaling. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-2 | PTM retransmission for multicast | - Support PTM retransmission for multicast. | 33-2-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-3 | PTP retransmission for multicast | - Support PTP retransmission for multicast | 33-2-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-2-x | Multiple G-RNTIs for group-common PDSCHs | Capability on number of G-RNTI for groupcast FFS details. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-1 | Slot-level repetition for group-common PDSCH | - Support slot-level repetition for group-common PDSCH for multicast. FFS whether to split FG 33-1-1 into two FGs for semi-static slot-level repetition and dynamic slot-level repetition | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-2 | FDM-ed unicast PDSCH and group-common PDSCH | - Support FDM between one unicast PDSCH and one group-common PDSCH in a slot. FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-2-1 | HARQ-ACK codebook for FDMed unicast and multicast | - Support FDM-ed Type-1 HARQ-ACK codebook for multicast. - Support FDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-3-2 | Yes |  |  | Per UE | No | No |  | Component 1: {Support, not support} Component 2: {Support, not support} | Optional with capability signalling |
| 33. NR_MBS | 33-3-3 | Intra-slot TDM-ed unicast PDSCH and group-common PDSCH | - Support TDM between one unicast PDSCH and one group-common PDSCH in a slot. - Support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - Support TDM among N (N>1) group-common PDSCHs in a slot per CC - Support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - The UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s). FFS whether/how to separate the capability for HARQ-ACK codebook | 33-1, 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-3-3-1 | HARQ-ACK codebook for intra-slot TDM-ed unicast PDSCH and group-common PDSCH | 1. Support TDM-ed Type-1 HARQ-ACK codebook for multicast. 2.Support TDM-ed Type-2 HARQ-ACK codebook for multicast. | 33-3-3 | Yes |  |  | Per UE | No | No |  | Component 1: {Support, not support} Component 2: {Support, not support} | Optional with capability signalling |
| 33. NR_MBS | 33-4 | NACK-only based HARQ-ACK feedback for multicast | - Support NACK-only based HARQ-ACK feedback. | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1 | SPS group-common PDSCH for multicast | - Support one SPS group-common PDSCH configuration for multicast - FFS: HARQ-ACK feedback for SPS group-common with PDCCH scheduling and SPS release PDCCH. FFS whether/how to separate the above capabilities from FG 33-5-1 | 33-2 | Yes |  |  | Per Band | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1-1 | Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling | - Support ACK/NACK based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1-2 | Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling | - Support NACK-only based HARQ-ACK feedback for SPS group-common PDSCH without PDCCH scheduling. | 33-5-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration | - Support up to [N>1] SPS group-common PDSCH configuration per CFR for multicast, and up to K SPS group-common PDSCH configuration in a cell group - Support up to [M>=1] activated SPS group-common PDSCH configurations among the N SPS group-common PDSCH configurations per CFR for multicast | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-1 | DL priority indication for multicast in DCI | 1. Support of priority indicator field configured in DCI formats 1_1 with CRC scrambled with G-RNTI for multicast. FFS whether/how to separate the above capability from FG 33-6-1 | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-2 | Two HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities for multicast or for unicast and multicast at a UE. | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed different priorities for multicast or for unicast and multicast at a UE. | 33-6-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-6-3 | More than one PUCCH for HARQ-ACK transmission for multicast or for unicast and multicast within a slot | 1. Supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast or for unicast and multicast with different priorities in a slot. | 33-6-1, 33-6-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-7 | Supporting group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 1. Supports the function of group-common DCI indicating the enabling/disabling [ACK/NACK based] HARQ-ACK feedback | 33-2 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-8 | Determining PDSCH reception candidate occasions, for TDM-ed unicast and multicast , and for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource | - Supports to determine PDSCH reception candidate occasions as below, for TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets, - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast | 33-3-3, 33-3-3-1 | Yes |  |  | Per UE | No | No |  |  | Optional with capability signalling |