3GPP TSG RAN WG1 #107	R1-2111152

e-Meeting, 11 – 19 November, 2021

Agenda item:		8.16.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On UE features for further enhancements on NR-MIMO

Document for:		Discussion and Decision

# Introduction

In RAN1#106bis-e the first baseline has been agreed for a number of UE features. In this contribution we present our views on the UE features for further enhancements on NR-MIMO.

# Discussion

Detailed comments on UE features are listed below. For reference, the baseline agreements from RAN1#106bis are available in the Annex.

- 23-1-1:

    - Confirm the FG. Details to be further discussed.

- 23-1-2:

    - Confirm the FG. Details to be further discussed.

    - Add 2-24 and 2-29 as pre-requisites

  - 23-1-3:

    - Confirm the FG. Details to be further discussed.

- 23-2-1:

    - No independent signaling for component 1, as it is not allowed to individually disable components of an FG. No need to separate the FG though, support to component 1 is given by support to the FG itself.

- 23-2-2:

    - Clarify if FR2-only support is supposed to include FR2-2 as well.

- 23-2-3:

    - Confirm the FG. Details to be further discussed.

- 23-3-1:

    - Confirm the FG. Details to be further discussed.

    - Pre-requisites missing, including on CG-PUSCH.

  - 23-3-2:

    - Confirm the FG. Details to be further discussed.

  - 23-3-1-1:

    - Confirm the FG. Details to be further discussed.

  - 23-6-1:

    - Confirm the FG. Details to be further discussed.

  - 23-6-2:

    - Confirm the FG. Details to be further discussed.

  - 23-6-3:

    - Confirm the FG. Details to be further discussed.

  - 23-6-4:

    - Confirm the FG. Details to be further discussed.

- 23-7-2:

    - Confirm the FG. Details to be further discussed.

  - 23-7-3:

    - Confirm the FG. Details to be further discussed.

  - We do support 23-6-1a, 23-6-2a, and 23-9-1a

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# Conclusion

In this contribution we have presented our views on UE features for further enhancements on NR-MIMO, with the following proposal:

Proposal: Consider the observations and modifications proposed above for the next version of the corresponding RAN1 UE features list.

# References

- Chair’s Notes RAN1#106bis-e, Oct. 2021

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1 | Unified TCI for [intra- and inter-cell] beam management | [For both intra- and inter-cell beam management:] 1. Support of joint DL/UL TCI update [and separate DL/UL TCI update] with their components (configuration mechanism, QCL rules, applicable source and target signals) [and Common cross-CC TCI update and activation (involving RRC common TCI state pool)] 2. Support of association between TCI state and UL PC settings [for PUCCH, PUSCH, and SRS] [(PLRS and other, including handling of beam [alignment /misalignment] for PLRS)] 3. Supported mode of [MAC-CE/MAC-CE+DCI]-based beam TCI state indication [(including TCI state activation, use of DCI formats 1_1/1_2 with and without DL assignment)] [4. The maximum number of configured TCI state pools across all BWPs and all CCs in a band] [5. The maximum number of configured TCI states across all BWPs and all CCs in a band] [6. The maximum number of configured TCI states per CC in a band] [7. Support number of MAC-CE activated joint TCI states across all BWPs and all CCs in a band] [8. The maximum number of MAC-CE activated separate DL and UL TCI states across all BWPs and all CCs in a band] [9. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE [10. Support any DL RS that is a valid target DL RS of a Rel-15/16 TCI state based on the Rel-15/16 QCL rules can be configured as a target DL RS of Rel-17 DL TCI (hence the Rel-17 DL TCI state pool) [11. The minimum beam application time in Y symbols] [12. Support of association between a TCI state and PL-RS, where the “beam alignment” between the DL source RS in the UL or (if applicable) joint TCI state to provide spatial relation indication and the PL-RS is assumed by the UE] [12. The maximum number of PCI(s) different from serving cell PCI that can be associated with activated TCI states] |  |  |  |  | [ber band] |  |  |  | {Support, Not support} [4. candidate values {1, …, 32} 5. candidate values {64, 72, 80, 96, 128, 192, 256, 128*4*32} 6. candidate values {8, 16, 32, 64, 128, 256} 63. candidate values FFS {no, with DA, w/o DA, both} 7. candidate values {1, 2, 3, 4, 5, 6, 7, 8} 8. candidate values {1, 2, 3, 4,5,6,7, 8} Note 1: for component 7 & 8, if two TCI states are in one codepoint, it should be counted as 2.] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-2 | Inter-cell measurement and reporting (for inter-cell BM [and mTRP]) | 1. Support of [L1-RSRP/beam] measurement and report on SSB(s) with PCI(s) different from serving cell PCI 2. Support of [L1-RSRP/beam] reporting of up to Kmax=4 SSBRI-RSRP[,SS-RSRP] [pairs/beams], with at least one [pair/beam] associated with a PCI different from serving cell PCI [in one report instance] [3. The number of [RRC-configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR1] [4. The number of [RRC- configured/ MAC-CE activated] PCI(s) different from serving cell PCI for beam measurement] [in FR2] [5. The max number of SSB resources configured to measure L1-RSRP within a slot across all PCI(s) different from serving cell PCI] [6. The max number of SSB resources configured to measure L1-RSRP across all PCI(s) different from serving cell PCI] [7. Support on that SSB(s) with PCI(s) different from serving cell PCI configured for L1 beam measurement and report are not included in SSBs with PCIs configured for L3 mobility measurement] | [2-24, 2-29] |  |  |  |  |  |  |  | {Support, Not support} [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-3 | MPE mitigation | 1. Support of enhanced reporting for MPE mitigation (with SSBRI/CRI and [associated] P-MPR in PHR reporting) [2. Support of reporting up to Nmax P-MPR and SSBRI/CRI pairs] [3. Maximum number of candidate RS(s) configured in a RRC pool for MPE mitigation] |  |  |  |  |  |  |  |  | {Support, Not support} [2. Candidate value of {1,2,4}] [3. Candidate value at least includes {64}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1 | [Multi-TRP] PDCCH repetition | 1. Support of PDCCH repetition (based on two linked SS sets associated with corresponding CORESETs) [with non-SFN scheme TDM and FDM] [including PDCCH repetition for Type 3 CSS] 2. Support of reporting one [or more] number(s) as required number of BDs for the two PDCCH candidates 3. [If 3 or {2, 3} is reported in component 2, support of whether the individual candidate is monitored or not when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 3 BDs] [If 2 or {2,3} is reported in component 2, the individual candidate is monitored when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET for the case that the linked PDCCH candidates is 2 BDs.] FFS: 4. Maximum number of such overlaps in 3 Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduling scheduled component carrier |  |  |  |  |  |  |  |  | Component1: details FFS {Support, Not support} Component2: details FFS {2, 3, [{2,3}]} Component3: details FFS {monitored, not monitored} Component4: details FFS [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for [CORESET monitoring in] PDCCH [repetition] | Support of determining two QCL-TypeD for overlapping CORESETs in the same CC or for intra-band CA [when UE is configured with PDCCH repetition] | [23-2-1, 23-6-1, 23-6-2] |  |  |  |  |  |  | FR2 only | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A and B) [2. Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A and B,] [3. Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2] [4. Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.)] [5. Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.)] [6. support of sequential mapping for single for single DCI based M-TRP PUSCH repetition Type A] [7. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] [8. Support of A-CSI report on two PUSCH repetitions] [9. Support of SP-CSI report on two PUSCH repetitions] [10: Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission] [11. Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) FFS: discuss details (to be discussed after type A is stable_ |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) 2. Support of PUCCH repetition scheme 3 (intra-slot repetition) 3. Support of cyclic mapping for beam mapping/power control parameter set mapping [when the number of repetitions is larger than 2] for PUCCH repetitions scheme 1 and 3 when the number of repetitions is larger than 2 4. Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats [1_0] / 1_1 / 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-4 | IntCell-mTRP | 1. Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info including other cell info in RRC TCI state config [2. Support of X>1 (max number of PCIs different from serving cell)] [3. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR1/case1] (X1)] [SSB time domain positions or periodicity of additional PCIs is not exactly the same as serving cell PCI)] [4. Supported max number of RRC-configured PCIs different from serving cell PCI for [FR2/case2] (X2)] [SSB time domain positions and periodicity are exactly the same among the additional PCIs and the same as serving cell PCI] [5. default case to be supported, e.g., case2 with X2=1] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP Beam reporting enhancements | - Support a single L1-RSRP reporting instance consisting of N of the maximum number of beam groups and M=2 beams per [pair/group] [in one report instance] - [Support of enhanced group-based reporting for Rel-17 M-TRP] - [Maximum number of SSB and CSI-RS resources for measurement in both CMR sets within a slot across all CCs] - [Maximum number of configured SSB and CSI-RS resources for measurement in both CMR sets across all CCs] - [Support beam measurement on two CMR resource sets] - [Support of enhanced group-based reporting for Rel-17 intra-cell and inter-cell mTRP] |  |  |  |  |  |  |  |  | Candidate values: {1,2,3,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2 | Maximum number of BFD-RS resources per set for MTRP BFR enhancements | - Support of the maximum number of BFD-RS resources per set - [Support of Rel-17 M-TRP BFR based on two BFD-RS sets] - [Maximum number of CSI-RS resources of both BFR-RS sets across all CCs] - [Maximal number of different SSBs of both BFD-RS sets across all CCs] - [Maximal number of different CSI-RS and/or SSB resources of both NBI-RS sets across all CCs] - [Support up to two PUCCH-SR resources for MTRP BFRQ] - [Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP] - [Support of [single DCI/multi-DCI] based BFR enhancement] |  |  |  |  |  |  |  |  | [Candidate values: {1, 2, …}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) | 1. [Support of scheme A for PDCCH] [and default QCL assumption with two TCI states for PDCCH] 2. Support of scheme A for PDSCH [scheduled by [single TRP/ Scheme A] PDCCH] [and default QCL assumption with two TCI states for PDSCH] 3. Support of dynamic switching between single-TRP and SFN PDSCH scheme A for DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1a | Dynamic switching - scheme A | Support of dynamic switching between single-TRP and SFN PDSCH scheme A by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) | - [Support of scheme B for PDCCH] and PDSCH - Support of scheme B for PDSCH [scheduled by [single TRP/Scheme B] PDCCH] - Support of dynamic switching between single-TRP and SFN PDSCH scheme B for by TCI state field in DCI formats 1_1, 1_2 | [23-6-1] |  |  |  |  |  | [FR1 only] |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2a | Dynamic switching - scheme B | Support of dynamic switching between single-TRP and SFN PDSCH scheme B by TCI state field in DCI formats 1_1, 1_2 |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-6-3 | Simultaneous activation of two TCI states for PDCCH across multiple CCs [(HST/URLLC)] | Support of simultaneous activation of two TCI states [for CORESETs with the same CORESET ID] in all BWPs across a set of configured component carriers [by single MAC-CE] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-4 | Default beam setup [for HST] | 1. Support of PDSCH reception using default beam for Rel-17 enhanced SFN scheme when PDSCH is scheduled with offset less than threshold FFS: 2. Support PDSCH reception using default beam for Rel-17 enhanced SFN scheme when TCI field is not present in DCI [when PDSCH is scheduled with offset equal or larger than the threshold] 3. Support aperiodic CSI-RS reception using default beam for Rel-17 enhanced SFN scheme when scheduling offset is less than threshold FFS: 4. Support of single-TRP PUCCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 5. Support of single-TRP PUSCH transmission using default beam when enhanced SFN PDCCH transmission scheme is configured FFS: 6. Support of single-TRP SRS resource transmission using default beam when enhanced SFN PDCCH transmission scheme is configured |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of Maximum number of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Nmax=1 - FFS: Support of Maximum number of NZP CSI-RS resources per CMR group: Ks,max=[2,4] - FFS others (supported options; values for X, codebook types, number of ports of CMRs, CMR sharing among NCJT and sTRP measurement hypotheses for FR1, two CMR groups with Ks=K1+K2 CMRs in CSI-RS resource set, reporting of 2 PMIs, 2 RIs and 2 Lis for NCJT measurement hypothesis …) | FFS |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-2 | Support of max # of Tx ports [per source/across two CMRs] [in a resource set for Multi-TRP CSI] [and max # resources] | [A list of supported combinations, each combination is {max # of Tx ports per source in a resource set for Multi-TRP CSI, max # resources in a resource set for Multi-TRP CSI}] {24 ports, 32 ports} [Note: same number of ports among CMRs] |  |  |  |  |  |  |  |  | {Support, Not support} [{4, 8, 12, 16, 24, 32}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-3 | Support of max # More than two resources in a resource set for Multi-TRP CSI | FFS exact candidate values, Ks,max is up to 8 |  |  |  |  |  |  |  |  | {Support, Not support} [candidate values are {2, 3, 4, 5, 6, 7, 8}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-4 | Support of Nmax=2 for Multi-TRP CSI | [Support of maximum number 2 pairs of CMR pairs Nmax=2 resources configured in a CSI-ReportConfig for Multi-TRP CSI measurement for CSI Enhancement for Multi-TRP] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-5 | Whether two CMRs from a CMR pair configured for a NCJT measurement hypothesis can be used for Single-TRP measurement hypotheses CMR sharing | [Support a NZP CSI-RS resource referred by both a CMR pair configured for NCJT measurement hypothesis and a CMR configured for Single-TRP measurement hypothesis] [FR2 only] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | Support of determining aperiodic SRS location based on available slot [and dynamically based on Rel-17 introduced new field in DCI Format 0_1/0_2/1_1/1_2] |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-2 | Triggering SRS only in DCI 0_1/0_2 | Support of triggering SRS in DCI 0_1/0_2 without data and without CSI |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-3 | SRS Antenna switching for >4Rx | 1. Support of SRS antenna switching xTyR with y>4 [2. Report whether the uplink TX switching impact to downlink receiving in a band] [3. Report whether the UL Tx is switched together with UL Tx in another band] |  |  |  |  |  |  |  |  | [{1T6R, 1T8R, 2T6R, 2T8R, 4T6R, 4T8R}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-4 | Maximum 2 SP [and 1 periodic] SRS sets for antenna switching | Support of maximum [2] [SP] SRS resource sets [and maximum 1 periodic SRS resource set] for antenna switching |  |  |  |  |  |  |  |  | {Support, Not support} [Note1: - Applies for all supported xTyR where y<=8 - For xTyR where y>4, if UE does NOT support this feature, support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - For xTyR where y<=4, if UEs that do not support this feature, follow Rel-15 on the number of resource sets for periodic and semi-persistent SRS] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-5 | Increased repetition for SRS | Support of increased repetition patterns (8, 10, 12, 14 symbols) for SRS resource |  |  |  |  |  |  |  |  | {Support, Not support} [(N_symbol, R) = {(8, 1), (8, 2), (8, 4), (8, 8), (12, 1), (12, 2), (12, 3), (12, 4), (12, 6), (12, 12), (10, 1), (10, 2), (10, 5), (10,10), (14, 1), (14, 2), (14, 7), (14, 14)}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-6 | Partial frequency sounding of SRS | Support of partial [(RB level)] frequency sounding for SRS |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-7 | Start RB location hopping for partial frequency SRS | Support of start RB location hopping in partial [(RB level)] frequency sounding SRS transmission [across different SRS frequency hopping periods for periodic/semi-persistent SRS] | [23-8-6] |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-8-8 | Comb-8 SRS | Support of comb-8 for MIMO SRS other than for positioning |  |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-1 | Basic Features of Further Enhanced Port-Selection Type II Codebook (FeType-II) | - [{Max # of Tx ports in one resource, Max # of resources and total # of Tx ports}] to support Port-selection FeType-II with Mv=1 - Support rank 1,2 - Support parameter combinations with Mv=21 - FFS others | FFS |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-1a | Codebook concurrency of Type I codebook and Rel-17 FeType II codebook | Supported mixed codebook types | [23-9-1] |  |  |  |  |  |  |  | [Codebook 1 = {Type I SP, Type I MP} {Codebook 2, Codebook 3} = {{FeType II PS, NULL}, {Type II, FeType II PS },{eType II R=1, FeType II PS }}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-2 | Support of Mv=2 for FeType-II | [1. Support rank 1,2] 2. Support parameter combinations with Mv=2 [3. {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support Port-selection FeType-II with Mv=12] | [23-9-1] |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-9-3 | Support of rank 3, 4 for FeType-II | Support of rank 3, 4 for FeType-II | [23-9-1, 23-9-2] |  |  |  |  |  |  |  | {Support, Not support} | Optional with capability signalling |