3GPP TSG-RAN WG1#107-e	Tdoc R1-2111192

e-Meeting, November 11-19, 2021

Agenda Item:	8.16.3

Source:	Ericsson

Title:	Rel-17 UE Features for IIoT/URLLC

Document for:	Discussion, Decision

# Introduction

In this contribution, Rel-17 UE Features for IIoT/URLLC are discussed. For each enhancement aspect, the candidate UE features are proposed in each section below.

# Discussion

## UE Feedback Enhancements for HARQ-ACK

### SPS HARQ-ACK deferral

[FL4] Medium priority question 2-2:

- Companies are encouraged to provide views on whether the type of FG 25-1 should be per UE or per band or per FSPC

| Ericsson | Per UE. It is not clear to show band differentiation would impact the feature and the corresponding testing. Also, considering RAN2 recommendation, FSPC should be avoided as much as possible (R2-2002378) |
| --- | --- |

Adopt “Per UE” type for FG 25-1.

### PUCCH repetition enhancements

In the following we address the remaining issues that were recommended by FL to be addressed in RAN1#107-e meeting [3].

[FL1] High priority question 3-2:

- Companies are encouraged to provide views on whether to split FG 25-3 and 25-3a per UCI type supported with sub-slot repetitions

| DOCOMO | We don’t think separate capabilities are needed for FG25-3 and 25-3a. Firstly, it has not been agreed that sub-slot based PUCCH repetition is supported for UCI types other than HARQ-ACK. Secondly, even if the UCI types are also supported, there should be no additional UE complexity because the same UE behavior on sub-slot based PUCCH repetition is applied to any UCI types. On the other hand, if only HARQ-ACK is supported in the end, it is fine to just add a note that it is only applicable to PUCCH carrying HARQ-ACK. |
| --- | --- |
| Nokia, NSB | No need to split the FGs. In addition 25-3a could be removed assuming a UE indicating support for dynamic PUCCH repetition indication (30-5) and sub-slot based PUCCH repetition (25-3) would then also support dynamic repetition indication for sub-slot based PUCCH (25-3a). |
| Ericsson | Agree with others. No need for split FGs. |

Do not split FG 25-3 and 25-3a per UCI type.

[FL4] Medium priority question 3-4:

- Companies are encouraged to provide views on whether the type of FGs 25-2 to 25-3a should be per UE or per FS or per FSPC

| Ericsson | Per UE. Same comment as before: It is not clear how band differentiation would impact the feature and the corresponding testing. Also, considering RAN2 recommendation, FSPC should be avoided as much as possible (R2-2002378) |
| --- | --- |

Adopt “Per UE” type for FG 25-3 and 25-3a.

[FL4] Low priority question 3-5:

- Companies are encouraged to provide views on whether/how to revise the prerequisite feature groups for FGs 25-2 to 25-3a

| Ericsson | - For 25-2, remove dependency of 4-23. FG 4-23 is for PF1/3/4, not for PF0/2 - For 25-3, remove dependency of 4-23. FG 4-23 is for slot based repetition, and not necessary condition for sub-slot based repetition. |
| --- | --- |

For FG 25-2, remove prerequisite of 4-23. FG 4-23 is for PF1/3/4, not for PF0/2

For FG 25-3, remove prerequisite of 4-23. FG 4-23 is for slot based repetition, and not necessary condition for sub-slot based repetition

### Semi-static HARQ-ACK CB for sub-slot

[FL4] Medium priority question 5-2:

- Companies are encouraged to provide views on whether the type of FG 25-8 should be per UE or per band or per FSPC

| Ericsson | Per UE. Same comment as before: It is not clear how band differentiation would impact the feature and the corresponding testing. Also, considering RAN2 recommendation, FSPC should be avoided as much as possible (R2-2002378) |
| --- | --- |

Adopt “Per UE” type for FG 25-8.

### PUCCH carrier switching

[FL4] Medium priority question 6-3:

- Companies are encouraged to provide views on whether the type of FGs 25-9 and 25-10 should be per UE or per BC or per FS

| Ericsson | Per UE. Same comment as before: It is not clear how band differentiation would impact the feature and the corresponding testing. Also, considering RAN2 recommendation, FSPC should be avoided as much as possible (R2-2002378) |
| --- | --- |

Adopt “Per UE” type for FG 25-9.

### Enhanced Type-3 HARQ-ACK codebook

[FL4] Medium priority question 4-3:

- Companies are encouraged to provide views on whether the type of FGs 25-4 to 25-7 should be per UE or per band or per FSPC

| Ericsson | Per UE. Same comment as before: It is not clear how band differentiation would impact the feature and the corresponding testing. Also, considering RAN2 recommendation, FSPC should be avoided as much as possible (R2-2002378) |
| --- | --- |

Adopt “Per UE” type for FG 25-4 to 25-7.

## CSI Feedback Enhancements

For FG 25-11 in the preliminary list [1], no prerequisite FG is stated. However, the 4-bit subband CQI reporting should be an enhancement of basic CSI feedback. Hence FG 2-32 “Basic CSI feedback” should be added as a prerequisite as shown below.

Additionally, the 4-bit subband CQI should be confirmed as a “per UE” capability.

Table 1: Proposed FG for enhanced CSI feedback.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-11 | 4-bits subband CQI | Subband CQI reporting with 4 bits per subband | 2-32 |

Add FG 2-32 “Basic CSI feedback” as a prerequisite  for FG 25-11 “4-bits subband CQI”.

Confirm FG 25-11 “4-bits subband CQI” as a “Per UE” capability.

## Enhancements for Unlicensed Band URLLC/IIoT

The remain issue with respect to UE features for operation on shared spectrum was whether to update the component description of FG 25-12 as the following (Details available at [FL4] Low priority question 8-5 in [3] )

Table 2: FG 25-12 with proposed updates for component description

| 25-12 | UE initiating a semi-static channel occupancy with configurations dependent on gNB semi-static channel access configurations | 1. Support initiating a semi-static channel access occupancy by the UE where the corresponding period is the same as, integer multiple of, or inter-factor of the period configured for a semi-static channel occupancy that can be initiated by gNB. 2. 9us sensing to initiate a semi-static CO or transmit after [a gap greater than 16us from] any transmission burst within a UE-initiated CO. 3. Determination of COT initiator assumption based on rules for configured UL 4. Validating COT initiator assumption indicated in UL scheduling DCI |
| --- | --- | --- |

From our perspective, the proposed additions are important to be included and tested to ensure proper intra-operability.

Adopt the proposed changes in red in Table 2 for FG 25-12.

## Intra-UE Multiplexing/Prioritization

For the topic of simultaneous PUCCH/PUSCH for CA, currently it has been agreed to support it for inter-band CA. For intra-band CA, it’s still FFS. Based on companies’ input, the main concern of supporting simultaneous PUCCH/PUSCH for intra-band CA is phase discontinuity if the PUCCH and PUSCH do not start and end at the same times. Phase discontinuity has been discussed since Rel-15. FG 6-23 was introduced for UE to indicate the incapability of PA phase discontinuity. As shown below, FG 6-23 can cover overlapping PUSCH-PUSCH as well as overlapping PUCCH-PUSCH. With this understanding, simultaneous PUCCH/PUSCH for intra-band CA should be supported for UEs that do not indicate the incapability FG 6-23. Thus we recommend to introduce the FG 25-18a, as shown in Table 5 below, with the note clarifying that FG 25-18a is applicable to UEs capable of handling PA phase discontinuity.

| 6-23 | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP |  | pa-PhaseDiscontinuityImpacts |
| --- | --- | --- | --- | --- |

Table 3: Proposed FG for simultaneous PUCCH/PUSCH for intra-band CA.

| Features | Index | Feature group | Components | Prerequisite feature groups | Note |
| --- | --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | 25-18a | Parallel PUCCH and PUSCH transmission across CCs in intra-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells for intra-band CA. | 6-5 | Applicable to UEs not indicating FG 6-23 |

For FG 25-14 and 25-15 in R1-2108679, the description should be aligned to avoid any confusion that there are any differences other than the LP vs HP of DG-PUSCH vs CG-PUSCH. Thus we recommend the editorial changes in Table 4. Furthermore, [25-14] and [25-15] should be merged into one feature group, with description “PHY prioritization of overlapping DG-PUSCH and CG-PUSCH of different PHY priorities”

For FG 25-18, FG 6-5 “Basic DL NR-NR CA operation” should be added as a prerequisite FG. Also “[at least]” can be deleted, since FG 25-18 describes inter-band CA only, i.e., intra-band CA is not a concern here.

Table 4: Proposed FG for Intra-UE Multiplexing/Prioritization.

| Features | Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- | --- |
| 25. NR_IIOT_URLLC_enh | [25-14] | HY prioritization of overlapping DG-PUSCH and CG-PUSCH of different PHY priorities PHY prioritization of overlapping low-priority DG-PUSCH and high-priority CG-PUSCH | Support PHY prioritization of overlapping for the case where low-priority DG-PUSCH and collides with high-priority CG-PUSCH on a BWP of a serving cell | 12-1 |
| 25. NR_IIOT_URLLC_enh | [25-15] (merge with [25-14]) | PHY prioritization of overlapping high-priority DG-PUSCH and low-priority CG-PUSCH | Support PHY prioritization of overlapping high-priority dynamic grant DG-PUSCH and low-priority configured grant CG-PUSCH on a BWP of a serving cell | 12-1 |
| 25. NR_IIOT_URLLC_enh | 25-18 | Parallel PUCCH and PUSCH transmission across CCs in inter-band CA | Support simultaneous PUCCH/PUSCH transmissions on different cells [at least] for inter-band CA. | 6-5 |

FG 25-18a is added for simultaneous PUCCH/PUSCH in intra-band CA.

Merge [25-14] and [25-15] into a single feature group with description “PHY prioritization of overlapping DG-PUSCH and CG-PUSCH of different PHY priorities”

Adopt the editorial changes to Components for FG [25-14], [25-15], 25-18.

## Propagation Delay Compensation

For the topic of propagation delay compensation, progress has been made in RAN1 discussion that enhancements are needed for UEs to provide time synchronization for IIoT use cases. In RAN2#115e, the following agreement was made:

Agreements (RAN2#115e)

1.	RAN2 assumes that gNB can perform pre-compensation.  RAN2 agrees to introduce signalling to enable/disable UE-side PDC.

2.	The gNB can enable/disable UE-side PDC via unicast-RRC signalling for Rel-17

Thus an FG needs to be introduced for such UEs. An example is shown in Table 7 below. The details of the FG can be updated once more design details are developed in RAN1 discussion, for example, to support TA-based method or RTT-based method.

A feature group is introduced for supporting propagation delay compensation in Rel-17.

Table 5: Exemplary FG for propagation delay compensation.

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation delay compensation | Support propagation delay compensation for time synchronization of the Uu interface | N/A | Yes | Per UE | No | No | N/A | Optional with capability signalling |

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt “Per UE” type for FG 25-1.

Proposal 2	Do not split FG 25-3 and 25-3a per UCI type.

Proposal 3	Adopt “Per UE” type for FG 25-3 and 25-3a.

Proposal 4	For FG 25-2, remove prerequisite of 4-23. FG 4-23 is for PF1/3/4, not for PF0/2

Proposal 5	For FG 25-3, remove prerequisite of 4-23. FG 4-23 is for slot based repetition, and not necessary condition for sub-slot based repetition

Proposal 6	Adopt “Per UE” type for FG 25-8.

Proposal 7	Adopt “Per UE” type for FG 25-9.

Proposal 8	Adopt “Per UE” type for FG 25-4 to 25-7.

Proposal 9	Add FG 2-32 “Basic CSI feedback” as a prerequisite  for FG 25-11 “4-bits subband CQI”.

Proposal 10	Confirm FG 25-11 “4-bits subband CQI” as a “Per UE” capability.

Proposal 11	Adopt the proposed changes in red in Table 2 for FG 25-12.

Proposal 12	FG 25-18a is added for simultaneous PUCCH/PUSCH in intra-band CA.

Proposal 13	Merge [25-14] and [25-15] into a single feature group with description “PHY prioritization of overlapping DG-PUSCH and CG-PUSCH of different PHY priorities”

Proposal 14	Adopt the editorial changes to Components for FG [25-14], [25-15], 25-18.

Proposal 15	A feature group is introduced for supporting propagation delay compensation in Rel-17.

# References

R1-2108679		Preliminary RAN1 UE features list for Rel-17 NR, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#106-e, August 2021.

R1-2110587		Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e	Moderators (AT&T, NTT DOCOMO, INC.)

R1- 2109709	Summary on UE features for enhanced IIoT and URLLC, Moderators (NTT DOCOMO, INC.), RAN1#106bis-e, October 2021.