3GPP TSG RAN WG1 Meeting #107-e		                                                      R1-2111230

e-Meeting, October 11th – 19th, 2021

Source:	CATT

Title:	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS

Agenda Item:	8.12.1

Document for:	Discussion and Decision

Introduction

According to the agreements that achieved in RAN1#106b-e meeting [1], this contribution discusses the following items for group scheduling mechanism for RRC_CONNECTED UEs:

- MBS Common frequency resource (CFR)

- Initial transmission and retransmission(s)

- SPS group-common PDSCH

- CORESET/SS/DCI design for NR MBS

- Blind decoding

- Simultaneous reception of unicast and multicast

Discussion

Common frequency resource

    - CFR configuration

For receiving MBS, a CFR can be confined within more than one BWP when more than one BWP are configured. For example, 4 dedicated unicast BWP are configured, and a CFR is defined in BWP1/2/3 to receive MBS, as shown in Figure 2.1-1. For the BWP that is not associated with a CFR (e.g. BWP4), UE will switch the active BWP to a certain BWP (e.g. BWP1, BWP2, and BWP3) which associated with a CFR to receive the MBS. In other words, MBS cannot be scheduled in a dedicated unicast BWP when the BWP is not associated with the CFR.

Figure 2.1-1: CFR is defined in BWP1, BWP2, and BWP3

Regarding the ‘CFR associated with UE active BWP’, two configuration options can be considered, as shown in Figure 2.1-2.

Figure 2.1-2: The configuration cases when CFR is associated with UE BWPs

- Option 1

In this option, when the CFR is defined in BWP1/2/3 as discussed above, the CFR configuration (i.e., cfr-Config-Multicast) will be configured for each BWP. In other words, the CFR configuration (i.e., cfr-Config-Multicast) will be configured more than one time when a CFR is associated with multiple BWP(s) as shown in Figure 2.1-2 (a).

- Option 2

In this option, the CFR configuration (i.e., cfr-Config-Multicast) is configured separately with BWP configuration. Each dedicated unicast BWP is associated with one CFR configuration (i.e., cfr-Config-Multicast). The CFR configuration needs to be associated with one dedicated unicast BWP by index, such as cfr-index, to indicate the correlation of  dedicated unicast BWP  and the CFR,  just as shown in Figure 2.1-2(b). When one of the CFR is associated with multiple BWP(s), the CFR configuration only needs to be set once for the UE. Furthermore, the gNB only needs to reconfigure the CFR once when it is need to be updated.

Comparing with Option 1, Option 2 can save RRC signaling. Thus, Option 2 is more preferred, in which the CFR configuration is associate with one dedicated unicast BWP through a CFR index.

Proposal 1: The CFR configuration (i.e., cfr-Config-Multicast) is configured separately with BWP configuration, and the CFR configuration can be associated with the dedicated unicast BWP via a CFR index.

If the new IE CFR-Config-Multicast is not configured for the active BWP, the UEs don’t know the multicast reception configuration corresponding to the active BWP, such as IE searchSpacesToAddModList-Multicast and searchSpace-Multicast. Thus, the UEs don’t know how/where to search for PDCCH candidates. In other words, the UE can’t perform multicast reception in the active BWP when the new IE CFR-Config-Multicast is not configured for the active BWP.

Proposal 2: The UE is not expected to receive multicast in the active BWP when the new IE CFR-Config-Multicast is not configured for the active BWP.

Initial transmission and retransmission(s)

    - LBRM/TBS determination for PTP retransmission of multicast

Agreement: Study the following options for the LBRM/TBS determination for PTP retransmission of multicast. - Option 1: based on the LBRM/TBS determination of the PTM initial transmission using same HPID and NDI. - Option 2: based on the LBRM/TBS determination of the legacy unicast PDSCH transmission.

It has been agreed that the LBRM/TBS (TBSLBRM) determination for GC-PDSCH and unicast PDSCH are configured separately by different higher layer parameters. The LBRM/TBS determinatio determination will have impact on the circular buffer and the starting points of RVs (Redundancy version). Specifically, when the LBRM (Limited buffer rate-mating) is enabled, the length of circular buffer Ncb is determined by the minimum value between the length of a coded block N and the length of limited buffer NLBRM, which can be denoted as  Ncb=min(N,NLBRM) . And the length of limited buffer NLBRM  can be derived by NLBRM=TBSLBRM/C×RLBRM, where  TBSLBRM is configured by higher layer parameters and RLBRM=2/3. For the starting points of RVs, it can be determined by Ncb and the lifting size ZC according to Table 1 given by Clause 5.4.2.1 in 38.212. For a certain RV, different values of N_cb will bring different starting positions.

Table 1: Starting position of different RVs,

|  | LDPC base graph 1 | LDPC base graph 2 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

For the LBRM/TBS determination for PTP retransmission of multicast, two options were discussed in the last RAN1 meeting. Option 1 is that using the LBRM/TBS determination of the PTM initial transmission with same HPID and NDI as that of PTP retransmission of multicast. The UE needs to differentiate PTP transmission of unicast and PTP retransmission of multicast. Regarding Option 2, the LBRM/TBS determination for PTP retransmission of multicast is determined by the higher layer parameters of unicast. It means that the values of LBRM/TBS for PTM initial transmission and PTP retransmission of multicast are determined by different higher layer parameters. In our view, the length of LBRM/TBS for PTM transmission depends on the lowest UE capability in one MBS group. Thus, the length of LBRM/TBS for PTP transmission is no more than PTM transmission. When the length of coded bits for a TB is larger than the length of limited buffer for PTM transmission, the starting point for a certain RV is different. If the UE requires to soft-combine the PTM initial transmission and PTP retransmission, an offset is needed to ensure adding the received coded bits to the correct position of buffer. Compared with option 1, option 2 is more complexity due to the computation of the offset of the buffer. Therefore, Option 1 is more reasonable.

Proposal 3: For the LBRM/TBS determination for PTP retransmission of multicast, option1 (based on the LBRM/TBS determination of the PTM initial transmission using same HPID and NDI) is preferred.

    - Retransmission schemes

It was agreed that PTM scheme 1 and PTP can be used as retransmission schemes when PTM scheme 1 is used as initial transmission. When PTM scheme 1 and PTP are simultaneously used as retransmission schemes for different UEs in the same MBS group, a duplicated reception may happen. PTM scheme 1 targets a group UEs, and all UEs in this group will receive the same TB. Apart from receiving the TB that re-transmitted by PTM scheme 1, some UE will also receive the same TB which re-transmitted by PTP when PTM scheme 1 and PTP are simultaneously used as retransmission schemes. It is noted that at this scenario the PTM scheme 1 and PTP as retransmission scheme share a HPID with un-toggled NDI. How to manage the HARQ-ACK feedback and the HARQ-ACK combination for the duplicated receptions needs to be further discussed. And the extra impact for the current specification should also be considered. This will bring extra specification work and is unnecessary. Therefore, simultaneous utilization of PTM scheme 1 and PTP retransmission for different UEs in the same MBS group is not supported.

Proposal 4: For retransmission scheme, PTM scheme 1 retransmission and PTP retransmission cannot be used simultaneously for different UEs in the same MBS group.

    - HARQ process management

An issue was raised in RAN1#104-e meeting that for a given HARQ process number whether the UE is expected to receive a new TB before the end of the expected transmission of HARQ-ACK of the previous TB. For example, when a UE doesn’t receive the TB#1 successfully and feedback NACK information, the gNB will retransmit the TB#1 with same HPID. If this HPID is used for transmitting a new TB#2 before the completion of the HARQ process, the UE may combine the TB#1 and TB#2 in one soft buffer mistakenly. Therefore, for a given HARQ process number, a UE is not expected to receive a new TB with the same HPID before the completion of the HARQ process of a previous TB.

Proposal 5: For a given HARQ process number, a UE is not expected to receive a new TB with the same HARQ process number before the completion of the HARQ process of a previous TB.

[High] Question 3-1a: Regarding the NDI conflict issue that different UEs in a group may have different NDI values for a certain HPID before performing an initial PTM transmission, which option should be adopted? - Option 1: Rely on gNB implementation to avoid such issue. - Option 2: Resolve this issue with potential specification enhancement.

In RAN1#104bis-e meeting, it has been reached the conclusion that how to allocate HARQ processes between unicast and multicast is up to gNB. In the conclusion, it is not limited that some value of HPID can only be used for MBS, e.g. HPID of 8-12 is only for MBS.

Thus, for Option 1, the NDI conflict issue can’t be completely avoided. Before the MBS happens, the gNB doesn’t know which UEs will be scheduled for the multicast reception and when the multicast service will be scheduled. The HPID will be fully utilized for the unicast service. Once the HPID of the TB received by different two UEs is the same before the MBS reception, the NDI conflict issue may happen. From this perspective, option 2 is a better choice to solve the NDI issue by potential specification enhancement.

Proposal 6: The NDI conflict issue needs to be solved though potential specification enhancement.

[High] Question 3-1b: If the answer is Option 2 for question 3-1a, which option do you prefer for the specification enhancement? - Option 1: When a G-RNTI DCI is received with a given HPID in the DCI, the data shall be considered new, i.e. be treated as if the NDI bit had been toggled, irrespective of actual NDI toggling, if the G-RNTI is different from the most recent earlier received RNTI (i.e. C-RNTI or another G-RNTI) of the same HPID. When the received G-RNTI is the same as the most recent use of the HPID, legacy NDI toggling is used to indicate new data or retransmission. - Option 2: Irrespective of earlier used RNTIs for the HPID, NDI bit ‘0’ means new data transmission, NDI bit ‘1’ means retransmission.

In the last meeting, there are two potential methods were proposed to solve the NDI conflict issue. Option 1 is based on the legacy NDI rules. It compares the received RNTI and the most recent earlier received RNTI of the same HPID are the same or not to indicate retransmission or new data.

For Option 2, it is used NDI bit ‘0’ to indicate new data transmission and NDI bit ‘1’ to indicate retransmission. However, it can cause wrongly soft combining issue when two PTM transmissions use same HPID.  As is shown in Figure 2.2-1, HPID=2 is used for Multicast-TB1 and Multicast-TB2 at the same time. The UE misses the detection the G-RNTI DCI of Multicast-TB2, then the retransmission is scheduled by gNB. In this case, UE will combine two different TBs of multicast in the same HARQ buffer and cause decoding failure. Thus Option 1 is more preferred for NDI scheme in multicast.

Figure 2.2-1: The issue proposed by the NDI rule in option2

Proposal 7: For NDI scheme of multicast, Option 1(When a G-RNTI DCI is received with a given HPID in the DCI, the data shall be considered new, i.e. be treated as if the NDI bit had been toggled, irrespective of actual NDI toggling, if the G-RNTI is different from the most recent earlier received RNTI (i.e. C-RNTI or another G-RNTI) of the same HPID. When the received G-RNTI is the same as the most recent use of the HPID, legacy NDI toggling is used to indicate new data or retransmission.) is preferred.

Figure 2.2-2: Multicast and Unicast transmission with same HPID and NDI

It has been agreed that the same HPID and NDI can be used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB. However, for a given HPID and a given NDI, the miss detection of the PDCCH of PTM initial transmission may cause UE to incorrectly soft-combine the received TB with PTP retransmission for multicast and the received TB PTP (re)transmission for unicast in the HARQ buffer. For example, HPID=2 and NDI=0 are used as both unicast and multicast transmission and the PTP is applied as the retransmission after the PTM scheme 1 transmission in multicast transmission, as shown in Figure 2.2-1.  When missing detects the group-common PDCCH, UE will soft combine the unicast service data and the PTP retransmission multicast service data. This will cause performance degradation. To solve this issue, two options can be considered:

- Option 1: A DCI field

In this option, a DCI field can be introduced in the DCI of PTP transmission to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

- Option 2: TB size

In this option, UE can differentiate PTP (re)transmission for unicast and PTP retransmission for multicast based on different TB sizes. gNB will schedule the different TB sizes for  any two consecutive PTP transmissions (include PTP transmission and retransmission). Then, the different TB size can be applied to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

Proposal 8: Down-selection from following options to solve the issue that the UE incorrectly soft-combine the received TB with PTP retransmission for multicast and the received TB with PTP (re)transmission for unicast when the HPID and NDI of multicast transmission and unicast transmission is same and the group-common PDCCH was missed.

- Option 1: introduce a field in DCI 1_1/1_2 of PTP transmission to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

- Option 2: use the different TB size of unicast and multicast to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

Supporting SPS group-common PDSCH for MBS

    - SPS activation/deactivation

Agreement: (104bis-e) Confirm the working assumption: For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state, - At least group-common PDCCH is supported - FFS: Whether and how to address the missed activation and deactivation - FFS: Whether UE-specific PDCCH is supported for activation/deactivation

It was agreed in RAN1#104bis-e meeting that the group-common PDCCH can be used for activation/deactivation of SPS group-common PDSCH. The activation/deactivation mechanism can be further considered as follows:

- Activation: When group-common PDCCH is used to indicate the shared PUCCH resource (e.g. NACK-only based feedback), the group-common PDCCH can be used for activation of SPS for all the UEs in the same group. However, when the separate PUCCH resource is configured among a group UEs, it is not feasible to let the group-common PDCCH indicate the k1 and PRI value for each UE. In this case,   UE-specific PDCCH can be used to indicate PUCCH resource (e.g. ACK/NACK based feedback) per UE as well as activate SPS procedure(s). Thus, UE-specific PDCCH can also be supported to activate SPS.

- Deactivation: Group-common PDCCH is used for deactivation of SPS for all the UEs in the same group. No matter what PDCCH is used for activation, to release a SPS procedure for MBS service, one PDCCH containing the same deactivation message is enough for all the UEs in a group. The benefit of UE-specific PDCCH deactivating the SPS is not clear for us and we did not see any use case.

Proposal 9: UE-specific PDCCH can be also used for SPS activation for MBS for RRC_CONNECTED UEs.

    - SPS configuration for MBS

Agreement:(106b-e) The association between a G-CS-RNTI and a SPS-Config-Multicast is indicated by the activation GC-PDCCH for SPS GC-PDSCH, i.e., a value of the HARQ process number field in a DCI format indicates an activation for a SPS GC-PDSCH configuration for multicast with a same value as provided by sps-ConfigIndex in a SPS-Config-Multicast. Agreement (106-e) If a SPS-config for MBS is configured in CFR, one G-CS-RNTI is associated with the SPS-config. FFS: Multiple G-CS-RNTIs associated with one SPS-config

In the RAN1#106b-e meeting, it has been agreed the association between a G-CS-RNTI and a SPS-Config-Multicast is indicated by the activation GC-PDCCH for SPS GC-PDSCH, similar to the legacy method in Rel.16. An issue was raised in the meeting that whether the value of sps-ConfigIndex in SPS configuration of multicast can be the same as that of unicast. It should be noted the total number of SPS configuration supported by a UE in a BWP for multicast and unicast is up to 8. And the bitwidth of HARQ process number field in a DCI format is 4. The HARQ process number field in a DCI format can indicate up to 16 different values of sps-ConfigIndex. There is no need to support the value of sps-ConfigIndex in SPS configuration of multicast is same as that of unicast.

Another remaining issue is that whether to support multiple G-CS-RNTIs associated with one SPS-config. If the multiple G-CS-RNTIs associated with one SPS-config is supported, it will cause HPID conflict between the SPS PDSCHs which corresponds to different SPS activation with different G-CS-RNTIs. Therefore, multiple G-CS-RNTIs associated with one SPS-config should not be supported.

Proposal 10：The UE is not expected that the values of sps-ConfigIndex in SPS configuration of multicast and unicast are the same value.

Proposal 11：Multiple G-CS-RNTIs associated with one SPS-config should not be supported.

    - Retransmission of PDCCH activation of SPS group-common PDSCH

Agreement: (105-e) For reliability of the group-common PDCCH activation of SPS group-common PDSCH, support at least one of the following alternatives. - Alt 1: retransmit the activation command via group-common PDCCH. - Alt 2: retransmit the activation command via UE-specific PDCCH. - Alt 3: retransmit the activation command via MAC-CE. - FFS other details. - Note: Down-selection can take into account the HARQ-ACK feedback scheme for SPS activation

When gNB configures a downlink assignment for SPS group-common PDSCH, UE can obtain the information of periodicity/PUCCH resource ID for HARQ-ACK feedback. After receiving group-common PDCCH activation of SPS group-common PDSCH, the UE can know where to receive group-common PDSCH without PDCCH and how to feedback HARQ-ACK. For the case that UE doesn’t receive the activation command (e.g. UE misses detecting the DCI or UE joins a new multicast group due to the mobility), gNB should retransmit the activation command to the UE. Considering gNB has already transmitted the activation command and SPS PDSCH is ongoing, the retransmitted activation command shall not conflict with first activation command. Three alternatives are discussed as following.

- Alt 1: retransmit the activation command via group-common PDCCH

In Alt 1, the group-common PDCCH is used to retransmit the activation command. To void the confliction and align with the first activation command, the scheduling information of the retransmitted activation command shall satisfy the following conditions:

- SLIV is equal to SLIV of the first activation command;

- k1 is equal to k1 of first activation command;

- Slot number that PDSCH transmission is equal to slot number that UE determines which is based on the first activation command and periodicity configured by RRC.

Since the group-common PDCCH is agreed for activation of SPS group-common PDSCH, the group-common PDCCH can also be supported for retransmitting the activation command. However, a potential issue should be considered. When receiving the on-going SPS PDSCH, the UE which has successfully received the first activation command can also receive the retransmitted activation command via group-common PDCCH. Due to the simultaneous reception between the on-going SPS PDSCH and the retransmitted activation command, the UE need to feedback two HARQ-feedback processes when Type-2 HARQ-ACK codebook is configured. One HARQ-feedback process is for the on-going SPS PDSCH, another is for the SPS PDSCH of the retransmitted activation command. Thus, extra clarification is needed in the current specification.

- Alt 2: retransmit the activation command via UE-specific PDCCH

In Alt 2, the UE-specific PDCCH to retransmit the activation command is applied. To align with the first activation command, the scheduling information of retransmitted activation command shall be satisfied following conditions:

- Slot number that PDSCH transmission is equal to the value that UE determines based on the  first activation command and periodicity configured by RRC;

- SLIV is equal to SLIV of the first activation command.

There are two potential use cases for Alt 2. One use case is that the gNB transmits SPS activation command via group-common PDCCH to a group of UEs and only part of them fail to receive the SPS activation command, then the gNB only retransmits the SPS activation command via UE-specific PDCCH to these UE. Another use case is that UE wants to join the MBS reception while the SPS PDSCH transmission is on-going, then the gNB can retransmits the SPS activation command via UE-specific PDCCH to the UE. Compared with group-common PDCCH of SPS activation command, UE-specific PDCCH of SPS activation command is more flexible, and will not affect the UEs who have successfully received the SPS activation command. In additional, gNB can transmit the UE-specific PDCCH of SPS activation to a specific UE via narrow beam, which can improve the reliability of the retransmission.

- Alt 3: retransmit the activation command via MAC-CE

In Alt 3, MAC-CE is used to retransmit the activation command. In this alterative, a new MAC-CE should be defined and RAN2 will be involved. This will cause high specification complexity. Moreover, MAC-CE command will introduce at least 3ms delay because the MAC-CE command is not valid immediately after UE receives it. Thus, Alt 3 is not supported.

Proposal 12: Both Alt 1 and Alt 2 can be supported for reliability of the group-common PDCCH retransmitted activation of SPS group-common PDSCH.

  - Alt 1: retransmit the activation command via group-common PDCCH

  - Alt 2: retransmit the activation command via UE-specific PDCCH

    - Retransmission schemes of SPS group-common PDSCH

Agreement: (104bis-e) The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP. - FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group

It was agreed in RAN1#104bis-e meeting that the retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP. When PTM scheme 1 and PTP are simultaneously used as retransmission for different UEs in the same MBS group, a duplicated reception may happen. As discussed in section 2.2.1, the UE can’t deal with the duplicated receptions. Therefore, simultaneous utilization of PTM scheme 1 and PTP retransmission for different UEs in the same MBS group shouldn’t be supported.

Proposal 13: For retransmission scheme for a given SPS group-common PDSCH，PTM scheme 1 retransmission and PTP retransmission cannot be used simultaneously for different UEs in the same MBS group.

For the activation/deactivation of SPS group-common PDSCH for MBS, the UE can feedback ACK/NACK information according to the reception of SPS group-common PDSCH. When the gNB receives ACK/NACK, it can know the receive transmission of activation command successfully. If gNB receives NACK corresponding to SPS group-common PDSCH, it may schedule retransmission of SPS group-common PDSCH via group-common PDCCH.The UE(s) that feedback the NACK information can receive the retransmission of the TB and soft combine the received TB of retransmission with the initial transmission.

Some UE (s) in the MBS group may miss detection the activation of SPS group-common PDSCH for MBS and corresponding SPS group-common PDSCH. The UE who miss the activation command may receive the retransmission of the SPS PDSCH. In order to improve the transmission efficiency of MBS, it is recommended to support the UE(s) that miss detection the activation of SPS group-common PDSCH for MBS can receive the retransmission of SPS group-common PDSCH as well.

Proposal 14: The UE(s) missing detection the activation of SPS group-common PDSCH for MBS and corresponding SPS group-common PDSCH can receive retransmission of  the SPS group-common PDSCH scheduled by DCI scrambled by G-CS-RNTI.

PDCCH

Working assumption: The maximum number of CORESETs per BWP is not increased for support of MBS, and the number of CORESETs configured within the CFR is left to gNB implementation.

The relation between CORESET for unicast and CORESET for multicast were discussed during last meeting. In our opinion, CORESET for unicast can be used for multicast if the CORESET is fully contained in the CFR, and the CORESET for MBS in the CFR can also be used for unicast transmission. This leads small impact on current CORESET utilization for both unicast and multicast. If a CORESET in CFR for MBS cannot be used by unicast, it implies that part of the dedicated unicast BWP cannot be used by unicast, which is too restricted for data reception.

Proposal 15: The CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

    - Search space

Agreement: For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, Alt 2 is supported: - Alt 2: support a Type-x CSS - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS. - FFS: Whether the Type-x CSS is a Type-3 CSS

It was agreed in RAN1#105 meeting that a Type-x CSS is supported for CSS of group-common PDCCH of PTM scheme1 for multicast in RRC_CONNECTED state and the monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets. The remaining issue is that whether the Type-x CSS can be a Type-3 CSS. If the Type-x CSS is regarded as a Type-3 CSS, the monitoring priority of Type-3 CSS for DCI format of group-common PDCCH need to be redesigned. In our opinion, a Type-3A/Type-MBS CSS can be introduced for DCI format of group-common PDCCH to avoid redesigning the monitoring priority of legacy Type-3 CSS.

Proposal 16: A Type-3A/Type-MBS CSS can be introduced for the CSS of group-common PDCCH of PTM scheme1 for multicast in RRC_CONNECTED state.

Figure 2.4-1: Type-3A/Type-MBS CSS of UE1 is dropped

For the UEs in the multicast group, the monitoring priority of Type-3A/ Type-MBS CSS is determined by the search space indexes of the Type-3A/Type-MBS CSS set. When the monitoring priority of Type-3A/Type-MBS CSS set for the UE is lower than other USS sets, the UE can’t allocate the PDCCH candidates for monitoring to Type-3A/ Type-MBS CSS due to the limitation of PDCCH monitoring capability. Thus, the UE in the multicast group can’t receive the group-common PDCCH of the Type-3A/Type-MBS CSS set, as the UE1 in Figure 2.4-1. When the Type-3A/Type MBS CSS of any UE in the multicast groups is dropped, there are two alternatives:

- Alt.1: The g-NB is not allowed scheduling the group-common PDCCH of this Type-3A/Type MBS CSS

- Alt.2: The g-NB is allowed scheduling the group-common PDCCH of this Type-3A/Type MBS CSS

For the Alt.1, gNB cannot schedule the group-common PDCCH within the slot where the Type-3A/Type MBS CSS of any UE in the multicast group is dropped, which will affect the flexibility of the network scheduling. Alt.2 allows the g-NB to schedule the group-common PDCCH within the slot where the Type-3A/Type MBS CSS of any UE in the multicast group is dropped, and re-transmits the dropped group-common PDCCH to the UE which the Type-3A/Type MBS CSS is dropped. However, when the UEs in the multicast group are configured with Type-2 HARQ-ACK codebook, the Alt.2 will cause the issue with Type-2 HARQ-ACK codebook determination of UEs which the group-common PDCCH is dropped. For example, as shown in the Figure 2.4-2, UE1 cannot receive the group-common PDCCHs with DAI=2, 3, 4 due to the Type-3A/Type MBS CSS of the group-common PDCCH was dropped within the consecutive 3 slots, and misses the group-common PDCCH with DAI=1. Although the g-NB expects to receive an 8-bit HARQ-ACK codebook, the UE1 can only generate a 4-bit HARQ-ACK codebook based on the received DAI, resulting in the HARQ-ACK codebook cannot be successfully decoded by g-NB. In order to avoid the issue about Type-2 HARQ-ACK codebook determination caused by Alt.2, we suggest the g-NB is not allowed scheduling the group-common PDCCH when the Type-3A/Type MBS CSS of the group-common PDCCH was dropped by any UE in the multicast group.

Figure 2.4-2: The worst case for Type-2 HARQ-ACK codebook determination in Alt.2

Proposal 17: If the Type-2 HARQ-ACK codebook is configured, the g-NB is not allowed scheduling the group-common PDCCH when the Type-3A/Type MBS CSS of the group-common PDCCH was dropped by any UE in the multicast group.

    - Blind decoding budget

Agreement: (104-e) The maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS. - FFS whether the budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration, which is similar to the method used for multi-DCI based multi-TRP in Rel-16.

It was agreed in RAN1#104-e meeting that the maximum number of monitored PDCCH candidates and non-overlapped CCEs per slot per serving cell defined in Rel-15 is kept unchanged for Rel-17 MBS. It is based on per UE’s capability whether the budget of BDs/CCEs of an unused CC can be used for the current occupied CC. When a UE supports multi-carrier and CA, this kind of BDs/CCEs budget sharing can be supported for MBS.

Proposal 18: The budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration.

    - DCI design

- First DCI

Agreement:(106-e) The first DCI format for GC-PDCCH uses the same fields as DCI format 1_0 with CRC scrambled by C-RNTI with the following modifications: - At least ‘Identifier for DCI formats’ is not needed. - FFS: Whether the field should be ignored and reserved, or should be removed. Agreement:(106b-e) The ‘TPC command for scheduled PUCCH’ field is not needed for the first DCI format for multicast. - FFS: Whether the field should be reserved or should be removed.

In the previous meeting, it was agreed that the ‘Identifier for DCI formats’ filed and ‘TPC command for scheduled PUCCH’ filed are not needed for the first DCI format for multicast. One remaining issue is that whether the field should be reserved or should be removed. If the field is reserved, it means the order of each field is keep as DCI format 1_0, and there is no meaning for this filed after decoding. If the field is removed, it means this field will be removed and the reserved bits corresponding to this field will be added at the end of the first DCI format.

According to the discussion of AI 8.12.2, the DCI indicating enabling/disabling HARQ0ACK feedback may occupy 1 bit (e.g. ‘0’ indicates enabling, ‘1’inicates disabling). Thus, from our point of view, ithe ‘Identifier for DCI formats’ can be redesigned as ‘HARQ enabled/disenabled’ field. In additional, some company proposed that the ‘TPC command for scheduled PUCCH’ field can repurpose for MCCH change notification. While, there is no consensus among companies that whether the first DCI format for multicast could be used for broadcast for IDLE/INACTIVE UEs. Thus, it reasonable for first DCI format to remove the ‘TPC command for scheduled PUCCH’ filed to facilitate the repurpose of the filed, and the number of the reserved bits is increased by 2 bits.

Proposal 19：The ‘Identifier for DCI formats’ filed can be reserved and redesigned as ‘HARQ enabled/disenabled’ field.

Proposal 20: The ‘TPC command for scheduled PUCCH’ filed should be removed from the first DCI format，and the number of the reserved bits is increased by 2 bits.

- Second DCI

Agreement:(106b-e) The ‘TPC command for scheduled PUCCH’ field is not needed for the second DCI format for multicast. FFS: Whether the field should be reserved or should be removed.

For the second DCI format, most of fields can reuse the existing fields in DCI format 1_1 with CRC scrambled by C-RNTI. In the RAN1# 106b-e meeting, it was agreed that the ‘TPC command for scheduled PUCCH’ field is not needed for the second DCI format for multicast. According to our understanding, the second DCI format is similar to DCI format 1_1 whose payload size is configured by higher layer parameters instead of a fixed value. Therefore, ‘TPC command for scheduled PUCCH’ field should be removed from the second DCI format for multicast.

For ‘Identifier for DCI formats’ field, it’s used to differentiate the types of DCI, i.e. DL DCI format or UL DCI format. The ‘Identifier for DCI formats’ field is not needed for the second DCI format for multicast and should be removed. For ‘Carrier indicator’ field, the value of this filed corresponds to the value indicated by CrossCarrierSchedulingConfig for the serving cell. However, the IE CrossCarrierSchedulingConfig is a UE specific parameter. It means it is difficult for UEs in a MBS group reach a same understanding of this field. Thus, it is better that the ‘Carrier indicator’ field is removed from the second DCI. For ‘Bandwidth part indicator’ field, the bitwidth of this filed is determined by the number of DL BWPs based on higher layer. Similar with the ‘Carrier indicator’ field, the ‘Bandwidth part indicator’ field should also be removed due to it is UE- specific. In additional, there is no motivation to support two code words which are scheduled by a single DCI for multicast. Therefore, the fields of ‘Modulation and coding scheme’, ‘New data indicator’ and ‘Redundancy version’ for the second TB are also not needed for the second DCI and should be removed.

Proposal 21：For the second DCI format, at least the following fields are not needed and should be removed.

      - ‘Identifier for DCI formats’

      - ‘TPC command for scheduled PUCCH’

      - ‘Carrier indicator’

      - ‘Bandwidth part indicator’

      - ‘Modulation and coding scheme’, ‘New data indicator’ and ‘Redundancy version’ for the second TB

    - DCI size budget

Agreement: Confirm the working assumption: Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS. - FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

In R15/R16, for a given cell, the number of different DCI sizes that configured to be monitored is up to 4, in where the number of different DCI sizes with C-RNTI is up to 3. The up to ‘3’ DCI size may include fall back DCI format, non-fall back DCI format and DCI format 2_2/2_3. The format 2_2/2_3 is TPC command for PUCCH/PUSCH and SRS and the size shall be aligned to CSS DCI 1_0 by padding zeros. The “1” DCI size includes DCI formats for the other purposes scheduling, and it includes DCI format 2_0/2_1/2_4/2_5/2_6. After step 4C of DCI size alignment in R15/R16, the possible of “3+1” DCI size budget may be shown as Table 2.4-1

Table 2.4-1: The possible of the “3+1” DCI size budget after step 4C.

| Number | DCI size | Including DCI format | Note |
| --- | --- | --- | --- |
| DCI size 1 counted as “C-RNTI” | DCI 1_0 size on CSS | DCI 0_0/1_0 on CSS DCI 0_0/1_0 on USS DCI 2_2/2_3 | Determining size of FDRA by CORESET#0/initial BWP; Appending zeros to DCI 2_2/2_3 if needed. |
| DCI size 2 counted as “C-RNTI” | Maximum size of DCI 1_2 and 0_2 | DCI 0_2/1_2 on USS | The small DCI size padding zero. |
| DCI size 3 counted as “C-RNTI” | Maximum size of DCI 1_1 and 0_1 | DCI 0_1/1_1 on USS | The small DCI size padding zero. |
| DCI size 1 counted as “other RNTI” | No more than 128 bits by RRC configuration | DCI 2_0/2_1/2_3 DCI 2_4/2_5/2_6. |  |

It has been agreed that two DCI formats (i.e. first DCI format and second DCI format) are supported for scheduling group-common PDSCH. First DCI format is based on DCI format 1_0, and second DCI format is based on DCI format 1_1. When UE performed DCI size alignment procedure, several potential options for the group-common PDCCH in where two DCI formats can be counted as “C-RNTI” or “other RNTI” are shown in Table 2.4-2.

Table 2.4-2: The potential options for “3+1” DCI size budget

|  | First DCI format | Second DCI format | Note& suggestion |
| --- | --- | --- | --- |
| Option 1 | counted as “other RNTI | counted as “other RNTI | Not preferred |
| Option 2 | counted as “C-RNTI” | counted as “C-RNTI” | Not preferred |
| Option 3 | counted as “C-RNTI” | counted as “other RNTI | Not Preferred |
| Option 4 | counted as “C-RNTI” | Configured by RRC | Preferred |

- Option 1: both first DCI format and second DCI format are counted as “other RNTI”

As discussed above, the number of different DCI size that configured is up to ‘4’, in where the number of different DCI size that configured with ‘C-RNTI’ is up to ‘3’. In this option, since G-RNTIs in both first and second DCI formats are counted as ‘other RNTI’, ‘2’ different DCI size will be configured with ‘other RNTI’. And the DCI formats for unicast can only have ‘2’ different DCI size budget. This will cause the DCI format 1_1/0_1 and the DCI format 1_2/0_2 cannot be configured simultaneously, which brings low flexibility. Thus, Option 1 is not preferred.

- Option 2: both first DCI format and second DCI format are counted as “C-RNTI”

In this option, during the DCI size alignment, the size of first DCI will be equal to that of DCI format 1_0 on CSS due to field of the first DCI format is based on DCI format 1_0. And the first DCI can reuse the current mechanism to determine the bitlength of FDRA, which is based on CORESET0/initial bandwidth part. Moreover, if the bandwidth of CFR (i.e. NCFR) is larger than  that of CORESET0/initial bandwidth part,  K can be the maximum value from set {1, 2, 4, 8} which satisfies K≤NCFR/NBWPinitial;otherwise, K=1. Thus, the G-RNTI I in the first DCI format can be counted as ‘C-RNTI’.

As discussed above, the second DCI format is based on DCI format 1_1, and the FDRA size is determined by PRBs number of CFR. The benefit is that all possible of PRBs combinations can be scheduled. Since the second DCI targets a group UEs, so it is difficult to align the second DCI to each UE’s DCI format 1_1. Thus, it is more feasible that the size of the DCI format 1_1 should be aligned to the second DCI format for MBS. However, this will bring some limitations during the DCI size alignment due to the size of DCI format 1_1 is normally larger than the second DCI.

- Option 3: First DCI format is counted as “C-RNTI”, second DCI is counted as “other-RNTI”

In this option, the G-RNTI in the first DCI format is counted as ‘C-RNTI’ as discussed in Option 2. For the second DCI format, the G-RNTI in the first DCI format is counted as ‘other-RNTI’.  Same reason as discussed in Option 2, the DCI size of “the other purposes scheduling such as 2_0/2_1/2_4/2_5/2_6” should be aligned to second DCI format size. It is noted that DCI size of other purposes scheduling may be up to 126 or 128 bits, so it is not easy to align the second DCI and DCI formats with the other purposes scheduling.

- Option 4: First DCI format is counted as “C-RNTI”, second DCI is counted as “other-RNTI” or “ C-RNTI” according to RRC configuration

As discussed in Option 2/3, the G-RNTI in the first DCI format is counted as ‘C-RNTI’. The size of second DCI format is fixed during the DCI size alignment procedure. Thus, whether the second DCI format with G-RNTI is counted as “C-RNTI” or “other RNTI” can be based on the sizes of the second DCI and the DCI with “X-RNTI” (“C-RNTI” or “other RNTI”). The G-RNTI can be counted as the RNTI with the larger DCI size among the DCI size is smaller than the second DCI. In other words, when both sizes of the DCI with “C-RNTI” and “other RNTI” are smaller than that of the second DCI, the G-RNTI can be counted as the RNTI with smaller DCI size difference. The DCI size difference is the size between second DCI and DCI with “C-RNTI”/ “other RNTI”. In order to guarantee a simpler DCI size alignment procedure, UE expect that at least one of the sizes of DCI with “C-RNTI” and “other RNTI” is smaller than the size of the second DCI.

For example, after RRC configuration, the size of DCI format 1_1 with C-RNTI and the size of DCI with ‘other RNTI’ are 20 bits and 50 bits, respectively. And the size of second DCI format is 30 bits. Then, G-RNTI in the second DCI will be counted as ‘C-RNTI’; If the size of DCI format 1_1 with C-RNTI and the size of DCI with ‘other RNTI’ are 20 bits and 28 bits, respectively, G-RNTI in the second DCI will be counted as “other RNTI”.

Proposal 22: For first DCI format, G-RNTI is counted as “C-RNTI”; DCI size is aligned to DCI 1_0 on CSS

Proposal 23: UE expect that at least one of the sizes of DCI with “C-RNTI” and “other RNTI” is smaller than the size of the second DCI.

Proposal 24: For second DCI format, G-RNTI can be counted as “C-RNTI” or “other RNTI” depending on RRC configuration.

Simultaneous reception of unicast and multicast

Agreement: For Rel-17 MBS UE, the UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b. - Note: Group-common PDSCH(s) are counted as unicast PDSCH(s).

It was agreed that the UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.  This maximum number is based on UE capability. gNB has all the information of UEs’ capability and each scheduled PDSCH for unicast and multicast. gNB should take every UE’s capability into account when scheduling PDSCHs in one slot. When the scheduled PDSCH for MBS is group-common received by all the UEs in the same group, some UEs may not be able to receive it successfully because the number of simultaneous received PDSCH in the slot is beyond their capability. When this case happens, some solutions have to be considered and studied, such as dropping rule by the UEs, or retransmission scheme can be used. For example, when UE capability is limited, the scheduled PDSCH who has higher priority will be receieved.

Proposal 25: When the simultaneous reception of unicast and multicast is beyond a UE’s capability, the dropping rule should be considered.

Agreement:(RAN1#104b-e) At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs - Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC - FFS: the value(s) of M - Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC - FFS: the value(s) of N - Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC - FFS: the value(s) of K and L

The TDM between multiple TDMed unicast PDSCHs and one group-common PDSCH in a slot has been supported in RAN1#104bis-e meeting. The reception of PDSCHs contains SPS PDSCH reception, where the rule for TDMed SPS PDSCHs reception in a slot has been specified in TS 38.214-5.1 for Rel-16 NR.

In the rule, Q is the set of SPS PDSCH(s) within the slot. The PDSCH with lowest configured sps-ConfigIndex will be received and designated as a survivor PDSCH, and then the PDSCH(s) overlapping (even partially) with the survivor PDSCH will be excluded from Q. The UE will repeat the above steps until Q is empty or the number of survivor PDSCH is equal to the number of PDSCH in a slot supported by UE.

Figure 2.5-1: Multicast and Unicast service in TDM mode for UEs in same group

When the total number of TDMed SPS PDSCHs of unicast and TDMed SPS PDSCHs of multicast exceed the limit of the UE capability, the method of UE receiving the SPS PDSCH(s) needs to be further studied. If the legacy rules in Rel-16 is reused for the MBS service, it may cause UEs in the same group have different understanding of the survivor multicast SPS PDSCH.

As shown in Figure 2.5-1, the sps-ConfigIndex of unicast is different for UE 1 and UE 2, UE 1 and  UE 2 are in the same MBS group, and the sps-ConfigIndex of multicast equals to 2. For UE1, the sps-ConfigIndex of unicast equals to 1. For UE2, the sps-ConfigIndex of unicast equals to 3. Assume that the maximum number of PDSCH in a slot supported by UE 1 and UE 2 is 1. Then, UE 1 will drop SPS group-common PDSCH of multicast, and UE 2 will drop SPS PDSCH of unicast. In this case, the reception of MBS service will be affected.

A simple solution is giving higher priority to the SPS group-common PDSCH of multicast when the UE simultaneously receives the TDMed SPS PDSCH of unicast and multicast in a slot, e.g. adding offset to SPS PDSCH of unicast. This solution can make sure that all the UE in the same group will receive multicast SPD PDSCH firstly.

Proposal 26: When the UE simultaneously receives the TDMed SPS PDSCH of unicast and multicast in a slot, it is suggested to receive the SPS group-common PDSCH of multicast in high priority, e.g. adding offset to SPS PDSCH of unicast.

Agreements: (RAN1#102-e) - For RRC_CONNECTED UEs, at least support FDM between unicast PDSCH and group-common PDSCH in a slot based on UE capability. - FFS: TDM or SDM in a slot.

In RAN1#102-e, it has been agreed to support FDM between unicast PDSCH and group-common PDSCH. In Rel-16, it specifies the schemes of receiving SPS PDSCH for TDM mode. However, the schemes of receiving SPS PDSCH for FDM mode are not clear yet.

When the simultaneous reception of FDMed SPS PDSCH of unicast and FDMed SPS group-common PDSCH of multicast in a slot is beyond a UE’s capability, a specified receiving rule is required. One possible method is described as following steps:

- Step 0-A: The legacy TDMed SPS PDSCHs reception rule in Rel-16 is applied to SPS PDSCHs of multicast to determine a set of SPS PDSCHs of multicast denoted as QM

- Step 0-B: The legacy TDMed SPS PDSCHs reception rule in Rel-16 is applied to SPS PDSCHs of unicast to determine a set of SPS PDSCHs of unicast denoted as Qu

- Step 1: The UE receives one SPS PDSCH with the lowest configured sps-ConfigIndex from the set Q=QM∪Qu of SPS PDSCHs and designate the received PDSCH as the survivor PDSCH

- Step 2: The survivor PDSCH in step 1 and any other PDSCH(s) overlapping in frequency domain with the survivor in step 1 are excluded from the set Q

- Step 3:  Repeat step 1 and 2 until the set Q of SPS PDSCH(s) is empty or the number of PDSCHs in a slot is equal to the value of UE’s capability

Proposal 27: When FDMed SPS PDSCH of unicast and FDMed SPS group-common PDSCH in a slot, the rules for SPS PDSCH reception in Rel-16 can be used as a baseline of FDMed SPS PDSCH of unicast and FDMed SPS group-common PDSCH reception.

Figure 2.5-2: Multicast and Unicast service in FDM mode

As shown in Figure 2.5-2, two cases may happen when receiving unicast SPS PDSCH. In Case 1, assuming the maximum number of PDSCH in a slot is 1, Multicast SPS ID is 3 and the smallest unicast SPS ID is 1, the multicast will not be received. In Case 2, multicast and unicast have overlapping in frequency domain, the multicast SPS PDSCH will also not be received due to its higher SPS ID. When the multicast is not received, UEs in the same group will not have a common understanding of multicast reception, just similar to the issue in TDM mode. Thus, adding offset of unicast is also needed when unicast and multicast can be supported in FDM mode.

Proposal 28: When FDMed SPS PDSCH of unicast and FDMed SPS group-common PDSCH in a slot, it is suggested to receive the SPS group-common PDSCH of multicast in high priority, e.g., adding offset to SPS PDSCH of unicast.

- Conclusion

In this contribution, to support group scheduling mechanism for RRC_CONNECTED UEs, we have the following observations and proposals:

Proposal 1: The CFR configuration (i.e., cfr-Config-Multicast) is configured separately with BWP configuration, and the CFR configuration can be associated with the dedicated unicast BWP via a CFR index.

Proposal 2: The UE is not expected to receive multicast in the active BWP when the new IE CFR-Config-Multicast is not configured for the active BWP.

Proposal 3: For the LBRM/TBS determination for PTP retransmission of multicast, option1 (based on the LBRM/TBS determination of the PTM initial transmission using same HPID and NDI.) is preferred.

Proposal 4: For retransmission scheme, PTM scheme 1 retransmission and PTP retransmission cannot be used simultaneously for different UEs in the same MBS group.

Proposal 5: For a given HARQ process number, a UE is not expected to receive a new TB with the same HARQ process number before the completion of the HARQ process of a previous TB.

Proposal 6: The NDI conflict issue needs to be solved though potential specification enhancement.

Proposal 7: For NDI scheme of multicast, Option 1 is preferred.

- Option1: When a G-RNTI DCI is received with a given HPID in the DCI, the data shall be considered new, i.e. be treated as if the NDI bit had been toggled, irrespective of actual NDI toggling, if the G-RNTI is different from the most recent earlier received RNTI (i.e. C-RNTI or another G-RNTI) of the same HPID. When the received G-RNTI is the same as the most recent use of the HPID, legacy NDI toggling is used to indicate new data or retransmission.

Proposal 8: Down-selection from following options to solve the issue that the UE incorrectly soft-combine the received TB with PTP retransmission for multicast and the received TB with PTP (re)transmission for unicast when the HPID and NDI of multicast transmission and unicast transmission is same and the group-common PDCCH was missed.

- Option 1: introduce a field in DCI 1_1/1_2 of PTP transmission to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

- Option 2: use the different TB size of unicast and multicast to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

Proposal 9: UE-specific PDCCH can be also used for SPS activation for MBS for RRC_CONNECTED UEs.

Proposal 10：The UE is not expected that the values of sps-ConfigIndex in SPS configuration of multicast and unicast are the same.

Proposal 11：Multiple G-CS-RNTIs associated with one SPS-config should not be supported.

Proposal 12: Both Alt 1 and Alt 2 can be supported for reliability of the group-common PDCCH retransmitted activation of SPS group-common PDSCH.

  - Alt 1: retransmit the activation command via group-common PDCCH

  - Alt 2: retransmit the activation command via UE-specific PDCCH

Proposal 13: For retransmission scheme for a given SPS group-common PDSCH，PTM scheme 1 retransmission and PTP retransmission cannot be used simultaneously for different UEs in the same MBS group.

Proposal 14: The UE(s) missing detection the activation of SPS group-common PDSCH for MBS and corresponding SPS group-common PDSCH can receive retransmission of  the SPS group-common PDSCH scheduled by DCI scrambled by G-CS-RNTI.

Proposal 15: The CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

Proposal 16: A Type-3A/Type-MBS CSS can be introduced for the CSS of group-common PDCCH of PTM scheme1 for multicast in RRC_CONNECTED state.

Proposal 17: If the Type-2 HARQ-ACK codebook is configured, the g-NB is not allowed scheduling the group-common PDCCH when the Type-3A/Type MBS CSS of the group-common PDCCH was dropped by any UE in the multicast group.

Proposal 18: The budget of BDs/CCEs of an unused CC can be used for group-common PDCCH to count the number of BDs/CCEs for UEs supporting CA capability based on configuration.

Proposal 19：The ‘Identifier for DCI formats’ filed can be reserved and redesigned as ‘HARQ enabled/disenabled’ field.

Proposal 20: The ‘TPC command for scheduled PUCCH’ filed should be removed from the first DCI format，and the number of the reserved bits is increased by 2 bits.

Proposal 21：For the second DCI format, at least the following fields are not needed and should be removed.

      - ‘Identifier for DCI formats’

      - ‘TPC command for scheduled PUCCH’

      - ‘Carrier indicator’

      - ‘Bandwidth part indicator’

      - ‘Modulation and coding scheme’, ‘New data indicator’ and ‘Redundancy version’ for the second TB

Proposal 22: For first DCI format, G-RNTI is counted as “C-RNTI”; DCI size is aligned to DCI 1_0 on CSS

Proposal 23: UE expect that at least one of the sizes of DCI with “C-RNTI” and “other RNTI” is smaller than the size of the second DCI.

Proposal 24: For second DCI format, G-RNTI can be counted as “C-RNTI” or “other RNTI” depending on RRC configuration.

Proposal 25: When the simultaneous reception of unicast and multicast is beyond a UE’s capability, the dropping rule should be considered.

Proposal 26: When the UE simultaneously receives the TDMed SPS PDSCH of unicast and multicast in a slot, it is suggested to receive the SPS group-common PDSCH of multicast in high priority, e.g. adding offset to SPS PDSCH of unicast.

Proposal 27: When FDMed SPS PDSCH of unicast and FDMed SPS group-common PDSCH in a slot, the rules for SPS PDSCH reception in Rel-16 can be used as a baseline of FDMed SPS PDSCH of unicast and FDMed SPS group-common PDSCH reception.

Proposal 28: When FDMed SPS PDSCH of unicast and FDMed SPS group-common PDSCH in a slot, it is suggested to receive the SPS group-common PDSCH of multicast in high priority, e.g., adding offset to SPS PDSCH of unicast.

- References

  - 3GPP TSG RAN WG1 Meeting RAN1 #106b-e chairman notes, Oct 11th – 19th, 2021.