3GPP TSG RAN WG1 #107-e                            		R1-2111257

e-Meeting, November 11th– 19th, 2021

Source:	CATT

Title:	Remaining issues on enhancements for UL-AoA positioning method

Agenda item:	8.5.2

Document for:	Discussion and Decision

Introduction

Most of the issues related to the enhancements for the UL-AoA positioning method were resolved in previous meetings. In this contribution, we further discuss the remaining issues related to the enhancements of UL-AoA (AoA/ZoA) measurement reporting for UL-AoA positioning.

UL-AoA measurements enhancements

The following agreements relating to the reporting of the UL measurements of the first arrival path in RAN1#106e[1]:

Agreement: - For the first arrival path measurements on SRS for positioning resource, - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one-gNB Rx-Tx time difference} - FFS additional option: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference} - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF - For the first arrival path measurements on SRS for MIMO resource, - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - FFS: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF - Note: The operation of SRS for MIMO is transparent to the UE

With the above agreements, the reporting of {one SRS-RSRP, multiple UL AoAs, one UL RTOA} and { SRS-RSRP, multiple UL AoAs, one gNB Rx-Tx Time Difference} is supported in Rel-17 for the 1st path. It is still FFS on whether to support reporting these measurements together with multiple SRS-RSRP for the 1st path.

However, RAN1 has so far not defined per-path SRS-RSRP. But, the definition of path-specific DL PRS RSRP was just agreed in the last RAN1 meeting after a long discussion of a few meetings. An LS was sent to RAN4  to check the details of the definition with potentially further changes on the definition of path RSRP.

Agreement: The measured path DL PRS RSRP for ith path delay is defined as the power of the received DL PRS signal configured for the measurement at the ith path delay of the channel response, and - path DL PRS RSRP for 1st path delay is the power corresponding to the first detected path - FFS: Whether the path RSRP measurement is normalized with PRS RSRP. - FFS: Whether the definition of the ith path delay (other than i=1) is required. - Note: UE may choose to use a time window to compute path DL PRS RSRP by UE implementation (there is no impact to specifications managed by RAN1 for this) - Note: This does not imply that the path delay has to be reported in DL-AoD positioning - Send LS to RAN4 to check the details of the definition and feedback if they identify any update is necessary

Therefore, there is a need for RAN1 to introduce path UL SRS-RSRP, similar to path DL PRS RSRP, at least for the first path. In the above agreement for path DL PSR RSRP, there is an “FFS: Whether the definition of the ith path delay (other than i=1) is required”. For path UL SRS RSRP, in our view, there is only to define the first path UL SRS-RSRP.

In addition, in our opinion, the ratio of the receiving power of the first arriving path to the total SRS PRSP with Rel-16 definition reflects the LOS path possibility of a SRS resource. It is beneficial to be used by LMF for improving the accuracy of UE positioning calculation. Therefore, normalization of the path SRS RSRP measurement with UL SRS RSRP could be supported.

Proposal 1: The measured path UL SRS RSRP for first path delay is defined as the power of the received UL SRS signal configured for the measurement at the first path delay of the channel response.

- The reported path UL SRS RSRP measurement is normalized with UL SRS RSRP.

- Note: TRP may choose to use a time window to compute first path UL SRS RSRP by TRP implementation.

- Note: This does not imply that the path delay has to be reported in UL-AoA positioning

If we want to consider reporting multiple RSRPs for a specific path for UL-AOA, there is a need to first define path multiple RSRP, which is so far not discussed. Also, so far it is unclear what kinds of benefits it may bring and what the impacts on the implementation are, if per path multiple RSRP measurements is introduced. Given that RAN1#107-e is the final meeting for RAN1, and we don’ have the definition of per path multiple RSRP, and it is unclear on the benefits and the impact. Thus, in our view, there is no need to further discuss and no need to support reporting path multiple RSRPs for UL-AOA.

Observation 1: There is no discussion of the definition of multiple UL SRS-RSRPs for the first arrival path.

Observation 2: The benefits and the impact of reporting multiple UL SRS-RSRPs for the first arrival path is unclear.

Proposal 2: No support of reporting multiple UL SRS-RSRPs for the first arrival path of an SRS for positioning resource in Rel-17.

Conclusions

In this contribution, we discussed the remaining issues related to the enhancements for UL-AoA positioning with the following proposal.

Proposal 1: The measured path UL SRS RSRP for first path delay is defined as the power of the received UL SRS signal configured for the measurement at the first path delay of the channel response

- The path UL SRS RSRP measurement is normalized with UL SRS RSRP.

- Note: TRP may choose to use a time window to compute path UL SRS RSRP by TRP implementation

- Note: This does not imply that the path delay has to be reported in UL-AoA positioning

Proposal 2: No support of reporting multiple UL SRS-RSRPs for the first arrival path of an SRS for positioning resource in Rel-17.

References

- 3GPP TSG RAN WG1 Meeting #106bis-e, RAN1 Chairman’s Notes.

- R1-2109225, Further discussion on enhancements for UL-AoA positioning method, CATT