3GPP TSG RAN WG1 #107-e		R1-2111288

e-Meeting, November 11th – 19th, 2021

Source:	OPPO

Title:	Discussion on Rel-17 Tx Switching enhancement

Agenda Item:	5.1

Document for:	Discussion and Decision

Introduction

In Rel-16, there was a work item for the support of UL Tx switching between two different NR carriers, i.e., the support of UL Tx switching between case 1 and case 2 [1]. By the great efforts of RAN1 and RAN4, Rel-16 specification supports the UL Tx switching between two different carriers for three scenarios: EN-DC, NR CA and SUL. On the basis of Rel-16 functionality, a Rel-17 package for further enhancement of UL Tx switching was approved in RAN#90e [2]. In the latest RAN1 meetings, some pertinent agreements were made as below [3][4][5]:

Agreement: - For a UE configured with higher layer parameter supplementaryUplink and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.3 of TS 38.214 is reused. Agreement: - For a UE configured with UL CA Option 1 and with 2Tx-2Tx UL Tx switching between two uplink carriers, the mechanism of uplink switching specified in S6.1.6.2 of TS 38.214 is reused with the following add-on. - When the UE is to transmit a 2-port transmission on one uplink carrier and if the preceding uplink transmission is a 2-port transmission on another uplink carrier, then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. Agreement: - For inter-band UL CA, if 2Tx-2Tx UL Tx switching between two uplink carriers is configured: - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1Tx on carrier 1 and 1Tx on carrier 2, the next UL transmission has a 2-port transmission on either carrier 1 or carrier 2. - If the current state of Tx chains is 0Tx on carrier 1 and 2Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 1. - If the current state of Tx chains is 2Tx on carrier 1 and 0Tx on carrier 2, the next UL transmission has a 1-port or 2-port transmission on carrier 2. - For other cases, the state of Tx chains of last UL transmission is assumed. Note: For SUL, UL CA option 1 and UL CA option 2, in RAN1 understanding, no spec change to power configuration and power control. Agreement For SUL and UL CA option 1, if 1Tx-2Tx UL Tx switching or 2Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured, the switching period is only applicable when the UL transmissions are switched between band A and band B. Agreement For inter-band UL CA, if 1Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured is configured: - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1 Tx on band A and 1Tx on band B, the next UL transmission has a 2-port transmission on at least one carrier on band B. - If the current state of Tx chains is 0 Tx on band A and 2Tx on band B, the next UL transmission has a 1-port transmission on the carrier on band A. - For other cases, the state of Tx chains of last UL transmission is assumed. Agreement For inter-band UL CA, if 2Tx-2Tx UL Tx switching between 1 carrier on band A and 2 carriers on band B is configured: - For option 2 of mapping between UL transmission ports and Tx chain - The switching period is only applicable in the following cases: - If the current state of Tx chains is 1Tx on band A and 1Tx on band B, the next UL transmission has a 2-port transmission on the carrier on band A or at least one carrier on band B. - If the current state of Tx chains is 0Tx on band A and 2Tx on band B, the next UL transmission has a 1-port or 2-port transmission on the carrier on band A. - If the current state of Tx chains is 2Tx on band A and 0Tx on band B, the next UL transmission has a 1-port or 2-port transmission on at least one carrier on band B. - For other cases, the state of Tx chains of last UL transmission is assumed. Agreement For UL-CA Option2, if UL Tx switching is triggered for 1-port transmission on a carrier and the state of Tx chains after the UL Tx switching is not unique, introduce a new RRC parameter to configure between 1) and 2) - 1) The state of Tx chains supporting 2Tx transmission on the carrier is assumed. - 2) 1Tx on carrier 1 and 1Tx on carrier 2 is assumed.

In this contribution, we continue to discuss RAN1 impacts of Rel-17 Tx switching enhancement and provide ours views on the corresponding designs.

Discussion

- Due to the new introduced Tx architectures, the UL Tx switching operations become very complicated and it is beneficial to differentiate different types of operation modes, e.g., 1Tx-2Tx switching mode or 2Tx-2Tx switching mode. In RAN1#106-e meeting, it was agreed to down-select one of two options. In RAN1#106bis-e meeting, some sub-bullets and notes were added to Option 2 during the discussion. Namely, the latest version of the two options are as below

- Option 1: - For a UE configured with UL Tx switching via uplinkTxSwitching, the maximum number of antenna ports among all configured P-SRS/A-SRS and activated SP-SRS resources is used to determine the operation mode, i.e. either 1Tx-2Tx switching mode or 2Tx-2Tx switching mode. - 2Tx-2Tx switching mode: when the maximum number is 2 for all uplinks configured with uplinkTxSwitching - 1Tx-2Tx switching mode: when the maximum number is 1 for any one uplink configured with uplinkTxSwitching - the switching gap duration for a triggered uplink switching is equal to the switching time capability value reported for the switching mode - Note: If the switching time capability value for 1Tx-2Tx switching mode is not reported by the UE, the value reported for 2Tx-2Tx switching mode is applied. - If any of the above SRS resources is configured with usage “noncodebook”, then the max number of 2 antenna ports are counted for the SRS resources during the determination of operation mode. - FFS how to determine the number of antenna ports for SRS resources. - Option 2: - For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode. - Note 1: gNB would not configure 1Tx-2Tx mode where the assumed 1Tx CC is also configured with non-codebook based MIMO. - Note 2: The new RRC configuration would not conflict with other active RRC configurations on Tx states. - Note 3: This RRC parameter is used to differentiate the table (mapping between UL transmission ports and Tx chain) and switching delay for 1Tx-2Tx vs 2Tx-2Tx for UE, and it doesn’t imply any restriction on application of non-codebook transmission together with UL Tx switching.

- From the technical point of view, both options can differentiate 1Tx-2Tx switching mode and 2Tx-2Tx switching mode. However, Option 1 is more complicated compared to Option 2. That is to say, Option 1 may lead to more implementation complexity for UE. Moreover, Option 1 will make the specification obscure and not friendly to the readers. There might be some concern on the introduction of new RRC parameter in Option 2. The additional signaling overhead of one RRC parameter is neglectable. Meanwhile, the official discussion of R17 RRC parameter list will start in this meeting. Thus, it is not late to introduce any new RRC parameter. In summary, we propose to support Option 2 for a simple design and readable specification.

Proposal 1: For the differentiation 1Tx-2Tx switching mode or 2Tx-2Tx switching mode, Rel-17 support Option 2, i.e.,

  - For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode

Conclusion

In this contribution, we discuss Rel-17 UL Tx switching enhancement. Based on the discussion, we have the following proposals:

Proposal 1: For the differentiation 1Tx-2Tx switching mode or 2Tx-2Tx switching mode, Rel-17 support Option 2, i.e.,

  - For a UE configured with UL Tx switching via uplinkTxSwitching, a new RRC parameter is used to indicate 1Tx-2Tx switching mode or 2Tx-2Tx switching mode

References

- RP-193266, RF requirements for NR frequency range 1 (FR1), Ericsson

- RP-202799, RF requirements enhancement for NR frequency range 1 (FR1), Huawei, HiSilicon

- Chairman’s notes of RAN1#105-e

- Chairman’s notes of RAN1#106-e

- Chairman’s notes of RAN1#106bis-e