3GPP TSG RAN WG1 #107-e	R1-2111336

e-Meeting, November 11th – 19th, 2021

Source:	OPPO

Title:	Discussion on UE PDSCH processing time for DCI format 1_2

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

In RAN1#104bis-e meeting, the issue of UE PDSCH processing time for DCI format 1_2 was raised but unfortunately, companies did not reach consensus. In this contribution, we continue to discuss how to include the new defined RRC parameters for DCI format 1_2 in section of UE PDSCH processing time.

Discussion

In Rel-15, UE PDSCH processing time depends on the UE reported capability as well as the configuration of additional DMRS and is defined by table 5.3-1 and table 5.3-2 in TS 38.214:

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB | dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |

In RAN1 #104bis-e, the issue of UE PDSCH processing time for DCI format 1_2 was raised and discussed in Rel-16 URLLC maintenance. During the heated discussion, Intel came up with the issue of UE PDSCH processing time for DCI format 1_0. According to TS 38.214, subclause 5.1.6.2, for PDSCH scheduled by DCI format 1_0, UE shall always assume dmrs-AdditionalPosition='pos2', irrespective of configuration of additional DMRS via higher layers. Then for a DL cell configured with Cap 2 processing time, the issue of right interpretation for PDSCH processing time and DMRS assumptions for PDSCH scheduled by DCI format 1_0 was widely discussed under Rel-15 maintenance session in RAN1 #105e and RAN1 #106e. Unfortunately, no conclusion has been drawn for both Rel-15 and Rel-16 and the discussion was closed without any consensus.

When receiving PDSCH scheduled by DCI format 1_0 or receiving PDSCH before dedicated higher layer configuration of any of the parameters dmrs-AdditionalPosition, maxLength and dmrs-Type, the UE shall assume that the PDSCH is not present in any symbol carrying DM-RS except for PDSCH with allocation duration of 2 symbols with PDSCH mapping type B (described in clause 7.4.1.1.2 of [4, TS 38.211]), and a single symbol front-loaded DM-RS of configuration type 1 on DM-RS port 1000 is transmitted, and that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE and in addition - For PDSCH with mapping type A and type B, the UE shall assume dmrs-AdditionalPosition='pos2' and up to two additional single-symbol DM-RS present in a slot according to the PDSCH duration indicated in the DCI as defined in Clause 7.4.1.1 of [4, TS 38.211], and - For PDSCH with allocation duration of 2 symbols with mapping type B, the UE shall assume that the PDSCH is present in the symbol carrying DM-RS.

Then back to the issue of absence of new DMRS parameters for DCI format 1_2, i.e. dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 in Rel-16 specification, the most controversial point is whether the PDSCH processing time is dependent on DCI formats. Based on that, there are two alternatives to include the new DMRS parameters:

- Alt 1: The PDSCH processing time is independent from DCI formats: to achieve that, the PDSCH would follow Cap 2 processing time or the shorted processing time of Cap 1 only when all of the following paramters are configured with 'pos0': dmrs-AdditionalPosition in dmrs-DownlinkForPDSCH-MappingTypeA and dmrs-DownlinkForPDSCH-MappingTypeB for DCI format 1_1, as well as, dmrs-AdditionalPosition in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 for DCI format 1_2. In other cases, the PDSCH decoding follows the longer processing time of Cap 1.

  - Pros: gNB does not need to handle the OoO HARQ feedback issue between two PDSCHs following different processing time and the pipelined UE implementation is easily guaranteed.

  - Cons: the scheduling latency would be enlarged even if no additional DMRS is configured for DCI format 1_2 and the requirements of URLLC would be more difficult to be fulfilled.

- Alt 2: The PDSCH processing time is dependent on DCI formats: That is, the PDSCH processing time could be different for PDSCH scheduled by DCI format 1_1 and 1_2 depends on the dmrs-AdditionalPosition configuration for DCI format 1_1 and 1_2 respectively.

  - Pros: this alternative is flexible and would be beneficial for reducing the scheduling latency for DCI format 1_2. For example, UE may report processing capability 2 and be configured with dmrs-AdditionalPosition = 'pos0' for DCI format 1_2 but dmrs-AdditionalPosition = 'pos1' for DCI format 1_1. In such a case, gNB can schedule HARQ-ACK for PDSCH scheduled by DCI format 1_2 following Cap 2 processing time and no need to restrict the scheduling of HARQ-ACK to follow the longer processing time of Cap 1.

  - Cons: When a first PDSCH following Cap 1 processing time is scheduled and followed by a PDSCH following Cap 2 processing time, gNB needs to be careful enough to avoid the OoO HARQ feedback issue and spec change is needed to guarantee the pipelined UE implementation is not affected.

Considering the pros and cons for the two alternatives, we have a slight preference on Alt 1 and suggest to:

Proposal 1: Adopt the following Text Proposal for UE PDSCH processing time for DCI format 1_2:

5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in allof dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured, or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured. 0; 3. 1; 4.5. 2; 9 for frequency range 1. <Unchanged parts are omitted>

Conclusion

In this contribution, we further discuss how to include the new defined RRC parameters for DCI format 1_2 in section of UE PDSCH processing time and the following proposal is made:

Proposal 1: Adopt the following Text Proposal for UE PDSCH processing time for DCI format 1_2:

5.3 UE PDSCH processing procedure time <Unchanged parts are omitted> Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in allof dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured, or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured. 0; 3. 1; 4.5. 2; 9 for frequency range 1. <Unchanged parts are omitted>

Reference

- R1-2104132	 Summary of email discussion [104b-e-NR-L1enh-URLLC-01] on remaining issues on PDCCH enhancements	Moderator (Huawei)

- R1-2108573 Summary of discussion on PDSCH processing time per Capability 2 and DCI format 1_0 Moderator (Intel Corporation)

- TS 38.214 v16.7.0