3GPP TSG RAN WG1 #107-e R1-2111336 e-Meeting, November 11th – 19th, 2021 CR-Form-v12.0. CHANGE REQUEST. 38.214; CR; rev; -; Current version:; 16.7.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on UE PDSCH processing time for DCI format 1_2. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_L1enh_URLLC; Date:; 2021-11-3. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; As discussed in our companion discussion document, the new defined RRC parameter for DCI format 1_2 (dmrs-AdditionalPosition in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2) has not been included in clause of UE PDSCH processing time in TS 38.214: Table 5.3-1: PDSCH processing time for PDSCH processing capability 1 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB; dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in either of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB or if the higher layer parameter is not configured. 0; 8; N1,0. 1; 10; 13. 2; 17; 20. 3; 20; 24. Table 5.3-2: PDSCH processing time for PDSCH processing capability 2 PDSCH decoding time N1 [symbols]. dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in both of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB. 0; 3. 1; 4.5. 2; 9 for frequency range 1.. -. Summary of change:; Include the new defined RRC parameter for DCI format 1_2 (dmrs-AdditionalPosition in dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2) in the clause of UE PDSCH processing time in TS 38.214. Consequences if not approved:; Incomplete specification. Clauses affected:; 5.3. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: This CR has isolated impact on UE PDSCH processing related to DCI format 1_2: If gNB is implemented according to this CR and UE is not, UE may not provide valid HARQ-ACK for PDSCH scheduled by DCI format 1_2. If UE is implemented in accordance to this CR while gNB is not, UE may not have enough processing time for PDSCH scheduled by DCI format 1_2. This CR's revision history:; This is the first version for this CR..

- 5.3	UE PDSCH processing procedure time

*** Unchanged text is omitted ***

Table 5.3-1: PDSCH processing time for PDSCH processing capability 1

|  | PDSCH decoding time N1 [symbols] |  |
| --- | --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in allof dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured | dmrs-AdditionalPosition ≠ 'pos0' in DMRS-DownlinkConfig in any of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured, or if the higher layer parameter is not configured |
| 0 | 8 | N1,0 |
| 1 | 10 | 13 |
| 2 | 17 | 20 |
| 3 | 20 | 24 |

Table 5.3-2: PDSCH processing time for PDSCH processing capability 2

|  | PDSCH decoding time N1 [symbols] |
| --- | --- |
|  | dmrs-AdditionalPosition = 'pos0' in DMRS-DownlinkConfig in all of dmrs-DownlinkForPDSCH-MappingTypeA, dmrs-DownlinkForPDSCH-MappingTypeB, dmrs-DownlinkForPDSCH-MappingTypeA-DCI-1-2 if configured, and dmrs-DownlinkForPDSCH-MappingTypeB-DCI-1-2 if configured |
| 0 | 3 |
| 1 | 4.5 |
| 2 | 9 for frequency range 1 |

*** Unchanged text is omitted ***