3GPP TSG RAN WG1 #107-e								R1-2111394

e-Meeting, November 11th – 19th, 2021

Agenda Item 	:	8.4.2

Source 	:	Sony

Title 	:	Considerations on UL time synchronisation

Document for 	:	Discussion and decision

- Introduction

RAN1 has been studying the issue of UL time and frequency synchronisation in NTN following the study [1].

In RAN1#103e, the following agreements were made [2]:

Agreement:

An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

In RAN1#104-e, the following was further agreed on UL time synchronization [3]:

Agreement:

An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris.

FFS: Operation of closed loop and open loop TA control

Agreement:

For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN.

FFS: Details of the combination of open and closed loop TA control

Conclusion:

It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation

In RAN1#104b-e, the following was further agreed on UL time synchronization [4]:

Agreement:

The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by:

Where:

- is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command.

  - FFS: details of NTA update/accumulation.

- is UE self-estimated TA to pre-compensate for the service link delay.

- is network-controlled common TA, and may include any timing offset considered necessary by the network.

- with value of 0 is supported.

  - FFS:  details of signaling including granularity.

- is a fixed offset used to calculate the timing advance.

Note-1: Definition of  is different from that in RAN1#103-e agreement.

Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A.

Note-3:  is the common timing offset X as agreed in RAN1 #103-e.

In RAN1#106-e, the following working assumption and agreements were made on UL time synchronization [5]:

Agreement:

- A validity duration configured by the network for satellite ephemeris data indicates the maximum time during which the UE can apply the satellite ephemeris without having acquired new satellite ephemeris.

  - FFS: Associated UE behaviour if the UE does not read the ephemeris within the validity duration.

- FFS: Whether the same validity duration can be applied for Common TA.

Agreement:

Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame.

- FFS: Whether this starting time is given by predefined rule or it is indicated by the Network

In RAN1#106b-e, the following working assumption and agreements were made on UL time synchronization [6]:

Agreement:

Confirm the working assumption:

Common TA may include parameter(s) indicating timing drift.

- The UE will apply common TA according to the parameters provided by the network (if any). No offset between the common TA according to the parameters provided by the network and the actual feeder link RTT is considered when defining UE UL timing error requirements.

Agreement:

In NTN, the Network may optionally indicate one or more of the following parameters:

- Common TA , Common TA drift rate and Common TA drift rate variation.

- FFS: Common TA third order derivative.

- FFS: Details of combination of Common TA parameters

Based on the above agreements, during the RRC_IDLE and RRC_INACTIVE states, the UE can calculate its timing advance (TA) from a combination of information broadcast to the UE by the network and a UE estimated delay based on its GNSS position. During RRC_CONNECTED state however, RAN1#104bis-e left the update and accumulation of  UE   as FFS. The UE   may be updated by both self-estimation from the UE GNSS-acquired position and the serving satellite ephemeris (open loop TA update) and from TACs issued by the network (closed loop TA update). Given the availability of open and closed loop     updates at the UE during RRC_CONNECTED state, there is an issue of how the UE uses both updates to derive an accurate TA for its UL synchronisation

For NGEO NTN, the high orbital speed of the satellite means that the TA will change rather more rapidly than in terrestrial networks therefore necessitating frequent closed loop TA updates from the network. Such TACs, besides consuming transmission resources, may also not be current enough when they arrive at the UE because of the large propagation delay between the gNB and the UE. TA variation due to satellite movement is quite predictable and this can be configured to the UE as a TA drift to minimise the frequency of TACs from the network.

In this contribution we discuss what in our view needs consideration in setting combining rules that the UE can use to combine the open and closed loop    updates. We also demonstrate, through a presentation of simulation results, the impact on throughput of configuring a TA drift parameter to the UE that can be used for updating the closed and open loop TAs without the need for frequent TACs. We also discuss what additional considerations are needed to complete the design of ephemeris information validity and regaining of UL synchronisation after loss due to ephemeris expiry.

- Discussion

  - Derivation of TA Error

Part of the TA arises from the orbital height of the satellite. The network may calculate this height at any time from the ephemeris information of the satellites. The network may also derive this height from an explicit location report of the satellite position. The full TA arises from the RTT between the gNB and the UE. The network, knowing the location of the satellite (estimated from the ephemeris or derived from location measurements such as onboard GNSS), the location of the gNB (or the reference point) and the location of the UE can calculate the RTT and therefore derive a TA correction (TAC) to signal to the UE. In RRC_CONNECTED state, the network can also command a RACH from the UE and detect the TA error from the arrival time of the preamble.

The UE has a current TA that it has been using for UL transmissions. With knowledge of its current GNSS location, the satellite ephemeris information and feeder link delay (or delay from the reference point to the satellite), the UE can also calculate its current TA error in an open loop manner. In the RAN1#104e agreement, a UE having both the open and closed loop TA errors should be able to combine these to get a more accurate TA value to apply in its UL transmissions. How this is done was left for further studies from RAN1#104bis-e.

  - Time dependent accuracy of TA calculation

The closed loop TA is derived at the gNB and signalled to the UE for example via a TAC. Given the large propagation time between the gNB and UE in NTN, during the time it takes for the TAC to arrive and be decoded at the UE, the propagation delay between the UE and gNB may have changed because of any relative movements between the satellite and the UE. This makes the TA already stale and so inaccurate at the time it is decoded. In order to conserve UE battery power, the network may configure the UE to send location reports to the network only at long intervals or when commanded. The accuracy of the closed loop TA derived using a UE GNSS location report will therefore depend on the time between the UE location report and when the network used the reported UE location to calculate the TA.

Similarly, at the UE, the inputs for calculation of its open loop TA include the GNSS location of the UE, satellite location either derived from the ephemeris information or explicitly signalled and the feeder link delay that may be configured using a common timing offset. The accuracy of the open loop TA therefore also depends on both the accuracy of these parameters and the elapsed time since they were derived and/or transmitted to the UE.

Observation 1: The accuracy of both open and closed loop TAs is impacted by the age of the parameters used in their calculation.

When the UE has both closed and open loop TAs, one may be more accurate than the other. This relative accuracy may arise from the age of the respective TAs i.e. when each TA was calculated.

Proposal 1: In setting combination rules, RAN1 should consider the relative age of open versus closed loop TAs.

When a particular parameter e.g. UE location needed for TA calculation is time varying, it may be useful to derive that parameter as close as possible to the time that it would be used. For example, the UE may be configured to derive its location for use in calculating its open loop TA just prior to any UL transmissions. Furthermore, knowing the rate of change of a given parameter such as the satellite location in NGEO NTN, the calculation can be done using a value of the parameter extrapolated to the time when the TA would be used. To gauge the age of the TA in a TAC, the UE may also need to know the time at which the TA was calculated.

Observation 2: Changes in satellite location due to orbital movement affect the propagation delay of the feeder link and can be configured to the UE as a drift rate.

Proposal 2: RAN1 should consider indicating the time at which a closed loop TA was calculated to the UE.

  - Coping with timing drift due to satellite movement

In RAN1#106-e meeting, indication  of common TA drift rate was agreed. In the last meeting, it was discussed whether RAN1 should specify how the UE calculates updates to the NTA_common. One option is up to UE implementation. However, we think if it is only up to UE implementation, misalignment could occur between the network and UE. To ensure there is no misalignment, RAN1 should specify the calculation and update method for common TA with the broadcasted common TA related parameters. If further optimization is required for common TA calculation and update at UE side, the optimisation could be up to UE implementation.

Proposal 3: RAN1 should specify calculation and update method for common TA with the broadcasted common TA related parameters.

  - Ephemeris Validity

On ephemeris information validity, we think the UE requires three pieces of information to calculate the remaining validity time of any ephemeris information it reads from the SI.

- Epoch time (T0) – when was this ephemeris information first generated

- Validity duration (TD) – what was the expected validity of the information when it was first generated.

- Current time (t) – the time at which the UE reads the ephemeris information from the SI.

With these three pieces of information, the UE can calculate the remaining validity time as:

tv = TD – (t - T0).

According to the RAN1#106e agreement, the validity duration (TD)  shall be configured by the network and broadcast in the SI. According to the second agreement, it is FFS whether the epoch time (T0) is given by an implied rule or is indicated by the network. One implied rule that has been discussed in the past was to consider T0 to be the start time of the frame, subframe or slot in which the SIB carrying the ephemeris is transmitted and so does not need to be explicitly configured. If this is the case, then the UE would in addition need to know the gNB-UE RTT in order to calculate the actual frame, subframe or slot number at the gNB at the time that the UE receives the SIB carrying the ephemeris of t = (T0 + (number of repetition)*si_periodicity + RTT/2) where the si_periodicity is configured in SIB1. The number of repetitions is not readily available to the UE. Furthermore, under some circumstances e.g. RRC-Idle state, the UE may not have enough information to calculate the gNB-UE RTT by the time it reads the SI. It may therefore be better to include the epoch time explicitly in the system information.

Proposal 4: Support explicit configuration of ephemeris validity epoch time in SI.

In RAN1#106bis, it was also agreed as follows regarding the serving satellite ephemeris information:

Agreement:

A single validity duration for both serving satellite ephemeris and common TA related parameters is defined at least if serving satellite ephemeris and common TA related parameters are signaled in the same SIB message.

Agreement:

The UE assumes that it has lost uplink synchronization if new or additional assistance information (i.e. serving satellite ephemeris data or Common TA parameters) is not available within the associated validity duration.

- FFS: details on how to acquire new or additional assistance information

Regarding the agreement on single validity duration for both serving satellite ephemeris and common TA related parameters, this holds only if serving satellite ephemeris and common TA related parameters are signaled in the same SIB. If they are NOT signaled in the same SIB, then we think that separate validity timers should be defined and their details transmitted in the respective SIBs carrying the relevant information because validation timer should be updated when information of serving satellite ephemeris and/or common TA are updated separately.

Proposal 5: If serving satellite ephemeris and common TA related parameters are not signalled in the same SIB message, separate validity timers should be defined and transmitted in different system information.

The FFS is based on expiry of serving satellite ephemeris data while the UE is in RRC-CONNECTED mode. With loss of UL synchronisation, any transmissions from the UE may cause interference on the network. Therefore, the UE ought not to be allowed to transmit. To minimise disruption to ongoing services, it is best for a UE that is aware that loss of UL synchronisation is imminent to take steps to acquire new assistance information prior to the loss of UL synchronisation.

Observation 3: A UE that loses UL synchronisation due to expiry of ephemeris or common TA information may still have DL synchronisation.

Observation 4: A UE should be able to know when it will lose UL synchronisation by inspection of its UL synchronisation validity timer(s).

Proposal 6: RAN1 should require a UE soon to lose UL synchronisation to enter an UL synchronisation refresh mode during which it will signal the network for assistance information to maintain UL synchronisation.

Proposal 7: The network should be able to send in a UE-specific manner assistance information for maintenance of UL synchronisation to a UE that reports imminent loss of UL synchronisation.

3	Conclusions

In this contribution, we discussed the considerations for deciding combining rules for open and closed loop TAs. We also discussed the need for TA adjustment at UE side with timing drift rate in order to reduce the frequency of TACs. We observed and proposed as follows:

Observation 1: The accuracy of both open and closed loop TAs is impacted by the age of the parameters used in their calculation.

Proposal 1: In setting combination rules, RAN1 should consider the relative age of open versus closed loop TAs.

Observation 2: Changes in satellite location due to orbital movement affect the propagation delay of the feeder link and can be configured to the UE as a drift rate.

Proposal 2: RAN1 should consider indicating the time at which a closed loop TA was calculated to the UE.

Proposal 3: RAN1 should specify calculation and update method for common TA with the broadcasted common TA related parameters.

Proposal 4: Support explicit configuration of ephemeris validity epoch time.

Proposal 5: If serving satellite ephemeris and common TA related parameters are not signalled in the same SIB message, separate validity timers should be defined and transmitted in different system information.

Observation 3: A UE that loses UL synchronisation due to expiry of ephemeris or common TA information may still have DL synchronisation.

Observation 4: A UE should be able to know when it will lose UL synchronisation by inspection of its UL synchronisation validity timer(s).

Proposal 6: RAN1 should require a UE soon to lose UL synchronisation to enter an UL synchronisation refresh mode during which it will signal the network for assistance information to maintain UL synchronisation.

Proposal 7: The network should be able to send in a UE-specific manner assistance information for maintenance of UL synchronisation to a UE that reports imminent loss of UL synchronisation.

References

[1]	3GPP TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), V16.0.0.

[2]	Chairman's Notes RAN1 #103-e, October-November 2020.

[3] 	Chairman's Notes RAN1 #104-e, January-February 2021.

[4] 	Chairman's Notes RAN1 #104b-e, April 2021.

[5]	Chairman's Notes RAN1 #106-e, August 2021.

[6]	Chairman's Notes RAN1 #106b-e, October 2021.