3GPP TSG-RAN WG1 Meeting #107-e	R1- 2111423

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.14

Source:	Ericsson

Title:	On UE features for IoT over NTN

Document for:	Discussion

# 1	Introduction

The work item on solutions for IoT over non-terrestrial networks (NTN) was approved at RAN#92 [1]. An overview of 3GPP non-terrestrial networks can be found in [2] and an overview of state of the art in LEO satellite access can be found in [3].

In this contribution, we provide comments on the endorsed list of UE features for IoT over NTN from RAN1#106bis-e [7].

# 2	Guidelines for UE capability definitions

- The discussion on defining UE features parameters for Rel-17 has now started. It is beneficial to recall RAN2’s guidelines for UE capability definitions, as documented in the RAN2 LS [4] and copied below:

1	Avoid defining “incapability” bits as they may cause interpretation issues

The definition of the capability should not say that “a UE setting the bit does not support Rel-16 feature X”. Such statements caused a lot of problems in Rel-15. One example was the pucch-F0-2WithoutFH that indicates that “the UE does not support PUCCH formats 0 and 2 without frequency hopping”.

2	Absence of a capability bit shall not imply support for something that is not yet defined

One example is “absence of channelBW means that the UE supports all BWs”. Since new BWs might be added later (and in a release independent manner) and maybe in some other specification, “all BWs” can become ambiguous. Any legacy UE would suddenly be expected to support the new bandwidth as well.

Another example is the default capability for UE power class and UL duty cycle. From signalling point of view, reducing the overhead is marginal (unless it is reported per-BandCombination or Per-Band-of-a-BandCombination as discussed in bullet 4 below). Instead, gNB faces difficulty in comprehending the implicit capabilities amongst UEs compliant with different versions in a release.

3 	Define baseline feature bits that covers the minimum parts of a feature and use additional bits for parts that can be used together with the baseline feature

If a feature has a baseline feature, the baseline feature must be clearly defined. It is not enough to write that “if the UE does not set the bit it does not support feature group x-y”.  If a capability bit for a certain “feature X” has been defined, all UEs that support “feature X” shall set the bit, and additional parts of the feature may not be supported by all UEs.

Capability signalling and rules to determine whether a UE supports a feature should not be mixed. For example, avoid to say

“In TDD band combinations, only UEs that set the featureX capability bit support feature. In FDD-only band combinations all UEs support feature X even if they do not set the bit.”

If later FDD-only band combinations are defined, where UEs are not required to support the feature, band combinations will not be release independent. An example where this caused problem in Rel-15 was the capability simultaneousRx-Tx for EN-DC.

4 	Minimize features “per-BandCombination” and “per-Band-of-a-BandCombination”

The number of features that require capability signaling with “per-BandCombination” and “per-Band-of-a-BandCombination” (FeatureSetPerCC) differentiation should be minimized. Such features cause an explosion in the size of the capability signaling and are even more severe when it comes to the number of possible configurations, making it impossible for the gNB to take all combinations into account.

It would be of help for RAN2 to be informed of the reasons why “per-BC” or “per-band-of-a-BC” was considered necessary, and whether other alternatives were considered.

5 	Avoid defining functionality that has no RRC configuration but is dependent on capability bits.

The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting  the reported capabilities.

A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

6	UE capabilities defined in specifications must be self-contained

RAN2 would also like to point out that once the “snapshot” of defined Rel-16 capabilities have been captured in RAN2 TR 38.822, RAN2 does not plan to update this TR. Discussions (on any WG capabilities) should be based on the actual specification text. RAN2 will document the capabilities in TS 3x.306, preferably with references to RAN1/RAN4 specifications and to corresponding RRC configurations (fields and information elements).

RAN2 understands that RAN1 uses the “feature IDs”, since they are the only available definitions during the initial stage of discussions. RAN2 would like to request that once the RRC signalling is in place and can be mapped to each “feature ID”, the actual field names used in the signalling are also utilized in the RAN1/RAN4 capability discussions (to avoid inter-WG confusion e.g. in LSs).

7	Rationale for necessity of both xDD and FRx differentiations for per-UE capability

RAN2 did not discuss the RAN1 LS on XDD-FRX Differentiation (R1-1913579/R2-2000013) at the RAN2#109-e meeting, but RAN2 would anyway appreciate to be provided with rationale in case Per-UE capability with both FDD/TDD and FR1/FR2 differentiations is deemed as necessary for a feature.

# 3	Detailed comments on the initial list of UE features for IoT over NTN

Our proposed changes on the list of UE features for IoT over NTN are made in the table below, with blue highlighting. The changes are also summarized in the rightmost column of the table. A general comment not reflected in the table is that the UE features should be separate for eMTC and NB-IoT.

eMTC and NB-IoT should have separate UE capabilities.

Adopt the changes in the table.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional | Ericsson comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS implementation [1-1. UE report the validity duration of GNSS] [1-2. UE specific TA calculation in RRC_IDLE/RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris.] [1-3. UE applies common TA according to the parameters provided by the network (if any)] [1-4. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops.] 2. [Indicate/receive] serving Satellite position state vector X,Y,Z in ECEF (m) and serving Satellite velocity state vector VX,VY,VZ in ECEF (m/s) [2-1. Derive epoch time of serving satellite ephemeris Epoch time is implicitly known] from implicit epoch time] 3. Receive ephemeris orbital parameter for the serving satellite 4. UE specific TA calculation in RRC_IDLE state based on its GNSS-acquired position and the serving satellite ephemeris. 5. UE specific TA calculation [in [RRC_IDLE and RRC_CONNECTED state] based on its GNSS-acquired position and the serving satellite ephemeris. 6. UE applies common TA [in RRC_IDLE and RRC_CONNECTED] according to the parameters provided by the network (if any) [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops X. [Common TA estimation is based on indication of common TA drift rate] 8. In RRC_IDLE state calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 9. in RRC_CONNECTED state, calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. 10. Support a validity timer of UL synchronization is configured by the network 11. Support UL transmission segment duration configuration is configured by the network [and updating time and frequency pre-compensation between segments but not within segments during UL repetition] 12. UE receives cell specific K_offset 13. UE applies K_offset in timing relationship enhancements 14.UE estimates of UE-gNB RTT [15. delaying the starts of ra-ResponseWindow] |  | [No] | N/A | Release 17 UE cannot access [NTN/satellite] | Per UE | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: Serving satellite ephemeris Epoch time is implicitly known as a reference time defined by the starting time of a DL slot and/or frame. FFS: Whether this starting time is given by predefined rule or it is indicated by the network FFS: differentiation based on orbits such as LEO/MEO/GEO | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] | ”Type” should be Per UE |
| 2. LTE_NBIOT_eMTC_NTN | 2-82 | K_offset Enhancing timing relationships using a time offset | - Configuration of K_offset 1. UE receives UE specific K_offset 2. UE applies K_offset in timing relationship enhancements | 2-41 [, 2-3] | [No] | No N/A | UE does not know the offset to apply for UL transmission | Per UE | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Mandatory Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported | ”Type” should be Per UE UEs supporting NB-IoT/eMTC NTN must indicate this FG is supported |
| 2. LTE_NBIOT_eMTC_NTN | [2-310] | [UE reporting of information about the UE specific TA pre-compensation] | - [The exact content of UE reporting of information about the UE specific TA pre-compensation] - [frequency of the reports] - [granularity of the reported content] | [2-14] | No | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | Per UE | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO | Mandatory Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported | ”Type” should be Per UE UEs supporting NB-IoT/eMTC NTN must indicate this FG is supported |

# Conclusion

Based on the discussion in the previous sections we propose the following:

- eMTC and NB-IoT should have separate UE capabilities.

Adopt the changes in the table.

# References

RP-211601, WID on NB-IoT/eMTC support for NTN, RAN#92-e, June 2021.

X. Lin, S. Rommer, S. Euler, E. A. Yavuz, and R. S. Karlsson, “5G from space: An overview of 3GPP non-terrestrial networks,” to appear in IEEE Communications Standards Magazine, March 2021. Available at arXiv preprint arXiv:2103.09156.

X. Lin, S. Cioni, G. Charbit, N. Chuberre, S. Hellsten, and J.-F. Boutillon, “On the path to 6G: Embracing the next wave of low Earth orbit satellite access,” to appear in IEEE Communications Magazine, April 2021. Available at arXiv preprint arXiv:2104.10533.

R1-2001513, Guidelines for UE capability definitions.

R1-1909953, LS on ambiguity of UE L1 FDD/TDD FR1/FR2 capabilities.

R1-1911741, Reply LS on ambiguity of UE L1 FDD/TDD FR1/FR2 capabilities.

R1-2109918, ”Summary of UE features for IoT over NTN”, Moderator (AT&T), October 2021