3GPP TSG-RAN WG1 Meeting #107-e	Tdoc R1-2111469

e-Meeting, 11th – 19th November, 2021

Agenda Item:	8.2.6

Source:	Ericsson

Title:	Channel access mechanisms

Document for:	Discussion, Decision

# Table of Contents

Table of Contents	1

1	Introduction	1

2	Baseline listen before talk (LBT) design	2

2.1	Energy detection threshold enhancement	2

2.1.1	Pout in the EDT equation	3

2.1.1.1	Pout for COT sharing and UE-initiated COT	4

2.1.2	 Further adjustment to EDT threshold based on sensing beam and transmission beam for directional transmissions	4

2.1.3	Multi-channel/carrier LBT operation	6

2.2	Short control signaling transmissions (SCST)	7

2.2.1	SCST in downlink transmissions	8

2.2.2	SCST in uplink transmissions	9

3	Further enhancements to LBT mechanisms	10

3.1	Omni-directional LBT vs Directional LBT	11

3.1.1 	Multi-beam COT	14

3.2	Receiver assisted channel access and interference management	15

3.2.1	Scheme 4: Legacy L3-RSSI with potential enhancement	16

3.2.2	Scheme 1: L1-RSSI based receiver assistance	17

3.2.3	Scheme 2-2: gNB triggered PUSCH based RX assistance	19

3.3	 Usage of CAT2 LBT and COT sharing	20

3.3.1	 Gaps in Shared COT	20

3.3.2.	CAT2 LBT	21

3.4	 Sensing slot structures	23

3.5	No LBT/LBT mode switching	25

3.6 Indication to use LBT for contention exempt transmissions	25

4	Conclusion	26

5	References	29

# 1	Introduction

A revised WI [1] was approved in RAN #90-e to study and extend NR support in the frequency range of 52.6 GHz to 71 GHz [2]. The objectives for the WI, according to the outcome of the study item and leveraging FR2 design to the extent possible, are to extend NR operation up to 71 GHz considering both licensed and unlicensed operation. The WI description included the following updates:

Physical layer procedure(s) including [RAN1]:

  - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to license-exempt spectrum for frequencies between 52.6 GHz and 71 GHz.

    - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

    - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

    - Study, and if needed specify, energy detection threshold enhancement

In this contribution, we first outline the changes to baseline LBT design in ETSI HS EN 302 567 v2.2.1 and the corresponding impact to the work in 3GPP. We then examine further enhancements to LBT mechanisms. Finally, we provide conclusions on the potential enhancement beneficial to NR operation in 52.6 GHz to 71 GHz.

# 2	Baseline listen before talk (LBT) design

## 2.1	Energy detection threshold enhancement

RAN1#104-e Agreement:

The baseline ED threshold can be computed as

EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz)

Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS definition of Operating Channel BW

- FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi.

- FFS: EDT when the COT has time varying transmission beams and varying EIRP

The following agreement was made in RAN1#104-e[10] regarding the baseline Energy detection threshold according to EN 302 567 v2.2.1 [4].

RAN1#104b-e Working assumption:

For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

Furthermore, the following was agreed as a working assumption in RAN1#104b-e[16] regarding Pout.

The next sub-sections deal with the following FFSs in the above agreement.

- FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations)

- FFS: If Pout is max output EIRP of the device or instantaneous output EIRP

- FFS: definition of Operating Channel BW

- FFS: EDT when the COT has time varying transmission beams and varying EIRPs.

### 2.1.1	Pout in the EDT equation

The Pout and Pmax used in the equation is defined as follows in EN 302 567 v2.2.1 [4]

EN 302 567 Clause 4.2.5.3 
The LBT mechanism is as follows:

1) Before a single transmission or a burst of transmissions on an Operating Channel, the equipment that initiates

transmission shall perform a Clear Channel Assessment (CCA) Check in the Operating Channel.

…

7)The energy detection threshold for the CCA Check shall be -80 dBm + 10 × log10 (Operating Channel

Bandwidth (in MHz)) + 10 × log10 (Pmax / Pout) (Pmax and Pout in W EIRP) where Pout is the RF output

power (EIRP) and Pmax is the RF output power limit defined in clause 4.2.2.1.”

EN 302 567 Clause 4.2.2.1

4.2.2.1	Definition

The RF output power is the mean equivalent isotropically radiated power (EIRP) for the equipment during a transmission burst.”

4.2.2.2	Limit

The maximum RF output power is applicable to the system as a whole when operated at the highest stated power level. For a smart antenna system, the limit applies to the configuration that results in the highest EIRP. In case of multiple (adjacent or non-adjacent) channels the total RF output power of all channels shall be less than or equal to the limits in table 3.”

It is also worthy to note that the LBT mechanism is performed by the transmitter before a transmission or a burst of transmissions that could last a duration of 5ms (MCOT) according to the regulation. However, 5ms is a long time in the 60 GHz regime and the products in this band may transmit multiple bursts within a COT. This is evident since the use cases in the band involves serving multiple users with different power requirements, beam directions, link adaptation and/or different QoS requirements. In addition, most devices using the band may configure a fixed transmit power for a transmission burst (for a certain beam direction or a user). The devices then can estimate the EIRP per transmission burst depending on the beams in the burst.

Poutb3

eCCA

MCOT

Time

Tx EIRP

Transmission burst #1

Transmission burst #2

Transmission burst #3

Poutb1

Poutb2

gap

gap

From the above, it could be noted that RF output power in clause 4.2.2.1 is defined as the mean EIRP during a transmission burst. There are two points to note:a) Pout is the mean EIRP over a transmission burst b)there may be multiple transmission bursts within a COT with varying EIRPs, transmission beams. A typical example for such a COT is depicted in Figure 1 below.

Figure 1:	A typical COT with multiple transmission bursts using varying EIRPs

Therefore, from our understanding, the Pout used in the EDT equation should correspond to the highest mean EIRP in the COT if only one eCCA is performed for such multiple transmission bursts.

In BRAN #105-e, there were discussions regarding the complexity involved in calculating the mean EIRP for transmissions occurring later in a COT. It was suggested that at the time of initiation of the COT, the mean EIRP of the bursts occurring later may not be available to the device performing eCCA. From the definitions in the HS EN 302 567, Pout refers to the mean EIRP of transmissions that are already scheduled within a COT. It is also known that EIRP is measured over a certain duration (mean EIRP) and is a product of the conducted output power and the maximum beamforming gain. From Figure 1, it is established that there can be multiple transmissions or transmission bursts within a COT each with a mean EIRP. Therefore, we think that Pout can be attained by either using max EIRP or maximum of mean EIRPs of transmissions within a COT.   For the latter, this means that the maximum is determined over all the intended transmission bursts’ mean EIRP. If the transmission bursts’ mean EIRP cannot be ascertained at the beginning of the COT, we proposed to use the former. Therefore, in RAN1 106b-e[23], for progress’s sake, we proposed that a device may be able to estimate its Pout by using a crude product of the maximum TRP and an estimated antenna beamforming gain for the intended transmissions in the COT. We note that this definition calculates the Pout according to max EIRP as in the working assumption (i.e. more conservatively), but there is no need to penalize other devices that can calculate the mean EIRP as well. Therefore, we propose the following:

Confirm that Pout corresponds to the maximum or the maximum of the mean output power EIRPs of the intended transmissions or transmission bursts in a COT. The exact method to estimate Pout is left for implementation.

#### 2.1.1.1	Pout for COT sharing and UE-initiated COT

It was discussed in RAN1 #105-e[18] and RAN1 #106-e[21] whether to consider the maximum EIRP of the COT sharing device while calculating Pout to determine the EDT or not.

The argument in support of this proposal is that, if a COT sharing node uses larger EIRP than the COT initiating node, this must be included in the Pout calculation. However, it is not supported by the HS EN 302 567. In the harmonized standard, the equipment that initiates the COT needs to use only its own transmit power to determine EDT and does not include transmit powers from other responding devices. Furthermore, to prevent responding device from hogging the spectrum, there is an additional MCOT restriction for a COT. It is also worthy to note that, a stricter EDT based on responding device’s Pout may be deployed but it need not be mandated as the only option in 3GPP. There is no need to go beyond what the regulations require.

According to the regulations it is sufficient to use only the initiating device’s Pout to determine EDT.

In addition, if we consider the UE-initiated COT example discussed in the last meeting, we would like to highlight three issues. Firstly, gNB acts as the COT sharing node in a UE-initiated COT and may have higher EIRP than the UE that initiated the COT. However, gNB may also transmit in a different bandwidth than the bandwidth used by the UE that initiated the COT. Therefore, should bandwidth also be considered to determine EDT in addition to Pout? This adds unwanted complexity considering the limited benefit and necessity of LBT in this regime. This is also not aligned with the regulations. Secondly, the Pout estimated by an UE has an absolute power tolerance of at least ±12 dB. The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame (1 ms) at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20 ms. The tolerance includes the channel estimation error RSRP estimate. The minimum requirements are specified in Table 6.3.4.2-1 in TS 38.101-2 [19]. This already equates to a large variation in the determination of EDT. Thirdly, we question the necessity and/or benefits of UE-initiated COT in this regime.

The argument to use both EIRPs from the initiating and responding devices to determine Pout for a node initiating a COT is insufficient as the responding device may also use a different bandwidth than the initiating device.

Confirm that Pout is estimated only based on the node initiating the COT even for COT sharing cases.

### 2.1.2	 Further adjustment to EDT threshold based on sensing beam and transmission beam for directional transmissions

ED threshold defined in EN 302 567 is a function of the transmission beam’s EIRP, the maximum EIRP and the operating channel BW. This means that the beamforming gain of transmission beam is already included in the determination of EDT. It would be a violation of the regulations if further enhancement is made based on the beamforming or directivity gain. For e.g., if two antenna arrays have the same RF output power (EIRP), both the antenna array with the higher beamforming gain and the antenna array with the lower beamforming gain must have the same EDT according to regulations as it is based on the EIRP output power. Any adjustments done that would lower the EDT is not precluded and can be performed but there is no need to specify such a mechanism. Regarding the sensing beam’s beamforming gain inclusion in the EDT estimation, we need to understand the reference point while testing the eCCA both in ETSI and 3GPP. Which value of received energies (before or after antenna BF gain) is compared with the EDT to determine channel idle/busy in 3GPP and ETSI? If they are the same (either both before or both after BF gain in 3GPP and ETSI), then there is no need to change the EDT based on sensing BF gain. In the 5 GHz spec, it was explicitly specified that the RSSI at the antenna ports is compared with the EDT: “the received power shall be measured at the interface between the equipment and the antenna assembly”. For the testing, the antenna ports were connected via cables to perform the test, i.e. conducted tests were performed. However, in EN 302 567, there are no conducted tests but only radiated tests where the energy estimated at the antenna is measured OTA parallelly by a separate reference antenna and compared with the EDT to determine channel idle/busy. Therefore, we think that if the device’s sensing beamforming gain is included in the EDT determination, it

EN 302 567 v2.2.1:

5.3.7.2 Test Method

…

Figure 2: Test Set-up for verifying the adjacent channel rejection of an equipment

The Receive Power of the CW Blocking Signal should be calibrated, e.g. by replacing the UUT with a standard reference antenna connected to a Power Meter.

…

5.3.8.2 Test method

…

Step 1:

• The UUT may connect to a companion device during the test. The set-up for radiated testing will include signal generator able to simulate an interferer, spectrum analyser or oscilloscope with two channels, with each channel connected to separate directed antenna, one directed to UUT and the other to the companion device, UUT and the companion device to connect with, but the signal generator does not yet generate any signals at this point in time. The spectrum analyser or oscilloscope is used to monitor the transmissions of the UUT in response to the interference signal.

• When performing testing on a UUT with a directional antenna system (such as array capable of beam-forming), the wanted communication link (between UUT and companion device) and the interference test signals shall be aligned to the direction corresponding to the UUT's maximum EIRP.

…..

Step 4: Verification of reaction to the interference signal

• The analyser shall be used to monitor the transmissions of the UUT and the companion device on the selected operating channel after the interference signal was injected. This may require the analyser sweep to be triggered by the start of the interfering signal.

• Using the procedure defined in clause 5.3.8.3, it shall be verified that:

…..

Annex C (normative): Test sites and arrangements for radiated measurements

…..

C.3 Substitution antenna

The substitution antenna shall be used to replace the equipment under test in substitution measurements. For measurements below 1 GHz the substitution antenna shall be a half wavelength dipole resonant at the frequency under consideration, or a shortened dipole, calibrated to the half wavelength dipole. For measurements between 1 GHz and 4 GHz either a half wavelength dipole or a horn radiator may be used. For measurements above 4 GHz a horn radiator shall be used. The centre of this antenna shall coincide with the reference point of the test sample it has replaced. This reference point shall be the volume centre of the sample when its antenna is mounted inside the cabinet, or the point where an outside antenna is connected to the cabinet.

could potentially fail the ETSI regulatory tests.

The following can be observed from the highlighted text in EN 302 567 v2.2.1 testing clauses 5.3.7. and 5.3.8.

- The tests in EN 302 567 use omni/quasi omni sensing for testing the adaptivity clause.

- The interference power is calibrated using a single standard reference antenna according to clause 5.3.7.2, which is different from the internal antennas of the device. This reference antenna is not as sophisticated as the antenna panel in the test device, but a single omni/quasi-omni directional antenna, i.e., a horn antenna as stated in the Annex C.3 of EN 302 567 v2.2.1.

- The reference RSSI value at the receiver compared with the EDT to determine whether channel is idle/busy is the RSSI after the standard reference antenna. Therefore, the reference RSSI value already includes the physical antenna gains of the reference antenna, which is non-zero but not defined in EN 302 567 v2.2.1[4].

Therefore, if it is agreed to include sensing gain in the determination of EDT (or internal adjustment based on sensing beamforming gain), it requires an accuracy calibration of the compensation BF gain considering: i) the offset of the narrow sensing beam (compared to the test interference direction or the transmission beam) and ii) the offset between the physical directional gains from the test device’s antenna panel and the reference antenna. Without an accuracy calibration of the compensation BF gain, the resulting EDT could be higher than the one in HS or the internal adjusted RSSI value at the receiver is lower than the reference RSSI value after the reference antenna, and hence violates the HS’s requirements. However, defining such compensation would require quite large specification effort in RAN1. Moreover, sensing with an omni-directional antenna listens to interference in all the directions (friendly), whereas sensing with a directional antenna senses interference only in a particular direction and ignores interferences from other directions, which are not guaranteed to be harmless to the coexistence considering the hidden node issues and reflection of signals. We do not think it is reasonable to compensate the EDT or adjusting the internal RSSI at the receiver for directional LBT as well.

ED threshold defined in EN 302 567 v2.2.0 is a function of the transmission’s EIRP Pout, which includes the transmission beamforming gain. It does not include the sensing beamforming gain.

Including the sensing beam’s beamforming gain in EDT would require complex and accurate calibration of the compensation between the device’s antenna and the reference horn antenna used in ETSI EN 302 567 v2.2.1 regulatory test

Therefore, we propose the following-

Further adjustment on ED threshold based on the transmission and sensing beamforming gains could be up to implementation while not violating EDT requirements as per regulations.

### 2.1.3	Multi-channel/carrier LBT operation

Prior to discussing the agreements in RAN1, let us discuss the multi-carrier LBT operation in 5 GHz specifications.

In the first method, LBT was carried out per carrier. In the second method, a channel bonding scheme was employed where LBT was performed on the primary channel/carrier and then a short CAT2 LBT was performed on the secondary channels.

This was captured in 37.213 as follows.

“An eNB/gNB can access multiple channels on which transmission(s) are performed, according to one of the Type A or Type B procedures”

RAN1 #104-e Agreement:

Define Type A and Type B multi-channel channel access as:

- Type A: Perform independent eCCA for each channel

- Type B: Identify a primary channel and perform eCCA on the primary channel, while perform Cat 2 LBT for other channels in the last observation slot

Down-selection between

- Alt1: Support Type A multi-channel channel access only

- Alt2: Support both Type A and Type B multi-channel channel access.

Note: How eCCA is performed on each channel, and the BW of the channels over which eCCAs are performed are separately discussed

In RAN1 104-e, the following was agreed:

Type A multi-channel/carrier case allows multi-carrier LBT by performing channel access independently on each channel/carrier, which is compliant with the HS EN 302 567. We think that this could be re-used for the 60 GHz multi-channel/carrier LBT.

Support Alt1 in the agreement that allows only Type A multi-channel access from 37.213.

Type B multi-channel case corresponds to the channel bonding case as described above.  ETSI BRAN neither specifies CAT2 LBT nor channel bonding based multi-channel access in the HS EN 302 567.

The Type B channel access is further complicated in 60 GHz, as there are no nominal channel BW units that all the devices sharing the band would use. For example, this was 20 MHz in the 5 GHz domain. According to the regulation, a manufacturer can state through declaration of conformance the bandwidths they support. By adopting Type B multi-channel access, we are forcing an unnecessary restriction on what these BWs need to be and where the sensing needs to be done. Moreover, the channel bonding scheme worked well in 5 GHz because it was based on following the same channelization and the same unit of 20 MHz for LBT.

We should not consider Type B multi-channel access from 37.213 in this WI for three reasons. Firstly, there is no fixed channelization or nominal channel BW in 60 GHz. Secondly, any channel bandwidth, including multiple carriers with different carrier BWs for each carrier, is allowed according to the regulation if it is declared by the manufacturer. Thirdly, it is not straight-forward how to select the primary carrier for type B channel access as the HS EN 302 567 does not specify CAT2 LBT nor multi-channel/carrier operation. Consequently, a device may randomly choose a carrier with narrower bandwidth as its primary channel and perform CAT2 LBT on the wider bandwidth secondary channels. Therefore, there is too much specification effort in 3GPP RAN1 to allow Type B channel access.

Alternatively, ETSI HS EN 302 567 adequately captures the LBT BW to be used as the operating channel BW. Therefore, it is enough to define LBT bandwidth for a single carrier and allow Type A multi-channel option from 37.213 for multi-carrier operations.

ETSI regulation for 60 GHz bands do not support Type B multi-channel access.

Do not support Type B multi-channel access for NR operation in 52.6 GHz to 71 GHz.

## 2.2	Short control signaling transmissions (SCST)

The use of Short Control Signalling Transmissions shall be constrained as follows:

within an observation period of 100 ms,

the total duration of the equipment's Short Control Signalling Transmissions shall be less than 10 ms within said observation period.

Recently, ETSI TC BRAN added a separate clause for Short control signaling transmissions to HS EN 302 567. The details of short control signaling are described below.

Unlike EN 301 893 (5 GHz) and EN 303 687 (6 GHz), EN 302 567 specifies no limitation on the number of control transmissions but only on the total duration of short control transmissions during an observation period. From an HS compliance perspective, a margin of up to 10% control frame transmissions without performing an LBT is allowed. This permission should be taken into consideration when designing any LBT related changes for control channels/signals.

In HS EN 302 567, SCS transmissions have a duty cycle requirement but no limitations on the number of SCS transmissions within the observation period.

### 2.2.1	SCST in downlink transmissions

In RAN1#104b-e [16], there were two agreements on SCS transmissions in downlink.

104b-e Agreement:

- Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH.

  - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule

  - FFS: Whether this can be applied to all supported SCS or specific SCS.

  - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH

  - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

- FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc

104b-e Agreement:

For contention exemption short control signalling based DL transmission of SS/PBCH, further consider if the following signals/channels can be multiplexed with SS/PBCH block transmission.

- RMSI PDCCH and RMSI PDSCH

- Other broadcast PDSCH

- PDSCH without user-plane data

- PDCCH

- CSI-RS

- PRS

- Other signals/channels contained in Discovery Burst (i.e., exemption applies to Discovery Burst)

Note: Total exempted signals/channels should meet the restriction of 10% over any 100ms interval.

FFS: If contention exemption short control signalling based DL transmission is allowed when not multiplexed with SS/PBCH block transmission.

It has been agreed in RAN1#104b-e that Discovery Burst (including SS/PBCH), defined as in Rel-16 NR, is supported for NR Rel-17[14]. Therefore, we think that it is reasonable to extend the short control signaling transmissions exemption for SS/PBCH to the Discovery Burst.

Support extending the Short control signaling transmissions exemption to Discovery Burst as defined in Rel-16.

### 2.2.2	SCST in uplink transmissions

Agreement:

- Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS.

  - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals)

  - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell

  - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective

- FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

In RAN1 #105-e, the following was agreed for Short control signalling transmissions in UL.

EN 302 567 v 2.2.1 and EN 303 753 v0.0.4[20] grants an initiating device/ equipment to perform short control signalling transmissions according to the duty cycle requirement. It is evident that short control signalling transmissions are tested per “equipment” in the ETSI regulations and not per system or cell.

Furthermore, IEEE 802.11ad/ay devices also perform multiple actions using the short control signalling transmissions. IEEE 802.11ay organizes access to the medium in beacon intervals (BIs), like 802.11ad. A typical BI consisting of two main access periods: beacon header interval (BHI) and data transmission interval (DTI). The BHI enables beam training of unassociated DMG and EDMG STAs and network announcements through a sweep of multiple directionally transmitted frames. BHI’s are important to establish time synchronization and beamforming training. This can be performed via short control signaling transmissions according to the definitions from the regulations shown below. The BHI is followed by the DTI, which facilitates different types of medium access for data transmission and beamforming training. In the DTI, data frames can be exchanged either in contention-based access periods or scheduled periods.

The requirement of 10ms over 100ms is from one UE perspective. Therefore, we propose the following as Alt 2 in the above agreement is in accordance with the regulations. Alt 1 is a tighter requirement which need not be precluded by implementation.

Support Alt2 in which the short control signalling transmissions requirement of 10ms over 100ms duration is applicable to control and management transmissions from a single UE perspective

It is also worthy to note that, even in Alt.2, there is a need to define which device calculates and maintains the duty cycle budget for short control signalling transmissions. Furthermore, if the examples of Short control signalling transmissions (highlighted in the following quote from EN 302 567 v2.2.1) are to be considered, RACH messages fall perfectly within the scope. RACH is used for synchronization between gNBs and UEs.

EN 302 567 v 2.2.1:

4.2.6.2 Definition

Short Control Signalling Transmissions are transmissions used by the equipment to send management and control frames without sensing the channel for the presence of other signals.

4.2.6.3 Limits

The use of Short Control Signalling Transmissions shall be constrained as follows:

• within an observation period of 100 ms;

• the total duration of the equipment's Short Control Signalling Transmissions shall be less than 10 ms within said observation period.

Clause 5.3.8.2, step 4:

Apart from transmission of the frames for short control signalling (such as, for example, ACK/NACK

signals, beacon frames, other time synchronization frames and frames for beamforming) no frame shall

be initiated.

EN 303 753 v 0.0.4:

4.2.6.1	Definition

Short Control Signalling Transmissions are transmissions used by the equipment to send management and control frames, other time synchronization frames and frames for beamforming, without minimum requirements for antenna and beamforming gain, while still conforming to the output power requirements in clause 4.2.2.2.

4.2.6.2	Limits

The total duration of the equipment's Short Control Signalling Transmissions shall be less than 10 ms within an observation period of 100 ms. It is not required for equipment to implement Short Control Signalling Transmissions.

Moreover, based on EN 302 567, SCS transmissions have a duty cycle requirement but no limitations on the number of SCS transmissions within the observation period. The harmonized standards do not define any numerology nor the signal type nor the number of SCS transmissions under the SCST regime. Any control or management signals can be SCST, as long as the duty cycle requirement is met. Therefore, we have the following proposal in addition to the agreement in the last meeting.

Consistent with EN 302 567, a node can access the channel without LBT for control signal/channel transmissions, the total duration of which shall not exceed 10 ms within an observation period of 100ms. The following signals/channels shall be classified as short control signaling transmissions:

  - msg3 for the 4 step RACH and MsgB for the 2-step RACH

# 3	Further enhancements to LBT mechanisms

LBT has been used as a medium access mechanism for license-exempt spectrum in lower frequency ranges, e.g., 2.4 GHz and 5 GHz bands. However, since the millimeter wave frequency range is characterized by high radio propagation loss that require the use of directional transmission and reception via large antenna arrays, LBT does not operate satisfactory. Interference conditions in the 60 GHz band are considerably different compared to lower frequency bands. The following aspects dominate in the 60 GHz band:

- The transmission power limitation imposed by spectrum regulations and the attenuation characteristics around the 60 GHz range prevents radio signals to cause strong interference to other nodes located tens of meters away.

- In practical deployments, the signal from other transmitters are often blocked by objects further reducing the interference.

- Highly directional signal transmission is less likely to interfere other nodes even in the close vicinity, except for the nodes that lie directly in the transmission beam coverage. The probability of interference is further reduced for the nodes that employ directional reception.

- Highly directional transmission also makes it very difficult for a transmitter to correctly detect the interference level at intended receiver, and hence the fundamental assumption in classical LBT for interference avoidance no longer holds. Moreover, we demonstrated in [6] that the interference level at both transmitter and receiver is well below the LBT threshold, regardless of any difference between the sensed level at either node.

Therefore, the effectiveness and necessity of employing LBT to mitigate interference in the 60 GHz band is questionable.

The effectiveness of LBT itself as medium access mechanism for co-existence in unlicensed spectrum in 60 GHz band is questionable. Therefore, any further enhancement on LBT baseline from the HS need to be justified both on the performance gain and the required complexity.

## 3.1	Omni-directional LBT vs Directional LBT

In HS EN 302 567 there is no mention or reference to directional LBT. In IEEE 802.11ad and IEEE 802.11ay the common understanding is that a device performs omni-directional LBT or quasi-omni-directional LBT.  This is simpler for beam switching or sector sweeping. We propose to keep the same for 3GPP spec as well.

Common understanding in ETSI and IEEE 802.11ad and IEEE 802.11ay specs are omni-directional LBT or quasi-omnidirectional LBT

Directional LBT where the transmitter listens to the channel only in the direction(s) that it intends to transmit, has been discussed in the 60 GHz unlicensed SI. One common understanding is that directional LBT could increase the spatial reuse by reducing the exposed node problem. However, the transmitter already rarely defers against interferences due to high directional beamforming and pathloss. Thus, it seems to be unnecessary to optimize LBT in 60 GHz band by enabling directional LBT. To confirm that hypothesis, the performance of directional LBT compared to omni-directional LBT for different scenarios, i.e., indoor scenarios A, B and C were evaluated in [6]. The results revealed no benefits for using directional LBT as compared to operating an omni-directional LBT. Furthermore, operating without LBT still has the best throughput performance.

Simulation studies in general indicate no significant gain from using directional LBT.

In RAN1-106b-e[23], there was support from companies to study directional LBT. It was discussed whether and how to define the relationship between sensing and transmission beams. We agreed to support beam correspondence as a mechanism to define the relationship between sensing beam and transmission beams for UEs that support it. For gNBs, and UEs that do not support beamforming, the following proposal was discussed.

Agreement:

- When UE indicates a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, support the following behaviors

- If the UE is indicated to transmit with a beam corresponding to a certain SRI, the UE can use the same beam for sensing

- Assuming Rel.17 unified TCI framework, if the UE is indicated to transmit with a beam corresponding to a certain unified TCI, the UE can use the reception beam corresponding to the TCI for sensing

- FFS: The case when UE does not indicate a capability for beam correspondence

- Note: The UE should meet local regulatory requirements

Proposal 2.9.2-3

For the following situations

- gNB

- UE does not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}

- UE chooses to uses a different beam for sensing than the beam used for transmission,

- Specify necessary requirement/test procedure to guarantee sensing beam(s) “covers” the transmission beam(s)

- Some methods to define “cover” have been discussed in RAN1 (may further down select the list) and are considered as acceptable from RAN1 perspective

  - Alt-1A: the angle included in the [3] dB beamwidth of the transmission beam is included in the [X, FFS] dB beamwidth of the sensing beam.

  - Alt-1B:  the sensing beam gain measured along the direction of peak transmission direction is at least X [FFS] dB of the transmission beam gain

  - Alt-1C:  The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP.  The sensing beam gain measured along the chosen directions is at least X [FFS] dB of the transmission beam gain in those directions.

  - Alt-1D: The sensing beam gain is measured in one or more directions where the transmission beam EIRP is within A [FFS] dB of the peak EIRP and the sensing beam gain measured along the chosen directions is at least X [FFS] dB of the peak sensing beam gain

  - Alt-1E: Sensing beam has the minimum [3] dB beamwidth which at least contains all beam peak directions of transmission beams.

- Sending LS to RAN4 and inform them the above and request them to make the final choice

  - RAN4 choice may not be limited by the list above, but if different method is selected, RAN1 would like to have an opportunity to check as well

  - RAN4 can further decide for gNB or UE separately if such test or requirement is not needed or not practical and leave it for UE implementation

- Note: Supporting both alternatives or a combination of the two alternatives is not precluded

The main motivation to introduce the relationship between sensing and transmission beams is the concern that without this relationship the transmitter could sense in a direction which is different from the transmit direction resulting in miss detection of the interference in the intended transmit direction. However, in our view, it is intuitive that the sensing beam should be in the direction of the transmission beam. A device will be tested with this assumption in HS EN 302 567 and hence there is no need to define this relationship. In addition, it is worthy to note that the directional sensing/ LBT itself is not precluded by the regulations. If the sensing beam is the same as the directional transmission beam, the device can perform LBT using the beam and use the intended EIRP for the transmission to calculate EDT. If the transmission is performed in a direction different to sensing it will fail the tests in EN 302 567, where the interference signal shall be aligned to the direction of the transmission as described in Clause 5.3.8.2 of the HS.

Directional LBT is currently not precluded in the existing regulations. EN 302 567¨s tests intrinsically ensure sensing beam is in the direction of the transmission beam for devices equipped with directional antenna systems.

Notwithstanding our concerns for using beam correspondence as an alternative method to define “cover” between sensing and transmission beam, the latest agreement allows a set of UEs that supports beam correspondence tolerance requirement to be able to use it for directional sensing even though it is not clear how “cover” requirement is achieved. For the remaining UEs and gNBs it was proposed to support Alt1 and their options.

Discussion 2.9.2-1

Please provide your view to the following alternatives for sensing beam selection on the gNB side

- Alt A. Leave to gNB implement. There is neither RAN1 requirement nor RAN4 requirement

  - Question: How does this alternative satisfy the previous agreement on “3GPP specification consider defining at least the relative relationship between all applicable sensing beam(s) and the transmission beam(s) to define sensing beam for LBT, where at least sensing beam(s) “covers” the transmission beam(s)”?

- Alt B. Alt 1 in earlier agreement (RAN4 requirement based)

  - Further modify the Alt 1 in earlier agreement to clarify that, RAN4 can further decide for gNB if such test is not needed or not practical and leave it for gNB implementation

Moderator: For proponents of Alt A, please note the question above.

Moderator: From the discussion, we have relative majority on the Alt B. There are many companies supporting Alt. A. .The moderator would recommend to go with Alt B while adding the clarification above so RAN4 can make the decision if any requirements is needed

For gNBs, the following discussion further ensued in RAN1 #106b-e

It is worthy to note that there is no notion of directions/beams for sensing described in the TS 37.213 nor is there any beam correspondence requirement for gNBs in RAN4 and it will take a lot of specification effort to introduce them. Furthermore, it becomes highly complex to specify various beams for gNBs restricting their operation.

Notion of “beams” is non-existent for gNBs in RAN4.

We think Alt. 1 in earlier agreement (could be applicable for both gNB and UEs) may have been a potential compromise solution to support directional LBT with reasonable and feasible specification efforts from both RAN1 and RAN4 perspective if Alt. 2 was not supported. However, now with Alt.2 partly supported for UE with beam correspondence, we think that 3GPP does not need to pursue Alt.1 anymore considering time constraints and efforts to specify such requirements, especially for gNBs. If Alt. A is agreed, there is no need to specify anything in RAN1 or RAN4 which will speed-up the completion of the WI. 
If Alt B (i.e., Alt.1 of earlier agreement for gNB) is agreed, RAN4 needs to be involved sooner as we do not know how or where the requirements for gNB would be captured. Therefore, we propose the following

Support Alt. A in Discussion 2.9.2-1 from RAN1#106be, i.e., to leave the sensing beam selection on the gNB side up to implementation.

If Alt. B in Discussion 2.9.2-1 from RAN1#106be is agreed for gNBs, RAN1 sends an LS to RAN4 and RAN4 can further decide if such test or requirement is needed for gNBs or not.

For UEs that do not support beam correspondence capability, it might be difficult to put restrictions on them, as these are cheap devices that cannot even support beam correspondence requirement. However, to meet the sensing requirement in regulations, these devices need to perform sensing either using the widest beamwidth attainable (quasi-omni directional) or an omni-directional pattern.This would ensure that the bare minimum is possible to pass the regulatory test.

For UEs that do not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, at least support omni-directional LBT or quasi-omni-directional LBT.

### 3.1.1 	Multi-beam COT

RAN 1- 104-e Agreements:

Agreement:

For a COT with MU-MIMO (SDM) transmission, further consider the follow alternatives (down-select or support both)

- Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

Agreement:

Within a COT with TDM of beams with beam switching, down-select one or more of the following LBT operations

- Alt 1: Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold

  - FFS: Details on the definition of "cover"

- Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT

- Alt 3: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

Notwithstanding the discussions and potential proposals on directional LBT, the following agreement on supporting TDM and SDM transmissions was agreed in RAN1-104e.

However, we note that Alt 1 with “wide beam” as an omni-directional beam based LBT at the beginning of COT and no LBT for the following beams is the better option for the following reasons:

- Alt 1 (in both agreements) with omni-directional LBT is simpler and compliant with current HS (no need for LBT from the responding device(s)) in a friendly manner by performing omni-directional LBT (instead of wide sensing beam) at the beginning of COT.

- Alt 2 (in both agreements) and Alt3 (in the second agreement) would require the device(s) to perform LBT several times within a COT whenever the transmit direction changes, which adds unnecessary overhead leading to degraded performance. During the SI, many companies showed that LBT (either directional or omni-directional) degrade performance in most scenarios compared to no LBT.

- No further specification impact as there is no need to define “cover” for multi-beam case.

It was suggested that cat2 LBT may be used before switching to a new transmission beam in a COT could help to reduce the interference and improve system performance. We performed simulations in [] to verify this claim.

From the results, the following observations were made:

- The mean and 5th percentile throughputs are the highest for the case with no LBT.

- The performances of Alt1 with omni-LBT at the beginning of COT and Alt1 with directional LBT at the beginning of COT are similar.

- Alt2/3 with cat2 LBT per beam does not improve the system performance compared to Alt1. Alt2/3 even decreases the system performance (both for mean and 5th percentage throughputs) due to unnecessary LBT overhead.

We think it is easier to perform an omni-directional or quasi-omni directional “umbrella LBT” that covers all the possible beam directions at the beginning of the COT. Furthermore, there is no need to complicate channel access mechanism by having multiple LBT procedures for multiple beam directions. EN 302 567 does not define it, neither should 3GPP do that. In addition, there is also no need to define the relationship between the LBT sensing beam and the transmission beam if the LBT beam is omni-directional or quasi-omni-directional. As stated earlier, quasi-omni-directional LBT is also used by IEEE 802.11ad and IEE 802.11ay devices for operation in the 60 GHz band.

If any enhancements to better enable multiple beam transmissions within a COT when LBT mode is used can be agreed now, it is to support Alt 1 in principle for TDM and SDM case where a single LBT at the beginning of the COT is performed with the definition of “cover” meaning omni-directional or quasi-omni-directional.

RAN 1- 104b-e Agreements:

Agreement:

For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed in TDM fashion

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Agreement:

Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed one after another in time domain

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

In addition, further agreements regarding Alt 2 and Alt 3 above were agreed in RAN1-104b-e.

We note that, if the above Proposal 12 were to be agreed, the details of the above agreements in RAN1- 104b-e becomes invalid. However, if multiple alternatives are supported from the above agreement from RAN1 -104e, then we would like to highlight the following complexity involved with the approach. Since the alternatives in the above agreements are considered based on the assumption that directional LBT for each beam is supported, there needs to be an agreement on whether to support directional LBT as described in section 3.1 before we discuss the multi-beam COT. As discussed in the previous section, there is no notion of LBT/sensing related “beams” in the current 37.213 specification, nor have we agreed on how to specify directional LBT for a single beam case in RAN1. However, we have already agreed to consider performing independent per-beam LBT (Alt 2) for multi-beam COTs in RAN1 104-e. Therefore, in our opinion, we cannot make further agreements on the multi-beam case without agreeing on how to specify the single beam case.

All alternatives agreed to be considered for a COT with TDM and SDM of beams, depends solely on how directional LBT for a single beam would be specified.

RAN1 needs to decide on whether and how to specify directional LBT for single sensing beam case before further discussing multiple sensing beams.

## 3.2	Receiver assisted channel access and interference management

For receiver assisted channel access and interference management, 5 different schemes (i.e. Scheme 1, Scheme 2-1, Scheme 2-2, Scheme 3 and Scheme 4) were intensively discussed in RAN1#106bis-e. The result of the discussion was summarized in the following summary [23]:

##### Summary: 2.6.2-7 For the topics discussed in 2.6.2, here is a summary - For scheme 1, the latest discussion in sin 2.6.2-6. There is majority to support introducing L1-RSSI feature, but there are 6 companies objecting (ZTE, vivo, LGE, Samsung, Huawei/HiSilicon, Oppo) - For Scheme 2-1, gNB triggers the UE to transmit PUCCH/SRS with CCA/eCCA as RX assistance, we don’t have consensus to support - We don’t have consensus to support single DCI trigger PUCCH/SRS with CCA/eCCA and PDSCH after the PUCCH/SRS transmission - We don’t have consensus to support introducing DCI to trigger PUCCH/SRS with CCA/eCCA without PDSCH as well - For Scheme 2-2 (gNB triggered PUSCH based RX assistance), we don’t have consensus to introduce in the spec the restriction that the gNB should abandon the DL transmission if PUSCH is not detected - On the other hand, gNB triggers PUSCH with CCA/eCCA is already supported in the spec - For Scheme 3 (new RTS/CTS type signaling based RX assistance), there is consensus we don’t support - For Scheme 4 (enhancements to L3-RSSI), we have majority to support, but there are objections from TCL

The summary suggests that Scheme 2-1 and Scheme 3 should be precluded due to lack of consensus to support the schemes. Therefore in this section we mainly focus our discussion on Scheme 1, Scheme 2-2 and Scheme 4. The discussion in this section is organized in descending order based on our preference.

Do not continue to discuss Scheme 2-1 and Scheme 3 for receiver assisted channel access.

### 3.2.1	Scheme 4: Legacy L3-RSSI with potential enhancement

RSSI and Channel Occupancy (CO) measurement was introduced in Rel-16 for NR-U. RSSI and channel occupancy measurements are performed within RMTC which is configured for the UE via RRC, measured as the linear average of the total received observed per configured duration and channel bandwidth. Higher layers configure the centre frequency (rmtc-Frequency), the reference numerology and the measurement duration (i.e., which OFDM symbol(s) should be measured by the UE). The measurement channel bandwidth is specified in TS 37.213. The ASN.1 code below shows the RSSI and CO measurement configuration defined in TS 38.331 (V16.4.1).

RMTC-Config-r16 ::=                 SEQUENCE {

rmtc-Periodicity-r16                ENUMERATED {ms40, ms80, ms160, ms320, ms640},

rmtc-SubframeOffset-r16             INTEGER(0..639)                                                 OPTIONAL,   -- Need M

measDurationSymbols-r16             ENUMERATED {sym1, sym14or12, sym28or24, sym42or36, sym70or60},

rmtc-Frequency-r16                  ARFCN-ValueNR,

ref-SCS-CP-r16                      ENUMERATED {kHz15, kHz30, kHz60-NCP, kHz60-ECP},

...

}

Ref-SCS-CP in RMTC-Config is used to indicate a reference subcarrier spacing and cyclic prefix for measDurationSymbols in RMTC-Config, which is the number of consecutive symbols for which the Physical Layer reports samples of RSSI. It is observed that the current RSSI and CO measurement doesn’t support the SCSs for FR2 (i.e., 120 kHz) and the extended SCSs for FR 2-2 (i.e., 480 and 960 kHz). To support RSSI and CO measurement in FR 2-2, it is a natural and straightforward solution to extend ref-SCS-CP in RMTC-Config to include 120, 480 and 960 kHz SCSs.

Furthermore, the UE configured with RSSI and CO measurement needs to know the channel bandwidth over which the RSSI measurement should be performed. For NR unlicensed operation in 5/6 GHz, a unique channel bandwidth for RSSI measurement is specified in TS 37.213 as a contiguous set of resource blocks (RBs) on which a channel access procedure is performed, which is determined by relevant regional regulations. Hence explicit bandwidth configuration is not needed. However, for NR unlicensed operation in the spectrum beyond 52.6 GHz, regulatory definition of channel access bandwidth doesn’t exist. Therefore, there needs to be a way to inform the UE about the channel bandwidth for RSSI measurement.

One possible solution for indicating RSSI measurement bandwidth to the UE is to specify the bandwidth in the relevant 3GPP specification. The measurement bandwidth can be specified as the maximum or the minimum channel bandwidth, or any of the intermediate channel bandwidths, defined by RAN4. However, this solution effectively prohibits gNB from indicating the most suitable channel bandwidth for RSSI measurement. Alternatively, the RSSI measurement bandwidth can be specified as the currently active DL bandwidth part configured to the UE, which, in our view, is also too restrictive because the gNB might want to indicate the UE to probe a channel on a different frequency with a different bandwidth.

To achieve configuration flexibility of RSSI measurement bandwidth in various deployment scenarios, we propose to introduce a new field in RMTC-Config to indicate the channel bandwidth for RSSI measurement. The RSSI measurement bandwidth can be indicated by an index pointing to an entry in a list of candidate channel bandwidths defined by RAN4 or directly indicated by an enumerated number from the list. The channel bandwidth indicator, together with the existing center frequency (rmtc-Frequency), explicitly provides a complete frequency domain configuration for RSSI measurement. In our view, this solution provides sufficient configuration flexibility with simple RRC configuration extension.

Signalling details of the RRC configuration for RSSI and CO measurement should be left to RAN2.

Enhance the RSSI and CO measurement configuration (RMTC-Config) in Rel-16 to support NR unlicensed operation in FR2-2 in Rel-17. The enhancement includes:

- Extending the current ref-SCS-CP to include SCS 120, 480 and 960 kHz

- Introducing an indication of channel bandwidth for RSSI measurement. The enumeration of channel bandwidths should include the maximum and the minimum channel bandwidth and the intermediate channel bandwidths defined by RAN4.

- The signalling details of the RRC configuration for RSSI and CO measurement should be decided by RAN2

Another open issue for supporting RSSI and CO measurement in FR2-2 is how UE should setup its spatial reception parameter (i.e., receive beam) for the measurement. RSSI and CO measurement is configured to UEs in connected mode to probe persistent interference from other networks in the close vicinity that severely affects their downlink performance. In FR 2-2 where directional reception is most likely employed by UEs in connected mode, the receive beam for receiving downlink channels should be used for RSSI measurement, so that the measurement result can effectively reflect the interference level experienced by the UE. In this regard, we don’t see any benefit for gNB to explicitly indicate receive beam for RSSI measurement in the RSSI measurement configuration.

For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

For this problem, the CLI-RSSI measurement defined in the current specs can be used as a guide, due to its similarity with RSSI and CO measurement. In TS 38.133, the spatial reception parameter configuration for CLI-RSSI measurement is specified as follows:

Following the same principle, when performing RSSI and CO measurement in FR2-2, the UE can assume the configured RSSI measurement resources are QCL-ed with Type-D to one of the latest received PDSCH and the latest monitored CORESET.

For RSSI and CO measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with Type-D to either the latest received PDSCH or the latest monitored CORESET.

### 3.2.2	Scheme 1: L1-RSSI based receiver assistance

In Scheme 1, a new L1 measurement quantity, L1-RSSI, is introduced to report interference level at the UE to the gNB. The scheme mostly re-uses the legacy CSI reporting framework. The current NR specification supports various CSI reporting mechanisms, including periodic CSI reporting, semi-persistent CSI reporting and aperiodic CSI reporting, for various purposes in different application scenarios. In our view, these legacy CSI reporting mechanisms can be supported for L1-RSSI reporting. Furthermore, the legacy CSI reporting methods (i.e., CSI on PUCCH and CSI on PUSCH) can be also re-used for L1-RSSI reporting.

In the remaining of the section, we will discuss the details on how L1-RSSI reporting can be done by leveraging the legacy CSI reporting mechanisms.

L1 report quantity

NR in Rel-15/16 only specifies L1-RSRP and L1-SINR in CSI measurement to be reported on PUCCH/PUSCH. This is configured by setting the higher layer parameter reportQuantity to 'cri-RSRP' or 'cri-SINR', respectively (see TS 38.214 Section 5.2.1.4.2). For L1-RSSI reporting, a new Layer 1 report quantity of L1-RSSI can  be introduced so that the UE can measure and report interference level in terms of received signal strength. L1-RSSI can be a quantized absolute value of RSSI as defined in TS 38.215, or a quantized relative value referring to a configured energy detection threshold.

Measurement resource

The L1-RSSI measurement can be done in time domain or frequency domain.

RSSI measurement in frequency domain can be based on CSI-IM resources defined in the current specs. According to the current specs, the UE can be configured with one or more CSI-IM resource set configuration(s) as indicated by the higher layer parameter CSI-IM-ResourceSet. Each CSI-IM resource set consists of at least one CSI-IM resource. A CSI-IM resource is defined as a comb structure in frequency domain across certain frequency band (specified by startingRB and nrofRBs), which occupies two (pattern 0) or four (pattern 1) contiguous sub-carriers in every RB in the frequency band. To support RSSI measurement in frequency domain, the legacy CSI-IM resource can be re-used.

A drawback with RSSI measurement in frequency domain is that the received signal needs to be FFT-ed into the frequency domain before the signal on the configured CSI-IM resources can be extracted and used for RSSI estimation. It is beneficial from UE implementation and processing latency perspective to perform RSSI measurement directly in the time domain. It is worth to note that RSSI measurement in time domain is essentially equivalent to energy detection in clear channel assessment (CCA).

To support L1-RSSI measurement in time domain, the measurement resource needs to occupy the entire measurement bandwidth, which can be achieved by one of the following options:

Option 1: Configure the UE with 3 pattern-1 (or 6 pattern-0) CSI-IM resources with different sub-carrier locations so that the combination of the CSI-IM resources occupies the full measurement bandwidth

Option 2: Introduce a new CSI-IM resource pattern that occupies all sub-carriers in an RB and use it for RSSI measurement purpose.

Option 3: Introduce ‘empty’ OFDM symbols as a new CSI-RS type and use it for RSSI measurement purpose

The above options are demonstrated in Figure 6 in one RB with two OFDM symbols. Considering the configuration complexity and standardization cost, we think Option 2 is the best way forward.

Figure 4: L1-RSSI measurement resources

Measurement beam

In Rel-15/16, the configuration for CSI-IM resource doesn’t indicate QCL information for CSI-IM based interference measurement. The underlying assumption is that the UE should assume the CSI-IM resource is QCL-ed with respect to TypeD with the corresponding NZP CSI-RS resource for channel measurement. For L1-RSSI measurement that doesn’t involve NZP CSI-RS based channel measurement, the spatial reception parameter (i.e., receive beam) needs to be specified in some way.

One possible solution can be to introduce QCL-info to the legacy CSI-IM resource configuration, similar to that for NZP-CSI-RS resource in the current specification. Alternatively, the same principle proposed in the previous section for determining receive beam for L3-RSSI measurement can be reused. That is to say, the UE can assume the configured RSSI measurement resources are QCL-ed with Type-D to one of the latest received PDSCH and the latest monitored CORESET.

L1-RSSI measurement latency

In the current NR specification, the CSI reporting delays are quite long mainly due to the excessive CSI processing latency. For instance, it is up to 11 slots for 120 kHz SCS and could be up to 88 slots for 960 kHz SCS based on the latest agreement on L1 processing timeline. Considering L1-RSSI measurement at the UE can most likely be done within a much shorter time compared to a complete CSI calculation, a much shorter  CSI processing delay requirement can be defined for L1-RSSI measurement. At least for RSSI measurement in time domain, the processing latency should be on-par with that for energy detection for LBT.

Based on above analysis, we propose the following:

If RAN1 decides to support Scheme 1, the following enhancements on the current CSI reporting can be considered:

- Introduce a new report quantity of L1-RSSI, which can be a quantized absolute value of RSSI, or a quantized relative value referring to a configured energy detection threshold.

- Introduce a new CSI-IM resource pattern that occupies the entire operating bandwidth for L1-RSSI measurement.

- Introduce QCL information to CSI-IM resource configuration for L1-RSSI measurement or specify that the configured RSSI measurement resources are QCL-ed with Type-D to one of the latest received PDSCH and the latest monitored CORESET.

- Define a new CSI processing latency requirement for L1-RSSI reporting.

### 3.2.3	Scheme 2-2: gNB triggered PUSCH based RX assistance

With Scheme 2-2, gNB first transmits a UL grant to indicate the UE to perform CCA and report the result in the scheduled PUSCH. UE performs CCA for the scheduled UL transmission and if LBT passes, transmits the receiver assistance information in the PUSCH (explicit feedback) or transmits user data and/or BSR with the PUSCH (implicit feedback). Based on the CCA result or data received on the PUSCH, the gNB determines whether it should continue to schedule and transmit the PDSCH.

In the last meeting, the below summary was documented regarding Scheme 2-2:

- For Scheme 2-2 (gNB triggered PUSCH based RX assistance), we don’t have consensus to introduce in the spec the restriction that the gNB should abandon the DL transmission if PUSCH is not detected

  - On the other hand, gNB triggers PUSCH with CCA/eCCA is already supported in the spec

According to this summary, there is no consensus among companies to introduce the restriction that the gNB should abandon the DL transmission if PUSCH is not detected. The conclusion implies that it is completely up to gNB implementation whether or not to schedule a PUSCH to probe the interference level at the UE prior to a PDSCH transmission. The remaining question is that whether reporting of the interference level at the UE should be done via explicit or implicit feedback.

For the explicit feedback approach, where the UE explicitly reports CCA result (either single-bit binary value or multi-bit quantified value) in the scheduled PUSCH, at least the report quantity and how it is measured needs to be specified in the specs. Moreover, with explicit feedback, Scheme 2-2 becomes very similar to Scheme 1 with aperiodic L1-RSSI reporting. The major difference compared to Scheme 1 is the report quantity. For Scheme 1, the report quantity is L1-RSSI, while for Scheme 2-2 with explicit feedback, one could view the report quantity as the result of a CCA procedure. Therefore, we think Scheme 2-2 with explicit feedback should be discussed in the context of Scheme 1.

For the implicit feedback approach, the UE transmits the scheduled PUSCH only if LBT passes, which is according to the conventional PUSCH transmission procedure for unlicensed operation. From the gNB’s perspective, the CCA result at the UE can be inferred based on whether the scheduled PUSCH is detected or not. Therefore, Scheme 2-2 with implicit feedback is already supported in the current specs and whether or not to apply the scheme can be completely up to gNB implementation. There is no need for RAN1 to continue discuss Scheme 2-2 with implicit feedback.

Discuss Scheme 2-2 with explicit feedback in the context of Scheme 1.

Conclude that Scheme 2-2 with implicit feedback is already supported by the current specs.

## 3.3	 Usage of CAT2 LBT and COT sharing

### 3.3.1		Gaps in Shared COT

Agreement:

On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives

- Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration

- Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission.

  - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA

  - Further down-select between the following options:

    - Option 1: Y=8 us (motivated by need to operate in all regions)

    - Option 2: Y=a multiple number of OFDM symbols

    - Option 3: gNB determines Y (for example, according to local regulation)

  - Cat. 2 LBT is a UE capability

- The usage of the two alternatives is a gNB choice and depends at least on local regulations.

Note: Alt. 3 is motivated by the regulations in Japan, but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan.

Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed

Note: Other use cases of Cat 2 LBT will be separately discussed

The following was agreed in RAN1- 106-e[21]

Regulations for 60 GHz in Japan mandate channel sensing for all the transmissions with transmit power above 10 dBm[22]. However, the details of what “channel sensing” means is not clear. In addition, this poses a problem in COT sharing where the responding device is also expected to perform “sensing” before responding to the initiating device’s transmissions. Therefore, in addition to Cat 1 (no LBT) based COT sharing mechanism, it was agreed to support Cat 2 LBT as in Alt 3. However, the usage of Alt 1 or Alt 3 is a gNB choice and hence does not mean that Cat2 LBT is mandatory for COT sharing in other regions.

It is worthy to note that, use of CAT3 LBT is also an option for operation in Japan and CAT2 LBT is a UE capability feature

It is worthy to note that in Rel-16, there was a gap definition used to determine usage of CAT2 LBT, however in 60 GHz, there is no gap value defined. Also, there is no requirement in ETSI regions to use CAT2 LBT for COT sharing case. It was also clear from Rel-16 discussions that there is no reliable way for UE to know when gNB transmission ends in the previous DL transmission and hence even if the gap is specified UEs won’t be able to estimate them. Therefore, we think that gNB can determine Y according to the regulatory domain and that there is no need to specify or signal it to the UE.

There is no reliable way for UEs (responding device) to know the end of the transmissions by the gNB (initiating device), to estimate the gap Y in the COT sharing case

For the maximum gap Y between initiating device and responding device in a COT sharing case for regions where sensing is needed, we support Option 3 with no need for gNB to specify or signal Y to the UE.

In Rel-16 the UE follows the DCI to know whether CAT2 LBT needs to be used before a PUSCH transmission or not. This could be reused for Rel-17 in FR2-2 as well.

Reuse Rel-16 framework for indication of CAT2 LBT in the DCI.

### 3.3.2.	CAT2 LBT

Agreement:

For Cat 2 LBT, down-select from the following alternatives

- Alt 1: Do not introduce Cat 2 LBT for 60GHz unlicensed band operation

- Alt 2: Introduce Cat 2 LBT for 60GHz unlicensed band operation

Agreement:

If Cat 2 LBT is introduced, the following use cases can be further studied:

- Resume transmission after a gap Y:  Cat 2 LBT may be used to resume transmission by the initiating device within the COT after a gap Y (FFS the value of Y)

- COT sharing: Cat 2 LBT may be used before transmission by a responding node sharing a COT

- Multi-Beam LBT:  Cat 2 LBT may be used before switching to a new transmission beam (not used in earlier part of the COT) in a COT with TDM beams, or resume a previously used transmission beam after a gap Z (FFS the value of Z)

- Rx-Assistance:  Cat 2 LBT may be used for sensing at the receiver as a responding device for Rx-Assistance measurements and associated signalling

Other use cases not precluded.

FFS if Cat 2 LBT is mandated for each use case or not.

In addition to the above agreement, the following agreement was made in RAN1#104-e regarding CAT2 LBT

First of all, it should be noted that the evaluations done during the SI phase do not encourage the use of Cat3 LBT/eCCA (8us +5us*rand (0,3) = 23us max, the contention window is fixed with minimum value of 3) as it results in aggregated throughput degradation when LBT is used. In our view, adding further complexity and overhead in terms of Cat2 LBT (on top of the already redundant Cat3 LBT at the beginning of COT) is not only necessary but futile. Furthermore, EN 302 567 does not mandate any Cat2 LBT.

Cat2 LBT is not specified in HS EN 302 567

Cat2 LBT has been touted as useful mechanism for various use cases in RAN1 104-e as in the above agreement. In the following, we analyze the usefulness of cat2 LBT case-by-case.

- Resume transmission after a gap in COT

It is unclear if Cat2 LBT inside COT is necessary because Cat3 LBT itself did not have a significant impact in the simulation studies due to limited interference in the 60GHz band. Moreover, if the initiating device could not fill in some big gaps between transmission bursts, it could simply perform the already supported Cat3 LBT (which is not much longer than Cat2 LBT considering CWS is only 3) to obtain a new COT.

- COT sharing

In RAN1 104b-e [15], we presented the performance evaluation for COT sharing using Cat2 LBT at the responding device, in addition to Cat3 LBT at the initiating device. In the simulations, Cat2 LBT was performed at the responding device when the gap between the previous transmission and the current transmission is more than 8us using Scenario A, which has the largest number of nodes among the indoor scenarios (A, B and C).

It was observed in [15] that, there are no gains obtained due to performing Cat2 LBT at the responding device for COT sharing when the initiating device performs both omni-directional and directional Cat3 LBT. The mean and 5th percentile throughput of the system either remains unchanged or is slightly lowered when performing Cat2 LBT at the responding device, regardless of whether it was directional or omni-directional. It can also be noted that the type of LBT (omni-directional or directional Cat3) at the initiating device also seem to have no significant impact on the system throughput.

- Multi-Beam LBT

As we analyze in section 3.1.1, cat2 LBT before switching to a new transmission beam is not necessary. In this contribution, we also simulate the benefit of using Cat2 LBT for TDM beam switching in section 3.1.1. The simulation results show that cat2 LBT for beam switching does not improve the system performance and even decreases the system throughput (both for mean and 5th percentage throughputs) due to unnecessary LBT overhead.

- Rx-Assistance

In RAN1 103-e we presented the performance evaluation for ideal receiver assisted LBT (RAL) [6]. In the evaluated procedure, Cat3 LBT procedure is performed at the receiver. It is also worthy to note that the LBT result at the receiver is assumed to be available instantly at the transmitter without accounting for any overhead for exchanging this information between the transmitter and the receiver. According to the results, the Cat3 LBT at the receiver reduces the system throughput (both mean and 5th%ile) even for the most ideal scenario. We postulated that if a more conservative Cat3 LBT (23 µs) did not yield any performance improvement for receiver-assistance in channel access, Cat2 LBT (8 µs) will likely not.

Simulation studies show that there is no gain using Ca2 LBT compared to no LBT for the proposed used cases.

Do not support Cat2 LBT for any of the use cases in 52.6 GHz to 71 GHz. It is not precluded to do CAT2 LBT in addition to CAT3 LBT requirements.

Agreement:

For energy measurement in 8us deferral period, at least a single measurement within 8us is performed, and the measurement duration is selected from one of the following alternatives:

- Alt 1: At least 3+X us (FFS X, such as X=1).

- Alt 2: At least X us, where X is the same as the minimum measurement duration in a 5 us observation slot and is within the 5 us observation slot.

- Alt 3: At least a contiguous duration of X+Y us where the Y us part of the measurement is done at the end of the first 3 us and X is the same as the minimum measurement duration in a 5 us observation slot and is at the beginning of the 5 us duration.

Agreement:

- For energy measurement in 8us deferral period, Alt 2 is supported while Alt 1 and Alt 3 can be considered as gNB/UE implementation (Alt. 1/2/3 are defined as per previous agreement)

3.4	 Sensing slot structures

In RAN1 106-e[21], the following was agreed regarding the sensing structures for LBT.

Regarding the sensing periods of 8us deferral period and 5us sensing slot for eCCA, EN 302 567 does not specify any sensing structure but these parameters were adopted from 802.11ad specifications.

5us slot is definition of aSlotTime, and 8 µs is sum of aSIFSTime (3 µs) + aSlotTime (5 µs). Furthermore, in IEEE 802.11ad amendment (incorporated in IEEE 802.11-2020), a SIFSTime and aSlotTime is calculated as

8us deferral period in IEEE 802.11ad and IEEE 802.11ay consists of the 5us observation slot at the end of the 8us period

Based on the above, since ETSI HS requirements were developed from IEEE 802.11ad, it is sufficient for 3GPP to also follow the same for a 8us deferral period. Moreover, this also allows companies to perform Alt1 and Alt3 as implementations if the 5us observation slot is located at the end of the 8us period.

For energy measurement in 8 µs deferral period, 5us observation slot is located at the end of the 8us deferral period like IEEE 802.11ad/ay.

Regarding 5 µs measurement periods, aSlotTime (5 µs) is defined as including aCCAtime. aCCATime is defined as “the maximum time (in microseconds) the CCA mechanism has available to assess the medium to determine whether the medium is busy or idle.” This is specified as a maximum duration of 3us in the 802.11ad-2012 specifications as shown in Figure 5. However, this is changed to “implementation dependent” in the recently released 802.11-2020 specifications, Table 20-30 Figure 6. Therefore, we think these values should be the same in 3GPP specifications as well. However, in 3GPP, we need to define only a minimum duration and not a maximum duration.

The minimum measurement duration X within a 5 µs observation slot can be left for implementation with the maximum value as 3µs

Figure 5:	 802.11ad-2012 specification: aCCATime value specified as less than 3 us

Figure 6:	Recent 802.11-2020 specification showing aCCATime value for 802.11ad as “Implementation-dependent”.

## 3.5	No LBT/LBT mode switching

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

Both LBT and no LBT channel access modes has been agreed to support for NR operation in 52.6 GHz to 71 GHz. The mechanism to indicate which LBT mode to use by the UEs has been discussed last meetings. The following agreement regarding LBT mode indication have also been agreed in RAN 1 #105-e.

For the 52.6 – 71 GHz band, there is a unique scenario where a part of the operating band can be both licensed and unlicensed. Hence, the UE will need to know a priori if the cell uses LBT to be able to interpret the bits of the MIB correctly. This is different from Rel-16 where there are no overlapping licensed / unlicensed bands, and LBT is always assumed to be used in unlicensed band.

An alternative to a priori knowledge, is that the MIB bits must be repurposed in the same way both for licensed (or no-LBT) and unlicensed (with-LBT) operation, i.e., only bits that are unused also for licensed operation can be repurposed. In this option no bits are repurposed and the MIB is interpreted according to the Rel-15 definitions, i.e., as in licensed bands. It can be specified that for the 52.6 – 71 GHz band, the size of DCI 1_0 is defined to be the same for both operation with and without LBT (including licensed operation). Preferably to limit the number of combinations in 38.212, the size of DCI 1_0 can be aligned with the size for Rel-16 shared spectrum channel access, i.e., for the case of licensed operation. Following this approach two additional bits would be added for the licensed case.

The other option is to define two different DCI_1_0 sizes like Rel-16. The UE needs to perform two blind decodes when receiving Type 0 PDCCH that schedules SIB1. Once the UE has read SIB1, the mode of operation is known (LBT on/off), and no additional blind decodes are needed when decoding DCI_1_0.

Based on the analysis above our view is that supporting two blind decodes for a UE is complicated and not well motivated. Further for the sake of simplicity and forward compatibility we propose to not repurpose any bits when operating in this frequency range. So, to summarize we make the following proposal:

Cell-specific system information indication of LBT ON/OFF is included in SIB1

    - Define same DCI_1_0 sizes for both LBT on/off (licensed and unlicensed operation)

## 3.6 Indication to use LBT for contention exempt transmissions

Regulations for 60 GHz in Japan mandate channel sensing for all transmissions with transmit power above 10 dBm[22]. However, the details of what “channel sensing” means is not clear. This poses a unique problem for short control signalling transmissions that are basically exempt from performing sensing before transmissions. Therefore, we think that it is useful to indicate if LBT needs to be used before contention exempt transmissions or not. Furthermore, this also lets devices estimate if LBT needs to be used for cases where the 10% limit is reached.

We have already agreed that RACH msg1 and Msg A are included in contention exempt transmissions. This is not possible in regions like Japan as mentioned before. Therefore, the UE needs to know if it should perform LBT before transmitting RACH or not. This can be indicated in the SIB1 like no LBT/LBT indication which will help the UEs to perform sensing for RACH and other short control signalling transmissions if this bit is set.

Enabling use of LBT for contention exempt transmissions is indicated in SIB1. The type of LBT (CAT3 or CAT2 LBT) to be used can be left for implementation.

# 4	Conclusion

In this contribution, we first discuss the baseline changes to LBT design that need to be addressed due to the change in the harmonized standard HS EN 302 567. Our analysis shows that there is no significant gain from doing LBT but we propose enhancements to different LBT mechanisms, its usage and design, when used. Finally, we discuss a few potential enhancements that are beneficial to NR operation in 52.6 GHz to 71 GHz. The following observations can be drawn:

Observation 1	According to the regulations it is sufficient to use only the initiating device’s Pout to determine EDT.

Observation 2	The argument to use both EIRPs from the initiating and responding devices to determine Pout for a node initiating a COT is insufficient as the responding device may also use a different bandwidth than the initiating device.

Observation 3	ED threshold defined in EN 302 567 v2.2.0 is a function of the transmission’s EIRP Pout, which includes the transmission beamforming gain. It does not include the sensing beamforming gain.

Observation 4	Including the sensing beam’s beamforming gain in EDT would require complex and accurate calibration of the compensation between the device’s antenna and the reference horn antenna used in ETSI EN 302 567 v2.2.1 regulatory test

Observation 5	ETSI regulation for 60 GHz bands do not support Type B multi-channel access.

Observation 6	In HS EN 302 567, SCS transmissions have a duty cycle requirement but no limitations on the number of SCS transmissions within the observation period.

Observation 7	The effectiveness of LBT itself as medium access mechanism for co-existence in unlicensed spectrum in 60 GHz band is questionable. Therefore, any further enhancement on LBT baseline from the HS need to be justified both on the performance gain and the required complexity.

Observation 8	Common understanding in ETSI and IEEE 802.11ad and IEEE 802.11ay specs are omni-directional LBT or quasi-omnidirectional LBT

Observation 9	Simulation studies in general indicate no significant gain from using directional LBT.

Observation 10	Directional LBT is currently not precluded in the existing regulations. EN 302 567¨s tests intrinsically ensure sensing beam is in the direction of the transmission beam for devices equipped with directional antenna systems.

Observation 11	Notion of “beams” is non-existent for gNBs in RAN4.

Observation 12	All alternatives agreed to be considered for a COT with TDM and SDM of beams, depends solely on how directional LBT for a single beam would be specified.

Observation 13	It is worthy to note that, use of CAT3 LBT is also an option for operation in Japan and CAT2 LBT is a UE capability feature

Observation 14	There is no reliable way for UEs (responding device) to know the end of the transmissions by the gNB (initiating device), to estimate the gap Y in the COT sharing case

Observation 15	Cat2 LBT is not specified in HS EN 302 567

Observation 16	Simulation studies show that there is no gain using Ca2 LBT compared to no LBT for the proposed used cases.

Observation 17	8us deferral period in IEEE 802.11ad and IEEE 802.11ay consists of the 5us observation slot at the end of the 8us period

Based on the extensive analysis and observations provide in this contribution, we propose

Proposal 1	Confirm that Pout corresponds to the maximum or the maximum of the mean output power EIRPs of the intended transmissions or transmission bursts in a COT. The exact method to estimate Pout is left for implementation.

Proposal 2	Confirm that Pout is estimated only based on the node initiating the COT even for COT sharing cases.

Proposal 3	Further adjustment on ED threshold based on the transmission and sensing beamforming gains could be up to implementation while not violating EDT requirements as per regulations.

Proposal 4	Support Alt1 in the agreement that allows only Type A multi-channel access from 37.213.

Proposal 5	Do not support Type B multi-channel access for NR operation in 52.6 GHz to 71 GHz.

Proposal 6	Support extending the Short control signaling transmissions exemption to Discovery Burst as defined in Rel-16.

Proposal 7	Support Alt2 in which the short control signalling transmissions requirement of 10ms over 100ms duration is applicable to control and management transmissions from a single UE perspective

Proposal 8	Consistent with EN 302 567, a node can access the channel without LBT for control signal/channel transmissions, the total duration of which shall not exceed 10 ms within an observation period of 100ms. The following signals/channels shall be classified as short control signaling transmissions:

1	msg3 for the 4 step RACH and MsgB for the 2-step RACH

Proposal 9	Support Alt. A in Discussion 2.9.2-1 from RAN1#106be, i.e., to leave the sensing beam selection on the gNB side up to implementation.

Proposal 10	If Alt. B in Discussion 2.9.2-1 from RAN1#106be is agreed for gNBs, RAN1 sends an LS to RAN4 and RAN4 can further decide if such test or requirement is needed for gNBs or not.

Proposal 11	For UEs that do not indicate a capability for beam correspondence with beamCorrespondenceWithoutUL-BeamSweeping ={1}, at least support omni-directional LBT or quasi-omni-directional LBT.

Proposal 12	If any enhancements to better enable multiple beam transmissions within a COT when LBT mode is used can be agreed now, it is to support Alt 1 in principle for TDM and SDM case where a single LBT at the beginning of the COT is performed with the definition of “cover” meaning omni-directional or quasi-omni-directional.

Proposal 13	RAN1 needs to decide on whether and how to specify directional LBT for single sensing beam case before further discussing multiple sensing beams.

Proposal 14	Do not continue to discuss Scheme 2-1 and Scheme 3 for receiver assisted channel access.

Proposal 15	Enhance the RSSI and CO measurement configuration (RMTC-Config) in Rel-16 to support NR unlicensed operation in FR2-2 in Rel-17. The enhancement includes:

	Extending the current ref-SCS-CP to include SCS 120, 480 and 960 kHz

	Introducing an indication of channel bandwidth for RSSI measurement. The enumeration of channel bandwidths should include the maximum and the minimum channel bandwidth and the intermediate channel bandwidths defined by RAN4.

The signalling details of the RRC configuration for RSSI and CO measurement should be decided by RAN2

Proposal 16	For RSSI and CO measurement in FR2-2, UE can assume the configured RSSI measurement resources are QCL-ed with Type-D to either the latest received PDSCH or the latest monitored CORESET.

Proposal 17	If RAN1 decides to support Scheme 1, the following enhancements on the current CSI reporting can be considered:

Proposal 18	Discuss Scheme 2-2 with explicit feedback in the context of Scheme 1.

Proposal 19	Conclude that Scheme 2-2 with implicit feedback is already supported by the current specs.

Proposal 20	For the maximum gap Y between initiating device and responding device in a COT sharing case for regions where sensing is needed, we support Option 3 with no need for gNB to specify or signal Y to the UE.

Proposal 21	Reuse Rel-16 framework for indication of CAT2 LBT in the DCI.

Proposal 22	Do not support Cat2 LBT for any of the use cases in 52.6 GHz to 71 GHz. It is not precluded to do CAT2 LBT in addition to CAT3 LBT requirements.

Proposal 23	For energy measurement in 8 µs deferral period, 5us observation slot is located at the end of the 8us deferral period like IEEE 802.11ad/ay.

Proposal 24	The minimum measurement duration X within a 5 µs observation slot can be left for implementation with the maximum value as 3µs

Proposal 25	Cell-specific system information indication of LBT ON/OFF is included in SIB1

i.	Define same DCI_1_0 sizes for both LBT on/off (licensed and unlicensed operation)

Proposal 26	Enabling use of LBT for contention exempt transmissions is indicated in SIB1. The type of LBT (CAT3 or CAT2 LBT) to be used can be left for implementation.

# 5	References

RP-202925, Revised WID: Extending current NR operation to 71 GHz

RP-193229, New WID on Extending current NR operation to 71 GHz

3GPP TR 38.808 Technical Report, "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on supporting NR from 52.6 GHz to 71 GHz (Release 17)" v1.0.0, December 2020.

EN 302 567 v2.2.1, Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum, Accessed on 2021-1-18 https://www.etsi.org/deliver/etsi_en/302500_302599/302567/02.02.01_60/en_302567v020201p.pdf,

3GPP TS 38.101-1 Technical Specification, “5G; NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone (3GPP TS 38.101-1 version 16.5.0 Release 16)”, Accessed on 2021-1-18  https://www.etsi.org/deliver/etsi_ts/138100_138199/13810101/16.05.00_60/ts_13810101v160500p.pdf

R1-2007983 Channel access mechanism; 3GPP TSG-RAN WG1 Meeting #103-e, 26th October to 13th November 2020

R1-2008976, Channel access mechanism for 60 GHz unlicensed operation, Huawei, HiSilicon; 3GPP TSG-RAN WG1 Meeting #103-e, 26th October to 13th November 2020

R1-2009760, FL summary#10 for channel access of 52.6GHz to 71GHz band, Moderator (Qualcomm); 3GPP TSG-RAN WG1 Meeting #103-e, 26th October to 13th November 2020

R1-2007984, Evaluation results for NR in 52.6 - 71 GHz, Ericsson

R1-2102081 Email discussion summary#3 for channel access for 52.6-71GHz, Moderator (Qualcomm);3GPP TSG-RAN WG1 Meeting 104-e, January 25th – February 5th, 2021

ERC REC (70)03 Relating to the use of Short-Range Devices (SRD) https://docdb.cept.org/download/25c41779-cd6e/Rec7003e.pdf

IEEE Std 802.11ad™-2012: IEEE Standard for Information Technology—Telecommunications and Information Exchange between Systems; Local and Metropolitan Area Networks—Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications; Amendment 3: Enhancements for Very High Throughput in the 60 GHz Band

IEEE Std 802.11™‐2020: IEEE Standard for Information Technology—Telecommunications and Information Exchange between Systems; Local and Metropolitan Area Networks—Specific Requirements Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications

R1-2104460, "Initial Access Aspects," Ericsson, RAN1#105-e, May 2021.

R1-2102793, “Channel Access Mechanisms”, Ericsson, RAN1#104b-e, April 2021.

R1-2104040, FL summary#3 for channel access mechanism for 52.6GHz to 71GHz, Moderator (Qualcomm), RAN1_104b, April 2021.

RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86, Sitges, December 2019

R1-2106193, Contribution summary of channel access mechanism for 52.6GHz-71GHz band, ver03, Moderator (Qualcomm Incorporated), RAN1_105e, May 2021.

5G NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone (3GPP TS 38.101-2 version 16.5.0 Release 16)

EN 303 753 v0.0.4, Wideband Data Transmission Systems (WDTS) for Mobile and Fixed Radio Equipment operating in the 57 - 71 GHz band; Harmonised Standard for access to radio spectrum, 2021-06

R1-2108554 FL summary for channel access of 52.6GHz to 71GHz band_ver03, Moderator (Qualcomm Incorporated), RAN1_106e, August 2021.

ARIB STD-B117 v1.0, “LOW POWER DATA COMMUNICATION SYSTEM/60 GHz-BAND WIRELESS LAN FOR VERY HIGH THROUGHPUT DATA COMMUNICATIONS,” Sept. 2016.

R1-2110540, FL summary of channel access mechanism for 52.6 GHz – 71 GHz band, ver 2, Moderator (Qualcomm Incorporated), RAN1#106bis-e, October 2021.