3GPP TSG RAN WG1 #107-e																			R1-2111473

e-Meeting, November 11th – 19th, 2021

Source: 	Intel Corporation

Title:	Discussion on physical layer aspects of small data transmission

Agenda item:	5.2

Document for:	Discussion and Decision

# Introduction

At the RAN1#106-e meeting, the following agreements and conclusions were made for CG-SDT [1].

Agreement

Multi-layer PUSCH transmission is not supported for CG-SDT.

Agreement

When SSB set indication is absent, UE assumes the SSB set includes all actually transmitted SSBs configured by SIB1.

Agreement

- RAN1 confirms that common PUCCH resources (i.e. those that are shared with non-SDT UEs) can also be used for HARQ-ACK feedback for Msg4 /MsgB and subsequent SDT transmissions.

- RAN1 thinks there is no need for any other PUCCH resources than common PUCCH resources shared with non-SDT UEs.

Agreement

BFD/BFR procedure is not required for SDT in Rel-17.

- FFS: whether or not to support reporting the beam change to gNB.

Agreement

For CG-SDT, the UE can assume the PDCCH carrying the DCI has the same DM-RS antenna port quasi co-location properties as for a SSB associated to the CG PUSCH transmission e.g. for detection of retransmission DCI in response to a CG PUSCH transmission.

Conclusion

No need to define UL/DL pattern type of validation rule specific for paired spectrum at least for non-RedCap UEs.

- FFS the case for RedCap UEs

Conclusion

It is RAN1’s common understanding that dynamic grant based retransmission has already been supported.

Conclusion

RA-SDT resource cannot be configured on non-initial BWP.

Conclusion

From RAN1’s perspective, there is no other L1 configuration for RA-SDT and CG-SDT to support subsequent data transmission.

Agreement

The pathloss for CG-SDT PUSCH power control can be determined by the measurement of selected SSB associated with the CG PUSCH.

Conclusion

- RAN1 cannot reach a consensus on whether to confirm RAN2 agreement that CG-SDT resource can be configured on separate SDT BWP.

- Capture the following in the LS: the concern is on the necessity.

Conclusion

- RAN1 cannot reach consensus on reusing CG-DFI mechanism for CG-SDT for operation in licensed band.

Agreement

- Mapping ratio of SSB to CG PUSCH is configured per CG configuration.

  - FFS whether to restrict the same value for all CG configuration and/or allow different value for different CG configurations.

- For the candidate value set of SSB to CG PUSCH mapping ratio, support at least {1, 2, 4, 8, 16}

  - FFS {1/8,1/4,1/2}

Agreement

- RAN1 confirms the working assumption in RAN2 that UE-specific search space is configured for UEs performing CG-SDT. This does not exclude the configuration of CSS for UEs performing CG-SDT.

- CORESET for UE performing RA-SDT should be a common CORESET.

Agreement

A CG PUSCH occasion is not valid if it overlaps with any valid PRACH occasion.

- FFS overlapping between CG PUSCH occasions and MsgA PUSCH occasion

In the contribution, we present our views on remaining issue for physical layer aspects of small data transmission.

# Discussion on remaining issues for CG-SDT

## Mapping between SSB and CG-PUSCH resource

For CG-SDT, association between CG resource and SSB is needed for proper multi-beam operation. At the RAN1#106b-e meeting, it was agreed that mapping ratio of SSB to CG PUSCH is configured per CG configuration. In addition, it was agreed to support at least {1, 2, 4, 8, 16} for the candidate value set of SSB to CG PUSCH mapping ratio. It is for further study whether to restrict the same value for all CG configuration and/or allow different value for different CG configurations [1].  Note that for the mapping ratio of SSB to CG PUSCH, when N < 1, each SSB index is mapped to 1/N consecutive valid CG PUSCH resources. If N ≥1, each valid CG PUSCH resource is associated with all the N SSB index.

In our view, it may not be desirable to additionally support one to many mapping between SSB to CG-PUSCH resource, where one SSB is associated with multiple CG-PUSCH resources. Given the fact that CG-PUSCH configuration is configured in RRC release message and is UE specific, transmission of CG-PUSCH would be contention free. In this case, it may not be necessary to allow UE to randomly select one CG-PUSCH resource among the configured CG-PUSCH resources associated with a single SSB. Hence, in our view, association of multiple CG-PUSCHs with a single SSB may not be needed for CG-SDT operation.

Further, given that different CG-PUSCH configurations may be targeted for different use cases, it may be more desirable to configure different mapping ratios for different CG-PUSCH configurations, which can provide good flexibility on the mapping between SSB and CG-PUSCH resource. Note that for a CG-PUSCH configuration, gNB may configure different number of DMRS resources, periodicity or amount of REs allocated for a CG-PUSCH resource. The same design principle can also apply for the mapping ratio between SSB and CG-PUSCH resource. For instance, a larger value of mapping ratio may be configured for a CG-PUSCH configuration so as to reduce latency for CG-PUSCH transmission.

Proposal 1

- For the mapping ratio of SSB to CG PUSCH resource

  - Different mapping ratios can be configured for different CG configurations.

  - Do not support mapping ratio < 1.

Note that if only one DMRS resource is configured for a CG-PUSCH configuration, multiple CG-PUSCH resources are multiplexed in a TDM manner. For CG-SDT operation, after UE selects an appropriate SSB and determines the CG-PUSCH resource for data transmission according to the mapping between SSB and CG-PUSCH resource, UE may need to wait for a few CG-PUSCH periods to send the uplink data. This may not be desirable in terms of latency, especially considering that typically CG-SDT operation would be limited to a short period of time.

To further reduce the latency for CG-SDT operation, it is more beneficial to support multiple CG-PUSCH occasions in a CG-PUSCH period for a CG-PUSCH configuration. Similar to MsgA PUSCH configuration as defined for 2-step RACH, multiple CG-PUSCH occasions can be multiplexed in TDM and FDM manner. Figure 1 illustrates one example of multiple CG-PUSCH occasions in a CG-PUSCH period.

Figure 1. Multiple CG-PUSCH occasions in a CG-PUSCH period

Proposal 2

- Multiple PUSCH occasions in a CG-PUSCH period are supported for a CG-PUSCH configuration.

To improve the coverage for PUSCH transmission during CG-SDT, repetition can be supported for CG-PUSCH transmission, which is similar to what was defined for CG-PUSCH configuration in Rel-15/16. Further, for CG-SDT operation, the repetitions are considered as a bundle of transmission occasions that are mapped to a same SSB.

One issue as brought up during the discussion was that when one of the CG-PUSCH repetitions is dropped due to invalidation rule, the number of repetitions for different CG-PUSCH resources for a CG-PUSCH configuration may be different, which would lead to unbalanced coverage performance for different CG-PUSCH transmissions. In our view, even with different number of repetitions associated with different SSBs, CG-PUSCH coverage performance can still be improved. In case of decoding failure of initial CG-PUSCH transmission, gNB may schedule the retransmission of CG-PUSCH to ensure the performance. Hence, special handling of the CG-PUSCH repetitions for CG-SDT operation is not needed.

Proposal 3

- Repetition of CG-PUSCH is supported.

  - The repetitions are considered as a bundle of transmission occasions that are mapped to a same SSB.

## Tx beam for PUCCH

At the RAN1#106b-e meeting, it was agreed that UE can assume the PDCCH carrying the DCI has the same DM-RS antenna port quasi co-location properties as for a SSB associated to the CG PUSCH transmission e.g., for detection of retransmission DCI in response to a CG PUSCH transmission [1].

For subsequent DL data transmission during CG-SDT, certain mechanisms may need to be defined to determine the Tx beam of PUCCH carrying HARQ-ACK response of the corresponding PDSCH. Following similar design principle as defined for 2-step RACH, during CG-SDT, UE transmits PUCCH carrying HARQ-ACK feedback in response to a PDSCH with a same spatial domain transmission filter in a same active UL BWP as a last PUSCH transmission.

Proposal 4

- For CG-SDT, UE transmits the PUCCH carrying HARQ-ACK feedback in response to a PDSCH with a same spatial domain transmission filter as a last PUSCH transmission.

# Conclusions

In this contribution, we presented our views on remaining issue for physical layer aspects of small data transmission. Further, we summarize the proposals as follows:

Proposal 1

- For the mapping ratio of SSB to CG PUSCH resource

  - Different mapping ratios can be configured for different CG configurations.

  - Do not support mapping ratio < 1.

Proposal 2

- Multiple PUSCH occasions in a CG-PUSCH period are supported for a CG-PUSCH configuration.

Proposal 3

- Repetition of CG-PUSCH is supported.

  - The repetitions are considered as a bundle of transmission occasions that are mapped to a same SSB.

Proposal 4

- For CG-SDT, UE transmits the PUCCH carrying HARQ-ACK feedback in response to a PDSCH with a same spatial domain transmission filter as a last PUSCH transmission.

# References

- Chairman’s notes, RAN1 #106bis e-Meeting, October 2021