3GPP TSG RAN WG1 #107-e	R1-2111513

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.10.2

Source: 	Intel Corporation

Title: 	Support for Case#6 and Case#7 Timing Alignment

Document for:	Discussion and decision

# Introduction

In this contribution, based on previous discussion and agreements, we further elaborate on timing alignment to support IAB-node’s simultaneous operation.

# Support for Case#6 and Case#7 Timing Alignment

## Case#6 Timing for Simultaneous MT-TX/DU-TX

Regarding Case#6 timing, RAN1#106bis-e has made the following agreements.

- RAN1 to down-select in RAN1#107-e one of the following for an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode:

  - Alt 1: no change or enhancement to the Rel-16 OTA synchronization specification is supported in Rel-17 for Case 6 timing.

  - Alt 2: in Rel-17 the Rel-16 OTA synchronization specification is updated to support OTA synchronization for an IAB-node operating solely in Case 6 timing during IAB-MT Tx.

    - FFS range of T_delta.

- NOTE: this is to provide a feasible solution to the RAN1#103-e agreement: “An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode”

We also list RAN1#103e and RAN1#106e agreement regarding Case#6 timing as below.

- RAN1#103-e agreement: “An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode”.

- RAN1#106-e agreement: “For Case 6 timing at a given IAB-node, the IAB-MT Tx timing is set by the node to the timing obtained by the node’s DL Tx.”

When an IAB-node operating in Case#6 timing,

- RAN1#103e agreement mainly focus on an IAB-DU DL TX timing control.

- RAN1#106e agreement mainly focus on an IAB-MT’s UL TX timing control, i.e., IAB-MT does not apply TA in UL TX control and directly sets UL TX timing to IAB-DU’s DL TX timing.

Regarding “an IAB-node operating solely in Case#6 timing during IAB-MT TX”, we don’t think that is a valid/possible definition due to the following reasons.

        - Parent node will always have a Case#1 UL RX reference point, to manage access UEs and Case#1 child IAB-nodes.

        - For an IAB-node based on OTA timing synchronization, Case#6 timing solely is not possible.

When an IAB-node (based on OTA timing synchronization) initially powers on, there is no IAB-DU DL TX timing. An IAB-MT needs to perform initial access/RACH under parent node’s Case#1 TA control before it obtains DL TX timing. In other words, Case#6 timing is possible only when an IAB node already has DL TX timing.

        - Even for an IAB-node with GNSS/GPS based DL TX timing, IAB-MT TX with Case#6 timing is controlled by the parent node and only switch to Case#6 timing with explicit indication.

RAN1#106e has agreed that an IAB-node is explicitly indicated by the parent node when Case#6 timing is performed at the IAB-node at least for specific time resources. An IAB-MT still needs to perform initial access/RACH under parent node’s Case#1 TA control, before it is explicitly indicated by its parent to switch to Case#6 timing.

Hence, “an IAB-node operating solely in Case#6 timing during IAB-MT TX” is not a valid/possible definition and an IAB-MT TX only switch to Case#6 timing from Case#1 timing with parent explicit indication.

We listed the operation details in our understanding of two alternatives as below.

In Alt. 1:

- During Case#6 mode, IAB-MT UL TX timing is not controlled by TA and always set to IAB-DU DL TX timing.

- During Case#6 mode, to obtain/maintain DL TX timing, parent node can transmit Case#1 TA (does not expect response) and Case#1 Tdelta for IAB-node to calculate DL TX timing:  TAcase12+Tdelta,case1=Tp.

In Alt. 2:

- During Case#6 mode, IAB-MT UL TX timing is not controlled by TA and always set to IAB-DU DL TX timing.

- During Case#6 mode, to obtain/maintain DL TX timing, parent node can transmit Case#6 Tdelta where Tdelta,case6= Tp2 for IAB-node to calculate DL TX timing: 2Tdelta,case6=Tp. Tdelta range needs to be changed accordingly.

Note that Tdelta,case6= Tp2 comes from:  TAcase62+Tdelta,case6=Tp and TAcase6=Tp.

The difference of Alt.1 and Alt.2 are the second bullets regarding obtain/maintain DL TX timing in Case#6 mode. Note that it is a common understanding for both alternatives that the IAB-MT’s UL TX timing ignores TA commands and is not controlled by TA commands in Case#6 timing. But in Alt.1, TA commands are still useful to decide IAB-node’s DL TX timing. Since Alt.1 is feasible and requires no specification change and Alt.2 needs additional specification change of Tdelta range, Alt. 1 is preferred.

Observation 1: An IAB-MT’s UL TX timing is not controlled by TA commands in Case#6 timing.

Observation 2: “An IAB-node operating solely in Case#6 timing during IAB-MT TX” is not a valid/possible definition and an IAB-MT TX only switch to Case#6 timing from Case#1 timing after parent explicit indication.

Observation 2: The operation details of the two Case#6 timing alternatives are listed as below.

In Alt. 1:

- During Case#6 mode, IAB-MT UL TX timing is not controlled by TA and always set to IAB-DU DL TX timing.

- During Case#6 mode, to obtain/maintain DL TX timing, parent node can transmit Case#1 TA (does not expect response) and Case#1 Tdelta for IAB-node to calculate DL TX timing:  TAcase12+Tdelta,case1=Tp.

In Alt. 2:

- During Case#6 mode, IAB-MT UL TX timing is not controlled by TA and always set to IAB-DU DL TX timing.

- During Case#6 mode, to obtain/maintain DL TX timing, parent node can transmit Case#6 Tdelta where Tdelta,case6= Tp2 for IAB-node to calculate DL TX timing: 2Tdelta,case6=Tp. Tdelta range needs to be changed accordingly.

Proposal 1: Support Alt 1 for Case#6 timing (No change or enhancement to the Rel-16 OTA synchronization specification is supported in Rel-17 for Case 6 timing).

## Case#7 Timing for Simultaneous MT-RX/DU-RX

Regarding Case#7 timing, RAN1#106bis-e has made the following agreements.

- Case 7 UL timing offset is indicated by the parent-node via MAC-CE.

- The granularity of Case 7 UL timing offset is the same as the UL TA granularity.

There is still one issue regarding the range of Case#7 UL timing offset. Although symbol-level alignment is supported for Case#7 timing, it will be beneficial to have an offset range that can also support slot-level alignment.

In Figure 2.1, we show an IAB-node’s timing relationship (slot-level alignment) with parent node operating in Case#7 timing:

- TAcase1: The original Case#1 TA at IAB-MT to make parent node operating at Case#1 timing mode

- TAcase7: The absolute Case#7 TA at IAB-MT to make parent node operating at Case#7 timing mode

- Tp0: parent-MT RX propagation delay

- Tp: IAB-MT RX propagation delay

The shifting offset from Case#1 TA to absolute Case#7 TA is defined as:

TAoffset, Case7=TACase1-TACase7

Figure 2.1: IAB-node timing relationship details with parent node operating at Case#7 timing

We derive TAoffset, Case7 value for slot-level alignment (which can be considered as the maximum offset value comparing with symbol-level alignment) with the following steps.

        - IAB-MT RX timing is Tp behind DL TX timing

IAB-MT TX Case#1 timing is TACase1 ahead of IAB-MT RX timing

        - IAB-MT TX Case#1 timing is TACase1-Tp ahead of DL TX timing

        - Parent-DU RX Case#1 timing is TACase1-2Tp ahead of DL TX timing

        - Parent-MT RX timing is Tp0 behind DL TX timing

Parent-DU RX Case#7 timing is the same as parent-MT RX timing

        - Parent-DU RX Case#7 timing is Tp0 behind DL TX timing

        - Parent-DU RX Case#1 timing is TACase1-2Tp ahead of DL TX timing (from Step1)

Parent-DU RX Case#7 timing is Tp0 behind DL TX timing (from Step 2)

        - Parent-DU RX Case#1 timing is TACase1-2Tp+Tp0 ahead of Parent-DU RX Case#7 timing

        - IAB-MT TX Case#1 timing is TACase1-2Tp+Tp0 ahead of IAB-MT TX Case#7 timing

        - TAoffset, Case7=TACase1-2Tp+Tp0

        - TAcase1=2Tp+Tg                               from Rel-16 IAB discussion [1] as in Figure 2.4

TAoffset, Case7=TACase1-2Tp+Tp0      from Step 3

        - TAoffset, Case7=Tp0+Tg

Figure 2.4: TAcase1/Tp/Tg relationship for Case#1 timing [1]

The Case#7 timing offset range can be designed according to TAoffset, Case7=Tp0+Tg.

RAN1#106e has also agreed that an IAB-node is explicitly indicated by the parent node when Case#7 timing is performed at the parent node and signaling details are FFS. There can be two alternatives for this explicit indication signaling.

- Alt. 1: TAoffset, Case7 is transmitted every time parent node switches from Case#1 timing to Case#7 timing and can be used as explicit indication from parent node.

- Alt. 2: TAoffset, Case7 is not transmitted every time parent node switches from Case#1 timing to Case#7 timing. Additional signaling is needed to explicitly indicate that parent node is performed at Case#7 timing.

As TAoffset, Case7=Tp0+Tg may not be static, we prefer Alt.1.

Observation 3: Although symbol-level alignment is supported for Case#7 timing, it will be beneficial to have an offset range that can also support slot-level alignment.

Proposal 2: The Case#7 timing offset range can be determined according to the maximum value with slot-level alignment:  TAoffset, Case7=Tp0+Tg ,where Tp0 is parent-MT RX propagation delay and  Tg is the switching gap between UL RX and DL TX at the parent node.

Proposal 3: There can be two alternatives for explicit indication by the parent node when Case#7 timing is performed at the parent node:

- Alt. 1: TAoffset, Case7 is transmitted every time parent node switches from Case#1 timing to Case#7 timing and can be used as explicit indication from parent node.

- Alt. 2: TAoffset, Case7 is not transmitted every time parent node switches from Case#1 timing to Case#7 timing. Additional signaling is needed to explicitly indicate that parent node is performed at Case#7 timing.

Alt. 1 is preferred.

# Conclusion

In this contribution, we discussed mechanisms to support Case#6 and Case#7 timing for IAB-node’s simultaneous operation.  It is summarized by the following observations and proposals.

Observation 1: An IAB-MT’s UL TX timing is not controlled by TA commands in Case#6 timing.

Observation 2: “An IAB-node operating solely in Case#6 timing during IAB-MT TX” is not a valid/possible definition and an IAB-MT TX only switch to Case#6 timing from Case#1 timing after parent explicit indication.

Observation 2: The operation details of the two Case#6 timing alternatives are listed as below.

In Alt. 1:

- During Case#6 mode, IAB-MT UL TX timing is not controlled by TA and always set to IAB-DU DL TX timing.

- During Case#6 mode, to obtain/maintain DL TX timing, parent node can transmit Case#1 TA (does not expect response) and Case#1 Tdelta for IAB-node to calculate DL TX timing:  TAcase12+Tdelta,case1=Tp.

In Alt. 2:

- During Case#6 mode, IAB-MT UL TX timing is not controlled by TA and always set to IAB-DU DL TX timing.

- During Case#6 mode, to obtain/maintain DL TX timing, parent node can transmit Case#6 Tdelta where Tdelta,case6= Tp2 for IAB-node to calculate DL TX timing: 2Tdelta,case6=Tp. Tdelta range needs to be changed accordingly.

Observation 3: Although symbol-level alignment is supported for Case#7 timing, it will be beneficial to have an offset range that can also support slot-level alignment.

Proposal 1: Support Alt 1 for Case#6 timing (No change or enhancement to the Rel-16 OTA synchronization specification is supported in Rel-17 for Case 6 timing).

Proposal 2: The Case#7 timing offset range can be determined according to the maximum value with slot-level alignment:  TAoffset, Case7=Tp0+Tg ,where Tp0 is parent-MT RX propagation delay and  Tg is the switching gap between UL RX and DL TX at the parent node.

Proposal 3: There can be two alternatives for explicit indication by the parent node when Case#7 timing is performed at the parent node:

- Alt. 1: TAoffset, Case7 is transmitted every time parent node switches from Case#1 timing to Case#7 timing and can be used as explicit indication from parent node.

- Alt. 2: TAoffset, Case7 is not transmitted every time parent node switches from Case#1 timing to Case#7 timing. Additional signaling is needed to explicitly indicate that parent node is performed at Case#7 timing.

Alt. 1 is preferred.

# References

[1] R1-1905842, “LS on clarification of OTA timing alignment for IAB”, Xi’an, China, April 2019.