3GPP TSG RAN WG1 #107-e			                                   R1-2111620

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.8.1.1

Source: 	CMCC

Title: 	Discussion on enhancements on PUSCH repetition type A

Document for:	Discussion and Decision

Introduction

In the RAN1 #106bis-e meeting, the enhancements on PUSCH repetition type A were discussed. The discussion focused on the maximum repetition number and the repetition counting on the basis of available slot. Several agreements have been achieved [1].

In this contribution, we provide our views on the enhancements of PUSCH repetition type A, especially on increasing the maximum number of repetitions and the number of repetitions counted on the basis of available slots.

Discussion

2.1 Increasing the maximum number of repetitions

In the last meeting, 3 issues were discussed during the meeting, the value of the maximum number of repetitions, RRC parameters to be extended for supporting the increased maximum number, and DCI formats supporting the repetition factors indicated/configured via TDRA lists.

RRC parameters to be extended for supporting the increased maximum number

In Rel-16, the numberOfRepetitions was introduced and the repetition factor was enhanced to maximum 16. But the RRC based configurations of repK and pusch-AggregationFactor remains maximum repetition factor of 8, the same as Rel-15. The parameter numberOfRepetitions could cover both CG-PUSCH and DG-PUSCH and can overwrite the repetition number configured by both repK and pusch-AggregationFactor. Then there is no need to further enhance the repetition factors of repK and pusch-AggregationFactor.

And in the last meeting, DCI format 0_1 and 0_2 was agreed to support the PUSCH repetition with the increased maximum repetition number configured in TDRA lists.

Agreement - DCI format 0_1 and DCI format 0_2 support Rel-17 PUSCH repetition Type A with the increased maximum repetition numbers configured in TDRA lists.

Then the increased maximum repetition number could be supported based on current agreements and specification. There is no need to further enhance other parameters, such as repK and pusch-AggregationFactor, for the increased maximum repetition number.

Observation 1:

Considering that numberOfRepetitions can fully support the function of increased maximum number of repetitions for both CG- and DG-PUSCH, the motivation to further enhance the pusch-AggregationFactor and repK is not strong.

DCI formats supporting the repetition factors indicated/configured via TDRA lists

For the specific DCI formats which could be used to support the indication of increased maximum repetition number, it was discussed during the last meeting. And an detailed analysis table was proposed by the FL. Only DCI format 0_1 and DCI format 0_2 were agreed to support for both CG and DG-PUSCH. Most companies proposed not to extend the repetition indications to the DCI format 0_0. For the numberOfRepetitions, it seems that the group do not have a consistent understanding about the behavior of Type 1 CG-PUSCH. From our understanding, the type 1 CG cannot be supported by the parameter of numberOfRepetitions. Then the DCI format 0_1 and 0_2 cannot support the type 1 CG repetitions. Despite of that, current Rel-17 can support the DG and CG PUSCH, there is no need to further enhance other parameters. As DCI format 0_1 and 0_2 could support DG-PUSCH with numberOfRepetitions, there is no need to further enhance the pusch-AggregationFactor as a fallback mode. And it is similar for the DCI format 0_1 and 0_2 for the Type 2 CG-PUSCH. Then there is no need to enhance repK for the Type 2 CG-PUSCH activated by DCI format 0_1 and 0_2.

Proposal 1:

No other DCI formats other than DCI format 0_1 and 0_2 is specified to support the increased maximum repetition number in Rel-17.

In the last meeting, FL’s proposals are raised as below,

FL proposal 1 to Issue#1-2: Agree the following as a package: - Rel-17 does not support up to 32 repetitions for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0. - Rel-17 does not support up to 32 repetitions configured by Rel-17 pusch-AggregationFactor for DG-PUSCH scheduled by DCI format 0_1/0_2. - For Type 1 CG-PUSCH and Type 2 CG-PUSCH activated by DCI format 0_1/0_2, select one from the following two alternatives: - Alt 2: - Rel-17 supports up-to-32 repetitions configured by Rel-17 repK for Type 1 CG-PUSCH. Rel-17 does not support up-to-32 repetitions configured by Rel-17 numberOfRepetitions for Type 1 CG-PUSCH. - Note: The TDRA list for Type 1 CG-PUSCH is kept un changed (i.e., use the TDRA list for DCI format 0_0). - Note: Need to introduce repK-r17. - Rel-17 supports up-to-32 repetitions configured by Rel-17 repK for Type 2 CG-PUSCH activated by DCI format 0_1/0_2. - Alt 3-b: - Rel-17 does not support up-to-32 repetitions configured by Rel-17 repK for Type 1 CG-PUSCH and for Type 2 CG-PUSCH activated by DCI format 0_1/0_2. - Rel-17 supports up-to-32 repetitions configured by Rel-17 numberOfRepetitions for Type 1 CG-PUSCH, where the TDRA list for DCI format 0_1 or 0_2 is reused. - Note: DCI format 0_0 field bit widths are kept unchanged. - Note: Need a mechanism to change the TDRA list for Type 1 CG-PUSCH, from the one for DCI format 0_0 to the one for DCI format 0_1 or 0_2.

The first two bullets is reasonable, since DCI format 0_0 as a fallback DCI should compatible to legacy UEs. Then no new function should be introduced such as enhanced maximum repetition number. For the 2nd bullet, as the numberOfRepetitions could be enhanced with maximum repetition number and supported by the DCI based indications, there is no need to enhance the Rel-15 RRC parameters. And the Rel-16 has already enhance the maximum repetition numbers to 16 but without enhancement to the pusch-AggregationFactor. Then the enhancement for pusch-AggregationFactor is not clear.

Proposal 2:

Support the first two bullets,

- Rel-17 does not support up to 32 repetitions for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- Rel-17 does not support up to 32 repetitions configured by Rel-17 pusch-AggregationFactor for DG-PUSCH scheduled by DCI format 0_1/0_2.

For the Type 1 CG-PUSCH, as it only reuse the TDRA table configured for DCI format 0_0, it can only reuse the repK and pusch-AggregationFactor for the indication of repetition factor. Alt 2 is to enhance the maximum repetition number through repK for Type 1 CG-PUSCH. And consequently, as the Type 2 CG-PUSCH also supports to use the repK. The enhanced repK also applies to Type 2 CG-PUSCH activated by DCI format 0_1/0_2, when the numberOfRepetitions is not configured. Alt 3-b does not enhance the parameter repK, but changes the associated TDRA table. Either way could work to support enhanced maximum repetition number for Type 1 CG-PUSCH. But it seems that increasing certain numbers in repK has less impact to the specification. For the Alt 3-b, Type 1 CG-PUSCH will associate with different tables for Rel-16 and Rel-17.

Observation 2:

Compared with Alt 3-b, it seems that Alt 2 has less impacts to the specification.

In the TDD configuration of 7D1S2U, the maximum 16 repetitions could support 4 actual repetitions. And when the maximum repetition number increased to 32 as in the working assumption, actual 8 repetitions can happen. However, when the available slot based repetition is configured, 16 and 32 actual repetitions could happen, which is the most direct and efficient way to execute the repetitions in TDD system. From the perspective of FDD system, since the uplink slots are consecutive, there is almost no difference between counting on the basis of available slot and consecutive slots. Then the counting on the basis of available slots could also be used for the FDD system. Considering the low efficiency of consecutive slot counting method, the repetition counting on the basis of available slots should be used as much as possible. It is proposed that this feature should be as mandate feature of CE UE capabilities.

Observation 3:

Though increase the repetition number on the basis of consecutive slot counting could increase the actual repetition number, it is not efficient in the TDD systems.

Proposal 3:

The counting of repetitions based on available slots should be used as much as possible. Counting on the basis of available slots for repetition should be as mandate feature of CE UE capability.

2.2 The definition of available slots for the PUSCH type A repetition

In the last meeting, the 2-step procedure was agreed for the Rel-17 PUSCH repetition counted on the basis of the available slots.

Agreement Take Option 1-B as an agreement for the procedure of Rel-17 PUSCH repetitions counted on the basis of available slots. - Alt 1-B consisting of two steps - Step 1: Determine available slots for K repetitions based on RRC configuration(s) in addition to TDRA in the DCI scheduling the PUSCH, CG configuration or activation DCI - Step 2: The UE determines whether to drop a PUSCH repetition or not according to Rel-15/16 PUSCH dropping rules, but the PUSCH repetition is still counted in the K repetitions. - FFS: Rel-17 PUSCH dropping rules are also applied if introduced in other WI(s) Agreement - Only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the determination of available slots. - Any other RRC configuration is not considered for the determination of available slots.

It was agreed that only tdd-UL-DL-ConfigurationCommon, tdd-UL-DL-ConfigurationDedicated and ssb-PositionsInBurst are considered for the available slots. But the dynamic indicated SFI could also change number of available uplink slots. And the miss detection of SFI could induce different understanding between UE and gNB. How to deal with the SFI indication should be clarified.

Proposal 4:

The operation to deal with SFI indication should be clarified or solved in the 2nd step of dropping rules.

HD-FDD RedCap UEs

It was observed that the due to the half-duplex capability of the HD-FDD Redcap UE, additional collision rules are being defined in the AI of RedCap [2]. And it may have some impact for the 2 step procedure of the available slot repetitions. And it was concluded that all types of UEs are taken into account in the scope of Rel-17 CovEnh WI. And the collision handling between PSUCH and SSB for HD-FDD RedCap UEs in the CE WI depends on the outcome of RedCap WI. Further definition or clarification of the 2-step procedure should be done when RedCap WI has clear conclusions.

Proposal 5:

Further definition or clarification of the 2-step procedure of available slot based PUSCH repetition type A should be discussed when RedCap WI has clear conclusions.

Conclusions

In this contribution, we provide our views on the enhancements of PUSCH repetition type A. The observations and proposals are as below.

Observation 1:

Considering that numberOfRepetitions can fully support the function of increased maximum number of repetitions for both CG- and DG-PUSCH, the motivation to further enhance the pusch-AggregationFactor and repK is not strong.

Observation 2:

Compared with Alt 3-b, it seems that Alt 2 has less impacts to the specification.

Observation 3:

Though increase the repetition number on the basis of consecutive slot counting could increase the actual repetition number, it is not efficient in the TDD systems.

Proposal 1:

No other DCI formats other than DCI format 0_1 and 0_2 is specified to support the increased maximum repetition number in Rel-17.

Proposal 2:

Support the first two bullets,

- Rel-17 does not support up to 32 repetitions for DG-PUSCH scheduled by DCI format 0_0 and for Type 2 CG-PUSCH activated by DCI format 0_0.

- Rel-17 does not support up to 32 repetitions configured by Rel-17 pusch-AggregationFactor for DG-PUSCH scheduled by DCI format 0_1/0_2.

Proposal 3:

The counting of repetitions based on available slots should be used as much as possible. Counting on the basis of available slots for repetition should be as mandate feature of CE UE capability.

Proposal 4:

The operation to deal with SFI indication should be clarified or solved in the 2nd step of dropping rules.

Proposal 5:

Further definition or clarification of the 2-step procedure of available slot based PUSCH repetition type A should be discussed when RedCap WI has clear conclusions.

References

- [1] Draft Report of 3GPP TSG RAN WG1 #106bis-e, e-Meeting, October 11th – 19th, 2021

- [2] RP-211991, CMCC, Discussion on the HD-FDD type UE in NR coverage enhancements, 3GPP TSG RAN Meeting #93-e, September 13 - 17, 2021