3GPP TSG RAN WG1 Meeting #107-e														        R1-2111649

e-Meeting, November 11th – 19th, 2021

Agenda item:		8.11.1.1

Title:						NR Sidelink Resource Allocation for UE Power Saving

Source:				Fraunhofer HHI, Fraunhofer IIS

Document for:		Discussion

# Introduction

The work item for NR V2X was approved in RAN#86 and revised in RAN#90e [1], and the following objectives were identified in relation to resource allocation enhancements for reducing power consumption:

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

2. Resource allocation enhancement:

- Specify resource allocation to reduce power consumption of the UEs [RAN1, RAN2]

  - Baseline is to introduce the principle of Rel-14 LTE sidelink random resource selection and partial sensing to Rel-16 NR sidelink resource allocation mode 2.

  - Note: Taking Rel-14 as the baseline does not preclude introducing a new solution to reduce power consumption for the cases where the baseline cannot work properly.

  - This work should consider the impact of sidelink DRX, if any.

Based on these objectives, the following agreements and working assumptions were made in the previous meeting [2]:

- When UE performs periodic-based and contiguous partial sensing schemes in a mode 2 Tx pool with periodic reservation for another TB (sl-MultiReserveResource) enabled, - For a resource (re)selection procedure triggered by periodic transmission (Prsvp_TX≠0) in slot n, TA and TB for the CPS monitoring window is defined according to one of the followings: - n+TA is M logical slots earlier than slot ty0SL, and n+TB is Tproc,0SL+Tproc,1SL slots earlier than ty0SL, where ty0SL is the first slot of the selected Y candidate slots of PBPS, and Tproc,0SL, Tproc,1SL are in units of physical time/slots. - By default, M is 31 unless (pre-)configured with another value. - For the periodic sensing occasion(s) (PSO(s)) that a UE needs to additionally monitored in PBPS, it shall be (pre-)configured jointly for all Preserve values. - The UE is not required to monitor PSOs earlier than n–T0 if the UE is triggered to do resource (re)selection in slot n, where T0 is (pre-)configured - When UE performs at least contiguous partial sensing in a mode 2 Tx pool for a resource (re)selection procedure triggered by aperiodic transmission (Prsvp_TX=0) in slot n, TA and TB for CPS monitoring window and a candidate resource set (SA) is initialized according to potentially one of the following approaches (final decision in RAN1#107-e). Other approaches are not precluded and the details in each approach can still be updated. - Approach 1: (SA is initialized based on at least slots with PBPS and/or CPS results and guarantee a minimum of M slots for CPS) - The UE selects a set of Y’ candidate slots with corresponding PBPS and/or CPS results (if available) within the RSW. - FFS how to handle the case if the total number of Y’ candidate slots is less than a (pre-)configured threshold Y’min without dropping the aperiodic transmission - FFS whether the Y’ candidate slots for aperiodic transmission is the same as the Y candidate slots in PBPS for periodic transmission of another TB(s) - FFS whether/how to prioritize/select resources based on partial sensing results. - FFS: How to select Y’ in case of CPS only - Candidate resource set (SA) is initialized to the set of all single-slot candidate resources in the selected Y’ candidate slots. - For the CPS monitoring window [n+TA, n+TB]: - TA and TB are both selected such that UE has sensing results for a minimum of M consecutive logical slots before ty0, where ty0 is the first slot of the selected Y’ candidate slots. - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-)configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - FFS: RSW in case of CPS only - Approach 2: (SA is initialized based on all candidate single-slot resources and guarantee a minimum of M slots for CPS) - Candidate resource set (SA) is initialized to the set of all candidate single-slot resources in [n+TB+Tproc,0+Tproc,1, n+T2], where TB is selected by the UE such that length of [n+TB+Tproc,0+Tproc,1, n+T2] ≥ T2min. - Tproc,0, Tproc,1 are in units of physical time/slots - FFS whether/how to prioritize/select resources based on partial sensing results (if PBPS is performed). - For the CPS monitoring window [n+TA, n+TB]: - TA = X - FFS value X for TA including X=1 and negative value - TB is selected such that UE has sensing results for a minimum of M consecutive logical slots before the start of (n+TB+Tproc,0+Tproc,1). - FFS: By default, M is 31 unless (pre-)configured with another value, or M is (pre-)configured based on transmission priority - FFS the range of (pre-) configured M from a TBD lowest value up to 30 - FFS: how to handle the case when the minimum M slots for CPS cannot be guaranteed - Approach 3: (independent approach for different case) - When UE additionally performs periodic-based partial sensing in the resource pool, the above Approach 1 applies. - When UE does not perform periodic-based partial sensing in a resource pool that does not allow resource reservation for another TB, the above Approach 2 applies. Working Assumption - In a resource pool (pre-)configured to enable partial sensing, when UE is configured with partial sensing by its higher layer, the resources for which the UE performs re-evaluation and/or pre-emption checking are for the initial transmission and retransmissions of every TB according to Rel-16 specification based on partial sensing results. - Same as in Rel-16, for periodic transmission, re-evaluation check is not applied to the resources that have been signalled in current period or previous periods, except that it is up to UE implementation whether to apply re-evaluation check to the resources in non-initial reservation period that have been signalled neither in the immediate last nor in the current period. - The resource in the main bullet is the set of resources (r0,r1,r2,…) and/or the set of resources (r0',r1',r2',…) for re-evaluation and/or pre-emption checking, respectively, which has been agreed in RAN1 #106-e.

The power saving feature is expected to be required for battery-based UEs, for example, pedestrians and cyclists as well as public safety UEs using V2X applications. In this contribution, we discuss approaches for power saving on NR sidelink based on principles from Rel-14 LTE sidelink, as well as based on new solutions, which leverage mechanisms introduced in NR Rel-16.

# Contiguous Partial Sensing

In the previous meeting [2], there were discussions regarding the sensing and selection windows for contiguous partial sensing. The different scenarios described are as follows:

- Contiguous partial sensing for a periodic transmission in a resource pool with periodic transmissions enabled,

- Contiguous partial sensing for an aperiodic transmission in a resource pool with periodic transmissions enabled,

- Contiguous partial sensing for an aperiodic transmission in a resource pool with periodic transmissions disabled.

While an agreement was made with regards to the first scenario, the remaining two were merged into one agreement with three possible approaches listed. With regards to the definition of the sensing window [n+TA, n+TB], it was agreed in previous meetings that the values of TA and TB depend on different operating scenarios or conditions such as the nature of the transmission, its remaining PDB etc.

In Rel-16, TRIV and FRIV parameters in the SCI were used to reserve resources for retransmissions, and the resource reservation period parameter was used to indicate the periodicity for periodic traffic [3]. Since a UE would decode the 1st stage SCI when carrying out sensing, it can use these parameters to adapt its sensing window for contiguous partial sensing to estimate the probable resource reservations and identify occupied resources. This would allow the UE to monitor shorter intervals of time and would be capable of predicting the sensing information in time slots where it did not sense, using the available resource reservation information.

Depending on the UE's need to save power and the amount of sensing information needed for successful transmissions, a trade-off can be made by the UE by identifying an ideal sensing detection rate. This can be defined as a percentage of time slots, where future resources are reserved by other UEs using SCIs that the UE is able to detect within a defined monitoring window size. For example, a higher detection rate would mean that the UE carries out sensing over the maximum configured duration of the sensing window, but at the expense of saving power, and vice versa. An example of this can be seen in Fig. 1. Multiple such detection rates can be configured, and selected in a per resource pool manner, so that the sensing window selected by a UE is known to other UEs using the resource pool.

Figure 1: Depiction of adaptable sensing window for contiguous partial sensing.

In order to understand the advantages of using such a scheme, we have modelled the TRIV formula and depicted the detection rate for different monitoring window sizes. It can be seen in Fig. 2, that by reducing the monitoring window size, the number of time slots that the UE can successfully identify as reserved based on received SCIs decrease. The blue dots indicate the time slots that the UE can successfully detect resource reservations, while the red dots indicate the instances that the UE cannot extrapolate. However, even with a window size of 25 time slots, UEs can achieve a detection rate as high as 96.6%, while a shorter window size of 15 time slots would permit the UE to achieve only a detection rate of 72.4%. This is primarily due to the design of the TRIV formula, which shows that the gain in the detection rate is much lesser as the window size is increased past half the window size, since it is at the edge of the “triangle” seen in the figures below. This would allow the UE to extrapolate resource reservations in future time slots without actually being required to monitor them.

| CPS Monitoring Window Size: 15 Detection Rate: 72.4% | CPS Monitoring Window Size: 20 Detection Rate: 87.4% | CPS Monitoring Window Size: 25 Detection Rate: 96.6% |
| --- | --- | --- |

Figure 2: Modelling of different CPS monitoring window sizes.

Based on this analysis, the maximum size of the CPS monitoring window can be maintained to be 32 time slots based on Rel-16, however, the minimum size should be dependent at least on the priority of the transmission and the remaining PDB and should be adaptable based on these parameters and the intended detection rate. This value cannot be left up to UE implementation since it could result in performance degradation and thus needs to be (pre-)configured within a range of values that a UE can select from.

Proposal 1: For contiguous partial sensing, we propose that the size of the CPS monitoring window should be (pre-)configured based on the priority of the transmission and the remaining PDB.

# Random Resource Selection without Sensing

In RAN1#104bis-e [4], it was agreed to support random resource selection as a power saving scheme for both periodic and aperiodic transmissions. It was especially beneficial in LTE when power saving UEs were expected to only transmit, and not receive, thereby eliminating the power-consuming sensing and reception procedures. This is similar to Type A and Type B UEs, where these UEs are not capable of performing reception of any SL signals and channels (with the exception of PSFCH and S-SSB for Type B UEs).

The outstanding issues related to random resource selection are whether only Type A UEs are restricted to using random resource selection, and its subsequent effect on the resource pool utilization.

## Resource Pool Restrictions for Random Resource Selection

It was agreed that resource pools in Rel-17 can be configured to enable random resource selection only, or in combination with full or partial sensing. With regards to mixed-mode resource pools, enabling both random resource selection and sensing in the same resource pool would place UEs carrying out random resource selection at a disadvantage, since these UEs could select resources that have already been reserved by UEs that have carried out sensing. This would result in a high risk of resource collisions, and can also trigger resource re-evaluation in other sensing UEs, since these UEs have determined that a resource that they had reserved has been taken up by a power saving UE carrying out random resource selection.

Based on these issues, it would be possible to restrict UEs that carry out random resource selection from using mixed resource pools with random resource selection enabled only for those transmissions that are above a pre-defined priority threshold. This can be implemented similar to how the priority threshold for pre-emption is defined in Rel-16. This could apply to either Type A or D UEs, as long as they do not select resources based on sensing procedures within the given resource pool.

UEs that carry out random resource selection for transmissions that are of low priority can use resource pools with only random resource selection enabled. This would avoid any resource collisions with other sensing UEs and the issues of triggering re-evaluation. Another issue that has to be borne in mind is that resource pools with only random resource selection configured cannot be used by Rel-16 UEs, since it would result in further resource collisions, and hence can be used only by these Rel-17 power saving UEs.

The use of the exceptional pool for these low priority transmissions is also not an optimum solution since the exceptional pool is meant to be used by all UEs with or without sensing capabilities for special cases, and forcing low priority transmissions here would only result in more collisions due to the high traffic load on the exceptional resource pool.

Proposal 2: Due to the risk of collisions and unnecessary triggering of resource re-evaluation, we support that UEs carrying out random resource selection are permitted to use a mixed resource pool only when transmitting a TB with a priority that is higher than a pre-defined threshold.

Proposal 3: We propose that UEs carrying out random resource selection and transmitting low priority transmissions should use a resource pool that is configured with random resource selection only.

Another aspect that has to be considered regarding the resource pool design to be used for random resource selection is the usage of the PSFCH. Since Type A UEs cannot perform reception of the PSFCH, it does not make sense for such a resource pool to have PSFCH configured. Hence, in order to avoid resource under-utilization, we propose that Type A UEs carrying out random resource selection transmit in resource pools with PSFCH disabled.

Proposal 4: We propose that resource pools with random resource selection enabled are configured with PSFCH disabled.

## Transmission Restrictions for Random Resource Selection

Since HARQ feedback-based transmissions are not possible for Type A UEs, they can resort to blind retransmissions to maintain the reliability of their transmissions. However, since the maximum number of blind retransmissions that a UE can carry out are up to 32, as per Rel-16, it should not result in the UEs transmitting each packet 32 times in order to ensure their reliability. This will result in the flooding of the resource pool and lead to further collisions. Hence, we propose that the maximum number of retransmissions to be carried out by Type A UEs are limited based on the priority of the transmission. In this case, packets with high priority can be retransmitted up to a maximum of 32 times, while low priority transmissions can be restricted to a lower number. This will ensure that reliability is maintained for transmissions by Type A UEs, without resulting in collisions due to a high number of retransmissions.

Proposal 5: For UEs carrying out random resource selection and are not capable of PSFCH reception, we propose to restrict the maximum number of blind retransmissions to be carried out based on the priority of the transmission.

# Sidelink DRX

One of the objectives of the Rel-17 WID [1] is the introduction of SL DRX, which should enable a UE to configure discontinuous reception on the sidelink based on RAN2 inputs. Having DRX on NR sidelink allows power saving UEs to use the battery more efficiently. DRX operation facilitates UE power reduction through RF chain circuit ON/OFF switching, where UEs can receive only during the ON duration of the DRX cycle to save power.

For SL DRX to work efficiently, the DRX configurations between the gNB and the UEs need to be aligned when operating in Mode 1. Similarly, in Mode 2, DRX alignment is required between UEs to transmit to and receive from each other. Another essential aspect of SL DRX is the alignment between partial sensing windows and the SL DRX durations.

## Alignment between Partial Sensing and SL DRX Durations

In this case, the selection windows and the DRX ON durations need to be aligned for UEs to successfully carry out the sensing and resource (re-)selection procedure while at the same time conserving power. This requirement is further accentuated when the UE is carrying out partial sensing, where the candidate resources need to align with the SL DRX durations for UEs to successfully receive the PSCCH. If these resources are to be allowed to be scheduled outside the DRX active period, it will diminish the power saving gains that the UE could achieve by using SL DRX in the first place, especially since sensing is one of the most power-hungry processes at a UE.

Regarding the discussion on the candidate resources being within the active time of the RX UE [5], we prefer to have at least a subset of these resources within the DRX ON duration, or what was referred to as the current active time. The future active time, which is essentially the duration of the inactivity timer after the DRX ON duration, is activated by the RX UE only on receiving an initial transmission. Hence for the initial transmission at least, the candidate resources need to be within the DRX ON duration for the RX UE to be able to receive the transmission.

Proposal 6: We propose that the candidate resources are aligned with the SL DRX ON duration in order to maximize the power saving gains.

Based on the discussions in RAN2 [6], a Mode 2 UE would receive SL DRX configurations from the TX UE in the case of unicast transmissions, or would use pre-configured SL DRX configurations in the case of broadcast and groupcast transmissions. In the case of unicast transmissions, the TX UE would carry out partial sensing based on its configured active durations, and provide this DRX configuration to the RX UE. Hence it would be the responsibility of the UE to align its partial sensing occasions according to the received DRX configuration. In order to carry out partial sensing for groupcast or broadcast transmissions, the UE has to align its partial sensing intervals with the pre-configured DRX active durations by selecting the most relevant set of pre-configured DRX configurations.

Proposal 7: In the case of Mode 2, we propose that the RX UE aligns its partial sensing occasions according to the received SL DRX configurations, either from the TX UE in the case of unicast, or from pre-configuration in the case of groupcast or broadcast transmissions.

## Impact of Reduced Sensing due to SL DRX

Another challenge with using SL DRX is the reduced sensing time a UE has to determine resources for its transmissions. Having intermittent ON/OFF durations might affect the ability of UEs operating in Mode 2 to reliably select resources as the UE's sensing duration is decreased. Therefore, a trade-off between the reliability requirements and power saving needs to be considered such that the UEs can adapt sleep and awake timing depending on the required reliability of the use cases.

An option for UEs using SL DRX to obtain reliable sensing results would be for the UE to receive assistance information for the time intervals when it was not able to carry out sensing. The UE would need to fill in the gaps due to the SL DRX inactive period using the sensing results obtained from the assistance information messages.

Proposal 8: In the case of inadequate sensing results, we propose that the UE uses assistance information messages in order to obtain the required sensing information for carrying out reliable resource selection.

# Possible Further Solutions for Power Saving

Apart from the aforementioned solutions to reduce power consumption, we have described a few solutions in this section that could go one step further in ensuring power saving, and negate the drawbacks of the baseline solutions.

## Assisted Resource Allocation

An aspect that could improve the power saving abilities of UEs, aside from the baseline solutions carried forward from LTE, is by using inter-UE coordination. It was already found to be feasible and beneficial in terms of enhancing the reliability and reducing the latency of the system [4]. It can also be coupled with partial sensing and SL DRX to increase the reliability of resource allocation, while at the same time reducing power consumption.

Inter-UE coordination can be carried out by RSUs, group lead UEs within groups, or even another UE to provide a set of resources to power constrained UEs in order to use these resources for their own transmissions. These resources could be a pool of reserved resources from which the UEs can select randomly or specific resources that UEs can directly use for their transmissions. These UEs can also merge their limited sensing results with the assistance information received in order to determine the best available resources for their transmissions. This would be advantageous when UEs are restricted to only limited sensing results due to SL DRX configurations.

These assistance messages can be triggered by request, as described in our accompanying contribution [7], based on which any of these assistance entities can transmit resources pro-actively. This would enable low power UEs to reduce, or skip entirely, the sensing and resource selection process, resulting in significant power saving gains.

A comparative analysis of this solution with the other baseline solutions can be seen in the table below:

|  | Assisted Resource Allocation | Partial Sensing | Random Resource Selection |
| --- | --- | --- | --- |
| Collision Probability | Low, since UEs carrying out full sensing can provide power saving UEs with resources for transmission. | Medium, since UEs might not have complete sensing results to carry out resource selection without collisions. | Medium to high, depending on the resource pool configurations and congestion scenario. |
| Power Saving | High, since UEs can completely avoid carrying out sensing, and receive selected resources for their own transmissions. | Medium, since UEs still have to carry out partial sensing. | High, since UEs are not carrying out the sensing process. |
| SL DRX Alignment | Largely unaffected, since UEs just need to receive the assistance information message during the DRX ON duration. | Direct impact, since the DRX durations need to be aligned with partial sensing intervals for obtaining sensing results. | Unaffected. |
| Overhead | Low to medium, especially by using an implicit triggering mechanism, where the resources are provided using the TRIV/FRIV format. | Low to medium, if used in conjunction with DRX. None, if used without DRX. | None. |

Table 1: Comparative analysis of assisted resource allocation with baseline power saving solutions.

Proposal 9: We propose to utilize assistance information from assistance entities, providing a set of resources that power saving UEs can use for increased reliability in their resource selection procedure.

## Location-based Power Saving

Rel-17 is targeted at VRUs, public safety UEs and commercial UEs that require power consumption to be minimized. These particular UEs are actively transmitting and receiving only when in a specific defined location. For example, pedestrian UEs are required to be active while close to roads and intersections and not when indoors. Similarly, public safety UEs need to be active only when at emergency locations and not otherwise.

Inactive P-UEs

Inactive P-UEs

Active P-UEs

Active P-UEs

Figure 3: Depiction of active and inactive P-UEs based on their location – active outdoors, inactive indoors.

In order to leverage this aspect, it is possible to restrict a power saving UE's activity based on its location. By defining pre-configured locations or regions, these UEs wake up and carry out transmissions only when they are within these regions. It is also possible to utilize the DRX feature to wake up in these regions and combine it with assisted resource allocation to maximize the power saving capabilities.

This solution works on top of any of the baseline or other solutions described. Enabling location-based activation and de-activation of sidelink operations for PS UEs would ensure a higher degree of power saving, while not having to sacrifice on any of the features of these solutions.

Proposal 10: Since power saving UEs are required to be active based on their location, we propose to enable these UEs to wake up and carry out transmissions only when they are in a pre-configured region.

## Operational Sidelink Bandwidth Adaptation

A SL-BWP is configured with different SL transmission and reception resource pools. Typically, UEs have to monitor and carry out sensing on all configured receive resource pools within an active SL-BWP. One way to reduce power consumption is that the power saving UEs indicate their preferred transmit and/or receive resource pools based on their functionalities. This way, UEs are not required to sense the resource pools that are not configured, resulting in reduced power consumption.

The agreement made in RAN1#103e [8] regarding the possibility to configure a resource pool for partial sensing only facilitates this, where the resource pools to be used by power saving UEs can be configured to span only a smaller bandwidth within the active SL-BWP. Since they are defined at a system level, other UEs are also aware of the resource pools being used by power saving UEs. This would enable other transmitter UEs to adapt their transmission bandwidth and use the configured resource pools for power saving UEs in order to ensure that these UEs can receive the data.

Another method to reduce power consumption is to employ a similar adaptive sensing solution, and extend it to the frequency domain. Shortened sensing frequency regions that can be contiguous or discontiguous across the resource pool can be used by the UE to carry out sensing and extrapolate the remaining sensing information dependent on a sensing detection rate.

Proposal 11: We propose that power saving UEs should support bandwidth adaptation by operating over shortened frequency regions or by carrying out partial sensing over a smaller bandwidth.

# Conclusions

The following proposals have been made in this document:

Proposal 1: For contiguous partial sensing, we propose that the size of the CPS monitoring window should be (pre-)configured based on the priority of the transmission and the remaining PDB.

Proposal 2: Due to the risk of collisions and unnecessary triggering of resource re-evaluation, we support that UEs carrying out random resource selection are permitted to use a mixed resource pool only when transmitting a TB with a priority that is higher than a pre-defined threshold.

Proposal 3: We propose that UEs carrying out random resource selection and transmitting low priority transmissions should use a resource pool that is configured with random resource selection only.

Proposal 4: We propose that resource pools with random resource selection enabled are configured with PSFCH disabled.

Proposal 5: For UEs carrying out random resource selection and are not capable of PSFCH reception, we propose to restrict the maximum number of blind retransmissions to be carried out based on the priority of the transmission.

Proposal 6: We propose that the candidate resources are aligned with the SL DRX ON duration in order to maximize the power saving gains.

Proposal 7: In the case of Mode 2, we propose that the RX UE aligns its partial sensing occasions according to the received SL DRX configurations, either from the TX UE in the case of unicast, or from pre-configuration in the case of groupcast or broadcast transmissions.

Proposal 8: In the case of inadequate sensing results, we propose that the UE uses assistance information messages in order to obtain the required sensing information for carrying out reliable resource selection.

Proposal 9: We propose to utilize assistance information from assistance entities, providing a set of resources that power saving UEs can use for increased reliability in their resource selection procedure.

Proposal 10: Since power saving UEs are required to be active based on their location, we propose to enable these UEs to wake up and carry out transmissions only when they are in a pre-configured region.

Proposal 11: We propose that power saving UEs should support bandwidth adaptation by operating over shortened frequency regions or by carrying out partial sensing over a smaller bandwidth.

# References

- RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics, 3GPP RAN#90e, December 2020.

- Chairman’s notes, 3GPP RAN1#106bis-e, October 2021.

- TS 38.212 v16.4.0, "Multiplexing and channel coding (Release 16)", October 2020.

- Chairman’s notes, 3GPP RAN1#104bis-e, April 2021.

- R1-2110511, “Moderator summary for [106bis-e-NR-R17-Sidelink-03] Reply LS to R1-2108710”, Moderator (InterDigital), 3GPP RAN1#106bis-e, October 2021.

- Chairman’s notes, 3GPP RAN2#113e, January-February 2021.

- R1-2111650, "Resource Allocation Enhancements for Mode 2", Fraunhofer HHI, Fraunhofer IIS, 3GPP RAN1#107-e, November 2021.

- Chairman’s notes, 3GPP RAN1#103e, October-November 2020.