3GPP TSG RAN WG1 #107-e			R1-2111720

e-Meeting, November 11th – November 19th, 2021

Agenda item:	8.1.2.3

Source: 	Samsung

Title: 	Enhancements on beam management for multi-TRP

Document for:	Discussion and Decision

# Introduction

Agreements from RAN1 #106-e [1] Agreement For aperiodic report of beam reporting option 2, - When associated with aperiodic resource setting, extend the existing RRC parameter CSI-AssociatedReportConfigInfo to be configured with two CMR resource sets where each may be configured with their corresponding QCL information. - FFS: Detailed association scheme - When associated with periodic/semi-persist resource setting, the resource setting comprises two CMR resource sets. Conclusion There is no consensus to support M>2 beams per group for beam reporting option 2 in Rel.17. Agreement Support differential L1 RSRP reporting as a UCI reduction scheme for beam measurement/reporting option 2. Agreement Differential reporting across all beam groups in a CSI-report - Including 1-bit indicator of the CMR set associated with the largest RSRP value in all groups - NOTE: best beam is assumed in the 1st group - 1-bit indicating CMR set with higher RSRP value (e.g. 0 indicating 1st SSBRI/CRI from 1st CMR set, 1 indicating 1st SSBRI/CRI from 2nd CMR set); UCI payload partitioning = 7/4 bits for 1st/2nd SSBRI/CRI in first beam group; 4 bits for all beams in other groups; Agreement For multi-TRP BFR, a single MAC-CE is used at least for BFRQ for all TRPs in all CCs in a cell group, which includes - Indices of failed BFD-RS set (as an indication of failed TRP link) - Indices of CC containing the failed TRP link - An indicator whether a new candidate beam is identified in the NBI-RS set associated with the failed BFD-RS set, and a resource indicator representing the new candidate beam (if identified) based on the number of NBI-RS resources in the corresponding NBI-RS set. - FFS: Content of MAC-CE related to SpCell when transmitted on msg3, msgA - Note: MAC-CE signaling design details are up to RAN2 - The term “failed TRP link” is used here for discussion purposes only Agreement The maximum number of BFD-RS resources per set is a UE capability, including a possible candidate value of 1 in Rel.17. Agreement Support the following BFD-RS configurations in Rel.17 for UEs with one activated TCI state per CORESET: - Implicit configuration: - M-DCI: - BFD-RS set k (k = 0, 1) is derived based on X TCI of CORESETs with CORESETPoolIndex = k - FFS: value of X (determined in spec or UE capability), and TCI selection rule when the number of CORESETs with CORESETPoolIndex = k exceeds X (e.g. reuse RLM RS selection rule) - FFS: CORESETs with more than 1 activated TCI states Conclusion BFD-RS configurations in Rel.17 for UEs with one activated TCI state per CORESET via implicit configuration for S-DCI mTRP is not supported in Rel-17. Agreement For extension of the existing RRC parameter CSI-AssociatedReportConfigInfo for the purpose of M-TRP beam reporting option 2, - Introduce a second ‘resourcesForChannel’ in CSI-AssociatedReportConfigInfo Agreement For option 2 with differential reporting - For each reported beam group other than the 1st beam group, the same SSBRI/CRI ordering as the 1st beam group is assumed. Agreement - For the case of all CORESETs with 1 activated TCI state per CORESET, after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with the failed BFD-RS set reported in the MAC-CE for TRP-specific BFR is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD-RS set - FFS: How to associate CORESET(s) with failed BFD-RS set - FFS: SCS configuration of 28 symbols - FFS: Update of QCL assumption for other DL channels/RSs, UL spatial filter/power control assumption for PUCCH, and other UL channels/RSs - FFS: the case of CORESETs with 2 activated TCI states per CORESET. - The above applies to SCell and SpCell - The above applies at least for the multi-DCI case Agreement Support the following BFD-RS configurations in Rel.17 for UEs with one activated TCI state per CORESET: - Explicit configuration of BFD-RS resources in BFD-RS set k, k = 0, 1 - FFS: CORESETs with more than 1 activated TCI state. Agreements from RAN1 #106bis-e [2] Agreement Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. - FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case. Agreement RACH-based transmission can be triggered on a SpCell at least in the following scenarios - Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell - FFS: other scenarios - Scenario 2: at least one TRP fails on SpCell - Scenario 3: at least one pre-defined TRP fails on SpCell - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available - Scenario 5: If MAC-CE based reporting does not work (details FFS) - Scenario 6: When no PUCCH-SR is configured Agreement To associate BFD-RS set k and NBI-RS set j - Alt-1: 1-to-1, fixed in spec - Whether NBI-RS configuration is mandatory is separate discussion Conclusion Design of MAC-CE related to SpCell when transmitted on msg3, msgA is up to RAN2. Agreement For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with CORESETPoolIndex k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD -RS set k (k=0,1) in the MAC-CE for TRP -specific BFR - The above applies to Scell and SpCell - The above applies for the multi-DCI case Agreement SCS of the 28 symbols is the smallest SCS of the active DL BWP for the response reception CC and of the active DL BWP (s) of the CC(s) with the failed TRP link(s) reported in BFR MAC CE. Agreement For RACH-based transmission at least when all BFD-RS sets fail in SPCell, CBRA is supported

In this contribution, we provide our views on several items related to the beam management enhancements for multi-TRP based on the agreements and progress from past RAN1 meetings such as those from RAN1 #106-e [1] and RAN1 #106bis-e [2].

# Beam measurement and reporting enhancements

In RAN1 #103-e [3], three options (Option-1, Option-2 and Option-3) for beam reporting enhancements were proposed for multi-TRP operation. In RAN1 #104-e [4], Option-2 was agreed and Option-1 and Option-3 are for further study. For Option-2, it has been agreed in RAN1 #105-e that the maximum number of groups/pairs per CSI-report (denoted by Nmax) could be {1, 2, 3, 4} depending on a UE capability. Furthermore, the number of beam groups N in a CSI-report is configured by higher layer RRC signalling. How to form/construct the CSI report/UCI for the beam group reporting is still open. Depending on various UE side’s configurations/settings such as how/which antenna panels are activated/deactivated and/or their orientation condition, not all of the N beam groups have satisfying beam metrics. That is, some of the N beam groups could be redundant, which would unnecessarily increase the decoding effort at the gNB side and UCI payload. To solve the above issues, the Rel. 15 two-part CSI reporting/UCI can be used. For instance, part 1 is of a fixed payload size, which includes the beam group report with the highest beam metrics; part 2 includes the remaining beam group reports. Part 1 is transmitted in its entirety before the transmission of part 2. The gNB could decode only part 1 if, e.g., the corresponding beam metrics are already good enough. The gNB could also decode both parts if necessary.

Proposal 1: For Option-2 group based beam reporting, support reporting in a CSI-report a two-part beam group report using the Rel. 15 two-part CSI/UCI.

As discussed above, for the group based beam reporting for multi-TRP, the UE could report more than one pairs/groups of beams with each pair/group associated with a receiving hypothesis at the UE, e.g., whether the UE would activate all of their RX panels or only one of the RX panels, not to mention that the UE may activate/deactivate their RX panels due to various other reasons such as power saving, MPE mitigation and etc. In addition, a TRP-RX panel correspondence can be established such that the UE could use different panels to correspondingly receive from different TRPs. Such a RX panel-specific operation cannot be transparent to the network, implying that certain RX panel specific information need to be sent to the network. For instance, reporting the RX panel activation/deactivation information/status along with the beam report(s) to the network should be considered for the group based beam reporting enhancements in Rel. 17. Or at least, information capturing whether the reported beams are associated with different receive filters/panels is needed for the multi-TRP operation.

Proposal 2: Support UE panel/antenna related feedback/reporting for Rel. 17 multi-TRP beam management enhancements.

- The feedback/reporting includes at least the information related to whether the reported beams are associated with different receive filters or panels.

- The feedback/reporting could also include RX panel activation/deactivation status.

# BFR enhancements

In this section, some remaining open points for BFR enhancements are discussed. Regarding PUCCH-SR resource configuration for per-TRP BFR, it has been agreed in RAN1 #104-e that up to two PUCCH-SR resources can be configured in a cell group. Whether a single uplink spatial domain transmission filter or two separate uplink spatial domain transmission filters can be associated with a PUCCH-SR resource still needs further discussions. For a UE configured with two PUCCH-SR resources, how to select the PUCCH-SR resource to indicate the beam failure(s) could depend on the number of UL spatial filters associated with a PUCCH-SR resource. When a PUCCH-SR resource is associated with only one UL spatial filter and beam failure is detected/declared for one BFD RS beam set in a CC, the PUCCH-SR resource associated with the working/non-failed BFD RS beam set shall be selected to indicate the beam failure. When a PUCCH-SR resource is associated with two separate UL spatial filters, it could be up to UE’s implementation and/or subject to network’s indication to select which PUCCH-SR resource (associated with either working/non-failed BFD RS beam set or failed BFD RS beam set) to indicate the beam failure. It has been agreed in RAN1 #105-e that one out of the four design alternatives (Alt. 2.5.2A, Alt. 2.5.2B, Alt. 2.5.2C and Alt. 2.5.2D) ought to be selected in RAN1 #106-e. In addition, it has been agreed in RAN1 #106bis-e that the association between the BFD RS beam sets and the PUCCH-SR resources is supported. According to the above analysis, Alt. 2.5.2C shall be supported.

Proposal 3: For the association between the BFD RS beam set and the PUCCH-SR resources,

- Support Alt. 2.5.2C, i.e., on the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation.

Some enhancements related to the TCI framework are needed. In Rel. 17 unified TCI framework, the TCI state update/switching is via DCI. Under this setting, the BFD RS resources configured via RRC cannot keep pace with the DCI based TCI state update/switching, resulting in misalignment between the configured BFD RSs and the actual control channel quality. Hence, more “dynamic” MAC CE shall be used to activate/update the BFD RS resources to minimize the impact of the misalignment. Furthermore, after the UE has received the BFRR, at least the QCL assumption for PDSCH (in addition to PDCCH) associated with the failed BFD RS set should be updated as under the Rel. 17 unified TCI framework, the PDCCH and PDSCH would share the same TCI state indication. Hence, we support

Proposal 4: Under Rel. 17 unified TCI framework,

- Update/activate BFD RSs via MAC CE

- Update at least QCL assumption for PDSCH (in addition to the already agreed PDCCH), after the UE receives BFRR.

# Conclusion

In this contribution, we provide the following proposals regarding Rel. 17 BM enhancements for multi-TRP:

Proposal 1: For Option-2 group based beam reporting, support reporting in a CSI-report a two-part beam group report using the Rel. 15 two-part CSI/UCI.

Proposal 2: Support UE panel/antenna related feedback/reporting for Rel. 17 multi-TRP beam management enhancements.

Proposal 3: For the association between the BFD RS beam set and the PUCCH-SR resources,

- Support to configure association between BFD RS beam sets and PUCCH-SR resources.

- Support Alt. 2.5.2C, i.e., on the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation.

Proposal 4: Under Rel. 17 unified TCI framework,

- Update/activate BFD RSs via MAC CE

- Update at least QCL assumption for PDSCH (in addition to the already agreed PDCCH), after the UE receives BFRR.

# References

- Chairman Notes, 3GPP RAN1#106-e, e-Meeting, August, 2021.

- Chairman Notes, 3GPP RAN1#106bis-e, e-Meeting, October, 2021.

- Chairman Notes, 3GPP RAN1#103-e, e-Meeting, November, 2020.

- Chairman Notes, 3GPP RAN1#104-e, e-Meeting, February, 2021.