3GPP TSG RAN WG1 #107-e			R1-2111773

e-Meeting, November 11th – 19th, 2021

Agenda Item:		8.17.5

Source:				Samsung

Title:					UE features for NR positioning enhancements

Document for:	Discussion

# Introduction

This contribution includes preliminary RAN1 UE features for Rel-17 NR provided from WI rapporteurs in [1]. This contribution discusses our views for some of the items.

# PRS measurement outside MG

## 27-3-3

| 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |
| --- | --- | --- |

In R16, the corresponding part for a) in point 2, “Type 1 – sub-slot/symbol level buffering” is as follows:

- T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms

Similarly, there should be a set of values for T with smaller values than that in R16 for bullet a).

Proposal 1: Support smaller numbers for T  in the existing UE PRS processing capability (N, T).

- The numbers include at least {1ms, 2ms, 4ms}, FFS other values

# LOS/NLOS Indicator

| 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements FFS: whether to have separate capability component for hard and soft indication |
| --- | --- | --- |

According to the previous RAN1 agreement, the LOS/NLOS indicator was agreed for all the positioning methods:

Agreement:

- Support LoS/NLoS indicators which are reported to the LMF for DL and DL+UL positioning measurements taken at UE for UE-assisted positioning or UL and DL+UL measurements at the TRP for NG-RAN assisted positioning.

  - Reporting from UE is subject to UE capability

- Positioning assistance data from LMF is enhanced for UE-based positioning by including LoS/NLoS indicators.

- FFS: Other kinds of positioning assistance data enhancements

- For LoS/NLoS detection method(s), there is no additional measurement IEs or assistance data outside of LoS/NloS indicator reporting (i.e., Option 6 from prior agreement).

- Note 1: No RAN4 requirements are expected for the LoS/NLoS indicators in RAN1’s understanding

- Note 2: LoS/NLoS indicators can be complementary to outlier rejection algorithms.

Agreement:

- Support LoS/NLoS indicators which are reported to the LMF for DL and DL+UL positioning measurements taken at UE for UE-assisted positioning or UL and DL+UL measurements at the TRP for NG-RAN assisted positioning.

  - Reporting from UE is subject to UE capability

- Positioning assistance data from LMF is enhanced for UE-based positioning by including LoS/NLoS indicators.

- FFS: Other kinds of positioning assistance data enhancements

- For LoS/NLoS detection method(s), there is no additional measurement IEs or assistance data outside of LoS/NloS indicator reporting (i.e., Option 6 from prior agreement).

- Note 1: No RAN4 requirements are expected for the LoS/NLoS indicators in RAN1’s understanding

- Note 2: LoS/NLoS indicators can be complementary to outlier rejection algorithms.

Thus, we suggest using the wording in the agreement for the explanation.

In addition, as we discussed in the corresponding tdoc [2], we think it is necessary to separate the UE capability for supporting the soft or the hard indication.

Proposal 2：Adopt the following change:

Support reporting LoS/NLoS indicator to LMF with separate capability component for hard and soft indication

# Conclusion

This contribution discusses the UE features for ePos. Observations and proposals are summarized as follows:

Proposal 1: Support smaller numbers for T  in the existing UE PRS processing capability (N, T).

- The numbers include at least {1ms, 2ms, 4ms}, FFS other values

Proposal 2：Adopt the following change:

Support reporting LoS/NLoS indicator to LMF with separate capability component for hard and soft indication.

# References

- R1-2108679, Preliminary RAN1 UE features list for Rel-17 NR, RAN1#106-e, Aug, 2021.

- R1-2111742, Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation, RAN1#107, Samsung, Nov. 2021