3GPP TSG RAN WG1 #107                	                                           R1-2111775

e-Meeting, November 11th – 19th, 2021

Agenda Item:		8.16.7

Source:			Samsung

Title:			UE features for UE power saving enhancements

Document for:		Discussion and decision

# Introduction

This contribution includes comments for Rel-17 UE features for supporting UE power saving enhancements, as drafted in R1-2109713 [1].

# 2. Comments to FG 29-2: TRS resources for idle/inactive UEs

For the components, the following agreement regarding the functionality was missing.

Agreements (From RAN1#102-e)

Idle/inactive UE may use the TRS/CSI-RS occasion(s) that are shared to it for functionalities such as:

-           AGC, time/frequency tracking

-           FFS: RRM measurement for serving cell, RRM measurement for neighbor cell, paging reception indication

We suggest to add “to support at least AGC, time/frequency tracking using available TRS resources in configured occasions”.  UE may also use the available TRS resources for RRM measurement by implementation, which has been discussed in RAN1.

Proposal 1: Add “to support at least AGC, time/frequency tracking using available TRS resources in configured occasions” as a part of the component.

For the FFS point, we don’t see the need to separate the capability for receiving L1 indication for TRS availability from FG 29-2. We agreed that gNB has to provide explicit availability indication in order for UE to utilize configured TRS resources. Also, L1 based availability indication is the only indication method supported as SIB based indication is not possible.

Proposal 2: No need to separate the capability for receiving L1 indication for TRS availability

# 3. Comments to FG 29-3: PDCCH monitoring adaptation within an active BWP

According to the progress in RAN1#106bis, there can be up to 5 PDCCH monitoring adaptation cases. Whether or not to support any of the PDCCH monitoring adaptation cases can be based on UE capability on PDCCH monitoring behaviors.

FG 29-3 can be split into the following four combination of PDCCH monitoring adaptation behaviours, depending on the progress in RAN1#107-e meeting:

- PDCCH skipping only, i.e. Beh 1/1A,

- SSSG switching only, i.e. Beh 2/2A/2B,

- Both PDCCH skipping and SSSG switching with 2 SSSGs, i.e. Beh 1A/2/2A, and

- Both PDCCH skipping and SSSG switching with 3 SSSGs, i.e. Beh 1A/2/2A/2B.

Proposal 3: Split into multiple sub-FGs with different combination of PDCCH monitoring adaptation behaviours. Including

- Beh 1/1A,

- Beh 2/2A/2B,

- Beh 1A/2/2A, and

- Beh 1A/2/2A/2B.

# 4. Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: Add “to support at least AGC, time/frequency tracking using available TRS resources in configured occasions” as a part of the component.

Proposal 2: No need to separate the capability for receiving L1 indication for TRS availability

Proposal 3: Split into multiple sub-FGs with different combination of PDCCH monitoring adaptation behaviours. Including

- Beh 1/1A,

- Beh 2/2A/2B,

- Beh 1A/2/2A, and

- Beh 1A/2/2A/2B.

# 5. Reference

[1] R1-2109713, Summary on UE features for UE power saving enhancements