3GPP TSG RAN WG1 #107-e	 																						    R1-2111810

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.16.5

Source:	Moderator (AT&T)

Title:	Summary of UE features for NR positioning enhancements

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion/approval [107-e-R17-UE-features-ePos-01] during RAN1 #107-e. According to the Chairman’s Notes:

[107-e-R17-UE-features-ePos-01] Email discussion UE features for NR positioning enhancements – Ralf (AT&T) - 1st check point: November 15 - Final check point: November 19

The following was discussed and/or agreed during RAN1 #107-e within the scope of [107-e-R17-UE-features-ePos-01]. All proposals are based on the latest RAN1 UE features list for Rel-17 NR in [1].

# Summary of Contributions Submitted to RAN1 #107-e

The following is the moderator’s summary of contributions submitted to RAN1 #107-e in this agenda item.

| 27. NR_pos_enh | 27-1-1 | Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE [for DL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | [13-3] | No |  | Mitigation of UE Rx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1,2,4,6,8,12,16,24,32}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - DL-TDOA and Multi-RTT reporting could have separate capabilities, in that for Multi-RTT, it is likely that UE only support RxTx TEG reporting instead of UE Rx TEG reporting, according to the agreement. - The reporting type could be per band. There is no need to have a per-UE capability for this if per band is agreed. The reporting Rx TEG ID in the measurement report could be numbered within the UE, i.e. the same Rx TEG ID means the same Rx TEG for different bands. - The maximum number per band should be reduced to 8. - If the reporting type is per band, we think that even if the number is 1, UE should still support Rx TEG ID reporting for the measurement in that PRS received in different bands may be associated with the same or different Rx TEG ID. - No need to have a separate row indicating support of the TEG reporting. If UE does not report the maximum number, UE does not support the feature (for a band). Perhaps we can put the number as the first component of the overall support of the feature. For example, the FG could be renamed as “Support of UE Rx TEG reporting”, and the first component is the maximum number of Rx TEGs. |
| ZTE [3] | Agreements: Parameter Description; Values in specifications (e.g., TS 37.355, TS 38.455); Values that can be signaled as part of UE Capability; Comments. The maximum number of UE RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT]; [32]; [1, 2,4,6,8,12,16,24,32] FFS: per UE/band /FL/FR; The parameter is used for supporting DL-TDOA and/or Multi-RTT. The maximum number of UE TxTEGs [for UL-TDOA and/or Multi-RTT]; [8]; [1, 2,4,6,8] FFS: per UE/band /FL/FR; The parameter is used for supporting UL-TDOA and/or Multi-RTT. The maximum number of UE-RxTx TEGs; [256]; [1, 2,4,6,8,12,16,24,32,64, 128, 256] FFS: per UE/band /FL/FR; The parameter is used for supporting Multi-RTT. - Note: Above proposal does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets. DL measurements from different bands may experience different timing error since the group delay mainly relies on hardware and different bands may correspond to different RF chains. Hence, this UE feature should be provided per band regardless of DL TDOA or RTT positioning solutions. That is, this FG should be shared for both DL TDOA and Multi-RTT positioning. Different UE receive antennas maybe correspond to different timing errors especially for UEs without well calibration. Considering UE may have up to 8 receive antennas, it is reasonable to support the candidate values {1, 2, 4, 8} for maximum number of UE-RxTEGs. Value=1 is to indicate that the timing error differences between all DL measurements are within a certain margin. The prerequisite should be FG 13-1 (common DL PRS Processing Capability) since UE-RxTEGs is for DL PRS processing and is independent of positioning solutions. Proposal: - The report granularity is per band. - The candidate values are {1, 2, 4, 8} - Value=1 is to indicate that the timing error differences between all DL measurements are within a certain margin - Shared for both UE-assisted DL TDOA and Multi-RTT positioning - The prerequisite is FG 13-1(common DL PRS Processing Capability) 27. NR_pos_enh; 27-1-1; Maximum number of UE-RxTEGs; The maximum number of UE-RxTEG, which is supported and reported by UE for DL TDOA and/or Multi-RTT positioning Value=1 indicates that the timing error differences between all DL measurements are within a certain margin; 13-1; No; Mitigation of UE Rx timing delays is not supported; per band; n/a; n/a; n/a; The candidate values are {1,2,4,8} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] | o Add FG 13-3 as pre-requisite, as this is an enhancement to Rel-16 DL-TDOA. o Rx TEG is not depending on the positioning technique (DL-TDOA, Multi-RTT), hence no need for separate row. |
| OPPO [5] | In Rel-17, in order to mitigate the Tx and Rx timing errors, a new concept of TEG (timing error group) is introduced. Based on TEGs, several enhanced solutions are introduced for DL TDOA, UL TDOA and multi-RTT positioning methods. In Rel-17, three types of TEGs are introduced, i.e., Rx TEG, Tx TEG and RxTx TEG. Thus, we should define the UE capability signaling to support UE to report it corresponding capabilities. During RAN1#106bis-e meeting, four feature groups were identified as below: - 27-1-1: Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] - 27-1-2: Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] - 27-1-3: Maximum number of UE-RxTxTEGs for Multi-RTT - 27-1-4: The maximum Number of UE Rx TEGs for measuring the same DL PRS resource The TEGs are depending on the hardware implementation. Typically, a UE may use different hardware for different bands. In this case, the TEGs will be different for different bands. Thus, it is natural to support the above-mentioned four feature groups as “per-band”. For the feature groups 27-1-1/2/3, some clarification is needed, e.g., - Alt.1: If the reported value reflects the total number of TEGs among all possible PFLs, it should be clarified that only up to (reported values / # of supported PFLs) TEGs can be available for a PFL - Alt.2: If the reported value reflects the maximum number of TEGs in one PFL, we should make it clear In our understanding, both alternatives can work. We prefer Alt.2 since it is more concise. Based on the above discussion, we have the following proposals: Proposal: UE feature groups 27-1-1 - Per band - for UE-assisted DL TDOA and/or Multi-RTT positioning - Adding some wording to reflect the reported value is for a PFL - The candidate values are [1, 2, 4, 6, 8] - The prerequisite feature groups: 13-3 or 13-4 |
| Intel Corporation [6] |  |
| Samsung [7] |  |
| Qualcomm Incorporated [8] | The support of the feature should be separated from the number of TEGs the UE have. The TEG is a feature that is considered across all the bands (per UE), whereas whether the UE supports the reporting of this feature is a per-band capability. Proposal: Support two separate rows for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” • Per band reporting Proposal: If a UE supports the RxTEG capability with a value > 1, if the UE does not include an RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement. Proposal: If a UE supports the RxTEG capability with a value=1, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements. 27. NR_pos_enh; 27-1-1; Maximum number of UE-RxTEGs for UE-assisted DL TDOA positioning; The maximum number of UE-RxTEG, which is supported and can be reported by UE for UE-assisted DL TDOA positioning Note 1: If UE supports this capability with the values > 1, and if if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement Note 2: If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.; 13-3, 27-1-1b; No; UE-RxTEG reporting is not supported; Per UE; n/a; n/a; n/a; The candidate values are {1,2,4,6,8,12,16,24,32} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-1b; Support of UE-RxTEG reporting for UE-assisted DL-TDOA; Support of UE-RxTEG reporting for UE-assisted DL-TDOA; 13-3a; No; Mitigation of UE Rx timing delays is not supported in this band; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-1c; Maximum number of UE-RxTEGs for Multi-RTT positioning; The maximum number of UE-RxTEG, which is supported and reported by UE Multi-RTT positioning Note 1: If UE supports this capability with the values > 1, if if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement Note 2: If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.; 13-4; No; UE-RxTEG reporting is not supported; Per UE; n/a; n/a; n/a; The candidate values are {1,2,4,6,8,12,16,24,32} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-1d; Support of UE-RxTEG reporting for UE-assisted DL-TDOA; Support of UE-RxTEG reporting for Multi-RTT positioning; 13-4a, 27-1-1c; No; Mitigation of UE Rx timing delays is not supported in this band; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signaling. |
| vivo [4] | - Issue 1: whether to have a value=1 to indicate UE Rx timing errors is well calibrated - We can support a value of 1, but we don’t think the value of 1 represents ‘well calibrated’. We think value=1 only represents that all the timing errors for UE Rx (e.g., based on RF chains and antenna panel) is grouped in the same group such that timing error difference in the same group is within a certain margin, rather than ‘well calibrated’. Even if some UEs cannot be ‘well calibrated’, they may support Rx TEG number of 1. - Issue 2: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning - Our answer is no. The UE capability is up to RF design such as RF chains or panel, we don’t see the need to differentiate to different values of the same feature for different positioning method. It is also no needed to have separate FGs for DL-TDOA and/or Multi-RTT positioning. Therefore, the brackets in the feature group name and related descriptions should be deleted. - Issue 3: regarding the type of UE feature of Rx TEG, FFS: Per UE or per band - Our preference is per band. The UE capability is up to RF design which is band/frequency dependent. For example, we don’t think the TEG capabilities are the same in FR1 and FR2. - Issue 4: The candidate values of UE Rx TEGs - According to agreements in RAN1#106bis-e, at least N=[2,3,4,6,8] Rx TEGs can be supported to measure the same DL PRS resource (FG27-1-4). In addition, the maximum value of N depends on UE capability of Rx TEGs per band. Then, considering ‘per band’ type for both feature 27-1-1 and 27-1-4 may be supported, so we think the candidate values of these 2 feature groups should be the same except the value of 1. Therefore, the candidate values of {1,2,3,4,6,8,12,16,24,32} in FG27-1-1 can be supported. Agreement: Make the following modification on the previous agreement made in RAN#106e: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability - The TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. - Note: All RSTD measurements are relative to a single reference timing. In addition, we also have another comment - Additional comment 1: change the name of the feature group to ‘Support of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning’ - This is similar to the way Rel-16 UE features is written. For example, in the feature group of 13-3 ‘DL PRS Resources for DL-TDOA’, the corresponding table is shown below. In addition, we also don’t see the need to have separate row for ‘Support of UE Rx TEGs…’ and ‘Maximum number of UE Rx TEGs…’. 13. NR Positioning; 13-3; DL PRS Resources for DL-TDOA; - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} - Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} - Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}. Therefore, we propose - Regarding UE feature group of Rx TEG (27-1-1), support the following: - Support value of 1 of UE-RxTEG, but value of 1 doesn’t indicate ‘well calibrated’. - Do not have separate values/FG for DL TDOA and/or Multi-RTT positioning. - The type of this feature is per band. - Change the candidate values to {1,2,3,4,6,8,12,16,24,32} - Change the name of this feature group to ‘Maximum number Support of UE-RxTEGs…’. |
| CATT [6] | For FG 27-1-1 (Maximum number of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning), the values of the maximum number of UE-RxTEGs should be [1,2,4,6,8,12,16,24,32], and the FG should be per UE. If the value=1, it indicates UE Rx timing errors is well calibrated. Regarding the issue of whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning, we prefer to keep the same values for both DL TDOA and Multi-RTT positioning and don’t need to have separate values or FGs for them. 27. NR_pos_enh; 27-1-1; Maximum number of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning; The maximum number of UE-RxTEG, which is supported and reported by UE for DL TDOA and/or Multi-RTT positioning The values = {1,2,4,6,8,12,16,24,32}. - Value=1 to indicate UE Rx timing errors is well calibrated. If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report.. |

| 27. NR_pos_enh | 27-1-2 | Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, 13-8] | FFS |  | Mitigation of UE Tx timing delays is not supported | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8, 16, 32}] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | For FG 27-1-2, we suggest to split the FG into two three FGs. - FG 27-1-2 deals with the UE SRS TEG reported to gNB, which can be applicable to both UL-TDOA and Multi-RTT. The reporting granularity can be per FS. - FG 27-1-2a deals with the UE SRS TEG reported to LMF for UL-TDOA. The reporting granularity can be per band for the configured CA band combination, so that LMF could request gNB with a proper number of SRS resources to cover the Tx TEG number. - FG 27-1-2b deals with the UE SRS TEG reported to LMF for Multi-RTT. The reporting granularity can be per band for the configured CA band combination, so that LMF could request gNB with a proper number of SRS resourcs to cover the Tx TEG number. - The maximum number of per band for the new three FGs should be reduced to 4. - If the reporting type is per band, we think that even if the number is 1, UE should still support Tx TEG ID reporting for the association and measurement in multi-RTT in that SRS transmitted in different bands may be associated with the same or different Tx TEG ID. - No need to have a separate row indicating support of the TEG reporting. If UE does not report the maximum number, UE does not support the feature (for a band). Perhaps we can put the number as the first component of the overall support of the feature. For example, the FG could be renamed as “Support of UE Tx TEG reporting”, and the first component is the maximum number of Tx TEGs. |
| ZTE [3] | Similarly, for FG27-1-2 on the mitigation of UE TX timing delays, we think the report granularity should be per band regardless of UL TDOA and Mutli-RTT positioning. Since UE usually only has at most 4 transmit antennas for UL transmission, the candidate values of maximum number of UE-TxTEGs should be {1, 2, 4}. Because UE-TxTEG information is not applicable for SRS with MIMO usage, and should be shared for both UL TDOA and Multi-RTT, we think the prerequisite should be only 13-8. In addition, this UE capability should let gNB know since gNB needs to decide UL measurement results based on the information of UE TxTEGs. Moreover, for UL TDOA based on positioning SRS, i.e. UE supports FG 13-8, if UE reports this FG 27-1-2 with value > 1, that means it should also support that the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA. It should also support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE. There is no reason to introduce an extra UE FG. Otherwise, this FG 27-1-2 will be useless if UE is not able to provide the association information of UL SRS resources for positioning with Tx TEGs. Proposal: The report granularity is per band. - The candidate values are {1, 2, 4} - Value=1 is to indicate that the timing error differences between all UL transmissions are within a certain margin - Shared for both UE-assisted UL TDOA and Multi-RTT positioning - The prerequisite is FG 13-8 (for positioning SRS) - This UE capability report should let gNB know - If UE supports this capability with the value > 1, - It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA - It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT 27. NR_pos_enh; 27-1-2; Maximum number of UE-TxTEGs; The maximum number of UE-TxTEG which is supported and reported by UE for UL TDOA and/or Multi-RTT positioning Value=1 indicate that the timing error differences between all UL transmissions are within a certain margin If UE supports this capability with the values > 1, - It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA if UL TDOA is supported by UE, - It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE; 13-8; Yes; Mitigation of UE Tx timing delays is not supported; per band; n/a; n/a; n/a; The candidate values are {1, 2, 4} Need for location server to know if the feature is supported; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] |  |
| OPPO [5] | In Rel-17, in order to mitigate the Tx and Rx timing errors, a new concept of TEG (timing error group) is introduced. Based on TEGs, several enhanced solutions are introduced for DL TDOA, UL TDOA and multi-RTT positioning methods. In Rel-17, three types of TEGs are introduced, i.e., Rx TEG, Tx TEG and RxTx TEG. Thus, we should define the UE capability signaling to support UE to report it corresponding capabilities. During RAN1#106bis-e meeting, four feature groups were identified as below: - 27-1-1: Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] - 27-1-2: Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] - 27-1-3: Maximum number of UE-RxTxTEGs for Multi-RTT - 27-1-4: The maximum Number of UE Rx TEGs for measuring the same DL PRS resource The TEGs are depending on the hardware implementation. Typically, a UE may use different hardware for different bands. In this case, the TEGs will be different for different bands. Thus, it is natural to support the above-mentioned four feature groups as “per-band”. For the feature groups 27-1-1/2/3, some clarification is needed, e.g., - Alt.1: If the reported value reflects the total number of TEGs among all possible PFLs, it should be clarified that only up to (reported values / # of supported PFLs) TEGs can be available for a PFL - Alt.2: If the reported value reflects the maximum number of TEGs in one PFL, we should make it clear In our understanding, both alternatives can work. We prefer Alt.2 since it is more concise. Based on the above discussion, we have the following proposals: Proposal: UE feature groups 27-1-2 - Per band - for UL TDOA and/or Multi-RTT positioning - Adding some wording to reflect the reported value is for a PFL - The candidate values are [1, 2, 4, 6, 8] - The prerequisite feature groups: 13-4 or 13-8 |
| Intel Corporation [6] |  |
| Samsung [7] |  |
| Qualcomm Incorporated [8] | The support of the feature should be separated from the number of TEGs the UE have. The TEG is a feature that is considered across all the bands (per UE), whereas whether the UE supports the reporting of this feature should be a per-band capability. Also, the reporting of this feature for UL-TDOA and the M-RTT features need to be separated because there are different reporting mechanisms. For UL-TDOA, the UE reports this information through RRC to the serving gNB, and for Multi-RTT, it reports this information through LPP to the LMF. Proposal: Split the FG 27-1-2 into 2 rows, one for Multi-RTT and the other for UL-TDOA. Proposal: With regards to the FG 27-1-2, support two separate FGs for “Support of UE-TxTEG reporting for UL-TDOA”, and “Support of UE-TxTEG reporting for M-RTT” • Per band reporting 27. NR_pos_enh; 27-1-2; Maximum number of UE-TxTEGs for UL TDOA; The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA; 13-8, 27-1-2b; Yes; UE-TxTEG reporting is not supported; Per UE; n/a; n/a; n/a; The candidate values are {1, 2, 4, 8, 16, 32} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-2b; Support of UE-TxTEG reporting for UL-TDOA; Support of UE-TxTEG reporting for UL-TDOA positioning; 13-8; Yes; Mitigation of UE Tx timing delays is not supported; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-2c; Maximum number of UE-TxTEGs for Multi-RTT positioning; The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning Note 1: If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource Note 2: If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.; 13-4, 13-8; No; Mitigation of UE Tx timing delays is not supported; Per UE; n/a; n/a; n/a; The candidate values are {1, 2, 4, 8, 16, 32} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-2d; Support of UE-TxTEG reporting for Multi-RTT; Support of UE-TxTEG reporting for Multi-RTT positioning; 13-8; No; UE-TxTEG reporting is not supported; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signaling. |
| vivo [4] | - Issue 1: whether to have a value=1 to indicate UE Tx timing errors is well calibrated - Similar to the views of Rx TEG, we can support a value of 1, but we don’t think the value of 1 represents ‘well calibrated’. - Issue 2: whether to have separate values/FGs for UL TDOA and/or Multi-RTT positioning - Firstly, we think the same values should be supported if the UE can support Tx TEG in UL-TDOA and Multi-RTT positioning. - Then, regarding whether to have separate FGs for UL TDOA and/or Multi-RTT positioning, our answer is yes. As concluded in the working assumption, in UL-TDOA method, UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB; while for Multi-RTT positioning, UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to the LMF. For example, in multi-RTT method, it is no need for the serving gNB to know whether UE can support Tx TEG; but for UL-TDOA method, both serving gNB and LMF need to know whether UE can support Tx TEG. Therefore, it is better to have separate FGs for UL-TDOA and/or Multi-RTT positioning. Working assumption: - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. - The serving gNB should forward the association information provided by the UE to the LMF. - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB. - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF. - FFS: Mitigation of UE Tx timing errors when Multi-RTT, UL-TDOA and/or DL-TDOA are used.. - Issue 3: regarding the type of UE feature of Tx TEG, FFS: Per UE or per band or per FS - Similar to the UE feature of ‘Rx TEG’, we think the type of Tx TEG is per band. In addition, we also have another comment - Additional comment 1: Change the name of the feature group to ‘Support of UE-Tx TEGs for…’ instead of ‘Maximum number of …’. - This is similar to the way Rel-16 UE features is written as mentioned in Rx TEG feature. Therefore, we propose - Regarding UE feature group of Tx TEG (27-1-2), support the following: - Have separate FGs for UL-TDOA and Multi-RTT positioning. - Note: if UE can support Tx TEG for both UL-TDOA and Multi-RTT positioning, the maximum number of UE-Tx TEGs should be the same. - Support value of 1 of UE-TxTEG, but value of 1 doesn’t indicate ‘well calibrated’. - The type of the feature is per band. - Change the name of this feature group to ‘Maximum number Support of UE-TxTEGs…’. |
| CATT [6] | For FG 27-1-2 (Maximum number of UE-TxTEGs for UL TDOA and/or Multi-RTT positioning), the values of the maximum number of UE-TxTEGs should be [1,2,4,6,8], and the FG should be per UE. If the value=1, it indicates UE Rx timing errors is well calibrated. Regarding the issue of whether to have separate values/FGs for UL TDOA and/or Multi-RTT positioning, according to the working assumption above, for UL TDOA, UE may provide the association information of UL SRS resources for positioning with Tx TEGs to the gNB and then gNB forward such information to LMF; for Multi-RTT, UE may provide the association information of UL SRS resources for positioning with Tx TEGs directly to LMF, therefore, we prefer to have separate FG 27-1-2a and FG 27-1-2b for UL TDOA and Multi-RTT positioning , respectively. 27. NR_pos_enh; 27-1-2a; Maximum number of UE-TxTEGs for UL TDOA; The maximum number of UE-TxTEG, which is supported and reported by UE for UL TDOA. The values = {1,2,4,6,8}. - Value=1 to indicate UE Tx timing errors is well calibrated. If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF.. 27. NR_pos_enh; 27-1-2b; Maximum number of UE-TxTEGs for Multi-RTT positioning; The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning. The values = {1,2,4,6,8}. - Value=1 to indicate UE Tx timing errors is well calibrated. If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the gNB.. |

| 27. NR_pos_enh | 27-1-3 | Maximum number of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4, 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, [128]}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - The reporting type could be per band. There is no need to have a per-UE capability for this if per band is agreed. The reporting Rx TEG ID in the measurement report could be numbered within the UE, i.e. the same Rx TEG ID means the same Rx TEG for different bands. - The maximum number per band should be reduced to 32. - If the reporting type is per band, we think that even if the number is 1, UE should still support RxTx TEG ID reporting for the measurement in that PRS received in different bands may be associated with the same or different RxTx TEG ID. - No need to have a separate row indicating support of the TEG reporting. If UE does not report the maximum number, UE does not support the feature (for a band). Perhaps we can put the number as the first component of the overall support of the feature. For example, the FG could be renamed as “Support of UE RxTx TEG reporting for multi-RTT”, and the first component is the maximum number of RxTx TEGs. |
| ZTE [3] | For FG 27-1-3 on the mitigation of UE Rx+TX timing delays, the report granularity should also be per band as aforementioned reason. The candidate values should take into account DL measurements and UL transmissions, which at least should include the number of multiplications between the number of UE-RxTEGs and UE-TxTEGs. So, the candidate values can be {1, 2, 4, 8, 16, 32}, where value=1 is to indicate that the ‘Rx timing error+Tx timing error’ differences between all combinations of measurement and transmission are within a certain margin. Regarding the prerequisite, we think both FG 13-4(DL PRS Resources for Multi-RTT) and FG 13-8(SRS Resources for Positioning) should be included as Multi-RTT positioning is based on both DL PRS and UL positioning SRS. Proposal: - The report granularity is per band - The candidate values are {1, 2, 4, 8, 16, 32} - Value=1 is to indicate that the ‘Rx timing error+Tx timing error’ differences between all combinations of measurement and transmission are within a certain margin - The prerequisite is FG 13-4(for DL PRS) and 13-8 (for positioning SRS) 27. NR_pos_enh; 27-1-3; Maximum number of UE-RxTxTEGs for Multi-RTT; The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning Value=1 is to indicate that the ‘Rx timing error+Tx timing error’ differences between all combinations of measurement and transmission are within a certain margin; 13-4, 13-8; No; Mitigation of UE RxTx timing delays is not supported; per band; n/a; n/a; n/a; The candidate values are {1, 2, 4, 8, 16, 32, 64} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] |  |
| OPPO [5] | In Rel-17, in order to mitigate the Tx and Rx timing errors, a new concept of TEG (timing error group) is introduced. Based on TEGs, several enhanced solutions are introduced for DL TDOA, UL TDOA and multi-RTT positioning methods. In Rel-17, three types of TEGs are introduced, i.e., Rx TEG, Tx TEG and RxTx TEG. Thus, we should define the UE capability signaling to support UE to report it corresponding capabilities. During RAN1#106bis-e meeting, four feature groups were identified as below: - 27-1-1: Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] - 27-1-2: Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] - 27-1-3: Maximum number of UE-RxTxTEGs for Multi-RTT - 27-1-4: The maximum Number of UE Rx TEGs for measuring the same DL PRS resource The TEGs are depending on the hardware implementation. Typically, a UE may use different hardware for different bands. In this case, the TEGs will be different for different bands. Thus, it is natural to support the above-mentioned four feature groups as “per-band”. For the feature groups 27-1-1/2/3, some clarification is needed, e.g., - Alt.1: If the reported value reflects the total number of TEGs among all possible PFLs, it should be clarified that only up to (reported values / # of supported PFLs) TEGs can be available for a PFL - Alt.2: If the reported value reflects the maximum number of TEGs in one PFL, we should make it clear In our understanding, both alternatives can work. We prefer Alt.2 since it is more concise. Based on the above discussion, we have the following proposals: Proposal: UE feature groups 27-1-3 - Per band - Adding some wording to reflect the reported value is for a PFL - The candidate values are [1, 2, 4, 6, 8, 12, 16, 32, 64] - The prerequisite feature groups: 13-4 or 13-8 |
| Intel Corporation [6] |  |
| Samsung [7] |  |
| Qualcomm Incorporated [8] | The support to UE RxTx TEG reporting should be a per-band-combo capability reporting since the PRS and the SRS may be on a different band. Proposal: With regards to the FG 27-1-3, support a separate FG for “Support of UE-RxTxTEG reporting for M-RTT” • Per band combination reporting 27. NR_pos_enh; 27-1-3; Maximum number of UE-RxTxTEGs for Multi-RTT; The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning Note 1: If UE supports this capability with the values > 1, and if if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement Note 2: If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.; 13-4, 13-8; No; Mitigation of UE RxTx timing delays is not supported; Per UE; n/a; n/a; n/a; The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, 128} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-3b; Support of UE-RxTxTEG reporting for Multi-RTT; Support of UE-RxTxTEG reporting for Multi-RTT positioning.; 13-8, 27-1-3; No; UE-RxTxTEG reporting is not supported; Per band combination; n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signaling. |
| vivo [4] | - Issue 1: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated - Similar to the views of Rx TEG, we can support a value of 1, but we don’t think the value of 1 represents ‘well calibrated’. - Issue 2: regarding the type of UE feature of RxTx TEG, FFS: Per UE or per band - Considering that UE Rx chain and Tx chain may be associated with different RF chain of different band, so we think it is better to define the type of RxTx TEG feature as ‘band combination’. In addition, we also have another comment. - Additional comment 1: change the name of the feature group to ‘Support of UE-RxTxTEGs…’ instead of ‘Maximum number of UE-RxTxTEGs…’ - This is similar to the way Rel-16 UE features is written as mentioned before. Therefore, we propose - Regarding UE feature group of RxTx TEG (27-1-3), support the following: - Support value of 1 of UE-RxTxTEG, but value of 1 doesn’t indicate ‘well calibrated’. - The type of the feature is per band combination. - Change the name of this feature group to ‘Maximum number Support of UE-RxTxTEGs…’ . |
| CATT [6] | For FG 27-1-3 (Maximum number of UE-RxTxTEGs for Multi-RTT), the values of the maximum number of UE-RxTxTEGs should be [1,2,4,6,8,12,16,24,32,64,128,256], and the FG should be per UE. If the value=1, it indicates UE Rx timing errors is well calibrated. 27. NR_pos_enh; 27-1-3; Maximum number of UE-RxTxTEGs for Multi-RTT; The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning The values= {1,2,4,6,8,12,16,24,32,64,128,256}. - Value=1 to indicate UE RxTx timing errors is well calibrated. If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning. |

| 27. NR_pos_enh | 27-1-4 | The maximum Number of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - The reporting type could be per band. - No need to have a separate row indicating support of the TEG reporting. If UE does not report the maximum number, UE does not support the feature (for a band). Perhaps we can put the number as the first component of the overall support of the feature. For example, the FG could be renamed as “Support of receiving the same DL PRS resource with multiple Rx TEGs, and the first component is the maximum number of Rx TEGs. - The prerequisite FG should be updated as FG 27-1-1. |
| ZTE [3] | For FG 27-1-4 on maximum Number of UE Rx TEGs for measuring the same DL PRS resource, straightforwardly, FG 27-1-1 should be its prerequisite and per band basis. The candidate values have been agreed in AI 8.5.1 in last meeting. Agreement: Make the following modification on the previous agreement made in RAN#106e: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability - The TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. - Note: All RSTD measurements are relative to a single reference timing. However, UE supports N1 RX TEGs for measuring the same DL PRS resource doesn’t mean UE can support N1 RX TEGs for measuring the same DL PRS resource simultaneously. Hence, a new component should be included in FG 27-1-4, the candidate values represent UE capability of the number of UE Rx TEGs for measuring the same DL PRS resource simultaneously, which depends on the maximum number of UE receive panels or the support maximum number of UE receive beams at a same time. The candidate values can be 2 or 4. With such UE capability, LMF can recommend gNB the appropriate PRS parameters including resource or resource set repetition factor, PRS periodicity etc.. For example, if UE reports N1 = 8 for measuring the same DL PRS resource and N2= 2 for measuring the same DL PRS resource simultaneously, then it is sufficient to configure 4 PRS resource repetitions to let UE report all 8 RX TEGs for the same resource as UE can process one resource instance with two RX TEGs simultaneously. However, if UE reports N1 = 8 and N2= 1, then it may need to configure 8 PRS resource repetitions to let UE report all 8 RX TEGs for the same resource. Proposal: - The report granularity is per band - The candidate values are { 2, 3, 4, 6, 8} - One new component or one new FG is needed for maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP simultaneously, - The candidates are {2, 4}. 27. NR_pos_enh; 27-1-4; The maximum Number of UE Rx TEGs for measuring the same DL PRS resource; Component 1: the maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. Component 2: the maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously; 27-1; No; Mitigation of UE Rx timing delays by using different Rx TEGs are not supported; per band; n/a; n/a; n/a; For component 1: the candidate values are {1, 2, 4, 8} For component 2: the candidate values are {1, 2} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] |  |
| OPPO [5] | In Rel-17, in order to mitigate the Tx and Rx timing errors, a new concept of TEG (timing error group) is introduced. Based on TEGs, several enhanced solutions are introduced for DL TDOA, UL TDOA and multi-RTT positioning methods. In Rel-17, three types of TEGs are introduced, i.e., Rx TEG, Tx TEG and RxTx TEG. Thus, we should define the UE capability signaling to support UE to report it corresponding capabilities. During RAN1#106bis-e meeting, four feature groups were identified as below: - 27-1-1: Maximum number of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] - 27-1-2: Maximum number of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] - 27-1-3: Maximum number of UE-RxTxTEGs for Multi-RTT - 27-1-4: The maximum Number of UE Rx TEGs for measuring the same DL PRS resource The TEGs are depending on the hardware implementation. Typically, a UE may use different hardware for different bands. In this case, the TEGs will be different for different bands. Thus, it is natural to support the above-mentioned four feature groups as “per-band”. For the feature groups 27-1-1/2/3, some clarification is needed, e.g., - Alt.1: If the reported value reflects the total number of TEGs among all possible PFLs, it should be clarified that only up to (reported values / # of supported PFLs) TEGs can be available for a PFL - Alt.2: If the reported value reflects the maximum number of TEGs in one PFL, we should make it clear In our understanding, both alternatives can work. We prefer Alt.2 since it is more concise. Based on the above discussion, we have the following proposals: Proposal: UE feature groups 27-1-4 - Per band - The candidate values are [1, 2, 4, 6, 8] |
| Intel Corporation [6] |  |
| Samsung [7] |  |
| Qualcomm Incorporated [8] | Not all UEs will be able to measure the same PRS resource simultaneously with multiple TEGs. For example, in FR2, measuring simultaneously with both panels should be a separate feature than measuring in a TDM fashion a PRS resource with the 2 panels. Proposal: Introduce a separate row for Multi-RTT and DL-TDOA for the FG 27-1-4 • Per band reporting Proposal: Introduce separate rows with the maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP simultaneously for Multi-RTT and DL-TDOA • Per band reporting 27. NR_pos_enh; 27-1-4; The maximum Number of UE Rx TEGs for measuring the same DL PRS resource in UE-assisted DL-TDOA; The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP in UE-assisted DL-TDOA.; 27-1-1b; No; Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP; Per band; n/a; n/a; n/a; The candidate values are {1, 2, 4, 8} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-4b; The maximum Number of UE Rx TEGs for measuring the same DL PRS resource in Muti-RTT; The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP in Multi-RTT.; 27-1-1d; No; Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP; Per band; n/a; n/a; n/a; The candidate values are {1, 2, 4, 8} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-4c; The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously in UE-assisted DL-TDOA; The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP simultaneously in UE-assisted DL-TDOA; 27-1-4; No; Measuring the same instance of a DL PRS resource simultaneously with multiple RxTEGs is not supported; Per band; n/a; n/a; n/a; The candidate values are {1, 2, 4, 8} Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-1-4d; The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously in Multi-RTT; The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP simultaneously in Multi-RTT; 27-1-4; No; Measuring the same instance of a DL PRS resource simultaneously with multiple RxTEGs is not supported; Per band; n/a; n/a; n/a; The candidate values are {1, 2, 4, 8} Need for location server to know if the feature is supported; Optional with capability signaling. |
| vivo [4] | - Issue 1: regarding the type of this UE feature, FFS: Per UE or per band - Our preference is per band. The UE capability is up to RF design which is band/frequency dependent. For example, we don’t think the TEG capabilities are the same in FR1 and FR2. - Issue 2: the candidate values of this UE feature. - Firstly, the values of this feature should be >1. - Then, according to agreements in RAN1#106bis-e, at least N=[2,3,4,6,8] should be captured in the list. Agreement: Make the following modification on the previous agreement made in RAN#106e: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability - The TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. - Note: All RSTD measurements are relative to a single reference timing. - Issue 3: whether to have separate row for ‘The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously’. - Our preference is yes. The measurement results of different UE Rx TEGs simultaneously may be better for LMF to calculate Rx timing error difference, since simultaneous measurement does not introduce additional errors, such as errors caused by timing drift. Whether the UE supports this feature may affect the decision of the LMF request. In addition, we also have an additional comment - Additional comment 1: change the name of the feature group to ‘Support of multiple Rx TEGs for measuring the same DL PRS resource’ - This is similar to the way Rel-16 UE features is written as mentioned before. We propose to change the name of the feature group to ‘Support of multiple Rx TEGs for measuring the same DL PRS resource’ instead of ‘The maximum Number of UE Rx TEGs for measuring the same DL PRS resource’. In addition, we also don’t see the need to have separate row for ‘Support of…’ and ‘Maximum number of…’. - Additional comment 2: a typo, that is, change the column of ‘Prerequisite feature groups’ to ‘27-x1’. Proposal: - Regarding UE feature group of multiple Rx TEGs for the same PRS resource (27-1-4), support the following: - The type of the feature is per band. - Change the candidate values to {1,2,3,4,6,8 }. - Have separate row for ‘The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously’. - Change the name of this feature group to ‘Support of multiple the maximum Number of Rx TEGs for measuring the same DL PRS resource’. - Change the column of ‘Prerequisite feature groups’ to ‘27-x1’. |
| CATT [6] | For FG 27-1-4 (The maximum Number of UE Rx TEGs for measuring the same DL PRS resource), the values of the maximum number of UE-RxTEGs for measuring the same DL PRS resource should be [2,3,4,6,8], and the FG should be per UE. 27. NR_pos_enh; 27-1-4; The maximum Number of UE Rx TEGs for measuring the same DL PRS resource; The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. The values = {2,3,4,6,8}. |

| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - DL-TDOA, DL-AoD, and Multi-RTT reporting could have separate capabilities. - The reporting type could be per band. |
| ZTE [3] | FG 27-2-1 should be only for first path RSRP as the current agreement for additional path RSRP is only applicable for DL TDOA and Multi-RTT. Moreover, based on the agreement made in RAN1#105-e meeting, first path RSRP for DL AoD should be applicable for both UE assisted and UE based positioning. So the component 1 for this FG should be support of PRS RSRP measurement of the first path for both UE based and UE assisted positioning. The second component is to report M first path PRS RSRP measurements per TRP for UE assisted DL-AOD positioning. Based on the agreement in RAN1#106b-e meeting, M should be {2, 4, 8, 16, 24}. Like FG 13-2 for DL-DOA, this FG should be per UE. Proposal: - The report granularity is per UE - The prerequisite is FG 13-2 - It is only for DL-AOD, and only for first path RSRP - Component 1: Support of PRS RSRP measurement of the first path for UE based or UE assisted positioning - Component 2: Candidate values for first path PRS RSRP measurements per TRP for UE assisted DL-AOD positioning are {2,4,8,16,24} - A separate FG is needed for path RSRP report on DL TDOA and Multi-RTT 27. NR_pos_enh; 27-2-1; DL PRS RSRP of the first path for DL-AoD; Component 1: Support PRS RSRP of the first path for DL-AoD positioning method Component 2: Candidate values for first path PRS RSRP measurements per TRP for UE assisted DL-DOA positioning are {2,4,8,16,24}; 13-2; No; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] | - Add FG 13-4, 13-8 as pre-requisite, as this requires SRS resources for positioning - Add ”13-2 or 13-3” as pre-requisite, as it requires PRS reception for DL-AoD or DL-TDOA - Add value 24, as given by RAN1 agreement from RAN1#106bis-e: Agreement: The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. - For UE-A DL-AOD, support reporting more than 8 up to 16 M first path PRS RSRP measurements per TRP, where M is a UE capability - FFS: Values of M. Candidate values include {2,4,8,16,24}. - FFS: Whether M is always equal to N - Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps. - Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation |
| OPPO [5] | Rel-17 supports PRS RSRP measurement of the first path for DL-AoD as in the following agreement: Agreement: For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) the PRS RSRP of the first path - FFS: Details of measurement and reporting of PRS RSRP of the first path. During the Rel-17 dicussion, the PRS RSRP measurement of the first path was discussed and agreed only for the DL-AoD positioning method, but not for DL-DTOA or multi-RTT. However, in the positioning measurement result reporting, the UE can also report PRS RSRP result in measurement reporting of DL-DTOA and Multi-RTT. In our view, because the initial motivation for RSRP of the first path was DL-AoD method and it was discussed and agreed only for DL-AoD. Therefore, we shall not request the UE to support it for other positioning method. In UE feature, we shall only define the UE capability of PRS RSRP measurement of the first path for the DL-AoD positioning method. Proposal 5: In UE feature, the UE capability of PRS RSRP measurement of the first path for DL-AoD positioning method is separate capability. 27-2-1; [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD]; Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] The maximum number of first path PRS RSRP per TRP: candidate value: 2,4,8,16,24 [Note: Applicable for DL-TDOA and Multi-RTT]. |
| Intel Corporation [6] |  |
| Samsung [7] |  |
| Qualcomm Incorporated [8] | This row should be related only to DL-AoD, where the earliest path-RSRP for this method is a main new measurement that is added in NR Rel-17. Note that, even though path-RSRP can be related to the DL-TDOA, Multi-RTT, for these methods, such features are considered within the multipath positioning features. Different requirements may exist for path-RSRP associated with DL-AoD compared to the path-RSRP for the multipath positioning (which, as agreed in RAN4, there will not be any requirements). Proposal: With regards to the FG 27-2-1, should only be applicable to UE-assisted DL-AoD. Path-RSRP reporting for DL-TDOA/Multi-RTT should be a separate FG. Proposal: Introduce an additional row (FG 27-2-1b) for the maximum number of first path RSRP measurements the UE can report. 27. NR_pos_enh; 27-2-1; DL PRS RSRP of the first path for UE-assisted DL-AoD; Support of measuring and reporting the PRS RSRP of the first path for UE-assisted DL-AoD positioning method; 13-5; No; PRS RSRP of the first path is not supported; per band; n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-2-1b; Maximum number of the first path PRS RSRP measurements on different PRS resources for UE-assisted DL-AoD positioning; Maximum number K of the first path PRS RSRP measurements on different PRS resources from the same TRP for UE-assisted DL-AoD positioning; 27-2-1; No; Per Band; n/a; n/a; n/a; The candidate values are {2,4,8,12, 16,24} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| vivo [4] | Firstly, we would like to discuss the remaining issue with the yellow highlight of the 27-2-1 UE feature. For the issue, whether adding a note about “Applicable for DL-TDOA and Multi-RTT”, a new agreement has been agreed upon in RAN1#106bis-e meeting as follows, that is reporting the PRS RSRP of the first path and additional path DL-TDOA, and multi-RTT has been supported. But, until now, AoD methods only support reporting the PRS RSRP of the first path. If the UE feature is different for DL-TDOA and Multi-RTT, we propose adding a new capability 27-2-* for reporting the PRS RSRP of the first path and additional path DL-TDOA, and multi-RTT. Agreement: Support reporting the path RSRP for the first path and for additional paths as part of DL-TDOA, UL-TDOA, and multi-RTT reporting enhancements. - FFS: Support introducing a request from the LMF to the UE/TRP when the path-RSRP for additional paths is desired to be reported. - FFS: Support of path RSRP for additional paths as part of DL-AoD.. In addition, in our view, the UE capability should be a per UE capability which should be dependent on baseband process capability other than only supported by some bands. So, the capability can be proposed as follow - Modify the 27-2-1 with the blue highlight for the support of DL PRS RSRP of the first path for DL-AoD as the following Table. - Introduce a new UE capability 27-2- * with the blue highlight for the support of DL PRS RSRP of the first path and additional path for DL-TDOA and Multi-RTT as the following Table. 27. NR_pos_enh; 27-z2-1; Support of [UE-assisted] DL PRS RSRP of the first [or additional] path for DL-AoD; UE’s capability to Support providing the of [measuring and reporting the] PRS RSRP of the first [or additional] path [for DL-AoD positioning method] [Note: Applicable for DL-TDOA and Multi-RTT]; [13-2 or 13-3]; No; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-z2-*; Support of [UE-assisted] DL PRS RSRP of the first or additional path for DL-TDOA and Multi-RTT; UE’s capability to Support providing the of [measuring and reporting the] PRS RSRP of the first or additional path for DL-TDOA and Multi-RTT; 13-3 or 13-4; No; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| CATT [6] |  |

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS | n/a | [The candidate values are {12, 16, 32, 64}] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - The reporting type could be per band. - There is no such need to indicate the K values, but rather UE only reports support of more than 8 DL PRS RSRPs. Network may indicate that more than 8 is desired, and it is up to UE to select which RSRPs are reported, and it can be more than 8 if requested by the LMF. If it is necessary to indicate the K values, we suggest to also add 24, making it {12, 16, 24, 32, 64} - No need to have additional capability to limit the number of RSRPs per Rx beam in the capability unless we made progress on the measurement reporting enhancement. |
| ZTE [3] | For FG 27-2-2, we have the following suggestion based on the agreement made in RAN1#106b-e meeting. We think the requisite should be only FG 13-2. If UE reports this FG, it doesn’t need to report FG 13-5(report up to 8 RSRP) anymore. Proposal: - The report granularity is per UE - The candidate values are {16, 24} - The prerequisite is FG 13-2 27. NR_pos_enh; 27-2-2; DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning; Support reporting K> 8 DL PRS RSRP measurements per TRP.; 13-2; No; UE report of more than 8 DL PRS-RSRP is not supported.; Per UE; n/a; Yes; n/a; The candidate values are {16, 24} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] | - Add FG 13-4, 13-8 as pre-requisite, as this requires SRS resources for positioning - Add FG 13-5 as pre-requisite as this extends the Rel-16 parameter space. |
| OPPO [5] | Regarding the maximal number of DL PRS RSRP per TRP for DL AoD, we have agreed that the candidate values can be {16, 24}. If the UE does not report this capability, the maximal number of DL PRS RSRP is 8 by default. In rel-17, we agreed to support first path PRS RSRP for DL AoD and the maximal number per TRP is UE capability. The candidate values can be {2,4,8,16, 24}, which shall be captured in UE capability. Furthermore, as agreed, the maximal number of DL PRS RSRP associated with the same Rx beam index is up to UE implementation. Therefore, that shall not be included in UE capability. 27-2-2; DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning; Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K The candidate values for K is 16, 32. If K is not reported, default value is 8. Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index. |
| Intel Corporation [6] |  |
| Samsung [7] |  |
| Qualcomm Incorporated [8] | 27. NR_pos_enh; 27-2-2; DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning; Support reporting K> 8 DL PRS RSRP measurements per TRP.; 13-5; No; UE report of more than 8 DL PRS-RSRP is not supported.; per band; n/a; FFS; n/a; The candidate values are {12, 16, 24} Need for location server to know if the feature is supported; Optional with capability signaling. |
| vivo [4] | The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting up to N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. - For UE-A DL-AOD, support reporting up to M first path PRS RSRP measurements per TRP, where M is a UE capability - FFS: Values of M. Candidate values include {2,4,8,16,24}. - FFS: Whether M is always equal to N - Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps. - Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation. Based on the above agreement in RAN1#106bis-e meeting, two UE capabilities should be introduced. One is for the support of up to N DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE, another is the support of up to M first path PRS RSRP measurements on different PRS resources from the same TRP supported by the UE. We would like to note the capability 27-2-2 should add the description that ‘K DL PRS RSRP measurements is on different PRS resources from the same TRP’ to align the Rel-16 capability 13-5 and clarify that the number K is measurement capability for a TRP. In addition, the candidate value only includes {16, 24} based on the above agreement. So, we propose - Modify the 27-2-2 with the blue highlight for the support of DL PRS RSRP reporting for more than 8 measurements as the following Table. - Introduce a new UE capability 27-2- * for the support of up to M first path PRS RSRP measurements per TRP as the following Table. 27. NR_pos_enh; 27-z2-2; Support of DL PRS RSRP reporting for more than 8 measurements on different PRS resources from the same TRP [for UE-assisted DL-AoD positioning]; UE capability to Support reporting K> 8 DL PRS RSRP measurements per TRP on different PRS resources from the same TRP . FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index; [13-5]; No; UE report of more than 8 DL PRS-RSRP is not supported.; FFS: Per UE or per band; n/a; n/a FFS; n/a; [The candidate values are {12, 16,24 32, 64}] Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-z2-*; Support of DL PRS RSRP reporting M first path PRS RSRP measurements on different PRS resources from the same TRP[for UE-assisted DL-AoD positioning]; UE capability to Support reporting M first path PRS RSRP measurements on different PRS resources from the same TRP . FFS: the values of K •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. Note: Additional capability may be added: •FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index; [13-5]; No; UE report of more than 8 DL PRS-RSRP is not supported.; FFS: Per UE or per band; n/a; n/a FFS; n/a; [The candidate values are {2,4,8,16,24} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| CATT [6] | The following agreement was achieved related to UE capability for DL-AoD enhancements in RAN1#106bis-e. Agreement: The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. - For UE-A DL-AOD, support reporting more than 8 up to 16 M first path PRS RSRP measurements per TRP, where M is a UE capability - FFS: Values of M. Candidate values include {2,4,8,16,24}. - FFS: Whether M is always equal to N - Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps. - Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation. For FG 27-2-2 (The maximum Number of UE Rx TEGs for measuring the same DL PRS resource), according to the agreement above, we prefer two separate FGs for DL PRS RSRP reporting and first path DL PRS RSRP reporting, respectively. Therefore, the original FG 27-2-2 can be divided into two FGs, i.e., FG 27-2-2a and FG 27-2-2b. Then, FG 27-2-2a and FG 27-2-2b can be used for DL PRS RSRP reporting and first path DL PRS RSRP reporting, respectively. For up to K DL PRS RSRP reporting per TRP, the values of K should be [16, 24]. For up to M first path DL PRS RSRP reporting per TRP, the candidate values include [2,4,8,16,24]. Both of the two FGs should be per UE. Proposal: Make the following modifications to FG 27-2-2 in order to reflect the agreement related to UE capability for DL-AoD enhancements in RAN1#106bis-e: 27. NR_pos_enh; 27-2-2a; DL PRS RSRP reporting for up to K measurements for UE-assisted DL-AoD positioning; Support reporting up to K DL PRS RSRP measurements per TRP. The values of K = {16,24}. •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps.. 27. NR_pos_enh; 27-2-2b; First path DL PRS RSRP reporting for up to M measurements for UE-assisted DL-AoD positioning; Support reporting up to M first path DL PRS RSRP measurements per TRP. The candidate values of M include {2,4,8,16,24}. - FFS: Values of M. - FFS: Whether M is always equal to K.. |

| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3}] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - The reporting type could be per UE with FR1/FR2 differentiation. - It should be common for all positioning methods (DL-TDOA, DL-AoD, and Multi-RTT) and all positioning measurements (DL RSTD, DL PRS-RSRP, UE Rx – Tx time difference, and DL PRS-RSRPP). |
| ZTE [3] | FG 27-3-1 is to support M < 4 measurement samples where one sample corresponds to one PRS resource measurement occasion/instance. Based on the previous agreement, at least M = 1 is supported. In our view, M = {2, 3} can further provide more flexibility between performance reliability and positioning latency reduction. Hence, we suggest further agreeing M = 2 and 3. Here is our proposal Proposal: - The report granularity is per UE - The candidate values are {1, 2, 3, 4} 27. NR_pos_enh; 27-3-1; M-sample measurements; The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set; 13-1; No; Per UE; n/a; n/a; n/a; [The candidate values are {1, 2, 3, 4} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] | - Add FG 13-4, 13-8 as pre-requisite, as this requires SRS resources for positioning - Add FG 13-1 as pre-requisite |
| OPPO [5] | Furthermore, for a UE supporting M-sample measurement, the LMF can indicate the UE to report a result based on M-sample measurement. From the UE perspective, it is not feasible that the LMF indicate different M-sample measurements for different simultaneous NR positioning measurements. For instance, the following case shall not happen: the LMF indicates the UE to report DL TDOA measurement result based on M-sample measurement and requests the UE to report DL AoD measurement based on 4-sample measurement. Proposal: In UE feature, we shall clarify that the UE expects the LMF to indicate same M-sample or 4-sample measurement for all the NR positioning measurement at the same time and add one note in FG 27-3-1 to clarify that: 27. NR_pos_enh; 27-3-1; M-sample measurements [of DL PRS measurement on single DL PRS period/occasion]; The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Note: a UE reporting M value expects the LMF configure same the M-sample measurement for all the simultaneous positioning measurement configurations.. |
| Intel Corporation [6] |  |
| Samsung [7] |  |
| Qualcomm Incorporated [8] | 27. NR_pos_enh; 27-3-1; M-sample measurements of DL PRS; The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1 If the UE does not provide the capability, the UE support M=4 only.; 13-1; No; per band; n/a; n/a; n/a; The candidate values are 1 Need for location server to know if the feature is supported; Optional with capability signaling. |
| vivo [4] | For us, we agree with the assumption that supports M=4 only if the UE does not provide the capability, and prefer the candidate values are {1,4}. Besides, we think multiple sample filter or one sample measurement reporting should be a per UE capability which should be dependent on baseband process capability. So, we propose - Modify the 27-3-1 with the blue highlight related to M-sample measurements as the following Table 27. NR_pos_enh; 27-u3-1; Support of M-sample measurements [of DL PRS measurement on single DL PRS period/occasion]; The capability to support providing reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M= [1[,2-34]. FFS: other values. If the UE does not provide the capability, the UE [is assumed] to support M=4 only.; [13-1]; No; Per UE; n/a; n/a; n/a; [The candidate values are {1, 2, 3, 4}] Need for location server to know if the feature is supported.; Optional with capability signaling. |
| CATT [6] |  |

| 27. NR_pos_enh | 27-3-2 | DL PRS measurement outside MG [and in a PRS processing priority window] - processing types | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window] Candidate values: {Type 1A, Type 1B, Type 2}. Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options [Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP] | [13-1] | FFS |  |  | FFS: Per UE or per band | n/a | n/a | n/a | Need for location server to know if the feature is supported FFS: Separate feature group for a UE to declare support of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - The reporting type could be per UE with FR1/FR2 differentiation. If indeed there is need for per band reporting, different bands, on which the feature is supported, should be corresponding to a single type. It will complicate the issue if UE reports different PRS processing types on different bands, i.e. the introduction of per band granularity should only be used to identify the bands that UE does not support the feature. - No need for separate FGs for different processing types, as it results in UE supporting different processing types at the same time, which overcomplicates this issue and require network to indicate which type is applied. |
| ZTE [3] | For FG 27-3-2 on support of DL PRS measurement outside MG, we suggest to clarify that DL PRS with higher priority for PRS measurement should be contained in a PRS processing priority window. Further, within a PRS processing window, UE measurement should be inside the active DL BWP with PRS having the same numerology as the active DL BWP. In addition, since it has been agreed the priority indicator in indicated by gNB, this UE capability should at least let gNB know. Proposal: For 27-3-2 on support of DL PRS measurement outside MG - The report granularity is per UE - Need for gNB to know if the feature is supported. - Clarify that: within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP In Rel-16, during the measurement window, the N ms of PRS symbols may be located in anywhere within the duration of T ms as shown in following figures. Figure 1a Figure 1b Figure 1c As defined in the formula of TS 38.133 for the measurement period, the component Tlast is the measurement duration for the last PRS sample, including the sampling time and processing time. The Tlast is to consider the cases that PRS resources from different sets are not concentrated on the same MG instance or PRS resources appear in the end of the processing window (e.g. Figure 1c shown above). This component leads to additional latency for the sampling and processing of the last PRS sample. As we are trying to reduce the latency as much as possible, it’s not acceptable to take additional time after the end of the PRS processing window. In order to reduce the latency for DL PRS measurement in the PRS processing window, the location information report should be ready right after the end of the PRS processing window. That is, UE has to finish all the DL PRS receiving and computation in the PRS processing window to make full use of its hardware resources. A Type 1 PRS processing capability is shown in the Figure 2 below, a PRS processing window is divided into a PRS buffering window and a PRS computation window. UE is only expected to receive the DL PRS in the PRS buffering window. Then, based on the buffered DL PRS, UE can compute/process the DL PRS in the PRS computation window to get ready for a location information report by the end of PRS processing window. According to this understanding, UE has to report its capability with at least one combination of {N, T} under the following interpretations, - A PRS processing window (with duration L) is divided into a PRS buffering window (with duration L-(T-N)) and a PRS computation window (with duration T-N). The PRS computation window starts right after the end of the PRS buffering window. - UE shall take (T-N) ms of time to process up to N ms of symbols containing PRS resources received by UE in the PRS buffering window - UE is not expected to be configured a PRS processing window with duration smaller than T-N. Figure 2 Type 1 PRS processing capability A Type 2 PRS processing capability is shown in Figure 3 below. UE may not need to buffer all the DL PRS before starting processing the DL PRS. That is, UE can do DL PRS receiving and processing simultaneously. Therefore, UE only needs to reserve enough time to process the latest DL PRS resource used for the location information report, which is quite similar to the CSI reference resource defined for CSI report. PRS computation time (Tcompute) is understood by the following, - A time span (Tspan) is calculated from an end of the latest DL PRS resource in the PRS processing window that is used for a location information report to the end of the PRS processing window - The value of Tspan is not expected to be smaller than the PRS computation time (Tcompute) . Figure 3 Type 2 PRS processing capability In summary, we propose to include the two types of PRS processing capability as two components of FG 27-3-2. Then, the feature of PRS processing window outside MG can work. Proposal: Add the following type 1 and type 2 processing capabilities as two components of FG 27-3-2 - Type 1 PRS processing capability: UE has to report its capability with at least of the combination {R, P}, - A PRS processing window (with duration L) is divided into a PRS buffering window (with duration L-(T-N)) and a PRS computation window (with duration T-N). The PRS computation window starts right after the end of the PRS buffering window. - UE shall take (T-N) ms of time to process up to N ms of symbols containing PRS resources received by UE in the PRS buffering window - UE is not expected to be configured a PRS processing window with duration smaller than T-N. - Type 2 PRS processing capability: UE has to report its capability of PRS computation time (Tcompute) - A time span (Tspan) is calculated from an end of the latest DL PRS resource in the PRS processing window that is used for a location information report to the end of the PRS processing window - The value of Tspan is not expected to be smaller than the PRS computation time (Tcompute). 27. NR_pos_enh; 27-3-2; DL PRS measurement outside MG and in a PRS processing priority window; Component 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing priority window. Component 2: Candidate values: {Type 1A, Type 1B, Type 2} Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Component 3: - Support Type 1 PRS processing capability: UE has to report its capability with at least of the combination {N, T}, - A PRS processing window (with duration L) is divided into a PRS buffering window (with duration L-(T-N)) and a PRS computation window (with duration T-N). The PRS computation window starts right after the end of the PRS buffering window. - UE shall take (T-N) ms of time to process up to N ms of symbols containing PRS resources received by UE in the PRS buffering window - UE is not expected to be configured a PRS processing window with duration smaller than T-N. - Support Type 2 PRS processing capability: UE has to report its capability of PRS computation time (Tcompute) - A time span (Tspan) is calculated from an end of the latest DL PRS resource in the PRS processing window that is used for a location information report to the end of the PRS processing window - The value of Tspan is not expected to be smaller than the PRS computation time (Tcompute). Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options. Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP.; 13-1; Yes; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] | - Add FG 13-4, 13-8 as pre-requisite, as this requires SRS resources for positioning - Confirm the FG, details to be decided later. - Add FG 13-1 as pre-requisite - Per UE capability, the processing itself can be a per band indication |
| OPPO [5] |  |
| Intel Corporation [6] |  |
| Samsung [7] |  |
| Qualcomm Incorporated [8] | With regards to MG-less PRS processing, if the UE reports a Type-1A MG-less PRS capability it could be able to report the highest PRS processing capability for a given latency, or vice versa, the shortest latency for a given number of PRS resources per slot, since the UE would have all the processing/memory resources of the modem to finish the processing. On the other hand, for Type-1B or Type-2, the UE would have to share the modem processing/memory resources with other functionalities, which will increase the latency or decrease the PRS processing capabilities. Depending on the use-case, it may be more appropriate for the UE to be allowed to report that it is capable of multiple of such PRS processing, and let the LMF recommend to the serving gNB which one is needed on different scenarios. Proposal: A UE should be able to report multiple of the Type-1A, Type-1B, Type-2 MG-less PRS processing capabilities, each one associated with a different PRS processing capability, to the LMF. There haven’t been any discussion yet on the maximum number of PFLs that a UE would be processing within a single PRS processing window. For example, • a UE may receive assistance data that have 2 PFLs with the same SCS on the same BWP and could be encompassed by the same PRS processing window. • a UE may receive assistance data that have 2 PFLs on 2 active BWPs on different CCs. A Type-1A UE may get a single processing window that could encompass both those PFLs, or a Type-1B/2 UE may get separate PRS processing window on different CCs, each one enabling the measurement of a different PFL. Without complicating the feature, and following the NR Rel-16 single-PFL processing window, we make the following proposal: Proposal 13: Inside each single instance of a PRS processing window, a single PFL can be measured. This is applicable to all Types of MG-less PRS processing. Proposal 14: Simultaneously processing of multiple PRS processing windows on different CCs/Bands is not supported for the MG-less PRS processing feature: The UE is not expected to be activated with multiple PRS processing windows that overlap. 27. NR_pos_enh; 27-3-2; DL PRS measurement outside MG and in a PRS processing priority window- processing types; 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG [and in a PRS processing priority window] Note: - Type 1A refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 1B refers to DL PRS being prioritized over other DL signals/channels in all OFDM symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. - Type 2 refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP Note: Inside each single instance of a PRS processing window, a single PFL can be measured.; 13-1; FFS; per band; n/a; n/a; n/a; Need for location server to know if the feature is supported Candidate values: A UE may declare any one or multiple from the set {Type 1A, Type 1B, Type 2}. Total 6 options - 1 bit for Type 1A - 1 bit for Type 1B - 1 bit for Type 2; Optional with capability signaling. |
| vivo [4] | Working assumption: Subject to UE capability, support PRS measurement outside the MG, within a PRS processing window, and UE measurement inside the active DL BWP with PRS having the same numerology as the active DL BWP. - Inside the PRS processing window, subject to the UE determining that DL PRS to be higher priority, support the following UE capabilities: - Capability 1: PRS prioritization over all other DL signals/channels in all symbols inside the window. - Cap. 1A: The DL signals/channels from all DL CCs (per UE) are affected. - Cap. 1B: Only the DL signals/channels from a certain band/CC are affected. - FFS: band or CC - Capability 2: PRS prioritization over other DL signals/channels only in the PRS symbols inside the window - A UE shall be able to declare a PRS processing capability outside MG. - FFS: Details of capability signalling (e.g., per UE or per band, etc.) - For the purpose of this feature, PRS-related conditions are expected to be specified, with the following to be down-selected: - Alt. 1: Applicable to serving cell PRS only - Alt. 2: Applicable to all PRS under conditions to PRS of non-serving cell. - Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options.. In our view, based on the above work assumption, the brackets around “and in a PRS processing priority window” should be removed. We think the per UE or per band capability will depend on the Type(i.e, Type 1A, Type 1B, Type 2). And we share the same view that Type 1A is per UE capability and Type 1B is per cc/band capability. But we observe there is no granularity definition for Type 2. So, at least, we think Type 2 should illustrate which is per UE or per CC capability. Besides, considering the symmetry of Type 1 and Type 2, we propose Type 2 is divided into Type 2A and Type 2B based on the granularity (i.e., per UE or per cc). Proposal: - For UE capability related to PRS processing window (27-3-2), the brackets around “and in a PRS processing priority window” should be removed. - For UE capability related to PRS processing window (27-3-2), to divide Type 2 into Type 2A and Type 2B based on the granularity (i.e., per UE or per cc). - Type 2A refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. The DL signals/channels from all DL CCs (per UE) are affected. - Type 2B refers to DL PRS being prioritized over other DL signals/channels only in DL PRS symbols within the PRS processing priority window. The DL signals/channels from certain DL CCs are affected. |
| CATT [6] |  |

| 27. NR_pos_enh | 27-3-3 | DL PRS Processing Capability outside MG - buffering capability | 1. DL PRS buffering capability: Type 1 or Type 2 a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz | 27-3-2 | No |  |  | Per band | n/a | n/a | n/a | FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - The FG should be broken into two FGs, one for PRS being higher priority, and one for PRS being lower priority, since the UE processing capability is apparently different for two cases. - The change of the Type 1 should be component 2 instead of component 1, i.e. 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz - b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz. - No need for separate PRS processing types, and we think the primary focus is to introduce single type per UE. - The prerequisite FG should be updated as FG 27-3-2. |
| ZTE [3] |  |
| Nokia/Nokia Shanghai Bell [4] | - Add FG 13-4, 13-8 as pre-requisite, as this requires SRS resources for positioning - Confirm the FG, details to be decided later. |
| OPPO [5] |  |
| Intel Corporation [6] |  |
| Samsung [7] | In R16, the corresponding part for a) in point 2, “Type 1 – sub-slot/symbol level buffering” is as follows: - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms Similarly, there should be a set of values for T with smaller values than that in R16 for bullet a). Proposal: Support smaller numbers for T in the existing UE PRS processing capability (N, T). - The numbers include at least {1ms, 2ms, 4ms}, FFS other values |
| Qualcomm Incorporated [8] | With regards to MG-less PRS processing, if the UE reports a Type-1A MG-less PRS capability it could be able to report the highest PRS processing capability for a given latency, or vice versa, the shortest latency for a given number of PRS resources per slot, since the UE would have all the processing/memory resources of the modem to finish the processing. On the other hand, for Type-1B or Type-2, the UE would have to share the modem processing/memory resources with other functionalities, which will increase the latency or decrease the PRS processing capabilities. Depending on the use-case, it may be more appropriate for the UE to be allowed to report that it is capable of multiple of such PRS processing, and let the LMF recommend to the serving gNB which one is needed on different scenarios. Proposal: A UE should be able to report multiple of the Type-1A, Type-1B, Type-2 MG-less PRS processing capabilities, each one associated with a different PRS processing capability, to the LMF. There haven’t been any discussion yet on the maximum number of PFLs that a UE would be processing within a single PRS processing window. For example, • a UE may receive assistance data that have 2 PFLs with the same SCS on the same BWP and could be encompassed by the same PRS processing window. • a UE may receive assistance data that have 2 PFLs on 2 active BWPs on different CCs. A Type-1A UE may get a single processing window that could encompass both those PFLs, or a Type-1B/2 UE may get separate PRS processing window on different CCs, each one enabling the measurement of a different PFL. Without complicating the feature, and following the NR Rel-16 single-PFL processing window, we make the following proposal: Proposal 13: Inside each single instance of a PRS processing window, a single PFL can be measured. This is applicable to all Types of MG-less PRS processing. Proposal 14: Simultaneously processing of multiple PRS processing windows on different CCs/Bands is not supported for the MG-less PRS processing feature: The UE is not expected to be activated with multiple PRS processing windows that overlap. 27. NR_pos_enh; 27-3-3; DL PRS Processing Capability outside MG - buffering capability if Type-1A is declared; 1. DL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {4, 6, 8, 16, 20, 30, 40} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 5, 6, 8, 10, 12, 16, 20} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz; 27-3-2; No; Per band; n/a; n/a; n/a; Optional with capability signaling. 27. NR_pos_enh; 27-3-3b; DL PRS Processing Capability outside MG - buffering capability if Type-1B is declared; 1. DL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz; 27-3-2; No; Per band; n/a; n/a; n/a; Optional with capability signaling. 27. NR_pos_enh; 27-3-3c; DL PRS Processing Capability outside MG - buffering capability if Type-2 is declared; 1. DL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz; 27-3-2; No; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| vivo [4] | Modify the 27-3-3 related to DL PRS processing capability outside MG-buffer capability as the following Table. 27. NR_pos_enh; 27-u63-3; DL PRS Processing Capability outside MG - buffering capability; 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level bufferingType 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: [{8, 16, 20, 30, 40, 80, 160, 320, 640, 1280}] ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz; 27-u5; No; Per band; n/a; n/a; n/a; FFS: Separate feature group for a UE to declare PRS processing capabilities of each of the Type-1A, Type-1B, Type-2” capabilities; Optional with capability signaling. |
| CATT [6] |  |

| 27. NR_pos_enh | 27-4-1 | LOS/NLOS Indicator for UE-assisted positioning | Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements FFS: whether to have separate capability component for hard and soft indication |  | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate value are [0,1]] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - There is no strong need to have separate FGs for the soft and hard values of LoS/NLoS indicator, but we can live with being having a component indicate that. 1. LOS/NLOS indicator type: {softValue, hardValue, both} - There is no need to have separate FGs for RSTD and UE Rx – Tx time difference measurement, which means that this is common to all positioning methods and positioning measurements. - The reporting type could be per UE, as we do not see difference on the algorithm on the detection. - Adding another component would be helpful to indicate whether UE support resource-specific indicator or TRP specific indicator or both. 2. LOS/NLOS indicator granularity: {resourceSpecific, trpSpecific, both}. |
| ZTE [3] | For FG 27-4-1 on LOS/NLOS indicator, we think this FG should be reported regardless of positioning methods as UE judges LOS/NLOS just based on channel measurement. In our view, UE can just report whether to support or not support this FG. If support, it will be up to UE to report any value from 0 to 1. Proposal: - The report granularity is per UE. - This reporting LoS/NLoS indicator to LMF is regardless of positioning methods - The FG is just to report support of LOS/NLOS indicator 27. NR_pos_enh; 27-4-1; LOS/NLOS Indicator for UE-assisted positioning; Support reporting LoS/NLoS indicator to LMF; No; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] | - Need to add corresponding Rel-16 FGs on measurement reports as pre-requisites, as this is an add-on. - No need for separate capability for soft and hard indication. OK to list as component in this capability though. - No need to signal candidate values. |
| OPPO [5] | Rel-17 supports the UE to report more than 2 additional paths for DL-TDOA positioning measurement. That should be subject to the UE capability. In UE feature, the UE can report if the UE supports to report more than 2 additional paths and the UE can reports the maximal number of additional paths for DL-TDOA positioning measurement. Regarding the maximal number of additional paths, 4 shall be one candidate value. Proposal: Reporting > 2 additional paths for DL-TDOA is UE capability and the UE can report the maximal number of additional paths for DL-TDOA: - The candidate values for maximal number additional paths are 2 and 4. It was also agreed in Rel-17 that UE can report more than 2 additional paths for Multi-RTT positioning measurement. That should be subject to the UE capability too. In UE feature, the UE can report if the UE supports to report more than 2 additional paths and the UE can reports the maximal number of additional paths for multi-RTT positioning measurement. Similarly, 4 shall be a candidate value for the maximal number of additional paths. Proposal: Reporting > 2 additional paths for Multi-RTT is UE capability and the UE can report the maximal number of additional paths for multi-RTT: - The candidate values for maximal number additional paths are 2 and 4. In Rel-17, the UE can report a single NLOS/LOS indicator for DL and DL+UL positioning measurement. As agreed, this feature is subject to UE capability. For the candidate value of the single NLOS/LOS indicator reported by the UE shall be 0 and 1 which are used to indicate whether the positioning measurement is from a NLOS or LOS path. Proposal: In UE capability, the UE can report supporting reporting hard value for LOS/NLOS indicator: [0, 1] and update the FG 27-4-1 as follows: 27-4-1; LOS/NLOS Indicator for UE-assisted positioning; Support reporting LoS/NLoS indicator to LMF [for RSTD and UE Rx-Tx time difference measurements to LMF for DL and DL+UL positioning] FFS: whether to have separate capability component/FG for RSTD and UE Rx-Tx time difference measurements - Supported values for LOS/NLOS indicator: hard value or soft value. FFS: whether to have separate capability component for hard and soft indication. |
| Intel Corporation [6] |  |
| Samsung [7] | According to the previous RAN1 agreement, the LOS/NLOS indicator was agreed for all the positioning methods: Agreement: - Support LoS/NLoS indicators which are reported to the LMF for DL and DL+UL positioning measurements taken at UE for UE-assisted positioning or UL and DL+UL measurements at the TRP for NG-RAN assisted positioning. - Reporting from UE is subject to UE capability - Positioning assistance data from LMF is enhanced for UE-based positioning by including LoS/NLoS indicators. - FFS: Other kinds of positioning assistance data enhancements - For LoS/NLoS detection method(s), there is no additional measurement IEs or assistance data outside of LoS/NloS indicator reporting (i.e., Option 6 from prior agreement). - Note 1: No RAN4 requirements are expected for the LoS/NLoS indicators in RAN1’s understanding - Note 2: LoS/NLoS indicators can be complementary to outlier rejection algorithms. Thus, we suggest using the wording in the agreement for the explanation. In addition, as we discussed in the corresponding tdoc [2], we think it is necessary to separate the UE capability for supporting the soft or the hard indication. Proposal：Adopt the following change - Support reporting LoS/NLoS indicator to LMF with separate capability component for hard and soft indication |
| Qualcomm Incorporated [8] | 27. NR_pos_enh; 27-4-1; LOS/NLOS Indicator for UE-assisted positioning; Support reporting LoS/NLoS indicator to LMF; No; per UE; n/a; n/a; n/a; The candidate value are [0,1] Need for location server to know if the feature is supported; Optional with capability signaling. |
| vivo [4] | In RAN1 #106-e meeting, the following agreements for LoS/NLoS indicators has been achieved. Agreement: - Support LoS/NLoS indicators which are reported to the LMF for DL and DL+UL positioning measurements taken at UE for UE-assisted positioning or UL and DL+UL measurements at the TRP for NG-RAN assisted positioning. - Reporting from UE is subject to UE capability - Positioning assistance data from LMF is enhanced for UE-based positioning by including LoS/NLoS indicators. - FFS: Other kinds of positioning assistance data enhancements - For LoS/NLoS detection method(s), there is no additional measurement IEs or assistance data outside of LoS/NloS indicator reporting (i.e., Option 6 from prior agreement). - Note 1: No RAN4 requirements are expected for the LoS/NLoS indicators in RAN1’s understanding - Note 2: LoS/NLoS indicators can be complementary to outlier rejection algorithms.. It has already been agreed that LoS/NLoS indicators reporting is supported for DL and DL+UL positioning measurements taken at UE. That is, whether the LoS/NLoS indicators is for RSTD (which is the measurement in DL positioning) and for UE Rx-Tx time difference (which is the measurement in DL+UL positioning), it should be both supported. However, it has also been agreed that for LoS/NLoS detection method(s), there is no additional measurement IEs or assistance data outside of LoS/NLoS indicator reporting (i.e., Option 6 from prior agreement). That is to say, how UE determines LoS/NLoS indicator completely depends on UE implementation, if a UE can do LoS/NLoS detection for DL positioning, it can also do LoS/NLoS detection for DL+UL positioning. Therefore, we think there is no need to have separate capability component for RSTD and UE Rx-Tx time difference measurements. Proposal: - For UE’s capability to support reporting LoS/NLoS indicator: - No need to have separate capability component for RSTD and UE Rx-Tx time difference measurements. For LoS/NLoS indicators, it has been agreed that a single-indicator can be reported and the supported values are a discrete set in the interval [0, 1], which does not preclude using binary values only which is up to UE/TRP implementation. Agreement: For LoS/NLoS indicators, a single-indicator can be reported and the supported values are a discrete set in the interval [0, 1]. - FFS: the number of discrete values to be supported - Note: This does not preclude using binary values only which is up to UE/TRP implementation - Note: Single-indicator means that one value in the interval [0, 1] is used for the LoS/NLoS indication. In the last RAN1 #106-bis-e meeting, there was discussion whether to introduce a separate UE capability to differentiate hard and soft values. However, no agreement was reached. As far as we are concerned, the intention of separate capability component for hard and soft indication is mainly to solve the ambiguousness caused by the same LoS/NLoS indicator value reported by different UEs. For example, UE1 is supportive of reporting LoS/NLoS indicator using binary values and UE2 is supportive of reporting LoS/NLoS indicator using discrete set, but there is no capability to indicate to gNB or LMF what type of LoS/NLoS indicator the UE is supportive. When UE1 and UE2 both report a LoS/NLoS indicator of 0, for UE1, it only means the link is detected as NLoS but the confidence is unknown; but for UE2, it means the link is detected as NLoS and the confidence is very high. However, the LMF cannot know the confidence information and may assume the two indicators are the same as they are both 0 and further use them in the same way, which is obvious unreasonable. Therefore, to deal with the above ambiguousness, we think two options can be adopted: - Option1: Support the additional UE capability of which type of LoS/NLoS indicators the UE is supportive. When gNB or LMF receives different UE capability, it can identify the meaning of 0 and 1 according to the reported different UE capability. - Option2: Support to differentiate the type of LOS/NLOS Indicator by the LoS-NLos-Indicator IE, which can include hard value and soft value. Then LMF can have the knowledge of whether 0 and 1 corresponds to hard value or soft value. Since the two above options are being discussed and does not reach an agreement, we think it is better to decide whether to have separate capability component for hard and soft indication after the two above options achieve an agreement. Proposal: It is better to decide whether to have separate capability component for hard and soft indication after the two above options achieve an agreement. |
| CATT [6] |  |

| 27. NR_pos_enh | 27-5-1 | [UE-initiated] on-demand PRS | UE’s capability to support UE-initiated on-demand DL PRS [request signalling] | [13-1] | No |  |  | Per UE | n/a | n/a | n/a | FFS: Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | We think that this feature can work even if the location server does not know if UE supports the feature, however LMF should anyway have control over whether UE should send the UE-initiated on-demand PRS request. |
| ZTE [3] | - The report granularity is per UE. - Need for location server to know if this FG is supported 27. NR_pos_enh; 27-5-1; UE-initiated on-demand PRS; UE’s capability to support UE-initiated on-demand DL PRS; 13-1; No; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia/Nokia Shanghai Bell [4] | - Add FG 13-1 as pre-requisite |
| OPPO [5] |  |
| Samsung [7] |  |
| Intel Corporation [6] | The UE-initiated on-demand DL PRS request from UE to LMF was agreed. The support of this functionality requires definition of a new FG family. Given that LMF initiated on-demand DL PRS request is also supported it is recommended to update FG 27-5-1. Proposal: Update FG 27-5-1 as follows: UE initiated on-demand DL PRS request |
| Qualcomm Incorporated [8] | 27. NR_pos_enh; 27-5-1; UE-initiated on-demand PRS; UE’s capability to support UE-initiated on-demand DL PRS request signalling; 13-1; No; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signaling. |
| vivo [4] | On-demand DL-PRS is not supported for UEs in Rel-15, Rel-16 and earlier versions. In Rel-17, for those UEs support on-demand DL-PRS, they need report their capabilities to network, so that network can distinguish which UE is supportive of on-demand DL-PRS. And these UEs can be configured and take measurements with on-demand DL-PRS. For on-demand DL-PRS, the UE capability can include the following aspects. - The new UE capability to suppport new LPP assistance data IE for on-demand DL-PRS configurations and potential new PRS parameters - The new UE capability to support providing UE preferred parameters for UE-initiated on-demand PRS. From our point of view, the 2nd UE capability has already been captured in the preliminary RAN1 UE feature list as following. 27. NR_pos_enh; 27-w5-1; Support of [UE-initiated] on-demand PRS; UE’s capability to support UE-initiated on-demand DL PRS [request signalling]; [13-1]; No; Per UE; n/a; n/a; n/a; FFS: Need for location server to know if the feature is supported.; Optional with capability signaling. Therefore, we propose. - Adding following new UE capability related to on-demand PRS in UE feature list - The new capability to suppport new LPP assistance data IE for on-demand DL-PRS configurations and potential new PRS parameters. For the 1st UE capability, as the new LPP assistance data IE for on-demand DL-PRS is transmitted from LMF to UE, it is surely reasonable for LMF to know this UE feature. For the 2nd UE capability, as RAN2 has reached an agreement as following, the UE initiated on-demand PRS is enabled by the UE request triggering a request from the LMF, LMF need this feature to know which UE is supportive of UE-initiated on-demand PRS and receive their requests. Therefore, we think the above two on-demand PRS UE features are needed for location server to know if the feature is supported. RAN2 #113bis-e - UE-initiated on-demand PRS request is enabled by enhancing LPP RequestAssistanceData. FFS how much control the network has over the UE request. - The UE-initiated mechanism is enabled by the UE request triggering a request from the LMF, and the actual PRS changes are requested by the LMF irrespective of whether the procedure is UE- or LMF-initiated. - Put the stage 2 description for UE-initiated and LMF-initiated PRS request under the same framework.. Proposal: - The two UE features for on-demand PRS are needed for location server to know if the feature is supported. |
| CATT [6] |  |

Other issues

| Company | Summary |
| --- | --- |
| Huawei/HiSilicon [2] | - We propose to add the following UE feature subject to progress in 8.5.1. 27. NR_pos_enh; 27-xx1; Support of MTW; Support of PRS measurement time window (PTW) configuration.; No; PRS measurement time window is not supported by UE; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling.. 27. NR_pos_enh; 27-xx2; Support of positioning SRS with antenna switching; Support of positioning SRS resource set configured with antenna switching 1. The number of positioning SRS resources in a positioning SRS resource set configured with antenna switching Note: The switching period follows the existing MIMO SRS antenna switching (15us as per R1-1710048).; 13-4; Yes; SRS positioning configured with antenna switching is not supported; Per FS; n/a; n/a; n/a; The candidate values are {1,2,4}.; Optional with capability signaling. 27. NR_pos_enh; 27-xx3; Support of PRS TEG association information for UE-based DL-TDOA; Support of reception of association between PRS and TRP Tx TEG for UE-based positioning; No; Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs; Optional with capability signaling. - We propose to add the following UE feature subject to progress in 8.5.4. 27. NR_pos_enh; 27-uu1; Support of lower Rx beam sweeping factor; Support of the lower Rx beam sweeping factor than 8 for FR2 1. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7}; No; UE only supports 8 as the Rx beam sweeping factor defined by RAN4.; Per band (FR2 only); n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signaling. 27. NR_pos_enh; 27-uu2; Support of UL MAC CE based MG activation request; 1. Support of using UL MAC CE to request measurement gap.; Yes; Using UL MAC CE to indicate PRS measurement to the gNB is not supported.; Per UE; No; No; No; Optional with capability signaling. 27. NR_pos_enh; 27-uu3; Support of DL MAC CE based MG activation; 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG; Yes; Using DL MAC CE to activate the preconfigured MG is not supported; Per UE; No; No; No; Optional with capability signaling.. 27. NR_pos_enh; 27-uu4; Support of DL MAC CE based PRS processing window activation; 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the PRS processing window; Yes; Using DL MAC CE to activate the preconfigured PRS processing window is not supported; Per UE; No; No; No; Optional with capability signaling. - We propose to add the following UE feature subject to progress in 8.5.5. 27. NR_pos_enh; 27-vv1; LOS/NLOS indicator for UE-based positioning assistance data; Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both}; No; Per UE; No; No; No; Need for location server to know if the feature is supported.; Optional with capability signaling.. 27. NR_pos_enh; 27-vv2; Additional path reporting for DL-TDOA; 1. Support of TOA reporting for more than 2 additional paths. 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported.; No; Per UE; No; No; No; Need for location server to know if the feature is supported.; Optional with capability signaling.. 27. NR_pos_enh; 27-vv3; Additional path reporting for DL-AoD; 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported.; No; Per UE; No; No; No; Need for location server to know if the feature is supported.; Optional with capability signaling.. 27. NR_pos_enh; 27-vv4; Additional path reporting for Multi-RTT; 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported.; No; Per UE; No; No; No; Need for location server to know if the feature is supported.; Optional with capability signaling.. - We propose to add the following UE feature subject to progress in 8.5.6. 27. NR_pos_enh; 27-ww1; Support of PRS measurement in RRC_INACTIVE; Support of PRS measurement in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference; Yes; Per UE; No; No; No; Optional with capability signaling.. 27. NR_pos_enh; 27-ww1a; Support of PRS measurement in RRC_INACTIVE state for DL-TDOA; Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA.; No; Per UE; No; No; No; Need for location server to know if the feature is supported.; Optional with capability signaling.. 27. NR_pos_enh; 27-ww1b; Support of PRS measurement in RRC_INACTIVE state for DL-AoD; Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD.; No; Per UE; No; No; No; Need for location server to know if the feature is supported.; Optional with capability signaling.. 27. NR_pos_enh; 27-ww1c; Support of PRS measurement in RRC_INACTIVE state for Multi-RTT; 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT.; No; Per UE; No; No; No; Need for location server to know if the feature is supported.; Optional with capability signaling.. 27. NR_pos_enh; 27-ww2; Support of positioning SRS transmission in RRC_INACTIVE state; 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component; Yes; Per band; n/a; n/a; n/a; Optional with capability signaling. 27. NR_pos_enh; 27-ww2a; Support of positioning SRS transmission in RRC_INACTIVE state; 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14}; 27-ww2; No; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported UE only reports the number on the PCell band.; Optional with capability signaling. 27. NR_pos_enh; 27-ww3; Support of positioning SRS configured outside UL BWP#0 configured by system information in RRC_INACTIVE state; 1. Support positioning SRS configured outside UL BWP#0 configured by system information in RRC_INACTIVE state 2. SRS switching period: following existing SRS-SwitchingTimeNR IE.; 27-ww2; Yes; Per band; n/a; n/a; n/a; Optional with capability signaling. 27. NR_pos_enh; 27-ww4; Supported positioning SRS bandwidth in RRC_INACTIVE; 1. Supported bandwidth for positioning SRS transmission in RRC_INACTIVE; 27-ww2; Yes; Per band; n/a; n/a; n/a; Optional with capability signaling. 27. NR_pos_enh; 27-ww4a; Supported positioning SRS bandwidth in RRC_INACTIVE; 1. Supported bandwidth for positioning SRS transmission in RRC_INACTIVE; 27-ww2, 27-ww2a; No; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-ww5; OLPC for positioning SRS in RRC_INACTIVE state; Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16; Yes; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-ww6; Spatial relation for positioning SRS in RRC_INACTIVE state; Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16; Yes; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| ZTE [3] | One more agreement was made in RAN1#106b-e meeting as follows. Hence, one new FG should be introduced. Agreement: Introduce a new UE capability on lower Rx beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers. - Send an LS to RAN4 to confirm.. Proposal: Introduce a new FG for support of lower Rx beam sweeping factor for FR2. - The report granularity is per UE - The candidate values are {1, 2, 4} For DL-TDOA and multi-RTT, it has been agreed to support N>2 additional paths from UE to LMF for DL-TDOA and multi-RTT. Hence, we think one new FG should be introduced to report whether UE supports additional path measurement. In our view, single value N = 4 is sufficient. The prerequisite should be FG 13-1 which is basic PRS processing capability. Proposal: Introduce a new FG on support of additional path measurement. - The report granularity is per UE. - This FG is applicable either DL-TDOA or Multi-RTT - UE can report up to 4 additional path measurement In additional, based on the following agreement, one more FG is needed for path RSRP including first path and additional path RSRP report for DL-TDOA or Multi-RTT. Agreement: Support reporting the path RSRP for the first path and for additional paths as part of DL-TDOA, UL-TDOA, and multi-RTT reporting enhancements. - FFS: Support introducing a request from the LMF to the UE/TRP when the path-RSRP for additional paths is desired to be reported. FFS: Support of path RSRP for additional paths as part of DL-AoD.. Proposal: Introduce a new FG on support of path RSRP reporting - The report granularity is per UE. - This FG is applicable either DL-TDOA or Multi-RTT - This FG is applicable for both first and additional path For UL or UL+DL positioning, UE has to let gNB know whether it supports SRS transmission in RRC inactive state. Then, gNB can decide whether to configure SRS transmission or trigger UE accessing in RRC connected state for UE in RRC inactive state. As for SRS time domain type, we don’t think this FG needs to differentiate periodic and semi-persistent SRS. In our view, if UE reports this FG and supports semi-persistent SRS in RRC connected state, UE has to also support semi-persistent SRS in RRC inactive state. Proposal: Introduce a new FG on support of SRS transmission in RRC inactive state - The report granularity is per UE. - Need for gNB to know if FG is supported - If UE reports this FG and supports semi-persistent SRS for positioning in RRC connected state (i.e. FG13-8b), UE has to also support semi-persistent SRS in RRC inactive state. For DL positioning in RRC inactive state, it has been agreed in RAN2 that RRC state is not exposed to LMF. That is, LMF will not be aware of whether UE is in RRC_INACTIVE or RRC_CONNECTED state, and gNB is not aware of NAS signaling including location request signaling or assistance data via LPP. Hence, it does not make sense to introduce an optional UE capability reported to either LMF or gNB that the UE supports NR DL positioning in RRC_INACTIVE state. - If such UE capability is reported to LMF, what do we expect LMF do? LMF even does not know whether UE is in RRC_INACTIVE state or not, so LMF cannot implement any operation corresponding to UE’s RRC states. - It is also inappropriate to report such UE capability to serving gNB because gNB does not decode LPP message, and it is not aware whether location request has been initiated or not. That is, gNB cannot trigger DL-PRS measurement and report no matter UE is in RRC_INACTIVE or RRC_CONNECTED. It is meaningless for UE to report such UE capability to gNB. In our view, a UE can be deemed to support NR DL positioning in RRC_INACTIVE state if it supports both SDT feature and DL/DL+UL positioning method. Specifically, if UE supports SDT feature, it implies that the UE can report DL-PRS measurement results via UL-SDT transparently to gNB without any extra UE capability dedicated for positioning. In other words, the UE capabilities of DL positioning designed for UE in RRC_CONNECTED state are also applied to UE in RRC_INACTIVE sate. Proposal: Do NOT support an additional UE capability for NR DL positioning in RRC_INACTIVE |
| Nokia/Nokia Shanghai Bell [4] | - Add new FG to cover the case of phase center offset compensation for TEGs. See details for proposed FG in the Annex. |
| OPPO [5] | To reduce the NR positioning measurement and reporting latency, the issue of M (< 4)- sample PRS processing was discussed. From the perspective of RAN1, by assuming that RAN4 agrees to support the M-sample measurement, it was agreed to support M-sample measurement as UE capability: Agreement: Subject to UE capability, support LMF to explicitly request UE to report the measurement with either M-sample or 4-sample, if RAN4 has supported M-sample measurement. - FFS signalling details.. Supporting M-sample measurement is a UE capability. A dedicated UE capability shall be introduced for this feature. The UE can report if the UE supports the M-sample measurement and the UE can also report the values of M that the UE can support. M = 1 shall be a candidate value. Proposal: Introduce a new UE capability for supporting M-sample measurement - the UE reports the supported M value. M = 1 is one candidate value. There were different views on the UE capability of DL PRS reception in RRC_INACTIVE state. The main reason not supporting this UE capability is that LMF is not aware of RRC state. On the other than, UE can report this UE capability to gNB and facilitate the gNB to do better decision. Thus, we have the following proposal: Proposal: Introduce a UE capability for the DL PRS reception in RRC_INACTIVE state - This UE capability is reported to gNB - FFS: additional sub-groups for this UE capability, e.g., PRS processing capability, supported positioning methods. SRS transmission for positioning is also supported for UE in RRC_INACTIVE state. This feature should also be reported by UE capability signaling Proposal: Introduce a UE capability for SRS transmission for positioning in RRC_INACTIVE state - This UE capability is reported to gNB - FFS: additional sub-groups for this UE capability, e.g., types of SRS, power control. In RAN1#106bis-e meeting, it is agreed that for RRC_INACTIVE UEs, SRS for positioning bandwidth, SCS and CP type are configured by RRC and can be different from that of initial UL BWP. This agreement should not mandate all UE to support this feature. Thus, we prefer to have a separate UE feature for it: Proposal: Introduce a UE capability for the following sub-group feature - For RRC_INACTIVE UEs, SRS for positioning bandwidth, SCS and CP type are configured by RRC and can be different from that of initial UL BWP configured by the system information - If not supported, UE in RRC_INACTIVE state can only transmit the SRS within the initial UL BWP with the same SCS and CP type |
| Samsung [7] |  |
| Intel Corporation [6] | RAN1 agreed to introduce a new UE capability on lower RX beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers. Support of reduced RX beam sweeping factor RAN1 agreed to introduce DL MAC CE for MG activation for the purpose of DL PRS measurements. The following new FG can be introduced to reflect it: Support of low latency MG activation request from LMF (DL MAC CE for MG activation request) Finally, RAN1 agreed to introduce UL MAC CE for MG activation request by UE. The following new FG can be introduced to reflect it: Support of low latency MG activation request from UE (UL MAC CE for MG activation request) Additional FGs may need to be defined for NR latency reduction support, once more progress on design options is made in RAN1. Proposal: - Define the following new FGs for NR positioning with lower latency - 27-3-2: Support of reduced RX beam sweeping factor for FR2 - Support of RX beam sweeping factor < 8 - 27-3-3: Support of low latency MG activation request from LMF - Support of DL MAC CE for MG activation request - 27-3-4: Support of low latency MG activation request from UE - Support of UL MAC CE for MG activation request In addition, the extension of the set of existing values for DL PRS processing capabilities within MG, in particular the processing window T can be beneficial for latency reduction and set of values {1 ms, 2 ms, 4 ms} can be introduced. Proposal: - Introduce additional values {1ms, 2ms, 4 ms} for parameter T defined for UE DL PRS processing capabilities within MG - Add new values for R17 UEs for Rel-16 FG 13-1 Common DL PRS Processing Capability - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms Additional FGs need to be introduced reflecting LOS/NLOS mitigation framework developed by RAN1. At least the following FGs need to be specified. - LoS/NLoS indicator for DL-TDOA positioning / DL PRS RSTD measurement - LoS/NLoS indicator for DL-AOD positioning / DL PRS RSRP measurement - LoS/NLoS indicator for Multi-RTT Each of the above FGs can be further split into two FGs related to soft / hard value for LOS/NLOS metric. In addition, dedicated FG needs to be introduced for LoS/NLoS assistance information to support UE-based positioning. Proposal: - Replace FG 27-4-1 with multiple FGs associated with NR positioning LOS/NLOS mitigation capabilities per positioning method - LoS/NLoS soft/hard value indicator for DL-TDOA positioning / DL PRS RSTD measurement - LoS/NLoS soft/hard value indicator for DL-AOD positioning / DL PRS RSRP measurement - LoS/NLoS soft/hard value indicator for Multi-RTT - Introduce additional FG indicating that UE can use LoS/NLoS assistance information for UE based positioning - LoS/NLoS assistance information for UE-based positioning It was agreed by RAN1 to support reporting the path RSRP for the first path and for additional paths as part of DL-TDOA, UL-TDOA, and multi-RTT reporting enhancements. This agreement require definition of at least 4 new FGs: Proposal: - Define FGs associated with the per path DL PRS RSRP measurement support instead of single FG 27-2-1 - First path DL PRS RSRP for DL-TDOA positioning - First path DL PRS RSRP for DL-AOD positioning - First path DL PRS RSRP for Multi-RTT positioning - Additional path DL PRS RSRP for DL-TDOA positioning - Additional path DL PRS RSRP for DL-AoD positioning - Additional path DL PRS RSRP for Multi-RTT positioning Support of NR positioning by RRC_INACTIVE UEs has been added in Rel.17 for DL, UL and DL+UL positioning methods. The corresponding FGs need to be added to UE feature list. The first family of the FGs that needs to be added is associated with NR positioning measurement capabilities for UEs in RRC_INACTIVE state. Proposal: - Define FGs associated with NR positioning measurements in RRC-INACTIVE state - FG 27-x2-1: DL PRS measurement for DL-AoD in RRC_INACTIVE state - FG 27-x2-2: DL PRS measurement for DL-TDoA in RRC_INACTIVE state - FG 27-x2-3: UE Rx-Tx measurement for Multi-RTT in RRC_INACTIVE state - FG 27-x2-4: SS-RSRP RRM measurements for NR E-CID positioning in RRC_INACTIVE state - FG 27-x2-5: SS-RSRQ RRM measurements for NR E-CID positioning in RRC_INACTIVE state - FG 27-x2-6: CSI-RSRP RRM measurements for NR E-CID positioning in RRC_INACTIVE state - FG 27-x2-7: CSI-RSRQ RRM measurements for NR E-CID positioning in RRC_INACTIVE state - FG 27-x2-8: Simultaneous DL-AoD and DL-TDoA processing in RRC_INACTIVE state - FG 27-x2-9: Simultaneous DL-AoD and Multi-RTT processing in RRC_INACTIVE state In Rel.16, the FGs for DL PRS resources for various NR positioning techniques were defined [3]. The same FGs are in general applicable for NR positioning by RRC-INACTIVE UEs. Proposal: - The Rel.16 NR positioning FGs (13-2/2a/2b, 13-3/3a/3b, 13-4/4a/4b) for DL PRS resources and different NR positioning techniques are reused and defined as pre-requisites for FGs associated with NR positioning measurements by RRC-INACTIVE UEs RAN1 has agreed on SRS for positioning transmission by RRC_INACTIVE UEs. The discussion on type of SRS for positioning to be supported (except periodic SRS for positioning) is still open. The support of OLPC and spatial relationship was agreed to be reused from Rel.16. The support of SRS for positioning by RRC_INACTIVE UEs should be reflected in UE features. gNB should be at least aware about support of SRS for positioning transmission in RRC_INACTIVE state. Another aspect that needs to be discussed is that for RRC_CONNECTED UEs SRS for positioning is configured within active UL BWP. RAN1 agreed that SRS for positioning in RRC-INACTIVE state can be configured outside of initial UL-BWP and have different numerology settings. This may require redefinition of the FGs for SRS for positioning resources for RRC-INACTIVE UEs (as there is no motivation to introduce parameters that are defined per UL BWP). Based on discussion, we have following proposal. Proposal: - Define new FGs for SRS for positioning support in RRC_INACTIVE state: - FG 27-x3-1: SRS for positioning transmission in RRC_INACTIVE state - FG 27-x3-2: Support of SRS for positioning resources in RRC_INACTIVE state - FG 27-x3-3: [Support of periodic SRS for positioning resources in RRC_INACTIVE] - FG 27-x3-4: [Support of semi-persistent SRS for positioning resources in RRC_INACTIVE] - Further discuss whether to reuse R16 FGs for SRS for positioning OLPC (FGs 13-9/9a/9b/9e/9f) and spatial relation (FGs 13-10/10a/10b/10d/10e/10f) or define new FGs for support of SRS for positioning in RRC_INACTIVE state (considering R16 FGs defined for RRC_CONNECTED state), i.e.: - OLPC - FG 27-x4-1: OLPC for SRS for positioning based on DL PRS from serving cell - FG 27-x4-2: OLPC for SRS for positioning based on DL PRS from neighboring cells - FG 27-x4-3: OLPC for SRS for positioning based on SSB from serving cells - FG 27-x4-4: Pathloss estimate maintenance for serving cell - FG 27-x4-5: Pathloss estimate maintenance across all cells - Spatial relation - FG 27-x5-1: Spatial relation for SRS for positioning based on SSB from serving cell - FG 27-x5-2: Spatial relation for SRS for positioning based on CSI-RS from serving cell - FG 27-x5-3: Spatial relation for SRS for positioning based on DL PRS from serving cell - FG 27-x5-4: Spatial relation for SRS for positioning based on SSB from non-serving cells - FG 27-x5-5: Spatial relation for SRS for positioning based on DL PRS from non-serving cells - FG 27-x5-6: Spatial relation maintenance |
| Qualcomm Incorporated [8] | We make the following general comments with regards to the first draft of the UE feature list on NR Positioning Enhancements: Observation: The feature group for the support of a feature should be separated from the feature group for values associated to the feature (as it was done in NR Rel-16 UE feature discussion). Observation: UE capabilities related to RRC Inactive Positioning are missing. Observation: Any additional information in the assistance data should be associated with a UE capability so that the LMF would know that the UE can accept the new assistance data. Proposal: Unless otherwise stated, the type for a UE feature should be at least per band (if not with finer granularity type), given the potential UE testing differentiation among licensed, unlicensed, and NTN band. Transmission of SRS in RRC inactive is a new feature and should be clearly be associated with a UE capability. It has very specific UE impacts, for example, for RTT positioning, a UE would need to keep the group delay calibration between the PRS and the SRS. Such feature is only specified in NR Rel-16 for PRS and SRS in RRC Connected state. Keeping the same (or even different) group delay calibration if the PRS and/or SRS are in RRC Inactive would be a new UE feature that needs additional implementation efforts. Also, in RAN4, new requirements may need to be defined for measurement accuracy, measurement period and reporting. For these requirements to be really testable, there should be the corresponding capability bits. Proposal: Introduce a per-band UE capability for UEs transmitting SRS in RRC inactive state. With regards to DL PRS processing, similar to the SRS transmission, a UE may be capable of different number of PRS resources per slot, different maximum PRS resource configurations, different group delay calibration for RSTD measurement (related to RAN4 margins for RSTD measurements). Proposal: A per-band DL positioning capabilities should be defined in inactive state, e.g. including at least - DL PRS processing capability in inactive state - UE Rx-Tx measurement reporting - DL RSTD measurement reporting - RSRP measurement reporting - Path-RSRP measurement reporting If there is no UE capability for PRS processing or SRS transmission in RRC inactive, there will be NO way to test whether a feature is supported or not. A gNB doesn’t know whether the UE will be able to do PRS and/or SRS transmission; it will be a non-existent, non-deployable, and non-testable feature. Observation: No UE capability for PRS reception or SRS transmission in RRC inactive, will render the feature practically non-existent, non-deployable and non-testable. There have not been introduced any UE capabilities for the multipath positioning features for the timing domain methods. We make the following proposals: Proposal: Introduce the following UE capabilities: • Support of Maximum number N>2 additional path reporting • Support of path-RSRP reporting for additional Paths for Multi-RTT • Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA • Support of path-RSRP reporting for first Path for Multi-RTT • Support of path-RSRP reporting for first Path for UE-assisted DL-TDOA • Support of LOS/NLOS indicators in the Assistance Data for UE-based DL-AoD In the previous RAN1 meeting, the following agreement was reached with regards to Batch Measurement Reporting: Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.. Even though this feature hasn’t been completed yet, we think it is important to put a placeholder in the UE features on the maximum number of measurement instances which can be included in a single measurement. Proposal: Introduce a per-UE capability on the maximum number of measurement instances which can be included in a single measurement report with the values {2,4,5,8,10,16,20,32}. |
| vivo [4] | Agreement: For the beam/antenna information to be optionally provided to the LMF by the gnodeB, decide to support one of the following options: - Option 2.1: The gNB reports quantized version of the relative Power/Angle response per PRS resource per TRP - The relative power is defined with respect to the peak power of that resource - FFS: How many relative power levels can be included (e.g., single -3 dB power-levels, multiple power-levels, etc). - Option 2.2: The gNB reports quantized version of the relative Power between PRS resources per angle per TRP. - The relative power is defined with respect to the peak power in each angle - For each angle, at least two PRS resources are reported. - FFS: support of multiple levels of quantization - FFS: how the report is constructed - FFS: overhead reduction mechanisms, including reusing of associated-dl-PRS-ID as a way of signaling that 2 TRPs have the same beam information - The gNB beam/antenna information can optionally be provided to the UE by the LMF - Note: Up to RAN2 & RAN3 the signaling/procedures on how the LMF receives this information from the gNBs - Send an LS to RAN2 & RAN3 with this agreement. In RAN1#106-e meeting, the above agreement has been reached. It can be found the gNB beam/antenna information can be optionally be provided to UE. We would like to confirm by the majority whether the word “optionally” is dependent on UE capability. That is only when the UE can use the beam information in positionings, the LMF can provide the gNB beam/antenna information to UE in unicast. If it is, a new capability or assistance data request information is needed to be introduced. For example Nr-AoD-enhancementWithBeamInformation ENUMERATED {supported} Proposal: - To discuss whether to introduce a new UE capability for UE to request the gNB beam/antenna information from UE to the LMF. Agreement: Support the following options (in the agreement made in RAN1#106-e) for a new mechanism of MG activation request for the purpose of positioning. - Option 2: by UE ( UL MAC CE) - Select only one of UCI and UL MAC CE in RAN1#106bis-e - Option 1: by LMF (via an NRPPa message) - Note: This is transparent to the UE Agreement: Support the following option (from the agreement made in RAN1#106-e) for a new MG activation procedure to be performed by the gNB for the purpose of positioning. - Option 2: DL MAC CE - FFS: Deactivation process. Based on the above agreement in RAN1#106bis-e meeting, two new mechanisms of MG activation request for the purpose of positioning. One is through UL MAC CE by the UE, another is through NRPPa by the LMF. For UE capability, in our view, it is needed for gNB to know whether supporting the new mechanisms of MG activation request through UL MAC CE by the UE. Similarly, a new MG activation by DL MAC CE also needs to make gNB know whether the UE supports the new mechanisms of MG activation through DL MAC CE. So, we propose - Introduce a new UE capability for the support of UE to request the MG by UL MAC CE. - Introduce a new UE capability for the support of UE to be activated an MG by DL MAC CE. Potential new UE capabilities for PRS measurement need to be considered for inactive state positioning, which should include the following at least: - DL PRS processing capability in inactive state. In inactive state, in order to save power, the UE may turn off some functions and modules, so the PRS processing capability may be limited. For example, the maximum supported PRS bandwidth per frequency layer may be smaller than connected state; the maximum number of supported frequency layers may be smaller than connected state; the durationOfPRS-Processing capability (N, T) for any P(≥T) time window may also be different with connected state, etc. In addition, considering PRS processing capability in connected state is defined assuming measurement gap is configured, while in inactive state, there may be no measurement gap is assumed, so that the PRS processing capability without measurement gap should be considered in inactive state. Therefore, the structure of DL PRS processing capability outside MG (feature 27-u6) in connected state can be reused for inactive state, but the specific values may be different. In addition, regarding whether to reuse the UE capability of ‘PRS measurement outside MG’ (feature 27-u5), our view is not to reuse it. The reason is that in inactive state, other DL signals are important signals (e.g. SSB, SIB1, COREST0, MSG2/MSGB, paging, etc.) for initial access which should be treated as high priority, it is weird to introduce a capability based on ‘higher priority for PRS measurement’ in inactive state. - DL PRS resource capability in inactive state. This capability is related to the number of TRPs, PRS resources, set, frequency layers to be supported. The following table is an example. We think we can reuse the structure in connected state, but the specific values may be different. NR-DL-PRS-ResourcesCapability-r16 ::= SEQUENCE { maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16 INTEGER (1..2), maxNrOfTRP-AcrossFreqs-r16 ENUMERATED { n4, n6, n12, n16, n32, n64, n128, n256, ...}, maxNrOfPosLayer-r16 INTEGER (1..4), dl-PRS-ResourcesCapabilityBandList-r16 SEQUENCE (SIZE (1..nrMaxBands-r16)) OF DL-PRS-ResourcesCapabilityPerBand-r16, dl-PRS-ResourcesBandCombinationList-r16 DL-PRS-ResourcesBandCombinationList-r16, ... }. - DL PRS QCL processing capability in inactive state. This capability is related to QCL information support, e.g. whether support of SSB from neighbor cell as QCL source of a DL PRS. If QCL procedure in connected state is reused in inactive state, this capability can also be reused. - DL measurements capability in inactive state. This capability is related to PRS measurement and report, e.g. the maximum number of DL RSTD/RSRP per pair of TRPs. We think we can reuse the structure in connected state, but the specific values may be different. However, based on previous RAN2’s conclusion, the UE state (connected or inactive) is transparent to LMF, and whether UE capability for DL positioning is useful to LMF is unclear. We think whether to support reporting the capability to LMF is more like a RAN2 issue and better for RAN2 to discuss first. Then, based on RAN2’s conclusion, the detailed components can be discussed by RAN1. Proposal: - It is up to RAN2 to determine whether to include DL positioning capability in inactive state in UE feature list. Different from DL positioning, in UL or UL+DL positioning, the use of gNB to obtain the SRS transmission capability of UE in the inactive state is clear. Therefore, for UL and DL+UL positioning, firstly, the capability of SRS for positioning transmission in inactive state should be support and reported to the serving gNB. In addition, the general capability of SRS in connected state can be reused. However, regarding the capability of semi-persistent SRS, it is concluded in RAN1#106bis-e: it is up to RAN2 to confirm support of semi-persistent SRS for positioning by RRC_INACTIVE UEs. Therefore, whether to support semi-persistent SRS remains to be confirmed. Then, for aperiodic SRS, it seems to be supported by a few companies, and whether not to support it should be further confirmed. Besides, considering the UE state is transparent to LMF, similar to DL positioning, whether to report this capability to LMF is more like a RAN2 issue and better decided by RAN2. Proposal: - For UE capability related to UL positioning in inactive state, support to reuse UE capability structure of SRS for positioning transmission in connected state. - It is up to RAN2 to confirm whether to support capability of semi-persistent SRS in inactive state. - Whether not to support aperiodic SRS in inactive state should be further confirmed. - The capability should be reported to the serving gNB at least, and whether to report this capability to LMF is up to RAN2. - The specific values of UE capability of SRS for positioning in inactive state may be different from connected state. 27. NR_pos_enh; 27-v2a; Additional path reporting N>2 for UE-assited DL-TDOA and/or Multi-RTT; Maximum number N>2 additional path reporting; No; Per Band; n/a; n/a; n/a; N = {4,6,8} Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-v2e; Support of path-RSRP reporting for first Paths for Multi-RTT; Support of path-RSRP reporting for first Path for Multi-RTT; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-v2e; Support of path-RSRP reporting for additional Paths for Multi-RTT; Support of path-RSRP reporting for additional Paths for Multi-RTT; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-v2e; Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA; Support of path-RSRP reporting for first Path for UE-assisted DL-TDOA; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-v2e; Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA; Support of path-RSRP reporting for additional Paths for UE-assisted DL-TDOA; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-c1; Support of Transmission of periodic SRS in RRC Inactive State; Support of Transmission of periodic SRS in RRC Inactive State; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Need for gNB to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-c2; Support of Transmission of Semi-persistent SRS in RRC Inactive State; Support of Transmission of Semi-persistent SRS in RRC Inactive State; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Need for gNB to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-c3; Support of PRS processing in RRC inactive state; Support of DL PRS processing in RRC inactive state; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Need for gNB to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-c4b; DL PRS processing capabilities in RRC inactive state; 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-c4c; Support of DL RSTD measurement in RRC Inactive state; Support of DL RSTD measurement in RRC Inactive state; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-c4c; Support of UE Rx-Tx measurement in RRC Inactive state; Support of UE Rx-Tx measurement in RRC Inactive state; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-c4c; Support of UE Rx-Tx measurement in RRC Inactive state; Support of RSRP measurement in RRC Inactive state; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-c4c; Support of UE Rx-Tx measurement in RRC Inactive state; Support of path-RSRP measurement in RRC Inactive state; No; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-d4; Support beam Information in the Assistance Data for UE-based DL-AoD; Support beam Information in the Assistance Data for UE-based DL-AoD; No; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-d5; Support of LOS/NLOS indicators in the Assistance Data for UE-based DL-AoD; Support of LOS/NLOS indicators in the Assistance Data for UE-based DL-AoD; No; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. 27. NR_pos_enh; 27-d5; Maximum number of measurement instances which can be included in a single measurement report; Maximum number of measurement instances which can be included in a single measurement report; No; Per UE; n/a; n/a; n/a; Candidate values {2,4,5,8,10,16,20,32}. Need for location server to know if the feature is supported.; Optional with capability signaling. |
| CATT [6] | The following working assumption and agreement were achieved related to UE capability for mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors in RAN1#106bis-e. Working assumption: - For mitigating UE Tx timing errors for UL TDOA, subject to UE’s capability, support the serving gNB to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE supports multiple UE Tx TEGs for UL TDOA. - The serving gNB should forward the association information provided by the UE to the LMF. - FFS: whether to support the serving gNB to forward the association information to the neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for UL TDOA to serving gNB. - For mitigating UE Tx timing errors for Multi-RTT, subject to UE’s capability, support the LMF to request a UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE supports multiple Tx TEGs for Multi-RTT. - FFS: whether to support the LMF to forward the association information to the serving and neighboring gNBs - UE should report its capability of supporting multiple UE Tx TEGs for Multi-RTT directly to the LMF. - FFS: Mitigation of UE Tx timing errors when Multi-RTT, UL-TDOA and/or DL-TDOA are used. Agreement: - Support the following parameters and values related to the accuracy enhancement for mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors: Parameter Description; Values in specifications (e.g., TS 37.355, TS 38.455); Values that can be signaled as part of UE Capability; Comments. The maximum number of UE RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT]; [32]; [1, 2,4,6,8,12,16,24,32] FFS: per UE/band /FL/FR; The parameter is used for supporting DL-TDOA and/or Multi-RTT. The maximum number of UE TxTEGs [for UL-TDOA and/or Multi-RTT]; [8]; [1, 2,4,6,8] FFS: per UE/band /FL/FR; The parameter is used for supporting UL-TDOA and/or Multi-RTT. The maximum number of UE-RxTx TEGs; [256]; [1, 2,4,6,8,12,16,24,32,64, 128, 256] FFS: per UE/band /FL/FR; The parameter is used for supporting Multi-RTT. - Note: Above proposal does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets. Agreement: Make the following modification on the previous agreement made in RAN#106e: - Subject to UE capability, support the LMF to request a UE to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. - N=[2, 3, 4, 6, 8] (FFS: other values), where the maximum value of N depends on UE capability - The TRP can be either a “RSTD” reference TRP or a neighbour TRP - FFS: details of the signalling, procedures, and UE capability - The timestamps of the multiple RSTD measurements in the same measurement report can be the same or different. - Note: All RSTD measurements are relative to a single reference timing - Support the LMF to request a TRP to optionally measure the same SRS resource of a UE with M different TRP Rx TEGs and report the corresponding multiple RTOA measurements. - M = [2, 3, 4, 6, 8] (FFS: other values) - FFS: details of the signalling, procedures - The timestamps of the multiple RTOA measurements in the same measurement report can be the same or different.. The FGs 27-1-1, 27-1-2, 27-1-3 and 27-1-4 in section 6 of R1-2109915 should be changed according to the above working assumption and agreement. The following agreement was achieved related to UE capability for latency reduction in RAN1#106bis-e. Agreement: Introduce a new UE capability on lower Rx beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers. - Send an LS to RAN4 to confirm.. According to the above agreement, we prefer to add a new FG 27-3-4 to reflect the new UE capability on lower Rx beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers. Proposal: Adding the following new FG 27-3-4 in order to reflect the agreement related to UE capability for latency reduction in RAN1#106bis-e: 27. NR_pos_enh; 27-3-4; Lower Rx beam sweeping factor; Support lower Rx beam sweeping factor (<8) to reduce the PRS measurement latency for FR2 positioning frequency layers.. |

# Discussion/Approval Items during RAN1 #107-e — First Checkpoint

After review of contributions submitted to RAN1 #107-e in this agenda item, the following topics were identified by the moderator for discussion/approval during RAN1 #107-e.

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 27-1-1

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 27. NR_pos_enh | 27-1-1 | Maximum number Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE [for UE assisted DL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | 13-1 [,one or more of {13-3, 13-4}], 27-1-1b | No |  | Mitigation of UE Rx timing delays is not supported UE-RxTEG reporting is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1,2, 3,4,6,8,12,16,24,32}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Basically support. Regarding the ‘Prerequisites’, we wonder why ‘27-1-1b’ is included. |
| ZTE | For candidate values, we think 3 is not a typical case, and prefer to remove it. For the following wording, we are not convinced why RxTEG-ID should be always reported if only 1 UE-RxTEG is supported at UE side. This just wastes signaling overhead. In our view, ‘Value=1 indicates that the timing error differences between all DL measurements are within a certain margin’ should replace the following part. if UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements. |
| Qualcomm | - We have a main problem with this proposal which needs a clarification: - This proposal, may mean that TEGs have “boundaries” at the band-level; Eg, it may not allow, for inter-frequency measurements, to say that 2 TOA measurements, even though they are on different bands, they are the same TEG. Lets take the example of DL-TDOA reporting. Within a single IE, measurements across all 4 PFLs could be included. If, lets say, a UE reports a capability of “8” RxTEGs for each one of 4 bands, and the UE is configured with 4 PFLs, each one on a different band, will it mean that the RxTEG-IDs in the report will be {0,1,…7}, or will it be {0,1,…31}? - If it is the former understanding, we have serious concerns, as such a solution doesn’t allow to signal that 2 measurements from different bands have the same or different TEG. It will always be assumed that they have different TEGs. - To ZTE: The NOTE doesn’t say that if value=1, then UE-RxTEG is always reported. It is not true that if “RxTEG=1”, it means that all the measurements are within a certain margin. There can be cases that the UE may not be able to provide a RxTEG, even if “value=1”. So, saying that, if the reports value=1, it means that all the measurements are in the same TEG, is not a valid statement. Even if “RxTEG=1”, a UE should be able to report which measurements have the same timing errors. - On the interpretation when the capability is not supported: We think it should be complemented as follows: “UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements” |
| Huawei, HiSilicon | Generally OK even though we prefer to have separate capabilities for DL-TDOA and Multi-RTT, since the band entry for the two methods could be different. Reply to QC: To our understanding, the “per band” reporting on capability does not imply the RxTEG ID associated with measurement report is grouped per band, but rather it is per UE. It means that even if UE reports 8 for two bands, the RxTEG ID reported associated with the measurement can be the same for PRS on two bands, implying that the two measurements have the timing error difference within the margin. |
| CATT | According to the following agreement, we prefer the candidate values of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning should be up to 32. Agreement: - Support the following parameters and values related to the accuracy enhancement for mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors: Parameter Description; Values in specifications (e.g., TS 37.355, TS 38.455); Values that can be signaled as part of UE Capability; Comments. The maximum number of UE RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT]; [32]; [1, 2,4,6,8,12,16,24,32] FFS: per UE/band /FL/FR; The parameter is used for supporting DL-TDOA and/or Multi-RTT. The maximum number of UE TxTEGs [for UL-TDOA and/or Multi-RTT]; [8]; [1, 2,4,6,8] FFS: per UE/band /FL/FR; The parameter is used for supporting UL-TDOA and/or Multi-RTT. The maximum number of UE-RxTx TEGs; [256]; [1, 2,4,6,8,12,16,24,32,64, 128, 256] FFS: per UE/band /FL/FR; The parameter is used for supporting Multi-RTT. - Note: Above proposal does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets. |
| Nokia, NSB | Generally OK. The note should be updated to say: If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports the same Rx-TEG-ID associated with both measurements, otherwise, no assumption can be made about the timing error differences between these measurements. |
| Qualcomm | - We also prefer different entries for TDOA/MRTT. It was written in the paper, but i noticed, i didn’t include that in the previous reply. - To Huawei/all: If all have the same understanding on what the “per band” means here and the differenation with the measurement report, then we can be OK assuming the following is added as a note: - Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31. |
| OPPO | - Regarding the value =1, we share the similar view as ZTE. - This feature should be “per band per band combination”. For typical UE implementation, the hardware may be shared by some bands. For example, if UE is working on Band A, it can use 4 Rx antennas. If UE is working on Band A and Band B, then UE may retune 2 antennas to Band B and use 2 antennas for each band. In some sense, it is similar to the UE capability of MIMO layers which is FSPC - We are open to one joint feature or two separate features for DL-TDOA and Multi-RTT |
| ZTE2 | @QC, for the note in red as below, can you clarify what a kind of scenario is assumed in your assumption? i.e. when UE reports it supports 1 UE-RxTEG, why sometimes the PRS receive timing errors are not within a margin, and sometimes are. If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an Rx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements. |

# Issue 2: FG 27-1-2

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 27. NR_pos_enh | 27-1-2 | Maximum number Support of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, 13-8], 27-1-2b | FFS Yes |  | Mitigation of UE Tx timing delays is not supported UE-TxTEGs for UL TDOA is not supported | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 6, 8, 16, 32}] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | per band | n/a | n/a | n/a | The candidate values are {1, 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Regarding FG of ‘Support of UE-TxTEGs for UL TDOA’, both serving gNB and LMF need to know if this feature is supported. Therefore, similar to R16 UE feature, this feature of ‘Support of UE-TxTEGs for UL TDOA’ reported to serving gNB and LMF can be divided to 2 FGs. The feature reported to serving gNB can have a separate row corresponding to FG27-2-1; and the feature reported to LMF can be combined with FG27-1-2a, e.g. 27-1-2a Support of UE-TxTEGs for Multi-RTT and/or UL TDOA positioning Then, regarding the ‘Prerequisites’, we wonder why ‘27-1-2b’ is included. |
| ZTE | - We don’t’ support 27-1-2a. The maximum UE-TxTEGs should be the same for UL TDOA and Multi-RTT. - Moreover, the following red part should be replaced by ‘Value=1 indicate that the timing error differences between all UL transmissions are within a certain margin’ If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources. - The following two notes should be included in 27-1-2 for clarification It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA if UL TDOA is supported by UE, It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE |
| Qualcomm | - To ZTE: One way or the other, there needs to be possible for a UE to signal that TxTEG is supported for UTDOA or MRTT, but not both, since the reporting is happening with different mechanisms! We cannot assume that if the UE supports TxTEG reporting through RRC, it also supports this reporting through LPP, and vice versa. - The “notes” should be added in both the fields. - On the interpretation: If the capability is not supported, what it means: We think it should be complemented with: “UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” |
| Huawei, HiSilicon | We prefer to have another capability indicating support of TxTEG feature for UL-TDOA (instead of the number of Tx TEGs) reported to the LMF, so that LMF is aware that UE supports the TEG feature for UL-TDOA, and request gNB to ask UE to provide it in RRC. We have preference to consider per FS for FG 27-1-2, since the TxTEG number per band is subject to CA band combination. The maximum number could be different for band under different band combination. |
| CATT | We are fine with two separate FGs, i.e., FG27-1-2 for UE-TxTEGs for UL TDOA, and FG27-1-2a for UE-TxTEGs for Multi-RTT, since maybe UE don’t support both UE-TxTEGs via LPP and via RRC. |
| Nokia, NSB | We are generally fine with the proposal. Agree with Qualcomm regarding the notes. |
| OPPO | - These features should be “per band per band combination”. For typical UE implementation, the hardware may be shared by some bands. For example, if UE is working on Band A, it can use 4 Rx antennas. If UE is working on Band A and Band B, then UE may retune 2 antennas to Band B and use 2 antennas for each band. In some sense, it is similar to the UE capability of MIMO layers which is FSPC - To ZTE’s comment on the issue of “value=1”, The UL is different from DL. For UL, the timing may be adjusted by gNB command and/or UE autonomous adjustment. Thus, the NW cannot assume that the UL timing delays are within a certain margin for all the measurement |
| ZTE2 | @QC, Please note that the current FG 27-1-2 needs to let both gNB and LMF know. I assume this FG will split into 2 as vivo commented. In such case, why do we further need FG 27-1-2a? @OPPO, I think we should focus on measurements nearby in time domain in which the same TEGID refers to the same timing error margin. Otherwise, if two measurements are far away from each other, even the same TEGID cannot ensure the same timing error margin. |

# Issue 3: FG 27-1-3

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 27. NR_pos_enh | 27-1-3 | Maximum number Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4,or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 64, [128]}] Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | OK |
| ZTE | The following red part should be replaced by ‘Value=1 is to indicate that the ‘Rx timing error+Tx timing error’ differences between all combinations of measurement and transmission are within a certain margin’ If UE supports this capability with the values > 1, and if if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements |
| Qualcomm | - We disagree with the comment from ZTE. Even if “value=1” is indicated, it doesn’t mean that ALL the measurmenets are within the same error. AuE may not be able to keep the same timing error for some measurements, and the UE should be allowed to not report a TxTEG. - Same concern as for the case of RxTEG and multiple PFL scenarios: A UE reports a single IE with all measurements across all PFLs. Is the assumption of the proponents of this feature that the RxTxTEG IDs will be {0,…,127} or {0,31}? |
| Huawei, HiSilicon | To QC: We think reporting should be separated discussed with the capability. The RxTX ID in the report is numbered per UE, instead of locally within a positioning frequency layer. |
| CATT | According to the following agreement, we prefer the candidate values of UE-RxTEGs for UE-assisted DL TDOA and/or Multi-RTT positioning should be up to 256. Agreement: - Support the following parameters and values related to the accuracy enhancement for mitigating UE Rx/Tx and/or gNB Rx/Tx timing errors: Parameter Description; Values in specifications (e.g., TS 37.355, TS 38.455); Values that can be signaled as part of UE Capability; Comments. The maximum number of UE RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT]; [32]; [1, 2,4,6,8,12,16,24,32] FFS: per UE/band /FL/FR; The parameter is used for supporting DL-TDOA and/or Multi-RTT. The maximum number of UE TxTEGs [for UL-TDOA and/or Multi-RTT]; [8]; [1, 2,4,6,8] FFS: per UE/band /FL/FR; The parameter is used for supporting UL-TDOA and/or Multi-RTT. The maximum number of UE-RxTx TEGs; [256]; [1, 2,4,6,8,12,16,24,32,64, 128, 256] FFS: per UE/band /FL/FR; The parameter is used for supporting Multi-RTT. - Note: Above proposal does not constrain in any way how features and feature sets are defined. The values in the table above may or may not be signalled to be different for different features or feature sets. |
| Qualcomm | - To Huawei/all: If all have the same understanding on what the “per band” means here and the differenation with the measurement report, then we can be OK assuming the following is added as a note: Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to 256. |
| OPPO | - The feature should be “per band per band combination”. For typical UE implementation, the hardware may be shared by some bands. For example, if UE is working on Band A, it can use 4 Rx antennas. If UE is working on Band A and Band B, then UE may retune 2 antennas to Band B and use 2 antennas for each band. In some sense, it is similar to the UE capability of MIMO layers which is FSPC - To ZTE’s comment on the issue of “value=1”, The UL is different from DL. For UL, the timing may be adjusted by gNB command and/or UE autonomous adjustment. Thus, the NW cannot assume that the UL timing delays are within a certain margin for all the measurement |
| ZTE2 | @QC, again, can you clarify what a kind of scenario is assumed in your assumption? i.e. when UE reports it supports 1 TEG, why sometimes UE may not be able to keep the same timing error for some measurements, and can keep in some other times |

# Issue 4: FG 27-1-4

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 27. NR_pos_enh | 27-1-4 | The maximum Number Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4, 6, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | The candidate values are {1,2} | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | Support. And for the candidate values of FG 27-1-4a, adding a bracket may be better. |
| ZTE | OK |
| Qualcomm | - Increase 27-1-4a values to {1,2,4,8} |
| Huawei, HiSilicon | Do not understand the motivation of adding the capability of “simultaneous” one. Even the measurement request does not include “simultaneous” indication. We do not support 27-1-4a. |
| CATT | We can put FG27-1-4a to FFS and decide whether to keep it in next meeting. In our point of view, it should be UE’s capability to use multiple Rx TEGs to measure the same DL PRS resource at the same time. |
| OPPO | These features should be “per band per band combination”. For typical UE implementation, the hardware may be shared by some bands. For example, if UE is working on Band A, it can use 4 Rx antennas. If UE is working on Band A and Band B, then UE may retune 2 antennas to Band B and use 2 antennas for each band. In some sense, it is similar to the UE capability of MIMO layers which is FSPC |
| ZTE2 | @CATT, yes, we think the FG 27-1-4a means UE’s capability to use multiple Rx TEGs to measure the same DL PRS resource at the same time @Huawei, the reason for FG 27-1-4a is: UE supports N1 RX TEGs for measuring the same DL PRS resource doesn’t mean UE can support N1 RX TEGs for measuring the same DL PRS resource simultaneously, the latter one depends on the maximum number of UE receive panels or the support maximum number of UE receive beams at a same time. The candidate values can be 1 or 2 or 4 in our view. With such UE capability, LMF can recommend gNB the appropriate PRS parameters including resource or resource set repetition factor, PRS periodicity etc.. For example, if UE reports N1 = 8 for measuring the same DL PRS resource and N2= 2 for measuring the same DL PRS resource simultaneously, then it is sufficient to configure 4 PRS resource repetitions to let UE report all 8 RX TEGs for the same resource as UE can process one resource instance with two RX TEGs simultaneously. However, if UE reports N1 = 8 and N2= 1, then it may need to configure 8 PRS resource repetitions to let UE report all 8 RX TEGs for the same resource |
| Huawei, HiSilicon2 | To ZTE: There is no requirement on the UE side, nor is there any explicit signaling to indicate the measurement behaviour on “simultaneous measurement using multiple Rx TEGs”. This feature will be useless. |

# Issue 5: FG 27-2-1

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | 1.) Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] 2.) The maximum number of first path PRS RSRP per TRP [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3], 13-4, 13-5, 13-8] | No |  | PRS RSRP of the first path is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: 2,4,8,16,24 Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | OK |
| ZTE | This is for DL-AOD. So the prerequisite should not be 13-3, 13-4 and 13-8. Regarding whether 13-5 can be the prerequisite, more discussion is needed. It depends on whether path RSRP is normalized based on regular RSRP. |
| Qualcomm | - Per band - Keep the UE-assisted in the name of the FG - Keep 13-2, and 13-5 as prerequisites. |
| CATT | OK. |
| OPPO | We suggest to keep the UE-assisted since there is no DL PRS RSRP reporting for UE-based method and we support “Per band” |
| vivo | We prefer the capability is per UE, otherwise the values may exceed the candidate values of DL PRS RSRP in FG 27-2-2 if FG 27-2-2 is per UE capability considering it is the enhancement of 13-5 |

# Issue 6: FG 27-2-2

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS Yes | n/a | [The candidate values are {12, 16, 24, 32, 64}] If K is not reported, default value is 8. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | - The following note should be removed. If the K value is not reported, the maximum value depends on FG 13-5. If K is not reported, default value is 8. - The prerequisite should not include 13-5. If K is reported, 13-5 is not needed anymore |
| Qualcomm | Same views with ZTE |
| CATT | For FG 27-2-2, according to the following agreement, we prefer two separate FGs for DL PRS RSRP reporting and first path DL PRS RSRP reporting, respectively. Therefore, the original FG 27-2-2 can be divided into two FGs, i.e., FG 27-2-2a and FG 27-2-2b. Then, FG 27-2-2a and FG 27-2-2b can be used for DL PRS RSRP reporting and first path DL PRS RSRP reporting, respectively. For up to K DL PRS RSRP reporting per TRP, the values of K should be [16, 24]. For up to M first path DL PRS RSRP reporting per TRP, the candidate values include [2,4,8,16,24]. Both of the two FGs should be per UE. The following agreement was achieved related to UE capability for DL-AoD enhancements in RAN1#106bis-e. Agreement: The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. - For UE-A DL-AOD, support reporting more than 8 up to 16 M first path PRS RSRP measurements per TRP, where M is a UE capability - FFS: Values of M. Candidate values include {2,4,8,16,24}. - FFS: Whether M is always equal to N - Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps. - Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation. Therefore, we propose to make the following modifications to FG 27-2-2 in order to reflect the agreement related to UE capability for DL-AoD enhancements in RAN1#106bis-e: 27. NR_pos_enh; 27-2-2a; DL PRS RSRP reporting for up to K measurements for UE-assisted DL-AoD positioning; Support reporting up to K DL PRS RSRP measurements per TRP. The values of K = {16,24}. •Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps.. 27. NR_pos_enh; 27-2-2b; First path DL PRS RSRP reporting for up to M measurements for UE-assisted DL-AoD positioning; Support reporting up to M first path DL PRS RSRP measurements per TRP. The candidate values of M include {2,4,8,16,24}. - FFS: Values of M. - FFS: Whether M is always equal to K.. |
| Nokia, NSB | Actually 13-5 is needed as pre-requisite, as otherwise a gNB not supporting 27-2-2 would not receive indication that UE supports 13-5 either. Hence, the pre-requisite is needed for backward compatibility. Based on the from the following agreement (106bis-e) the only confirmed candidate values are {16, 24}, as commented by ZTE above. Agreement: The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. - For UE-A DL-AOD, support reporting more than 8 up to 16 M first path PRS RSRP measurements per TRP, where M is a UE capability - FFS: Values of M. Candidate values include {2,4,8,16,24}. - FFS: Whether M is always equal to N - Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for same or different timestamps. - Note: the maximum number of DL PRS RSRP associated with the same Rx beam index is up to the UE implementation |
| OPPO | Re the first path DL PRS RSRP, we share the same understanding as CATT that another FG is needed for that. Re the candidate value for K in the proposal: they looks fine and we suggest to keep. Please note in RAN1 agreement, the candidate values are FFS. |
| vivo | We think the capability should be per UE since it is Rel-16 capability enhancement and corresponding Rel-16 capability is per UE The candidate value only include 16 and 24 in agreement, other values should be removed |

# Issue 7: FG 27-3-1

After review of contributions submitted to RAN1 #107-e in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in red

| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| vivo | OK |
| ZTE | - The yellow part in the third column can be removed. M-4 samples must on multiple PRS resource instances. - The prerequisite should be only 13-1 as this FG is common for all DL related positioning methods based on PRS. |
| Qualcomm | - Per band - Keep 13-1 as prerequire. |
| CATT | OK. |
| Nokia, NSB | We are generally OK |
| OPPO | Generally Ok with couple of suggestions: - Remove the wording in [] in the 3rd column. - Re FFS per UE vs per band: This feature should be per band. |

# Issue 1: New FGs

There have been close to a hundred proposals for new FGs. These will be discussed in the second week of RAN1 #107-e.

# Discussion/Approval Items during RAN1 #107-e — Second Checkpoint

Based on the comments/questions/suggestions received by the first checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 3 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 27-1-3

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-3 | Maximum number Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4,or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 12, 16, 24, 32[, 64, 128, 256]}] Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement [If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements] Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to 256. FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | OK. Just a typo in the blue note: “256” -> “255”. |
| H3C | We would like to clarify how much bits are used for indicating the maximum number of UE-RxTxTEG, If only 4 bits are used, we suggest keeping specific value 256 for the maximum number of UE-RxTxTEG. |
| vivo | One typo on the third bullet in comment: more “if” in the third paragraph If UE supports this capability with the values > 1, and if if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement |
| Huawei,HiSilicon | To H3C: The bit width is determined after the value range is settle, not vice versa. Support. To potential comments on per FS reporting (maybe from OPPO), we would like to address that issue here. We understand RF architecture for measurement may depend on CA configuration. For example, if UE needs to process data and PRS at the same time on different CCs (for PRS measurement without MGs with Capability 2 UE), the Rx number may be reduced, however for other cases, e.g. measurement inside MG and capability 1A outside MG, the Rx number may not be affected. Even if there is dependence on the CA configuration with Rx TEG, reporting the maximum value potentially used for this band does not harm the UE implementation, since a larger number of Rx TEGs does not mean that UE is more capable, and I think reporting 1 Rx TEG should be corresponding to the most capable UE. With this understanding, over-reporting the number of TEGs does not put higher requirement at UE, and reporting per band should be OK. Another issue for per FS reporting is that LMF does not know the current UE CA band combination, and would require large overhead to report all potential CA band combinations to the LMF, (which has not been introduced in LPP yet) and LMF will eventually need to inquire the gNB on the current CA band combination, which was discussed in Rel-16. |
| ZTE | Support. If we have the note marked in blue, we think the candidate values should not include 64, 128, 256 . |
| Ericsson | Support. We agree with Huawei comment that per FS reporting is problematic when the capability is reported to the LMF. Per band should be sufficient here. |
| CATT | There is one typo in the note in BLUE, i.e., the “256”at the end of the note should be “255”, and there should be the brackets for the 255, since we have not agree the the number can be 256 yet. We preferred changes as follows, Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to [255]256. |

# Issue 2: FG 27-1-4

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting incl. the yellow highlighting

| 27. NR_pos_enh | 27-1-4 | The maximum Number Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4, 6, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| H3C | We are fine this proposal. |
| Huawei, HiSilicon | Not OK with 27-1-4a as commented earlier. |
| ZTE | Support both. To Huawei, please read our reply in section 3.4 for 27-1-4a. Basically, the FG is useful to let LMF determine more efficient PRS configuration. |
| Ericsson | We understand the intention of introducing feature 27-1-4a and we are supportive of it. It is for simultaneous measurement of the same DL PRS using different UE Rx TEGs. But then in 27-1-4, we should clarify what is the assumption for measurement. Other than simultaneous reception, the UE may receive measure the same DL PRS resource in the following ways: - Option 1: different symbols in the same slot of the same DL PRS resource - Option 2: different repetitions of the same DL PRS resource - Option 3: different occasions of the same DL PRS resource It would be good to clarify that feature 27-1-4 supports one or more of the above options. |
| CATT | We are fine with both 27-1-4 and 27-1-4a. In our point of view, it should be UE’s capability to use multiple Rx TEGs to measure the same DL PRS resource at the same time. |

# Issue 3: FG 27-2-1

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | 1.) Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] 2.) The maximum number of first path PRS RSRP per TRP [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3], 13-4, 13-5, 13-8] | No |  | PRS RSRP of the first path is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: 2,4,8,16,24 Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| H3C | We are fine with this proposal. |
| vivo | We would like to note that the candidate value of 13-5 includes the {1,2,3,4,5,6,7,8}, if 27-2-1 only includes 2.4.8, in some cases, the path RSRP capability may be larger than PRS RSRP capability. In addition, we prefer the capability is per UE since 13-5 is per UE capability |
| Huawei, HiSilicon | Why do we need two components here? I guess if UE does not report the feature, this feature is not supported by the UE, which should be common understanding to handle backward compatibility. |
| ZTE | Support. To Huawei, if we remove the first component, candidate value 1 may need be included in the second component. |
| CATT | We think we only need component 2, and don’t need component 1 for this FG. |
| Nokia, NSB | This FG should apply to UE-based operation as well. |

# Issue 4: FG 27-2-2

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS Yes | n/a | [The candidate values are {[12,] 16, 24[, 32, 64]}] If K is not reported, default value is 8. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| H3C | We are fine with this proposal |
| vivo | We think the capability should be per UE since it is RSRP reporting(13-5) enhancement and corresponding Rel-16 capability(13-5) is per UE The candidate value only includes 16 and 24 in agreement, other values should be removed Agreement: The agreement from RAN1#106e on the number of DL PRS RSRP measurements per TRP is extended as follows: - For UE-A DL-AOD, support reporting more than 8 up to 16 N DL PRS RSRP measurements per TRP, where N is UE capability and candidate values include {16,24}. |

# Issue 5: FG 27-3-1

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| CATT | We are fine with this proposal. |
| vivo | We don’t think the candidate value of 4 should be included, since based on the note ‘If the UE does not provide the capability, the UE [is assumed to] support M=4 only’, M=4 can be implicitly indicated. |
| Huawei, HiSilicon | Remove 4 in the candidate value. |
| Nokia, NSB | We are generally fine with the proposal, but “on single DL PRS period/occasion” should be removed, especially if M=4 is included. |

# Issue 6: New FGs

Individually, almost 100 new FGs have been proposed in RAN1 #107-e [2]-[10]. After review of the contributions, the proposals from Huawei/HiSilicon [2] seem to cover all but one, namely, the one in the Annex of [5]. Additional new FGs not covered by the below at all can be proposed by individual companies in their comments in the respective table.

The moderator thus proposes to take the new FGs proposed in [2],[5] as starting point of the discussion. The goal is not so much to agree any of these, but to find a starting point for discussion, given the almost 100 proposals submitted to RAN1 #107-e. Companies should therefor focus on features that need a new FG (third column), rather than the exact details provided by [2],[5] which are marked yellow for that reason. Furthermore, companies may agree on the need for one or more rows for a feature, but not on the number of rows proposed by [2],[5]. In that case, the moderator will capture the feature and leave the number of rows FFS, using either Alt. 1 or Alt. 2 based on the majority view. Alt. 1 and Alt. 2 are perfectly identical and simply represent two ways of capturing the same. In a nutshell, the number of rows for a new FG is secondary at this point, as are the detailed components and candidate values, types, and other details. The purpose of this exercise is to find a basic table of new FGs that can serve as starting point for future discussions, e.g., at RAN1 #108-e, similar to how the rapporteur’s input served as starting point for initial discussions at RAN1 #106bis-e.

Alt. 1

| 27. NR_pos_enh | 27-x-y | FG Name | 1. Component 1 [2. Component 2] |  |  |  |  |  |  |  |  | FFS: Whether to split FG 27-x-y into multiple FGs | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 2

| 27. NR_pos_enh | 27-x-y | FG Name 1 | 1. Component 1 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | [27-x-z] | [FG Name 2] | 1. Component 2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

Based on the above, please indicate which of the following proposed new FGs you support in principle incl. whether you agree with the proposed split into multiple rows or whether you prefer a different split as explained in Alt. 1 and Alt. 2 above, or whether you don’t support any new rows for a given feature mentioned below. Please use the FG numbers below to refer to specific rows in your comments. (The FG numbering below is provisionally and will be aligned after further progress is made.)

From [2]:

| 27. NR_pos_enh | 27-xx1 | Support of MTW | Support of PRS measurement time window (PTW) configuration. |  | No |  | PRS measurement time window is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-xx2 | Support of positioning SRS with antenna switching | Support of positioning SRS resource set configured with antenna switching 1. The number of positioning SRS resources in a positioning SRS resource set configured with antenna switching Note: The switching period follows the existing MIMO SRS antenna switching (15us as per R1-1710048). | 13-4 | Yes |  | SRS positioning configured with antenna switching is not supported | Per FS | n/a | n/a | n/a | The candidate values are {1,2,4}. | Optional with capability signaling |
| 27. NR_pos_enh | 27-xx3 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning |  | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs | Optional with capability signaling |
| 27. NR_pos_enh | 27-uu1 | Support of lower Rx beam sweeping factor | Support of the lower Rx beam sweeping factor than 8 for FR2 1. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| 27. NR_pos_enh | 27-uu2 | Support of UL MAC CE based MG activation request | 1. Support of using UL MAC CE to request measurement gap. |  | Yes |  | Using UL MAC CE to indicate PRS measurement to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-uu3 | Support of DL MAC CE based MG activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-uu4 | Support of DL MAC CE based PRS processing window activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the PRS processing window |  | Yes |  | Using DL MAC CE to activate the preconfigured PRS processing window is not supported | Per UE | No | No | No |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-vv1 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-vv2 | Additional path reporting for DL-TDOA | 1. Support of TOA reporting for more than 2 additional paths. 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-vv3 | Additional path reporting for DL-AoD | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-vv4 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1 | Support of PRS measurement in RRC_INACTIVE | Support of PRS measurement in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference |  | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww2 | Support of positioning SRS transmission in RRC_INACTIVE state | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww2a | Support of positioning SRS transmission in RRC_INACTIVE state | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} | 27-ww2 | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported UE only reports the number on the PCell band. | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww3 | Support of positioning SRS configured outside UL BWP#0 configured by system information in RRC_INACTIVE state | 1. Support positioning SRS configured outside UL BWP#0 configured by system information in RRC_INACTIVE state 2. SRS switching period: following existing SRS-SwitchingTimeNR IE. | 27-ww2 | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww4 | Supported positioning SRS bandwidth in RRC_INACTIVE | 1. Supported bandwidth for positioning SRS transmission in RRC_INACTIVE | 27-ww2 | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww4a | Supported positioning SRS bandwidth in RRC_INACTIVE | 1. Supported bandwidth for positioning SRS transmission in RRC_INACTIVE | 27-ww2, 27-ww2a | No |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww5 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 27. NR_pos_enh | 27-ww6 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |

From [5]

| 27.NR_pos_enh | 27-1-5 | Phase center offset compensation for TEGs | Indicates if the UE supports PCO compensation as part of the TEG margin calculation. FFS: Separate capabilities for UE Rx TEG, UE Tx TEG, UE RxTx TEG. | [13-3,13-41,3-8] | No |  | PCO errors are not calibrated as part of TEGs | [Per UE] | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | - Discussion for the 27-3-2 and 27-3-3 FGs is missing from this round, but these were already “yellow color” in the previous round. - The following suggested FG from R1-2112250 were not included in the new FG list: 27-c4b; DL PRS processing capabilities in RRC inactive state; 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz. Support beam Information in the Assistance Data for UE-based DL-AoD; Support beam Information in the Assistance Data for UE-based DL-AoD. 27-d5; Maximum number of measurement instances which can be included in a single measurement report; Maximum number of measurement instances which can be included in a single measurement report. - Supportive to include such row (or split as shown below): - 27-xx3, 27-uu1, 27-uu2, 27-uu3, 27—uu4, 27-vv1, ,27-vv4, , 27-ww2, 27-ww5, 27-ww1a, 27-ww1b, 27-ww1c, 27-ww5, 27-ww6 - 27-vv2 should be split as: 27-vv2; Additional path reporting for DL-TDOA; 1. Support of TOA reporting for more than 2 additional paths. 2. Maximum Number of additional paths. Additional path RSRP reporting for DL-TDOA; 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported.. Path RSRP reporting for the first path for DL-TDOA; Support Path RSRP reporting for the first path for DL-TDOA. Similarly for 27.vv3 - Not supportive to be included - 27-1-5, 27-ww1, 27-ww4, 27-ww4a - Not agreed in main Positioning sessions, but if agreed, we are supportive: - 27-xx1 - Not agreed in main positioning sessions, and we are not supportive of the feature - 27-xx2, 27-vv3 - Still being discussed in main session if this feature will be kept inside Rel-17. We prefer to downscope the feature from this release. - 27-ww3 |
| vivo | Within above new feature list, we think the following FGs should be support first, since they are based on previous agreements and stable. - 27-xx3, 27-uu1, 27-uu2, 27-uu3, 27-vv1, 27-vv2, 27-vv4, 27-ww1a, 27-ww1b, 27-ww2, 27-ww5, 27-ww6 - For 27-vv2 and 27vv4, we share the similar view with Qualcomm, both of them should be split. In addition, the following feature groups are missed, which are based on previous agreement. - Support of pre-configured MG(s) in RRC Agreement(RAN1#107e) Preconfiguration of MG(s) in RRC is supported from RAN1 perspective. - Each MG in the preconfiguration is associated with an ID - The information in the UL MAC CE for MG activation request by the UE can be one ID associated with the preconfiguration of the MG - Send an LS to RAN2 and RAN3. - Support DL PRS processing in RRC_INACTIVE state; Support DL PRS processing outside and inside of the initial DL BWP in inactive state Agreement(RAN1#107e) For UE in RRC_INACTIVE state can support DL PRS processing outside and inside of the initial DL BWP: - For DL PRS processing outside of the initial DL BWP, the SCS, CP type of DL PRS can be the same or different as for the initial DL BWP - For DL PRS processing inside of the initial DL BWP, the SCS, CP type of DL PRS is the same as for the initial DL BWP. - Potential impact of retuning time and expected RSTD assistance information on DL PRS reception performance is up to RAN4 - UE capability(ies) will be defined for DL PRS processing in RRC_INACTIVE state - details are FFS - Send an LS to RAN4 on agreed by RAN1 UE behavior for reception of DL PRS in RRC INACTIVE state. - Support of semi-persistent SRS for postioning in inactive state Agreement(RAN2#116e): Proposal 6: SRS for positioning in RRC_INACTIVE state can be configured through the following ways: - RRCRelease with SuspendConfig (13/13) - SDT DL RRC message, i.e. Msg B / Msg 4 of RA-SDT (9/13) - WA: pre-configure positioning SRS in RRC_CONNECTED (9/13) FFS detailed signalling for these approaches. Proposal 8: Support SP SRSp for positioning in RRC_INACTIVE state. (12/13) Proposal 9: SP Positioning SRS Activation/Deactivation MAC CE is reused for triggering SRSp transmission in RRC_INACTIVE. (12/12) Proposal 10: AP SRSp is not supported for positioning in RRC_INACTIVE state. (11/13) |
| Huawei, HiSilicon | Agree with QC that - Discussion for the 27-3-2 and 27-3-3 FGs is missing from this round, but these were already “yellow color” in the previous round. Support based on the current progress: xx3, uu1, uu2, uu3 vv1, vv2, vv4 ww1, ww1a, ww1b, ww1c, ww2, ww2a, ww3, ww4, ww4a, ww5, ww6 To QC’s comments For split vv2 and vv4, we are open to discuss the format suggested by QC. Ok with c4b and d5 suggested by QC, but the typo in component 2 of c4b should be corrected. “a) Type 1 – sub-slot/symbol level buffering” To vivo’s comments Support of RRC preconfiguration of multiple MGs could be listed as one component in 27-uu2 and 27-uu3. We do not support separate capability bits for inside BWP#0 and outside BWP#0 for PRS measurement in RRC_INACTIVE state. Ok with adding semi-persistent SRS, but we need separate FGs after ww2 and ww2a. |
| ZTE | Not support: - 27-xx2 (not agreed in Rel-17) - 27-uu4 (since it is similar as 27-xx1, and not agreed yet) - 27-vv3 (not agreed in Rel-17 yet) - 27-ww1a, ww1b, ww1c (these three should be already covered by 27-ww1, the prerequisite can be Rel-16 TDOA, AOD and RTT) - 27-ww2a (it is the same as ww2) - 27-ww3 (ww1 has include this regardless of BWP#0 or not) - 27-ww4, 4a (4 and 4a are the same. Both of them are not needed because BWP doesn’t rely on UE capability as usual) - 27-ww5 and ww6 (it is enough to reuse Rel-16) - 27-1-5 (not agreed in Rel-17) Should be supported based on agreements - 27-xx1 (should clarify MTW outside MG); 27-xx3; 27-uu1; 27-uu2, 27-uu3; 27-vv1, - 27-ww1 (this is reported to gNB) - 27-ww2 - One new row for following as Qualcomm suggested (has been agreed) Support beam Information in the Assistance Data for UE-based DL-AoD; Support beam Information in the Assistance Data for UE-based DL-AoD. - 27-d5 as Qualcomm suggested (has been agreed) Suggest revision - 27-vv2 and vv4 should be combined, the prerequisite can be Rel-16 DL-TDOA and Multi-RTT Comments: - For 27-c4b added by QC (vivo also mentioned), it can be discussed together with 27-ww1, the details depend on if this FG is reported to gNB or LMF - For vivo’s comment on semi-persistent SRS in RRC inactive, we disagree. One FG is enough for both P and SP-SRS. |
| Ericsson | At least the following features are not agreed yet to be supported in Rel-17 positioning (although discussions are ongoing as of RAN1#107e): 27-xx1, 27-xx2, 270uu4 |
| CATT | As pointed out by companies above, a lot of FGs have not been agreed yet, therefore, we prefer to only capture the agreed FGs, and put the potential FGs into the brackets. |
| Nokia, NSB | Clarification needed: - Xx1: it should be PTW in FG name, instead of MTW, as those are different features. Do not support: - Xx2: not agreed to be supported in the WI - Uu4: not agreed to be supported in the WI - Ww3: not agreed to be supported in the WI Not needed as separate FG: - Vv4: can be merged with vv2 - Ww1a, ww1b, ww1c: can be components of ww1 To QC: Please note the following agreement related to 27-1-5: Agreement: (RAN1#104-e) The following definitions are used for the purpose of discussion of internal timing errors (these terms are not agreed to be included in the specifications): - Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error. - Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error. It was clearly agreed that PCO impact may be part of the TEG concept. This capability is needed to inform the network if a UE is supporting this feature or not. |
| Huawei, HiSilicon | For xx1, the intention is MTW, sorry for the typo. For a couple of FGs that are considered duplicated, the understanding from our side is one FG is reported to gNB, while another is reported to LMF. ww1 gNB while ww1a/1b/1c LMF ww2 gNB while ww2a LMF We understand some proposals are subject to further consideration, we agree to only agree to those with agreements. |

In addition, companies proposed these new rows/FGs.

| [Please add any rows not captured above here] |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-c4b | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh |  | Support beam Information in the Assistance Data for UE-based DL-AoD | Support beam Information in the Assistance Data for UE-based DL-AoD |  |  |  |  |  |  |  |  |  |  |
| 27. NR_pos_enh | 27-d5 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report |  |  |  |  |  |  |  |  |  |  |

# Discussion/Approval Items during RAN1 #107-e — Third Checkpoint

Based on the comments/questions/suggestions received by the second checkpoint, the following are the revised proposals and/or proposed agreements by the moderator. Companies submitted the following views on the moderator’s proposals.

[Please submit all comments/questions/suggestions here, late comments/questions/suggestions submitted in Section 4 will not be considered]

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 1: FG 27-1-3

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-3 | Maximum number Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4,or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 12, 16, 24, 32[, 64, 128, 256]}] Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement [If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements] Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to [255] FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 2: FG 27-1-4

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting incl. the yellow highlighting

| 27. NR_pos_enh | 27-1-4 | The maximum Number Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4, 6, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | 27-1-4 FG name could be updated as Support of multiple UE Rx TEGs for measuring the same DL PRS resource We disagree with introducing 27-1-4a. No request from LMF or measurement period requirement is affected by this FG. The time stamp associated with the measurement can be the same or different, and network cannot instruct UE to only provide the same time stamp if UE supports 27-1-4a. Our comments earlier were not addressed. ZTE’s comments on LMF determine the PRS configuration is not valid. We do not see how this impacts the LMF initiated on-demand. If this is about repetition factor, we do not think it should consider potential Rx TEG switching if simultaneous is not possible. |
| ZTE | @Huawei for 27-1-4a, We don’t need to consider Rx TEG switching. If UE reports the 27-1-4a is 2, that means UE maintains 2 RxTEGs for simultaneous reception of a PRS resource occasion. This is help LMF determination for more efficient PRS configuration. If UE reports value 1 for this FG 1-4a, at least two resource instance/repetition factor should be configured to let UE use both RxTEGs. If UE reports value 2, one resource instance/repetition factor is enough. Then, what is not clear? |

# Issue 3: FG 27-2-1

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | 1.) Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] 2.) The maximum number of first path PRS RSRP per TRP [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3], 13-4, 13-5, 13-8] | No |  | PRS RSRP of the first path is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: [2,4,8,16,24] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | As commented before, the FG should apply to UE-based operation as well. For example change FG name to: [UE-assisted] and UE-based DL PRS RSRP of the first [or additional] path [for DL-AoD] |
| Qualcomm | We don’t agree to have this FG applicable to UE-based. It may appear as if the UE is expected to report measurements. If it becomes a 3rd component that is {0,1} it may be OK for us. |

# Issue 4: FG 27-2-2

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS Yes | n/a | [The candidate values are {[12,] 16, 24[, 32, 64]}] If K is not reported, default value is 8. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |

# Issue 5: FG 27-3-1

Proposed Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

[Please only comment in the table if you are NOT okay with the proposed agreement]

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | Now that there are no candidate values is it correct understanding that UE reporting this FG would support all values supported in spec? That is fine for us, just want a confirmation. |
| Qualcomm | Our understanding is that the only value in the spec that has been agreed ins {1}. If more values are agreed then, we would have to put back the candidate values. Since that topic is still open, wouldn’t it be more appropriate to keep it in []? |

# Issue 6: New FGs

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-xx3 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning |  | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-uu1 | Support of lower Rx beam sweeping factor | Support of the lower Rx beam sweeping factor than 8 for FR2 1. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-uu2 | Support of UL MAC CE based MG activation request | 1. Support of using UL MAC CE to request measurement gap. |  | Yes |  | Using UL MAC CE to indicate PRS measurement to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-uu3 | Support of DL MAC CE based MG activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-vv1 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-vv2 | Additional path reporting for DL-TDOA | 1. Support of TOA reporting for more than 2 additional paths. 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | It should be: Additional path reporting for UE-Assisted DL-TDOA |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-vv4 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | One FG for vv2 and vv4 is enough. The prerequisite can include Rel-16 DL-TDOA and Multi-RTT, i.e. FG 13-4 and 13-6. If supports additional path measurement for TDOA, there is no reason not to support for RTT. |
| Qualcomm | We prefer to split the features vv2 and vv4 as it is shown now. |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-ww2 | Support of positioning SRS transmission in RRC_INACTIVE state | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | There is a very related debate happening in the 8.5.6 agenda, whether we would have 2 capabilities for SRS transmission in RRC Inactive. It seems there is a compromise that is getting some traction to write it us. This may come up in the main session today/tomorrow, and we should wait for this discussion to see what we ll do with this row. Proposal 3.3-4 - The following options are supported for SRS for positioning transmission by RRC_INACTIVE Ues: - Option 1: - Subject to UE capability, a UE may be configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP. - Option 2: - Subject to UE capability (prerequise is the first capability), a UE may be configured with an SRS for Positioning where the following parameters are additionally configured for the transmission of the SRS for Positioning during the RRC_INACTIVE state: frequency location and bandwidth, SCS, CP length. - The UE shall not transmit the SRS for Positioning when it is expected to perform UL transmissions in the initial UL BWP in RRC_INACTIVE state. - RAN1 assumes that - SRS for positioning for Ues in RRC_INACTIVE state is configured using the SRS-PosResourceSet IE |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-ww5 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Don’t support. Rel-16 UE features can be completely reused. |
| QUalcomm | We think it is needed |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-ww1a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | We prefer to agree with 27-ww1 along with 1a/1b/1c. They should not be agreed if 27-ww1 is not agreed. |
| ZTE | Support 27-ww1 is enough. The prerequisite of ww1 can be Rel-16 TDOA, AoD and RTT. If UE can support a positioning method, e.g. TDOA, when it reports support 27-ww1, it should also support TDOA in RRC inactive. Hence, we don’t need many rows for each positioning methods. |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-ww1b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | We prefer to agree with 27-ww1 along with 1a/1b/1c. They should not be agreed if 27-ww1 is not agreed. |
| ZTE | Support 27-ww1 is enough. The prerequisite of ww1 can be Rel-16 TDOA, AoD and RTT. If UE can support a positioning method, e.g. TDOA, when it reports support 27-ww1, it should also support TDOA in RRC inactive. Hence, we don’t need many rows for each positioning methods. |
| Qualcomm | We agree with HW/HiSi, we would like to have ww1/ww1a/ww1b/ww1c. |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-ww1c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | We prefer to agree with 27-ww1 along with 1a/1b/1c. They should not be agreed if 27-ww1 is not agreed. |
| ZTE | Support 27-ww1 is enough. The prerequisite of ww1 can be Rel-16 TDOA, AoD and RTT. If UE can support a positioning method, e.g. TDOA, when it reports support 27-ww1, it should also support TDOA in RRC inactive. Hence, we don’t need many rows for each positioning methods. |
| Qualcomm | We agree with HW/HiSi, we would like to have ww1/ww1a/ww1b/ww1c. |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-ww6 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| ZTE | Don’t support. Rel-16 UE features can be completely reused. If UE support a spatial relation reference, it is natural also to support in RRC inactive when it supports SRS transmission in RRC inactive |
| QUalcomm | We think it is needed |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-c4b | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei, HiSilicon | Component 2 should be changed to 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms |
| ZTE | This should be discussed together with FG 27-ww1. We don’t support this for now. If this UE capability is reported to gNB, FG 27-ww1 is enough since gNB doesn’t need PRS processing capability. |

Proposal: Add the following new row/FG to the 27-x-y family for NR_pos_enh

| 27. NR_pos_enh | 27-d5 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Nokia, NSB | It is not clear why we would need such a FG. Do not support. |
| Qualcomm | We replied in the corresponding email thread. In short, this is a new feature added; the LMF is not aware if the UE can do this or not, some indication is needed whether the UE can (or will) start reporting accordingly. |

# Conclusion

After further discussion on the RAN1 email reflector and during several GTW online sessions the following was agreed as part of this email discussion:

Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-1 | Maximum number Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE [for UE assisted DL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | 13-1 [,one or more of {13-3, 13-4}], 27-1-1b | No |  | Mitigation of UE Rx timing delays is not supported UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | FFS: Per UE or [per band or FS] | n/a | n/a | n/a | [The candidate values are {[1,]2[, 3],4,6,8[,12,16,24,32]}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement [If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an the same Rx-TEG-ID associated with both the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.] Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-2 | Maximum number Support of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, ] 13-8], 27-1-2b | FFS Yes |  | Mitigation of UE Tx timing delays is not supported UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 16, 32}] [Need for location server to know if the feature is supported] Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT [and/or UL TDOA] positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | [per band per FS] | n/a | n/a | n/a | The candidate values are {[1, ] 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource [If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.] [Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-xx3 | Support of PRS TEG association information for UE-based DL-TDOA | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning |  | No |  | Positioning calculation assistance data containing association between PRS and TRP Tx TEG is not supported by UE | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Agreement: Support the LMF to provide the association information of DL PRS resources with Tx TEGs to a UE for UE-based positioning if the TRP has multiple TEGs | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-uu1 | Support of lower Rx beam sweeping factor | 1. Support of the lower Rx beam sweeping factor than 8 for FR2 2. Number of Rx beam sweeping factors: {1,2,3,4,5,6,7} |  | No |  | UE only supports 8 as the Rx beam sweeping factor defined by RAN4. | Per band (FR2 only) | n/a | n/a | n/a | Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-uu2 | Support of UL MAC CE based MG activation request | 1. Support of using UL MAC CE to request measurement gap. |  | Yes |  | Using UL MAC CE to indicate PRS measurement to the gNB is not supported. | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-uu3 | Support of DL MAC CE based MG activation | 1. Support of preconfiguration of MGs in RRC 2. Support of using DL MAC CE to activate the MG |  | Yes |  | Using DL MAC CE to activate the preconfigured MG is not supported | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-vv1 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type: {softValue, hardValue, both} 2. LOS/NLOS indicator granularity {resourceSpecific, trpSpecific, both} |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-vv2 | Additional path reporting for UE-assisted DL-TDOA | [1. Support of TOA reporting for more than 2 additional paths.] 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-vv4 | Additional path reporting for Multi-RTT | 1. Support of TOA reporting for more than 2 additional paths 2. Support of path RSRP reporting for additional paths if path RSRP reporting is supported. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-ww2 | Support of positioning SRS transmission in RRC_INACTIVE state [for initial BWP] | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} 4. FFS: Applicability for initial BWP OLPC for SRS for positioning based on SSB from the last serving cell (the cell that releases UE from connection) is part of this FG. Note: no dedicated capability signaling is intended for this component |  | Yes |  |  | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG with yellow background to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-ww5 | OLPC for positioning SRS in RRC_INACTIVE state | Same as LPP OLPC-SRS-Pos-r16 RRC OLPC-SRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-ww1 | Support of [PRS measurement in RRC_INACTIVE] | Support of PRS measurement in RRC_INACTIVE Note: UE supporting this feature may support at least one from DL RSTD, DL PRS-RSRP, or UE Rx – Tx time difference |  | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-ww1a | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA | Support of PRS measurement in RRC_INACTIVE state for DL-TDOA Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-TDOA. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1b | Support of PRS measurement in RRC_INACTIVE state for DL-AoD | Support of PRS measurement in RRC_INACTIVE state for DL-AoD Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for DL-AoD. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |
| 27. NR_pos_enh | 27-ww1c | Support of PRS measurement in RRC_INACTIVE state for Multi-RTT | 1. Support of PRS measurement in RRC_INACTIVE state for Multi-RTT 2. Support of positioning SRS transmission in RRC_INACTIVE state. Note: Other PRS capabilities follows the same as the RRC_CONNECTED state for Multi-RTT. |  | No |  |  | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signaling. |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-ww6 | Spatial relation for positioning SRS in RRC_INACTIVE state | Same as LPP SpatialRelationsSRS-Pos-r16 RRC SpatialRelationsSRS-Pos-r16 |  | Yes |  |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-c4b | DL PRS processing capabilities in RRC inactive state | 1. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) Type 1 – sub-slot/symbol level buffering b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms 3. Max number of DL PRS resources that UE can process in a slot under it a) FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b) FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Add the following new row/FG to the 27-x-y  family for NR_pos_enh

| 27. NR_pos_enh | 27-d5 | Maximum number of measurement instances which can be included in a single measurement report | Maximum number of measurement instances which can be included in a single measurement report |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- FG number/ID will be fixed by moderator before LS is sent to RAN2

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-1-3 | Maximum number Support of UE-RxTxTEGs for Multi-RTT | The maximum number of UE-RxTxTEG, which is supported and reported by UE for Multi-RTT positioning FFS: the values (>1) FFS: whether to have a value=1 to indicate UE RxTx timing errors is well calibrated [If a UE support this capability with the values > 1, the UE supports reporting of UE RxTx TEG ID with UE Rx-Tx time difference measurements for Multi-RTT positioning] | [13-4 or 13-8] | No |  | Mitigation of UE RxTx timing delays is not supported | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 12, 16, 24, 32[, 64, 128, 256]}] Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if the UE does not include RxTxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE RxTx timing delays for this measurement [If value=1 is indicated by the UE, the UE RxTx timing errors differences between two measurements are within a margin only if the UE reports an RxTx-TEG-ID associated with the measurements, otherwise, no assumption can be made about the timing error differences between these measurements] Note: The “per band” reporting on this capability does not imply, that the RxTxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTxTEG ID can span from 0, up to [255] FFS: Separate row for “Support of UE-RxTxTEG reporting for Multi-RTT” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements: Adopt the following changes highlighted in chromatic formatting incl. the yellow highlighting

| 27. NR_pos_enh | 27-1-4 | The maximum Number Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4, 6, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | 1.) Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] 2.) The maximum number of first path PRS RSRP per TRP [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3], 13-4, 13-5, 13-8] | No |  | PRS RSRP of the first path is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: [2,4,8,16,24] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-2-2 | DL PRS RSRP reporting for more than 8 measurements for UE-assisted DL-AoD positioning | Support reporting K> 8 DL PRS RSRP measurements per TRP. FFS: the values of K Note: Multiple RSRPs corresponding to same or different Rx Beam index should be able to be reported for a given PRS resource for different timestamps. FFS: Additional capability may be added to limit the maximum number of DL PRS RSRP associated with the same Rx beam index | [13-5, 13-2] | No |  | UE report of more than 8 DL PRS-RSRP is not supported. | FFS: Per UE or per band | n/a | FFS Yes | n/a | [The candidate values are {[12,] 16, 24[, 32, 64]}] If K is not reported, default value is 8. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreements: Adopt the following changes highlighted in chromatic formatting

| 27. NR_pos_enh | 27-3-1 | M-sample measurements [of DL PRS measurement on single DL PRS period/occasion] | [The capability to support reporting a measurement based on measuring M samples (instances) of a DL PRS resource set] M=1[,2-3]. FFS: other values. If the UE does not provide the capability, the UE [is assumed to] support M=4 only. | [13-1, 13-4, 13-8] | No |  |  | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4}] If the UE does not provide the capability, the UE [is assumed to] support M=4 only. Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# References

- R1-2110587, Updated RAN1 UE features list for Rel-17 NR after RAN1 #106bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2110855, Rel-17 UE features for positioning enhancements, Huawei/HiSilicon

- R1-2110963, UE features for NR positioning enhancements, ZTE

- R1-2111053, Discussion on UE features for NR positioning enhancements, vivo

- R1-2111156, On UE features for NR positioning enhancements, Nokia/Nokia Shanghai Bell

- R1-2111239, Further discussion on Rel-17 UE features for NR Positioning enhancements, CATT

- R1-2111295, UE features for NR positioning enhancements, OPPO

- R1-2111529, UE Features for NR Positioning Enhancements, Intel Corporation

- R1-2111773, UE features for NR positioning enhancements, Samsung

- R1-2112250, Discussion on Positioning Enhancements Features, Qualcomm Incorporated