3GPP TSG RAN WG1 #107-e			R1-2111833

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.2.2

Source:	InterDigital, Inc.

Title:	Discussions on PDCCH monitoring enhancements

Document for:	Discussion and Decision

# Introduction

In RAN1#106bis-e [1], following agreements on PDCCH monitoring enhancements were made:

Agreement: - Multi-slot PDCCH monitoring is based on slots within a slot group - Each slot group consists of X consecutive slots - Slot groups are consecutive and non-overlapping - The start of the first slot group in a subframe is aligned with the subframe boundary - The start of each slot group is aligned with a slot boundary - Reporting the BD/CCE budget for X=4/8 slots (for 480/960 kHz resp.) is mandatory (if UE supports the corresponding SCS), and is optional for X=[2]/4 slots (for 480/960 kHz resp.) - There is a common BD budget for all search spaces - FFS: Search space configuration - For Group (1) SS - A SS is configured to be within YGroup1 consecutive slots within a slot group of X slots - The location of the YGroup1 consecutive slots within a slot group of X slots is based on a time offset within the slot group based on slot index n0 determined for Group (2) monitoring such that the YGroup1 slots overlap the YGroup2 slots - The location of the YGroup1 consecutive slots within a slot group of X slots is maintained across different slot groups (unless n0 changes) - BD attempts for all Group (1) SSs are restricted to fall within the same YGroup1 consecutive slots - For Group (2) SS - A SS is configured to be within YGroup2 consecutive slots within a slot group of X slots - The location of the YGroup2 consecutive slots within a slot group of X slots is maintained across different slot groups (unless n0 changes) - The reported capability indicates the BD/CCE budget within Y=max(YGroup1, 2) slots per slot group - Support the following values of YGroup1 and YGroup2 - For X=8: (YGroup1,YGroup2) = (4,2), (2,2), (1,[1 or 2]) - For X=4: (YGroup1,YGroup2) = (2,2), (1,[1 or 2]) - For X=2: (YGroup1,YGroup2) = (1,[1 or 2]) - Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS

In this contribution, we discuss potential issues and associated standards impacts on PDCCH monitoring for new SCSs in 52.6 – 71GHz.

# Discussions

In RAN1#106bis-e [1], support of consecutive and non-overlapping slot groups was agreed with aligned slot/subframe boundaries. In addition, , two search space groups were defined as follows:

- Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS

- Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS

Based on the two search space groups, two possible options were discussed for a search space configuration:

- Option A

  - A SS can be configured to be within Y consecutive slots within a slot group of X slots; the Y consecutive slots can be located anywhere within the slot group of X slots

    - The location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups

- Option B

  - For Group (1) SS, a SS can be configured to be within Y consecutive slots within a slot group of X slots

    - The location of the Y consecutive slots can be anywhere within a slot group of X slots

  - For Group (2) SS, a SS can be configured as per Rel-15, and monitoring locations with respect to slot groups are unrestricted

In contrast to Option B, in Option A, the location of the Y consecutive slots within the slot group of X slots is maintained across different slot groups regardless of Group (1) SS and Group (2) SS. Main intention of Option A is to avoid back-to-back PDCCH monitoring issue as shown in Figure. 1.

Figure 1: Potential back-to-back monitoring of search spaces

However, some companies didn’t see back-to-back monitoring as a serious issue considering relatively large monitoring periodicity of Group (2) SS. In addition, Rel-15 UE also supports any symbol locations of a slot group as shown in the below [4].

- For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot.

- For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot.

As a compromise, following search space configuration is provided by the feature lead and agreed as an FFS point.

- Search space configuration - For Group (1) SS - A SS is configured to be within YGroup1 consecutive slots within a slot group of X slots - The location of the YGroup1 consecutive slots within a slot group of X slots is based on a time offset within the slot group based on slot index n0 determined for Group (2) monitoring such that the YGroup1 slots overlap the YGroup2 slots - The location of the YGroup1 consecutive slots within a slot group of X slots is maintained across different slot groups (unless n0 changes) - BD attempts for all Group (1) SSs are restricted to fall within the same YGroup1 consecutive slots - For Group (2) SS - A SS is configured to be within YGroup2 consecutive slots within a slot group of X slots - The location of the YGroup2 consecutive slots within a slot group of X slots is maintained across different slot groups (unless n0 changes) - The reported capability indicates the BD/CCE budget within Y=max(YGroup1, 2) slots per slot group - Support the following values of YGroup1 and YGroup2 - For X=8: (YGroup1,YGroup2) = (4,2), (2,2), (1,[1 or 2]) - For X=4: (YGroup1,YGroup2) = (2,2), (1,[1 or 2]) - For X=2: (YGroup1,YGroup2) = (1,[1 or 2]) - Group (1) SS: Type 1 CSS with dedicated RRC configuration and type 3 CSS, UE specific SS - Group (2) SS: Type 1 CSS without dedicated RRC configuration and type 0, 0A, and 2 CSS

Given the limited time, approval of the FFS point is preferred, however, larger values should be supported for YGroup2 to ensure flexibility of Type 1 CSS without dedicated RRC configuration and type 0, 0A and 2 CSS.

Proposal 1: Approve the FFS point with the following values of YGroup1 and YGroup2:

- For X=8: (YGroup1,YGroup2) = (4,2), (2,2), (1, 2)

- For X=4: (YGroup1,YGroup2) = (2,2), (1, 2)

- For X=2: (YGroup1,YGroup2) = (1, 2).

# Summary

In this contribution, we discussed the issues for PDCCH monitoring enhancements of NR in 52.6 – 71 GHz. From the discussions, we made a following proposal:

Proposal 1: Approve the FFS point with the following values of YGroup1 and YGroup2:

- For X=8: (YGroup1,YGroup2) = (4,2), (2,2), (1, 2)

- For X=4: (YGroup1,YGroup2) = (2,2), (1, 2)

- For X=2: (YGroup1,YGroup2) = (1, 2).

# References

- “Final Report of 3GPP TSG RAN WG1 #106bis-e”, 3GPP RAN1 Meeting #106bis-e, e-Meeting, October 11th – 19th, 2021,

- 3GPP TR 38.807 (v16.0.0): “Study on requirements for NR beyond 52.6 GHz (Release 16)”,

- 3GPP TS 38.213 (v16.4.0), “Physical layer procedures for control (Release 16)”,

- 3GPP TS 38.822 (v16.0.0), “User Equipment (UE) feature list (Release 16)”.