3GPP TSG RAN WG1 #107-e			R1-2111841

e-Meeting, November 11th – 19th, 2021

Agenda item:	8.8.1.3 Joint channel estimation for PUSCH

Source: 	Sierra Wireless

Title: 	Design Considerations for Joint channel estimation for PUSCH

Document for: 	Discussion and decision

# Introduction

This tdoc discusses design considerations for the following coverage enhancement objectives in the WID [1]:

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified, if necessary, by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

# Time Domain Window (TDW)

The following agreement was made in RAN1 #106-e meeting:

Down-select one of the following options in this meeting:

Option 1:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

Option 1’:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

    - If L is not configured, the configured TDW length is equal to all repetitions

    - If L is not configured, default behavior should be defined, e.g., the configured TDW length is equal to all repetitions

Option 3’:

- Whether the window length L of the configured TDW can be longer than maximum duration is subject to UE capability.

  - If UE is capable of L being longer than maximum duration,

    - The maximum value of the window length L of the configured TDW is the duration of all repetitions.

      - FFS: whether L cannot be other values other than the duration of all repetitions, if it is longer than the maximum duration.

    - If L is longer than the maximum duration, UE does not expect dynamic events.

      - FFS: details of dynamic events

Further progress was attempted by email discussions with this being the last proposal by the feature lead:

Proposal 15-v2: Down select one of the following options in RAN1 #107-e.

Option 1’-a:

Ÿ   The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

‐    If L is not configured, the default value and default behavior are is to be discussed, with default value not exceeding the maximum duration.

Option 1’-b:

Ÿ   The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

‐    If L is not configured, the default value and default behavior are to be discussed, with where default value may or may not exceeding the maximum duration.

The main bullet is the same for both option so this should be agreeable and only the default behaviour is controversial.

To configure the TDW, only the default behaviour and value when L is not configured is controversial.

Assuming the default behaviour for the options is defined as this:

option 1’-a  Default when L is not signaled = > Configured TDW  = max duration

option 1’-b  Default when L is not signaled = > Configured TDW  = # of repeats

With the above assumptions, there is no difference in the actual TDW’s that are generated (see below) when there are no dynamic events. The only difference is that with 1’-b, semi- static events are generated when TDW exceeds max duration.

Option 1’-a:

Configured TDW

Actual TDW

Events

Configured TDW

Actual TDW

Configured TDW

Configured TDW

Actual TDW

Actual TDW

Option 1’-b:

Configured TDW

Actual TDW

Events 		          * TDW exceeded max dur	     * TDW exceeded max dur	* TDW exceeded max dur

Actual TDW

Actual TDW

Actual TDW

The actual TDW’s generated between option 1’-a and 1’-b are the same, but 1’-b will generate semi-static events to create the Actual TDWs.

As mentioned by many companies, the sub bullet of option 1’-b seems to contradict the main bullet. This contradiction is caused due to the difference in understanding between the signaled value of L, referred to in the sub-bullet, and the configured TDW, which is what UE shall use in the main bullet so if we let

TDWconfigured 	Is the TDW used by UE (i.e. the main bullet)

Lconfigured  		Is the NW signaled value or default value of L  (i.e. the sub bullet)

For there to not be a contradiction, the specification must have a rule mapping L to Configured TDW where:

TDWconfigured = min (max dur, Lconfigured)

For the sub-bullet in option 1’-b to not contradict the main bullet, the follow specification rule would be needed:

TDWconfigured = min (max dur, Lconfigured)

where

TDWconfigured 	TDW used by UE

Lconfigured  	L signaled by NW or default value of L

With this rule, then option 1’-a and option 1’-b would have the exact same behaviour.

Option 1’-a:

TDWConfigured = LConfigured

Actual TDW

Events

Actual TDW

Actual TDW

Actual TDW

Option 1’-b:

LConfigured

Actual TDW

Events

Actual TDW

Actual TDW

Actual TDW

TDWconfigured = max duration

If option 1’-b has the rule TDWconfigured = min (max dur, Lconfigured), it will have the same actual TDWs with no events as option 1’-a

Considering dynamic events :

For example, let Max Dur =4 , repeats =16, with 2 Dynamic Events after R2 and R14

For UEs supporting restarting TDW after dynamic events:

Option 1’-a

LConfigured = 4 = max dur

Actual TDW

Event	*								     *

Actual TDW

Actual TDW

R1

R2

R4

R6

R8

R10

R12

R14

R3

R5

R7

R9

R11

R13

R15

R16

Actual TDW

Actual TDW

Actual TDW

Option 1’-b:

LConfigured = number of repeats = 16

Actual TDW

Event	*								     *

Actual TDW

Actual TDW

R1

R2

R4

R6

R8

R10

R12

R14

R3

R5

R7

R9

R11

R13

R15

R16

Actual TDW

Actual TDW

For UEs NOT supporting restarting TDW after dynamic events:

Option 1’-a:

LConfigured = 4 = max dur

Actual TDW

Event	*								     *

Actual TDW

Actual TDW

R1

R2

R4

R6

R8

R10

R12

R14

R3

R5

R7

R9

R11

R13

R15

R16

Actual TDW

Option 1’-b:

LConfigured = number of repeats= 16

Actual TDW

Event	*								     *

R1

R2

R4

R6

R8

R10

R12

R14

R3

R5

R7

R9

R11

R13

R15

R16

Summary

UE’s supporting Dynamic events:

Option 1’-a:  	2 actual TDW len=4, 4 actual TDW len =2

Option 1’-b:	3 actual TDW len=4, 2 actual TDW len =2

Similar performance

UE’s NOT supporting Dynamic events:

Option 1’-a:  	2 actual TDW len=4, 2 actual TDW len =2

Option 1’-b:	0 actual TDW len=4, 1 actual TDW len =2

Option 1’-b has severely degraded performance

For UEs that do NOT support restarting TDW after dynamic events, Option 1’-b when L>> max duration will have severely degraded performance vs option 1’-a

With option 1’-b it is possible for the UE and gNB to be out of sync if UE misses the dynamic event. This can be avoided if gNB conservatively assumes UE missed the events, however this leads to the same performance as option 1’a.

If gNB conservatively assumes UE missed the dynamic event, the performance of 1’-b and 1’-a will be the same.

Given the above analysis, option 1’-a is preferred:

Select Option 1’-a:

The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

If L is not configured, the default value is to be discussed, with default value not exceeding the maximum duration.

# UE capability of restarting DMRS bundling

The following agreement has been made last meeting:

Down-select one of the following options:

   Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

   Option 2: UE capability of restarting DMRS bundling is applied to both semi-static events and dynamic events.

WRT events, only the following agreement has been made:

Support at least the following events that violate power consistency and phase continuity.

‐   Dropping/cancellation based on Rel-15/16 collision rules.

‐   DL slot or DL reception/monitoring based on semi-static DL/UL configuration for unpaired spectrum.

‐   Gap between two PUSCH/PUCCH transmissions exceeds 13 symbols.

‐   FFS: Rel-17 collision rules.

‐   FFS: Other UL transmission in between PUSCH/PUCCH transmissions.

‐   FFS: Transmission parameters need to be changed due to network-indicated operations, including: Tx power, UL beam/TPMI, and RB allocation.

‐   FFS: TPC command.

‐   FFS: TA adjustment.

‐   FFS: The actual TDW reaches the maximum duration.

‐   FFS: Frequency hopping.

‐   FFS: Precoder cycling.

‐   FFS: other events.

‐   FFS: whether events are semi-static events or dynamic events.

‐   FFS: the time duration of an event.

There have been no agreements made WRT which events are considered semi-static vs dynamic events

With the assumption that semi-static events are events which are known to occur before the start of the transmission and dynamic events are only known after the start of the transmission, then the above list seems to already include both semi-static and dynamic events. The spirt of the original WA relating to TDW is under the premise that restarting after dynamic events is a UE capability so that UEs w/o this capability will always be able to pre-determine where its actual TDWs start/end before the transmission starts which is in line with option 1.

Assuming semi-static event are events which are known before the start of the transmission and dynamic events are only known after the start of the transmission then

   Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

# Conclusions

- To configure the TDW, only the default behaviour and value when L is not configured is controversial.

The actual TDW’s generated between option 1’-a and 1’-b are the same, but 1’-b will generate semi-static events to create the Actual TDWs.

For the sub-bullet in option 1’-b to not contradict the main bullet, the follow specification rule would be needed:

TDWconfigured = min (max dur, Lconfigured)

where

TDWconfigured 	TDW used by UE

Lconfigured  	L signaled by NW or default value of L

If option 1’-b has the rule TDWconfigured = min (max dur, Lconfigured), it will have the same actual TDWs with no events as option 1’-a

For UEs that do NOT support restarting TDW after dynamic events, Option 1’-b when L>> max duration will have severely degraded performance vs option 1’-a

If gNB conservatively assumes UE missed the dynamic event, the performance of 1’-b and 1’-a will be the same.

- Select Option 1’-a:

The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

If L is not configured, the default value is to be discussed, with default value not exceeding the maximum duration.

There have been no agreements made WRT which events are considered semi-static vs dynamic events

Assuming semi-static event are events which are known before the start of the transmission and dynamic events are only known after the start of the transmission then

   Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

# References

RP-202928 China Telecom, “New WID on NR coverage enhancements”

# Previous Agreements

The following Agreements have been made:

For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation at least for the following case:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant

  - FFS details (including possible other cases)

For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for TB processing one TB processed over multiple slots

    - It’s subject to UE capability

For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

FFS: additional specification enhancements on top of that defined to support repetition Type A

Only for single layer transmissions

Subject to UE capability

FFS: Over back-to-back PUSCH transmissions with different TBs

Joint channel estimation over non-back-to-back PUSCH transmissions within one slot is not supported.

Definition of the maximum duration: a maximum time duration during which UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

- FFS whether or not such a definition is necessary for RAN1 specifications.

  - Note: whether such a definition is to be specified in RAN4 specifications is up to RAN4.

- FFS the maximum duration may be reported by UE.

- Note: it is understood that for a UE, the maximum duration is no less than the time domain window duration

Send LS to RAN4 asking the following questions

- For joint channel estimation, is there a maximum duration during which UE is able to maintain power consistency and phase continuity under certain tolerance level? If any, how long is it?

  - What factors determine the maximum duration?

  - Whether the maximum duration should be the same for different cases for both PUSCH and PUCCH?

  - Whether the maximum duration is dependent on the modulation order of transmission, e.g., QPSK, 16QAM, 64QAM?

  - Whether the maximum duration is dependent on UL waveform (DFT-s-OFDM vs. OFDM)?

  - Whether the maximum duration is band specific?

  - Besides the factors listed above, whether or not the maximum duration is further dependent on UE capabilities (e.g., multiple possible values for a given set of factor(s)), and if so, whether the UE should report such a duration

- For back-to-back PUSCH transmissions within one slot, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A with consecutive slots

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

Joint channel estimation over back-to-back PUSCH transmissions with different TBs within one slot is not supported.

- Joint channel estimation for PUSCH transmissions is enabled or disabled via RRC configuration for a UE

  - FFS: whether additional dynamic signaling is needed to enable/disable joint channel estimation for PUSCH transmissions

  - Note: the enabling of such a feature is subject to certain prerequisites

  - FFS RRC parameter details (including explicit vs. implicit configuration)

- FFS For joint channel estimation for PUSCH, the time domain window is not explicitly enabled or disabled separately from joint channel estimation.

Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

Confirm the following working assumption

Working assumption:

- For non-back-to-back PUSCH transmissions (at least for the case of the same TB) across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type A scheduled by dynamic grant or configured grant.

  - Over non-back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

    - Only for single layer transmissions

    - Subject to UE capability

  - FFS: Over non-back-to-back PUSCH transmissions with different TBs

  - FFS: Over non-back-to-back PUSCH transmissions for TBoMS

  - For the non-back-to-back PUSCH transmissions, it is defined as at least when there is no UL transmission between the two successive PUSCH transmissions

  - Subject to UE capability with details FFS (e.g., separate vs. joint capability for type A & type B, w.r.t. OFF power requirements, etc.)

- FFS: Joint channel estimation over non-back-to-back PUSCH transmissions with other uplink transmissions between the two successive PUSCH transmissions across consecutive slot.

- Joint channel estimation for PUSCH transmissions and the time domain window are jointly enabled or disabled via RRC configuration for a UE.

  - Note: Enabling/disabling of joint channel estimation for PUSCH transmissions means enabling/disabling of DMRS bundling for PUSCH transmissions under the condition of power consistency and phase continuity.

- UE should not perform TA adjustment during the time domain window.

‐   FFS: UE does not expect to receive TA command to indicate TA adjustment during the TDW.

‐   FFS: UE ignores any TA command which indicates TA adjustment during the TDW.

‐   FFS: UE performs TA adjustment after the TDW if it receives any TA command indicating TA adjustment during the TDW.

Working assumption:

For joint channel estimation for PUSCH repetition type A of PUSCH repetitions of the same TB, all the repetitions are covered by one or multiple consecutive/non-consecutive configured TDWs.

Each configured TDW consists of one or multiple consecutive physical slots.

The window length L of the configured TDW(s) can be explicitly configured with a single value

‐   FFS: The maximum value of L

‐   FFS: Solutions to error propagation issue if  L is longer than the maximum duration is to be discussed further.

‐   FFS: The window length L is configured per UL BWP

The start of the first configured TDW is the first PUSCH transmission

‐   FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission.

The start of other configured TDWs can be implicitly determined prior to first repetition.

‐   FFS: The configured TDWs are consecutive for paired spectrum/SUL band

‐   FFS: The start of the configured TDWs for unpaired spectrum is implicitly determined based on semi-static DL/UL configuration.

The end of the last configured TDW is the end of the last PUSCH transmission.

‐   FFS: The end of the configured TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission.

Within one configured TDW, one or multiple actual TDWs can be implicitly determined:

‐   The start of the first actual TDW is the first PUSCH transmission within the configured TDW.

o    FFS: The first available slot/symbol, or the first physical slot/symbol for the first PUSCH transmission.

‐   After one actual TDW starts, UE is expected to maintain the power consistency and phase continuity until one of the following conditions is met, then the actual TDW is ended.

o    The actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

FFS: The end of the actual TDW is the last available slot/symbol, or the last physical slot/symbol for the last PUSCH transmission.

o    An event occurs that violates power consistency and phase continuity

FFS: The events may include e.g., a DL slot based on DL/UL configuration for unpaired spectrum, the actual TDW reaches the maximum duration, DL reception/monitoring occasion for unpaired spectrum, high priority transmission, frequency hopping, precoder cycling.

FFS: The end of the actual TDW is the last available slot/symbol of the PUSCH transmission right before an event such that the power consistency and phase continuity are violated.

‐   If the power consistency and phase continuity are violated due to an event, whether a new actual TDW is created is subject to UE capability of supporting restarting DMRS bundling.

o    If UE is capable of restarting DM-RS bundling, one new actual TDW is created after the event,

FFS: The start of the new actual TDW is the first available slot/symbol for PUSCH transmission after the event.

o    If UE is not capable of restarting DM-RS bundling, no new actual TDW is created until the end of the configured TDW.

o    FFS: UE capability of restarting DMRS bundling is applied only to dynamic event or not

Note 1: A ‘configured TDW’ refers to a time domain window whose length can be configured to ‘L’ and whose start and end is determined as described above.

Note 2: An ‘actual TDW’ refers to a time domain window during whose entire duration the DM-RS bundling is actually applied. An ‘actual TDW’ duration is always less than or equal to the ‘configure TDW’ duration.

Note 3: Whether the terms ‘configured TDW’ and ‘actual TDW’ are revised to other terms and if such terminology is used in specifications is to be further discussed.

It is agreed that

- For PUSCH repetition type A counting based on physical slots

  - The start of the first configured TDW is the first physical slot for the first PUSCH transmission.

  - The end of the last configured TDW is the last physical slot for the last PUSCH transmission.

- For PUSCH repetition type A counting based on available slots

  - The start of the first configured TDW is the first available slot for the first PUSCH transmission.

  - The end of the last configured TDW is the last available slot for the last PUSCH transmission.

  - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Conclusion

- Joint channel estimation over PUSCH transmissions across non-consecutive slots is not supported in Rel-17.

- For DG-PUSCH, Type1 CG-PUSCH and Type2 CG-PUSCH, the window length L of the configured TDW is at least configured by RRC.

- FFS: For DG-PUSCH and Type2 CG-PUSCH, whether the window length L of the configured TDW can be indicated by DCI or indicated by TDRA table with one additional entry.

- The window length L of the RRC configured TDW is configured separately for PUSCH and PUCCH.

  - For PUSCH, L is configured per BWP.

FFS whether the window length L can be configured with each row in the TDRA table

- For PUSCH repetition type A counting based on physical slots

  - The configured TDWs are consecutive, where the start of other configured TDWs is the first physical slot right after the last physical slot of a previous configured TDW.

- For PUSCH repetition type A counting based on available slots

  - The configured TDWs are determined based on available slots, where start of a configured TDWs is the next first available slot after the conclusion last available slot of a previous configured TDW.

  - Note: The determination of available slots for PUSCH repetition Type A is defined in AI 8.8.1.1.

Working Assumption

Support Actual TDW Option 2b’:

- The start of the first actual TDW is the first available symbol (at least determined by TDRA table) in available slot for the first PUSCH transmission in an available slot within the configured TDW.

- The end of the actual TDW is

  - the last available symbol (at least determined by TDRA table) in available slot for the last PUSCH transmission in an available slot within the configured TDW if the actual TDW reaches the end of the last PUSCH transmission within the configured TDW.

  - the last available symbol (at least determined by TDRA table) in available slot of the PUSCH transmission right before the event if an event occurs that violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

  - For UE capable of restarting DM-RS bundling, the start of the new actual TDW is the first available symbol (at least determined by TDRA table) in available slot for PUSCH transmission after the event violates power consistency and phase continuity, and the PUSCH transmission is in an available slot.

         For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

    - if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A

For non-back-to-back PUSCH transmissions across consecutive slots (no uplink transmission in the middle of two PUSCH transmissions), support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following case:

  - Over non-back-to-back PUSCH transmissions for one TB processed over multiple slots

    - It’s subject to UE capability

    - if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A

Down-select one of the following options:

   Option 1: If DM-RS bundling is supported, UE is mandatory to support restarting DM-RS bundling due to semi-static events. UE capability of restarting DMRS bundling is applied only to dynamic events.

   Option 2: UE capability of restarting DMRS bundling is applied to both semi-static events and dynamic events.

Support at least the following events that violate power consistency and phase continuity.

‐   Dropping/cancellation based on Rel-15/16 collision rules.

‐   FFS: Rel-17 collision rules.

‐   DL slot or DL reception/monitoring based on semi-static DL/UL configuration for unpaired spectrum.

‐   FFS: Other UL transmission in between PUSCH/PUCCH transmissions.

‐   Gap between two PUSCH/PUCCH transmissions exceeds 13 symbols.

‐   FFS: Transmission parameters need to be changed due to network-indicated operations, including: Tx power, UL beam/TPMI, and RB allocation.

‐   FFS: TPC command.

‐   FFS: TA adjustment.

‐   FFS: The actual TDW reaches the maximum duration.

‐   FFS: Frequency hopping.

‐   FFS: Precoder cycling.

‐   FFS: other events.

‐   FFS: whether events are semi-static events or dynamic events.

‐   FFS: the time duration of an event.

Down-select one of the following options in this meeting:

Option 1:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

Option 1’:

- The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

    - If L is not configured, the configured TDW length is equal to all repetitions

    - If L is not configured, default behavior should be defined, e.g., the configured TDW length is equal to all repetitions

Option 3’:

- Whether the window length L of the configured TDW can be longer than maximum duration is subject to UE capability.

  - If UE is capable of L being longer than maximum duration,

    - The maximum value of the window length L of the configured TDW is the duration of all repetitions.

      - FFS: whether L cannot be other values other than the duration of all repetitions, if it is longer than the maximum duration.

    - If L is longer than the maximum duration, UE does not expect dynamic events.

      - FFS: details of dynamic events

By email latest version:

Option 1’-a:

   The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

‐   If L is not configured, the UE assumes the default value where the default value does not exceed the maximum duration.

Option 1’-b:

   The maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements.

‐    If L is not configured, the UE assumes the default value, with where the default value may or may not exceeding the maximum duration.