3GPP TSG- RAN WG1 Meeting #107-e                                              R1- 2111856

# e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.1.2.3

Source:	Apple Inc.

Title:	On Multi-TRP Beam Management Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #106b meeting, the following agreements on multi-TRP beam management enhancement have been achieved.

Agreement - Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. - FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR - A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case. Agreement - RACH-based transmission can be triggered on a SpCell at least in the following scenarios - Scenario 1: When beam failure is detected on all BFD-RS sets on the SpCell - FFS: other scenarios - Scenario 2: at least one TRP fails on SpCell - Scenario 3: at least one pre-defined TRP fails on SpCell - Scenario 4: at least one TRP fails and no PUCCH-SR is configured, and no UL grant is available - Scenario 5: If MAC-CE based reporting does not work (details FFS) - Scenario 6: When no PUCCH-SR is configured Agreement To associate BFD-RS set k and NBI-RS set j - Alt-1: 1-to-1, fixed in spec - Whether NBI-RS configuration is mandatory is separate discussion Conclusion Design of MAC-CE related to SpCell when transmitted on msg3, msgA is up to RAN2. Agreement For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with CORESETPoolIndex k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD -RS set k (k=0,1) in the MAC-CE for TRP -specific BFR - The above applies to Scell and SpCell - The above applies for the multi-DCI case Agreement SCS of the 28 symbols is the smallest SCS of the active DL BWP for the response reception CC and of the active DL BWP (s) of the CC(s) with the failed TRP link(s) reported in BFR MAC CE. Agreement #### For RACH-based transmission, at least when all BFD-RS sets fail in SPCell, CBRA is supported

In this contribution, we provide some discussion on multi-TRP beam management enhancement.

# Discussion

## Beam Measurement and Report

### UE panel relation info report

In RAN1 #104, option 2 on beam reporting was agreed with a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously.

One open issue is whether to support UE to indicate the beams are received by a single panel or multiple panels. This could be a useful information to decide the maximum possible rank. It is reasonable to support such report and in addition, UE can also report the maximum number of layers per Rx beam.

Proposal 1: Support UE reports whether beams are received with different RX beams or not

- Support UE reports maximum number of layers per Rx beam as a UE capability

### Effective time window for a group-based beam report

For the enhanced group-based beam report, the beams reported in one pair can be received simultaneously. However, after UE reports one beam pair, it does not mean the two beams can always be received simultaneously. An effective time window for each beam report instance needs to be defined. Only the reported beams within the effective time window can be assumed to be received simultaneously. Thus, a UE capability on the duration for the effective time window for a group-based beam report should be supported.

Proposal 2: The beams in a beam pair reported in a group-based beam report instance can be simultaneously received by UE within an effective time window.

- The duration of the effective window can be reported by UE capability

### Group-based L1-SINR report

Another open issue is whether to support L1-SINR based on option 2. It might be possible that option 2 could provide some benefit for inter-beam interference. However, with regard to potential overlap between beam pairs, it is impossible for UE to measure inter-beam interference, due to UE Rx beam constraints. Figure 1 shows one example for this UE Rx beam issue. Thus, the benefit to support L1-SINR based on option 2 is questionable. The key motivation for current option 2 is to report whether 2 beams can be received simultaneously.

Figure 1: Potential issue for inter-beam interference measurement

Figure 2 and Figure 3 illustrates the performance between L1-RSRP and L1-SINR based beam selection for mTRP operation. It can be observed that without inter-beam interference measurement, the performance for L1-SINR could be quite similar to L1-RSRP, while with inter-beam interference measurement, there can be some performance loss for cell edge UEs. Assuming each CMR set contains N CMRs, and there are M beam at UE side, the UE effort to identify the best beam pair could be as follows:

- For L1-RSRP, UE needs to measure 2*N*M L1-RSRP

- For L1-SINR without inter-beam interference, UE needs to measure 2*N*M L1-SINR

- For L1-SINR with inter-beam interference, UE needs to measure N*M*N*M L1-SINR

Thus L1-SINR related group-based beam reporting enhancement cannot provide performance benefit, but it increases the UE complexity, which should not be supported. The detail simulation assumption is in Table A-1.

Figure 2: Geometry SINR for the first panel

Figure 3: Geometry SINR for the second panel

Proposal 3: Do not support option 2 to measure L1-SINR for beam measurement enhancement.

## BFR Enhancement

### Beam failure detection

It was agreed that 2 BFD RS sets can be used for TRP-specific BFR. One open issue is how to derive the BFD RS. With regard to high latency to update the BFD RS by RRC, it is better to consider some mechanism like DCI/MAC CE based BFD RS update, which can be aligned with TCI states. One possible way is to use the same approach as PL-RS, where a BFD RS selected from the candidate 2 BFD RS sets can be associated with a TCI state as shown in Figure 2. Then it can support more flexible BFD RS update and UE can derive potential failed BFD RS set for single-DCI mode.

Figure 2: BFD RS and TCI state association

Proposal 4: Support to configure a BFD RS index selected from the 2 candidate BFD RS sets configured by RRC to be associated with a TCI state

For single DCI mode, since dynamic switching between sDCI and sTRP mode is supported, when it is switched back to sTRP mode, UE does not need to perform BFD. UE may still maintain the BFD counter and timer within a short time in case UE switches back to sDCI mode.

Proposal 5: For sDCI mode, UE should not be required to always keep TRP-specific BFR when it switches to sTRP mode

- UE may maintain the BFD counter and timer within a time window in case it switches back to sDCI mode

### Candidate Beam Detection

With regard to inter-cell multi-TRP operation, the non-serving cell SSB can be configured for candidate beam detection. As different cells may have different Tx power, the RSRP threshold for candidate beam detection can be configured in a cell-specific manner.

Proposal 6: Support to configure non-serving cell SSB for candidate beam detection.

Proposal 7: Support to configure cell-specific candidate beam detection RSRP threshold.

### Beam failure recovery request procedure

In RAN1 #106b, the following agreement on 2 PUCCH-SR is achieved.

Agreement - Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR. - FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR - A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.

One open issue is which PUCCH-SR should be triggered when one BFD RS set fails. Since the association is configured by gNB, the associated PUCCH-SR should be triggered when corresponding BFD RS set fails. Whether the associated PUCCH-SR and the BFD RS set belong to the same TRP or not can be transparent to UE. If multiple PUCCH-SRs are triggered, legacy SR multiplexing/dropping rule can be reused.

Proposal 8: When one BFD RS set fails, UE can trigger the associated PUCCH-SR

- If multiple PUCCH-SRs are triggered, legacy SR multiplexing/dropping rule can be reused

## QCL-TypeD collision handling

In Rel-15, some QCL-TypeD collision handling rules have been defined as shown in Table 1.

Table 1: QCL-TypeD collision handling rules

| Signal 1 | Signal 2 | Behavior |
| --- | --- | --- |
| PDSCH (scheduling offset before threshold) | PDCCH | Prioritize PDCCH. |
| PDSCH | SSB not used for RLM/CBD | gNB should ensure QCL. |
| CSI-RS not used for RLM/BFD/RLM | PDCCH | Not allowed for CSI-RS with repetition=on, for other CSI-RS, gNB should ensure QCL. |
| CSI-RS not used for RLM/BFD/CBD | SSB not used for RLM/CBD | gNB should ensure same QCL. |
| Aperiodic CSI-RS for CSI (scheduling offset below threshold) | Other signal | Prioritize other signal. |
| PDCCH | PDCCH | Follow a priority rule |
| SSB/CSI-RS for RLM/BFD | Other DL signal | Not allowed |
| SSB/CSI-RS for CBD | Other DL signal | Not allowed |

In Rel-17, inter-cell mTRP operation is going to be supported, which would create new cases:

- Case 1: SSB + SSB from a different PCI

- Case 2: SSB + PDSCH/PDCCH/CSI-RS QCLed with SSB from a different PCI

The 2 new cases above need to be resolved. For Case 1, a simple dropping rule should not be helpful, since UE needs to measure different SSBs from different cells. Case 2 may be connected with Case 1, but UE may not be able to receive both signals at the same time especially if the SSB is used for RLM/BFD/CBD or the SSB is from a third cell,

Proposal 9: For simultaneous reception, RAN1 should prioritize the following new cases, which has not been discussed in Rel-15:

- Case 1: SSB + SSB from a different PCI

- Case 2: SSB + PDSCH/PDCCH/CSI-RS QCLed with SSB from a different PCI

# Conclusion

In this contribution, we discussed multi-TRP BM enhancement. Based on the discussion, the following proposals have been achieved.

Beam report

Proposal 1: Support UE reports whether beams are received with different RX beams or not

- Support UE reports maximum number of layers per Rx beam as a UE capability

Proposal 2: The beams in a beam pair reported in a group-based beam report instance can be simultaneously received by UE within an effective time window.

- The duration of the effective window can be reported by UE capability

Proposal 3: Do not support option 2 to measure L1-SINR for beam measurement enhancement

BFR

Proposal 4: Support to configure a BFD RS index selected from the 2 candidate BFD RS sets configured by RRC to be associated with a TCI state

Proposal 5: For sDCI mode, UE should not be required to always keep TRP-specific BFR when it switches to sTRP mode

- UE may maintain the BFD counter and timer within a time window in case it switches back to sDCI mode

Proposal 6: Support to configure non-serving cell SSB for candidate beam detection.

Proposal 7: Support to configure cell-specific candidate beam detection RSRP threshold.

Proposal 8: When one BFD RS set fails, UE can trigger the associated PUCCH-SR

- If multiple PUCCH-SRs are triggered, legacy SR multiplexing/dropping rule can be reused

Simultaneous reception

Proposal 9: For simultaneous reception, RAN1 should prioritize the following new cases, which has not been discussed in Rel-15:

- Case 1: SSB + SSB from a different PCI

- Case 2: SSB + PDSCH/PDCCH/CSI-RS QCLed with SSB from a different PCI

# Appendix – Simulation assumption

Table A-1: Simulation Assumption

| Parameter | Value |
| --- | --- |
| Scenario | Dense Urban Macro |
| Number of BS | 21 |
| Number of UE | 210 |
| gNB antenna architecture (M, N, P, Q) | (4, 8, 2, 2) |
| UE antenna architecture (M, N, P, Q, Mg, Ng) | (1, 4, 2, 2, 1, 2) |
| Number of beams per TRP | 32 |
| Number of beams per UE panel | 4 |
| UE speed | 3 km/h for indoor and 30km/h for outdoor |
| Indoor UE percentage | 80% |
| System bandwidth | 80 MHz |
| Carrier frequency | 30 GHz |
| Subcarrier spacing | 120 kHz |
| UE and panel orientation | Vertical but random in azimuth |