3GPP TSG RAN WG1 #107-e	                                         	      R1-2111857

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.1.2.4

Source:	Apple Inc.

Title:	Views on Rel-17 HST enhancement

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Within the scope of multi-TRP enhancement, one sub-objective is the High-Speed Train (HST) enhancement as highlighted as quoted below from [1]

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline - Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In the last RAN1 meeting RAN1#106bis e-meeting, the following agreement is reached [2]

Agreement Support combination of Rel-17 SFN PDCCH scheme 1 and single-TRP PDSCH - This is optional UE feature - Note: The support of such combination scheme is for URLLC use-case only. Agreement Enhanced SFN (scheme 1 or TRP-based pre-compensation scheme) for PDCCH and PDSCH is configured by using separate per-BWP RRC parameters - In Rel-17, all downlink BWPs (except initial BWP and FFS: BWP-DownlinkCommon) within a CC should be the same configuration of SFN scheme Working Assumption Reuse legacy Rel-16 RRC parameters simultaneousTCI-UpdateList1, simultaneousTCI-UpdateList2 to define set of the serving cells which can be addressed by a single MAC CE for activation of two TCI states of CORESET with the same CORESET ID for all the BWPs. Agreement If CSI-RS other than those configured with repetition set to 'on' is overlapping in the time domain with CORESET with two TCI states, support the first TCI state of the CORESET as the default TCI assumption for the CSI-RS. Agreement When SFN PDSCH is not configured by RRC, for PDSCH reception scheduled by DCI format 1_0, 1_1, 1_2, if the time offset between the reception of the DL DCI and the corresponding PDSCH is smaller than the threshold timeDurationForQCL, - For DCI format 1_1/1_2, support both configurations with and without TCI state field. - [If enableTwoDefaultTCIStates is not configured,] for both cases with and without TCI state field, - If enhanced SFN PDCCH transmission scheme 1 is configured and the lowest CORESET ID in the latest slot is indicated with two TCI states, select the 1st TCI state of the two TCI states of the CORESET as default beam for the PDSCH reception - FFS: Whether above applies for TRP-based pre-compensation if TRP-based pre-compensation is agreed to be support in FR2 - Otherwise, UE applies the one active TCI state of the CORESET with the lowest controlResourceSetId in the latest slot when receiving the PDSCH Agreement For CSS associated with SFN CORESET, study the following alternatives and down-select in RAN1#107e: - Alt 2: UE doesn’t expect PDCCH candidates in CSS to be associated with CORESET activated with two TCI states, except for CSS type 3 associated with CORESET configured with scheme 1 - Alt 3: If PDCCH candidates in CSS 0/0A/1/2/3 are associated with CORESET that activated with two TCI states, the first TCI state is applied for the CSS reception, except for CSS type 3 associated with CORESET configured with scheme 1. - For CSS type 3 associated with CORESET configured with scheme 1, both TCI states can be applied for the CSS reception. Agreement When CORESET is indicated with two TCI states - One BFD RS pair for SFN CORESET is counted as two BFD RSs - FFS: Increase the maximum number of monitored BFD RSs to X. - X is UE capability - X = 2, 3, 4, FFS other values of X Agreement When two TCI states are activated for a CORESET, NBI RS can be configured as follows - Alt 4-1: Using the existing Rel-15 NBI configuration based on single SSB / CSI-RS resource - FFS addition support of Alt 4-2: two new beam identification CSI-RS resource sets / new beam identification CSI-RS resource pairs or SSB pairs

In this contribution, we provide our views on the Rel-17 FeMIMO HST enhancement.

# High Doppler shift in HST deployment

The major difficulty or challenge in HST deployment comes from handling of high Doppler shift due to the combination of high device movement speed and potential deployment at high band. When a UE in the high-speed train travels at very high speed between two TRPs, UE would observe a high positive Doppler shift to one of the TRP, and high negative Doppler shift to the other TRP.

We use an example to demonstrate the issues that we need to solve for HST deployment

- Train Speed: 400km/hr

- Carrier frequency: 5GHz

- Location of the UE: equal distance between two TRP

- 30kHz SCS

Based on the above assumptions, the device will observe around +1850Hz Doppler shift from one TRP and -1850Hz Doppler shift from the other TRP. As results, when two TRPs perform SFN transmission, the composite signal at the UE side will experience 3700Hz sinusoid fading as illustrated in Figure 1.

From Figure 1., it is easy to observe that the channel experiences a very fast fading, i.e., the channel completes a complete sinusoid cycle within about half a slot, i.e., ~ 8 symbols.

Figure 1 Amplitude of composite SFN channel

However, after further looking at the HST problem, we have the following two observations which lead to two directions for the HST enhancement

- Even though the composite channel appears to fade very fast, it is primarily due to the high Doppler shift instead of Doppler spread. Therefore, if UE can estimate the Doppler shift, UE can improve its channel estimate by performing parametric enhanced channel estimate

  - This leads to the DMRS QCL enchantment

- Even with perfect channel estimation, the channel capability still suffers due to the channel variation. If gNB can pre-compensate for the Doppler shift and perform the right phase compensation, gNB can ensure the composite channel to always have the strongest amplitude without Doppler shift

  - This leads to the gNB doppler pre-compensation enhancement

We will discussion the two directions at high level in the following two sub-sections

## HST-SFN scheme 1

To assist the UE channel estimation, gNB needs to transmit two TRS, one TRS from each TRP. Then, UE can estimate the individual Doppler shift from each TRP which can be used in the parametric channel estimation to improve the channel estimate quality. The main issue we need to resolve is how to indicate that DMRS is QCL to two TRS.

In Rel-16, for Single-DCI based Multi-TRP solution, we already introduced TCI codepoint enhancement to allow a single TCI point to contain up to 2 TCI states. Furthermore, TCI codepoint, together with RRC configuration, TDRA field in DCI and antenna ports configuration in DCI, is used to enable 5 different Single-DCI Multi-TRP schemes, including (1) SDM (2) FDMSchemeA (3) FDMSchemeB (4) TDMSchemeA (5) Scheme 4.

As part of the remaining issue from the last meeting, i.e.,  RAN1#104be, we agreed that

- Switching between HST scheme 1 and Rel-15 DPS (Dynamic Point Selection) can be DCI based, but it is UE optional feature

- Switching between HST scheme 1 and Rel-16 Single-DCI Multi-TRP scheme 1a (SDM) is semi-static (RRC) based, whether to support dynamic switching (DCI based) between HST-SFN scheme 1 and Rel-16 Single-DCI Multi-TRP scheme 1a is for FFS

Therefore, we have the following proposals

Proposal 1: Do not support DCI based dynamic switching between HST-SFN scheme 1 and Rel-16 Single-DCI Multi-TRP scheme 1a

## HST-SFN with gNB Doppler shift pre-compensation

For gNB to perform efficient Doppler shift pre-compensation, gNB needs to know the Doppler shift between the UE and each TRP. To obtain Doppler shift, there are two directions

- gNB acquires the Doppler shift based on the UE SRS sounding

- gNB acquires the Doppler shift based on the UE measurement reporting

There are still other remaining issues on the following three topics (1) mixed mode of PDCCH and PDSCH for HST operation (2) Default beam enhancement (3) BFD/RLM

Proposal 2: For mixed Single-TRP and Multi-TRP operation mode configuration for PDCCH and PDSCH operation

- Do not support HST-SFN with pre-compensation PDCCH together with sTRP PDSCH

Proposal 3: If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, regarding the configuration of RS for BFD

- For implicit configuration, RS of CORESETs with both single and two TCI states can be used, the selection of the RS is up for UE implementation while all the RSs are counted as the total number of RSs for the maximum number of RSs UE can monitor for BFD

- For explicit configuration, support defining CSI-RS resource or SSB pairs as BFD RS under the condition that hypothetic PDCCH BLER measurement is up for UE implementation without further RAN4 requirement

# Conclusion

In RAN#86 meeting, Rel-17 FeMIMO WI was approved in RP-193133 [1]. Four main areas haven been identified for the Rel-17 FeMIMO, namely

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2

- Enhancement on SRS, targeting both FR1 and FR2

- Enhancement on CSI measurement and reporting

Within the scope of multi-TRP enhancement, one sub-objective is the High-Speed Train (HST) enhancement. In this contribution, we provide our views on the Rel-17 FeMIMO HST enhancement with the following proposals

Proposal 1: Do not support DCI based dynamic switching between HST-SFN scheme 1 and Rel-16 Single-DCI Multi-TRP scheme 1a

Proposal 2: For mixed Single-TRP and Multi-TRP operation mode configuration for PDCCH and PDSCH operation

- Do not support HST-SFN with pre-compensation PDCCH together with sTRP PDSCH

Proposal 3: If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, regarding the configuration of RS for BFD

- For implicit configuration, RS of CORESETs with both single and two TCI states can be used, the selection of the RS is up for UE implementation while all the RSs are counted as the total number of RSs for the maximum number of RSs UE can monitor for BFD

- For explicit configuration, support defining CSI-RS resource or SSB pairs as BFD RS under the condition that hypothetic PDCCH BLER measurement is up for UE implementation without further RAN4 requirement

# Reference

- RP-193133, "New WID: Further enhancements on MIMO for NR", 3GPP TSG RAN Meeting #86, Sitges, Spain, December 9-12, 2019

- Chairman Notes, 3GPP TSG RAN WG1 #106b-e e-Meeting, October 11th – 19th, 2021