3GPP TSG RAN WG1 #107-e		R1-2111863

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.2.3

Source:	Apple Inc.

Title:	Discussion on Enhancements for PUCCH formats 0_1_4

Document for:	Discussion/Decision

# Introduction

Based on the SI [1] [2], an updated WID was agreed to in RAN #90-e with the following objectives [3] :

- In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

  - Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

Due to the regulatory limits in shared spectrum, the single Resource Block (RB) transmission required by some PUCCH formats may result in a limit on the transmit power even if the UE is capable of transmitting at higher power levels. To achieve higher transmit power for PUCCH transmission, the WID has the following objective:

- Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

In this contribution, we address enhancements to PUCCH formats 0, 1 and 4 for the new SCS 480 kHz and 960 kHz in the new frequency range.

# Discussion

## PUCCH Background and Motivation

In Rel-15/Rel-16 NR, the following PUCCH formats are specified (see Table 1) [4]:

Table 1: PUCCH Formats

| Format | Payload | Payload Size | Duration (symbols) | Resource Blocks | DMRS | Waveform | Multiplexing |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | SR/HARQ-ACK | 1 / 2 bits | 1 or 2 | 1 | No | CP-OFDM | CG sequence + cyclic shift |
| 1 |  |  | 4 to 14 | 1 | Yes |  | CG sequence + TD-OCC |
| 2 | SR/HARQ-ACK, CSI | > 2 bits | 1 or 2 | 1-16 |  |  | None |
| 3 |  |  | 4 to 14 | 1-16 |  | DFT-S-OFDM |  |
| 4 |  |  | 4 to 14 | 1 |  |  | CG sequence + FD-OCC |

As seen from the table, Formats 0, 1 and 4 are transmitted in a single Resource Block (RB) while Formats 2 and 3 can be transmitted with more that 1 RB.

As an example, European regulations [5] require a maximum power spectral density when operated at the highest power spectral density level (PSL) and a maximum RF output power applicable when operated at the highest stated power level. The spectral Power Density Limit (PDL) is 23 dBm/MHz and the maximum power level is 40 dBm with different values for other regulatory administrations. For the agreed upon SCSs for NR operation between 52.6 GHz and 71 GHz, Table 2 shows the corresponding power levels assuming a single RB is transmitted with the UE transmitting at the PDL.

Table 2 : EIRP with 1 RB for 23 dBm/MHz PDL

As seen in the table, with the 23 dBm/MHz PDL, the PUCCH transmission is unable to transmit at the maximum power level of 40 dBm if the transmission is limited to 1 RB only. Ideally, we would want to ensure that the # RBs are set to make the transmission power level limited as opposed to PDL.

As such, in RAN1 #103-e [7], RAN1 #104-e [8], RAN1 #104bis-e [9], RAN1 #105-e [10], RAN1 #106-e [11], and RAN1 #106-bis-e [12] ,agreements were made to support allocation of N_RB contiguous RBs for PF0, PF1 and PF4. The remaining issue to finalize the design of the PUCCH Resource Sets prior to RRC configuration.

## PUCCH Resource Sets Prior to RRC Configuration

To define the PUCCH Resource Sets prior to RRC configuration that may be used during initial access or in scenarios where the UE has not been allocated any dedicated PUCCH resources, it is necessary to modify the existing Table defined in table 9.2.1-1 of 38.213 (See Table 3 below) [6]. The current table assumes PF0/1 with NRB =1 and based on the current agreements in RAN1, NRB >1 needs to be defined for different SCSs. In RAN1 #106-e, the following agreement was made:

Agreement: - For PUCCH resource sets prior to RRC configuration, support a parameter in SIB1 that indicates the number of RBs for enhanced (multi-RB) PUCCH format 0/1

While in RAN1 #106-bis-e, the following agreements were made:

### Agreement: - Reuse the existing Rel-15/16 PUCCH configuration Table 9.2.1-1 in 38.213 for configuration of PUCCH resource sets prior to dedicated PUCCH configuration for multi-RB PUCCH formats 0/1 - As previously agreed, the number of RBs for each PUCCH resource in a set is N_RB which is signaled in SIB1 - The lowest-indexed RB for each PUCCH resource is a function of N_RB - The following example change to 38.213 Section 9.2.1 can be recommended to the editor of 38.213 to use at the editor’s discretion (subject to resolution of the below FFS on the value of X) ---- Start ---- If rPUCCH8=0 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the lowest PRB index of the PUCCH transmission in the first hop as RBBWPoffset∙X +rPUCCHNCS∙NRB and the lowest PRB index of the PUCCH transmission in the second hop as NBWPsize-RBBWPoffset∙X -1+rPUCCHNCS∙NRB, where NCS is the total number of initial cyclic shift indexes in the set of initial cyclic shift indexes - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as rPUCCHmodNCS If rPUCCH8=1 and a UE is provided a PUCCH resource by pucch-ResourceCommon and is not provided useInterlacePUCCH-PUSCH in BWP-UplinkCommon - the UE determines the lowest PRB index of the PUCCH transmission in the first hop as NBWPsize-RBBWPoffset∙X -1+rPUCCH-8NCS∙NRB and the lowest PRB index of the PUCCH transmission in the second hop as RBBWPoffset∙X +rPUCCH-8NCS∙NRB - the UE determines the initial cyclic shift index in the set of initial cyclic shift indexes as ---- End ---- ### Agreement: - In the RAN1#106bis-e agreement on construction of PUCCH resource sets prior to dedicated PUCCH configuration, the following is supported at least for PUCCH resource set indices 0 .. 14 in Table 9.2.1-1 (Alt-1 in the agreement): - FFS: Down select to one of the following alternatives for PUCCH resource set index 15 - Alt-a: - Alt-b: Alternative handling (to be defined)

With this agreement, the Table that indicates the PUCCH resource sets before dedicated PUCCH resource configuration in Rel-15 can be directly re-used. The UE assumes that the number of RBs indicated in SIB1 can be used with any row of the table.

Table 3: PUCCH resource sets before dedicated PUCCH resource configuration

| Index | PUCCH format | First symbol | Number of symbols | PRB offset | Set of initial CS indexes |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 12 | 2 | 0 | {0, 3} |
| 1 | 0 | 12 | 2 | 0 | {0, 4, 8} |
| 2 | 0 | 12 | 2 | 3 | {0, 4, 8} |
| 3 | 1 | 10 | 4 | 0 | {0, 6} |
| 4 | 1 | 10 | 4 | 0 | {0, 3, 6, 9} |
| 5 | 1 | 10 | 4 | 2 | {0, 3, 6, 9} |
| 6 | 1 | 10 | 4 | 4 | {0, 3, 6, 9} |
| 7 | 1 | 4 | 10 | 0 | {0, 6} |
| 8 | 1 | 4 | 10 | 0 | {0, 3, 6, 9} |
| 9 | 1 | 4 | 10 | 2 | {0, 3, 6, 9} |
| 10 | 1 | 4 | 10 | 4 | {0, 3, 6, 9} |
| 11 | 1 | 0 | 14 | 0 | {0, 6} |
| 12 | 1 | 0 | 14 | 0 | {0, 3, 6, 9} |
| 13 | 1 | 0 | 14 | 2 | {0, 3, 6, 9} |
| 14 | 1 | 0 | 14 | 4 | {0, 3, 6, 9} |
| 15 | 1 | 0 | 14 |  | {0, 3, 6, 9} |

However, with PUCCH Resource Set 15, any value of X > 1 results in a PRB offset of NBWPsize/4∙NRB  ≥ NBWPsize/4∙2 implying there may be no PUCCH resources available for transmission. Secondly, for the resources available for PUCCH transmission, 4NRB resource blocks are required to provide up to 16 PUCCH resources. To resolve this problem, there are two options:

- Option 1: X = N_RB, which implies that by gNB implementation, PUCCH Resource Set 15 will not be usable if N_RB > 1 and the UE will not expect the gNB to configure this value.

- Option 2: X=1, which leaves NBWPsize/2 RBs available for PUCCH resources. It will be up to gNB implementation to configure  the values of NBWPsize and NRB to ensure that there are enough PUCCH resources.

To allow for some use of PUCCH Resource set 15, option 2 is the better choice.

Proposal 1: For the construction of PUCCH resource sets prior to dedicated PUCCH configuration, X = 1  for PUCCH resource set index 15.

- It will be up to gNB implementation to configure  the values of NBWPsize and NRB to ensure that there are enough PUCCH resources.

# Conclusion

In this contribution, we have discussed enhancements to PUCCH formats PF0, PF1 and PF4 for NR  operation in the band > 52.6 GHz and have the following proposal:

Proposal 1: For the construction of PUCCH resource sets prior to dedicated PUCCH configuration, X = 1  for PUCCH resource set index 15.

- It will be up to gNB implementation to configure  the values of NBWPsize and NRB to ensure that there are enough PUCCH resources.

# Reference

- RP-193259, New SID: Study on supporting NR from 52.6GHz to 71 GHz

- 3GPP TR 38.808 V1.0.0 (2020-12, Study on supporting NR from 52.6 GHz to 71 GHz (Release 17)

- RP-202925,  3GPP Work Item Description on Extending current NR operation to 71GHz

- 3GPP TS 38.211 V16.4.0 (2020-12), Physical channels and modulation, (Release 16)

- Draft ETSI EN 302 567 v2.1.21 (2020-12), "Multiple-Gigabit/s radio equipment operating in the 60GHz band: harmonized standard for access to radio spectrum"

- 3GPP TS 38.213 V16.4.0 (2020-12), Physical layer procedures for control, (Release 16)

- Chairman’s Notes, RAN1 #103-e, October 2020

- Chairman’s Notes, RAN1 #104-e, January 2021

- Chairman’s Notes, RAN1#104bis-e, March 2021

- Chairman’s Notes, RAN1 #105-e, May 2021

- Chairman’s Notes, RAN1 #106-e, August 2021

- Chairman’s Notes, RAN1 #106-bis-e, October 2021

- R1-2110449 FL Summary #2 for [106bis-e-NR-52-71GHz-03] Email discussion/approval on enhancements for PUCCH formats 0/1/4	Moderator (Ericsson)