3GPP TSG RAN WG1 #107-e	R1- 2111875

e-Meeting, November 11th – 19th, 2021

Agenda Item:	8.5.2

Source:	Apple Inc.

Title:	Accuracy enhancements for UL-AoA positioning technique

Document for:	Discussion, Decision

# Introduction

In 3GPP RAN#90-e, a work item for Rel-17 NR positioning enhancements was approved and was updated in RAN#91-e [1]. One of the objectives of Rel-17 positioning WI, is to specify solutions to enhance positioning accuracy, as it can be found below [1]:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

In this contribution, we present our views on potential techniques and required signalling to enhance NR positioning accuracy for angle-based positioning technique. More specifically, in this paper we discuss UL-AoA positioning technique and in our companion contribution [2], we consider solutions to enhance accuracy for DL-AoD positioning technique.

# Enhancements to UL-AOA Reporting and Measurements

In RAN1#106-e, it was agreed that [3]

Agreement:

Reporting of one UL-RTOA and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning and per SRS resource for MIMO in a single gNB report to LMF is supported

- The above measurements are associated with SRS resource ID which is also reported to LMF

- FFS: Reporting of RSRP for the first arrival path

- Note: The use of SRS for MIMO resource is transparent to the UE

- FFS: Reporting of gNB Rx-Tx

Agreement:

Reporting of one gNB Rx-Tx time difference and multiple UL-AOAs measurements for the first arrival path per SRS resource for positioning in a single gNB report to LMF is supported

- The above measurements are associated with SRS resource ID which is also reported to LMF

- FFS: Reporting of RSRP for the first arrival path

Later in RAN1#106b-e, it was agreed that [4]

Agreement:

- For the first arrival path measurements on SRS for positioning resource,

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one-gNB Rx-Tx time difference}

  - FFS additional option: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference}

  - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF

- For the first arrival path measurements on SRS for MIMO resource,

  - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - FFS: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA}

  - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF

  - Note: The operation of SRS for MIMO is transparent to the UE

In our view, to mitigate the NLoS impact, UL-AOA measurements by TRP can be restricted to the first detected. Although it should be noted that even under this technique, which is mainly TRP implementation-based, the multi-path effect is not completely mitigated. An example is shown in Figure 1, where the pair link 1, although associated with LoS Tx/Rx beams at UE/TRP, is experiencing smaller RSRP than that of pair link 2 which is associated to the reflection path. In this example, due to blockage, the first arrival path on beam 2 may be not detected (e.g., completely blocked), so the first “detected” arrival path based on pair link 2 is indeed a NLoS path. To mitigate this issue, gNB may report the UL-AoA measurements associated with the first detected path from two positioning SRS resources, one with the smallest UL-RTOA and another one with the largest SRS-RSRP, if the two metrics belong to two different positioning SRS resources. Based on what we discussed, the following is proposed:

Proposal 1: For UL-AOA positioning, when multiple positioning SRS resources are received, gNB shall report UL-AoA measurements associated to the first arrival path of two positioning SRS resources; the one with the largest SRS-RSRP and another one with the shortest UL-RTOA.

Figure 1: UL-AoA measurements associated to the first detected path of SRS with shortest RTOA and SRS with largest SRS-RSRP.

# Conclusions

In this contribution, we provided our thoughts on potential techniques to enhance the positioning accuracy for the NR UL-AoA positioning technique. Based on views shared in this paper, we make the following proposals:

Proposal 1: For UL-AOA positioning, when multiple positioning SRS resources are received, gNB shall report UL-AoA measurements associated to the first arrival path of two positioning SRS resources; the one with the largest SRS-RSRP and another one with the shortest UL-RTOA.

# References

- RP-210903, “New WID on NR Positioning Enhancements”, March 2021

- R1- 2111876, Accuracy enhancements for DL-AoD positioning technique, Apple Inc.

- Chairman’s Notes RAN1#106-e final

- Chairman’s Notes RAN1#106b-e final